3GPP TSG RAN WG1 #112bis-e	                              R1-2303251

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Discussion on disabling of HARQ feedback for IoT NTN

Agenda item:	9.9.3

Document for:	Discussion & Decision

- Introduction

During last RAN1 meeting, the respective configurations and scheduling restrictions on disabling of HARQ feedback in IoT NTN have been discussed, and some progress were achieved [1].

In this contribution, we will further discuss and share our views on disabling of HARQ feedback for IoT NTN, including the detailed configuration/indication and other enhancements on disabling of HARQ feedback for IoT NTN.

- Discussion

  - 2.1 Configuration/indication of disabling HARQ feedback

Disabling of HARQ feedback is beneficial to throughput improvement and latency reduction. In addition, disabling of HARQ feedback may especially bring more benefits to the GEO network scenarios (e.g. GEO with RTT about 500 ms) due to the transmission waiting time is reduced.

For the disabling of HARQ feedback in IoT NTN, one key issue is the configuration of enabling/disabling HARQ feedback for both eMTC NTN and NB IoT NTN. Several options from different perspectives were discussed during previous RAN1 meetings, and the working assumption on reusing RRC signalling for CE Mode A as well as using two merged options for NB-IoT NTN and eMTC NTN for CE Mode B was modified and agreed in the last RAN1 meeting [1].

RAN1#110 Agreement For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling. - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field). - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above. RAN1#110 Agreement For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above RAN1#110b-e Agreement For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select ONE from the following options at RAN1#111: - Option 6a-1: Support RRC signaling configured between Option 1 and Option 3 - Option 6a-4: Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission RAN1#112 Agreement - Confirm the following working assumption with the following update: Working assumption - For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - FFS#1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes. - FFS#2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. - RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”. - For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

### 2.2.1 DCI-based overridden mechanism/direct indication

Considering the configuration of enabling/disabling HARQ feedback for NB IoT NTN and eMTC NTN for CE Mode B, the bitmap to indicate the configuration per HARQ process in NR NTN can be reused for RRC-based mechanism, and the main issue is the detailed configuration of the DCI direct indication and DCI-based overridden mechanism. Whether DCI-based overridden mechanism can be applied to semi-statically HARQ enabled process or HARQ disabled process or both for single TB has been discussed in the last RAN1 meeting, and the following alternatives are proposed for further consideration.

- RAN1#112 Agreement For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity - Alternative 1: applies to both semi-statically HARQ enabled and disabled processes - Alternative 2: only applied to semi-statically HARQ disabled processes - Alternative 3: only applied to semi-statically HARQ enabled processes

From our perspective, DCI-based mechanism can be enabled to support dynamic enabling/disabling configuration for the HARQ process. Since there are concerns on MAC CE activation issue and other important message for NB IoT NTN and eMTC NTN for CE Mode B with very limited number of HARQ process, especially for single HARQ process, the DCI indication method is supported to provide more flexibility. Considering that disabling of HARQ feedback is beneficial to throughput improvement and latency reduction, it is reasonable to also apply the DCI-based overridden mechanism on semi-statically HARQ enabled process, which can provide more flexibility and avoid the latency caused by RRC reconfiguration.

Proposal 1. For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, DCI-based overridden mechanism can be applied to both semi-statically HARQ enabled and disabled processes.

Besides, multiple TBs scheduled by a single DCI has been supported for NB-IoT and eMTC, the per HARQ process bitmap via UE specific RRC signaling can be directly applied on the multiple TBs cases, the main issue is whether/how DCI direct indication or DCI-based overridden mechanism can be applied for multiple TBs cases. We think it is reasonable to apply DCI-based overridden mechanism/indication on multiple TBs scheduled by a single DCI to provide flexibility and throughput gain as well. Furthermore, one indication applied to all TBs scheduled by a DCI can be a straightforward way with less specification effort, and the same indication mechanism can be applied to both single TB and multiple TBs cases. The detailed indication method will be discussed in the next section.

Proposal 2. DCI-based overridden mechanism and DCI direct indication can be applied to the cases of multiple TBs scheduled by a single DCI, and single indication can be applied to all scheduled TBs.

### 2.2.2 DCI indication of HARQ feedback

Considering the DCI-based overridden/direct indication of enabling/disabling HARQ feedback, two options were proposed in the last RAN1 meeting and listed below, whether to introduce a new DCI field or reuse the existing DCI field needs to be further discussed.

- RAN1#112 Agreement For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc - Option 1: Indication by adding one field in DCI - Option 2: Indication by reusing/reinterpreting existing field in DCI

For the overridden/direct indication in DCI to indicate the enabled/disabled HARQ feedback, it is preferred not to introduce additional DCI field in order to minimize the standard impact and blind detection complexity. Regarding the limited number of HARQ process of NB-IoT NTN and eMTC NTN for CE Mode B, i.e., one or two HARQ process, it is reasonable to reinterpret the existing DCI filed, such as reinterpreting one dedicated state in the existing field of DCI format N1 and DCI format 6-1B to indicate enabling/disabling of HARQ feedback for NB IoT NTN and eMTC NTN for CE mode B respectively, or 1 dedicated bit in the existing DCI field to indicate the enabling/disabling feedback of HARQ process.

Proposal 3. When DCI-based overridden mechanism or DCI direct indication is enabled, the existing DCI field can be reinterpreted to indicate the enabling/disabling of HARQ feedback for the HARQ process.

From our perspective, the selection of DCI field reused for the indication of HARQ process enabling/disabling should minimize the impact on the existing indication information and scheduling flexibility, one of the state in the HARQ ACK resource related field (i.e., “1111” in the HARQ-ACK resource field of DCI format N1 and “11” in the HARQ-ACK resource offset of DCI format 6-1B) can be reinterpreted for the indication of HARQ feedback enabling/disabling, and other states are still interpreted as the legacy manner. When the DCI-based overridden mechanism or DCI direct indication is enabled, if the value of the filed is set to all 1, the HARQ process will be configured with disabled HARQ feedback, otherwise, the default configuration of HARQ feedback for the process is performed. Specifically, for single TB scheduling, if the dedicated state is configured in the HARQ resource field, the HARQ process for scheduling the single TB should be configured with disabling of HARQ feedback. For multiple TBs scheduling with a single DCI, if dedicated state is configured in the HARQ resource field, all the HARQ processes for scheduling the multiple TBs will be configured with disabling of HARQ feedback.

Besides, considering the semi-statistic bitmap configuration via UE specific RRC signalling, it is specified the bit(s) set to 1 identify HARQ processes with disabled DL HARQ feedback and the bit(s) set to 0 identify HARQ processes with enabled DL HARQ feedback, the similar indication method can be applied for identifying DCI indication. That is using one dedicated bit in the existing filed to indicate enabling/disabling of HARQ feedback for the HARQ process. For example, if the MSB of the field is set to “1”, the HARQ process will be configured with disabling of HARQ feedback, and if the MSB of the field is set to “0”, the HARQ processes will be configured with enabled DL HARQ feedback.

Proposal 4. For DCI-based overridden mechanism or DCI direct indication, the following options can be considered by reinterpreting the existing DCI filed.

- Option 1. One of the state in the existing HARQ-ACK resource field in DCI format N1 or HARQ-ACK resource offset of DCI format 6-1B can be reinterpreted to indicate the disabling of HARQ feedback for the HARQ processes.

- Option 2. Using one dedicated bit in the existing DCI field to indicate enabling/disabling of HARQ feedback for the HARQ process.

  - 2.2 HARQ feedback for scheduling multiple TBs

  - To support multiple TBs scheduling with a single DCI, the issue on the HARQ feedback for scheduling multiple TBs has been discussed during previous meetings. As up to 2 TBs scheduled by a single DCI in NB IoT and up to 8 TBs scheduled by a single DCI for eMTC has been supported, the HARQ feedback for each TB needs to be further discussed, especially for the cases when part of TBs correspond to HARQ enabled processes and the others correspond to HARQ disabled processes. Several options on whether/which feedback should be reported for HARQ disabled process were proposed and shown as follows [3].

[Proposal 5-1a]: - For NBIoT two TBs scheduled by single DCI, the following UE behaviors are considered for the downlink transmission with HARQ process disabled： - Option 1: ACK is assumed/reported for the downlink transmission with HARQ process disabled regardless of decoding results of corresponding transmission - Option 2: HARQ feedback is reported only for downlink transmission with HARQ process enabled (e.g., HARQ feedback is not reported for downlink transmission with HARQ process disabled) - Option 3: HARQ feedback is reported or not depending on the other TBs HARQ-enabled/HARQ-disabling scheduled by DCI - Other options are not excluded [Proposal 5-2a]: - At least for eMTC FDD/HD-FDD multiple TBs scheduled by single DCI without HARQ bundling, the following UE behaviors are considered for the downlink transmission with HARQ process disabled： - Option 1: ACK is assumed/reported for the downlink transmission with HARQ process disabled regardless of decoding results of corresponding transmission - Option 2: HARQ feedback is reported only for downlink transmission with HARQ process enabled (e.g., HARQ feedback is not reported for downlink transmission with HARQ process disabled) - Other options are not excluded - FFS: scenarios for eMTC FDD/HD-FDD multiple TBs scheduled by single DCI with HARQ bundling

From our understanding, the HARQ feedback of feedback-disabled HARQ process for scheduling multiple TBs in IoT NTN can refer to the codebook enhancement method of disabling HARQ feedback in Rel-17 NTN. When multiple TBs are scheduled by a single DCI, for HARQ feedback in NB-IoT over NTN and semi-persistent HARQ feedback in eMTC over NTN, the UE can consistently report ACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Proposal 5. For two TBs scheduled by a single DCI in NB-IoT over NTN, the UE can report ACK for the feedback-disabled HARQ process regardless of decoding results of corresponding NPDSCH.

Proposal 6. For multiple TBs scheduled by a single DCI in eMTC over NTN, the UE can report ACK for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

- Conclusions

In this contribution, we share our views on the issues of disabling of HARQ feedback for IoT NTN. The observations and proposals are summarized as follows:

Proposal 1. For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, DCI-based overridden mechanism can be applied to both semi-statically HARQ enabled and disabled processes.

Proposal 2. DCI-based overridden mechanism and DCI direct indication can be applied to the cases of multiple TBs scheduled by a single DCI, and single indication can be applied to all scheduled TBs.

Proposal 3. When DCI-based overridden mechanism or DCI direct indication is enabled, the existing DCI field can be reinterpreted to indicate the enabling/disabling of HARQ feedback for the HARQ process.

Proposal 4. For DCI-based overridden mechanism or direct indication, the following options can be considered by reinterpreting the existing DCI filed.

- Option 1. One of the state in the existing HARQ-ACK resource field in DCI format N1 or HARQ-ACK resource offset of DCI format 6-1B can be reinterpreted to indicate the disabling of HARQ feedback for the HARQ processes.

- Option 2. Using one dedicated bit in the existing DCI field to indicate enabling/disabling of HARQ feedback for the HARQ process.

Proposal 5. For two TBs scheduled by a single DCI in NB-IoT over NTN, the UE can report ACK for the feedback-disabled HARQ process regardless of decoding results of corresponding NPDSCH.

Proposal 6. For multiple TBs scheduled by a single DCI in eMTC over NTN, the UE can report ACK for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

- References

- Chairman's Notes RAN1#112.

- Chairman's Notes RAN1#110bis-e.

- R1-2301804, “FLS#1 on disabling of HARQ feedback for IoT NTN”, Athens, Greece, February 27th - March 3rd, 2023.