3GPP TSG RAN WG1 #112bis-e  	                            R1-2303252

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Discussion on improved GNSS operations for IoT NTN

Agenda item:	9.9.4

Document for:	Discussion & Decision

- Introduction

During last RAN1 meeting, potential GNSS measurement schemes in connected states were discussed, some progress were achieved on GNSS measurement for long connection times in IoT NTN [1].

In this contribution, we will discuss and share our views on improved GNSS operations during long connection times for IoT NTN, including detailed GNSS measurement schemes and corresponding configurations during long connection time.

- Discussion

  - 2.1 GNSS measurement schemes

Considering the assumption that UE calculates timing advance and frequency adjustment based on acquired GNSS position during long transmission in NB-IoT and eMTC, since the GNSS has a limited validity duration, timing advance value calculated based on GNSS position fix may be not available or sufficiently accurate when the GNSS becomes outdated during the long transmission. Thus, one key issue is how to ensure the GNSS positioning accuracy in long transmission times.

According to the discussion during previous RAN1 meetings, the corresponding agreements about GNSS measurement triggering and GNSS measurement schemes in long connection time are listed as below.

RAN1#110b-e Agreement Support eNB to at least aperiodically trigger UE to make GNSS measurement. RAN1#110b-e Agreement If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used. RAN1#111 Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing RAN1#112 Agreement - On the length of GNSS measurement gap, which is aperiodically triggered by eNB, the gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. - FFS: whether the gap duration is configured by eNB, or the gap duration is equal to the latest reported GNSS position fix time duration. RAN1#112 Agreement - On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, RAN1 can down select one of the following alternatives: - Alt 1: the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value - Alt 2: the start time should be based on the current GNSS validity duration with delay or without delay

It is supported that when eNB aperiodically triggers UE to perform the GNSS measurement, the GNSS measurement gap during long connection time can be configured to ensure that UE in RRC_CONNECTED state will still be able to perform timing and frequency pre-compensation based on accurate GNSS position without impact on NB-IoT/eMTC module.

As simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed in IoT NTN, a common understanding between UE and eNB that data transmission will automatically skip the GNSS measurement gap should be achieved, which can depend on the eNB’s implementation. That is the UE may autonomously stop NB-IoT/eMTC operation and start the GNSS operations for a new position fix, then switch to NB-IoT/eMTC operation after GNSS measurement being finished, the timeline for this process should be aligned between UE and eNB.

Proposal 1. If GNSS measurement gap is applied to enable a new GNSS position fix during long connection times, the common understanding between UE and eNB that UE may automatically perform GNSS operation and NTN NB-IoT/eMTC operation on non-overlapping slot/frame should be achieved.

To minimize the delay and overhead of UE to reacquire a new GNSS position fix during long connection times, detailed GNSS measurement schemes to support UE perform GNSS operation and NTN NB-IoT/eMTC operation on non-overlapping slot/frame need to be further studied. For the configuration of GNSS measurement gap, the length and start time of GNSS measurement gap have been discussed in the last RAN1 meeting, the configuration and alternatives listed above need to be further discussed and down-selected.

For the configuration of GNSS gap duration, it has been agreed that the GNSS measurement gap length can be determined based on the reported GNSS position fix time duration for measurement, which should be equal to or larger than the latest UE reported GNSS position fix time duration to make sure that UE can finish GNSS measurement within the GNSS measurement gap. Furthermore, the gap duration can be configured by network to achieve the common understanding between eNB and UE, especially when the GNSS gap duration is longer than the UE reported GNSS position fix time duration.

Proposal 2. If the GNSS measurement is triggered aperiodically by eNB, and UE re-acquires GNSS position fix within a GNSS measurement gap, the length of GNSS measurement gap should be equal to or larger than the latest UE reported GNSS position fix time duration, which can be configured by eNB.

Another issue on the GNSS measurement gap aperiodically triggered by eNB is when to start the GNSS measurement, as both UE and eNB already know when the original GNSS position fix will expire based on the reported GNSS validity duration, one option is the start time of GNSS measurement gap can be configured at the expiration of the current GNSS validity to avoid excessive GNSS measurement and power consumption.

Another option is that UE will finish the GNSS measurement before the GNSS expiration time to ensure that the new GNSS position fix will be available when the current GNSS position fix becomes outdated, which is sufficient to maintain the UL synchronization, but it may lead to a relatively shorter duration for data transmission.

Proposal 3. If the GNSS measurement is triggered aperiodically by eNB, and UE re-acquires GNSS position fix with a GNSS measurement gap, the start time of GNSS measurement gap can be configured based on the current GNSS validity duration with the following options.

- Option 1. The start time of GNSS measurement gap can be configured at the expiration time of the current GNSS validity duration

- Option 2. The start time of GNSS measurement gap can be configured earlier than the expiration of the GNSS validity duration, and GNSS measurement will be completed before the GNSS validity duration expires.

The GNSS measurement gap aperiodically triggered by MAC CE command is suitable to allow a quick update of GNSS measurement, which can provide scheduling flexibility as the GNSS position fix being performed on demand. And it is agreed later that UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger signalling. While the latency and signalling overhead to perform new GNSS measurement need further discussions.

From our perspective, regarding the scenarios where the GNSS validity duration and UE mobility are stable, the GNSS measurement gap can also be configured periodically by network without any triggering signalling, and the configuration can be indicated by RRC signalling to enable UE acquire new GNSS position fix during long connection times, which is also feasible to allow UE require GNSS position fix and avoid the interference of new GNSS measurement operation to active NB-IoT/eMTC operation with the benefit of less signalling overhead.

As shown in Figure 1, considering the long time transmission, the procedure of GNSS measurement can be first performed up to UE’s implementation in the preparation period before NB-IoT/eMTC active duration, and no specification change is needed. After the connection established, assist information reported by UE can be used to facilitate network determining the configuration of GNSS measurement gap. Based on the reported remaining GNSS validity duration, the network can determine the start time of the first GNSS measurement after the original GNSS expires. The period of the GNSS measurement gap can be determined based on the reported GNSS validity duration, and the length of each GNSS measurement gap is equal to the reported GNSS position fix time duration for measurement. As a result, UE can perform GNSS measurement periodically and automatically switch between GNSS and NB-IoT/eMTC operation for the service link according to the periodic configuration of network, which can save signalling overhead of triggering GNSS measurement.

Figure 1. Illustration of periodic GNSS measurement gaps

Furthermore, UE may not need to perform GNSS measurement in each GNSS measurement gap configured by eNB if the current GNSS position fix can be maintained for a longer time. In this case, UE needs to report indication before the start of GNSS measurement to indicate eNB that the corresponding GNSS measurement will not be performed, which can avoid unnecessary power consumption of GNSS measurement.

Proposal 4. For GNSS measurement in RRC connected, GNSS measurement gap can also be configured periodically by network and indicated through RRC signalling.

- UE may not perform GNSS measurement at each configured GNSS measurement gap, and the indication of not performing the GNSS measurement should be reported before the start of GNSS measurement gap.

  - 2.2 GNSS assistance information

In order to support the GNSS measurement being performed efficiently, it is necessary to enable UE to report effective assistance information. The agreements about GNSS assistance information reporting achieved during the previous RAN1 meetings are shown as below.

RAN1#109-e Agreement UE reports additional GNSS assistance information and further study the detailed GNSS assistance information, including e.g. GNSS position fix measurement time - Note: Since RAN1 agreed that GNSS validity duration is reported by UE in Rel-17, it is already included in GNSS assistance information. RAN1#110 Agreement GNSS assistance information that UE reports to eNB at least consists of: - GNSS position fix time duration for measurement - GNSS validity duration RAN1#110b-e Agreement UE reports GNSS position fix time duration for measurement at least during the initial access stage - which message carries this information is up to RAN2 RAN1#110b-e Agreement In connected mode, UE may report GNSS validation duration with MAC CE. RAN1#112 Agreement UE reports only one GNSS position fix time duration for GNSS measurement at least when moving to RRC connected state.

Regarding the GNSS measurement time duration, it has been agreed that UE reports GNSS position fix time duration for measurement at least during the initial access stage, and only one GNSS position fix time duration for GNSS measurement is reported when moving to RRC connected state. From our perspective, the GNSS position fix time duration indicates the UE capability to finish a GNSS measurement, which may not change dynamically for each GNSS measurement during the connected state.

Considering the GNSS validity duration reported by UE in connected mode, the GNSS validity duration is mainly determined based on UE mobility and position accuracy requirements, whether the GNSS validity duration will frequently update or not depends on UE characteristics. Therefore, during the long connection times, if the GNSS validity duration needs to be updated due to the significant changes of UE mobility, UE can report the updated GNSS validity duration before the next triggering of GNSS measurement if the GNSS measurement is triggered by eNB, then the length of GNSS measurement gap can be determined based on the updated GNSS validity duration. And if a new validity period is not reported, the GNSS gap duration will be configured based on the previous validity duration.

Proposal 5. In connected mode, if the GNSS measurement is aperiodically triggered by eNB, UE will report the updated GNSS validity duration after completing GNSS measurement only when the GNSS validity duration changes.

## 2.3 Success/failure of GNSS measurement

The issue on whether UE needs to indicate the success/failure of GNSS measurement has been discussed during the last RAN1 meeting, and it is agreed that in order to inform the eNB that UE finish the GNSS measurement successfully, at least the two alternatives listed below can be considered.

RAN1#112 Agreement - The following alternatives can be considered to inform eNB the success of GNSS measurement at UE side after GNSS measurement in RRC connected. - Alt-1: The UE will report the new GNSS validity duration - Alt-2: The reception of any UL transmission from the UE at eNB after the GNSS measurement

For Alt-1, whether UE will report the GNSS validity duration after each GNSS measurement should be discussed first. If it is specified that the new GNSS validity duration will be reported after each GNSS measurement, the information of the new GNSS validity duration can implicitly indicate the success of GNSS measurement. However, we think only the changed GNSS validity duration needs to be reported, in this way, using the GNSS validity duration reporting to inform the success of GNSS measurement may not be efficient for all GNSS measurements.

For Alt-2, considering the scenario where large UL repetitions are transmitted during the long connection, as the timing of NB-IoT/eMTC operation and GNSS operation is aligned between eNB and UE, the continued UL transmission after UE finishing the GNSS measurement based on the network configuration can inform eNB that UE already completes the GNSS measurement successfully.

Moreover, if the information of new GNSS validity duration or UL transmission data is not available to transmit, an indication of successfully re-acquiring GNSS position fix can be used to indicate eNB the GNSS measurement has been finished.

Proposal 6. To inform eNB the success of acquiring GNSS position fix, the indication of success after GNSS measurement can be considered if implicit indication is not sufficient.

Another issue is to identify the UE behavior when UE fails to complete the GNSS measurement. According to the conclusion from RAN2 made in Rel-17, when the GNSS fix becomes outdated in RRC_CONNECTED mode, the UE goes to IDLE mode. From our perspective, the similar principle should be followed for long connection times in IoT NTN, UE needs to have the valid GNSS position fix in the connected mode, if UE fails to re-acquire GNSS position fix within the configured GNSS measurement gap, and can not complete the autonomous GNSS acquisition until the GNSS validity duration expires, the UE should enter IDLE mode.

Proposal 7. If UE can not re-acquire the new GNSS position fix successfully until the GNSS validity duration expires, the UE goes to IDLE mode.

  - 2.4 GNSS measurement in C-DRX

The issue on whether UE can re-acquire GNSS position fix during inactive state of C-DRX has been discussed during the previous RAN1 meetings. Considering one motivation for IoT NTN is investigating power efficient operation in order to support extended battery lifetime, mechanism for GNSS position fix updating that can achieve a trade-off between power saving and effective synchronization performance based on GNSS position should be investigated.

From our views, when the duration of inactive state of C-DRX is longer than the GNSS fix time duration for measurement, it is possible that UE can perform GNSS measurement automatically and report the GNSS validity duration during the inactive state of C-DRX, which can ensure accurate GNSS position fix and may save power consumption of alternate GNSS measurement and data transmission during the long connection.

Furthermore, we think it is efficient that UE only re-acquires GNSS position fix right before turning to active state. UE can finish the GNSS measurement before returning into active state to avoid additional impact on data transmission in active state, which can also avoid unnecessary power consumption caused by premature GNSS measurement.

Proposal 8. UE can autonomously re-acquire GNSS position fix during inactive state of C-DRX when the inactive state of C-DRX is longer than the GNSS fix time duration for measurement.

- Conclusions

In this contribution, we mainly share our views on improved GNSS operations for a new position fix for UE pre-compensation during long connection times. The proposals are summarized as follows:

Proposal 1. If GNSS measurement gap is applied to enable a new GNSS position fix during long connection times, the common understanding between UE and eNB that UE may automatically perform GNSS operation and NTN NB-IoT/eMTC operation on non-overlapping slot/frame should be achieved.

Proposal 2. If the GNSS measurement is triggered aperiodically by eNB, and UE re-acquires GNSS position fix within a GNSS measurement gap, the length of GNSS measurement gap should be equal to or larger than the latest UE reported GNSS position fix time duration, which can be configured by eNB.

Proposal 3. If the GNSS measurement is triggered aperiodically by eNB, and UE re-acquires GNSS position fix with a GNSS measurement gap, the start time of GNSS measurement gap can be configured based on the current GNSS validity duration with the following options.

- Option 1. The start time of GNSS measurement gap can be configured at the expiration time of the current GNSS validity duration

- Option 2. The start time of GNSS measurement gap can be configured earlier than the expiration of the GNSS validity duration, and GNSS measurement will be completed before the GNSS validity duration expires.

Proposal 4. For GNSS measurement in RRC connected, GNSS measurement gap can also be configured periodically by network and indicated through RRC signalling.

- UE may not perform GNSS measurement at each configured GNSS measurement gap, and the indication of not performing the GNSS measurement should be reported before the start of GNSS measurement gap.

Proposal 5. In connected mode, if the GNSS measurement is aperiodically triggered by eNB, UE will report the updated GNSS validity duration after completing GNSS measurement only when the GNSS validity duration changes.

Proposal 6. To inform eNB the success of acquiring GNSS position fix, the indication of success after GNSS measurement can be considered if implicit indication is not sufficient.

Proposal 7. If UE can not re-acquire the new GNSS position fix successfully until the GNSS validity duration expires, the UE goes to IDLE mode.

Proposal 8. UE can autonomously re-acquire GNSS position fix during inactive state of C-DRX when the inactive state of C-DRX is longer than the GNSS fix time duration for measurement.

- References

- Chairman's Notes RAN1#112, February 27 - March 3, 2023.

- Chairman's Notes RAN1#111, November 14th – 18th, 2022.

- Chairman's Notes RAN1#110bis-e, October 10th – 19th, 2022.

- Chairman's Notes RAN1#109-e, May 9th – 20th, 2022.

- Chairman's Notes RAN1#110, August 22nd – 26th, 2022.