3GPP TSG RAN WG1 #112bis-e			                                   R1-2303254

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.10.2

Source: 	CMCC

Title: 	Discussion on timing advance management to reduce latency

Document for:	Discussion and Decision

Introduction

In the RAN1#112 meeting, a few agreements had been achieved for the TA acquisition before cell switching command [1]. The agreements are listed in the related sections.

In this contribution, we provide our views on the TA management in the mobility scenarios.

Discussion

Shorten the interruption time during the handover can reduce the latency of the RACH procedure for UL synchronization. If the uplink synchronization can be achieved before the handover procedure or before the interruption time, this part of latency for the mobility can be reduced.

Agreement On mechanism to acquire TA of the candidate cells, the following solutions can be further studied: - RACH-based solutions - e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd - RACH-less solutions - e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Two category solutions are agreed for further study. RACH-based solutions are mainly based on the RACH procedure or PRACH to acquire the uplink timing. PDCCH ordered RACH depends on serving cell’s decision for the acquisition of TA of the candidate cells. And RACH-less solutions were subject to the scenarios that zero TA or the candidate cell and the serving cell have the same TA. To acquire the TA of candidate cells, UE based TA estimation could be considered. Based on the TA estimation from UE, the RACH-less solutions can work.

RACH-based solutions

In the last meeting, a few agreements related to the PDCCH ordered RACH were achieved. The details are as below.

Agreement For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement - Note: Definition of candidate cells is up to RAN2 Agreement - For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC.

It was agreed that whether RAR needs to be received is configured by RRC and depends on UE’s capability. If the RAR is not configured or indicated, the TA value of the candidate cell is indicated in the cell switch command. If the RAR is configured or indicated, whether the RAR is from the serving cell or candidate cell is still under discussion.

When the reception of RAR is not configured, whether UE should re-transmit PRACH is till FFS. In this condition, the TA value of candidate cell is indicated in the cell switch command. The TA value is transferred from the candidate serving cell which has received the PDCCH ordered PRACH. If the candidate cell does not receive the PRACH, it can also inform that to the serving cell. Then the serving cell have the full knowledge of whether the PRACH is received by the candidate cell or not. When the PRACH is not correctly received by the candidate cell, the serving cell can trigger the re-transmission of PRACH to the UE. There is no need to support UE oriented PRACH retransmission.

Proposal 1:

When the reception of RAR is not configured, the PRACH re-transmission could be triggered or ordered by the serving cell. There is no need to support the UE oriented PRACH re-transmission.

The transmit power of subsequent PRACH triggered by PDCCH order is still FFS. For the initial PDCCH ordered PRACH transmission, the transmit power depends on the targeted received power and the pathloss. The UE should have the knowledge of the EPRE of candidate cell and then calculate the pathloss. Not only the targeted received power of PRACH but also the DL EPRE should be informed to the UE.

If the first transmission of PDCCH ordered RACH fails, and the serving cell should trigger a re-transmission of the PRACH. Currently there is no indication of the re-transmission of PDCCH ordered PRACH. And there is also no indication to UE to increase the transmit power of the PDCCH ordered PRACH. It should be further study how to indicate UE to increase the transmit power of the retransmission or sub-sequent transmission of the PDCCH ordered RACH. One straightforward solution is to indication in the DCI that this is a re-transmission or the sub-sequent transmission of the PDCCH ordered RACH. Or it is indicated in DCI field increasing the transmit power of the PRACH. Then if the UE still has additional power for transmission, it can increase the transmit power in certain step.

Proposal 2:

The targeted received power of PRACH and the DL EPRE of the candidate cell should be informed to the UE for the PDCCH ordered RACH.

Proposal 3:

It should be further discussed whether the sub-sequent PRACH triggered by PDCCH order should be identified and whether there is a need to indicate in the DCI to increase the transmit power of PRACH.

When the reception of the RAR is configured, it should be discussed that from which cell should the RAR be received. If the RAR is received from the candidate cell, the procedure is similar with the one in legacy. Additional latency would be introduced, such as that the UE has to wait within the legacy RAR window for the response. Since this is the procedure before the cell switching, the configuration of the candidate cells is not activated. The RAR related information should be configured to the UE. And candidate cell which received the PRACH should be able to identify that this is not a RACH procedure but for the early acquisition of the TA. Considering that the LTM should be applied in the ideal backhaul scenarios, then the transmission delay of the TA values over the backhaul link can be ignored. From the perspective of complexity and the latency, it is preferred to receive the RAR from the serving cell.

Proposal 4:

It is preferred to receive the RAR from the serving cell when the reception of RAR is configured.

Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

Then content of the RAR for the TA acquisition should be simplified compared to the legacy RAR. In the current specification, the MAC payload of RAR contains the absolute TA, UL Grant and Temporary C-RNTI. For the TA acquisition, only the absolute TA is useful. And UL Grant and Temporary C-RNTI are not needed. It is supported that the TA values of the candidate cells should be contained in the RAR. If multiple TAs are supported, it should be identified the TA values corresponds to which candidate cells. Then the candidate cell ID should be included in the RAR.

Proposal 5:

At least the candidate cell ID or indication should be contained in the RAR.

Agreement on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

The UE autonomous re-transmission of PRACH was discussed in the last meeting. And Two alternatives would be downselected. In legacy, the UE has to wait for the response from the target cell within the RAR window in the RACH procedure. If the UE does not receive the RAR within the window, the UE autonomous re-transmit the PRACH. The RAR window length is configured in RRC, which is from from 1 to 80 slots in Rel-15. When will the target cell send the RAR depends on gNB’s implementation. If the UE autonomous re-transmission is introduced, following the legacy procedure, the UE has to wait a period which corresponds to the length of the RAR window and then re-transmit the PRACH. This is not efficient for the TA early acquisition. If the TA values are not acquired by the serving cell, it can directly order another PRACH transmission through PDCCH. There is no need to wait for the RAR window. Thus the UE autonomous re-transmission is not needed.

Proposal 6:

The UE autonomous re-transmission is not needed when PDCCH ordered RACH is used for the TA acquisition.

RACH-less solutions

The RACH-less solutions were only used under certain scenarios in legacy. The first scenario is when the TA of the target cell is the same as the one in the current serving cell. The second scenario is that the TA value of target cell equals zero which usually happens in the small cell. In those scenarios, the initial TA could be acquired based on either the TA of current cell or the DL reception timing. If the DL reception timing difference could be acquired by UE, UE could calculate the uplink TA to the candidate cells. The initial TA derived by UE based on the timing difference can be used for the first transmission to the target cell. And the target cell can adjust the TA values based on the first uplink transmission. The Rx timing difference based solutions can be extended for more using scenarios, e.g. not only for the TA=0 and the latest TA value of the source cell, compared to the LTE scenarios.

Proposal 7:

Rx timing difference based solution should be supported among the RACH-less solutions.

In the last meeting, one working assumption is achieved.

Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

As we discussed above, the working assumption should be confirmed to support the UE based TA measurement. And the other impacts on RAN1 specification can be discussed.

There are several issues can be discussed. First, whether the gNB have the knowledge that the UE have derived the TA of the candidate cells and can be switched to the candidate cells. After the DL synchronization before cell switch, the UE with the capability can be considered that it has acquired the timing differences between the multiple candidate cells and current serving cell during synchronization procedure. Then two options can be further discussed. One is that after the DL synchronization before the cell switching, the serving cell can confirm that the UE has already derived the TAs of the candidate cells based on the UE capability and can be switched to any candidate cells. In this condition, only the candidate cell id and without the TA value need to be carried in the cell switching command. The other option is that UE report the estimated TA values or the timing difference to the gNB. And the gNB indicate that actual TA within the cell switching command. The actual TA values can be derived with additional information from gNB, such as some implementation-based timing gap of the candidate cells, synchronization errors between gNBs, and others.

Proposal 8:

Confirm the working assumption to support the UE based TA measurement.

Proposal 9:

Two options can be considered for the UE based TA measurement.

- Option 1: UE can derive the TA of the candidate cells without any reporting and confirmation from the serving cell.

- Option 2: UE should report the timing gap or estimated TA to the serving cell. And the serving cell can indicate the actual TA values within the cell switching command.

Association between TA/TAG and candidate target cell

Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

In the two TAs under the multiple TRP scenario, the configuration of the two TRPs could be delivered to the UE with two TAs. All the configurations are from the serving cell and configured to the UE. The additional TA/TAG could associate with a configured TCI state. But in the mobility scenario, the UE only has the configuration of its current serving cell. All the configurations of the candidate cell are still unknown or not activated. The UE cannot directly associate the TA or TAG with some deactivated parameters and event implicitly. If the configuration of the candidate cell could be provided directly to the UE and serving cell. Then the UE could connect the TA or TAG directly to the parameters of the candidate cell.

Proposal 10:

The Alt 2, associating TA/TAG and the candidate cell explicitly, is slightly preferred.

Conclusions

In this contribution, we provide our views on the TA management in the mobility scenarios. The proposals are listed as below.

Proposal 1:

When the reception of RAR is not configured, the PRACH re-transmission could be triggered or ordered by the serving cell. There is no need to support the UE oriented PRACH re-transmission.

Proposal 2:

The targeted received power of PRACH and the DL EPRE of the candidate cell should be informed to the UE for the PDCCH ordered RACH.

Proposal 3:

It should be further discussed whether the sub-sequent PRACH triggered by PDCCH order should be identified and whether there is a need to indicate in the DCI to increase the transmit power of PRACH.

Proposal 4:

It is preferred to receive the RAR from the serving cell when the reception of RAR is configured.

Proposal 5:

At least the candidate cell ID or indication should be contained in the RAR.

Proposal 6:

The UE autonomous re-transmission is not needed when PDCCH ordered RACH is used for the TA acquisition.

Proposal 7:

Rx timing difference based solution should be supported among the RACH-less solutions.

Proposal 8:

Confirm the working assumption to support the UE based TA measurement.

Proposal 9:

Two options can be considered for the UE based TA measurement.

- Option 1: UE can derive the TA of the candidate cells without any reporting and confirmation from the serving cell.

- Option 2: UE should report the timing gap or estimated TA to the serving cell. And the serving cell can indicate the actual TA values within the cell switching command.

Proposal 10:

The Alt 2, associating TA/TAG and the candidate cell explicitly, is slightly preferred.

References

[1] Chair’s notes RAN1#112, 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023