3GPP TSG RAN WG1 #112bis-e	                            R1-2303255

e-Meeting, April 17th – April 26th, 2023

Source: 	CMCC

Title:	Discussion on L1 signal design and procedure for LP-WUS

Agenda item:	9.11.3

Document for:	Discussion & Decision

- Introduction

In the last meetings, some basic principles were agreed with respect to the following aspects [1], waveform modulation, synchronization methods, monitoring behaviors, etc. In this contribution, we would like to share our further views on these topics. Here are following discussion.

- Discussion

2.1 Waveform and modulation

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

Four candidate waveforms for the OOK-based LP-WUS have been provided in accordance with the agreement from the last meeting. Among them, we prefer to support multi-bit options over single-bit options. Although the single-bit scheme is the simplest LP-WUS generation way, the data rate is too low and hard to meet the higher data rate requirements for LP-WUS due to the limited efficiency. On the contrary, the multi-bit scheme can improve resource efficiency by adjusting the M size to meet the LP-WUS data rate requirements. Therefore, it is more reasonable to set the multi-bit OOK options as the high-priority option to be studied.

Proposal 1. Multi-bit OOK options can be considered as the high-priority option to be studied.

Considering the cost of the structure, the LP-WUS needs to co-exist with the legacy NR signal to reuse the existing gNB transmitter hardware. Besides that, some key factors also affectthe modulation option selection. To compare the multi-bit OOK options more clearly, the analysis of each multi-bits options is provided in Table 1.

Table 1. Analysis of multi-bits OOK generation options

|  | Pros | Cons |
| --- | --- | --- |
| Option OOK-2 | Easy to implement | Highest receiver complexity |
| Option OOK-3 | Robust to frequency selective fading | - High receiver complexity - Additional sub-carrier filter |
| Option OOK-4 | Low receiver complexity | Higher sample rate |

It is easy to find that option OOK-2 and option OOK-3 generate the LP-WUS through parallel sub-carriers, which requires the LP-WUR to receive multiple parallel sub-carriers simultaneously. The complexity of LP-WUR will increase due to the existence of multiple parallel envelope detection chains, which violates the design principle. Meanwhile, although option OOK-3 is more stable against frequency selective fading via multi-tone design, the additional sub-carrier filter with a narrow pass-band is needed which is hard to implement. Option OOK-4 has the simplest receiver complexity among the three options which can also reduce the cost of LP-WUR. The shortcoming of Option-4 is it also requires a high sample rate on the time domain, but this is not a critical issue for LP-WUR complicity. Therefore, based on the comprehensive consideration, we prefer to support option OOK-4 for further study.

Proposal 2. Support Option OOK-4 for further study.

To improve the ability to resist noise and enhance reliability, it is worth discussing how to encode LP-WUS. One notable approach is the use of Manchester Encoding to prevent the loss of clock synchronization and analog link bit errors from low frequency shifts. The ON and OFF symbols are transmitted in pairs of either (ON, OFF) or (OFF, ON) which is helpful to estimate the accurate detection of the state of signal per unit of time and avoid missing detection. The LP-WUS receiver can compare the received energy in the first and second half of the pair to make a decision, which can reduce the cost and enhance the reliability.

Proposal 3. Manchester Encoding can be the candidate method to enhance the reliability of MC-ASK(OOK) based LP-WUS.

2.2 Bandwidth location

Based on the discussion result of the previous meeting, it is worth analyzing the advantages and disadvantages of restricting LP-WUS bandwidth location. The advantage of the LP-WUS bandwidth at a certain location is a lower requirement for the hardware and easier to implement than flexible location. The disadvantage of bandwidth restriction is it will cause the scheduling restriction when multiplexing with other NR signals. Therefore, the flexible location of LP-WUS is necessary to improve resource utilize efficiency and is more reasonable to be studied.

Another issue that needs discussion is whether the LP-WUS should always be confined within the initial DL BWP. Generally speaking, the size of the initial DL BWP is limited and precious. Much essential information such as the SSB, SIB1, RMSI, is contained in the initial DL BWP for the initial access process. Due to the LP-WUS being a low-efficient signal, it is unnecessary to occupy the limited BWP resource. However, the LR can transmit independently of MR. When LP-WUS transmitting separately, the LP-WUS bandwidth can be flexibly located inside or outside the initial DL BWP. Therefore, whether the LP-WUS bandwidth is inside or outside of DL BWP can be considered.

Proposal 4. Support flexibility configuration of LP-WUS bandwidth location. Both inside and outside initial DL BWP can be considered.

2.3 Structure of the LP-WUS

Generally, there are two options for LP-WUS structure:

- Option1: Sequence based

- Option2: Payload based

For option 1, the different LP-WUS sequences can be used to wake different UE-groups or POs. It is hard for LP-WUS to expand more functions based on this structure as its limited sequence size/number. Sync or system information may hard to be delivered correctly by LP-WUS sequence. The benefit of this structure is it is very simple but the information conveyed may be limited.

For option 2, this structure can convey more information compared with option 1, e.g., convey Group ID or UE ID to identify the target UE for waking up which is very useful information for paging. In addition, this structure can also attach CRC which is a useful solution to reduce the FAR. This structure can provide higher transmission efficiency than option 1. As a consequence, we prefer to support the payload based structure scheme and the size of each part of the structure also needs to be studied.

Proposal 5. Support payload based LP-WUS structure and study the payload size.

2.4 Synchronization

Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

The simple structure of the LP-WUS receiver may cause clock drift, which also affects the synchronization with the gNB. Three synchronization options were proposed at the last meeting to address this issue. To satisfy the low cost and low power consumption requirements of the LP-WUS, we need to make a trade-off between the power consumption and the signal performance and find a balanced option to keep LP-WUS in sync.

For option 1, design an aperiodic signal transmitted as part of LP-WUS, meaning that the LP-WUS may contain a dedicated preamble as a sync partition and transmit with the LP-WUS all the times to self-adjust time error and reduce the FAR. The drawback of this option is that the WUR needs to be detected all the time, which results in more power consumption. Otherwise, it may lead to a full miss match due to large time drift.

For option 2, design a periodic signal transmitted separately from LP-WUS which is helpful for UE to adjust the timing to receive the LP-WUS signal through LP-SS. In contrast to option 1, this method can achieve a large power gain as the transmission is periodic. It also helps in resetting the clock and reducing timing ambiguities around the WUS monitoring occasions with more stringent criteria to avoid the full mismatch. In addition, the LP-SS may make an effect on relaxing the serving cell measurement in further study. The drawback of this method is that it is only a coarse synchronization option and it is hard to fully match the MR and LR, it only reduces the time drift but does not eliminate the time drift.

For option 3, when combining option 1 and option 2, the performance of LP-WUS may have a significant improvement. LP-WUS can sync with NR via coarse synchronization and fine synchronization. However, the structure of the LP-WUS will be more complex and the power consumption may be higher than the expected value due to satisfy the outstanding performance. While high-quality synchronization may reduce some miss-detection consumption, further research is needed to compare the network overhead, power consumption and detection accuracy of each method in computing and simulation to find a balanced approach to help LP-WUS synchronization.

Proposal 6. Support Option 3 as the synchronization scheme to further study.

2.5 Function and procedure

In Rel-16/17, UE power saving techniques are specified for RRC RRC_IDLE/RRC_INACTIVE states and RRC_CONNECTED state, including, Rel-16 DCP, Rel-17 PEI and Rel-17 PDCCH skipping/SSSG switching. Regarding the Rel-18 LP-WUS, the function in IDLE/RRC_INACTIVE and RRC_CONNECTED state should also be discussed separately.

2.4.1 RRC_IDLE/RRC_INACTIVE state

When UE is in RRC_IDLE/RRC_INACTIVE state, paging is the basic function that can help UE wake up at the correct time. How to paging based on LP-WUS and how to work with other legacy power saving methods remains to be investigated. In the last meeting, three paging wake up methods have been proposed [2]. We compared each of these options to find an appropriate paging wake up method.

For option 1, LP-WUS replaces PEI to trigger MR whether to monitor PO. When LP-WUR received a positive LP-WUS indicator, MR will start to receive paging PDCCH. The LP-WUS can carrier per-group information that each occasion is associated with one or more POs. LP-WUS on this option also needs to carry more than one bit of information and requires a high transition rate. It is worth studying and evaluating the payload and the power saving gain of different options.

For option 2, LP-WUS performs as an indicator to trigger MR whether to monitor PEI. The advantage of this method is it can reduce the paging consumption of the LR and decrease the FAR due to the high paging accuracy. Meanwhile, this method has a low requirement for data rate, it just makes a micro impact on traditional paging procedure which is easier to be implemented. However, this method can not provide significant power saving gain on paging as UE also needs to monitor PEI and PO, and also leads to a high latency as the big gap between LP-WUS and the PEI.

For option 3, LP-WUS carries the UE-ID directly. LP-WUS itself supports paging message delivery and can each UE can wake up its MR if comparing UE-ID conveyed in the LP-WUS matches its ID. Then, the MR will initiate PRACH directly to establish the RRC connection. Based on this option, the MR is no longer required to receive paging PDCCH. This option reduces the paging latency and provides the best UE power saving but requires the LP-WUS to carry a large payload size. However, it also has stringent data rate requirements for transmitting UE-ID, which is unfriendly to basic modulation methods like OOK or FSK.

Above all, we need to make a trade-off between options. Option 2 can provide high accuracy of paging and it is easier to be implemented by co-existing with the existing paging method. However, it doesn’t have a significant power saving gain and suffers from long paging latency. Option 3 has a high demand on the data rate to transmit its UE-ID, but it has the most significant power saving gain and less latency. Option 1 can balance the requirements of power saving and latency, which is worth to be further studied.

Proposal 7. For UEs in RRC_IDLE/RRC_INACTIVE state, LP-WUS can replace PEI to trigger MR whether to monitor PO.

2.4.2 RRC_CONNECTED state

Rel-16 DCP and Rel-17 PDCCH skipping/SSSG switching are used to indicate UE the PDCCH monitoring adaptation to reduce the PDCCH monitoring power consumption. For LP-WUS in RRC_CONNECTED state, a similar function can be provided. But there are also some restrictions and different designs in previous solutions, e.g., the Rel-17 PDCCH skipping/SSSG switching can only be used within DRX, the Rel-16 DCP is a group-common DCI, but Rel-17 PDCCH skipping/SSSG switching is UE-specific DCI. Since we design a new LP-WUS in Rel-18 to replace this function, a joint design for LP-WUS is needed to reduce the workload and spec impacts. For example, the Rel-18 LP-WUS can be used independently from DRX configuration, and whether the LP-WUS in RRC_CONNECTED state is based on UE-group or UE-specific can be further studied.

Proposal 8. For UE in RRC_CONNECTED state, LP-WUS is used to indicate UE whether to monitor PDCCH or not similar to Rel-16 DCP and Rel-17 PDCCH skipping/SSSG switching and strive a joint LP-WUS design for all functions.

2.6 Coverage

The structure of the LP-WUS receiver is simpler than the general main radio receiver, which only retains the basic components to satisfy the core receiver functions. The design concept also leads to the worse sensitivity of the LP-WUR receiver than the general main radio receiver. As a consequence, the coverage gap exists between LP-WUS and NR signal/channel. This will lead to the situation that the UE could receive the NR signal but could not receive the LP-WUS. The coverage hole will affect the UE could not keep the low-power consumption state. Therefore, although the coverage performance between the LP-WUR and the main radio is difficult to match due to the poor receiver sensitivity of LP-WUR, the gap still needs to be catch up as much as possible.

Proposal 9. Coverage of LP-WUS/WUR should be comparable with the main radio.

If the coverage gap between LP-WUR and MR cannot be avoided, how to handle the partial coverage issue of LP-WUS needs to be further studied. It is reasonable to consider how to trigger the transition between LP-WUS and other UE power saving techniques. For example, when UE is located at the cell center, the LR and MR can be received at the same time, the UE could apply the LP-WUS by setting the LP-WUS as the first-priority power-saving method in this situation. When the UE moves from the cell center to the cell edge, due to the smaller coverage of LP-WUS, it is reasonable to set a decision scheme to help UE switch the power saving function. One solution is setting the RSRP and RSRQ tolerance threshold value of LP-WUS. When the measured value is lower than the threshold, it means UE at the coverage hole and needs to apply other power saving techniques such as PEI or DCP. Then, UE could turn off the LP-WUR until the measurement RSRP/RSRQ value reaches the LP-WUR threshold in the next measurement cycle. If the measurement value is higher than the threshold, which represents UE could apply the LP-WUS rather than other power saving techniques. This method can help UE always on the suitable power-saving mode to overcome the coverage hole between LP-WUR and MR.

Proposal 10. LP-WUS jointly applied with other paving saving techniques to overcome the coverage hole issue.

- Conclusions

In this contribution, some considerations of L1 signal design and procedure for low power WUS are discussed and the following proposals are made.

Proposal 1. Multi-bit OOK options can be considered as the high-priority option to be studied.

Proposal 2. Support Option OOK-4 for further study.

Proposal 3. Manchester Encoding can be the candidate method to enhance the reliability of MC-ASK(OOK) based LP-WUS.

Proposal 4. Support flexibility configuration of LP-WUS bandwidth location. Both inside and outside initial DL BWP can be considered.

Proposal 5. Support payload based LP-WUS structure and study the payload size.

Proposal 6. Support Option 3 as the synchronization scheme to further study.

Proposal 7. For UEs in RRC_IDLE/RRC_INACTIVE state, LP-WUS can replace PEI to trigger MR whether to monitor PO.

Proposal 8. For UE in RRC_CONNECTED state, LP-WUS is used to indicate UE whether to monitor PDCCH or not similar to Rel-16 DCP and Rel-17 PDCCH skipping/SSSG switching and strive a joint LP-WUS design for all functions.

Proposal 9. Coverage of LP-WUS/WUR should be comparable with the main radio.

Proposal 10. LP-WUS jointly applied with other paving saving techniques to overcome the coverage hole issue.

- References

Chairman notes RAN1#112

R1-2302213 Summary #3 of discussions on L1 signal design and procedure for low power WUS