3GPP TSG RAN WG1 #112bis-e			                                   R1-2303258

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.12.3

Source: 	CMCC

Title: 	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and Decision

Introduction

In the RAN1#112 meeting, the enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM were discussed. And several agreements have been achieved [1]. The agreements are listed in the corresponding sections.

In this contribution, we provide our views on the dynamic switching between DFT-S-OFDM and CP-OFDM.

Discussion

The main issue should be solved under this scope is to specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM. In legacy spec, transform precoder is configured via RRC signaling like transformPrecoder or msg3-transformPrecoder. For DCI format 0_0, msg3-transformPrecoder is applied, and it is also applied to the DCI format 0_1 and 0_2 if the transformPrecoder is absent.

In the last meeting, indication mechanism for dynamic waveform switching was discussed, and the following agreement was made.

Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI.

DCI format 0_0 performs a very important character in network, because it is a fallback DCI. The fallback DCI format support only basic NR feature, to save bit width and fulfil a better robustness. For the above reason, any modification for fallback DCI needs carefully design. After several round discussion, we agreed to aid new 1-bit field in DCI to indicate the dynamic waveform indication for scheduled PUSCH transmission. This bit field obviously can not be identified by the legacy UE, and the increased payload would decrease the coverage and the successful decoding rate for PDCCH to a certain extent. In sum, DWS should not be applied to the DCI format 0_0.

Proposal 1:

Dynamic waveform switching is not applied to the DCI format 0_0.

Msg3 is one of the UL transmission in RACH procedure. The initial transmission is scheduled by the RAR UL grant in Msg2, and the re-transmission is scheduled by DCI format 0_0 scrambled by TC-RNTI. For legacy network, the waveform for Msg3 is indicated by the RRC parameter msg3-transformPrecoder. Since this is the very early of the UE access to the network, gNB have little information about the UE situation, changing waveform is not very reasonable. Introduce indication in RAR and DCI format 0_0 also have some difficulty, especially the DCI 0_0, which we proposed not to support DWS in the above section. We therefore propose not to support DWS for Msg3 in Rel-18.

Proposal 2:

Dynamic waveform switching in Rel-18 is not applicable to msg3 PUSCH.

After the indication mechanism is decided, DCI size alignment issue should be discussed to finish the whole procedure. DCI size alignment is a very critical issue in NR network. The UE ability can only handle several (like 4) different DCI size, so the DCI size of different format have a exquisite mechanism to meet the requirement. This procedure is done by adding the padding bits in the last of the smaller DCI format. The DCI scrambled by C-RNTI and the DCI scrambled by CS-RNTI also have a size alignment issue. According to the TS38.212, this alignment is performed field by field, to make the NDI field locate in the same location, to help the UE understand which type of transmission this DCI indicated.

Observation 1:

Field-by-field size alignment and per DCI format alignment are all existed in current spec.

At the early step, only DCI size alignment between two DCI indicate different waveforms is discussed. Two possible methods could be considered For DCI format 0_1/0_2 containing dynamic waveform indication.For option1, bit width of each field is set to the maximum between the bit width of the field if transform precoding is disabled and the bit width of the field if transform precoding is enabled, if different. For option2, bit width of DCI is set to the maximum between the bit width of the DCI if transform precoding is disabled and the bit width of the DCI if transform precoding is enabled, if different. Both options seems workable, but the details should be further discussed. For example, if the alignment procedure is operated per DCI bit field, then the new 1-bit field for dynamic waveform indication could be carried in any location in DCI. If the alignment procedure is operated per DCI, then the new 1-bit field for dynamic waveform indication could be carried in front of any other bit fields whose bit width are influenced by transform precoding, otherwise UE could not decided each width of the bit field.

In the last meeting, the issue about DCI size alignment between the DCI scrambled by C-RNTI and the DCI scrambled by CS-RNTI is discussed, and the options expand to 4.

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

Figure 1.  4 Options for DCI size alignment

Figure 1 shows the different options for DCI size alignment. Option 3 and option 4 is introduced in offline discussion, aiming for solving the alignment problem for CS-RNTI. Option 3 seems a compromise solution, as it not only make allowances for the DCI scrambled by CS-RNTI but also have same advantage which option 1 have. Almost all the bit field influenced by waveform located following NDI, so the Option3 can have similar performance with option1, if there is a proprietary bit field adding into DCI with DFT-S-OFDM.

Proposal 3:

Support option 3. Whether there is a bit field which bit width is influenced by DWS need further check.

Another issue needs to discuss is the options for handling of FDRA type configuration when dynamic waveform switching is configured. The following alternatives are identified by the FL.

- Alt-1: Configuration of FDRA type to Type 0 is error case - MSB of FDRA set to 0 is error case if DFT-S-OFDM is indicated and FDRA type is set to dynamicSwitch - Alt-2: If FDRA type is configured to Type 0, UE applies FDRA type 1 for DFT-S-OFDM and FDRA type 0 for CP-OFDM - Alt-3: Introduce additional RRC parameter for FDRA type applicable to DFT-S-OFDM

From our understanding, to activate the dynamic waveform switching, RRC signalling is needed. gNB should have clear understanding about the collision between FDRA and DWS, so it should have  opportunity to change the FDRA configuration and activate DWS at the same time. Alt-1 seems workable.

Proposal 4:

Support Alt-1. Configuration or indication of FDRA type to Type 0 is error case if DWS indicates DFT-S-OFDM.

In the last meeting, assistive message for dynamic waveform switching was discussed, and the following agreement was made.

Agreement Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching: - Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform - Target waveform can be same or different from waveform of an actual PUSCH transmission - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission - FFS determination of target waveform, target RB allocation, target modulation order - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms - PHR triggering enhancements, e.g. - Network-triggered PHR - PH becomes lower (higher) than a threshold - PHR triggered by waveform switching - Reporting of recommended waveform or request to switch waveform - Other solutions not precluded

In the legacy spec, power headroom reporting is a periodic event. According to the TS 38.321, a Power Headroom Report (PHR) shall be triggered if the phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB, or phr-PeriodicTimer expires. In a typical cell configuration, a UE may report its PHR every 100 subframes. gNB may have no idea about the UE’s power reduction due to the waveform switching. To solve this problem, two options could be considered. The first option is adding a event triggered PHR, e.g. , after the waveform switching, UE reports its PHR instead of waiting until any of the legacy events occur. The second option is based on UE capability reporting. Power reduction due to the waveform switching should be reported to the gNB when a UE reporting its capability of dynamic waveform switching.

Proposal 5:

Enhancements to assist the scheduler in determining waveform switching is necessary. Two options could be considered.

Option 1: a new event to trigger PHR.

Option 2: UE capability reporting.

If the DCI size alignment issue is apply, after UE enable the dynamic waveform switching, the bit width of the DCI with transform precoding would be enlarged. This would somehow decrease reliability for DCI detection in some extreme coverage limited scenario. For a UE which is suppose to be suffered from a long-term coverage limitation, it should have possibility to fallback to the legacy DCI, which helps better DCI successful detection rate.

Proposal 6:

Study potential enhancement to enable UE fallback from DWS to legacy DCI.

Conclusions

In this contribution, we provide our views on the dynamic switching between DFT-S-OFDM and CP-OFDM. The observations and proposals are as below.

Observation 1:

Field-by-field size alignment and per DCI format alignment are all existed in current spec.

Proposal 1:

Dynamic waveform switching is not applied to the DCI format 0_0.

Proposal 2:

Dynamic waveform switching in Rel-18 is not applicable to msg3 PUSCH.

Proposal 3:

Support option 3. Whether there is a bit field which bit width is influenced by DWS need further check.

Proposal 4:

Support Alt-1. Configuration or indication of FDRA type to Type 0 is error case if DWS indicates DFT-S-OFDM.

Proposal 5:

Enhancements to assist the scheduler in determining waveform switching is necessary. Two options could be considered.

Option 1: a new event to trigger PHR.

Option 2: UE capability reporting.

Proposal 6:

Study potential enhancement to enable UE fallback from DWS to legacy DCI.

References

- [1] Draft Report of 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023