3GPP TSG RAN WG1 #112bis-e			R1-2303260

e-Meeting, April 17th – April 26th, 2023

Source:	CAICT

Title:	Discussion on timing advance management to reduce latency

Agenda Item:	9.10.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #112 meeting several agreements of Timing advance management to reduce latency were accepted during the meeting [1]:

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

On whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives.

- Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1)

  - Alt 2: UE autonomous Re-transmission of PRACH is allowed,

    - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

In this contribution, we further discuss more details of the PDCCH-order based PRACH including RACH based TA acquisition and TA updating mechanism.

- RACH based TA acquisition

For PDCCH ordered RACH for candidate cell(s), RAR reception can be configured/indicated is agreed at last meeting.

- If RAR reception is not configured, TA value of candidate cell is indicated in cell switch command. For the remaining issues of whether UE should re-transmit PRACH when reception of RAR is not configured/indicated and how UE determine the transmit power of subsequent PRACH triggered by PDCCH order, we think different with PDCCH ordered RACH for serving cell, UE should not re-transmit the preamble with power ramping according to the legacy procedure, since the TA value of candidate cell is only indicated by serving cell in cell switch command, which means the TA is sent until the cell switch command is sent, the re-transmission of PRACH may bring waste of resource. On the other hand, it is better that the serving cell can trigger PDCCH order RACH again with power ramping for subsequent PRACH transmission. In other words, network can trigger another RACH transmission using a new PDCCH order with high power if PRACH is not received reliably, and re-transmission is not required. So we prefer UE should not re-transmit PRACH when reception of RAR is not configured/indicated, and Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) is more preferred.

Proposal 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) .

- If reception of RAR is configured/indicated (with RAR), whether RAR is received from serving cell or candidate cell, we think the RAR reception from candidate cell before cell switch command is not needed because UE may not need to communicate with candidate cell before cell switch, and receiving RAR from candidate cell may need new procedures. So we prefer that if reception of RAR is configured/indicated(with RAR), RAR is received from serving cell.

Proposal 2: If reception of RAR is configured/indicated (with RAR),  RAR is received from serving cell.

- TA updating Mechanism

- An agreement has been made that the serving cell can trigger the UE to update the TA for a candidate cell by updating the TA using RACH procedure. There are two options of TA updating mechanism, the first option is UE based updating, which means the UE may be configured to maintain the timer for TA validity after sending preamble or receiving the RAR message, upon expiration of the timer, the UE may inform the source cell to trigger TA update. The second option is network based updating, which means the candidate cell may share the time duration of the TA validity timer with the serving cell, and the  serving cell may maintain the timer for TA validity.

- For this issue, at least for PDCCH ordered based TA acquisition scheme, as mentioned above, we think the TA indication at serving cell is better than candidate cell, so for the TA updating mechanism, we also think it is better the TA validity timer is maintained in the serving cell. When the Timer expires, the source cell can trigger the PDCCH ordered PRACH transmission to acquire new TA information.

Proposal 3: For TA updating mechanism, the network based updating is supported.

- Conclusion

In this contribution, we give following proposals on PDCCH ordered RACH for candidate cell(s):

Proposal 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) .

Proposal 2: If reception of RAR is configured/indicated (with RAR),  RAR is received from serving cell.

Proposal 3: For TA updating mechanism, the network based updating is supported.

- References

- Chairman notes of RAN1#112