3GPP TSG RAN WG1#112-bis-e	           					     R1-2303263

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.5.1.1

Source: 	Lenovo

Title: 	Discussion on SL PRS Aspects

Document for:	Discussion

# Introduction

The Rel-18 NR expanded and improved positioning WID was agreed upon during the RAN#98-e [1] meeting, where one of the objectives included the design for SL PRS support as well as corresponding power control mechanisms for SL positioning as highlighted below:

The specific objectives of this work item are: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

During the RAN1#112 [2] meeting, the following agreements were reached in the context of SL-PRS design:

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095 Agreement A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. Agreement Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities. Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

This contribution provides a detailed discussion into the various SL physical layer procedures and reference signal design in order to support SL Positioning.

# SL Positioning Physical Layer Structure

## Pseudo-Random Sequence (Gold Sequence)

During the previous RAN1#112 meeting [2], the gold sequence defined by r(n) containing the output sequence c(i) defined in Clause 5.2.1 of TS 38.211 was agreed upon. In addition, several options on how to generate the 12-bit SL-PRS sequence ID given by nID,seqSL-PRS were agreed upon for further down-selection. It was already agreed that the value range of the SL-PRS sequence ID would be nID,seqSL-PRS∈0,1,…,4095. Option 1 seems the most straight-forward as the knowledge of identities associated with each UE are known to the higher-layers and the same method may be re-used as with the DL-PRS, however there may be some ambiguity with respect to source (initiator-UE) or destination (responder-UE) SL PRS transmission. In this case Option 5 considering destination ID may also be a feasible option as well. In Option 6 considering the source UE ID may also be a feasible option, however this requires some additional padding which may not always be desirable, and it is unclear if the MSBs or LSBs of the Source ID are used.

Proposal 1: RAN1 to further discuss and down-select among one of the following Options to generate the 12-bit SL-PRS sequence ID (nID,seqSL-PRS):

- Option 1: Higher-layer configured parameter

- Option 5: Based on 12 LSBs of destination ID.

## SL PRS Resource Definition

A key issue raised in the previous meeting was the definition of a SL PRS resource, which also further raised in AI 9.1.5.3. It has already been agreed that the SL-PRS is defined directly with respect to and contained within a single SL BWP and carrier.  Therefore, the following resource hierarchical options can be further considered to define a SL PRS resource: SL CarrierSL BWPSL Resource PoolSL PRS Resource Set SL PRS Resource. Further discussion may also be needed on the definition of SL PRS resources with respect to sub-channels as this represents the default frequency allocation for SL communications.

Proposal 2: RAN1 to define the SL PRS resource hierarchy in terms of SL Carrier SL BWPSL Resource PoolSL PRS Resource Set SL PRS Resources. Further consider the definition of a SL PRS resource with respect to one or more sub-channels.

## SL-PRS Resource Mapping

### PSCCH/Control Information associated with SL-PRS

A further consideration is the SL PRS resource mapping to the physical resources. The PSCCH is transmitted in the same slot as the corresponding PSSCH, and the transmission of the SL-PRS in relation to the current SL operation needs further discussion.  Normally, the PSCCH and PSSCH can be sent in every slot of a resource pool, where the PSCCH is multiplexed in the same slot with the associated PSSCH in non-overlapping resources. It is transmitted from the second symbol in the slot and begins from the lowest PRB within a sub-channel(s) used by the PSSCH. The number of symbols for PSCCH is pre-configured for each resource pool and can be 2 or 3. In the frequency domain, PSCCH occupies a pre-configurable number of PRBs. The number of symbols and PRBs for PSCCH allows for different allocations in time and frequency domains, depending on the size of the sub-channel. For example, with a large sub-channel, PSCCH can occupy more PRBs and only need 2 symbols, while with a smaller sub-channel, it may need more symbols to occupy fewer PRBs.

In terms of SL positioning or ranging, multiple SL PRS transmissions to multiple UEs can be multiplexed within a given slot provided each UE is configured with a frequency offset (RE offset). In one example, the PSCCH and SL-PRS are multiplexed in time domain as shown in Figure 1-Option A, while in Option B of Figure 1, the PSCCH only occupies a subset of PRBs of a subchannel and there is a FDM between PSCCH and SL-PRS in the PSCCH symbols. The remaining region of the slot is dedicated to SL-PRS transmission. This illustrative example enables multiplexing of PSCCH and SL-PRS within the same slot. For Option A, RE offset is required between the two SL-PRS transmissions in the frequency domain, while in Option B, has a custom RE offset configuration for supporting PSCCH transmitted in the first set of PRBs within a subchannel and the SL-PRS will not be mapped in the overlapping PSCCH REs.

NOTE: The above is described with respect to a dedicated resource pool, where in this case a single stage SCI is sufficient to carry the desired control information associated to a SL-PRS transmission. In the case of shared resource pool, the control information structure may be different, e.g., use of a 2-stage SCI.

Figure 1: PSCCH and SL-PRS multiplexing options within a slot

Proposal 3: Support PSCCH which carries a single stage SCI associated with SL-PRS transmission(s) for a dedicated resource pool.

Proposal 4: Consider the support of the following options for PSCCH and SL-PRS multiplexing in a dedicated resource pool:

- Option A: PSCCH occupying all configured subchannels

- Option B: PSCCH occupying a subset of PRBs within a given subchannel

The options of dedicated pool and shared pool structure were discussed during the study phase. However, it was agreed that the use of a shared pool will have to ensure backward compatibility, which may be a challenging endeavour considering the additional complexity involved to support such backward compatibility. This would imply that the SL-PRS would have to be multiplexed with SL data transmissions. Therefore, further study is required in order to support shared resource pool sharing.

The selection and reservation/indication of SL-PRS resources is best indicated using the sidelink control information (SCI), which among other aspects, assists other UEs determine the resources used for transmitting SL-PRS. The reservation/indication of SL-PRS resource using the SCI is also more dynamic and can be transmitted in a low latency manner, when compared to higher-layer signalling, especially considering Scheme 1 and 2 procedures.  Another further aspect for consideration is to understand whether there is a need to support a single SCI or two-stage SCI. The first stage SCI traditionally carriers the key control information associated with the PSSCH and 2nd stage SCI. In the case of the SL Positioning, the 1st or single stage SCI can may at least convey the time-frequency SL-PRS information including reservation/indication information. Other further information may include comb pattern size, number of symbols, whether repetitions of SL-PRS are enabled or not.

Proposal 5: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc.

### Comb Size (N) and Symbol Length (M) Design

During the previous RAN1#112 meeting [2], the following permutations of comb sizes for dedicated and shared resource pools are supported as shown in Table 1.

Table 1: Supported Comb sizes according to SL-PRS resource pool type

| Comb Sizes (N) | Dedicated Resource Pool | Shared Resource Pool |
| --- | --- | --- |
| 1 | FFS | Supported |
| 2 | Supported | Supported |
| 4 | Supported | Supported |
| 6 | Supported | FFS |
| 8 | FFS | FFS |
| 12 | FFS | FFS |
| > 12 | Not Supported | Not Supported |

In the case of a dedicated resource pool, there is sufficient resource availability within a slot to accommodate comb size of N= 8 and N=12 respectively, although in the case of a shared resource pool such higher comb size including N= {6, 8 and 12} may be challenging to provision. In the case of a comb size of 1 in a dedicated resource pool, may also be feasible albeit with a lower resource efficiency in the case of a single UE transmitting SL-PRS with a lower accuracy.

Proposal 6: Support the following options of SL-PRS comb sizes (N) in the following cases:

- For a dedicated resource pool, support N = {1,8,12}

- For a shared resource pool, support N= {6} and do not support N= {8,12}

The symbol length design is inter-dependent with the yet to be finalized comb sizes. Agreements were already made to support SL-PRS patterns for both dedicated and shared SL-PRS resource pools. Fully staggered SL-PRS patterns with M=N have been agreed to be supported with M ={2,4}, while M={6} remains FFS. On the remaining FFS point, further consideration can be given to support M={6} at least for a dedicated SL-PRS resource pool. In addition, pending an agreement on Comb size N=8, the corresponding symbol length of M=8 may also be supported, i.e. SL-PRS pattern (M, N) = (8, 8).

Proposal 7: At least for a dedicated resource pool, support at least M=6 for a fully staggered SL-PRS, i.e. (M, N) = (6, 6). Additional M=8 may also be supported pending support of N=8 for a dedicated resource pool.

According to the Uu positioning design a maximum comb-size of 12 could be supported, however due to the nature of SL slot structure, this may be challenging to support. The reason being that an AGC may occupy 1 symbol, PSCCH may occupy 2 or 3 symbols, Tx-Rx turnaround time (Guard symbol) may occupy a further 1 symbol, the number of available symbols for SL-PRS may be 9 or 10 symbols, depending on the type of configuration. This takes into account any potential overhead including whether SCI is transmitted along with SL-PRS and how SCI and SL-PRS are multiplexed as shown in Figure 1-Option A and Option B. The maximum supported M symbol lengths may range from 10-12 symbols depending on the SCI-SL-PRS multiplexing in the case of a dedicated resource pool. As result, the maximum symbol length is dependent on the comb size, start SL-PRS symbol and overall slot structure depending the configuration of overhead symbols.

Proposal 8: RAN1 to further consider M>N SL-PRS fully staggered patterns, e.g., M= {10,11,12} depending on the maximum supported comb size, start symbol in a slot and slot structure configuration, e.g., number of AGC, PSCCH and Tx-Rx turnaround symbols.

#### SL-PRS Comb-based UE Multiplexing

During the previous RAN1#112 meeting [2], it was agreed that comb-based UE multiplexing was supported at least for the dedicated resource pool. In the case of a shared resource pool, the complexity of utilising comb-based UE multiplexing may increase, and therefore it is recommended to restrict comb-based UE multiplexing to the SL-PRS dedicated resource pool.

Proposal 9: Restrict comb-based UE multiplexing to only the dedicated SL-PRS resource pool.

It has also been agreed that a single same SL-PRS pattern among different UEs can be supported. The comb pattern design should further consider whether multiple user multiplexing and multiple comb pattern multiplexing e.g., SL-PRS patterns = {2,4},{2,6},{2,8,{4,8} (in line with partial staggering) in a slot is possible to increase the resource efficiency and at the same time obtain good detection performance (See Figure 2 for a simple illustration not drawn to scale). Multiple users can be multiplexed in a slot with different RE offset as shown in Figure 2. The resource selection procedure is simplified when same comb size in a slot is used, e.g., in Mode-2, while when allocating different comb size to different UEs in a slot, further consideration should be given in order to avoid overlapping SL-PRS REs.

Proposal 10: Support different (multiple) SL-PRS comb (M, N) pairs for comb-based UE multiplexing in a slot together with any associated restrictions, e.g., avoid overlapping SL-PRS REs, where applicable.

Figure 2: Example illustration of SL-PRS user multiplexing in a slot

#### SL-PRS TDM-based Multiplexing

It was also further agreed that TDM-based multiplexing of SL-PRS from different UEs in a slot is supported at least for dedicated resource pools. In a dedicated resource pool, if data transmission is not supported, this may potentially generate dummy transmissions with blank symbols along with a SL-PRS transmission within a slot. For example, aside from one symbol AGC, one symbol Tx-Rx turnaround time (Guard symbol) and 2 symbols potential PSCCH transmission (if any), it needs 8, 6 and 4 symbols of dummy transmissions (blank symbols) corresponding to the transmission of 2, 4 and 6 SL-PRS symbols, respectively within a 14 symbols slot. It obviously wastes large amounts of resources and may be resource inefficient. A solution to this is to adopt a mini-slot structure considering various symbol length of SL-PRS e.g., for 2, 4 and, 6 symbols, which can be TDMed in a consecutive manner at least for a dedicated SL Positioning resource pool. In this way, different UEs may be multiplexed or the overall total number of potential SL-PRS transmission occasions within one slot are further increased. Further discussion is required on the feasibility of this approach to the shared resource pool.

Proposal 11: Mini-slot of various symbol length of SL-PRS e.g., 2, 4, and 6 can be further studied at least for dedicated resource pools. FFS the applicability to shared resource pools.

## Resource Pool Aspects

In Rel-16 NR sidelink, multiple resource pools are configured with a SL BWP and only one SL BWP is configured in a sidelink carrier which is common for all sidelink devices. The DL PRS configuration consists of positioning frequency layer since the DL PRS is not limited to within a DL BWP configuration so that the DL PRS transmission can be measured over a wide enough bandwidth when compared to the BWP configuration. The resource pool design may be restrictive since the SL PRS is limited according to the resource pool size and is not attractive for forward compatibility in the case aggregating different SL PRS resources.   However, the agreement made during the RAN1#110-bis-e, confines the SL-PRS bandwidth to that of the resource pool based on two alternatives. In general, enhanced positioning performance arises from the use of as much as bandwidth as possible, and therefore the motivation of Alt. 1 of using a smaller bandwidth than a resource pool is not clear. We therefore support Alt. 2, where the bandwidth of the SL-PRS shall be the same as that of the resource pool.

Proposal 12: In the case of a dedicated resource pool, the bandwidth of SL-PRS shall be the same as that of the resource pool.

Power control and Interference

The following work item was objective regarding power control was agreed upon in [1] was made with respect to SL positioning power control and interference aspects:

SL Power Control WI Objective - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]..

Further agreements were also made in RAN1#112 [2], to use open loop power control (OLPC) mechanisms as a starting point for SL PRS power control. Power control procedures for SL positioning are beneficial for several reasons including interference management, power conservation, improved spectral efficiency as well signal reception reliability.  To improve the Uu PRS detectability of further out base stations with respect to the serving cell, muting of the Uu PRS was specified, which also served to mitigate inter-cell interference. Furthermore, additional power control measures were also enabled to further improve detectability of the PRS, e.g., power boosting. Due to the short range of SL, interference amongst nodes transmitting SL PRS may also degrade positioning performance. The current SL power control methods rely on open loop methods, to manage the transmit power and in the context of SL positioning further work is also required in terms of controlling the interference between anchor nodes, which are in close proximity.

Power control mechanisms should be enabled from the transmitting UE, which may include an Anchor UE or Target-UE as well among a group of transmitting UEs, in the case of performing positioning techniques, which involve multiple nodes, e.g., SL -TDoA and SL-RTT from multiple nodes. The SL positioning transmitting UEs may compute the SL pathloss reference based on the reported SL PRS RSRP or any other SL RS RSRP. For in-coverage scenarios or partial coverage scenarios, the DL pathloss reference may also be used in line with existing SL power control mechanisms.

Proposal 13: Support SL PRS open loop power control mechanisms based on SL and DL pathloss references and SL PRS and/or other RS RSRP reporting.

In addition, for methods which require frequent bi-directional SL-PRS transmissions, e.g., SL-RTT, it would be beneficial for the transmitting UE to share power control related transmitted parameters with the receiving UE, e.g., SL PRS transmit power, to minimize overhead, e.g., re-computing the pathloss reference based on the RSRP. This would be applicable if the positions of both of UEs remain the same at the point of SL-PRS transmissions.

Proposal 14: Support power control parameter(s) sharing, e.g., SL RS/PRS transmit among UEs performing SL positioning.

There is a need to also employ SL PRS power control and the adaptation of SL PRS transmit power may be depending on the determination of the SL pathloss reference with respect to other anchor nodes. In a SL positioning scenario, requiring two or more anchor nodes, have knowledge of the other anchor’s node SL PRS transmit power characteristics may help in adapting the SL PRS transmit power of the concerned anchor node. Figure 3 is an illustration of enabling one anchor node to adapt its SL PRS transmit power with respect to other anchor nodes participating in the SL positioning session (e.g., performing SL-TDoA). According to Figure 3, the Anchor UE 1, can utilise the information received from other anchor nodes to adapt its SL PRS transmit power to the Target-UE depending on its distance with respect to the Target-UE and minimizing any interference impacts.

Figure 3: Power control amongst Anchor Node UEs for SL positioning, e.g., SL-TDoA

Since the pseudo-random sequence (Gold sequence) has been selected for SL-PRS, potential further interference impacts need to be mitigated. As with the Uu interface PRS interference mitigation muting may assist in reducing the impacts of interference. A muting configuration may apply to all configured, e.g., periodical SL PRS transmitted from Anchor devices or may apply to a subset of SL PRS resources within a whole configured set of SL PRS resources.

Proposal 15: RAN1 to support muting of SL-PRS transmissions. FFS further details such as the type of muting configurations, etc.

# Conclusion

The proposals relating to the SL Positioning RS design are summarized as follows:

Proposal 1: RAN1 to further discuss and down-select among one of the following Options to generate the 12-bit SL-PRS sequence ID (nID,seqSL-PRS):

- Option 1: Higher-layer configured parameter

- Option 5: Based on 12 LSBs of destination ID.

Proposal 2: RAN1 to define the SL PRS resource hierarchy in terms of SL Carrier SL BWPSL Resource PoolSL PRS Resource Set SL PRS Resources. Further consider the definition of a SL PRS resource with respect to one or more sub-channels.

Proposal 3: Support PSCCH which carries a single stage SCI associated with SL-PRS transmission(s) for a dedicated resource pool.

Proposal 4: Consider the support of the following options for PSCCH and SL-PRS multiplexing in a dedicated resource pool:

- Option A: PSCCH occupying all configured subchannels

- Option B: PSCCH occupying a subset of PRBs within a given subchannel

Proposal 5: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc.

Proposal 6: Support the following options of SL-PRS comb sizes (N) in the following cases:

- For a dedicated resource pool, support N = {1,8,12}

- For a shared resource pool, support N= {6} and do not support N= {8,12}

Proposal 7: At least for a dedicated resource pool, support at least M=6 for a fully staggered SL-PRS, i.e. (M, N) = (6, 6). Additional M=8 may also be supported pending support of N=8 for a dedicated resource pool.

Proposal 8: RAN1 to further consider M>N SL-PRS fully staggered patterns, e.g., M= {10,11,12} depending on the maximum supported comb size, start symbol in a slot and slot structure configuration, e.g., number of AGC, PSCCH and Tx-Rx turnaround symbols.

Proposal 9: Restrict comb-based UE multiplexing to only the dedicated SL-PRS resource pool.

Proposal 10: Support different (multiple) SL-PRS comb (M, N) pairs for comb-based UE multiplexing in a slot together with any associated restrictions, e.g., avoid overlapping SL-PRS REs, where applicable.

Proposal 11: Mini-slot of various symbol length of SL-PRS e.g., 2, 4, and 6 can be further studied at least for dedicated resource pools. FFS the applicability to shared resource pools.

Proposal 12: In the case of a dedicated resource pool, the bandwidth of SL-PRS shall be the same as that of the resource pool.

Proposal 13: Support SL PRS open loop power control mechanisms based on SL and DL pathloss references and SL PRS or other RS RSRP reporting.

Proposal 14: Support power control parameter(s) sharing, e.g., SL RS/PRS transmit among UEs performing SL positioning.

Proposal 15: RAN1 to support muting of SL-PRS transmissions. FFS further details such as the type of muting configurations, etc.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, RAN#98-e, Dec. 2022.

- R1-2302065, “Session notes for 9.5 Expanded and Improved NR Positioning”, Ad-Hoc Chair (Huawei), RAN#112, Feb. 2023.

- TR 38.859, “Study on expanded and improved NR positioning (Release 18)”, V1.0.0., Dec. 2022.

- 3GPP TS38.211, “Physical channels and modulation”, V17.3.0, Sept. 2022.