3GPP TSG RAN WG1#112-bis-e	           					     R1-2303264

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.5.1.2

Source: 	Lenovo

Title: 	Discussion on SL Positioning Measurement and Reporting

Document for:	Discussion

# Introduction

The Rel-18 NR expanded and improved positioning WID was agreed upon during the RAN#98-e [1] meeting, where one of the objectives included the measurement and associated reporting procedures for SL positioning as highlighted below:

The specific objectives of this work item are: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

During the RAN1#112 [2] meeting, the following agreements were reached in the context of SL positioning measurement and reporting design:

Agreement SL PRS reference signal received power (SL PRS-RSRP) - is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. Agreement SL PRS reference signal received path power (SL PRS-RSRPP), - is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. Agreement SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN Agreement Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. Agreement Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3 Agreement Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE). Agreement Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL-PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded Agreement LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). - LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. - No specification impact for how to set this indicator. - From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18. Agreement Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e. Note: this does not imply a different measurement report content for reporting to LMF or to UE. Table 6.2 Collection of measurement report content reporting to LMF; reporting to UE. SL-PRS based Rx-Tx measurement. SL-PRS based RSTD measurement. SL-PRS based RSRP measurement. SL-PRS based RSRPP measurement. SL-PRS based RTOA measurement. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement. etc.

This contribution provides a detailed discussion into the measurement and reporting procedures in order to support SL Positioning.

# SL Positioning Techniques

During the RAN1#112 meeting [2], a set of SL Positioning measurements were agreed upon. The agreed measurements support the following SL positioning techniques as follows:

- SL timing-based positioning methods

  - SL-TDoA

  - SL-RTT type solutions

- SL angular-based positioning methods

  - SL-AoA

## SL-TDoA

NR DL-TDoA and UL-TDOA has already been employed since Rel-16 on the Uu-interface and usually requires at least 3 or more anchor nodes (including a reference node) and can be easily extended to support SL absolute positioning of the target-UE. The anchor nodes may comprise of RSUs with known fixed locations and are usually deployed alongside a highway or at traffic intersections in the context of V2X scenarios. Another approach would be to utilise static UEs with fixed known locations to perform SL-TDoA.  Some challenges include achieving the tight synchronisation requirements among RSUs and UEs as well as localizing highly mobile UEs. An additional challenge would involve the mobility of anchor nodes along with the target UE. Therefore, enhancements in terms transferring of synchronization-related information and associated mitigation need to be considered.

In order to avoid inefficient signalling, the DL-TDoA-like and UL-TDoA-like operation can be jointly utilised to measure the absolute position of the UE. In this manner the RSTD and RTOA measurements can be both leveraged within a single SL positioning session. Key issues such as procedures for coordinating such a combined TDoA method and joint SL RSTD and SLRTOA measurement need to be further considered.

Proposal 1: RAN1 to support SL-RSTD and SL-RTOA physical layer measurements in any respective order within a single SL positioning session.

### SL RTOA Measurements

SL-TDoA has been agreed to be introduced to support UL-TDOA-like operations. During RAN1#112 [2], the SL-PRS based RTOA was defined with respect to a RTOA reference time. The selection of the nominal timing for performing SL RTOA measurements was an FFS issue and largely depends on the coverage of the group of UEs involved in this UL-TDOA like procedure. For example, if the group of UEs involved in deriving the RTOA measurement are all in-coverage, it would be beneficial to select SFN 0, while if all UEs are out-of-coverage, then it would be more appropriate to select DFN 0. The partial coverage scenario would depend on whether target-UE is in-coverage or out-of-coverage. This allows flexible selection of the nominal timing depending on the scenario.

Proposal 2: Support flexible selection of reference timing for deriving the SL RTOA measurements depending on the scenario, e.g., SFN 0 for all in-coverage group of UEs or DFN 0 for all out-of-coverage group of UEs.

Another remaining issue is the source of reference timing, which also somewhat depends on the coverage of the group of UEs (including target-UE) involved in deriving the SL RTOA measurement. The source of reference timing for general SL communication includes network-based timing, e.g., gNB and GNSS-based timing, and UE-based timing. These sources can be re-used along with a prioritization scheme associated to each reference timing source.

Proposal 3: Sources of the reference timing for SL RTOA measurements may support network-based timing, e.g., gNB, GNSS-based timing and UE-based timing. FFS prioritization of reference timing sources.

### SL RSTD Measurements

Similarly DL-TDoA operations has also been agreed to be supported for SL-TDOA. For the DL-TDOA-like operation of SL-TDoA, the SL reference signal time difference measurements (RSTD) between a reference anchor UE/node and another anchor UE/node needs to be considered. SL reference signal time difference (SL RSTD) may defined as the SL relative timing difference between the UE (node) j and the reference UE (node) i, defined as TSubframeRx,j – TSubframeRx,i, where: TSubframeRx,j is the time when the UE receives the start of one subframe from UE (node) j and TSubframeRx,i is the time when the UE receives the corresponding start of one subframe from UE (node) i that is closest in time to the subframe received from UE (node) j. Additionally, one or more SL PRS resources can be used to determine the start of one subframe from a UE. The target-UE measures the SL RSTD with respect to the reference anchor node or a UE in order to derive the RSTD measurement.

Proposal 4: SL RSTD may be defined as SL relative timing difference between the UE (node) j and the reference UE (node) i, defined as TSubframeRx,j – TSubframeRx,i, where: TSubframeRx,j is the time when the UE receives the start of one subframe from UE (node) j and TSubframeRx,i is the time when the UE receives the corresponding start of one subframe from UE (node) i that is closest in time to the subframe received from UE (node) j.

#### Reference UE Details

A reference anchor node or UE should be selected with respect to of available SL anchor UE/s/nodes in order successfully measure the SL RSTD. This selection mechanism can be viewed in conjunction with the broader mechanism in which candidate anchor UEs are selected in order to perform SL-TDOA. The mechanisms used to select anchor UEs may also be extended to the selection of reference anchor UEs/UEs. In addition, criteria to select reference UEs may involve PHY measurements, e.g., SL PRS RSRP/RSRPP measurements, link quality, etc and would require further discussion  Further RAN2 coordination may also be needed.

Proposal 5: Selection of a reference UEs for performing SL RSTD measurements may use the same procedures used to select anchor UEs. FFS reference UE selection criteria. RAN2 coordination may also be required.

### SL-TDoA Synchronization Assistance Information

The aim of the exchange of synchronization assistance information is to improve the performance of SL-TDoA as opposed to developing new SL synchronization mechanisms, which require additional specification efforts. For example, RSUs may share RTD information to the target-UE to assist in compensating for synchronization-related offsets.

Observation 1: Exchange of Synchronization assistance information can be considered beneficial for improving overall SL-TDOA performance.

Support is therefore required for signalling the relative time differences (RTD) between a reference anchor UE/node and the other respective anchor UEs/nodes. Such information will assist in the correction of any synchronization errors, e.g., real-time differences arising from the transmission of SL-PRS from the different anchor nodes. Traditional SL makes of the synchronisation source selection in order transmit/receive over the SL and examples of such sources includes GNSS, gNB and UE-based sources. The type of positioning technique may influence the synchronization source selection, especially in the case of SL-TDOA, where the anchors would ideally need to select the same synchronisation source.

Proposal 6: RAN1 to support synchronization error compensation based on SL synchronisation source selection. FFS the details for synchronization error compensation e.g., sharing of RTD information between anchors and target-UE, SL synchronisation source selection, etc.

### Expected SL RSTD and SL RTOA Measurement Details

In order to enable accurate measurement of the SL RTOA and SL RSTD measurement, it is beneficial that the anchor UE or target-UE be configured with a search window or time interval in order to expect the SL RTOA and SL RSTD measurement. Although, this requires a prior approximate location of the UE, this could serve as important assistance information for the UE measurement. Further discussion may be needed on how the this approximate location may be obtained based on SL RS measurements, zone information, etc.

Proposal 7: RAN1 to support the provision of SL RSTD and SL RTOA expected search window information for anchor UEs or target-UE.

## SL-RTT

Multi-cell Round trip time (Multi-RTT) has also been supported on the Uu interface since Rel-16 to perform position estimation using the round trip signal time measurements at the gNB (using gNB Rx-Tx time difference measurements) and at the UE (using UE Rx-Tx time difference measurements). RTT requires at least one anchor or non-anchor to determine its absolute or relative location estimate, and the positioning performance is enhanced as more nodes are involved or in cases where additional SL-PRS is transmitted, e.g., Double-sided SL-RTT. In the case of SL positioning, two variants of RTT can be supported including single-sided and double-sided SL-RTT and are described below.

### Single-sided (SS) SL-RTT Measurements

The single-sided (one-way) RTT method is similar in principle to the Multi-RTT technique whereby an initiator UE or multiple initiator UEs transmit SL PRS signal(s) and the responder UE transmits a reply signal, which may include a corresponding SL PRS transmission, and thereafter the responder UE performs the single-sided RTT measurement considering the receive and transmit timing of the SL PRS. The overall procedure is illustrated in Figure 1. The signal propagation time between a pair of UEs is used to derive the distance/range between the devices as a function of the UE-1 Ranging Round Duration (A) and UE-2 Reply Time Duration (B) (which is similar to the UE Rx-Tx time difference measurement), where ε is sum of the absolute clock frequency offset errors, each independently measured at the Initiator and Responder UE. This can be calculated as follows:

Tpropogation-delay=A-B-ε2,

Figure 1: Single-sided SL-RTT procedure

It can be noted that the propagation delay computation between two SL UEs is also dependent on timing errors such as clock offset at each UE, which require compensation for an accurate range estimate. The performance can potentially degrade due to lack of clock synchronization at either Initiator or Responder UE.

Observation 2: SS SL-RTT is susceptible to UE clock offset errors at the Initiator and Responder UE.

### Double-sided (DS) SL-RTT Measurements

The double-sided (two-way) RTT method draws from the same principles of one-way RTT but can eliminate the effects of the clock offset errors via the introduction of 2nd stage RTT round as noted in Figure 2. This method is essentially a two-stage RTT procedure where the TX-UE of the 1st phase is the Initiator UE and the TX-UE of the 2nd phase is the Responder UE. This brings about an improved distance/ranging estimate between the pair of UEs and provides robust timing measurements in the presence of uncompensated clock offsets, which is an issue for the SS SL-RTT method. This ranging method can also operate based asymmetric reply times, depending on the SL traffic and SL resource availability.

The signal propagation time (Tpropagation-delay) can be used to derive the range between the Initiator-UE and Responder-UE as a function of the TX-Device RTT Round Durations (A and D) and RX-Device Reply Time Durations (B and C) and is represented as follows:

Tpropogation-delay=A×D-B×C(A+B+C+D),

The independent clocks at the Initiator and Responder UEs can be assumed to run at a factor of δ1 and δ2 , respectively with respect to an ideal/master clock, which is assumed to be constant over the course of the two-way SL-RTT exchange and has been taken into account.

Figure 2: Double-sided SL-RTT procedure

Observation 3: DS SL-RTT can compensate for UE clock offsets at both Initiator and Responder UEs.

### Definition of SL-RTT Measurements

The definition of SL-RTT measurements can be generalized irrespective of whether single-side or double-side SL-RTT is employed. According to the RAN1#112 agreement, the following alternatives are to be down-selected for the definition of SL-PRS Rx-Tx measurement:

- Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement

- Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

The preference would be to support Alt 1, where the actual SL-PRS transmission times are used to derive the Rx-Tx time difference measurement as opposed to the legacy Rel-16/Rel-17 definitions. The actual SL PRS transmission and reception timings provide more accurate and meaningful information, when deriving the Rx-Tx time difference measurement.

Proposal 8: RAN1 to support Alt.1 in defining the SL PRS-based Rx-Tx time difference measurement, where the actual SL-PRS transmission times are incorporated in the definition.

### SL-PRS transmission order for SL-RTT Measurements

Another remaining issue was the SL-PRS transmission order applicable to double-sided RTT. The double-sided RTT can re-use the framework of the agreed UE Rx-Tx time difference measurements by combining measurements from both the Responder UE and Initiator UE perspective. As noted in Figure 2, further discussion is required on how to re-use the UE Rx-Tx time difference measurement when the SL PRS transmission order is not fixed, which may move away from the principle of doubles-sided RTT. Therefore, it is recommended to restrict the transmission order of SL-PRS in order re-use the definition of the UE Rx-Tx time difference measurement. However, for the scenario where the transmission order is not restricted, this can be considered as another version of deriving the SL-RTT and not necessarily double-sided RTT. The transmission from each perspective can be modified depending on the number of SL-PRS transmissions from the initiator UE or responder UE perspective.

Proposal 9: In terms of the SL UE Rx-Tx time difference measurement definition for double-sided RTT, the order of SL-PRS transmission for double-sided RTT is restricted.

Proposal 10: Support creating an alternative SL-RTT definition where the order SL PRS transmission is not restricted.

### Multi-UE SL-RTT in a SL Positioning Group

SL-RTT can also exploit the derivation of absolute position estimates from multiple UEs within a SL positioning group. Further discussion is also needed on whether the SL-RTT definition is impacted based on the target-UE performing SL-RTT with multiple UEs in a group. This enables one-to-many and many-to-one SL PRS transmissions to enable Multi-UE SL-RTT. This can be applicable to both single-sided and double-sided SL-RTT variations.

Proposal 11: Support Mult-UE SL-RTT within a SL Positioning Group, where both one-to-many (groupcast) and many-to-one (unicast) SL-PRS transmissions are enabled.

## SL-AoA

### SL-AoA Measurements

SL-AoA may be defined in terms of the azimuth and zenith (vertical) of UE with respect to a reference direction. The reference direction/axes may include either LCS or GCS coordinate systems. LCS information may require translation to the GCS in order to share such measurements with other UEs/nodes. During RAN1#112 [2], it was further agreed that both the GCS and LCS are supported when performing the SL Azimuth and Zenith of Arrival measurements.

The location of translation function may depend on where the measuring UE and/or UE/node that receives such SL-AoA information. The translation function can assist in processing the AoAs using a global reference using the sequence of 3 rotation angles, α, β and γ. In the case that the target-UE is determining the relative direction/orientation with respect to another UE, the SL-AoA computed in the LCS may be sufficient, provided that the relative reference direction/orientation is already known at the target-UE.

Proposal 12: For direct ranging for relative direction/orientation estimation with respect to another UE/node computed at the target-UE, SL-AoA and SL-ZoAs in LCS may be sufficient.

Proposal 13: RAN1 to discuss the location of the LCS-to-GCS translation function, e.g., at the target-UE or anchor UE, which affect the signalling of the 3 rotation angles α, β and γ.

Additional assistance information by SL-AoA measurement UE, e.g., target-UE or anchor UE can be used to improve the accuracy of SL-AoA measurements. This information may include expected SL-AoA ranges based on a priori location information as well as per path indications to support enhanced SL-AoA measurements in the presence of multipath.

Proposal 14: RAN1 to support SL-AoA assistance information for improving the accuracy of SL-AoA, e.g., expected SL-AoA ranges, SL-AoA path indications.

# SL Positioning Measurement Report Content

During RAN1#112 [2], a measurement report content template shown in Table ?? was agreed upon associated to each of the SL Positioning measurements.

Table 1: SL Positioning Measurement Report Content

| No. | SL Positioning Measurement | Reporting to LMF | Reporting to UE |
| --- | --- | --- | --- |
| A | SL-PRS based Rx-Tx measurement | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS Rx-Tx measurement 4) SL-PRS measurement quality, e.g., timing quality 5) LOS/NLOS Indicator | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS Rx-Tx measurement 4) SL-PRS measurement quality, e.g., timing quality 5) LOS/NLOS Indicator |
| B | SL-PRS based RSTD measurement | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSTD measurement 4) SL-PRS measurement quality, e.g., timing quality 5) LOS/NLOS Indicator | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSTD measurement 4) SL-PRS measurement quality, e.g., timing quality 5) LOS/NLOS Indicator |
| C | SL-PRS based RSRP measurement | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSRP measurement 5) LOS/NLOS Indicator | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSRP measurement 5) LOS/NLOS Indicator |
| D | SL-PRS based RSRPP measurement | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSRPP measurement 5) LOS/NLOS Indicator | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSRPP measurement 5) LOS/NLOS Indicator |
| E | SL-PRS based RTOA measurement | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSTD measurement 4) SL-PRS measurement quality, e.g., timing quality 5) LOS/NLOS Indicator | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS RSTD measurement 4) SL-PRS measurement quality, e.g., timing quality 5) LOS/NLOS Indicator |
| F | SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement 4) LOS/NLOS Indicator | 1) Measurement ID to unambiguously identify the SL-PRS resource associated to the measurement, e.g., based on, SL PRS resource set IDs, SL PRS resource IDs 2) UE/entity ID which performed the measurement, e.g., UE-ID, source-ID, Destination-ID, FFS need for anchor UE ID, server UE ID 3) Timestamp of SL-PRS Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement 4) LOS/NLOS Indicator |

Proposal 15: RAN1 to support SL Positioning Measurement Report Content as described in Table 1.

# SL Common Measurement, Processing and Reporting

SL positioning measurements can be performed by the target-UE in a similar fashion to the current Uu positioning framework. The target-UE is usually configured separately to report certain measurements based on the positioning method and can also be extended to the SL positioning framework. RAN2 has further agreed in the following functionality for the SLPP positioning protocol:

TR 38.859 SL Positioning Messaging Functionality SLPP is introduced to support at least the following functionalities: - SL Positioning Capability Transfer - SL Positioning Assistance Data exchange - SL Location Information Transfer - Error handling - Abort

This follows a similar positioning framework as the Uu interface. It is therefore straightforward to perform measurement configuration and reporting via the Location Information transfer message. The location information transfer can transfer the supported SL positioning measurements and in cases where the absolute/relative position and/or range information has been computed, the resulting location information. In addition, these measurements and location information can be requested and reported per positioning method.

Proposal 16: SL Location Information transfer message may be used to request and transfer SL positioning measurements and location information and requested/reported per SL positioning method. Send LS to RAN2.

Figure 5 is a general conceptual view of the measurement, processing and reporting flow for SL positioning.

Figure 5: Measurement, processing and reporting flow for SL positioning

A portion of the time has to also be dedicated for processing the SL PRS measurements depending on different UE capability types. The measurement UE may indicate to the configuration entity, the duration of SL PRS symbols N in units of ms that it may process every T ms assuming a maximum SL PRS bandwidth. The type of capabilities can affect the amount of SL PRS resources, a UE can process in a given time as well as the latency of processing the SL PRS. The configuration entity may then configure a set of SL PRS resources based on the SL PRS processing capabilities of the UE.

Proposal 17: RAN1 to consider different SL PRS processing capabilities depending on the UE-type.

Another key issue is the prioritization of SL positioning measurements depending on the transmitting entity/node, e.g., anchor-UEs, initiator-UE, responder -UE or others UEs, which are deemed mobile or static. It is assumed that the transmitted SL-PRS originates from nodes known to be generally mobile (depending on the class of mobility, e.g., low speeds/high speeds), even though there can be fixed SL transmitters such as roadside units (RSU) or UEs that are temporarily stationary. Given the distributed nature of SL, there may be a host of candidate nodes available for selection by a SL measurement UE. There is a therefore a need to prioritize SL-PRS measurements that originate from a SL-PRS transmitter whose position is well known with good accuracy/confidence interval/good reliability.

Proposal 18: RAN1 to further study the prioritization of SL positioning measurements depending on the known position and accuracy of the transmitting SL UE.

The SL Positioning framework should also be capable of supporting different types of reporting of both SL positioning measurements and absolute/relative location estimate. RAN2 has already agreed during the RAN2#119-bis-e meeting, that the new potential SL positioning layer comprises functionality to perform location information exchange, which may include measurement reporting and location estimate exchange. Table 2 shows the various reporting types depending on the reporting configuration.

Table 2: Types of SL Positioning Reporting

| SL Positioning Reporting Types | Overview |
| --- | --- |
| One-shot | - The UE/device performing measurements may report to the positioning calculation entity in a one-shot manner to support immediate reporting. - The UE (positioning calculation) entity may report the absolute/relative location estimate to the requesting node/network entity, e.g., UE, LMF, etc. |
| Triggered | - The UE/device performing measurements may report to the positioning calculation entity based on defined event-based criteria, e.g., based on a location change/cell change. - The UE/device performing measurements may report to the positioning calculation entity upon trigger of a configured event. |
| Periodic | - The UE/device may provide periodic positioning measurement or location estimate reports with the same time interval between two consecutive reports. |

Proposal 19: Support different SL Positioning reporting types including one-shot, triggered (event-based) and periodic reports, where such reports may be based on higher-layer signalling.

# Joint Uu and SL Positioning Measurements

In order to support Hybrid positioning model using both Uu and SL interfaces, it also necessary to discuss the relationship of the Uu PFL with SL PRS resource pool in order to efficiently process and measure both Uu and SL PRS. A UE supporting both legacy LPP positioning (Uu positioning) and SL positioning may perform joint Uu and SL PRS measurement and processing depending on the time instance in which both the Uu and SL measurements are available for measurement and processing. The goal would be to minimize the time required to perform existing Uu and new SL positioning measurements, in order to report or utilise the measurements in hybrid manner for positioning calculation.

Proposal 20: RAN1 to discuss the relationship between Uu PRS and SL PRS configuration and associated measurements for Uu and SL hybrid positioning. FFS whether such measurements are performed in a joint manner or separately measured and how these measurements are utilised.

# SL Positioning Reference Units UEs

The positioning reference unit (PRU) with a known fixed location has been extensively discussed Rel-17 Uu positioning to assist in improving the accuracy of the location estimate, especially for timing-based positioning methods. However, due to lack of time and consensus among WGs, this feature was not further specified aside from PRU UE definition in TS38.305.

In the context of SL and considering the challenges in achieving stable location accuracy performance for multiple mobile (mobile) nodes, the SL PRU can be considered to help mitigate timing clock/offsets errors using reference measurements from a fixed known location, e.g., an RSU may act as a PRU. This achieved via compensating the pseudo-range errors arising from the Rx/Tx timing delays from the transmitting and receiving UEs. UEs may be more prone to clock drifts at the transmitter and receiver end depending on the hardware implementations.

Observation 4: Transmitter and receiving UEs may be susceptible to Tx/Rx errors impacting positioning performance due to uncompensated clocks based on different hardware implementations.

The positioning calculation entity (e.g., UE or network node) can exploit the PRU’s known precise location to compensate for timing-based errors such as delays embedded within the received measurements by applying (double) differential computations. This is similar in concept to the reference stations employed by GNSS systems to compensate for pseudo-range errors by the GNSS receivers by exploiting the fact that receivers in the same general area, e.g., within a few hundred km between the receivers, experience the same atmospheric impairments.  The actual known precise location of the reference stations is therefore an important pre-requisite.

Due to the distributed nature of SL and the type of environment, many of the pair of UEs may not have a good quality link, e.g., LOS, in order to perform accurate ranging. In such cases, such scenarios may require cooperation with respect to an assistance UE, in order to facilitate the ranging process. In another example, network assistance UEs, may also facilitate a SL positioning/ranging session between an in-coverage and out-of-coverage UE.

Proposal 21: RAN1 to further discuss the support of SL PRUs in SL positioning/ranging procedures.

# Conclusion

The following observations regarding potential SL positioning solutions have been noted as part of the discussion:

Observation 1: Exchange of Synchronization assistance information can be considered beneficial for improving overall SL-TDOA performance.

Observation 2: SS SL-RTT is susceptible to UE clock offset errors at the Initiator and Responder UE.

Observation 3: DS SL-RTT can compensate for UE clock offsets at both Initiator and Responder UEs.

Observation 4: Transmitter and receiving UEs may be susceptible to Tx/Rx errors impacting positioning performance due to uncompensated clocks based on different hardware implementations.

The proposals relating to the SL Positioning framework are summarized as follows:

Proposal 1: RAN1 to support SL-RSTD and SL-RTOA physical layer measurements in any respective order within a single SL positioning session.

Proposal 2: Support flexible selection of reference timing for deriving the SL RTOA measurements depending on the scenario, e.g., SFN 0 for all in-coverage group of UEs or DFN 0 for all out-of-coverage group of UEs.

Proposal 3: Sources of the reference timing for SL RTOA measurements may support network-based timing, e.g., gNB, GNSS-based timing and UE-based timing. FFS prioritization of reference timing sources.

Proposal 4: SL RSTD may be defined as SL relative timing difference between the UE (node) j and the reference UE (node) i, defined as TSubframeRx,j – TSubframeRx,i, where: TSubframeRx,j is the time when the UE receives the start of one subframe from UE (node) j and TSubframeRx,i is the time when the UE receives the corresponding start of one subframe from UE (node) i that is closest in time to the subframe received from UE (node) j.

Proposal 5: Selection of a reference UEs for performing SL RSTD measurements may use the same procedures used to select anchor UEs. FFS reference UE selection criteria. RAN2 coordination may also be required.

Proposal 6: RAN1 to support synchronization error compensation based on SL synchronisation source selection. FFS the details for synchronization error compensation e.g., sharing of RTD information between anchors and target-UE, SL synchronisation source selection, etc.

Proposal 7: RAN1 to support the provision of SL RSTD and SL RTOA expected search window information for anchor UEs or target-UE.

Proposal 8: RAN1 to support Alt.1 in defining the SL PRS-based Rx-Tx time difference measurement, where the actual SL-PRS transmission times are incorporated in the definition.

Proposal 9: In terms of the SL UE Rx-Tx time difference measurement definition for double-sided RTT, the order of SL-PRS transmission for double-sided RTT is restricted.

Proposal 10: Support creating an alternative SL-RTT definition where the order SL PRS transmission is not restricted.

Proposal 11: Support Mult-UE SL-RTT within a SL Positioning Group, where both one-to-many (groupcast) and many-to-one (unicast) SL-PRS transmissions are enabled.

Proposal 12: For direct ranging for relative direction/orientation estimation with respect to another UE/node computed at the target-UE, SL-AoA and SL-ZoAs in LCS may be sufficient.

Proposal 13: RAN1 to discuss the location of the LCS-to-GCS translation function, e.g., at the target-UE or anchor UE, which affect the signalling of the 3 rotation angles α, β and γ.

Proposal 14: RAN1 to support SL-AoA assistance information for improving the accuracy of SL-AoA, e.g., expected SL-AoA ranges, SL-AoA path indications.

Proposal 15: RAN1 to support SL Positioning Measurement Report Content as described in Table 1.

Proposal 16: SL Location Information transfer message may be used to request and transfer SL positioning measurements and location information and requested/reported per SL positioning method. Send LS to RAN2.

Proposal 17: RAN1 to consider different SL PRS processing capabilities depending on the UE-type.

Proposal 18: RAN1 to further study the prioritization of SL positioning measurements depending on the known position and accuracy of the transmitting SL UE.

Proposal 19: Support different SL Positioning reporting types including one-shot, triggered (event-based) and periodic reports, where such reports may be based on higher-layer signalling.

Proposal 20: RAN1 to discuss the relationship between Uu PRS and SL PRS configuration and associated measurements for Uu and SL hybrid positioning. FFS whether such measurements are performed in a joint manner or separately measured and how these measurements are utilised.

Proposal 21: RAN1 to further discuss the support of SL PRUs in SL positioning/ranging procedures.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, RAN#98-e, Dec. 2022.

- R1-2302065, “Session notes for 9.5 Expanded and Improved NR Positioning”, Ad-Hoc Chair (Huawei), RAN#112, Feb. 2023.

- TR 38.859, “Study on expanded and improved NR positioning (Release 18)”, V1.0.0., Dec. 2022.