3GPP TSG RAN WG1#112-bis-e	           					     R1-2303265

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.5.1.3

Source: 	Lenovo

Title: 	Discussion on SL Positioning Resource Allocation

Document for:	Discussion

# Introduction

The Rel-18 NR positioning evolution SID was agreed upon during the RAN#94-e [1] meeting, where one of the objectives included the support for resource allocation for SL Positioning for SL positioning as highlighted below:

The specific objectives of this work item are: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

During the RAN1#112 [2] meeting, the following agreements were reached in the context of SL positioning resource allocation:

- Agreement - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool. - Agreement For a dedicated resource pool for positioning: - The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214). - FFS: additional slots that can be used for SL PRS is not precluded - Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration - Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH). Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further). Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

This contribution provides a detailed discussion into the resource allocation mechanism in order to support SL Positioning.

# SL Positioning Resource Configuration and Allocation

## Resource Allocation

It should be possible to extend the current Mode 1 and Mode 2 resource allocation schemes in SL to configure SL PRS in all coverage scenarios, i.e., in-coverage, partial coverage and out-of-coverage. Mode 1 or Scheme 1 offers a centralized approach to configure SL PRS for in-coverage and partial coverage scenarios, while Mode 2 or Scheme 2 enables SL PRS configuration especially in out-of-coverage scenarios.

### Scheme 1

During the RAN1#112 [2], it was further agreed that the LMF provision of SL resources in not supported, which leaves the door open to the existing resource allocation scheme used in existing SL, i.e.,resource coordination via the gNB.

In this scheme, the gNB may configure the SL PRS resources in a centralised manner depending on the configured positioning technique. For techniques requiring relative position estimation amongst multiple pair of UEs (e.g., between an anchor node UE and target-UE), gNB coordination should be sufficient. The gNB may also coordinate SL PRS for multiple anchor node UEs when performing triangulation using timing-based methods such as SL-TDoA or angular-based position estimation, for the purposes of absolute location estimation. In contrast, using the LMF to schedule SL-PRS resources may be inefficient, however the LMF may receive information from the gNB as to which resources were allocated for SL-PRS transmission. In addition, depending on the type of location request the gNB may configure one of the following:

- Dynamic grants for one shot SL positioning estimates (based on one shot SL PRS transmissions) e.g., in the case of immediately triggered requests

- Configured grant schemes in the case that periodic location requests are required, e.g., Type 1 and 2 CG grants (based on periodic SL PRS transmissions), e.g., in the case of periodic location requests.

Proposal 1: In the case of Scheme 1, RAN1 to support the transmitting UE receiving a SL-PRS resource allocation information via the gNB through a dynamic grant, or through a configured grant type 1/type 2 from the gNB.

For certain positioning schemes such as SL-RTT where a reply SL-PRS signal is necessary for the computation of the Rx-Tx time difference measurement, it would be beneficial if the transmitting UE (Tx UE) may request a resource grant from the gNB for uni-directional SL-PRS transmissions and/or bi-directional SL PRS resources. This may be different from the existing scheme where a SL UE may only request for uni-directional resources, i.e., resources for transmissions from Tx UE to Rx UE. It should be noted both Tx and Rx UEs should be in-coverage to receive the resource grant. Alternatively, the Tx UE may share the allocated gNB grant to transmit SL-PRS from Rx UE to Tx UE with the Rx UE. Further study is required on types resource allocation grants that can be supported via bi-directional resource allocation.

Proposal 2: In the case of Scheme 1, RAN1 to support uni-directional and bi-directional request and allocation of resources by the gNB. In case of uni-directional resource allocation, SL-PRS resource from Tx UE to Rx UE is allocated. In case of bi-directional resource allocation, SL-PRS resource from 1) Tx UE to Rx UE 2) Rx UE to Tx UE is allocated. FFS types of resource allocation grants supported by bi-directional resource allocation by the gNB.

### Scheme 2

Mode 2 is beneficial for scenarios not requiring network infrastructure, where the Initiator UE may autonomously select and reserve resource for the purposes of SL positioning. This relies on a pre-configuration of resource pools within a UE. Given the distributed nature of this scheme, Mode 2 can sure stability of SL PRS for a given period by selecting and reserving resources for SL PRS transmission. However, this may depend on the type of SL PRS configuration (e.g., bandwidth, number of symbols, etc.).

The selection triggers may be different in the case of SL PRS when compared to the existing schemes for SL data (TB) transmission, where the selection is dependent on the generation of a new TB or a pre-emption mechanism depending on the SL traffic priority. Similarly, in the case SL Positioning, the QoS may also have an impact on the resource selection mechanism in Mode 2. The sensing window should also be able to accommodate a SL PRS transmission based on the accuracy requirements of the SL positioning service request.

Proposal 3:   RAN1 to use the existing Mode 2 sensing, (re-) selection and reservation as baseline for the transmission of SL-PRS in Scheme 2.

Further discussion may be required on the allocation of resources for the transmission and reception of higher-layer SL positioning messages, e.g., capability exchange, assistance data, location information. In this SL PRS and/or shared resource pools can be considered. In this case Scheme 1 and Scheme 2 can be supported to transmit SL positioning messages.

Proposal 4:   RAN1 to consider Scheme 1 and Scheme 2 for transmission and reception of SL positioning messages. FFS whether SL-PRS dedicated resource and/or shared resource pool may be utilised.

### SL Positioning IUC Resource Allocation

Inter-UE Coordination (IUC) scheme 1 and scheme 2 were introduced in Rel-17 NR sidelink communication. The IUC schemes can be used to indicate resource selection/reservation collision caused by the hidden node problem and half duplex issue in NR SL mode 2 resource allocation. For SL positioning, UE autonomous resource selection and relevant resource reservation mechanism (e.g., similar to legacy Mode 2 solution) are introduced. Therefore, the Rel-17 IUC framework can be considered to reuse and extended for SL-PRS transmission, especially for out-of-coverage scenarios.

In IUC scheme 1, a UE can provide a set of preferred or non-preferred resource to another UE by using SCI format 2-C and/or MAC CE. Based on current specification on SL, both SCI format 2-C and MAC CE are carried on PSSCH. For shared resource pool, PSSCH region is available and IUC scheme 1 is feasible. But in dedicated SL-PRS resource pool, whether PSSCH region is supported or not is still under discussion. From SL-PRS transmission point of view, in addition to SL-PRS, only PSCCH which carries SCI associated with SL-PRS transmission(s) is recommended. So techniques to support IUC scheme 1 for SL-PRS transmission in a dedicated SL-PRS resource pool should be further studied.

In IUC scheme 2, the reserved resource by a UE is indicated by another UE if a potential resource conflict is detected and the conflict information is carried by PSFCH. It has no problem on shared resource pool, in which the PSFCH region can be (pre-)configured based on legacy mechanism. But in the dedicated resource pool for SL-PRS transmission, PSFCH region may be excluded based on current agreements. So mechanisms to support IUC scheme 2 in a dedicated resource pool should also be further studied.

Proposal 5:  RAN1 to further consider IUC scheme 1 and 2 for dedicated SL-PRS resource pool and study details.

### Resource Allocation for a SL Positioning Group

According to TR 38.859 [3], it has been concluded by RAN2, that if SL Group Positioning is supported, it would be feasible to realize such a scenario based on the following text extract from the TR:

SL Positioning Group If it is determined to support group positioning, it is feasible to perform at least ranging with the estimate calculation at multiple UEs.

Furthermore, during the RAN2#121 meeting [4], the following agreement was reached:

RAN2#121 Agreement: RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.

It would be therefore beneficial from RAN1 perspective to understand the design considerations to support SL Positioning. In addition, the concept of a SL Positioning Server UE has been discussed on several occasions in RAN2 in order to play a coordinating role amongst UEs involved and performing SL Positioning. The above agreement shows that the SL Positioning Server UE may play an important resource coordinating role for allocation of SL-PRS resources.

Observation 1: SL Positioning requires involvement of multiple UEs and therefore coordination may be required in terms of SL Positioning Group, especially with regard to resource allocation.

A SL positioning group may comprise of a number of UEs including Anchor UEs, Target-UE, etc. and therefore tight coordination is beneficial amongst UEs depending on the type of positioning technique(s) being employed, e.g., SL-TDoA for 3 or more anchor UEs, SL-RTT, etc. Therefore, coordinated resource allocation schemes amongst member UEs of a SL positioning group should be further investigated over and above the existing SL positioning resource schemes. This may include recommending a set of resources (inter-UE coordination), scheduling/coordination by one member UE on behalf of other member group UEs, forwarding of resource grants from one member UE to other member UEs with or without gNB assistance, etc.

Proposal 6:  RAN1 to support enhanced resource allocation/coordination schemes for a SL Positioning Group that are based on the existing Scheme 1 and 2 procedures, e.g., resource configuration sharing by a member UE with other member UE(s), sensing information coordination, inter-UE coordination by member UE with other member UE(s), server UE coordinating SL-PRS resources for other UEs within a SL positioning group.

## SL PRS configuration/activation/deactivation/triggering

The following SL positioning resource configuration details were studied and concluded in TR 38.859 during the study phase [3]:

TR 38.859 SL PRS Configuration Outcomes [3] On the configuration/ activation/ deactivation/ triggering/ reservation of SL PRS, the study focused on the following options, with considerations on flexibility, overhead, latency, and reliability: - Option 1: High-layer-only signaling involvement in the SL PRS configuration. - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL PRS. - Based on the study, this option may correspond to: - A SL PRS configuration that is a single-shot or multiple shots. - A high-layer configuration that may be received from an LMF, a gNB, or a UE. - Option 2: High-layer and lower-layer signaling involvement in the SL PRS configuration. - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI. - For example, high layer signaling can be used for SL PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL PRS.

From the TX UE perspective, the SL PRS transmission(s), which may be supported include periodic, semi-persistent (with activation/deactivation commands) and aperiodic transmissions. During RAN1#110-bis-e [4], the aforementioned SL PRS transmission options were agreed for study. The semi-persistent SL-PRS transmission aligns well with the existing SL transmission procedures and can be easily extended. Additionally, a key difference between periodic SL-PRS and semi-persistent SL-PRS is which layer (lower layer or higher-layer) may handle the activation/deactivation of the SL-PRS periodic transmission. Therefore, it may be redundant to support both periodic and semi-persistent SL-PRS transmissions given that they achieve the same purpose of periodic SL PRS transmissions. Semi-persistent SL-PRS offers the ability to utilize existing SL configured grants for positioning. Therefore, it is recommended to support at least semi-persistent SL PRS transmissions.

In addition, aperiodic SL-PRS may exploit the dynamic grant scheduling already offered in SL communication, where the UE may request for resources from the gNB for SL-PRS transmission. The aperiodic SL-PRS transmission scheme may re-use the existing SL framework for dynamic grant configured transmissions to transmit SL-PRS in a one-shot manner. Further study may be required on details of such a UE request for resources and how it may be triggered.

Overall, the different options provide configurable flexibility for SL-PRS transmission, which may apply to different use cases, e.g., one-shot or multi-shot positioning.  In addition, there needs to be unambiguous identification of transmitting/initiator and receiving/responder UEs based on the SL-PRS cast type, e.g., unicast, groupcast or broadcast.

Proposal 7: RAN1 to at least support the following time domain behaviour for SL-PRS transmissions:

- Support of semi-persistent SL-PRS by re-using the existing SL framework as best as possible, e.g., using the configured grant scheduling mechanism with lower and/or higher layer activation/deactivation commands.

- Support of aperiodic SL-PRS by re-using the existing SL framework as best as possible, e.g., the existing dynamic grant scheduling framework based on a UE request of SL-PRS resources to the gNB.

- FFS the details of the UE request of SL-PRS resources including unambiguous identification of initiator UEs and responder UEs, e.g., defining new or re-using existing UE IDs.

Furthermore, the above SL-PRS time domain transmissions may be applicable to both Scheme 1 (similar to Mode 1 resource allocation procedures) and Scheme 2 (similar to Mode 2 resource allocation procedures).

Proposal 8: At least semi-persistent SL-PRS and aperiodic SL-PRS is applicable to both Scheme 1 and Scheme 2 resource allocation procedures.

The type of trigger may provide further information as to whether a particular SL-PRS transmission is a /semi-persistent or aperiodic in nature, which will have to align with the request/selection/reservation of SL PRS resources to be for Mode 1 and Mode 2 SL transmissions.

Proposal 9: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, gNB, LMF and the type of trigger, e.g., based on a higher-layer SL positioning/ranging service request. Send LS to RAN2 in relation to agreed SL-PRS triggering mechanism.

# Resource Pool Structure

A number of issues were raised during the RAN1#112 meeting [2] regarding both the dedicated and shared  SL positioning resource pool structure including slot structure, SCI content for scheduling SL-PRS, SL-PRS multiplexing with SL-PRS, bandwidth of the SL-PRS transmission, resource allocation granularity.

## SL PRS Resource Granularity

A key issue raised in the previous meeting was the actual definition of a SL PRS resource, which also defines the SL PRS resource granularity. In AI 9.5.1.1, it has already been agreed that the SL-PRS is defined directly with respect to and contained within a single SL BWP and carrier.  Therefore, the following resource hierarchical options can be further considered as a baseline to define a SL PRS resource: SL CarrierSL BWPSL Resource PoolSL PRS Resource Set SL PRS Resource.

Proposal 10: RAN1 to define the SL PRS resource hierarchy in terms of SL Carrier SL BWPSL Resource PoolSL PRS Resource Set SL PRS Resources.

Further discussion may also be needed on the definition of SL PRS resources with respect to sub-channels as this represents the default frequency allocation for SL communications. In the time domain, depending on whether TDM-based multiplexing is utilised, a slot-based or mini-slot/sub-slot-based resource allocation may be considered beneficial.

Proposal 11: Further consider the SL PRS resource allocation granularity in terms of one or more sub-channels and/or slots/mini slots.

## SCI Content for Scheduling SL-PRS

The selection and reservation/indication of SL-PRS resources is best indicated using the sidelink control information (SCI), which among other aspects, assists other UEs determine the resources used for transmitting SL-PRS. The reservation/indication of SL-PRS resource using the SCI is also more dynamic and can be transmitted in a low latency manner, when compared to higher-layer signalling, especially considering Scheme 1 and 2 procedures.  Another further aspect for consideration is to understand whether there is a need to support a single SCI or two-stage SCI. The first stage SCI traditionally carriers the key control information associated with the PSSCH and 2nd stage SCI. In the case of the SL Positioning, the 1st or single stage SCI can may at least convey the time-frequency SL-PRS information including reservation/indication information. Other further information may include comb pattern size, number of symbols, whether repetitions of SL-PRS are enabled or not.

Proposal 12: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc.

# Conclusion

The following observations regarding potential SL positioning solutions have been noted as part of the discussion:

Observation 1: SL Positioning requires involvement of multiple UEs and therefore coordination may be required in terms of SL Positioning Group, especially with regard to resource allocation.

Hence, the discussed proposals are summarized as follows:

Proposal 1: In the case of Scheme 1, RAN1 to support the transmitting UE receiving a SL-PRS resource allocation information via the gNB through a dynamic grant, or through a configured grant type 1/type 2 from the gNB.

Proposal 2: In the case of Scheme 1, RAN1 to support uni-directional and bi-directional request and allocation of resources by the gNB. In case of uni-directional resource allocation, SL-PRS resource from Tx UE to Rx UE is allocated. In case of bi-directional resource allocation, SL-PRS resource from 1) Tx UE to Rx UE 2) Rx UE to Tx UE is allocated. FFS types of resource allocation grants supported by bi-directional resource allocation by the gNB.

Proposal 3:   RAN1 to use the existing Mode 2 sensing, (re-) selection and reservation as baseline for the transmission of SL-PRS in Scheme 2.

Proposal 4:   RAN1 to consider Scheme 1 and Scheme 2 for transmission and reception of SL positioning messages. FFS whether SL-PRS dedicated resource and/or shared resource pool may be utilised.

Proposal 5:  RAN1 to further consider IUC scheme 1 and 2 for dedicated SL-PRS resource pool and study details.

Proposal 6:  RAN1 to support enhanced resource allocation/coordination schemes for a SL Positioning Group that are based on the existing Scheme 1 and 2 procedures, e.g., resource configuration sharing by a member UE with other member UE(s), sensing information coordination, inter-UE coordination by member UE with other member UE(s), server UE coordinating SL-PRS resources for other UEs within a SL positioning group.

Proposal 7: RAN1 to at least support the following time domain behaviour for SL-PRS transmissions:

- Support of semi-persistent SL-PRS by re-using the existing SL framework as best as possible, e.g., using the configured grant scheduling mechanism with lower and/or higher layer activation/deactivation commands.

- Support of aperiodic SL-PRS by re-using the existing SL framework as best as possible, e.g., the existing dynamic grant scheduling framework based on a UE request of SL-PRS resources to the gNB.

- FFS the details of the UE request of SL-PRS resources including unambiguous identification of initiator UEs and responder UEs, e.g., defining new or re-using existing UE IDs.

Proposal 8: At least semi-persistent SL-PRS and aperiodic SL-PRS is applicable to both Scheme 1 and Scheme 2 resource allocation procedures.

Proposal 9: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, gNB, LMF and the type of trigger, e.g., based on a higher-layer SL positioning/ranging service request. Send LS to RAN2 in relation to agreed SL-PRS triggering mechanism.

Proposal 9: RAN1 to consider the resource allocation triggering mechanism of SL PRS transmission(s) in terms of which entity, e.g., anchor-UE, target-UE, gNB, LMF and the type of trigger, e.g., based on a higher-layer SL positioning/ranging service request. Send LS to RAN2 in relation to agreed SL-PRS triggering mechanism.

Proposal 10: RAN1 to define the SL PRS resource hierarchy in terms of SL Carrier SL BWPSL Resource PoolSL PRS Resource Set SL PRS Resources.

Proposal 11: Further consider the SL PRS resource allocation granularity in terms of one or more sub-channels and/or slots/mini slots.

Proposal 12: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, RAN#98-e, Dec. 2022.

- R1-2302065, “Session notes for 9.5 Expanded and Improved NR Positioning”, Ad-Hoc Chair (Huawei), RAN#112, Feb. 2023.

- TR 38.859, “Study on expanded and improved NR positioning (Release 18)”, V1.0.0., Dec. 2022.

- R2-2301904, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), RAN2#121, Feb. 2023.