3GPP TSG RAN WG1#112-bis-e	           					     R1-2303268

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.5.5

Source: 	Lenovo

Title: 	Support for RedCap Devices

Document for:	Discussion

# Introduction

The Rel-18 NR expanded and improved positioning WID was agreed upon during the RAN#98-e [1] meeting, where one of the objectives included the support of Redcap devices as highlighted below:

The specific objectives of this work item are: - Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

During the RAN1#112 [2] meeting, the following agreements were reached in relation to RedCap positioning:

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported. Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Conclusion The scope for RedCap positioning includes FR1 and FR2. Agreement For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance. - Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same. R1-2302126 Draft LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs Ericsson [to be RAN1] Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources Agreement - The draft LS in R1-2302126 is endorsed with the addition of the two agreements above. Final LS in R1-2302127.

This contribution provides a further discussion into the support of frequency hopping for DL PRS reception and UL SRS transmission.

# RedCap Device positioning

The Redcap positioning target requirement has been evaluated as follows [2]:

For commercial use cases for both indoor and outdoor scenarios

- Horizontal positioning accuracy: (< 3 m) for 90% of Ues

- Vertical positioning accuracy: (< 3 m) for 90% of UEs.

For IIoT use cases:

- Horizontal positioning accuracy: (< 1 m) for 90% of Ues

- Vertical positioning accuracy: (< 3 m) for 90% of UEs.

Furthermore, the RAT-dependent positioning techniques supported in NR are summarized as follows:

- Timing-based positioning methods

  - DL-TDoA

  - Multi-RTT

  - UL-TDoA

- Angular-based positioning methods

  - DL-AoD

  - UL-AoA

The Redcap positioning performance evaluation considered Rx hopping of DL PRS, Tx hooping of UL SRS to evaluate DL-TDoA and UL-TDoA respectively. Moreover, the evaluation results for Rx hopping included phase offset between hops, with phase compensation, without phase compensation and minimum time gap between hops. Since the evaluation results are mostly with timing-based methods. In Rel-18 Redcap positioning, timing-based positioning technique can be prioritized compared to angular based methods

Proposal 1: Prioritize timing-based positioning technique such as TDoA, RTT for Redcap positioning.

Implementing phase offset compensation is up to each UE receiver, hence a UE can decide whether to combine hops if the phase offset is within a certain threshold and thereafter choose to apply phase compensation or not, depending on whether it is above a threshold, may be up to UE implementation.

Proposal 2: Phase offset compensation should be left to UE implementation

The subset of positioning reference signal length, LPRS {2,4, 6, 12}, comb size, Kcomb {2, 4, 6, 12} and such combination containing LPRS,KcombPRS is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}; defined in NR Rel-16 should be taken as a baseline for RedCap positioning evaluation while considering reduced Rx RF chain of 1 or 2 and associated bandwidths for the FR1 and FR2 link budget evaluation. The longer PRS length such 12 symbols maybe beneficial for the evaluation criteria in terms of coverage and accuracy improvement compared to the shorter PRS length which was primarily defined for shorter latency. Furthermore, the (N,T) processing capability for DL-PRS may vary for RedCap devices when compared to standard UE due to the lower hardware complexity.

Proposal 3: Support Redcap PRS processing capability with reduced bandwidths e.g., 20MHz for FR1 and 100MHz for FR2 including a reduced Rx antenna/RF chain of a single antenna.

Proposal 4: Support only longer PRS symbol lengths for RedCap devices {6,12} and FFS support PRS symbol lengths of {2, 4}.

Since the study concluded that the Rx frequency hopping for DL-PRS and Tx frequency hopping for UL-SRS is beneficial for DL-TDOA and UL-TDOA positioning techniques to mee the Redcap target requirement, then the specification work should restrict the frequency hopping only to timing based positioning techniques such as TDoA.  The coherent combining across different frequency hops improves RedCap positioning accuracy, given the inherent bandwidth limitation. The phase needs to be estimated and corrected before coherent combining which is upto the UE implementation.

Minimum timing offset between hops affect the overall positioning accuracy and hence there should be conifigured within 4ms which includes potential time to synchronize the RF chain. Also the complexity and capability of RedCap UEs to store the number of time domain DL-PRS sample across different hops to perform wideband measurement should be specified. Additionally, the impact of RF tuning arising during the measurement gap is a well-known issue and hence the UE needs to store DL-PRS samples from one sub-band prior to RF switching to another sub-band within one measurement gap and the stored DL-PRS samples would need to be coherently combined at the end of the measurement. The time taken to perform RF tuning will have an impact on the number of Rx positioning frequency hops which can be performed by the UE.

Proposal 5: Specify the capability on the amount of Rx hops to combine, storing the number of time domain DL-PRS samples across different hops for coherent Rx combining to achieve wideband DL-PRS measurement for RedCap devices.

Proposal 6: Specify the timing offset resulting from the reception of different positioning hops.

# Conclusions

The proposals are summarized as follows:

Proposal 1: Prioritize timing-based positioning technique such as TDoA, RTT for Redcap positioning.

Proposal 2: Phase offset compensation should be left to UE implementation.

Proposal 3: Support Redcap PRS processing capability with reduced bandwidths e.g., 20MHz for FR1 and 100MHz for FR2 including a reduced Rx antenna/RF chain of a single antenna.

Proposal 4: Support only longer PRS symbol lengths for RedCap devices {6,12} and FFS support PRS symbol lengths of {2, 4}.

Proposal 5: Specify the capability on the amount of Rx hops to combine, storing the number of time domain DL-PRS samples across different hops for coherent Rx combining to achieve wideband DL-PRS measurement for RedCap devices.

Proposal 6: Specify the timing offset resulting from the reception of different positioning hops.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, RAN#98-e, Dec. 2022.

- R1-2302065, “Session notes for 9.5 Expanded and Improved NR Positioning”, Ad-Hoc Chair (Huawei), RAN#112, Feb. 2023.

- TR 38.859, “Study on expanded and improved NR positioning (Release 18)”, V1.0.0., Dec. 2022.