3GPP TSG RAN WG1 #112bis-e   														     R1- 2303270

e-Meeting, 17th – 26th April, 2023

Agenda item:		5

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RAN4 LS on the UE SRS IL imbalance issue

Document for:		Discussion and Decision

# 1	Introduction

RAN4 has sent an LS to RAN1 [1], where the following action is requested.

ACTION: RAN4 respectfully ask RAN1 to consider above issue with, but not limited to the three resolutions listed in the Annex for their future study. Possible resolution in Annex - Enable UE report on the actual IL imbalance for each diversity branch used for SRS so network can use this information for AS-SRS based channel estimation accuracy improvement where the granularity of such report can be per SRS resource and either static or dynamic. - Utilize PCMAX, f, c via PHR type 3, where power imbalances across antenna ports are derived by comparing the maximum configured power of main branch to those of diversity branches, where such report is per transmission occasion per SRS (only a report of a single SRS resource is transmitted per transmission occasion). - Define UE measurements of downlink channels which are reported in order to assist the network in determining the difference between the UE insertion losses for two given antenna ports, where the network also does its own measurements of SRS channels. - RAN4 does not preclude other options. The above alternatives are considered from Release 18 onwards and for 8Rx capable UE’s, and possible applicability UE’s supporting 2RX or 4RX is FFS

# 2	Discussion

SRS antenna switching is an important feature to acquire DL CSI and beamforming information for TDD especially for most of cells with larger number of antenna ports. But this requires addition switching logic in UE Tx side, and it introduces additional insertion loss (IL) to the diversity branches than the main branch.

To solve the problem, RAN4 has defined a relaxation for 4Rx as a non-ideal factor in PCMAX_L, f, c, ∆TRxSRS (3-6dB) is the factor to relaxation. For 8 Rx case, higher IL values were proposed, so it is not efficient to define only power reduction. Also, channel estimation performance is highly important for higher rank transmission. To solve the problem, three options were discussed in RAN4.

  - Option1: Indication of actual IL

This approach requires that a UE shall always maintain power imbalance at respective antenna ports depending on the indicated actual ILs, i.e., ∆TRxSRS,p (p=0, 1, …). Since gNB utilizes the information as channel estimation correction factor, if the power imbalance at the antenna ports cannot be maintained, it just may lead to additional channel estimation error. The concept is illustrated in Figure 1. Note that hereafter this approach is called fixed power imbalance approach.

Figure 1: Precondition to utilize atual IL indication for Fixed power imbalance

If UE cannot maintain the condition, it means that the approach works only when the UE output power at reference antenna port(s) which delivers the highest power across ports is at maximum.

Observation 1: Fixed power imbalance approach (maintain power imbalance at respective antenna ports depending on the indicated actual ∆TRxSRS, p) works only when the UE output power at reference antenna port(s) which delivers the highest power across the ports is at maximum.

We discuss whether or not the above UE behaviour in terms of power per antenna port during SRS antenna switching is expected by the current specifications and UE implementations.

Specification aspect

The below formula is from TS 38.213.

[dB]

According to TS38.213, minimum unit to handle PHR for SRS is SRS resource set. It means that SRS resources in an SRS resource set uses exactly the same parameters in the above formula and the UE shall target the same power across all the antenna ports in the same SRS resource set.

Observation 2: TS38.213 expects the same output power across antenna ports (hereafter, this is called no power imbalance approach) in an SRS resource set. Hence, if UE is compliant to no power imbalance approach, indicated ∆TRxSRS shall not be used as error correction factor at gNB.

Implementation aspect

Though we noted that power per port in an SRS resource set should be the same from TS38.213 perspective, we discuss whether UE is able to exactly follow such an instruction or not. The same power across ports means that UE shall be able to compensate for the power to be lost due to ∆TRxSRS immediately after SRS antenna switching. In order to make the discussion more specific, let’s assume that supportedSRS-TxPortSwitch is set to t1r2, ∆TRxSRS is 3 dB for the 2nd antenna port, PA output power at 1st antenna port is 26 dBm (i.e., post PA IL is 3 dB) to achieve PC3 (23 dBm). If SRS antenna switch is conducted, to maintain the same power across the ports, the UE shall be able to compensate 3 dB by boosting PA output by 3 dB during 2nd SRS resource transmission, i.e., PA output power shall be 29 dBm at the instant. This might not be practical and if the UE was able to do that, the UE would declare PC2. If the required power, however, is below or equal to 20 dBm, the UE may be able to compensate for the lost power at 2nd SRS resource transmission since the PA output can be equal to or below 26 dBm, which brings 20 dBm at antenna port (26 dBm - Total loss of 6 (3 dB + 3 dB) dB). Hence, with typical implementation, it might not be practical for UEs to maintain no power imbalance across antenna ports specifically, when the required output at port exceeds Power Class – max {∆TRxSRS,p | p=0, 1, …m}.

Observation 3: Contrary to Observation 2, though TS38.213 expects no power imbalance across ports within the same SRS resource set, at least it may not be practical to perform it when the output power at the antenna port to deliver the highest maximum output power exceeds power class - max {∆TRxSRS,p | p=0, 1, …m}

The expected UE behaviour in terms of power is illustrated in Figure 2. It should be noted that if UE follows no power imbalance approach, gNB shall not use channel estimation error correction with ∆TRxSRS,p until the power at the highest power antenna port reaches maximum power, i.e., Power Class. If the power at the highest power port reaches the maximum, then, ∆TRxSRS,p can be utilized by gNB as error compensation factor.

Figure 2: No power imbalance (UE compensates for power to be lost due to ∆TRxSRS)

Next question would be if as far as the output power at reference antenna port to deliver the highest power across ports is below or equal to 20 dBm, is the no power imbalance approach feasible? Perhaps, it may not be always the case. Since the UE shall be able to achieve the same power immediately after one or two symbols (minimum GP (guard period)). Moreover, in some cases, ∆TRxSRS,p can be around 8 dB (if proposed value by some companies is true). This may increase probability that after the 1st SRS resource transmission, PA gain settings for 2nd SRS resource transmission may drastically change, e.g., the number of PAs to be used in series changes from one to two (even advanced PA technology would have more or less a similar point to change gain settings). Relation between PA output power, achievable power at antenna port and ∆TRxSRS for fixed power imbalance as well as no power imbalance are summarized in Figure 3. It’s noted that all the values are hypothetical.

Figure 3: No power imbalance and Fixed power imbalance

In fact, the current TS38.101-1 allows some exception due to RF Power amplifier mode changes for relative power tolerance requirements. Also, the tolerance is already quite large even with less than or equal to 20 ms, which is even larger than GP (one or two symbols) allowed for SRS antenna switching.

Observation 4: Even the power at antenna port to deliver the highest power is lower or equal to Power Class - max {∆TRxSRS,p | p=0, 1, …m}, the UE may not be able to maintain the same power across ports due to short GP.

Observation 5: From Observation 1 – 4, TS38.213 seems expect no power imbalance across ports and that is not fit to a solution by indication of actual ∆TRxSRS, while not all the UE may not be able to achieve no power imbalance approach. With the current specification, applicability of indication of ∆TRxSRS is extremely limited.

Proposal 1: No introduction of indication of ∆TRxSRS unless clear UE behaviour in terms of power per port as well as associated with requirements are defined.

  - Option 2: Type 3 PHR

In the current specification, the transmission power per port is the same within the same SRS resource set. Also, power control is handled per SRS resource set level. The Type 3 PHR value is applied all SRS resources in a SRS resource set in an occasion. Hence, with the current specifications, the problem cannot be solved.

To solve the problem, we should introduce new PHR operation reporting PHR per SRS port or SRS resource level. This is discussed for 8Rx case, this increases the UE’s reporting overhead. Since this insertion loss is assumed as constant value, multiple PHR reporting is equivalent to applying different IL values into each reporting, so such high overhead doesn’t make sense.

Observation 6: Type 3 PHR approach cannot mitigate power imbalance due to ∆TRxSRS due to the restriction that power control is per SRS resource set.

  - Option 3: Report channel (gNB to UE) measurement by UE

SRS antenna switching is introduced to acquire accurate channel state without abstraction. PMI cannot fully describe the exact channel but delivering limited information only. Though CQI can be measured by the UE, and reported additionally, but main problem of this issue is rather acquiring accurate channel information for DL beamforming than CQI acquisition. Also, CQI is dependent on DL beamforming assumption.

Thus, this approach doesn’t solve the problem but increase the UE complexity and the reporting overhead by using two operations both in UE and gNB.

Also, due to the measurement latency between SRS transmission and CSI-RS reception, and reporting latency between SRS transmission time and CSI reporting time, the performance cannot be guaranteed. This can be used for fixed/stable channel environment only.

Observation 7: Report channel (gNB to UE) measurement by UE cannot solve the IL imbalance problem, but marginal gain for CQI measurement. Though the channel consistency problem for measurement and reporting latency, the application scenario should be limited to fixed stable channel only.

# 3	Summary

Based on the discussion above, we think option 1 is most promising option over the other options for improving the channel accuracy problem in SRS antenna switching. However, there is still ambiguity on the UE operation. To support the option, the UE operation should be clarified in RAN1. Also, we should study the other aspect regarding to the SRS configuration etc.

Proposal 2: RAN1 to study and specify the UE behavior in terms of Tx power setting to support indication of SRS IL. Do not support only indication of IL without RAN1 specification update.

# 4	Conclusions

In this contribution, we have the following observations and proposals.

Observation 1: Fixed power imbalance approach (maintain power imbalance at respective antenna ports depending on the indicated actual ∆TRxSRS, p) works only when the UE output power at reference antenna port(s) which delivers the highest power across the ports is at maximum.

Observation 2: TS38.213 expects the same output power across antenna ports (hereafter, this is called no power imbalance approach) in an SRS resource set. Hence, if UE is compliant to no power imbalance approach, indicated ∆TRxSRS shall not be used as error correction factor at gNB.

Observation 3: Contrary to Observation 2, though TS38.213 expects no power imbalance across ports within the same SRS resource set, at least it may not be practical to perform it when the output power at the antenna port to deliver the highest maximum output power exceeds power class - max {∆TRxSRS,p | p=0, 1, …m}

Observation 4: Even the power at antenna port to deliver the highest power is lower or equal to Power Class - max {∆TRxSRS,p | p=0, 1, …m}, the UE may not be able to maintain the same power across ports due to short GP.

Observation 5: From Observation 1 – 4, TS38.213 seems expect no power imbalance across ports and that is not fit to a solution by indication of actual ∆TRxSRS, while not all the UE may not be able to achieve no power imbalance approach. With the current specification, applicability of indication of ∆TRxSRS is extremely limited.

Observation 6: Type 3 PHR approach cannot mitigate power imbalance due to ∆TRxSRS due to the restriction that power control is per SRS resource set.

Observation 7: Report channel (gNB to UE) measurement by UE cannot solve the IL imbalance problem, but marginal gain for CQI measurement. Though the channel consistency problem for measurement and reporting latency, the application scenario should be limited to fixed stable channel only.

Proposal 1: No introduction of indication of ∆TRxSRS unless clear UE behaviour in terms of power per port as well as associated with requirements are defined.

Proposal 2: RAN1 to study and specify the UE behavior in terms of Tx power setting to support indication of SRS IL. Do not support only indication of IL without RAN1 specification update.

# 5 References

- 3GPP R1-2302267 (R4-2303519), LS on the UE SRS IL imbalance issue