3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303275

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | 0 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Positioning Measurement Gap Activation Request in 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to RAN2’s agreement in RAN2#121, gNB indicates whether UE is configured to send UL MAC CE for positioning measurement gap activation/deactivation request via posMG-Request. In other words, the previous UE behaviour described in TS 38.214 is that the UE may be preconfigured with one or more measurement gaps and UE can request activation or deactivation of one of the preconfigured measurement gaps. With the newly added feature and for efficient resource utilization, the UE behaviour regarding measurement gap activation/deactivation in TS 38.214 should be updated as: UE may be preconfigured with one or more measurement gaps and UE can request activation or deactivation of one of the preconfigured measurement gaps only if the UE is configured by the network to request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in TS 38.214 that UE is only expected to request one of the preconfigured measurement gap if the UE is configured by the network to request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of UE behavior regarding preconfigured measurement gap activation/deactivation in TS 38.214 does not align with RAN 2’s new agreement. UE may still request for activation/deactivation of measurement gap regardless of network’s indication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.6.5	PRS reception procedure

< Unchanged parts are omitted >

The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When the UE is expected to measure the DL PRS resource, the UE may request a measurement gap via higher layer parameter NR-PRS-MeasurementInfoList [12, TS 38.331] or as specified in clause 6.1.3.40 of [10, TS 38.321]. The UE may be preconfigured with one or more measurement gaps each associated with an measPosPreConfigGapId. When the UE is configured via higher layer parameter posMG-Request [12, TS 38.331] to request activation or deactivation of a measurement gap as specified in clause 6.1.3.40 of [10, TS 38.321] it can request one of the preconfigured measurement gaps by referring to the measPosPreConfigGapId. The UE may have one of the preconfigured measurement gap(s) activated or deactivated as specified in clause 6.1.3.41 of [10, TS 38.321].

< Unchanged parts are omitted >