3GPP TSG RAN WG1 #112bis-e                                    R1-2303277

e-Meeting, April 17th – 26th, 2023

Source:         ZTE

Title:            Discussion on SL positioning measurements and reporting

Agenda item:    9.5.1.2

Document for:	 Discussion and Decision

Introduction

In RAN#98-e meeting, various features have been recommended for normative work [1] for support of sidelink (SL) ranging/positioning. The following shows the expected enhancements for SL positioning measurements and reporting in the WID:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

In this contribution, we provide our solutions for SL positioning measurements and reporting considering RTT-type solutions using SL, SL-AoA, SL-TDOA.

Discussion

For Rel-18 NR sidelink positioning, in order to obtain the location information of a target UE, positioning methods make use of reference signals (e.g. SL PRS) transmitted between UEs and the resulting measurements can be used to locate the target UE. Especially in some scenarios with anchor UEs, e.g. RSUs which can be utilized as TRPs for positioning purpose, then both time-based and angle-based positioning methods are applicable. For SL-AoD positioning method, similar to DL-AoD, the UE position is estimated based on SL PRS-RSRP and/or SL PRS-RSRPP measurements taken at the UE from multiple anchor UEs, along with knowledge of the beam index of the SL PRS and geographical coordinates of the anchor UEs. However, SL-AoD cannot be a high accuracy positioning method due to the lack of beam management functionality. The following agreements were made in previous RAN1 meetings where SL-AoD is not introduced in Rel-18 SL positioning but SL PRS RSRP and SL PRS RSRPP are defined in order to assist other positioning methods:

Agreement in RAN1#110b-e - From the potential candidate Positioning methods using at least SL measurements, at least the following should be introduced: - RTT-type solution(s) using SL - SL-AoA - SL-TDOA - FFS: SL-AoD Agreement in RAN1#111 SL-AoD is included as a potential candidate positioning method, and - SL-AoD should be deprioritized over the remaining methods that have been recommended to be introduced. Agreement in RAN1#112 SL PRS reference signal received power (SL PRS-RSRP) - is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. Agreement in RAN1#112 SL PRS reference signal received path power (SL PRS-RSRPP), - is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

In this section, we mainly focus on the discussion on RTT-type, SL-AoA and SL-TDOA method and the corresponding measurements.

2.1 SL-TDOA

### 2.1.1 SL-TDOA

The legacy DL-TDOA/UL-TDOA positioning methods need time synchronization information between a reference TRP and a list of neighboring TRPs in Rel-16/17 to meet the positioning accuracy requirements. For SL-TDOA positioning methods, the target UE position is estimated based on measurements taken from multiple anchor UEs (shown in Figure 2.1) and/or based on measurements at the target UE.

Figure 2.1: UL-TDOA like method for sidelink positioning

The following agreements and proposal were achieved/discussed in previous RAN1 meetings:

Agreement in RAN1#111 For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced. - A UE is not required to support both DL-TDOA-like operation and UL-TDOA-like operation Proposal in RAN1#112 Study whether and how to mitigate impact of synchronization errors between anchor UEs for TDoA measurement, considering the following alternatives: - Alt1: indication/reporting of e.g., synchronization source type, synchronization error or quality of synchronization information - Alt2: SL PRS transmission between anchor UEs - Alt3: (pre-)configuration and capability report of the clock source’s accuracy requirement/threshold. If a UE does not satisfy the requirement, it does not participate in the positioning. - Others are not precluded

Currently, the synchronization errors for sidelink transmission include both UE’s transmission timing error and propagation delay. Also, there is no perfect synchronization between UEs in current SL communication even if two UEs share the same synchronization reference source, and it’s hard to guarantee the SL positioning accuracy. In RAN1#112, 3 alternatives were agreed as solutions on how to mitigate synchronization errors. Analyses are present as follows:

Alt1: indication/reporting of e.g., synchronization source type, synchronization error or quality of synchronization information

For Uu positioning, the IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs, and NR-RTD-Info is contained in NR-PositionCalculationAssistance to enable UE-based downlink positioning.

Similar mechanism can be applied for SL positioning, for example, suppose a reference UE is selected, time synchronization information between a reference UE and a list of anchor UE (e.g. NR-SL-RTD-Info) can be provided to the target UE for UE-based positioning or provided to LMF for LMF-based positioning. The time synchronization information can include both time related information of the reference UE (including the quality of the timing of reference UE) and the relative synchronization time offset (including the quality) between reference UE and a list of anchor UEs.

As for the indication of synchronization source type (e.g. gNB, LMF), it can be a part of the procedures for discovering anchor UEs. For example, UEs shall report its synchronization source type and a deciding entity may determine some of the UEs as anchor UEs where they share the same synchronization reference source.

Alt2: SL PRS transmission between anchor UEs

In our view, SL PRS transmission between anchor UEs cannot make any significant improvement for SL synchronization especially when there is no LOS links between anchor UEs. Further, anchor UEs can transmission SL PRS to each other and there is no additional spec impact.

Alt3: (pre-)configuration and capability report of the clock source’s accuracy requirement/threshold. If a UE does not satisfy the requirement, it does not participate in the positioning.

Similar as reporting of synchronization source type, clock source’s accuracy may also be used in discovering anchor UEs. However, we support reporting synchronization information between reference UE and a list of anchor UEs including quality of the synchronization information. Clock source’s accuracy can be one of the factors that determine quality of the synchronization information.

Proposal 1: With regards to mitigate impact of synchronization errors between anchor UEs for SL-TDOA method, support reporting synchronization difference between reference UE and a list of anchor UEs.

- The quality of the synchronization difference can also be reported.

- Note: Clock source’s accuracy can be one of the factors that determine quality of the synchronization information.

### 2.1.2 Measurements

SL PRS based RTOA and RSTD measurement can be defined based on UL-TDOA and DL RSTD in Uu positioning [3]. The following agreement was reached in RAN1#112:

Agreement in RAN1#111 SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN

With regards to SL PRS based RTOA, there remains FFS on how to select between SFN or DFN. Whether a UE use SFN or DFN depends on what the synchronization source is. For example, UE chooses its synchronization source depending on synchronization priority, DFN is used if UE is synchronized to GNSS, otherwise SFN is used if UE is synchronized to a gNB. The synchronization procedure (or selection between SFN or DFN) happens before UE transmits SL PRS to another UE.

Moreover, according to the specification of SL communication [6], for slots that might belong to a sidelink resource pool, slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0. We do not need to specifically point out how to select between SFN or DFN when it comes to a slot index or frame number in SL.

The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where - 0≤tiSL<10240×2μ, 0≤i<Tmax, - the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

To sum up, we prefer to delete the FFS in the agreement and suggest the following proposal:

Proposal 2: SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where

- T0 is the nominal beginning time of SFN 0 or DFN0.

- tSL PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

For DL PRS RSTD, the corresponding definition is shown as follows:

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

Similar as DL PRS RSTD, SL PRS based RSTD measurement can be defined as the relative timing difference between anchor UEs and a reference UE. We have the following proposal:

Proposal 3: SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi,

- Where:

  - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j.

  - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j.

  - For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE.

2.2 SL RTT

### 2.2.1 SL RTT

As shown in Figure 2.2-1, SL-RTT positioning method makes use of measurements of sidelink signals received from one or multiple anchor UEs, measured by the target UE and the measurements at the one or multiple anchor UEs of sidelink signals transmitted from target UE. UE location is estimated based on measurements performed between two UEs.

Figure 2.2-1: SL PRS interaction between target UE and multiple anchor UEs

The following agreements were achieved in previous RAN1 meetings where both single-sided RTT (SS-RTT) and double-sided RTT (DS-RTT) methods were agreed for Rel-18 positioning specification.

Agreement in RAN1#110b-e - With regards to the RTT-type solutions using SL, down-select between the following 2 alternatives: - Alt. 1: it corresponds to a single-sided RTT method - Alt. 2: it may correspond to either a single-sided or double-sided RTT method - With regards to the double-sided RTT, - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift - Study the order of the SL PRS transmissions for double-sided RTT - Study the impact of UE mobility - FFS study whether there is or what is the spec impact of double-sided RTT method - Note: the above may correspond to RTT with one or multiple devices Agreement in RAN1#111 With regards to the RTT-type solutions using SL, both single-sided and double-sided RTT methods should be introduced - Strive to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT. - Note: a UE should be able to support single-sided RTT without having to support double-sided RTT

For SL-RTT positioning methods as shown in Figure 2.2-2, suppose TOFSS-RTT is the propagation time for a SL PRS transmitted between two UEs and multiplying TOFSS-RTT by the speed of light can yield the distance between two UEs.

TOFSS-RTT=12Tround-Treply

Figure 2.2-2: RTT/single-sided RTT (SS-RTT) for sidelink positioning

Even though SL-RTT does not require accurate synchronization between UEs, the performance and positioning accuracy of multi-RTT positioning method will be impacted by clock shift error of UE due to the limited oscillator accuracy of UE. Suppose the clock shift error of UE1 and UE2 are eUE1, eUE2 respectively. The estimated error of RTT ErrorSS-RTT is proportional to Treply whose unit normally is millisecond.

Tround=1+eUE1TroundTreply=1+eUE2Treply

TOFSS-RTT=12(Tround-Treply)

ErrorSS-RTT=TOFSS-RTT-TOFSS-RTT=≈12TreplyeUE1-eUE2

Observation 1: SL-RTT does not require perfect synchronization among different UEs but its accuracy can be impacted by UE’s clock shift error.

Figure 2.2-3: Double-sided RTT (DS-RTT) for sidelink positioning

As mentioned by other companies in RAN1#109-e meeting [2], introducing double-sided RTT (DS-RTT) as shown in Figure 2.2-3 will significantly improve positioning accuracy. The estimated error of RTT ErrorDS-RTT is proportional to TOFDS-RTT whose unit normally is nanosecond. By introducing DS-RTT, the positioning accuracy can be significantly improved.

TOFDS-RTT=12(Tround1-Treply1)

TOFDS-RTT=12(Tround2-Treply2)

TOFDS-RTT=Tround1×Tround2-Treply1×Treply2Tround1+Tround2+Treply1+Treply2

ErrorDS-RTT=TOFDS-RTT-TOFDS-RTT=1-1+eUE1+1+eUE221+eUE11+eUE2TOFDS-RTT=≈(eUE1+eUE2)2TOFDS-RTT

Extra signaling(s) might be needed for DS-RTT compared to SS-RTT, for example, a DS-RTT round-trip indicator should be designed to indicate whether DS-RTT or SS-RTT is used. Specially, a DS-RTT round-trip indicator containing configuration/triggering/activation/deactivation of the 1st and 2nd TX-to-RX round-trip can be carried in SCI. For example, as shown in Table 2.2-1, when the indicator is set to 0, it means the 1st round-trip SL PRS (SL PRS 1 in Figure 2.2-3) is transmitted by initiating UE (UE 1 in Figure 2.2-3). If SCI does not carry a DS-RTT round-trip indicator, DS-RTT positioning is not triggered and alternatively SS-RTT may be used.

Table 2.2-1: DS-RTT round-trip indicator

| DS-RTT round-trip indicator | Descriptions |
| --- | --- |
| 0 | Initiating the 1st round-trip of DS-RTT. |
| 1 | Triggering the 2nd round-trip of DS-RTT, and request the initial UE’s second SL PRS transmission for DS-RTT. |

Proposal 4: With regards to double-sided RTT for SL positioning,

  - Introducing a DS-RTT round-trip indicator in SCI to indicate the transmission of the 1st and 2nd TX-to-RX round-trip.

### 2.2.2 Measurements

Agreement in RAN1#112 For definition of SL PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL PRS transmission time is used for the definition of SL PRS based Rx-Tx time difference measurement - Alt2: SL PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. Agreement in RAN1#112 For SL PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3

According to TS 38.215 [3], UE Rx – Tx time difference is defined as the difference between received timing of downlink subframe and transmit timing of uplink subframe.

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

One of the main differences between the measurement report of DS-RTT and that of SS-RTT is: a single UE needs to report at least two Rx-Tx time difference and those two Rx-Tx time differences may satisfy a certain timing relationship. For example, two UE Rx-Tx time difference measurements can share a common time stamp, and the common time stamp can be the ending time stamp for the first UE Rx-Tx time difference and also the starting time stamp for the latter UE Rx-Tx time difference.

As shown in the Figure 2.2-3, if UE Rx – Tx time difference for SL DS-RTT is calculated as the difference between received timing and transmit timing of sidelink subframe, ideally the transmit timing of t0’s subframe is the same as that of t4’s subframe and the received timing of t1’s subframe is the same as that of t5’s subframe. Where t0, t1, t2, t3, t4, t5 are the UE received timing of SL PRS and UE transmit timing of SL PRS. We can derive the following equations:

Tround1=Treply2

Treply1=Tround2

If this is the case, DS-RTT have no remarkable superiority over SS-RTT. Therefore, UE Rx – Tx time difference for SL DS-RTT should be defined as the difference between SL PRS received timing and actual SL PRS transmit timing, instead of the difference between received timing subframe and transmit timing of sidelink subframe which is closest to the received timing.

Moreover, regarding how to minimize the impact of UE reference timing offset and mobility, actual SL PRS transmission time should be used for the definition of SL PRS based Rx-Tx time difference measurement even for single-sided RTT method. Consider this situation where the synchronization reference source is changed (e.g. change from GNSS to gNB) during RTT’s SL PRS transmission procedure, subframe boundary and SL PRS transmit timing can be changed according to TS 38.133 [7].

12.2.2 GNSS as synchronization reference source The requirements in this subclause are applicable when the reference timing used by the UE for V2X sidelink communication is derived from GNSS. The sidelink transmissions takes place before the subframe starting boundary as defined in TS 38.331 [2], where = 0 and=0. 12.2.3 NR Cell as synchronization reference source The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is a NR serving cell on a non-V2X sidelink carrier or a V2X sidelink carrier. The sidelink transmissions takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where = 0. If uplink transmission and sidelink transmission are in the same band, is defined in Table 7.1.2-2, otherwise is 0.

Using the actual SL PRS transmission time and actual receiving time (i.e. Tround, Treply) is beneficial to address this problem and further increase SL positioning accuracy. Also, in such case, same definition of UE Rx-Tx time difference can be applied for both single-sided RTT and double-sided RTT:

The UE Rx – Tx time difference for sidelink positioning is defined as TUE-RX – TUE-TX, where:

- TUE-RX is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time.

- TUE-TX is the UE transmit timing of sidelink subframe #j based on actual transmission time.

Proposal 5: With regards to the RTT-type solutions using SL, the same definition of UE Rx-Tx time difference for SL positioning (TUE-RX – TUE-TX) is applied for both single-sided or double-sided RTT method:

- TUE-RX is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time.

- TUE-TX is the UE transmit timing of sidelink subframe #j based on actual transmission time.

Specifically, because the actual SL-PRS may be transmitted in different slots, the UE Rx – Tx time difference value may be milli-seconds level. So, the UE Rx – Tx time difference for SL DS-RTT can be defined in two granularity levels. The total DS-RTT Rx-Tx time difference is the sum of the coarse granularity level UE Rx – Tx time difference and fine granularity level UE Rx – Tx time difference. For example: millisecond level/slot level Rx – Tx time difference can be reported along with nanosecond level Rx – Tx time difference.

Compared to single-sided RTT, double-sided takes more time to complete the positioning procedure. The longer time one SL positioning method takes, the higher the probability of performance degradation caused by UE mobility or timing changes. In order to reduce and control transmission times of double-sided RTT, a (pre-)configured measurement time window can be applied.

UE is able to obtain other UE’s SL PRS transmission pattern through LMF via LPP, or directly through UE-2-UE signaling (e.g. SLPP). According to SL communication specification in Rel-16/17, even though there are synchronization errors due to UE’s timing error and/or propagation time, two communicating UEs are synchronized at least at slot/subframe level as shown in Figure 2.2-4. Also, UE is aware of its own SL PRS transmission pattern/configuration, the coarse granularity level Rx-Tx time difference (slot level or subframe level) can be inferred from the SL PRS transmission slot-time of target UE and anchor UE. For example, suppose both UE1 and UE2 transmit SL PRS at slot boundaries, UE1 transmits SL PRS resource in slot #0, #4 with 4 slots as periodicity, UE2 transmits SL PRS in slot #1, #6 with 5 slots as periodicity. Then, from UE1 perspective, the coarse granularity level Rx-Tx time difference is one slot, two slots respectively.

Figure 2.2-4: SL PRS transmission between UE1 and UE2

Proposal 6: With regards to UE Rx – Tx time difference for SL positioning,

  - For double-sided RTT, a single UE needs to report at least two Rx-Tx time difference and those two Rx-Tx time difference measurements should be within a certain timing window.

  - The total DS-RTT Rx-Tx time difference is the sum of a coarse granularity level UE Rx – Tx time difference (slot or subframe level) and a fine granularity level UE Rx – Tx time difference (Tc level).

    - The coarse granularity level Rx-Tx time difference can be inferred from the SL PRS transmission configuration of target UE and anchor UE

2.3 SL-AoA

In the UL-AoA positioning method, the UE position is estimated based on UL-AoA (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP) of uplink radio signals taken at different TRPs, along with some other configuration information. The definition of UL Angle of Arrival (UL AoA) is as follows:

TS 38.215 UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE.

We have the following agreement related to SL-AoA positioning method:

Agreement in RAN1#112 Support both GCS and LCS for SL PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

According to the above agreement, the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS is an FFS. Since LCS orientation is generally different from the GCS orientation, we agree that mapping of LCS to the GCS is necessary for absolute positioning. However, AoA/ZoA relative to LCS without translating to GCS is applicable at least in ranging and relative positioning cases. For example, UE 1 transmits SL PRS to UE 2 where UE 2 is responsible for calculating the relative angle (azimuth angle and vertical angle). UE 2 can obtain AoA/ZoA using its own LCS orientation. Switching AoA/ZoA relative to LCS to AoA/ZoA relative to GCS is not necessary if UE 2 do not need to report this measurement to other entities. One the other hand, if UE 2 needs to report the measurement to other UE/LMF, it’s better to either translate it to GCS or provide extra information regarding its LCS orientation.

Proposal 7: AoA/ZoA relative to LCS without translating to GCS is applicable at least in ranging and relative positioning cases.

2.4 SL positioning measurement report

In Rel-16/17, UE position is estimated and calculated based on measurements and only high-layer may have the capability to calculate position. In other words, the location calculation is not performed at physical layer. Also, higher-layer-signaling (LPP signaling) [4] is used for location information request and report in NR Rel-16/17. Therefore, it is straightforward to support SL positioning measurement request/report in high-layer signaling.

In order to support multiple legacy positioning methods, at least UE Rx-Tx time difference, SL PRS RSRP (reference signal received power), SL-AoA (angle of arrival including both azimuth and zenith of arrival), SL PRS RSTD (reference signal time difference), SL PRS RTOA (Relative Time of Arrival) should be supported. Besides, referring to the discussion in Rel-17, SL PRS RSRPP (reference signal received path power) may also be introduced for accuracy improvement. The contents of SL positioning measurement report are studied and discussed in previous RAN1 meetings, the agreements shown in the following box were achieved accordingly.

Agreement in RAN1#110 For the content of the sidelink positioning measurement report, potential elements may include at least the following: - One or more sidelink positioning measurement(s) - Timestamp(s) associated with a sidelink positioning measurement - Quality metric(s) associated with a sidelink positioning measurement - Identification Information for a sidelink positioning measurement - FFS any detail for the above Agreement in RAN1#111 With regards to the Positioning methods supported using SL PRS measurements - at least the following measurements should be introduced: - SL PRS based Rx-Tx measurement - SL PRS based RSTD measurement - SL PRS based RSRP measurement - SL PRS based RSRPP measurement - SL PRS based RTOA measurement - SL PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement Agreement in RAN1#112 Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement in RAN1#112 Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE). Agreement in RAN1#112 Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL PRS resource ID/SL PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded Agreement in RAN1#112 LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). - LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. - No specification impact for how to set this indicator. - From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18.

### 2.4.1 Unified measurement report

According to current Rel-16/17 positioning design, UE measurements are requested/reported for each positioning method as shown in Table 2.4-1. LMF can firstly send a RequestLocationInformation message to UE to request location information, indicating the type of location information needed, accordingly, ProvideLocationInformation message sent from UE to LMF should match or be a subset of the location information requested by LMF.

Table 2.4-1: Measurements reported by UE for each positioning method in Rel-16/17

| Methods | Measurements performed at UE side | Location information elements [7] |
| --- | --- | --- |
| DL-TDOA | DL RSTD, Quality for each measurement, DL-PRS-RSRP, DL-PRS-RSRPP, Time stamp, LOS/NLOS information | NR-DL-TDOA-ProvideLocationInformation |
| RTT | UE Rx-Tx time difference, Quality for each measurement, DL-PRS-RSRP, DL-PRS-RSRPP, Time stamp, LOS/NLOS information | NR-Multi-RTT-ProvideLocationInformation |
| DL-AoD | DL-PRS receive beam index, DL-PRS-RSRP, DL-PRS-RSRPP, Time stamp, LOS/NLOS information | NR-DL-AoD-ProvideLocationInformation |

Apparently all three positioning methods which need measurement results transferred from UE to the LMF include measurement elements “DL-PRS-RSRP”, “DL-PRS-RSRPP”, “Time stamp” and “LOS/NLOS information”. It is not signaling-efficient enough since measurement results derived from timing-based positioning method and angle-based positioning method can be merged for location calculation. Similar as the measurement information transfer framework defined in TS 38.455 [5], we prefer SL positioning measurements report per UE in order to save signaling overhead, instead of measurement report per positioning method.

Proposal 8: SL positioning measurements is not reported per SL positioning method.

### 2.4.2 Measurement report contents

Agreement in RAN1#112 Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e. Note: this does not imply a different measurement report content for reporting to LMF or to UE. Table 6.2 Collection of measurement report content reporting to LMF; reporting to UE. SL-PRS based Rx-Tx measurement. SL-PRS based RSTD measurement. SL-PRS based RSRP measurement. SL-PRS based RSRPP measurement. SL-PRS based RTOA measurement. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement. etc.

In our understanding, the report structure in TS 38.455 can be reused, i.e. SL measurement report can consider at least the following information:

- One or more sidelink positioning measurement(s): UE Rx-Tx time difference, SL PRS RSRP, SL PRS RSRPP, SL-AoA (including both azimuth and zenith of arrival), SL PRS RSTD, SL PRS RTOA

- Time stamps at which the measurements are performed

- Quality metric(s): timing quality or measurement quality

- Location information and quality information of the location of a UE in sidelink positioning measurement report

- LoS/NLoS indicator

- Identification Information

  - UE identification information

  - SL PRS information: SL PRS resource ID and/or SL PRS resource set ID

Among the contents of SL measurement report, the present of SL positioning measurement elements (e.g. UE Rx-Tx time difference, SL PRS RSRP, SL PRS RSTD) can be set as optional and the presence of time stamp, timing quality, and SL PRS identification information should be set as mandatory. UE identification is mandatory for LMF-based positioning since LMF need to know the measurements’ corresponding anchor UE ID or UE pair information. For UE-based positioning, it is optional depending on scenarios. The following measurement report messages can be supported:

Proposal 9: The following elements for SL PRS based measurements can be reported to LMF or UE:

|  | Reporting to LMF | Reporting to UE |
| --- | --- | --- |
| SL PRS based measurement | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |

Next, we will focus on the details for some measurement contents.

- UE identification information

UE identification information can be reported along with measurements in order to let calculation entity aware of corresponding UE pair. In other words, it is necessary to let calculation entity know whether this measurement is obtained by transmit SL PRS from UE1 to UE2 or from UE 1 to UE 3.

When it comes to how to define UE identification information in SL positioning measurement report, all UE’s ID, PC5 unicast link identifier, V2X service identifier, UE’s application layer ID, destination layer-2 ID, source layer-2 ID, Source user info, target user info are potential candidates. Whether source ID, destination ID or other UE ID information can be up to RAN2’s design.

Proposal 10: UE identification information can be included in SL positioning measurement report.

- How to define this UE identification information is up to RAN2

- Time stamp

In Uu positioning, NR-TimeStamp defines the UE measurement associated time stamp where each time stamp mandatorily includes dl-PRS-ID, SFN and slot number within the SFN and optionally includes nr-PhysCellID, nr-ARFCN, nr-CellGlobalID.

NR-TimeStamp-r16 ::= SEQUENCE {

dl-PRS-ID-r16				INTEGER (0..255),

nr-PhysCellID-r16			NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16			NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16				ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-SFN-r16					INTEGER (0..1023),

nr-Slot-r16					CHOICE {

scs15-r16				INTEGER (0..9),

scs30-r16				INTEGER (0..19),

scs60-r16				INTEGER (0..39),

scs120-r16				INTEGER (0..79)

},

...

}

Similar mechanism can be applied for time stamp for SL positioning. As we mentioned in Section 2.1.2, slot index of the slots that might belong to a sidelink resource pool is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0. Therefore, time stamp for SL positioning can be defined either relative to SFN or DFN, if SFN is used, the time stamp should include SFN and slot number within the SFN and optionally include nr-PhysCellID, nr-ARFCN, nr-CellGlobalID; if DFN is used, the time stamp should include DFN and slot number within DFN or current UTC time obtained from GNSS. Moreover, in order to increase the accuracy of time stamp, symbol number within a slot can also be supported.

Proposal 11: Time stamp supported in SL positioning measurement report can be defined either relative to SFN or relative to DFN:

- SFN: including SFN and slot number within the SFN and optionally including nr-PhysCellID, nr-ARFCN, nr-CellGlobalID,

- DFN: including DFN and slot number within DFN or current UTC time obtained from GNSS

- For either SFN or DFN, symbol number within a slot can also be included in time stamp

### 2.4.3 Measurement report signaling

In RAN1#112, the following proposals were discussed but not agreed yet:

Proposal in RAN1#112 Support at least higher layer signaling for sidelink positioning measurement report and report triggering. - FFS on SCI based lower layer measurement report and report triggering Proposal in RAN1#112 Support at least one shot and periodic report for sidelink positioning measurement report. - FFS the difference between periodic report and semi-persistent report

Issue 1: higher layer signaling or lower layer signaling for SL positioning measurement report

Firstly, higher layer signaling for sidelink positioning measurement report and report triggering should be supported. The candidate higher layer signaling at least includes: PC5-RRC, PC5-S, SLPP and which of them is used should be up to RAN2. Physical layer is not suitable for SL measurement report since the location calculation entity is in the application layer.

Proposal 12: Only support higher layer signaling for sidelink positioning measurement report and report triggering.

Issue 2: time domain behavior of SL positioning measurement report

As defined in TS 37.355, LMF can request UE for periodic measurements reporting or triggered measurement reporting as follows:

CommonIEsRequestLocationInformation ::= SEQUENCE { locationInformationType LocationInformationType, triggeredReporting TriggeredReportingCriteria OPTIONAL, -- Cond ECID periodicalReporting PeriodicalReportingCriteria OPTIONAL, -- Need ON additionalInformation AdditionalInformation OPTIONAL, -- Need ON qos QoS OPTIONAL, -- Need ON environment Environment OPTIONAL, -- Need ON locationCoordinateTypes LocationCoordinateTypes OPTIONAL, -- Need ON velocityTypes VelocityTypes OPTIONAL, -- Need ON ..., [[ messageSizeLimitNB-r14 MessageSizeLimitNB-r14 OPTIONAL -- Need ON ]], [[ segmentationInfo-r14 SegmentationInfo-r14 OPTIONAL -- Need ON ]], [[ scheduledLocationTime-r17 ScheduledLocationTime-r17 OPTIONAL, -- Need ON targetIntegrityRisk-r17 TargetIntegrityRisk-r17 OPTIONAL -- Need ON ]] }

For higher layer triggered SL positioning measurement report, similar as Uu positioning, LMF or UE may request a UE for either periodic reporting or triggered reporting.

Proposal 13: With regards to time domain behavior for sidelink positioning measurement report, LMF or UE may request a UE for either periodic reporting or triggered reporting.

2.5 Joint SL and Uu positioning methods and measurements

According to RAN1’s discussion in SI phase, both SL-only-based positioning and joint SL & Uu positioning are studied and recommended for normative work.

Agreement in RAN1#111 From RAN1 perspective, at least the following 2 operation scenarios are recommended for normative work: - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

DL-TDOA-like method is a typical technique for joint positioning where both DL-PRS from multiple TRPs and SL PRS from multiple anchor UEs are transmitted to target UE. Take Figure 2.5-1 as an example, target UE’s position cannot be calculated by either Uu positioning or SL positioning since DL-TDOA-like positioning requires at least 3 anchor nodes wherein one of them is a reference node. In such case, we have to merge both Uu measurements (i.e. measured between target UE and TRP 1/2) and SL measurements (i.e. measured between target UE and anchor UE 1/2).

Figure 2.5-1: DL-TDOA-like joint SL & Uu positioning

Observation 2: Neither SL-TDOA nor DL-TDOA is workable under the situation of insufficient anchor TRPs or anchor UEs.

However, usually anchor UEs and TRPs are not accurately synchronized and reference nodes for SL and Uu positioning are selected separately. For Uu positioning, a reference TRP (e.g. TRP 1) is selected as reference node, Uu RSTD measurement is obtained by calculating relative timing difference between the anchor TRP and a reference TRP. For SL positioning, a reference UE (e.g. anchor UE 1) is selected as reference node, SL measurement RSTD is obtained by calculating relative timing difference between the anchor UE and the SL reference node. For UE-assisted positioning, DL measurement and SL measurement can be reported to LMF jointly. In order for the LMF to know the time difference or time synchronization error between Uu session and SL session, a special RSTD measurement can be introduced and reported from target UE to LMF. This special RSTD is used to specify the measurement of the relative timing difference between SL reference node and Uu reference node.

Proposal 14: Support DL-TDOA-like joint SL & Uu positioning:

- For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node.

Conclusion

In this contribution, we discuss potential solutions for SL positioning measurements and reporting considering RTT-type solutions using SL, SL-AoA, SL-TDOA, and we have the following observations and proposals:

Observation 1: SL-RTT does not require perfect synchronization among different UEs but its accuracy can be impacted by UE’s clock shift error.

Observation 2: Neither SL-TDOA nor DL-TDOA is workable under the situation of insufficient anchor TRPs or anchor UEs.

Proposal 1: With regards to mitigate impact of synchronization errors between anchor UEs for SL-TDOA method, support reporting synchronization difference between reference UE and a list of anchor UEs.

- The quality of the synchronization difference can also be reported.

- Note: Clock source’s accuracy can be one of the factors that determine quality of the synchronization information.

Proposal 2: SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where

- T0 is the nominal beginning time of SFN 0 or DFN0.

- tSL PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively

Proposal 3: SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi,

- Where:

  - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j.

  - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j.

  - For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE.

Proposal 4: With regards to double-sided RTT for SL positioning,

  - Introducing a DS-RTT round-trip indicator in SCI to indicate the transmission of the 1st and 2nd TX-to-RX round-trip.

Proposal 5: With regards to the RTT-type solutions using SL, the same definition of UE Rx-Tx time difference for SL positioning (TUE-RX – TUE-TX) is applied for both single-sided or double-sided RTT method:

- TUE-RX is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time.

- TUE-TX is the UE transmit timing of sidelink subframe #j based on actual transmission time.

Proposal 6: With regards to UE Rx – Tx time difference for SL positioning,

  - For double-sided RTT, a single UE needs to report at least two Rx-Tx time difference and those two Rx-Tx time difference measurements should be within a certain timing window.

  - The total DS-RTT Rx-Tx time difference is the sum of a coarse granularity level UE Rx – Tx time difference (slot or subframe level) and a fine granularity level UE Rx – Tx time difference (Tc level).

    - The coarse granularity level Rx-Tx time difference can be inferred from the SL PRS transmission configuration of target UE and anchor UE

Proposal 7: AoA/ZoA relative to LCS without translating to GCS is applicable at least in ranging and relative positioning cases.

Proposal 8: SL positioning measurements is not reported per SL positioning method.

Proposal 9: The following elements for SL PRS based measurements can be reported to LMF or UE:

|  | Reporting to LMF | Reporting to UE |
| --- | --- | --- |
| SL PRS based measurement | UE identification information (Mandatary for reporting to LMF) (Optional for reporting to UE) SL PRS resource/resource set ID info Mandatory SL PRS RSTD Optional SL PRS RTOA Optional SL PRS AOA Optional SL PRS ZOA Optional SL PRS RSRP Optional SL PRS RSRPP Optional SL Rx-Tx time difference Optional Time stamp Mandatory Timing quality Mandatory LOS/NLOS indicator … |  |

Proposal 10: UE identification information can be included in SL positioning measurement report.

- How to define this UE identification information is up to RAN2

Proposal 11: Time stamp supported in SL positioning measurement report can be defined either relative to SFN or relative to DFN:

- SFN: including SFN and slot number within the SFN and optionally including nr-PhysCellID, nr-ARFCN, nr-CellGlobalID,

- DFN: including DFN and slot number within DFN or current UTC time obtained from GNSS

- For either SFN or DFN, symbol number within a slot can also be included in time stamp

Proposal 12: Only support higher layer signaling for sidelink positioning measurement report and report triggering.

Proposal 13: With regards to time domain behavior for sidelink positioning measurement report, LMF or UE may request a UE for either periodic reporting or triggered reporting.

Proposal 14: Support DL-TDOA-like joint SL & Uu positioning:

- For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node.

Reference

- RP-223549 New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting #98-e.

- R1-2205457	Moderator Summary #2 for [109-e-R18-Pos-04] Email discussion on potential solutions for SL positioning	Moderator (Qualcomm)

- 3GPP TS 38.215 h30: "NR; Physical layer measurements (Release 17)"

- 3GPP TS 37.355 h40: "LTE Positioning Protocol (LPP) (Release 17)".

- 3GPP TS 38.455 h40: "NR Positioning Protocol A (NRPPa) (Release 17)".

- 3GPP TS 38.214 h50: “Physical layer procedures for data (Release 17)”

- 3GPP TS 38.133 i00: “Requirements for support of radio resource management”