3GPP TSG RAN WG1 #112bis-e                                    R1-2303278

e-Meeting, April 17th – 26th, 2023

Source:         ZTE

Title:            Discussion on resource allocation for SL positioning reference signal

Agenda item:    9.5.1.3

Document for:	 Discussion and Decision

Introduction

In RAN#98-e meeting, various features have been recommended for normative work [1] for support of sidelink (SL) ranging/positioning. The following shows the expected enhancements for SL-PRS resource allocation in the WID:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

In this contribution, we provide our views and solutions for SL-PRS resource allocation including SL-PRS resource pool, SL positioning resource allocation and cast type of SL-PRS transmission.

Discussion

Agreement in RAN1#111 With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility Proposal in RAN1#112 - For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) can be (pre-)configured in the only SL BWP of a carrier. In addition, - Alt. 1: It is not allowed to (pre-)configure both dedicated resource pool and shared resource pool in the SL-BWP of a carrier. - Alt. 2: It is allowed to (pre-)configure both dedicated resource pool and shared resource pool in the SL-BWP of a carrier. - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool. Agreement in RAN1#112 - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

For a SL-PRS resources pool, according to the agreements made in RAN1#111, either dedicated resource pool and/or a shared resource pool with SL communication can be (pre-)configured for SL-PRS. Support of both two kind of resource pools leads to a question on whether it is allowed to (pre-)configure both dedicated resource pool and shared resource pool in the SL-BWP of a carrier. In our understanding, a UE can be configured with one or more SL resource pools for SL positioning where the SL resource pools can be either dedicated resource pool or shared resource pool for more flexibility. Dedicated resource pool is used only for SL-PRS transmission in SL positioning, shared resource can be used for both SL-PRS transmission and PSSCH transmission. Moreover, shared resource pool and SL communication pool can also be used for SL positioning measurement report. Overall, it is up to higher layer (pre-)configuration on how to configure different resource pools for a UE.

Proposal 1: A UE can be (pre-)configured with one or more SL resource pools for SL-PRS transmission.

- The SL resource pools can be dedicated resource pool, shared resource pool.

## 2.1 Dedicated SL-PRS resource pool

### 2.1.1 Channels in dedicated SL-PRS resource pool

Agreement in RAN1#110b-e For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above Agreement in RAN1#112 For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

As shown in the above agreements, we discussed which channels can be included in the dedicated SL-PRS resource pool in RAN1#110b-e and RAN1#112 [6]. Among those 3 options, Option 1 (No other channel can be included beyond SL-PRS) is not pursued further.

For Option 2, dedicated SL-PRS resource pool includes PSCCH which carries SCI. And for Option 3, dedicated SL-PRS resource pool includes both PSCCH and PSSCH associated with SL-PRS transmission(s). The main discussion point is whether PSSCH is needed in a dedicated resource pool. From our perspective, we support Option 2 for the following reasons:

- Saving resources for SL-PRS: Including PSSCH in dedicated resource pool can cause a waste of resources. From frequency domain perspective, normally the bandwidth of PSSCH is small than that of SL-PRS. But for SL positioning, large bandwidth is required for high-accuracy positioning especially if timing-based positioning methods are used, the larger the bandwidth, the higher the positioning accuracy. From time domain perspective, extra AGC symbol between PSSCH and SL-PRS might be needed due to the mismatched/different transmission power. Moreover, if considering the symbols used by PSCCH, PSSCH, AGC symbol, gap symbol, available symbols for SL-PRS transmission in a slot can be limited and thus makes it difficult to support 8-symbol or larger-symbol SL-PRS.

- Reducing spec impact and work load: In SL communication, PSSCH is used for carrying SL-data and the 2nd stage SCI. The design of PSSCH will result in largely increasing spec impact and work load considering PSSCH resource allocation (e.g. symbols, sub-channels), the 2nd stage SCI, DMRS pattern in PSSCH, power control of PSSCH and etc. Since single stage SCI is sufficient for triggering/reserving SL-PRS resource(s), PSSCH shall not be transmitted in dedicated resource pool.

Proposal 2: For a dedicated resource pool for SL positioning, in addition to SL-PRS, PSCCH which carries SCI associated with SL-PRS transmission(s) is included in the resource pool.

- PSSCH is not included in a dedicated resource pool

### 2.1.2 Bandwidth of SL-PRS transmission

Agreement in RAN1#110b-e At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

Even though larger bandwidth brings higher positioning accuracy especially for timing-based positioning methods, different positioning scenarios may have different accuracy requirements. When the positioning accuracy requirement is not high, a smaller bandwidth compared to that of the resource pool can be applied to save frequency resource and further get power boosting gain. Moreover, for angle-based positioning methods, bandwidth may not be a critical influence factor of accuracy in some scenarios. The bandwidth of SL-PRS should be able to flexibly configured depending on different positioning requirements and different positioning methods. Given that we have two options for SL-PRS bandwidth:

- The bandwidth of SL-PRS transmission is indicated in SCI. For either a dedicated resource pool or a shared resource pool, the bandwidth can be same or smaller than that of the resource pool. For a shared resource pool, the bandwidth is the same as that of PSSCH if they are transmitted in the same slot.

- The bandwidth of SL-PRS transmission is up to SL PRS configuration in a resource pool. In such case, SCI only needs to indicate a SL-PRS resource ID (or based on the mapping between SCI and SL-PRS) where the SL-PRS resource is configured in resource pool and bandwidth is still indicated in higher layer signaling. There are two ways to guarantee SL-PRS bandwidth configuration flexibility:

  - The bandwidth of SL-PRS can be configured per dedicated resource pool. In other words, the bandwidth of SL-PRS is the same as that of the resource pool. A UE can be (pre-)configured with multiple resource pools to make sure it can transmit SL-PRS of various bandwidths.

  - In order to increase configuration flexibility, the bandwidth of SL-PRS can be the same or smaller than that of a resource pool. For example, bandwidth is one of the parameters configured for each SL PRS resource

We should first figure out how to configure SL PRS bandwidth and down-select between two options. In our understanding, dedicated resource pool will not have the situation when SL-data and SL-PRS are transmitted in the same slot, and dedicated resource pool does not need to apply the same 1st SCI which includes “frequency resource assignment” if PRS bandwidth can be informed by RRC signaling. Among Option (2a) and Option (2b), we prefer to apply similar to the bandwidth configuration of DL-PRS (i.e. configured per positioning frequency layer (PFL)), the bandwidth of SL-PRS is configured per resource pool.

Proposal 3: For a dedicated resource pool for positioning, with regards to the bandwidth of SL-PRS transmission, support the following option:

- Option (2): SL PRS bandwidth is up to higher layer configuration in a resource pool.

  - Option (2a): The bandwidth of SL-PRS can be configured per dedicated resource pool. In other words, the bandwidth of SL-PRS is the same as that of the resource pool.

### 2.1.3 Dedicated SL-PRS resource pool configuration

Agreement in RAN1#112 For a dedicated resource pool for positioning: - The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214). - FFS: additional slots that can be used for SL PRS is not precluded - Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

For dedicated SL-PRS resource pool design, we believe a carrier frequency or a BWP dedicated for SL-PRS can provide sufficient time-frequency resources for SL-PRS and thus results in high-accuracy positioning. However, As mentioned early, only one BWP and one carrier frequency is defined in Rel-16/17 SL communication. We may consider to separately specify frequency or BWP configuration for SL-PRS after completing specifying SL CA operation in NR sidelink evolution. Therefore, In Rel-18, we prefer dedicated SL-PRS resource pool associated with the BWP of a carrier that is the same as that of the SL communication resource pool. For resource pool level configuration as shown in Figure 2.1-1, the SL-PRS and SL-data belong to the same BWP but different resource pool configuration. Each pool/configuration can occupy a part of the BWP resource including time and frequency resources.

Figure 2.1-1: Dedicated SL-PRS configuration - resource pool level

Based on the agreement made in the last RAN1 meeting, a dedicated SL-PRS resource pool can be configured for reception of SL-PRS, or for transmission of SL-PRS which can be associated with either SL-PRS resource allocation scheme 1 or SL-PRS resource allocation scheme 2.

- Scheme 1 SL-PRS resource pool: indicates the resources by which the UE is allowed to transmit SL-PRS based on network scheduling on the configured BWP.

- Scheme 2 SL-PRS resource pool: indicates the resources by which the UE is allowed to transmit SL-PRS by UE autonomous resource selection on the configured BWP.

Set of slots belonging to a dedicated resource pool

Should dedicated SL-PRS resource pool be multiplexed in a TDM manner with the resource pool for sidelink communication? In our understanding, the relationship between different resource pools is totally up to high layer configuration as in Rel-16/17. If a dedicated resource pool for SL-PRS multiplexed in a TDM manner with the resource pool for sidelink communication is applied, the positioning latency can be high. The distribution principle of reserved slots is discretely and evenly distributed in SFN/DFN cycles (10240 ms), the interval between two reserved slots might be quite large. Moreover, if SL-PRS resource pool is not overlapped with any SL communication resource pools to make sure pool level TDM, the available slots for SL-PRS are limited. Therefore, this type of SL-PRS resource pool design is more suitable for services that do not require high positioning latency or for UEs with low velocity. Further, UE can be (pre-)configured by high layers with one or more SL-PRS resource pools, and multiple UEs can be involved in SL communication. To make sure pool level TDM, SL-PRS resource pool should not be overlapped with any SL communication resource pools, which makes SL-PRS available resources even more restricted. As usual, even though resource pools are configured in non-TDMed manner by RRC signaling, the actual resource allocations can still be TDMed based on the proper network scheduling or UE sensing.

Observation 1: For dedicated resource pool, using reserved slots of SL communication resource pool can increase the latency of SL positioning.

According to the agreement, the set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214). Therefore, the set of slots that may belong to a dedicated resource pool is denoted by (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS) where

-	0≤tiSL-PRS<10240×2μ, 0≤i<Tmax,

-	the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

-	the set includes all the slots except the following slots,

-	NS-SSB slots in which S-SS/PSBCH block (S-SSB) is configured,

-	NnonSL slots in each of which at least one of Y-th, (Y+1)-th, …, (Y+X-1)-th OFDM symbols are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or a higher layer parameter configured in dedicated resource pool (e.g. sl-TDD-Configuration in SL communication) if provided or sl-TDD-Config of the received PSBCH if provided, where Y and X are set by the higher layer parameters.

-	The reserved slots which are determined by the following steps.

1)	the remaining slots excluding NS-SSB slots and NnonSL slots from the set of all the slots are denoted by (l0,l1,⋯,l(10240×2μ-NS-SSB-NnonSL-1)) arranged in increasing order of slot index.

2)	a slot lr (0≤r<10240×2μ-NS-SSB-NnonSL) belongs to the reserved slots if r=m∙(10240×2μ-NS-SSB-NnonSL)Nreserved, here m=0,1,⋯,Nreserved-1 and Nreserved=10240×2μ-NSSSB-NnonSL mod Lbitmap where Lbitmap denotes the length of bitmap configured by higher layers.

-	The slots in the set are arranged in increasing order of slot index.

The UE determines the set of slots assigned to a dedicated resource pool as follows:

-	a bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used where Lbitmap the length of the bitmap is configured by higher layers.

-	a slot tkSL-PRS (0≤k<10240×2μ-NS-SSB-NnonSL-Nreserved) belongs to the set if bk'=1 where k'=k mod Lbitmap.

-	The slots in the set are re-indexed such that the subscripts i of the remaining slots  t'iSL are successive {0, 1, …, T'max-1} where T'max is the number of the slots remaining in the set.

Proposal 4: For the support of dedicated SL-PRS resource pool:

- Pool level TDMed restriction by RRC signaling between SL-PRS resource pool and SL communication pool should be avoided.

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214)

  - The set of slots (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS) that may belong to a dedicated resource pool includes all the slots except NS-SSB slots, NnonSL slots and reserved slots.

  - According to (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS), the set of slots assigned to a dedicated resource pool is based on a bitmap configured in the resource pool.

Configuration information for a dedicated resource pool

From our point of view, dedicated SL-PRS resource pool configured by higher layer should at least include the following parameters:

- SL-PRS resource pool level configuration: time resource of resource pool, frequency resource of resource pool

- SL-PRS configuration: contains SL PRS resource set/ SL PRS resource configuration, for example: the period of SL PRS resource/SL PRS resource set, number of symbols that a SL PRS occupied in a slot, start symbol of SL PRS in a slot, comb size and RE offset, priority of SL PRS resource/SL PRS resource set

- PSCCH configuration: candidates of time resource and frequency resource of PSCCH

- Synchronization configuration: Indicates the allowed synchronization reference(s) which is (are) allowed to use the configured SL-PRS resource pool

- Resource allocation scheme 2 related configuration: CBR config and its association relationship with SL-PRS transmission parameter, SL-PRS resource reservation period, sensing window, selection window, SL-PRS RSRP threshold

- Power control configuration: e.g., maximum SL-PRS transmission power, alpha value and P0 value for either sidelink pathloss based power control or downlink pathloss based power control

Proposal 5: A dedicated SL-PRS resource pool should at least include the following configurations:

- SL-PRS resource pool level configuration: time resource of resource pool, frequency resource of resource pool

- SL-PRS configuration: the period of SL PRS resource/SL PRS resource set, number of symbols that a SL PRS occupied in a slot, start symbol of SL PRS in a slot, comb size and RE offset, priority of SL PRS resource/SL PRS resource set

- PSCCH configuration: candidates of time resource and frequency resource of PSCCH

- Synchronization configuration: indicates the allowed synchronization reference(s) which is (are) allowed to use the configured SL-PRS resource pool

- Resource allocation scheme 2 related configuration: CBR config and its association relationship with SL-PRS transmission parameter, SL-PRS resource reservation period, sensing window, selection window, SL-PRS RSRP threshold

- Power control configuration: maximum SL-PRS transmission power, alpha value and P0 value for either sidelink pathloss based power control or downlink pathloss based power control

Specifically, in a dedicated SL-PRS resource pool, frequency resource occupied by multiple SCI is usually less than the bandwidth of SL-PRS resource. To make full use of frequency resource and randomize interference, Frequency resource occupied by SCI can be limited to a range of bandwidth of SL-PRS resource pool in a slot and multiple SCIs can be multiplexed in FDM manner.

As shown in Figure 2.1-2, multiple SL-PRS can be multiplexed in SL-PRS resource pool, where both frequency division multiplexing (FDM) manner and time division multiplexing (TDM) manner are supported for SL-PRS in dedicated resource pools. Each SL-PRS resource can be triggered/reserved by one SCI. Number of SCI configuration relies on the number of SL-PRS resources. In such case, the mapping between SCI and the associated SL-PRS can be configured between SCI location and PRS configuration/resources for each dedicated resource pool. To be more specific, multiple pairs of “SCI location and PRS configuration” can be configured in dedicated resource pool. The corresponding configurations at least include:

- Frequency range of SCI (e.g. starting PRB of SCI, PRB number of SCI)

- Comb offset of associated SL-PRS, symbol offset of associated SL-PRS.

For instance, as shown in the figure, four SCIs map to four SL-PRS resources respectively. If SCI 1 is selected by a UE 1, then SL-PRS 1 should be scheduled by the SCI 1. If another UE 2 selects SCI 2, the SL-PRS resource 2 should be scheduled accordingly. Then, the SL-PRS transmission collision between the two UEs are avoided if the two UEs select different SCIs.

Figure 2.1-2: Multiple SL-PRS multiplexing in either an FDM manner or a TDM manner in a slot of dedicated resource pool

Proposal 6: In order to avoid SL-PRS collision, support the mapping between each SCI and each SL-PRS configuration for a dedicated resource pool, where:

- Frequency resource occupied by SCI is limited to a range of bandwidth of dedicated resource pool and multiple SCIs can be multiplexed in FDM manner.

- The mapping relationship of “SCI location and SL-PRS configuration” can be configured in each dedicated resource pool.

  - SCI location refers to frequency range of SCI (e.g. starting PRB of SCI, PRB number of SCI)

  - SL-PRS configuration refers to comb offset or symbol offset.

### 2.1.4 SCI for SL-PRS

Proposal in RAN1#112 In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS, downselect between the following alternatives: - Alt. 1: A new single-stage SCI in PSCCH is introduced - Alt 2: A 2-stage SCI design is introduced Study at least the following aspects: - The contents of the SCI(s) (e.g., Time/Frequency resource assignment; Resource reservation period, SL-PRS resource indication, priority, source ID, destination ID) and mapping rule between PSCCH and SL-PRS. - Whether to support same-slot only or both same-slot and cross-slot SL-PRS reservation signaling

For SL communication, SCI are carried in both PSCCH and PSSCH. SCI carried on PSCCH is 1st stage SCI, which transports SL scheduling information. SCI carried in PSSCH is 2nd stage SCI, which transports SL scheduling information, and/or inter-UE coordination related information. However, for dedicated SL-PRS resource pool, two-stage SCI may not be needed anymore since SL-PRS will not transmitted in the same resource pool with SL-data which carried in PSSCH. As we analyze in section 2.1.1, PSSCH is not included in a dedicated resource pool in order to save resources for SL-PRS transmission and to reduce the work load. Therefore, we support introducing a new single-stage SCI in PSCCH for SL-PRS transmission.

Information carried in SCI for SL-PRS should at least include: SL-PRS priority (the priority value of SL-PRS can be configured as decimal to distinguish the service priority between positioning and communication), source ID, destination ID, cast type (unicast, groupcast, broadcast), resource reservation period, SL-PRS configuration information. Regarding the SL-PRS configuration information, there are two options:

- SCI indicates SL-PRS resource ID: this field can implicitly indicate number of symbols, comb size and RE offset, symbol offset of SL-PRS, bandwidth, SL-PRS sequence ID

- SCI indicates time and frequency resource assignment: apply similar design as the SCI 1-A in SL communication

Proposal 7: In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS:

- A new single-stage SCI in PSCCH is introduced

- Information carried in SCI for SL-PRS should at least include: SL-PRS priority, source ID, destination ID, cast type, resource reservation period, SL-PRS configuration information. Regarding the SL-PRS configuration information, down-select between the following two options:

  - SCI indicates SL-PRS resource ID: this field can implicitly indicate number of symbols, comb size and RE offset, symbol offset of SL-PRS, bandwidth, SL-PRS sequence ID

  - SCI indicates time and frequency resource assignment: apply similar design as the SCI 1-A in SL communication

### 2.1.5 Resource allocation granularity

Agreement in RAN1#110b-e - Study further the granularity of time-domain resource allocation for SL-PRS transmission.

In current Rel-16/17 SL communication, the granularity of time-domain resource allocation for SL-data transmission is slot-based allocation. However, applying the same slot-based allocation for SL-PRS transmission may not be very resource efficient. In our companion contribution [4], we discussed the frequency and time domain pattern of a SL-PRS resource. A SL-PRS resource within a slot occupies M SL PRS symbols (excluding AGC symbol and gap symbol) with comb size N in a slot. Also, a list of SL-PRS resources will be defined at least in dedicated SL-PRS resource pool, we suggest to use SL-PRS-resource-based allocation. Furthermore, compared with slot-based SL-PRS allocation, resource-based allocation is more extensible to FR2 in which multiple resources with different beams may be multiplexed in the same slot or across some slots.

Moreover, AGC training symbol and Rx/Tx turnaround symbol should also be considered for SL-PRS resource allocation if one AGC training symbol and/or one Rx/Tx turnaround symbol is allocated per SL-PRS resource.

Proposal 8: With regards to the granularity of time-domain resource allocation for SL-PRS transmission

- Support SL-PRS-resource-based allocation, i.e. M <=14 symbols with comb size N within a slot are configured for each SL-PRS resource

## 2.2 Shared resource pool(s) with sidelink communication for SL-PRS

Agreement in RAN1#110b-e With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

### 2.2.1 SL-PRS multiplexing within shared resource pool

Proposal in RAN1#112 In a shared resource pool, study the following options - Opt. 1: SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot - The PSSCH and SL-PRS bandwidth is the same - With regards to PSSCH and SL-PRS multiplexing - Alt. 1: Only TDMing is supported - Alt. 2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported - Discuss further whether PSFCH could also exist in this slot - Opt. 2: SL-PRS and associated PSCCH are included (no PSSCH) in a slot - The presence of SL-PRS is indicated in the 1st SCI in the PSCCH - Opt. 3: SL-PRS without PSCCH are included (no PSSCH) - SL-PRS may be reserved by a SCI siganling in the previous slot - In either option, study multiplexing option between PSCCH (if present) and SL-PRS are TDMed

For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), SL-PRS and SL-data transmission/reception share the same resource pools and same logic slots. In current SL specification, 1st stage SCI is carried in PSCCH and used for the scheduling of PSSCH and 2nd stage SCI on PSSCH. Also, the location of each PSCCH resource is fixed: in time domain, PSCCH starts from the 2nd symbol assigned for SL in the same slot with its associated PSSCH; in frequency domain, PSCCH starts from the first RB of the first sub-channel of its associated PSSCH. As shown in the above proposal in RAN1#112, support of a slot including only SL-PRS (without PSCCH and PSSCH) is not aligned with the basic slot structure designed for SL communication. Moreover, the number of reserved bits of the existing 1st stage SCI is too limited (i.e. {2,3,4} as configured by higher layer parameter sl-NumReservedBits) and not enough for SL-PRS scheduling for Opt.2. We support Option 1 as shown in Figure 2.2-1, 2nd stage SCI carried in PSSCH is needed for indicating SL-PRS resource configuration information, source ID, destination ID, etc.

Figure 2.2-1: Slot structure for shared resource pool

With regards to the bandwidth of SL-PRS, as presented in Section 2.1.2, whether the bandwidth of SL-PRS is the same as that of PSSCH depends how we configure SL-PRS bandwidth: either the bandwidth of SL-PRS transmission is indicated in SCI or is up to SL PRS configuration in a resource pool.

Due to the half-duplex restriction, the UE cannot transmit and receive sidelink resource at the same time. Furthermore, like Rel-16/17 PRS design, it is better not to support SL-PRS transmitted or received with SL-data at the same time for the purpose of low latency, low UE complexity and high positioning measurement accuracy. Figure 2.2-2 shows one example of FDM between SL-PRS and SL-data which we do not prefer in this Release, we suggest to only consider TDM between SL-PRS and SL-data.

Figure 2.2-2 FDM between SL-PRS and SL-data (not supported)

Proposal 9: In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot.

- With regards to the bandwidth of SL-PRS, down-select between (1) bandwidth is indicated in SCI; (2) bandwidth is up to higher layer configuration in a shared resource pool

- Only TDM between SL-PRS transmission and SL-data transmission is supported

### 2.2.2 SCI for SL-PRS

Proposal in RAN1#112 In a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication - FFS: Details for the new 2nd stage SCI format - Note: data can also be indicated using the 2nd stage SCI in addition to SL-PRS - Opt. 2: SCI1-A & existing 2nd stage SCI format used for SL-PRS indication - The presence of SL-PRS is indicated in the 1st SCI

In shared resource pool, at least the following information is transmitted by means of SCI: priority of SL-PRS, SL-PRS resource time and frequency indication (e.g. reservation period), source ID, destination ID, cast type indicator. The design should consider both SCI 1-A and 2nd stage SCI.

Currently, the fields indicated in SCI 1-A is shown in Table 2.2-1.

Table 2.2-1: Information transmitted by means of the SCI format 1-A [5]

SCI 1-A Field

Priority

Frequency resource assignment

Time resource assignment

Resource reservation period

DMRS pattern

2nd-stage SCI format

Beta_offset indicator

Number of DMRS port

Modulation and coding scheme

Additional MCS table indicator

PSFCH overhead indication

Reserved

Conflict information receiver flag

There are two ways to design 2nd stage SCI for SL-PRS transmission: (1) introducing a new 2nd stage SCI; (2) extend the existing 2nd stage SCI.

If a new 2nd stage SCI (e.g. SCI 2-D) is used for SL-PRS indication, it is possible to indicate both SL-PRS and SL-data in this new 2nd stage SCI. Therefore, we support to use one bit to indicate whether only SL-PRS is indicated or both SL-PRS and SL-data are indicated in this SCI. Similarly, extra bits are needed to indicates whether only SL-PRS, only SL-data, or both SL-PRS and SL-data are transmitted if we reuse the existing 2nd stage SCI.

If both SL-PRS and SL-data are indicated using the 2nd stage SCI, it is expected that the cast type, destination ID, source D are shared for both Tx UE’s SL-PRS transmission and SL-data transmission. One Tx UE is not expected to transmit SL data to UE1 but transmit SL-PRS to UE2 in order to save overhead/bits for 2nd stage SCI.

Proposal 10: In a shared resource pool, in addition to SL PRS transmission, the UE transmits

  - Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication

    - Use one bit to indicate whether only SL-PRS is indicated or both SL-PRS and SL-data are transmitted

  - Opt. 2: SCI1-A & existing 2nd stage SCI format used for SL-PRS indication

    - Extra bits are needed to indicates: whether only SL-PRS, only SL-data, or both SL-PRS and SL-data are transmitted

  - At least the following information is transmitted by means of SCI: priority of SL-PRS, SL-PRS resource time and frequency indication (e.g. reservation period), source ID, destination ID, cast type indicator.

With regards to SL-PRS priority indication in shared resource pool, three options should be considered where the priority value of SL-PRS can be integer, non-integer, or discontinuous integer, or discontinuous decimal:

- Option 1: priority indicator of SL-PRS is configured in each PRS configuration of a shared resource pool. For example, each SL-PRS configuration (e.g. SL-PRS resource) is associated with a priority value in a shared resource pool.

- Option 2: priority indicator of SL-PRS is indicated in SCI format 1-A from Tx UE to Rx UE

  - Option 2-1: reuse the current priority indicator in SCI 1-A with an addition parameter indicting the presence of SL-PRS.

  - Option 2-2: introduce a new priority table, each entry corresponds to a combination of priority level for SL-PRS and priority level for PSSCH. One example is shown in Table 2.2-2, where the highest bit represents whether SL-data or SL-PRS is present and the lowest three bits indicating the exact priority value.

- Option 3: priority indicator of SL-PRS is indicated in 2nd -stage SCI format from Tx UE to Rx UE

Table 2.2-2: New priority table indicated in SCI 1-A

|  | Value of priority field | Priority |
| --- | --- | --- |
| SL-data | 0000 | 1 |
|  | 0001 | 2 |
|  | 0010 | 3 |
|  | 0011 | 4 |
|  | 0100 | 5 |
|  | 0101 | 6 |
|  | 0110 | 7 |
|  | 0111 | 8 |
| SL-PRS | 1000 | 1 |
|  | 1001 | 2 |
|  | 1010 | 3 |
|  | 1011 | 4 |
|  | 1100 | 5 |
|  | 1101 | 6 |
|  | 1110 | 7 |
|  | 1111 | 8 |

Among those three options, we prefer Option 2 which indicating SL-PRS priority in SCI 1-A. In such case, it is beneficial to reuse the same sensing mechanism as legacy SL communication, for example, the RSRP threshold used in sensing is based on the priority value indicated in SCI 1-A.

Proposal 11: In a shared resource pool, priority indicator of SL-PRS is indicated in SCI format 1-A from Tx UE to Rx UE

- Priority value of SL-PRS can be integer, non-integer, or discontinuous integer, or discontinuous decimal

## 2.3 SL positioning resource allocation scheme 1

### 2.3.1 Scheme 1 SL-PRS resource allocation

The following agreement were achieved regarding scheme 1 SL-PRS resource allocation:

Agreement in RAN1#110b-e Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable Agreement in RAN1#112 Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

For the design of NG-RAN UE SL positioning architecture for scheme 1 SL-PRS resource allocation, LMF should also be part of sidelink positioning to manage the support of different location services for target UEs. Especially when UEs in one positioning session have more than one serving NG-RAN nodes (e.g. gNBs), it is necessary to use LMF to interact with multiple NG-RAN nodes for assistance data information transfer, the capability of NG-RAN nodes transfer and location information transfer. However, current sidelink communication does not need LMF’s participation and clearly LMF does not have any sidelink related information. In order to support LMF’s involvement for sidelink positioning and hybrid positioning, sidelink related information transfer between LMF and gNB should be supported (Figure 2.3-1 shows one possible signaling procedure).

Figure 2.3-1: Sidelink related information transfer between LMF and gNB

For the periodic SL-PRS configuration (with shares the significant part of current LPP), there exists an issue. Different UEs may belong to different gNB’s reach, when serving gNB provides SL-PRS configuration to scheme 1 UE for transmitting, the scheme 1 UEs with different serving gNBs may possibly acquire SL-PRS configuration with resource conflict. This will lead to an issue that, UEs will have interface with each other when sending the SL-PRS according to the overlapping SL-PRS configuration.

Figure 2.3-2. Example of different scheme 1 UEs in the same sidelink positioning session under different gNB’s coverage

For example, in Figure 2.3-2, UE 1 to UE m are all in coverage and in the same sidelink positioning session, in which, UE 1 and UE 2 have the common serving gNB as gNB 1, UE 3 has the serving gNB as gNB 2, and so on. UE 1 and UE 2 are configured to adopt scheme 1 and can receive the SL-PRS configuration from gNB 1, UE 3 is configured to adopt scheme 1 and can receive the SL-PRS configuration from gNB 2. UE 1 and UE 2 will not have resource conflict due to the serving gNB 1 can allocate well; however, UE 1/UE 2 and UE 3 will have SL-PRS resource conflict since gNB 1 and gNB 2 will not coordinate SL-PRS configuration of these three UEs. Under this situation, it is straightforward to support LMF to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session.

Specifically, regarding how to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session, we have the following solutions and corresponding analyses as shown in Table 2.3-1:

Table 2.3-1: LMF resolving SL-PRS resource conflict between different scheme 1 UEs

| Solution | Details |
| --- | --- |
| Solution 1: LMF to coordinate SL-PRS configuration of different gNBs | (1) LMF needs to send a request message via NRPPa to trigger each gNB to provide the SL-PRS configuration for scheme 1 UE (2) Each gNB provides the response message to LMF containing the configured SL-PRS configuration of each scheme 1 UE. (3a) LMF resolves the interference by LMF’s implementation, for example LMF deletes some of the overlapping SL-PRS resources of some of the gNBs. LMF provides the non-overlapped SL-PRS configuration to mode 1 UEs via LPP. (3b) LMF sends the feedback (e.g. collected all gNBs’ SL-PRS configuration, or preferred/non-preferred resources) to each gNB, gNB distribute the modified SL-PRS configuration to each scheme 1 UE. |
| Solution 2: LMF ask gNBs to coordinate with each other | (1) LMF sends a request message to gNB(s) and the request message is to request gNB(s) to coordinate SL-PRS configuration with other gNB (s) (2) The gNB sets up Xn interface with the other gNB(s) as requested by LMF, and the gNB sends the SL-PRS configuration of itself to other gNB via Xn interface. (3) After receiving one gNB’s SL-PRS configuration, the other gNB(s) can configure non-overlapped SL-PRS configuration. |

Among solution 1 and solution 2, we prefer solution 1 since solution 2 includes some Xn interface design where even the legacy Rel-16/17 positioning do not require the positioning information interaction between gNBs. Solution 1 only need to reuse the NRPPa procedure between LMF and gNB. Considering the UE may usually be configured with periodic, aperiodic or semi-persistent SL-PRS together, we think a unified design is desired, that is, regardless of periodic/aperiodic/semi-persistent SL-PRS (SL-PRS time-domain behavior), serving gNB of the UE should configure the SL-PRS configuration for UE. In other words, among procedure (3a) and (3b), we support (3b) in solution 1.

Proposal 12: For scheme 1 SL-PRS resource allocation, support SL-PRS configuration transfer between LMF and gNB.

### 2.3.2 DCI design

In sidelink resource allocation mode 1 according to TS 38.214 [3], for SL data transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported.

- For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0.

- For sidelink configured grant type 2, the configured grant (CG) is activated by a DCI format 3_0.

- For sidelink configured grant type 1, the PSSCH transmission follows the higher layer configuration.

Dynamic grant and CG type2, CG type1 in SL communication may correspond to aperiodic SL-PRS, semi-persistent SL-PRS and periodic SL-PRS. In order to support mode 1 resource allocation for SL positioning, DCI can be used to dynamically or semi-persistently schedule the SL-PRS resource(s). We should consider whether a new DCI format is needed: (1) DCI format 3_0 can be reused and extra fields related to SL-PRS transmission should be added to DCI format 3_0. (2) A new DCI format exclusively for SL-PRS can be introduced, the new DCI (e.g., with a CRC scrambled by ‘SL-PRS-RNTI’) can be designed to indicate/select several SL-PRS resource configurations.

At lease the following should be added in either DCI format 3-0 or a new DCI format: resource pool index (either dedicated resource pool or shared resource pool), SL-PRS resource configuration index (SL-PRS resource ID or SL-PRS resource ID + SL-PRS resource set ID if resource set is introduced), time gap indicating the gap between DCI reception and the first SL-PRS transmission scheduled by DCI, configuration index indicating which configured grant type 2 is activated by the DCI when UE is configured with multiple CG. As shown in Figure 2.3-3, the time gap between DCI reception and the first SL-PRS transmission is determined based on the information indicated in DCI. The time gap between the first SL-PRS transmission and the second transmission of a SL-PRS resource is determined according to SL-PRS resource configuration.

Figure 2.3-3: DCI scheduling SL-PRS resource

Proposal 13: Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from gNB through Dynamic grant, or through configured grant type 1/type 2

- Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format is needed. At lease the following should be added in either DCI format 3-0 or a new DCI format:

  - Resource pool index (either dedicated resource pool or shared resource pool for SL-PRS)

  - SL-PRS resource configuration index (SL-PRS resource ID or SL-PRS resource ID + SL-PRS resource set ID if resource set is introduced)

  - Time gap indicating the gap between DCI reception and the first SL-PRS transmission scheduled by DCI

  - Configuration index indicating which configured grant type 2 is activated by the DCI when UE is configured with multiple CG

## 2.4 SL positioning resource allocation scheme 2

In SL-PRS resource allocation mode 2, UE selects the candidate resource(s) for SL-PRS transmission in the selection window based on the sensing results. The following agreements were approved in previous RAN1 meetings, wherein companies are encouraged to study resource selection mechanism for SL-PRS, Intel-UE coordination (IUC) and SL-PRS congestion control mechanisms.

Agreement in RAN1#110 Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects: - Resource selection mechanism for SL-PRS - Inter-UE coordination - Aspects for congestion control mechanisms for SL-PRS Agreement in RAN1#111 For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. Agreement in RAN1#112 For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

### 2.4.1 Sensing-based resource allocation

For SL-PRS resource allocation mechanism in scheme 2, a sensing-based resource selection should be the baseline. The higher layer can request a UE to determine a subset of resources from which the higher layer will select resources for SL-PRS transmission. To trigger this procedure, in slot n, UE has to do sensing in the sensing window, and further select the candidate resource for transmission in the selection window based on the sensing results. Sensing is based on reservation information contained in the corresponding SCI.

For dedicated resource pool for SL-PRS transmission, we think the basic procedure of Rel-16 resource (re-)selection should be applied with minor modifications:

(1) Reservation interval of SL-PRS: the determination of reservation interval can refer to the periodicities defined for DL-PRS. The periodicity of DL-PRS can be configured as one of {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240}. Similarly, if the periodicity of SL-PRS can also be configured up to 10240ms, the maximum reservation interval can be set to 10240ms.

(2) Sensing window: the sensing window size for dedicated SL-PRS resource pool is associated with the reservation interval of SL-PRS. Currently, resource reservation period of SL-data can be defined as either {ms0, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000} or {1..99ms}. The maximum sensing window size defined in SL communication pool is 100ms or 1100ms. Also, in order to make sure at least one occasion appears in a sensing window, sensing window size for SL-PRS should be equal to or larger than the reservation interval. For example, if the maximum reservation interval of SL-PRS is 10240ms, sensing window size can be set to 10240ms or 10300ms. Moreover, multiple sensing window configurations can be set.

(3) Selection window: the selection window size is related to the QoS requirement for SL-PRS.

(4) Resource exclusion mechanism: Resource(s) exclusion is based on resources overlapping with reserved resource(s) of other UE whose RSRP measurement is larger than a RSRP threshold. The RSRP threshold can be (pre-)configured in each dedicated resource pool. If the priority of SL-PRS is indicated in SCI, the threshold should be selected based on the priority prioi in the decoded SCI and the priority prioj in the SCI to be transmitted. A resource is excluded if it’s SL-PRS RSRP measurement is above the RSRP threshold. Moreover, for scheme 2 UE autonomous selection of SL-PRS resources, whether the UE performs resource exclusion or not is not only related to the SL-PRS RSRP measurement and Tx priority (both UEs), but also related to whether the two UEs have positioning signaling interaction. For example, as long as two UEs have no positioning signaling interaction, UEs with different comb offset may transmit SL-PRS resources in the same time (e.g. same slot). However, if two UEs have positioning signaling interaction, due to the half-duplex restriction, UEs with different comb offset cannot transmit SL-PRS resources in the same slot/time. As shown in Figure 2.4-1, suppose the SL-PRS comb offset of UE 1 is 0, if UE1 detects in the sensing window that UE 2 (whose SL-PRS comb offset is 3) have time domain resource conflict, UE 1 and UE 2 cannot support an FDM manner for SL-PRS transmission in the selection window if they have to measure SCI/SL-PRS from each other.

Figure 2.4-1: SL-PRS resources with different comb offset multiplexed in the same slot

Proposal 14: For Scheme 2, with regards to sensing-based resource selection for SL-PRS, sensing is based on SCI:

- For shared resource pool, Rel-16/17 resource (re)-selection procedure is reused for SL-PRS

- For dedicated resource pool, the basic procedure of Rel-16 resource (re-)selection should be applied with minor modifications:

  - Reservation interval of SL-PRS: the determination of reservation interval can refer to the periodicities defined for DL-PRS.

  - Sensing window: Sensing window size should be equal to or larger than the reservation interval.

  - Selection window: the selection window size is related to the Qos requirement for SL-PRS.

  - Resource exclusion mechanism: whether the UE performs resource exclusion or not is not only related to the SL-PRS RSRP measurement and Tx priority (both UEs), but also related to whether the two UEs have positioning signaling interaction

    - Resource(s) exclusion is based on resources overlapping with reserved resource(s) of other UE whose RSRP measurement is larger than a RSRP threshold.

    - FDMed PRS multiplexing between UEs having positioning serving interaction should NOT be supported.

### 2.4.2 Random resource selection

Agreement in RAN1#112 With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed Agreement in RAN1#112 Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

In NR SL communication, allowed resource selection mechanism(s), i.e. full sensing only, partial sensing only, random resource selection only, or any combination can be configured in a SL communication resource pool as shown below:

c1: only full sensing allowed c2: only partial sensing allowed c3: only random selection allowed c4: full sensing+random selection allowed c5: full sensing+ partial sensing allowed c6: partial sensing + random selection allowed c7: full sensing+ partial sensing + random selection allowed.

Similar to Rel-17 NR SL communication, we support that either /both sensing based or/and random selection in Scheme 2 is/are allowed by (pre-)configured per resource pool. Therefore:

Proposal 15: Confirm the following working assumption:

- Sensing-based and random selection can be allowed in the same resource pool

### 2.4.3 Congestion control

Proposal in RAN1#112 For Scheme 2 SL-PRS resource allocation, - specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. Study at least the following aspects: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges

For shared resource pool, the design for SL-PRS in shared resource pool should ensure backward compatibility with legacy Rel-16/17 UEs. For RSSI, either a new SL-PRS RSSI is introduced or existing SL RSSI is reused with slight modification:

Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH or additionally for SL-PRS, starting from the 2nd OFDM symbol.

The definition of CR and CBR should reuse the legacy definition. Even if PSSCH and SL-PRS are transmitted within a slot, the priority of SL-PRS and that of PSSCH can be different. In such case, the CR can be evaluated separately for SL-PRS and PSSCH.

Proposal 16: For congestion control mechanism in shared resource pool:

- Either a new SL-PRS RSSI is introduced or existing SL RSSI is reused with slight modification

- The definition of CR and CBR should reuse the legacy definition in Rel-16/17 SL communication.

  - CR can be evaluated separately for SL-PRS and PSSCH

For dedicated resource pool, SCI is used to reserve/trigger/indicate SL-PRS resource. There are relationships between SCI and its associated SL-PRS resource(s) (e.g. one SCI resource/config is associated with one SL-PRS resource). SL-PRS CR and SL-PRS CBR can be based on either PSCCH/SCI or SL-PRS, or both PSCCH/SCI and SL-PRS. For example, as shown in Figure 2.4-2, each SCI configuration is associated with a SL-PRS resource where different SL-PRS resources apply a TDM pattern in a slot. Suppose there is only 4 SCI configurations and associated 4 SL-PRS resources in a slot, CR and CBR can be either calculated based on how many SCI configuration are occupied/busy or how many SL-PRS resources are occupied/busy.

Figure 2.4-2 each SCI configuration is associated with a SL-PRS resource, different SL-PRS resources apply a TDM pattern in a slot

SL-PRS congestion control is meant to determine SL-PRS transmission parameter(s) according to the channel busyness and the priority of SL-PRS transmission. For example, both SL-PRS CR (channel occupancy ratio) and SL-PRS CBR are the essential parameters for congestion control, where SL-PRS CBR mainly defines the channel busy ratio (e.g. based on RSRP, RSSI) within a (pre-)configured time window and SL-PRS CR defines the channel occupancy ratio of one SL-PRS transmission. If a UE is configured with higher layer parameter indicating CRSL-PRS, Limit and transmits SL-PRS, the UE shall ensure the following limits for any priority value k of SL-PRS (the lower the priority value k, the higher the priority):

i≥kCRSL-PRSi≤CRSL-PRS, Limitk

Where CRSL-PRSi is the CR evaluated within a time window for SL-PRS transmission with priority i (the priority can be indicated in SCI), CRSL-PRS, Limitk corresponds to the higher layer parameter that is associated with the priority k and the CBR range which includes the SL-PRS CBR measured with another time window.

The definition of SL-PRS CBR and CR is associated with SL-PRS resource allocation granularity and the SL-PRS resource configuration in a SL-PRS resource pool. Specifically, if SL-PRS resource allocation time granularity is slot and frequency granularity is sub-channel, defining SL-PRS CR and SL-PRS CBR based on the occupancy/portion of subchannel over multiple slots is feasible. On the other hand, if SL-PRS resource allocation granularity is SL-PRS resource and sub-channel is not defined in dedicated SL-PRS resource pool, it is not reasonable to define SL-PRS CR or CBR based on sub-channel.

Moreover, the definition of CR and CBR is also related to the SL-PRS resource configuration in a SL-PRS resource pool (e.g. related to whether the bandwidth and comb pattern of SL-PRS resources in a resource pool is the same or different, whether TDM-based multiplexing or comb-based multiplexing is supported).

- For comb-based multiplexing, SL-PRS CR and SL-PRS CBR are associated with the number of occupied SL-PRS/PSCCH resource(s) and comb size. The channel capacity is closely related to SL-PRS comb size. For example, if the comb size is 4, comb-based multiplexing will allow at most 4 UE multiplexing (supposing the number of symbols for the 4 SL-PRS resources are the same). On the other hand, if comb size is 6, it is possible to allow 6 SL-PRS resources’ multiplexing.

- For TDM-based multiplexing, the definition and configuration of SL-PRS CR and SL-PRS CBR is associated with the number of occupied SL-PRS/PSCCH resource(s) and the number of symbols for each SL-PRS resource. The channel capacity is closely related to SL-PRS symbol number within a slot. For example, if the symbol number is 2, TDM-based multiplexing can allow 4 UE multiplexing (i.e. 4 SL-PRS resources transmitted in a slot). However, if symbol number is 8, it is only possible to allow 1 SL-PRS resource in a slot.

In the current specification for SL communication, sidelink congestion control is used in sidelink resource allocation mode 2. There are mapping relationship between PSSCH transmission parameter (such as MCS, PRB number, retransmission number, MaxTxPower, CR limit), CBR ranges and priority ranges. Similarly, congestion control mechanisms for SL-PRS can be studied after the design of SL-PRS resource allocation and configuration is determined. For example, during UE SL-PRS resource selection procedure, UE can measure SL-PRS CBR (definition can be further determined). Combining the CBR measurement result and the SL-PRS transmission priority, the corresponding transmission parameter for SL-PRS can be further determined. The determination of SL-PRS transmission parameters related to congestion control is associated with the definition of CR and CBR for SL positioning. For example, if the definition of CR, CBR for SL positioning is associated with the number of SL-PRS resources, the SL-PRS transmission parameters should include the indication of SL-PRS resource including the comb offset, number of symbols in a slot, etc.

Proposal 17: For congestion control mechanisms in dedicated resource pool:

- SL-PRS CR and SL-PRS CBR should be based on

  - Alt.1: PSCCH which is associated with SL-PRS transmission

  - Alt.2: SL-PRS

  - Alt.3: both SL-PRS and PSCCH which is associated with SL-PRS transmission

- The definition of SL-PRS CBR and CR is associated with SL-PRS resource allocation granularity and the SL-PRS resource configuration in a SL-PRS resource pool.

  - For comb-based multiplexing, SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and comb size.

  - For TDM-based multiplexing, the definition and configuration of SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and the number of symbols for each SL-PRS resource.

- There are mapping between congestion measurements, SL-PRS priority and SL-PRS transmission parameters.

### 2.4.4 Inter-UE coordination

Proposal in RAN1#112 For Scheme 2, with regards to Inter-UE coordination for SL-PRS, - consider the following options: - Option 1: Use Rel-17 Scheme 1 IUC procedure as a starting point - Option 2: Use Rel-17 Scheme 2 IUC procedure as a starting point - Option 3: Use both Rel-17 Scheme 1 or and 2 IUC procedure as a starting point - Note: Companies are encouraged to describe the applicability of their proposals with regards to dedicated and shared resource pool, and the changes that may be needed - In addition to the options above, - study whether also conflict indication for the already used SL PRS resource(s), or other SL PRS conflict avoidance mechanisms (e.g. SL PRS muting, comb offset coordination, and SL PRS sequence selection)

In SL communication, UE-A can determine (one or more) resource conflict (IUC scheme2) or determine whether a set of resources are recommended (IUC scheme1) for UE-B’s SL-PRS transmission. Then, the UE-B can take the SL-PRS IUC information for sidelink positioning into consideration before its transmission.

For UE autonomous SL-PRS resource allocation, inter-UE coordination (IUC) information transfer for sidelink positioning among a number of UEs may be beneficial for maximum resource utilization and minimum resource conflicts. As shown in Figure 2.4-3, UE-A can determine (one or more) resource conflict or determine whether a set of resources are recommended for UE-B’s SL-PRS transmission. Then, the UE-B can take the SL-PRS IUC information for sidelink positioning into consideration before its transmission. Basically, Rel-17 Scheme 1 and 2 IUC procedure can be used as a starting point.

Figure 2.4-3: An example procedure for SL-PRS IUC

Moreover, some companies mentioned that IUC frame can be used for allocating SL-PRS resources or delivering SL-PRS configurations/resource allocation among UEs. In other words, one UE may reserve the SL-PRS resource(s) for another UE. For example, UE-B can request UE-A to allocate or reserve SL-PRS resource(s), UE-A may further reserve or allocate corresponding SL-PRS resource(s) and deliver this information to UE-B. However, we do not prefer this design. Technically speaking it is hard for a UE to be known about all the resource transmission information and resource conflict information (in one positioning session). One UE can recommend some SL-PRS resource or SL-PRS configurations, but it is up to transmission UE (e.g. UE-B) itself to determine how to use this information (e.g. whether directly use the resource recommended by UE-A or combine the IUC with its sensing results). As shown in Figure 2.4-4, suppose UE-A transmits SL-PRS IUC to UE-B, it is possible that UE-A may not be aware of the interference caused by UE-C’s transmission.

Proposal 18: Support SL-PRS IUC to maximize resource utilization and minimize resource conflicts.

- Reuse the Rel-17 Scheme 1 and 2 IUC procedure

- Do not support using IUC scheme to allocate SL-PRS resource(s)

2.5 SL-PRS configuration/triggering/activation/reservation

### 2.5.1 SL-PRS triggering

With regards to the configuration/activation/deactivation/triggering/reservation of SL-PRS, the following agreements were achieved in the previous RAN1 meetings.

Agreement in RAN1#109-e With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement in RAN1#110 - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement. Agreement in RAN1#112 In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Regarding the triggering of SL-PRS, at least UE’s own higher layer can trigger SL-PRS transmission, in other words, at least Option 1 of the agreement in RAN1#112 should be supported.

As for Option 2, UE-A can request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A, after UE-B receives the request signaling, it is still up to UE-B’s own higher layers to trigger SL-PRS transmission (i.e. Option 1). The use case of Option 2 is SL-RTT positioning method in our understanding. But when the initiating UE transmit SL-PRS and associated SCI to the second UE, the SCI can directly include a single-sided RTT or double-sided RTT indicator instead of requesting for another UE’s SL-PRS transmission.

Therefore, we support the following proposal:

Proposal 19: In Scheme 2, with regards to the triggering of SL-PRS:

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes another UE requests UE to transmit SL-PRS and UE’s own higher layer then triggers the corresponding SL-PRS transmission.

### 2.5.2 Periodic, semi-persistent or aperiodic SL-PRS

Agreement in RAN1#112 For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering

The above agreement mainly tried to reuse the existing mechanism of SL communication, and then the design of the TDRA field and the Resource reservation period field can be reused. Hence, we suggest the maximum number of reservations within one periodic instance or for the case without periodic reservation is the same as Rel-16/17 SL communication, i.e. 3.

Proposal 20: For SL PRS transmissions without periodic reservation or within a periodic instance, support the same maximum number, i.e. 3 of reservations as the legacy SL communications.

2.6 SL-PRS cast type

Agreement in RAN1#110b-e From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one) - Broadcast (as a working assumption). - FFS: Applicability of the above cast types Agreement in RAN1#111 - Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work. Agreement in RAN1#112 “Cast type indicator”, “Source ID, “Destination ID” fields (similar to SL communication) should be are included in SCI to support unicast, groupcast and broadcast SL-PRS transmissions. - FFS: Further details

According to the discussion during RAN1#110b-e and RAN1#111, unicast, groupcast and broadcast of SL-PRS transmission are supported for SL-PRS transmission. Similar as the design in SL communication, we support all three cast types including unicast, groupcast and broadcast for SL positioning.

Unicast, which is a basic cast type for SL positioning. Two UEs can establish a one-to-one unicast connection with each other and the source ID and destination ID should be reported correspondingly. Unicast is more suitable for relative positioning, e.g., SL-RTT.

Groupcast, which is a more signaling-efficient way for SL positioning compared to unicast. For example, a target UE can groupcast its SL-PRS transmission to multiple anchor UEs within a group (e.g., SL-TDOA method). In this way, the target UE can avoid transmitting SL-PRS one by one and thus save overhead and increase efficiency.

Broadcast: More suitable for RSUs’ periodic SL-PRS transmission. RSUs can broadcast SL-PRS and other neighboring UEs can use the corresponding measurements to calculate its location.

For SL-PRS resource pool in Rel-18, SCI which can contain cast type and source/destination UE information is used to trigger/reserve SL-PRS resource. For example, if the cast type indicated in SCI is unicast, that means the SL-PRS reserved by this SCI is only expected to be decoded by a particular UE. The cast type can also be indicated as groupcast or broadcast, in such case, one source UE is associated with multiple destination UEs and multiple destination UEs is expected to correctly decode and measure the SL-PRS transmitted from the source UE.

For example, as shown in Figure 2.6-1, if SL-PRS is configured in a dedicated SL-PRS resource pool, if the cast type indicated in Tx UE’s SCI is unicast, Rx UE should determine if its destination ID is equal to Tx UE’s source ID, (and/or determine if its source ID is equal to Tx UE’s destination ID), if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI.

Figure 2.6-1: Filtering for unicast SL-PRS transmission

As shown in Figure 2.6-2, if SL-PRS is configured in a dedicated SL-PRS resource pool, if the cast type indicated in Tx UE’s SCI is groupcast or broadcast, Rx UE should determine if its destination ID is equal to Tx UE’s destination ID, if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI.

Figure 2.6-2: Filtering for groupcast/broadcast SL-PRS transmission

Proposal 21: Support using SCI to indicate the cast type of SL-PRS transmission:

  - Rx UE uses the cast type and corresponding source/destination ID filtering to decide whether to process the SL-PRS associated with Tx UE’s SCI.

Conclusion

In this contribution, we discuss potential solutions for SL positioning, and we have the following observations and proposals:

Observation 1: For dedicated resource pool, using reserved slots of SL communication resource pool can increase the latency of SL positioning.

Proposal 1: A UE can be (pre-)configured with one or more SL resource pools for SL-PRS transmission.

- The SL resource pools can be dedicated resource pool, shared resource pool.

Proposal 2: For a dedicated resource pool for SL positioning, in addition to SL-PRS, PSCCH which carries SCI associated with SL-PRS transmission(s) is included in the resource pool.

- PSSCH is not included in a dedicated resource pool

Proposal 3: For a dedicated resource pool for positioning, with regards to the bandwidth of SL-PRS transmission, support the following option:

- Option (2): SL PRS bandwidth is up to higher layer configuration in a resource pool.

  - Option (2a): The bandwidth of SL-PRS can be configured per dedicated resource pool. In other words, the bandwidth of SL-PRS is the same as that of the resource pool.

Proposal 4: For the support of dedicated SL-PRS resource pool:

- Pool level TDMed restriction by RRC signaling between SL-PRS resource pool and SL communication pool should be avoided.

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214)

  - The set of slots (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS) that may belong to a dedicated resource pool includes all the slots except NS-SSB slots, NnonSL slots and reserved slots.

  - According to (t0SL-PRS,t1SL-PRS,⋯,tTmax-1SL-PRS), the set of slots assigned to a dedicated resource pool is based on a bitmap configured in the resource pool.

Proposal 5: A dedicated SL-PRS resource pool should at least include the following configurations:

- SL-PRS resource pool level configuration: time resource of resource pool, frequency resource of resource pool

- SL-PRS configuration: the period of SL PRS resource/SL PRS resource set, number of symbols that a SL PRS occupied in a slot, start symbol of SL PRS in a slot, comb size and RE offset, priority of SL PRS resource/SL PRS resource set

- PSCCH configuration: candidates of time resource and frequency resource of PSCCH

- Synchronization configuration: indicates the allowed synchronization reference(s) which is (are) allowed to use the configured SL-PRS resource pool

- Resource allocation scheme 2 related configuration: CBR config and its association relationship with SL-PRS transmission parameter, SL-PRS resource reservation period, sensing window, selection window, SL-PRS RSRP threshold

- Power control configuration: maximum SL-PRS transmission power, alpha value and P0 value for either sidelink pathloss based power control or downlink pathloss based power control

Proposal 6: In order to avoid SL-PRS collision, support the mapping between each SCI and each SL-PRS configuration for a dedicated resource pool, where:

- Frequency resource occupied by SCI is limited to a range of bandwidth of dedicated resource pool and multiple SCIs can be multiplexed in FDM manner.

- The mapping relationship of “SCI location and SL-PRS configuration” can be configured in each dedicated resource pool.

  - SCI location refers to frequency range of SCI (e.g. starting PRB of SCI, PRB number of SCI)

  - SL-PRS configuration refers to comb offset or symbol offset.

Proposal 7: In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS:

- A new single-stage SCI in PSCCH is introduced

- Information carried in SCI for SL-PRS should at least include: SL-PRS priority, source ID, destination ID, cast type, resource reservation period, SL-PRS configuration information. Regarding the SL-PRS configuration information, down-select between the following two options:

  - SCI indicates SL-PRS resource ID: this field can implicitly indicate number of symbols, comb size and RE offset, symbol offset of SL-PRS, bandwidth, SL-PRS sequence ID

  - SCI indicates time and frequency resource assignment: apply similar design as the SCI 1-A in SL communication

Proposal 8: With regards to the granularity of time-domain resource allocation for SL-PRS transmission

- Support SL-PRS-resource-based allocation, i.e. M <=14 symbols with comb size N within a slot are configured for each SL-PRS resource

Proposal 9: In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot.

- With regards to the bandwidth of SL-PRS, down-select between (1) bandwidth is indicated in SCI; (2) bandwidth is up to higher layer configuration in a shared resource pool

- Only TDM between SL-PRS transmission and SL-data transmission is supported

Proposal 10: In a shared resource pool, in addition to SL PRS transmission, the UE transmits

  - Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication

    - Use one bit to indicate whether only SL-PRS is indicated or both SL-PRS and SL-data are transmitted

  - Opt. 2: SCI1-A & existing 2nd stage SCI format used for SL-PRS indication

    - Extra bits are needed to indicates: whether only SL-PRS, only SL-data, or both SL-PRS and SL-data are transmitted

  - At least the following information is transmitted by means of SCI: priority of SL-PRS, SL-PRS resource time and frequency indication (e.g. reservation period), source ID, destination ID, cast type indicator.

Proposal 11: In a shared resource pool, priority indicator of SL-PRS is indicated in SCI format 1-A from Tx UE to Rx UE

- Priority value of SL-PRS can be integer, non-integer, or discontinuous integer, or discontinuous decimal

Proposal 12: For scheme 1 SL-PRS resource allocation, support SL-PRS configuration transfer between LMF and gNB.

Proposal 13: Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from gNB through Dynamic grant, or through configured grant type 1/type 2

- Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format is needed. At lease the following should be added in either DCI format 3-0 or a new DCI format:

  - Resource pool index (either dedicated resource pool or shared resource pool for SL-PRS)

  - SL-PRS resource configuration index (SL-PRS resource ID or SL-PRS resource ID + SL-PRS resource set ID if resource set is introduced)

  - Time gap indicating the gap between DCI reception and the first SL-PRS transmission scheduled by DCI

  - Configuration index indicating which configured grant type 2 is activated by the DCI when UE is configured with multiple CG

Proposal 14: For Scheme 2, with regards to sensing-based resource selection for SL-PRS, sensing is based on SCI:

- For shared resource pool, Rel-16/17 resource (re)-selection procedure is reused for SL-PRS

- For dedicated resource pool, the basic procedure of Rel-16 resource (re-)selection should be applied with minor modifications:

  - Reservation interval of SL-PRS: the determination of reservation interval can refer to the periodicities defined for DL-PRS.

  - Sensing window: Sensing window size should be equal to or larger than the reservation interval.

  - Selection window: the selection window size is related to the Qos requirement for SL-PRS.

  - Resource exclusion mechanism: whether the UE performs resource exclusion or not is not only related to the SL-PRS RSRP measurement and Tx priority (both UEs), but also related to whether the two UEs have positioning signaling interaction

    - Resource(s) exclusion is based on resources overlapping with reserved resource(s) of other UE whose RSRP measurement is larger than a RSRP threshold.

    - FDMed PRS multiplexing between UEs having positioning serving interaction should NOT be supported.

Proposal 15: Confirm the following working assumption:

- Sensing-based and random selection can be allowed in the same resource pool

Proposal 16: For congestion control mechanism in shared resource pool:

- Either a new SL-PRS RSSI is introduced or existing SL RSSI is reused with slight modification

- The definition of CR and CBR should reuse the legacy definition in Rel-16/17 SL communication.

  - CR can be evaluated separately for SL-PRS and PSSCH

Proposal 17: For congestion control mechanisms in dedicated resource pool:

- SL-PRS CR and SL-PRS CBR should be based on

  - Alt.1: PSCCH which is associated with SL-PRS transmission

  - Alt.2: SL-PRS

  - Alt.3: both SL-PRS and PSCCH which is associated with SL-PRS transmission

- The definition of SL-PRS CBR and CR is associated with SL-PRS resource allocation granularity and the SL-PRS resource configuration in a SL-PRS resource pool.

  - For comb-based multiplexing, SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and comb size.

  - For TDM-based multiplexing, the definition and configuration of SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and the number of symbols for each SL-PRS resource.

- There are mapping between congestion measurements, SL-PRS priority and SL-PRS transmission parameters.

Proposal 18: Support SL-PRS IUC to maximize resource utilization and minimize resource conflicts.

- Reuse the Rel-17 Scheme 1 and 2 IUC procedure

- Do not support using IUC scheme to allocate SL-PRS resource(s)

Proposal 19: In Scheme 2, with regards to the triggering of SL-PRS:

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes another UE requests UE to transmit SL-PRS and UE’s own higher layer then triggers the corresponding SL-PRS transmission.

- Proposal 20: For SL PRS transmissions without periodic reservation or within a periodic instance, support the same maximum number, i.e. 3 of reservations as the legacy SL communications.

Proposal 21: Support using SCI to indicate the cast type of SL-PRS transmission:

  - Rx UE uses the cast type and corresponding source/destination ID filtering to decide whether to process the SL-PRS associated with Tx UE’s SCI.

Reference

- RP-223549 New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting #98-e.

- 3GPP TS 38.331 h40: "Radio Resource Control (RRC) protocol specification (Release 17)".

- 3GPP TS 38.214 h50: "NR; Physical layer procedures for data (Release 17)".

- R1-2303276 Discussion on SL positioning reference signal

- 3GPP TS 38.212 h50: “Multiplexing and channel coding (Release 17)”

- Chair's notes RAN1#112 eom4