3GPP TSG RAN WG1 #112bis-e                                            R1-2303279

e-meeting, April 17th - 26th, 2023

Source:	ZTE

Title:	Discussion on carrier phase positioning

Agenda item:	9.5.2

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN WG1#112 [1], the following agreements were made for carrier phase positioning.

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from or based on the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: - enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF. - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

In this contribution, we provide our views on carrier phase positioning techniques from RAN1 perspective.

Physical layer measurement and signalling for carrier phase positioning

Carrier phase measurement configuration and/or report

  - 2.1.1 Integer ambiguity

As approved in previous meeting (3GPP RAN Meeting #98-e) [2], the carrier phase positioning is supported in Rel-18 WID, for UE-assisted positioning, existing DL PRS is used for carrier phase measurements. For carrier phase positioning, an integer part N and a fractional part Φ of carrier phase is required to calculate the target UE’s location for carrier phase positioning. Wherein, the integer part search range can be deduced from the timing based positioning technology if carrier phase positioning is not a standalone positioning method, i.e., carrier phase positioning is executed together with other legacy positioning technologies (such as TDOA), and the fractional part Φ is the carrier phase measured by the receiver (e.g., in the time domain or frequency domain based on the existing PRS).

In our previous contributions [3,4], it is proved that the integer search procedure is time-consuming especially when the integer ambiguity exceeds a certain level, which heavily affects the positioning performance. Taking this into consideration, it would be beneficial if network suggests an initial location to determine the scope of integer N. For UE-based or NG-RAN node assisted positioning, if LMF can get UE’s rough location, a proper range of N values can be informed to UE, further improve the positioning performance.

For the integer ambiguity problem, the following agreement was achieved in 3GPP TSG RAN WG1#112[1]:

Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF. - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

For option 3 and option 4, we believe that option 4 is more feasible, the reasons are as follows. There is no strong demand for UE/TRP to report the estimated integer ambiguity in LMF-based NR carrier phase positioning, because LMF usually has more location information or measurement results than UE/TRP. LMF holds all the measurement results that UE/TRP reported, not only from the target UE, and LMF can estimate/calculate a more reasonable integer ambiguity for the target UE.

Therefore, option 4 is our first preference. If there are other benefits proposed concerning UE/TRP report the estimated integer ambiguity, option 3 can also be supported as a compromise. Hence, we have the following proposal.

Proposal 1: For signalling concerning the integer ambiguity, at least support LMF providing the range of integer to UE for UE-based NR carrier phase positioning.

  - 2.1.2 Carrier phase report

For NG-RAN node-assisted or UE-assisted carrier phase positioning, TRP/UE should measure and report the carrier phase based on the received SRS/PRS, to get the fraction part of the carrier phase. As aforementioned, the existing positioning method, e.g. TDOA can help UE or LMF determine the integer part search range with affordable complexity, therefore, carrier phase measurement and report should be performed together with existing positioning method for UE-assisted and NG-RAN node assisted positioning.

For the carrier phase report problem, the following agreement was achieved in 3GPP TSG RAN WG1#112[1]:

Agreement For NR carrier phase positioning, at least support the following approach: - enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements

Among the four listed legacy positioning methods, it is feasible for UE/TRP to report the carrier phase measurements together with the measurements of RSTD and TDOA. Detailed reasons are listed as follows:

Lack of evaluation results on RTT: The simulation results provided in the proposals (submitted by different companies) during the SID stage concentrate more on RSTD and RTOA. RTT requires both the transmission of SRS and reception of PRS from UE perspective, some of the possible error sources are different from that of RSTD or RTOA. Therefore, it’s difficult to infer the positioning accuracy of RTT from existing simulation results. Extensive simulation results should be provided if there is strong interest on the support of RTT.

Indelible possible error sources of RTT: Similar as TDOA positioning procedure, a UE has to receive DL PRS and transmit UL SRS. The errors (include timing error, ARP error and phase error, etc) in DL PRS reception/UL SRS transmission process should be evaluated first, to prove that these errors can be reduced or eliminated with the help of PRU, especially if simultaneous measurements/transmissions for PRU and UE is enabled. However, the UE/gNB Rx-Tx phase difference error cannot be eliminated with current techniques. The performance of combining RTT and carrier phase positioning is unknown.

Hence, we have the following proposal:

Proposal 2: Support UE/TRP reporting carrier phase measurements together with the RSTD and/or RTOA measurements to LMF.

In general, there might be multi-PRS transmitted between the same TRP and UE pair with same or different PFL/carriers in a positioning process, and the measured carrier phases of these different (sub-)carriers might be different. The number of reported PRS carrier phase measurement, physical channel environment and PRS propagation multi-path could affect the accuracy of carrier phase evaluation. Taking these factors into consideration,  UE can report the uncertainty of phase measurement and further provides the corresponding phase measurement quality or phase measurement confidence, which is similar to the existing timing measurement quality or angle measurement quality. Hence, we have the following proposal:

Proposal 3: Support UE/TRP reporting the carrier phase measurement quality together with the phase measurement result.

Multi-path for carrier phase positioning

In last meeting, the following proposal was discussed but no consensus has reached:

Select one of the following options for NR carrier phase positioning: - Option 1: Report the carrier phase of the first path only - Option 2: Report the carrier phase of the first path, and optionally, the additional paths

As shown in Figure 1, the measured carrier phase on receiver side is related to the transmission path of the SRS/PRS. Compared to NLOS path, LOS path can reflect the distance relationship between the receiver and the transmitter more accurately. On the contrary, if UE/TRP measures the carrier phase of the received signal that is transmitted with NLOS path, the position of the target UE cannot be estimated with the measured carrier phase.

Figure 1. Simple example for LOS/NOS propagation path

Furthermore, reporting the carrier phase of the additional path will definitely increase the UE’s measurement and report load, and even we get the phase of additional path, the benefits of this report are unknown for carrier phase positioning. Hence, we have the following proposal:

Proposal 4: Support UE/TRP reporting the carrier phase of the first path only at least for Rel-18.

Enhancement on PRU assisted carrier phase positioning

  - 2.3.1 Multiple sub-carrier phase measurement

Our previous simulations show that there are multiple error sources, include ARP error, initial phase error, etc., may deteriorate the carrier phase positioning performance. To reduce the side effects of these errors on positioning performance, PRUs can help positioning system calibrate the carrier phase related errors. For a TRP, if the ARP or phase error corresponding to the same beam set remains unchanged, the explicit ARP error or carrier phase error can be evaluated with the help of one or more PRUs. A possible error calibration flow is shown in Figure 2.

Figure 2. ARP or carrier phase error calibration diagram

The ARP or carrier phase error calibration process is as follows (take ARP error as an example):

STEP 1: PRU/PRS selection

LMF should select several PRU(s) to help UE calibrate ARP error, and the criterion for PRU selection may include the following requirements:

- The selected PRU should be able to receive the same PRS as that of the target UE.

- The selected PRU should be in the LOS direction with the corresponding TRPs.

As shown in Figure 3, there are several PRUs located around a UE, and a TRP is transmitting multiple PRS during the positioning process. In this case, PRU 3 and PRU 4 are selected as reference PRUs to calibrate the error sources for the target UE. PRU 3, PRU 4 and target UE share the same ARP error in this deployment setting.

Figure 3. PRU selection example

STEP 2: Measurement report

PRU(s) measure and report the carrier phase of the received PRS.

STEP 3: Error calculation

After receiving the carrier phase reported by PRU, LMF can calculate the expected error source as follows:

Assume that the antenna location of TRP is (x,y), ARP error is (ex,ey), the actual ARP location is (X,Y)= (x+ex, y+ey). The following calculation describes the error estimation detail.

Suppose the coordinates of PRU 3 and PRU 4 are (x3,y3), (x4,y4) respectively. The PRS carrier phase measured by PRU 3 and PRU 4 are φ3, φ4 respectively. The actual distance between TRP and PRU 3 is:

D3=(X−x3)2+(Y−y3)2=λ(N3+φ3/2π)

The actual distance between TRP and PRU 4 is:

D4=(X−x4)2+(Y−y4)2=λ(N4+φ4/2π)

where λ is the wavelength of measured PRS, N3 and N4 are the integer parts of PRU 3 and PRU 4, combining the above formula, the actual TRP antenna location (X,Y) can be calculated. The estimated ARP error (ex,ey) can be calculated as follows:

ex=X−x, ey=Y−y.

STEP 4: Error delivery

After error calculation, LMF can deliver the estimated error to UE for further location calculation. In this way, UE can get more precise positioning result while calibrating the ARP error. The carrier phase error can also be calculated in the same way. Hence, we have the following proposal.

Proposal 5: For DL UE-based carrier phase positioning, the ARP or carrier phase error of the TRP can be delivered by LMF.

In PRU-assisted carrier phase positioning procedure, PRU knows the exact location where it camps on. Assume that the coordinates of TRP and PRU are (xT,yT), (xP,yP) respectively, and the PRS carrier phase measured by PRU is φp, the carrier phase error is Δφ, the distance between TRP and PRU is

D=(xT−xP)2+(yT−yP)2=λ[Np+(φp+Δφ)/2π]

Substituting the known coordinates and measured carrier phase to the above equation, the carrier phase error can be calculated. And the estimated carrier phase error can be reported by PRU to LMF.

Proposal 6: For UE-based carrier phase positioning, the carrier phase error can be reported by PRU to LMF.

  - 2.3.2 PRU carrier phase measurement report

In our previous contributions [3,4], it is proved that single/dual differential technique with PRU can improve the positioning accuracy in carrier phase positioning. And previous discussion in RAN1 meetings demonstrated that the use of PRU in carrier phase positioning is beneficial for both UE-based and UE-assisted NR carrier phase positioning. If PRU is adopted in a positioning process, PRU should report the carrier phase measurement results to LMF. But for UE-based carrier phase positioning, all of the measurement results should be collected by UE. Therefore, LMF can forward the PRU carrier phase measurement results to UE for further location estimation. Hence, we have the following proposal:

Proposal 7: For UE-based carrier phase positioning, support LMF forwarding the carrier phase measurement results of the PRU to the target UE.

  - 2.3.3 Enhancement on PRU procedures (regarding the LS from SA2)

In last meeting, SA2 sent an LS [4] concerning the potential enhancement on PRU as follows:

SA2 would like to seek RAN1 and RAN2’s feedback on such assumption and whether additional enhancement would be needed to support the simultaneous measurements for the target UE and PRU. SA2 would update the specification according to the RAN1 and RAN2 feedbacks if necessary. Additionally, SA2 also would like to know what location information can be provided by PRU, and whether RAN1 and RAN2 see benefit to allow PRU to provide its location information together with the location measurements reports.

And the following conclusion was made towards this LS:

Conclusion Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report.

In carrier phase positioning, it is beneficial for UE and PRU measure/transmit PRS/SRS simultaneously. The main benefits are listed as follows:

Faster positioning response time: Single/dual differential technique with PRU can improve the positioning accuracy in carrier phase positioning. For DL positioning method, this differential operation requires the measurement results both on UE and PRU’s carrier phase of the received PRS. And for UL positioning method, differential operations requires the TRP measures the SRS transmitted by UE and PRU. If UE and PRU measure PRS or transmit SRS at different time slots, the carrier phase differential operator have to wait until all the required measurement results are reported or collected. On the contrary, if simultaneous measurements/transmissions for multiple UEs is supported, LMF can calculate the UE’s location once upon receiving the measurement report of UE, PRU and TRP in LMF-based positioning method. For UL positioning, gNB can measure and report the carrier phase as soon as receiving the SRS transmitted by UE and PRU.

Higher positioning accuracy: It is more likely that two PRS/SRS carrier phase measurements will have the same time and frequency resources and similar channel environments if UE and PRU can transmit/measure the positioning related signals simultaneously, further improve the practicability of carrier phase differential operation among different PRS/SRS measurement results. If UE and PRU can measure the same PRS instance, the relative timing error, phase error, and measurement error of these two received signals can be minimized, thereby improving positioning accuracy.

Hence, we have the following observation:

Observation 1: Supporting a mechanism to ensure simultaneous measurements/transmissions for a target UE and a PRU is beneficial for improving the performance of carrier phase positioning.

It is also observed that simultaneous measurements/transmissions for a target UE and a PRU is beneficial for improving the performance of timing based positioning methods. But the current RAN1 specifications do not support such a mechanism, so there should be some enhancements to allow the UE and PRU measure/transmit positioning related signals to enable double differential operations. Towards this requirement, we have the following potential enhancements:

  - 2.3.3.1 DL enhancement on PRU procedure

LMF can configure the same measurement time slot(s)/window(s) for PRU and UE, which indicates when the PRS should be measured. If the PRS configuration details is transmitted in LPP from LMF to PRU and UE, the two PRS receivers will know the exact PRS transmission occasions. Furthermore, if the same measurement time slots/windows is configured for PRU and UE, these two terminals can measure the PRS simultaneously as configured.

In current specifications, PRU and UE can specify a PRS resource by identifying the exact PRS resource ID, but PRS is usually transmitted periodically, UE cannot specify the exact PRS instance with the configured information. If LMF can configure the same PRS instance to be measured, PRU and UE can measure the same PRS instance(s), further minimizing the relative timing error, phase error, etc., on carrier phase positioning.

Hence, we have the following proposal:

Proposal 8: For supporting a mechanism to ensure simultaneous measurements of DL PRS for a target UE and a PRU, the following options can be supported:

-LMF configures the same measurement time slot(s)/window(s) for PRU and UE.

-LMF configures the same PRS instance(s) to be measured by PRU and UE.

Note that the number of configured time slots/windows or PRS instances should take Nsample into account, which indicates the number of measurement samples.

  - 2.3.3.2 UL enhancement on PRU procedure

For UL enhancement on PRU procedures, on one hand, gNB can configure the same slot offset(s) towards PRU and UE for SRS transmission. In this way, PRU and UE can transmit the SRS for positioning as configured for periodic or semi-persistent SRS. On the other hand, gNB can trigger the positioning SRS transmission for PRU and UE through other downlink signalling. PRU and UE can start the SRS for positioning transmission once upon receive the trigger information. It is up to gNB implementation to ensure the simultaneous SRS triggering/activation from PRU and a target UE. No spec impact is expected in our view.

The key point is how to let gNB know in which slot(s) SRS from both PRU and the target UE should be transmitted. Hence, NRPPa signaling should be enhanced accordingly. For instance, LMF can recommend/request a list of slots or slot offsets or even time windows for potential SRS transmission, where the recommended/requested list of slots/slot offsets/windows can be the same for PRU and the target UE to suggest the simultaneous SRS triggering/activation/transmission.

Hence, we have the following proposal:

Proposal 9: For supporting a mechanism to ensure simultaneous transmissions of UL SRS for a target UE and a PRU, LMF can request a list of slots or windows for SRS transmission.

Multiple sub-carriers phase measurement

As described in our previous contributions [3,4], the carrier phase estimation error, PCO, ARP error, etc., may affect the carrier phase positioning accuracy. Single carrier phase measurement may result in some accidental errors/positioning error, further deteriorate the positioning performance. The simulation results in our previous contributions [3,4] show that the carrier phase from multiple sub-carriers in one carrier/PFL can improve performance compared with TDOA concerning the initial random phase. And dual differential carrier phase can mitigate the positioning side effects brought by ARP error. All of these verified that multiple phase measurement is beneficial for improving the positioning accuracy.

From another perspective, in carrier phase positioning procedure, each sub-carrier has its own carrier phase (Φ, fractional part). The carrier phase of the center sub-carrier could represent the carrier phase of the whole carrier/PFL. To support multiple sub-carrier phase measurement, a positioning carrier/PFL can be divided into several segments, and each segment could have its own carrier phase. Through the theoretical analysis in [4], it can be observed that the carrier phase error from two frequency segments could be removed. To realize carrier phase positioning with the help of multiple frequency segments, the center frequency and corresponding phase of different carrier segments need to be reported. Hence, we have the following proposal.

Proposal 10: Support measuring and reporting carrier phase of multiple sub-carriers/segments in one carrier/PFL.

- center frequency or λ should be informed for each selected sub-carrier/segment.

Regarding the multiple sub-carriers phase measurement problem, the following agreement was achieved:

Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF. - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

For multiple sub-carriers phase measurement, in option 1, there is an FFS concerning the details of reporting for multiple sub-carrier phase measurement. As shown in Figure 4, a PFL/carrier contains multiple sub-carriers, and UE may report the carrier phase of several sub-carriers in one measurement report.

Figure 4. UE reports the carrier phase of multiple sub-carriers

In Figure 4, N indicates the number of sub-carriers to be reported in each carrier phase measurement result, and M represents the number of carrier phase reports in each PFL. LMF can configure the requirement M or N for multiple sub-carriers phase measurement. In this way, UE can execute carrier phase measurement and report procedure as configured by LMF.

Hence, we have the following proposal:

Proposal 11: If a UE/TRP is supported to report the carrier phase measurements of more than one frequencies within a PFL/carrier, the following enhancement can be supported:

-LMF configures the number of PRBs/sub-carriers for each carrier phase measurement report.

-LMF configures the number of carrier phase measurement reports for each PFL/carrier.

Phase Error Group

A gNB (or TRP) or UE may be equipped with multiple antennas (including transmission antenna, reception antenna). Different antennas may have different phase error when performing carrier phase measurement. As the following figure, it is appropriate to measure carrier phase with the antenna(s) within a phase error group (PEG) which is similar as TEG defined in Rel-17. More specifically, one or more antennas within a phase error margin can be grouped into a PEG.

With the help of PEG, UE(for UE-based positioning method) or LMF(for UE-assisted/NG-RAN node-assisted positioning method) can choose appropriate antenna/PEG for location estimation with the measured and/or reported carrier phase. The phase error can be minimized, and the positioning accuracy can be definitely improved.

Hence, we have the following proposal.

Proposal 12: Phase Error Group(PEG) should be defined for carrier phase measurement, where phase measurement errors in a PEG is within a margin value.

Figure 5. Conception for PEG

Conclusion

In this contribution, we provide our views on carrier phase measurement based positioning techniques, observation and proposals are listed as follows:

Observation 1: Supporting a mechanism to ensure simultaneous measurements/transmissions for a target UE and a PRU is beneficial for improving the performance of carrier phase positioning.

Proposal 1: For signalling concerning the integer ambiguity, at least support LMF providing the range of integer to UE for UE-based NR carrier phase positioning.

Proposal 2: Support UE/TRP reporting carrier phase measurements together with the RSTD and/or RTOA measurements to LMF.

Proposal 3: Support UE/TRP reporting the carrier phase measurement quality together with the phase measurement result.

Proposal 4: Support UE/TRP reporting the carrier phase of the first path only at least for Rel-18.

Proposal 5: For DL UE-based carrier phase positioning, the ARP or carrier phase error of the TRP can be delivered by LMF.

Proposal 6: For UE-based carrier phase positioning, the carrier phase error can be reported by PRU to LMF.

Proposal 7: For UE-based carrier phase positioning, support LMF forwarding the carrier phase measurement results of the PRU to the target UE.

Proposal 8: For supporting a mechanism to ensure simultaneous measurements of DL PRS for a target UE and a PRU, the following options can be supported:

-LMF configures the same measurement time slot(s)/window(s) for PRU and UE.

-LMF configures the same PRS instance(s) to be measured by PRU and UE.

Proposal 9: For supporting a mechanism to ensure simultaneous transmissions of UL SRS for a target UE and a PRU, LMF can request a list of slots or windows for SRS transmission.

Proposal 10: Support measuring and reporting carrier phase of multiple sub-carriers/segments in one carrier/PFL.

- center frequency or λ should be informed for each selected sub-carrier/segment.

Proposal 11: If a UE/TRP is supported to report the carrier phase measurements of more than one frequencies within a PFL/carrier, the following enhancement can be supported:

-LMF configures the number of PRBs/sub-carriers for each carrier phase measurement report.

-LMF configures the number of carrier phase measurement reports for each PFL/carrier.

Proposal 12: Phase Error Group(PEG) should be defined for carrier phase measurement, where phase measurement errors in a PEG is within a margin value.

Reference

- Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0 (Athens, Greece, 27th February – 3rd March 2023).

- RP-223549, New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting #98-e.

- R1-2209215, Discussion on carrier phase measurement based positioning, ZTE, 3GPP RAN1#110b-e.

- R1-2212520, Discussion on carrier phase measurement based positioning, ZTE, 3GPP RAN1#111.

- R1-2301915, LS on PRU procedures, A2 #155, (Athens, Greece, 27th February – 3rd March 2023).