3GPP TSG RAN WG1 #112bis-e                                              R1-2303281

e-Meeting, April 17th – April 26th, 2023

Source:          ZTE

Title:            Discussion on BW aggregation for positioning

Agenda item:     9.5.4

Document for:	Discussion and Decision

- Introduction

In Rel-17, the technique of PRS/SRS bandwidth aggregation was evaluated and the performance gain was proved [1]. However, due to lack of feasibility check from RAN4, it was not specified in Rel-17 eventually.

In SI of NR Rel-18 positioning, RAN4 studied bandwidth aggregation for positioning measurement. The relevant conclusions are captured in the TR 38.859 [2].

In RAN#98e meeting, a WID on Expanded and Improved NR Positioning was approved for Rel-18 where one item is to specify bandwidth aggregation techniques [3][4].

In RAN1#112 meeting, the good progress was achieved for the topic of PRS/SRS bandwidth aggregation.

In this contribution, we provide our further views on the signaling and procedures to specify the aggregation of PRS/SRS across PFLs/carriers.

- PRS BW aggregation

  - Common transmission properties

For DL PRS bandwidth aggregation, as RAN4 has concluded that PRS from multiple PFLs (positioning frequency layers) should be transmitted in the same time from the same antenna, and the numerology of PFLs should be the same.

From PRS configuration signaling perspective, LMF should explicitly inform UE which two or three PFLs are linked for aggregation as shown in Figure 2.1-1, where the PRS resource and resources sets between PFL#i and PFL#j can be one-to-one mapping/linked for bandwidth aggregation.

Figure 2.1-1 One-to-one mapping for PRS resources between two PFLs

The following agreement has been agreed in RAN1#112 meeting which will enable this feature.

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs

Since the same antenna has been agreed for linked PRS resources across the aggregated PFLs, the condition of ‘The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin’ is not needed anymore from our view in order to avoid unnecessary discussion. Because TEG is defined per band, the same TEG and TEG margin value should be automatically applicable for the aggregated PRS resources across the two or three intra-band contiguous carriers.

For periodicity and slot offset, we believe the same values should be used. Otherwise, the following situation may be happened if the different periodicities are used for the two PFLs. In the case when M=4 measurement samples are used for PRS measurement, it is unclear how UE can get single timing measurement result based on such PRS configuration where only partial of M=4 PRS instances can be aggregated.

Figure 2.1-2 Different periodicities for the PRS resource sets across two PFLs

Similar as the periodicity and slot offset, we propose to use the same muting pattern to simplify the implementation for this feature.

Also, for the number of PRS resource sets and resources, we think the same numbers should be used across the aggregated PFLs. If the number of resources for the linked PRS resource sets are different as shown in the left side of Figure 2.1-3 where some resources are linked, but some other resources only exist in PFL#j, it is unclear what the use case or benefit is to support this scenario. In FR2, one resource usually represents one beam, it is also unclear how to fairly compare the beam quality if only partial beams are based on aggregated resources.

Figure 2.1-3 Different number of PRS resources or resource sets across two PFLs

With regard to NR-DL-PRS-SFN0-Offset, we think the same value may not be needed as SFN offset may not be necessarily the same between different PFLs.

As for how to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones, e.g, PFLs with different RE-offset configurations or PFLs with different point A, we think it is up to network implementation to ensure the uniform and contiguous PRS RE distribution.

Proposal 1: To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be further satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs

- The same periodicity and slot offset

- The same muting pattern

- The same number of PRS resource sets

- The same number of PRS resources

  - The PRS resources between the aggregated PFLs are one-to-one linked

  - Link configuration

The following agreement has been agreed in RAN1#112 meeting.

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis.

To inform UE which PFLs are linked for potential PRS bandwidth aggregation measurement, the existing assistance data should be enhanced as shown in the above agreement.

Since all TRPs/gNBs may not be upgraded simultaneously in the feature deployment, it is better to have flexibility configuring PFL aggregation only for a part of TRPs.

To keep both specification and implementation simple, we suggest per TRP basis for the link signaling between PFLs. We haven’t found the good use cases and benefit of per resource set or resource basis.

Proposal 2: For PRS bandwidth aggregation across PFLs, the signaling to link the PFLs is per TRP basis.

  - Measurement report

In Rel-16/17, for each measurement element of the current measurement report for DL-TDOA and multi-RTT measurement at UE side, i.e. in the first measurement element or an additional measurement element, UE needs to report TRP ID, RSTD or UE Rx-Tx time difference, PRS resource ID, PRS resource set ID, etc. The measurement should be based on a single PFL as UE can only process PRS of multiple PFLs in TDMed manner.

In Rel-18, when UE is configured with BW aggregated PRS via LPP assistance data, the following two options should be determined

- Option 1: UE measures the RSTD or UE Rx-Tx time difference always based on the aggregated PFLs.

  - In such case, the existing measurement element report signaling can be completely reused. The reported PRS resource set ID implies that UE measures PRS based on both the reported PRS resource set and the linked PRS resource set(s) of the same TRP, where the link association of aggregated PRS resources is configured by PRS assistance date.

- Option 2: UE has flexibility to report RSTD or UE Rx-Tx time difference based on either single PFL or aggregated PFLs

  - In such case, the existing measurement element is not enough. UE needs to newly inform LMF whether a measurement element is based on a single PFL or aggregated PFLs. If three aggregated PFLs are configured, UE needs to further inform LMF whether/which one or two or three PFLs are used for measurement and report.

Figure 2.3-1 One-to-one mapping for PRS resources between two PFLs

As shown in Figure 2.3-1, option 1 is simple without new signaling needed, but it is less flexible. For example, UE needs to switch into power saving mode for more important purpose. As long as single PFL measurement can satisfy the location requirement sometimes, option 2 provides more freedom for UE. Hence, we support the second bullet of the following agreement which was made in RAN1#112 meeting.

Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced

In addition, besides the joint RSTD and UE Rx-Tx time difference report, we believe the joint RSRP and RSRPP should be supported as well for TDOA and Mult-RTT positioning methods. In the case that UE uses single IFFT operation to measure the aggregated PRS resources, the joint RSRP and RSRPP can be calculated based on the equivalently larger bandwidth PRS. That is, the aggregated PRS is jointly operated as a whole.

In the case that PRS assistance data includes the link signaling for PFL aggregation, it has implied that UE may need to perform joint measurement across aggregated PFLs. Hence, there is no need to further use location information request message to indicate aggregation-based measurement.

Proposal 3: Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods

- In a measurement report element, single RSRP and RSRPP are reported for the PRS resources across aggregated PFLs

- In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement

- PRS assistance data with the link signaling for PFL aggregation implies that UE can perform joint measurement across aggregated PFLs

  - MG and PPW

In Rel-16, PRS measurement was only supported in MG (measurement gap).

In Rel-17, PPW (PRS processing window) was introduced to support PRS processing within active BWP in the condition that PRS numerology is aligned with active BWP and PRS bandwidth is within the active BWP. UE can measure PRS within the activated or configured PPW. Specifically, for each BWP, at most four PPWs can be configured by RRC signaling, and MAC CE can be used to active one for a BWP. Totally, at most four PPWs can be activated corresponding to four PFLs. Because Rel-17 UE cannot process PRS from multiple PFLs simultaneously, the multiple activated PPWs should be TDMed. As shown in the following Figure 2.4-1, single MAC CE can be used to activate two TDMed PPWs, where the UE only measure PRS in PFL1 within PPW1 and measures PRS in PFL2 within PPW2. In other words, the UE does not measure PRS in PFL2 within PPW1, and not measure PRS in PFL1 within PPW2 as well. Figure 2.4-2 illustrates the existing Rel-17 MAC CE structure.

Figure 2.4-1 MACCE activates TDMed PPWs In Rel-17

Figure 2.4-2 Rel-17 MACCE structure

In Rel-18, when UE is configured with bandwidth aggregated PRS via LPP assistance data, single MG can still be shared for the PRS in the aggregated PFLs since MG is configured for FR1 or FR2 or for UE. Therefore, there is no need to enhance the exiting Rel-17 specification.

For PPW, we think single PPW can be shared for the PRS in the aggregated PFLs to make PRS aggregation feasible and reduce MAC CE overhead. That is, PPW1 or PPW2 in Figure 2.4-1 can be shared for PRS reception in both PFL1 and PFL2. Then, UE can measure PRS in the aggregated PFLs simultaneously as shown in Figure 2.4-3. In the case of small gap between two active BWPs, the benefit in terms of location accuracy from the PRS aggregation in PPW is the same as that in MG, but the measurement latency is reduced.

Figure 2.4-3 Single shared PPW for two PFLs within two active BWPs

Proposal 4: Both MG and PPW are supported for PRS bandwidth aggregation measurement

- A shared MG or PPW is used for the aggregated PFLs.

Furthermore, the following issues should be discussed when PFLi and PFLj are linked via PRS configuration in assistance data.

- Question 1: whether the activated PPW should be always shared. In other words, it should be discussed if it is allowed that MAC CE can activate PPW(s) for single PFL measurement, i.e. Rel-17 behavior

  - From our view, it is beneficial for gNB to control PPW activation and select either Rel-18 PRS aggregation measurement or Rel-17 single PFL measurement, e.g. for power saving purpose, gNB can control UE sticking to Rel-17 UE behavior with single PFL PRS measurement, and for higher positioning accuracy purpose, gNB can control UE measuring PRS in the aggregated PFLs.

- Question 2: whether UE should always measure PRS based on the aggregated PFLs in the case the shared PPW is activated. In other words, it should be discussed if the UE can be flexible to select PRS BW aggregation measurement or single PFL measurement.

  - If UE has more flexibility to select the measurement scheme, the UE can control more depending on the real demand, e.g. for power saving or for higher location accuracy

Proposal 5: When UE is configured with BW aggregated PRS via LPP assistance data and PPW(s) is configured by gNB, the following options should be determined

- Option 1: It is only allowed that a shared PPW is configured/activated for PRS BW aggregation measurement

- Option 2: Either a shared PPW for PRS BW aggregation measurement or non-shared PPW(s) for single PFL measurement can be configured/activated

Proposal 6: When UE is configured with BW aggregated PRS via LPP assistance data and a shared PPW is activated, the following options should be determined

- Option 1: UE always measures the RSTD or UE Rx-Tx time difference based on the aggregated PFL in the shared PPW

- Option 2: UE has flexibility to measure RSTD or UE Rx-Tx time difference based on either single PFL or aggregated PFL in the shared PPW

  - On-demand PRS

For UE initiated on-demand PRS, Rel-17 supports preconfigured on-demand PRS where maximum 8 PRS configurations can be configured by LMF to UE as shown in the following box abstracted from 37.355. Then UE can further request the preferred configuration ID(s).

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ... } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... }

Alternatively, as shown in the following box abstracted from 37.355, UE can directly request the preferred PRS parameters for each PFL including the preferred PRS bandwidth, repetition factor, comb size, etc.

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-On-Demand-DL-PRS-PerFreqLayer-r17 NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE { dl-prs-FrequencyRangeReq-r17 ENUMERATED { fr1, fr2, ...}, dl-prs-ResourceSetPeriodicityReq-r17 ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40, p64, p80, p160, p320, p640, p1280, p2560, p5120, p10240, p20480, p40960, p81920, ...} OPTIONAL, dl-prs-ResourceBandwidthReq-r17 INTEGER (1..63) OPTIONAL, dl-prs-ResourceRepetitionFactorReq-r17 ENUMERATED {n2, n4, n6, n8, n16, n32, ...} OPTIONAL, dl-prs-NumSymbolsReq-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-CombSizeN-Req-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-QCL-InformationReqTRPlist-r17 DL-PRS-QCL-InformationReqTRPlist-r17 OPTIONAL, ... } dl-prs-ResourceBandwidthReq This field specifies the requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

In Rel-18, when PRS bandwidth aggregation is supported from network side, the following agreement has been made for on-demand PRS.

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

Based on the agreement, in the case there is preconfigured on-demand PRS, LMF should inform UE whether/which PFLs are linked/aggregated. Since one On-Demand-DL-PRS-Configuration is associated with one PFL, that is, LMF should inform UE whether/which On-Demand-DL-PRS-Configuration IDs are linked/aggregated.

As shown Step 1 in Figure 2.5-1, the grouping information should be informed to UE together with the Rel-17 on-demand PRS configurations. In the figure, it is assumed four on-demand PRS configurations are divided into two groups where configuration#1 and #2 are linked, and #3 and #4 are linked for aggregation.

In step 2, after UE receives the on-demand PRS configurations and the grouping information, UE can request the preferred on-demand PRS configuration ID(s) if the UE does not request PRS aggregation measurement, or can request the preferred group ID(s) if the UE needs PRS aggregation measurement for higher positioning accuracy.

Alternatively, in step 2, UE can directly request the preferred PRS bandwidth value regardless of on-demand PRS configuration where the case of the requested bandwidth larger than the maximum value of the single PFL, e.g. >272 PRBs implies that UE requests PRS bandwidth aggregation measurement for higher positioning accuracy.

Figure 2.5-1 On-demand PRS for PRS bandwidth aggregation

For LMF-initiated on-demand PRS, it only supports LMF directly requests preferred PRS parameters including bandwidth. Similar as Option 2 in Figure 2.5-1, Rel-18 can be enhanced to support that the requested bandwidth is larger than the maximum value of the single PFL, which implies that LMF requests PRS bandwidth aggregation.

Since the on-demand PRS signaling design heavily needs RAN2 and RAN3 work, we propose to send LS to RAN2 and RAN3.

Proposal 7: For UE-initiated on-demand PRS request for PRS bandwidth aggregation, support

- On-demand PRS configuration can include the group information where the linked on-demand PRS configuration IDs are in the same group for aggregation

- UE can request on-demand PRS configuration ID(s) or group ID(s)

- UE can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN2

Proposal 8: For LMF-initiated on-demand PRS request for PRS bandwidth aggregation, support

- LMF can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN3

- SRS BW aggregation

  - Common transmission properties

Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers

Similar as DL PRS, we think the SRS transmission in the aggregated carriers should be configured with the same periodicity, slot offset, the same number of SRS resource sets and resources.

As for how to maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones, we think it is up to network implementation for SRS resource allocation to ensure the uniform and contiguous PRS RE distribution.

As for how to ensure the same Tx PSD, i.e. power per subcarrier, we think the same pathloss RS, Po and alpha is necessary. It is hard to guarantee the same Tx PSD if the pathloss is different between two carriers because it is up to UE to calculate the path loss in the air and estimate the transmission power to compensate this part of loss. To ensure the same power in the case that pathloss is different, different Po/alpha should be allocated as well. However, pathloss may be dynamically changed, but Po/alpha is semi-statically configured.

Proposal 9: To enable SRS bandwidth aggregation, the following conditions should be further satisfied for the aggregated SRS resources across the aggregated carriers

- The same periodicityAndOffset, and slotOffset

- The same number of SRS resource sets

- The same number of SRS resources

  - The SRS resources between the aggregated carriers are one-to-one linked

- The same pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier)

  - Link configuration

Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked - SRS resources are per BWP per carrier configuration - FFS whether the link is per SRS resource set basis or per SRS resource basis.

To inform UE which carriers are linked for potential SRS bandwidth aggregation measurement, the existing RRC configuration should be enhanced as shown in the above agreement.

To keep both specification and implementation simple, we suggest per carrier basis for the link signaling. That is, a higher layer signaling is used to indicate which serving cells are linked, and then the SRS resources for positioning are one-to-one mapping between the linked carriers. So far, we haven’t found the good use cases and benefit of per resource set or resource basis.

Proposal 10: For SRS bandwidth aggregation across carriers, support higher layer signaling to indicate which carriers are linked, where the SRS resources for positioning between the linked carriers are one-to-one mapping

  - Measurement report

In Rel-16/17, gNBs should report TRP measurement results via NRPPa signaling in 38.455 as shown in the following box, where each TRP measurement result may be reported with cell ID and SRS resource ID together.

- 9.1.4.4 MEASUREMENT REPORT This message is sent by the NG-RAN node to report positioning measurements for the target UE. Direction: NG-RAN node LMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3; YES; reject. NRPPa Transaction ID; M; 9.2.4; -. LMF Measurement ID; M; INTEGER (1..65536, …); YES; reject. RAN Measurement ID; M; INTEGER (1..65536, …); YES; reject. TRP Measurement Response List; 1; YES; reject. >TRP Measurement Response Item; 1..<maxnoofMeasTRPs>; EACH; reject. >>TRP ID; M; 9.2.24; -. >>TRP Measurement Result; M; 9.2.37; -. >>Cell ID; O; NR CGI 9.2.9; The Cell ID of the TRP identified by the TRP ID IE.; YES; ignore. - 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>. >CHOICE Measured Results Value; M. >>UL Angle of Arrival; M; 9.2.38; -. >>UL SRS-RSRP; M; INTEGER (0..126); -. >>UL RTOA; M; 9.2.39; -. >>gNB Rx-Tx Time Difference; M; 9.2.40; -. >>Z-AoA; M; 9.2.67; YES; reject. >>Multiple UL-AoA; M; 9.2.71; YES; reject. >>UL SRS-RSRPP; M; 9.2.72; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore. - 9.2.73 SRS Resource type This IE contains the SRS resource type. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. CHOICE Reference Signal; M. >SRS. >>SRS Resource ID; M; INTEGER(0..63). >Positioning SRS. >>Positioning SRS Resource ID; M; INTEGER(0..63). SRS Port Index; O; ENUMERATED(id1000, id1001, id1002, id1003, …); This IE may be present if the SRS Resource ID IE is present, and is ignored otherwise.; YES; ignore.

In Rel-18 for the measurement report of multi-carrier SRS aggregation, the measurement results for a TRP including {UL TROA, UL SRS-RSRP/RSRPP, NB Rx-Tx Time Difference, Measurement Quality, Time Stamp, ARP ID, LoS/NLoS Information} should be based on the aggregated SRS in the multiple carriers. From specification perspective, those IEs in the existing 38.455 can be reused. To let LMF be clearer on SRS configuration, it is better to further report whether the measurement is based on SRS carrier aggregation, if yes, then further report on which carriers SRS are aggregated. For example, in some cases, SRS in one of aggregated carriers collides with other higher priority signaling, e.g. MsgA, gNB may only get positioning measurement results based on single carrier SRS transmission as shown in the left one of Figure 3.3-1.

In the case three carriers are configured for aggregation and SRS from one carrier is dropped as shown in the right one of Figure 3.3-1, TRPs may only get positioning measurement results based on two aggregated carriers.

Hence, we think the second bullet in the following agreement made in RAN1#112 meeting should be confirmed.

Figure 3.3-1 TRPs may only report measurement result based on a part of aggregated carriers

Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs

Moreover, besides the joint UL RTOA and gNB Rx-Tx time difference report, we believe the joint RSRP and RSRPP should be supported as well for UL-TDOA and Multi-RTT positioning methods. In the case that gNB uses single IFFT operation to measure the aggregated SRS resources, the joint RSRP and RSRPP can be calculated based on the equivalently larger bandwidth SRS. That is, the aggregated SRS is jointly operated as a whole.

Proposal 11: Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods

- Single joint RSRP or joint RSRPP is reported for the SRS resources across aggregated carriers

- SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement

  - SRS transmission Type

The following agreement has been agreed in RAN1#112 meeting.

Agreement At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation - Support single MAC CE activating positioning SRS resource sets across the linked carriers - FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused - FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way

    - RRC connected mode

In Rel-16/17, all types SRS for positioning including aperiodic, semi-persistent and periodic SRS are supported for UEs in RRC connected mode. In Rel-18, we suggest to stick to Rel-17 positioning and CA principle.

Proposal 12: For SRS bandwidth aggregation, besides semi-persistent and periodic SRS for positioning, support aperiodic SRS for positioning for UE in RRC connected mode.

For UEs in RRC connected mode, SRS is configured per BWP based on the existing Rel-15/16/17 specifications. In Rel-18, for UE supporting SRS BW aggregation, gNB can configure the association between the linked carriers for SRS for positioning by RRC signaling. This is very like the feature of per CC group update TCI state or spatial relation which was introduced in Rel-16 MIMO as in the following box.

- 38.321 - 6.1.3.29 Serving Cell Set based SRS Spatial Relation Indication MAC CE The Serving Cell Set based SRS Spatial Relation indication MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields: - SRS Resource's Cell ID: This field indicates the identity of the Serving Cell, which contains the indicated SP/AP SRS Resource. If the C field is set to 0, this field also indicates the identity of the Serving Cell which contains all resources indicated by the Resource IDi fields. The length of the field is 5 bits. The indicated Serving Cell is configured as part of a simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2 as specified in TS 38.331 [5], and this MAC CE applies to all the Serving Cells configured in the set simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2, respectively;

For example, one or more carrier lists for SRS BW aggregation can be configured by RRC signaling, where the linked SRS in different carriers should be transmitted in the same slot and the same symbol with the same numerology for SRS BW aggregation measurement.

For periodic SRS for positioning, the linkage between carriers configured by RRC is sufficient. After RRC configuration, SRS in different linked carriers are transmitted periodically in the same symbols. Then, TRPs can do SRS measurement and report based on the aggregated SRS transmission.

For semi-persistent SRS, MAC CE is used to activate/deactivate a positioning SRS resource set in a carrier in Rel-16/17 as shown in the following figure, where spatial relation can be also updated for each SRS resource in the resource set. The serving cell ID, i.e. carrier ID and the SRS resource set ID is carried in the first and second octets respectively. For Rel-18 SRS aggregation, we think the SRS resource sets with the same set ID in the linked carriers can be triggered by single MAC CE.

Figure 3.4-1: Rel-16/17 SP Positioning SRS Activation/Deactivation MAC CE

However, it should be discussed whether SRS resource sets in the RRC linked carriers should be always activated/deactivate together. Two options can be considered

- Option 1: When a carrier i is linked with a carrier j by RRC signaling for positioning SRS BW aggregation, a MAC CE activating/deactivating an SRS resource set with ID m in carrier i should also always activate/deactivate the SRS resource set with ID m in the carrier j.

- Option 2: When a carrier i is linked with a carrier j by RRC signaling for positioning SRS BW aggregation, a MAC CE activating/deactivating an SRS resource set with ID m in carrier i can either activate/deactivate the SRS resource set with ID m in carrier i (Rel-16/17 behavior) or activate/deactivate the SRS resource sets with ID m in both carrier i and carrier j (Rel-18 aggregation).

Figure 3.1-2: Rel-18 SP Positioning SRS Activation/Deactivation MAC CE for SRS BW aggregation

Option 1 does not need MAC CE enhancement but has less flexibility. In the case that the traffic load, the number of camped UEs, or the slot formats are different between linked carriers, it may not be easy to always guarantee simultaneous SRS resource transmission in the multiple linked carriers. For example, compared with carrier i, more UEs with more urgent UL traffic are camping in the carrier j, and then UL resource may be limited in carrier j. So gNB may only activate SRS transmission in carrier i rather than in both linked carriers for a UE without urgent UL traffic.

Compared with Option 1, Option 2 provides more dynamic selection since MAC CE can be used to further determine how many carriers for positioning SRS transmission should be activated.

Proposal 13: Support single MAC CE to activate/deactivate semi-persistent positioning SRS resource sets in the two or three linked carriers. When a carrier i is lined with a carrier j by RRC signaling, select one of the two options:

- Option 1: a MAC CE activating/deactivating an SRS resource set with ID m in carrier i should always activate/deactivate the SRS resource sets with ID m in both carrier i and carrier j.

- Option 2: a MAC CE activating/deactivating an SRS resource set with ID m in carrier i can either activate/deactivate the SRS resource set with ID m in carrier i or activate/deactivate the SRS resource sets with ID m in both carrier i and carrier j

For aperiodic SRS, similarly as semi-persistent SRS, it is natural to support single DCI triggering SRS resource sets in the linked carriers together for the sake of low DCI overhead. The mechanism can reuse the achievement in the discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI in agenda 9.9 of RAN1#111 meeting. In that agenda, it has already concluded that a single DCI can trigger SRS resource sets in multiple cells. The relevant agreements are listed below:

Agreement The types for below fields in DCI format 1_X are listed (R1-2212924): Field; Type; Details (for information only). HARQ process number; Type 2; Details in Section 7.1.1. MCS; Alt 1: Type 2 (without compression); Details in Section 7.1.2. BWP indicator; Type 1A; Details in Section 7.1.3. FDRA; Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2; Details in Section 7.1.4. VRB-to-PRB mapping; Type 1A; Details in Section 7.1.5. PRB bundling size indicator; Type 1A; Details in Section 7.1.6. Rate matching indicator; Type 1B (up to 4 bits); Details in Section 7.1.7. ZP CSI-RS trigger; Type 1B (up to 3 bits); Details in Section 7.1.8. Antenna port(s); Configurable between Type 1A and Type 2; Details in Section 7.1.9. TCI; Type 1B (up to 4 bits); Details in Section 7.1.10. DMRS sequence initialization; Type 1A; Details in Section 7.1.11. SRS request; Type 1B (up to 4 bits); Details in Section 7.1.12. SRS offset indicator; Type 1B (up to 3 bits); Details in Section 7.1.13. Agreement - The types for below fields in DCI format 0_X are listed: Field; Type; Details (for information only). HARQ process number; Type 2; Details in Section 7.2.1. MCS; Alt 1: Type 2 (without compression); Details in Section 7.2.2. BWP indicator; Type 1A; Details in Section 7.2.3. FDRA; Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2; Details in Section 7.2.4. Frequency hopping flag; Type 1A; Details in Section 7.2.5. TPC command for scheduled PUSCH; Type 2; Details in Section 7.2.6. Open-loop power control parameter set indication; Type 1A; Details in Section 7.2.7. Antenna port(s); Configurable between Type 1A and Type-2; Details in Section 7.2.8. Precoding information and number of layers; Configurable between Type 1A and Type-2; Details in Section 7.2.9. PTRS-DMRS association; Type 2; Details in Section 7.2.10. DMRS sequence initialization; Type 1A; Details in Section 7.2.11. SRS request; Type 1B (up to 4 bits); Details in Section 7.2.12. SRS resource indicator; Configurable between Type 1A and Type-2; Details in Section 7.2.13. SRS offset indicator; Type 1B (up to 3 bits); Details in Section 7.2.14. UL/SUL indicator; FFS; Details in Section 7.2.15. Where type 1B refers to the following: - Type 1B - Field size is dependent on number of rows in the configured table which contains combinations of indication for each cell in the set configured for the DCI format 0_X - FFS: maximum number of rows - Indicated bit is interpreted as pointing one row in the configured table

As shown in the above agreements made in multi-cell PDSCH/PUSCH with a single DCI agenda, positioning SRS in two or three intra-band contiguous carriers can be triggered by the SRS request field in a single DCI 0_X or 1_X.

Proposal 14: Support single DCI triggering aperiodic positioning SRS in two or three contiguous carriers for SRS aggregation.

- Try to reuse the achievement in the agenda of multi-cell PDSCH/PUSCH with a single DCI

    - RRC inactive mode

In Rel-17, semi-persistent and periodic SRS for positioning are supported for UE in RRC inactive mode. The positioning SRS configured by RRCRelease signaling, and the details are shown in following according to 38.331, where SRS configuration for NUL or SUL can be within or outside the initial BWP. It is noted that the SRS is assumed within the same serving cell of the initial BWP.

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-changeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

For Rel-18, to support SRS carrier aggregation, an additional SRS configuration in a serving cell other than the one of initial BWP should be introduced and informed by RRCRelease signaling. Because intra-band contiguous carrier aggregation is only supported among NUL carriers, the newly introduced SRS configuration only for NUL is sufficient. One example for new RRC IE is shown as follows.

SRS-PosRRC-InactiveConfig-r18 ::=       {

AdditionalServingcell_PosSRS-r18_list  SEQUENCE (SIZE (1..2)) OF AdditionalServingcell_PosSRS-r18

}

AdditionalServingcell_PosSRS-r18 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17            BWP                                                                 OPTIONAL,    -- Need S

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

}

Proposal 15: To support two or three carrier aggregation of positioning SRS transmission for UE in RRC inactive mode, introduce one or two NUL carriers with respective SRS configuration, where the newly introduced carrier(s) and the carrier of the initial BWP are intra-band NUL contiguous carriers.

  - Relationship with communication CA

In RAN1#112 meeting, the following agreement was made for further study.

Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

For the first bullet, we think it is better not to completely decouple the UE capabilities between UL communication CA and SRS bandwidth aggregation. Usually, UL communication CA capability also reflects whether UE can do simultaneous transmission in the two carriers. For positioning SRS bandwidth aggregation, the requirement should be stricter, such as the same antenna, the same symbol for SRS transmission aggregation. Hence, we think the feature of SRS bandwidth aggregation within two or three contiguous CC can still be based on the existing UE capability reporting granularity, e.g. per band, or per Feature set (i.e. per band per band combination).

For the second bullet, we support SRS outside BWP. That is, SRS can be configured outside the configured BWP, but the bandwidth should still be limited within the carrier of each BWP.

Proposal 16: Support UE capability on positioning SRS bandwidth aggregation per band or per band per band combination.

Proposal 17: Support to configure positioning SRS outside the corresponding BWP, but still within the corresponding carrier

  - Prioritization for power reduction

The following agreement has been agreed in RAN1#112 meeting.

Agreement Study potential power control enhancement of simultaneous transmission of SRS for SRS bandwidth aggregation especially in the case when the total uplink transmission power across multiple carriers exceeds P_c,max

In the current 38.213, if total UE transmit power in a transmission occasion i for operation with carrier aggregation exceed Pcmax(i), the UE prioritizes power allocation for the transmissions on the primary cell, or on the carrier with PUCCH configuration.

For the Rel-18 SRS bandwidth/carrier aggregation for positioning, if the same power prioritization rule is followed, the SRS transmission power over the aggregated carriers may not be the same, and the aggregation measurement at gNB side will be infeasible in this power limited case.

Hence, we suggest the same UE power prioritization for the positioning SRS transmission between the aggregated carriers regardless of priority cell or PUCCH.

Proposal 18: Support the same power prioritization for a positioning SRS transmission occasion between the aggregated carriers in the case when total UE transmit power in a transmission occasion i exceeds Pcmax(i).

- Conclusion

In this contribution, we provide our views on positioning across multiple frequency layers (carriers) techniques as the following.

Proposal 1: To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be further satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs

- The same periodicity and slot offset

- The same muting pattern

- The same number of PRS resource sets

- The same number of PRS resources

  - The PRS resources between the aggregated PFLs are one-to-one linked

Proposal 2: For PRS bandwidth aggregation across PFLs, the signaling to link the PFLs is per TRP basis.

Proposal 3: Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods

- In a measurement report element, single RSRP and RSRPP are reported for the PRS resources across aggregated PFLs

- In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement

- PRS assistance data with the link signaling for PFL aggregation implies that UE can perform joint measurement across aggregated PFLs

Proposal 4: Both MG and PPW are supported for PRS bandwidth aggregation measurement

- A shared MG or PPW is used for the aggregated PFLs.

Proposal 5: When UE is configured with BW aggregated PRS via LPP assistance data and PPW(s) is configured by gNB, the following options should be determined

- Option 1: It is only allowed that a shared PPW is configured/activated for PRS BW aggregation measurement

- Option 2: Either a shared PPW for PRS BW aggregation measurement or non-shared PPW(s) for single PFL measurement can be configured/activated

Proposal 6: When UE is configured with BW aggregated PRS via LPP assistance data and a shared PPW is activated, the following options should be determined

- Option 1: UE always measures the RSTD or UE Rx-Tx time difference based on the aggregated PFL in the shared PPW

- Option 2: UE has flexibility to measure RSTD or UE Rx-Tx time difference based on either single PFL or aggregated PFL in the shared PPW

Proposal 7: For UE-initiated on-demand PRS request for PRS bandwidth aggregation, support

- On-demand PRS configuration can include the group information where the linked on-demand PRS configuration IDs are in the same group for aggregation

- UE can request on-demand PRS configuration ID(s) or group ID(s)

- UE can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN2

Proposal 8: For LMF-initiated on-demand PRS request for PRS bandwidth aggregation, support

- LMF can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN3

Proposal 9: To enable SRS bandwidth aggregation, the following conditions should be further satisfied for the aggregated SRS resources across the aggregated carriers

- The same periodicityAndOffset, and slotOffset

- The same number of SRS resource sets

- The same number of SRS resources

  - The SRS resources between the aggregated carriers are one-to-one linked

- The same pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier)

Proposal 10: For SRS bandwidth aggregation across carriers, support higher layer signaling to indicate which carriers are linked, where the SRS resources for positioning between the linked carriers are one-to-one mapping

Proposal 11: Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods

- Single joint RSRP or joint RSRPP is reported for the SRS resources across aggregated carriers

- SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement

Proposal 12: For SRS bandwidth aggregation, besides semi-persistent and periodic SRS for positioning, support aperiodic SRS for positioning for UE in RRC connected mode.

Proposal 13: Support single MAC CE to activate/deactivate semi-persistent positioning SRS resource sets in the two or three linked carriers. When a carrier i is lined with a carrier j by RRC signaling, select one of the two options:

- Option 1: a MAC CE activating/deactivating an SRS resource set with ID m in carrier i should always activate/deactivate the SRS resource sets with ID m in both carrier i and carrier j.

- Option 2: a MAC CE activating/deactivating an SRS resource set with ID m in carrier i can either activate/deactivate the SRS resource set with ID m in carrier i or activate/deactivate the SRS resource sets with ID m in both carrier i and carrier j

Proposal 14: Support single DCI triggering aperiodic positioning SRS in two or three contiguous carriers for SRS aggregation.

- Try to reuse the achievement in the agenda of multi-cell PDSCH/PUSCH with a single DCI

Proposal 15: To support two or three carrier aggregation of positioning SRS transmission for UE in RRC inactive mode, introduce one or two NUL carriers with respective SRS configuration, where the newly introduced carrier(s) and the carrier of the initial BWP are intra-band NUL contiguous carriers.

Proposal 16: Support UE capability on positioning SRS bandwidth aggregation per band or per band per band combination.

Proposal 17: Support to configure positioning SRS outside the corresponding BWP, but still within the corresponding carrier

Proposal 18: Support the same power prioritization for a positioning SRS transmission occasion between the aggregated carriers in the case when total UE transmit power in a transmission occasion i exceeds Pcmax(i).

- Reference

- 38.857 Study on NR Positioning Enhancements (Release 17) V17.0.0

- 38.859 Study on expanded and improved NR positioning (Release 18) V18.0.0

- RP-223549, New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson

- RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corporation