3GPP TSG RAN WG1 #112bis-e                                    R1-2303282

e-Meeting, April 17th – April 26th, 2023

Source:         ZTE

Title:            Discussion on Positioning for RedCap UEs

Agenda item:    9.5.5

Document for:	 Discussion and Decision

Introduction

In 3GPP RAN1#112 meeting, some agreements were reached [1] as the following.

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

In this contribution, we provide our solutions for positioning for RedCap UEs considering PRS/SRS configuration, frequency overlapping between two hops, measurement / report, etc.

DL-PRS Rx hopping

In 3GPP RAN1#112 meeting, some agreements on DL-PRS Rx hopping were reached [1] as the following. We will discuss the issues with FFS.

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW

2.1 On UE processing capabilities

For UE processing capabilities, the number of hops and total bandwidth after hopping reception (or, equivalently, a capability of data buffering) should be considered.

The configuration for number of frequency hops shall consider both UE complexity and the corresponding positioning performance requirement. Basically, enlarging the number of frequency hops would improve positioning accuracy and bring more performance gain due to larger effective bandwidth. The simulation results for FR1 in InF-SH scenario for evaluating the impact of frequency hop number are shown in the following Figure 1. The evaluation assumptions are shown in the Appendix of [5].

Figure 1 Positioning accuracy for RedCap UE under different number of hops

According to the agreements made during SI phase, for IIoT use cases, the horizontal positioning accuracy requirement for 90% of UE is less than 1m. Based on our evaluation results, when the number of hops is equal to 4 or 5, the positioning accuracy in FR1 for a RedCap UE with a bandwidth of 20MHz is 0.88 or 0.65 m respectively for 90% UEs which can satisfy the target positioning requirements.

For UE supporting frequency hopping, UE needs to report the maximum number of supported frequency hops. For each hop, the supported maximum PRS bandwidth can refer to the existing UE capability. For instance, if UE reports that the supported maximum PRS bandwidth is 20MHz in FR 1, and UE reports the supported maximum number of frequency hops is 4 or 5, then network may configure at least 4 frequency hops to achieve measurement results which is equivalent to the result based on about 80MHz PRS bandwidth.

In addition, for phase rotation estimation between adjacent hops, UE may need to report whether it supports overlapping between adjacent hops. Alternatively, UE can report its capability on the maximum equivalent bandwidth after combing all hops.

Proposal 1: With regards to frequency hopping for positioning for RedCap UE, the number of hops should be configured by the network:

- UE reports a capability on the maximum number of supporting frequency hops to network, the candidates are {2, 3, 4, 5}

- UE reports a capability on the overlapping PRB(s) between adjacent hops or a capability on the maximum equivalent bandwidth after combing all hops

2.2 On single or multiple instances of MGs

According to measurement gap (MG) configuration for positioning in Rel-16/17, inside one measurement gap instance for DL-PRS processing, UE is not expected to process other DL channels or signals. In such case, it is straightforward to support UE to measure and process DL-PRS resources of multiple frequency hops during a configured measurement gap as shown in Figure 2.

Figure 2 DL-PRS frequency hopping inside MG

Furthermore, according to [2] as the following, a measurement gap (MG) can be configured within a long duration, e.g. 3ms, 6ms, 10ms and 20ms. For a frequency hopping with 5 hops and 20MHz for each, single MG duration is long enough for PRS measurement with all hops even considering Rel-18 RedCap UEs with 5MHz bandwidth BWP.  If multiple hops are measured in different MG occasions, the gap between hops may rely on the value of configured periodicity of MG which is too long to guarantee the performance.

TS38.331: PosGapConfig-r17 ::= SEQUENCE { ... mgl-r17 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

Hence, we have the following proposal.

Proposal 2: To ensure positioning performance, a single MG instance is used for PRS measurement of all hops.

2.3 On PPW

In case when data traffic load is low, UE can do PRS frequency hopping measurement within one PPW instance, which is similar to the operation in MG. Alternatively, a MAC CE from gNB to UE can be used to activate/deactivate a PPW with PPW ID=i associated with hop ID i and another a PPW with PPW ID=j associated with hop ID j. The association can be configured by RRC signalling. For example, suppose PPW 1 is associated with hop 1, PPW 2 is associated with hop 2, PPW 3 is associated with hop 3, then at least one of the following parameters for PPW configurations of PPW 1/2/3 should be the same: PPW periodicity, the length of PPW, the priority between PDCCH/PDSCH/CSI-RS and DL-PRS, PPW type. However, because a PPW is configured under a BWP, multiple PPW will involve BWP switching which requires a long switching delay. Hence, we have the following proposal.

Proposal 3: De-prioritize PPW for RedCap UE with PRS frequency hopping measurement.

2.4 Measurement and report with hopping

For DL-PRS frequency hopping, we prefer using the legacy mechanism without hopping transmission (i.e., with hopping reception). The transmitted DL-PRS is workable for both RedCap UEs and eMBB UEs. A RedCap UE can receive the corresponding DL-PRS in a frequency hopping manner as shown in Figure 3. If so, the spec impact will be relatively small. Moreover, even though DL-PRS frequency hopping is configured to UE for the sake of equivalent large bandwidth PRS transmission/reception/measurement, sometimes a UE does not have to combine the measurement from multiple frequency hops. For example, in the scenarios when PRS configuration is broadcast for all UEs in a serving cell but the UE’s location accuracy requirement is not high. In such case, UE should report hop information for the positioning measurement results, e.g., which frequency range corresponding hop(s) is/are used for the measurement.

Figure 3 DL-PRS frequency hopping reception

Proposal 4: For measurement report considering DL Rx frequency hopping, UE should report the corresponding hop indication for each measurement result, where the hop indication includes the frequency range.

In TS38.214, some parameters (such as the inter-slot repetition factor) for PRS reception are specified as the following.

- dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor.

For PRS reception with hopping, we suggest one or more of the following parameters to be introduced to facilitate UE’s PRS measurement.

      - Number of hops. This parameter will affect positioning accuracy. More hops will compose a larger bandwidth which brings a higher positioning accuracy.

      - Number of overlapped RB. This parameter is used for phase adjustment between adjacent hops. It will affect positioning accuracy.

      - Total bandwidth of all hops. Actually, this is equivalent to the number of hops (e.g., each hop has the same bandwidth). One of these two parameters can be finally selected.

Hence, we have the following proposal.

Proposal 5: For PRS reception with hopping, one or more of the following parameters in assistance data should also be introduced to facilitate UE’s PRS measurement.

      - Number of hops

      - Number of overlapped RB between hops

      - Total bandwidth of all hops

2.5 On-demanded PRS

In TS 38.305 [4], the on-demanded PRS is described as: on-Demand PRS transmission procedure allows the LMF to control and decide whether PRS is transmitted or not and to change the characteristics of an ongoing PRS transmission. The on-demand PRS transmission procedure can be initiated either by the UE or LMF. The actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated. In short, a UE (or LMF) can request to LMF (or TRP) for a “better” configuration of PRS. For example, a UE can request a larger bandwidth of PRS for higher positioning accuracy.

For a RedCap UE, it can request a larger bandwidth than that UE supports, this implies UE is requesting PRS hopping for the equivalently larger PRS bandwidth. Hence, we have the following proposal.

Proposal 6: For on-demand PRS, a RedCap UE can request a larger bandwidth that the UE supports, which implies PRS frequency hopping measurement is requested.

UL-SRS Tx hopping

In 3GPP RAN1#112 meeting, some agreements on UL-SRS Tx hopping were reached [1] as the following. We will discuss the SRS configuration related processing.

Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration FFS: hopping is configured within a SRS resource or across SRS resources

3.1 SRS configuration for hopping

For positioning SRS hopping for RedCap UE, there are two candidate solutions as shown in the FFS part in the above agreement:

- Frequency hopping within one resource

In this way, the frequency domain resource of one reference signal is divided into several parts, each part corresponds to a frequency hop, and several hops are received in different symbols with a combination as a whole (as shown in Figure 4). In other words, the three hopping parts refer to different symbols of one SRS resource.

Figure 4 Frequency hopping within one resource

- Frequency hopping within one resource set (across resources)

In this way, assuming one reference signal resource set contains several reference signal resources, then different reference signal resources may have different frequency range and correspond to different hops (as shown in Figure 5). Similarly, the received signals from different resources are concatenated for positioning related processing.

Figure 5 Frequency hopping across resources

In our view, both solutions are workable to support SRS frequency hopping for positioning. However, solution 1, i.e. SRS hopping within one resource may have less flexibility since the current SRS configuration for each resource is only limited to one slot, and then the SRS hopping is limited to intra-slot hopping case only. So we slightly prefer solution 2, i.e. frequency hopping across SRS resources.

Proposal 7: For RedCap UE, support frequency hopping across SRS resources.

3.2 Overlapping PRB

In the existing Rel-15/16/17, PT-RS is used to adjust phase rotation between OFDM symbols caused by phase noise in FR2 or Doppler shift. The motivation is the same as the design of overlapping PRBs between hops for RedCap UE for Rel-18 positioning here.

According to TS 38.214 [3] as the following, the density of PT-RS is related to the bandwidth allocated to data channel for good balance between phase tracking and RS overhead. Similarly, for RedCaP UE positioning with frequency hopping, the overlapped frequency resource should be related to the bandwidth of each hop. For example, the overlapped PRB for a RedCap UE with 20MHz bandwidth for each hop should be less than that for a RedCap UE with 40MHz bandwidth for each hop. Hence, we have the following proposal.

| TS38.214: |  |
| --- | --- |
| Scheduled bandwidth | Frequency density () |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

Proposal 8: For RedCap UE with positioning SRS, introduce partial overlapping in frequency for adjacent hops.

- The number of overlapping PRBs in adjacent hops is based on the bandwidth of each hop.

Moreover, in some cases (e.g. FR2 with phase noise impact or when UE has high mobility), using staggered pattern in partial overlapping part may not work well. In one of overlapping PRBs, PRS may only be present in some symbols especially in the case with large comb size, and then the UE cannot estimate phase rotation of adjacent OFDM symbols in one subcarrier.

As shown in the following figure where two symbol gap is assumed and the existing staggered pattern with comb size 4 is used in the overlapping PRB between two hops, phase rotation cannot be effectively estimated between symbol 4 and 7 as PT-RS did. That’s because the reference signal is not mapped on the same subbcarrier between symbol 4 and 7.

Figure 6 The staggered pattern used in the overlapping PRB

Therefore, an alternative solution is to apply non-staggered pattern in the overlapping part. Specifically, in overlapping PRB(s), PRS is transmit in all symbols, i.e. comb size 1. And then Receiver side can use the overlapping part to estimate accurate phase rotation.

Proposal 9: In the overlapping PRB(s), support SRS transmission with comb size 1 for effective estimation of phase rotation.

3.2 Measurement and report for hopping

In some cases, one or more hops may be dropped (e.g., because of collision with some other higher priority signals). For these cases, a combination of hop IDs (e.g., {{1}, {1,2}, {1,2,3}}) should be attached in a measurement result to indicate LMF on which hop(s) the measurement result is/are based.

Further in some scenarios, one or more hops may have low channel quality (e.g., low SINR). The measurement result(s) on this/these hop(s) may not be reliable. That is, a concatenation of one/some hop(s) might not be helpful. Hence, we have the following proposal.

Proposal 10: Support that TRPs can report the corresponding hop indication for each measurement result.

Conclusion

In this contribution, we provide our solutions on positioning support for RedCap UE techniques as the following proposals.

Proposal 1: With regards to frequency hopping for positioning for RedCap UE, the number of hops should be configured by the network:

- UE reports a capability on the maximum number of supporting frequency hops to network, the candidates are {2, 3, 4, 5}

- UE reports a capability on the overlapping PRB(s) between adjacent hops or a capability on the maximum equivalent bandwidth after combing all hops

Proposal 2: To ensure positioning performance, a single MG instance is used for PRS measurement of all hops.

Proposal 3: De-prioritize PPW for RedCap UE with PRS frequency hopping measurement.

Proposal 4: For measurement report considering DL frequency hopping, UE should report the corresponding hop indication for each measurement result, where the hop indication includes the frequency range.

Proposal 5: For PRS reception with hopping, one or more of the following parameters in assistance data should also be introduced to facilitate UE’s PRS measurement.

      - Number of hops

      - Number of overlapped RB between hops

      - Total bandwidth of all hops

Proposal 6: For on-demand PRS, a RedCap UE can request a larger bandwidth that the UE supports, which implies PRS frequency hopping measurement is requested.

Proposal 7: For RedCap UE, support frequency hopping across SRS resources.

Proposal 8: For RedCap UE with SRS, introduce partial overlapping in frequency for adjacent hops.

- The number of overlapping PRBs in adjacent hops is based on the bandwidth of each hop.

Proposal 9: In the overlapping PRB(s), support SRS transmission with comb size 1 for effective estimation of phase rotation.

Proposal 10: Support that TRPs can report the corresponding hop indication for each measurement result.

Reference

- 3GPP, Draft_Minutes_report_RAN1#112_v040, 3GPP RAN1#112, February 27 – March 3 2023.

- TS38.331, NR Radio Resource Control (RRC) protocol specification (Release 17) V17.1.0, 2022-06.

- TS38.214, NR Physical layer procedures for data (Release 17) V17.4.0, 2022-12.

- TS38.305, NG Radio Access Network (NG-RAN) Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17) V17.2.0, 2022-09.

- R1-2300808, Discussion on Positioning for RedCap UEs, ZTE, 3GPP RAN1#112, February 27 – March 3 2023.