- 3GPP TSG RAN WG1 #112bis-e                                                                                        R1-2303293

- E-meeting, April 17th – 26th, 2023

Source:	Rapporteur (ZTE)

Title:	Discussion on the UE feature for NCR

Agenda Item:	9.17.4

Document for:    Discussion

- Introduction

In RAN1#112 meeting, the RAN1 part of the work item Network Controlled Repeater has been completed. In this contribution, the layer-1 UE features for NCR are discussed.

- Discussion on NCR capabilities

  - Capabilities for NCR-Fwd

Agreement The following is supported to deliver the information to characterize the supported physical beam of NCR-Fwd for access link: Option-2: The information is informed to gNB and NCR via OAM - Note-1: In this option, how to characterize the beam information is based on implementation (e.g., declaration from NCR vendor). - Note-2: In this option, the beam(s) used by NCR-Fwd for access link is configured for gNB and NCR by OAM based on implementation. - The beam index in SCI corresponds to the configured beam(s) sequentially.

According to the agreement above, the information to characterize the physical beams for access link is informed to gNB and NCR via OAM. In addition, ON-OFF behavior of NCR-Fwd is implicitly determined by access link beam indication, therefore, the ON-OFF related capability of NCR-Fwd can also be informed to NCR and gNB via OAM. As for backhaul link beam indication, since the TCI state is selected from the same RRC configured list for NCR-MT, the corresponding capability can be defined for NCR-MT and reported via legacy RRC signalling.

In summary, the information exchange/delivering for the NCR-Fwd relevant capabilities including beam and ON-OFF can be delivered via implementation and there’s no need to define UE features for NCR-Fwd reported via RRC signalling.

Proposal 1: No need to define UE features for NCR-Fwd reported via RRC signalling.

  - Capabilities for NCR-MT

    - New UE features for NCR-MT

In normative phase, the side control information and associated signalling for NCR have been specified, including periodic, semi-persistent and aperiodic beam indication for access link, and dedicated MAC CE signalling for backhaul link, while NCR is ON within the time domain resource indicated in the access link beam indication. From NCR-MT’s perspective, the capabilities for reception of these side control information should be defined.

For periodic beam indication, it can be used to forward periodic signals including common and UE specific signals and for aperiodic beam indication, it can be used to forward the dynamically scheduled signals based on UE’s traffic. So, the reception of periodic and aperiodic indication should be defined as mandatory UE features for NCR-MT.

For semi-persistent beam indication for access link, it can be used as a supplementary in addition to periodic indication to forward the periodic signals which can be activated or deactivated. Then it can be further studied whether this feature can be mandatory or optional. In addition, it has been agreed that the MAC CE can optionally provide update for the Zy beam index configured in RRC, then the beam index update should be an separate optional capability for NCR-MT.

Moreover, in RAN1#110bis-e meeting, the following agreements have been achieved on defining the NCR capabilities for simultaneous UL transmission of C-link and backhaul link, adaptive beam for C-link/backhaul link and new signalling for backhaul link.

In addition, corresponding to adaptive beam, the fixed beam for C-link/backhaul link is default capability. It’s also agreed that simultaneous UL transmission of C-link and backhaul link should be NCR capability, and TDMed UL transmission of C-link and backhaul link is default capability.

If adaptive beams are adopted for C-link and backhaul link, new signaling via MAC CE can be optionally supported to indicate a beam(s) used for backhaul link from the set of beams for C-link, the new signalling is agreed as an optional NCR capability.

Agreement The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link Agreement If adaptive beams are adopted for C-link and backhaul link, new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link. - Predefined rule is used to define the beam in case there is no indication via the new signalling - FFS: Details of the predefined rule - FFS: Application of predefined rule for other cases - Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link. - The new signalling, if needed, is an optional NCR capability

For those capabilities agreed for backhaul link and C-link, since it’s more related to the NCR-MT’s behavior, it’s better to define these capabilities as NCR-MT’s UE features informed to gNB via RRC signalling.

Proposal 2:  Adopt the following new feature groups for NCR-MT.

| Features | Index | Feature group | Components | Prerequisite feature groups | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type(the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Periodic beam indication for access link | 1.Support periodic beam indication for access link |  | n/a | No | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Support |  | Mandatory without capability signaling |
|  | 43-2 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link |  | n/a | No | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Support |  | Mandatory without capability signaling |
|  | 43-3 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link |  | n/a | [Yes/No] | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Support |  | [Mandatory without/Optional with] capability signaling |
|  | 43-3a | Beam index updates for semi-persistent beam indication | 1.Support to update the beam index(es) in semi-persistent beam indication | 43-3 | n/a | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Support |  | Optional with capability signaling |
|  | 43-4 | Adaptive beam for NCR backhaul link/C-link | - Support adaptive beam for NCR C-link - Support adaptive beam for NCR backhaul link |  | n/a | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Support |  | Optional with capability signaling |
|  | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link |  | n/a | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Support |  | Optional with capability signaling |
|  | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-4 | n/a | Yes | The beam for backhaul link follows predefined rule. | Per NCR-MT | No | No | Support |  | Optional with capability signaling |

Furthermore, there is a pending issue on how to define the reference of slot offset for each time resource for aperiodic beam indication, which may have impact on UE features.

Agreement For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and: - Option-2: k refers to the offset value [defined by NCR-MT capability and/or declared by vendor]. - Note: This k is different from the parameter used to define the Slot offset for the time resource.

According to the agreement above, down-selection is required to define the offset value k, which can be defined by NCR-MT capability and/or declared by vendor. The offset value k may come from the timing in several parts, e.g., DCI decoding time of NCR-MT, MT to Fwd inter-module delay, potential ON-OFF or beam switching time of NCR-Fwd, it’s difficult to define the offset value as NCR-MT’s capability. Thus it’s more straightforward to go with the other direction, i.e., the offset value k can be declared by vendor by considering all possible impacted factors.

Proposal 3: For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and k refers to the offset value declared by vendor.

- Note: No need to define new capability for the offset value k.

- 2.2.2 Existing mandatory UE features for NCR-MT

RAN1#111 Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. Agreement - Legacy Rel-15 initial/random access procedure is supported for NCR-MTs in C link. - Note: No additional enhancement is necessary from RAN1 point of view. - The CRC bits of the PDCCHs carrying side control information are scrambled by a new dedicated RNTI - Applicable only for NCR-MT Agreement: For NCR-MT which can support adaptive beams in C link, - Rel-15 beam indication framework can be reused. - Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports.

Generally, NCR-MT is assumed to have simplified functionalities compared to legacy UE. Also, no additional complexity is expected compared to IAB-MT with the mandatory features defined in section 4. [2]. Meanwhile, BFD/BFR/RLM mechanisms have been agreed as optional functionalities in RAN1#111.

Then, the mandatory feature defined in Table 1 seems enough for NCR-MT.

Table 1: Layer-1 mandatory features for NCR-MT

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL | 1) CP-OFDM for DL 2) CP -OFDM for UL |
|  | 0-3 | DL modulation scheme | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |
|  | 0-4 | UL modulation scheme | 1) QPSK modulation 2) 16QAM modulation |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1) RACH preamble format 2) SS block based RRM measurement 3) Broadcast SIB reception including RMSI/OSI and paging |
| 2. MIMO | 2-1 | Basic PDSCH reception | 1) Data RE mapping 2) Single layer transmission 3) Support one TCI state |
|  | 2-5 | Basic downlink DMRS for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. |
|  | 2-6 | Basic downlink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |
|  | 2-12 | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission Single port, single resource SRS transmission (SRS set use is configured as for codebook) |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols |
|  | 2-16a | Basic uplink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |
|  | 2-32 | Basic CSI feedback | 1) Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2) 2Tx codebook for FR1 and FR2 3) 4Tx codebook for FR1 4) 8Tx codebook for FR1 when configured as wideband CSI report 7) a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |
|  | 2-50 | Basic TRS | 1) Support of TRS (mandatory) 2) All the periodicity are supported. |
|  | 2-52 | Basic SRS | 1) Support 1 port SRS transmission 2) Support periodic/aperiodic SRS transmission |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as "enabled" 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as "enabled" 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) SR/HARQ multiplexing once per slot using a PUCCH when SR/HARQ-ACK are supposed to be sent by overlapping PUCCH resources with the same starting symbols in a slot 8) HARQ-ACK piggyback on PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on 9) Semi-static beta-offset configuration for HARQ-ACK 10) Single group of overlapping PUCCH/PUCCH and overlapping PUCCH/PUSCH s per slot per PUCCH cell group for control multiplexing |
|  | 4-10 | Dynamic HARQ-ACK codebook | Dynamic HARQ-ACK codebook |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - One unicast PDSCH per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment for TDD 9) In TDD support at most one switch point per slot for actual DL/UL transmission(s) 10) DL scheduling slot offset K0=0 12) UL scheduling slot offset K2<=12 For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, interleaving for VRB-to-PRB mapping for PDSCH |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |
| 7. Channel coding | 7-1 | Channel coding | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |
| 8. UL TPC | 8-3 | Basic power control operation | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 9) PHR |

Proposal 4: NCR-MT supports the following layer-1 mandatory UE features defined in TR38.822.

- 0-1, 0-3, 0-4, 1-1, 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 4-10, 5-1 (only components 1/2/3/4/5/6/9/10/12), 6-1, 7-1, 8-3

- 2.2.3 Revised existing UE features for NCR-MT

RAN1#111 Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. Agreement The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link

As shown in above agreement, the RLM mechanism is changed to an optional functionality for the NCR-MT, so FG 1-3, 1-7 for SSB/CSI-RS based RLM should be changed to optional.

In addition, the existing UE features relevant to beam management can only be supported optionally if adaptive beam is supported, for example, in Rel-15, FG 2-21 periodic and FG 2-22 aperiodic beam report are mandatory features, but those features should be changed to optional since beam report is useless if backhaul/C-link beam is fixed.

Table 2: Revised existing features for NCR-MT

| Features | Index | Feature group | Prerequisite feature groups | Mandatory/Optional | Additional information |
| --- | --- | --- | --- | --- | --- |
| 1. Initial access and mobility | 1-3 | SS block based RLM | 1-1 | Mandatory with capability signalling which shall be set to '1' Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. |
|  | 1-7 | CSI-RS based RLM | 1-1, CSI-RS | Mandatory with capability signalling Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. |
| 2. MIMO | 2-21 | Periodic beam report | 43-4 for NCR | Mandatory with capability signalling for both FR1 and FR2 Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-22 | Aperiodic beam report | 43-4 for NCR | Mandatory with capability signalling for both FR1 and FR2 Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-23 | Semi-persistent beam report on PUCCH | 43-4 for NCR | Optional with capability signalling | FG 43-4 should be added in the prerequisite feature group. |
|  | 2-23a | Semi-persistent beam report on PUSCH | 43-4 for NCR | Optional with capability signalling | FG 43-4 should be added in the prerequisite feature group. |
|  | 2-24 | SSB/CSI-RS for beam measurement | 2-21, 2-22 or 2-23, 2-23a 43-4 for NCR | Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-4: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} On FR2, UE is mandated to signal either "3 only" or "both 1 and 3" On FR1, either "3 only" or "both 1 and 3" is mandatory with UE capability signalling. Component-5, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4 Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-25 | Beam reporting timing | 2-24 43-4 for NCR | Mandatory with capabilit Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8,14, 28} X4 is{14,28, 56} Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 43-4 for NCR | Mandatory with UE capability at least for FR2 Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-27 | Beam switching | 2-24 43-4 for NCR | Optional with capability signalling Candidate value set: {4, 7, 14} | FG 43-4 should be added in the prerequisite feature group. |
|  | 2-28 | A-CSI-RS beam switching timing | 43-4 for NCR | Optional with capability signalling Candidate values: {14, 28, 48, 224, 336} | FG 43-4 should be added in the prerequisite feature group. |
|  | 2-29 | Non-group based beam reporting | 43-4 for NCR | Mandatory with capability for both FR1 and FR2 candidate value set is {1, 2, 4} Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-29a | Group based beam reporting | 43-4 for NCR | Optional with capability signalling | FG 43-4 should be added in the prerequisite feature group. |
|  | 2-30 | Uplink beam management | 43-4 for NCR | Optional with capability signalling - Capability signalling shall be set to 1 if 2-20 is set to 0 - For the UE meeting the minimum peak EIRP and spherical coverage requirements without the uplink beam sweeping, this feature is optional Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} | FG 43-4 should be added in the prerequisite feature group. |
|  | 2-31 | Beam failure recovery | 1-7 for CSI-RS based BFD/BFR 43-4 for NCR | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} UE is mandated to support at least 32 for FR2 Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-59 | Configured spatial relations | 43-4 for NCR | Candidate value set: {4, 8, 16, 32, 64, 96} UE is mandated to report 16 or higher values. Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-60 | Active spatial relations | 2-59 43-4 for NCR | Mandatory with capability signalling Candidate value set: {1, 2, 4, 8, 14} Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-61 | Additional active spatial relation for PUCCH | 2-60 43-4 for NCR | Mandatory with capability signalling Optional with capability signalling for NCR | This FG is optional with capability signalling for NCR. FG 43-4 should be added in the prerequisite feature group. |
|  | 2-62 | Max number of downlink RS resources used for QCL type-D in the active TCI states and active spatial relation info | 2-4, 2-4a and 2-60 43-4 for NCR | Optional with capability signalling Candidate value set: {1,2,4,8, 14} | FG 43-4 should be added in the prerequisite feature group. |

Based on above, the following is proposed:

Proposal 5: The beam management related UE features (i.e., FG 2-21~FG 2-31 and FG 2-59~FG 2-62 in Rel-15) as defined in TR38.822 should be revised for NCR-MT as follows:

- The mandatory features for beam management should be changed to optional.

- FG 43-4 Adaptive beam for NCR backhaul link/C-link should be added in the prerequisite feature group.

- 2.2.4 Not supported UE features for NCR-MT

Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols - FFS: The behavior over these symbol if dynamic DL/UL operation is supported by NCR-MT and/or NCR-Fwd. Conclusion For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the dynamic DL/UL operation of NCR-MT and NCR-Fwd is not supported in Rel-18.

As agreed in RAN1#111, the dynamic DL/UL operation of NCR-MT and NCR-Fwd is not supported in Rel-18, therefore, the dynamic DL/UL operation related UE features are not supported for NCR.

Table 3: Not supported UE features for NCR-MT

| Features | Index | Feature group | Components | Additional information |
| --- | --- | --- | --- | --- |
| 3. DL control channel and procedure | 3-6 | Dynamic SFI monitoring | Adjust periodic and semi-persistent signal reception and transmission in response to detected dynamic UL/DL configuration | This FG is not supported for NCR. |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment | Component 7 of this FG is not supported for NCR. |
|  | 5-1a | UE specific RRC configure UL/DL assignment | Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment | This FG is not supported for NCR. |

Proposal 6: The dynamic DL/UL operation related UE features FG 3-6, FG 5-1a and Component 7 of FG 5-1 are not supported for NCR.

- Conclusions

In this contribution, discussion on RRC and MAC CE is provided with following proposals:

Proposal 1: No need to define UE features for NCR-Fwd reported via RRC signalling.

Proposal 2:  Adopt the following new feature groups for NCR-MT.

| Features | Index | Feature group | Components | Prerequisite feature groups | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type(the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Periodic beam indication for access link | 1.Support periodic beam indication for access link |  | n/a | No | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Support |  | Mandatory without capability signaling |
|  | 43-2 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link |  | n/a | No | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Support |  | Mandatory without capability signaling |
|  | 43-3 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link |  | n/a | [Yes/No] | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Support |  | [Mandatory without/Optional with] capability signaling |
|  | 43-3a | Beam index updates for semi-persistent beam indication | 1.Support to update the beam index(es) in semi-persistent beam indication | 43-3 | n/a | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Support |  | Optional with capability signaling |
|  | 43-4 | Adaptive beam for NCR backhaul link/C-link | - Support adaptive beam for NCR C-link - Support adaptive beam for NCR backhaul link |  | n/a | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Support |  | Optional with capability signaling |
|  | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link |  | n/a | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Support |  | Optional with capability signaling |
|  | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-4 | n/a | Yes | The beam for backhaul link follows predefined rule. | Per NCR-MT | No | No | Support |  | Optional with capability signaling |

Proposal 3: For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and k refers to the offset value declared by vendor.

- Note: No need to define new capability for the offset value k.

Proposal 4: NCR-MT supports the following layer-1 mandatory UE features defined in TR38.822.

- 0-1, 0-3, 0-4, 1-1, 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 4-10, 5-1 (only components 1/2/3/4/5/6/9/10/12), 6-1, 7-1, 8-3

Proposal 5: The beam management related UE features (i.e., FG 2-21~FG 2-31 and FG 2-59~FG 2-62 in Rel-15) as defined in TR38.822 should be revised for NCR-MT as follows:

- The mandatory features for beam management should be changed to optional.

- FG 43-4 Adaptive beam for NCR backhaul link/C-link should be added in the prerequisite feature group.

Proposal 6: The dynamic DL/UL operation related UE features FG 3-6, FG 5-1a and Component 7 of FG 5-1 are not supported for NCR.

Reference

- TR 38.822, NR; User Equipment (UE) feature list

- TS 38.306, NR; User Equipment (UE) radio access capabilities (Release 17)