3GPP TSG RAN WG1 #112bis-e	                                           R1-2303294

e-Meeting, April 17th - April 26th, 2023

Source:	ZTE

Title:	Discussion on coverage enhancement for NTN

Agenda Item:	9.9.1

Document for: Discussion

- Introduction

              - In RAN1#112 [1], UL coverage enhancements on PUSCH and PUCCH channels have been discussed to identify whether potential enhancements are needed with corresponding conclusion and agreements.

              - In this contribution, link-level simulations are performed with the corresponding techniques to evaluate the performance of VoIP on PUSCH and Msg4 HARQ-ACK on PUCCH at LEO-1200 with Set-1. Furthermore, the details of potential enhancement solutions are also elaborated.

- UL coverage evaluation

  - PUSCH for VoIP

      - According to the observation shown below, the performance of DMRS bundling will not be impacted by frequency error within ± 0.1 PPM but limited by the timing error and phase difference requirement in NTN.

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Proposal 2-2_v5 (for observation) For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots for carrier bandwidth bandwidth 5 MHz at the edge of the carrier bandwidth, if TA pre-compensation update, phase pre-compensation, and RX phase post-compensation are not assumed, and if 70.5 (us/s) timing drift rate is assumed. - FFS: whether to assume phase pre-compensation at UE and/or RX phase post-compensation at gNB

      - Based on our previous analysis in [2], when the segment length for pre-compensation is within 6ms, the phase continuity requirement for DMRS bundling can be maintained within a segment for LEO-1200 satellite at 30 degree elevation angle by assuming UL bandwidth of 360kHz. On the other hand, a phase jump could occur between adjacent segments due to the timing adjustment, which breaks the phase continuity requirement of DMRS bundling. Hence, a practical solution to enable DMRS bundling is to perform the DMRS bundling per segment, i.e., determine the DMRS TDW size based on the segment pre-compensation configuration.

      - Observation 1: DMRS bundling size can be determined based on the segment length for pre-compensation.

      - However, a 6ms segment is much shorter than maximum duration of a PUSCH transmission, e.g., 20 slots for VoIP, which limits the gain of DMRS bundling. Moreover, for UE at the edge of large carrier bandwidth, the segment length that can satisfy the phase continuity requirement may be even shorter as shown in the above agreement, which further reduces the gain of DMRS bundling. Hence, for some advanced UEs, e.g., UEs that can compensate the phase offset between segments, DMRS bundling across different segments should also be supported to achieve higher DMRS bundling gain. Of course, for UEs without the capability to perform DMRS bundling across different segments, the DMRS bundling size should still be limited by the segment length. A capability may be defined to distinguish the UEs and behaviors on determining DMRS bundling TDW.

      - Proposal 1: A capability can be defined to indicate whether UE can perform DMRS bundling across different pre-compensation segments.

      - W.r.t how to determine actual or nominal TDW for DMRS bundling, following options were discussed in RAN1#112 but no agreement achieved:

Proposal 2-3_v5 For NTN-specific PUSCH DMRS bundling, further discuss whether nominal/actual TDW determination defined in R17 can be reused for NTN or not, e.g., the following options. - Option 1: Actual TDW determination based on timing of TA pre-compensation update causing phase discontinuity and power inconsistency. UE determines the timing. - Nominal TDW is configured by gNB as in the existing spec. - UE reports information on the timing. FFS details - Actual TDW is determined by the reported information and events defined in R17. - New event is defined based on TA pre-compensation update. FFS details - Option 2: Nominal/actual TDW determination based on gNB configuration and/or indication - Option 2-1: Determined as in the existing spec. No spec change is assumed. - Nominal TDW is configured by gNB as in the existing spec. - Note: Based on gNB implementation, nominal TDW length can be configured to current timing drift, derived from satellite ephemeris, to allow UE to update time/frequency pre-compensation sufficiently often. - Actual TDW is determined by events defined in R17. - Note: no new event is defined for the TA pre-compensation update. - Option 2-2: Determined by new gNB configuration/indication. - FFS details - e.g., TA pre-compensation timing is indicated by gNB - e.g., further segmented configuration is provided by gNB - FFS: Whether new event is defined based on new gNB configuration/indication. - FFS: whether to define another new event to decide actual TDW (e.g., antenna switching, update of epoch time, etc.)

      - As analyzed above, TA adjustment will cause phase offset which may break the phase continuity requirement of DMRS bundling. Actually, in legacy TN, the MAC CE TA command is considered as an event to determine actual DMRS TDW, i.e., DMRS bundling phase difference limit is not though kept after a TA adjustment. Therefore,  a new event of segmented pre-compensation should be defined similarly to determine TDW. Besides, this new event can be implicitly coupled with antenna switching to increase performance gain by using the same antenna at each segment as an implementation. Hence, option 2-1, which reuses existing spec without considering the impact of TA pre-compensation, should not be considered. gNB cannot guarantee the phase different limit of DMRS bundling to be always satisfied without knowing when UE performs pre-compensation and whether UE has capability to mitigate phase offset between pre-compensation segments. Option 1 and option 2-2 both introduce new event for determining actual TDW and can be considered. However, in R17 IoT-NTN, segment pre-compensation mechanism has already been agreed and specified, where eNB is responsible for determining the segment length for pre-compensation. Hence, it is preferred to reuse such mechanism here, i.e., let gNB configure when to perform pre-compensation. Based on above analysis, we prefer Option 2-2 to determine actual TDW in NTN.

      - In addition, as discussed previously, UEs with advanced capability may be able to perform DMRS bundling across segments. For such types of UEs, the new event of segment pre-compensation is not needed. Therefore, UE should report the capability to network. Then network can determine whether to introduce the new event of segment pre-compensation to determine actual TDW. Furthermore, in order to evaluate the feasibility of Option 2-2 with UE capability of large DMRS bundling size across segments, the link-level simulation is evaluated. For UE capability of large DMRS bundling size across segments, through sample dropping and phase offset compensation, phase discontinuity caused by time/frequency compensation updates between segments can be within a tolerable range, which allows to configure TDWs that are longer than the segment length. As shown in Figure 2, JCE across segments can obtain ideal performance (e.g., TDW=20, SL = 1/2/4/6, with phase offset) when segmented pre-compensation and phase continuity between segments are both guaranteed. Therefore, in order to achieve the better performance, the actual TDW should be determined based on UE capability.

Proposal 2: For the determination of actual TDW based on new gNB configuration, at least the following behaviors should be required.

- UE reports capability on whether to support DMRS bundling across pre-compensation segments

- The new event of segmented pre-compensation is introduced for UEs not support DMRS bundling across pre-compensation segments.

      - Figure 2 Performance based on DMRS bundling cross UL pre-compensation segments

  - PUCCH for Msg4 HARQ-ACK

      - For ACK for Msg-4, since a UE does not have dedicated PUCCH resource configuration, the corresponding PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 1 in [3] for transmission of HARQ-ACK information on PUCCH, the number of symbols it can occupy is 4, 10, or 14. According to the design principle of PUCCH format 1, a longer symbol can provide better coverage performance, and the link budget requirements of NTN can be satisfied by simple repeated transmission. So, if PUCCH format 1 with less symbol number, e.g., 4 symbols, is assumed as the baseline, a larger number of repetitions is required, which will lead to a larger time delay for the RACH processes. Meanwhile, due to the decreased channel correlation, the performance gain brought by repeated transmission will also be degraded and the bundling will be difficult to be achieved. Then, only the PUCCH format-1 with 14 symbols are used for enhancement. Meanwhile, considering the standardization effort to define a new table, reusing Table 9.2.1-1 of TS 38.213 is preferred to minimum spec change.

Proposal 3: The PUCCH format-1 with 14 symbols should be used as the baseline for enhancement.

Table 1 PUCCH resource sets before dedicated PUCCH resource configuration from TS 38.213 Table 9.2.1-1

| Index | PUCCH format | First symbol | Number of symbols | PRB offset RBBWPoffset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 | NBWPsize4 | {0, 3, 6, 9} |

    - DMRS bundling for PUCCH

      - To improve the coverage performance and eliminate the coverage gap, we can consider supporting repeated transmission of Msg4 PUCCH and DMRS bundling to improve coverage performance. In RAN1 #110bis-e meeting, the benefits on the PUCCH has also been discussed with following agreement:

Agreement For PUCCH for Msg4 HARQ-ACK, - Support PUCCH repetition - Further discuss the specification impact for at least the following - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.) - Repetition factor - Repetition slot counting for FDD - Further study whether to enhance or support the following - Frequency hopping - DMRS bundling

      - In this section, further evaluation to justify the improvement of DMRS bundling along with repetition is done for PUCCH Msg4 HARQ-ACK with 1 bit payload, detailed simulation assumption is shown in Table 5.

      - As can be seen from the blue curve in Figure 7 and Table 2, the required SNR at 1% BLER is -7.1 dB when the number of repetitions is only 2. Further improvement can be achieved with FO = 0 Hz, that is, -8 dB. When the number of repetitions is increased to 4, the performance can be further improved to -10.72 dB at 1% BLER.

      - In addition, as shown in Figure 7 and the performance gap table of PUCCH Msg4 HARQ-ACK shown in Table 2, the required SNR at 1% BLER is -8.66 dB when the number of repetitions is only 2 with DMRS bundling performed, which can meet the requirement. Significant performance improvement can be achieved by DMRS bundling by enabling joint cross-slot channel estimation and frequency pre-compensation. With the increase of the number of repetitions, up to maximum 1.99 dB gain can be obtained by joint channel estimation. That is, higher performance gain can be achieved with more DMRS sharing in repetitions which can further improve the coverage performance of PUCCH for Msg4 HARQ-ACK.

      - How to indicate the request of DMRS bundling for PUCCH for Msg4 HARQ-ACK should be considered. A bit field of msg3 can be defined to indicate the request of DMRS bundling for PUCCH for msg4 HARQ-ACK transmission. Similar with PUSCH, the DMRS bundling time window may be same as a segment length for pre-compensation. If the time duration of PUCCH transmission (including repetitions) is shorter than the segment length for pre-compensation, all the DMRSs can be within the same bundle.

Proposal 4: The enhancement on DMRS bundling should also be applied for PUCCH with similar mechanism as PUSCH for VoIP.

Proposal 5: A bit field in Msg3 can be defined to indicate whether to support DMRS bundling for PUCCH for Msg4 HARQ-ACK.

      - Figure 3 Performance result of PUCCH for Msg4 HARQ-ACK with DMRS bundling

Table 2 Performance gap @ 1% BLER (PUCCH Format-1 with 1bit Msg4 HARQ-ACK)

| PUCCH format-1 (1bit) for Msg4 HARQ-ACK/NACK in LEO-1200-30°with satellite parameter-set1, with CNR= - 8.1 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number of Trans and FO(ppm) | Required SNR (dB) (only rep) | Gap (dB) (only rep) | Required SNR (dB) (rep + JCE) | Gap (dB) (rep + JCE) | JCE gain(dB) (rep only) vs (rep + JCE) |
| 1 w/ 0.1 | -3.6 | -4.5 | - | - | - |
| 2 w/ 0.1 | -7.1 | -1 | - | - | - |
| 2 w/ 0 | -8 | -0.1 | -8.66 | 0.56 | 0.66 |
| 4 w/ 0 | -10.72 | 2.62 | -12.71 | 4.61 | 1.99 |
| Note：The red gap value indicates that the CNR requirement is not met. |  |  |  |  |  |

    - Repetition for PUCCH

      - Determination on reporting indication

      - In RAN1 #112[1], the options to determine whether to transmit the reporting indication was discussed and following working assumption was agreed.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

      - According to the content discussed in the previous meeting, the repetition factor is dynamically configured by the gNB, which means that the gNB can configure the repetition factor more accurately based on the measurement results of the uplink signal. Through threshold, UE can be based on measurement results to decide the most suitable repetition factor. Unfortunately, this has been rejected at the last meeting.

      - Therefore, in our understanding, if the UE does not need to determine the repetition factor that best fits the current channel environment based on the measurement results, then the UE only needs to report whether it has the capability to repeat, regardless of whether a threshold is configured. Conversely, if the threshold for reporting PUCCH repetition is the same as R17 Msg3 repetition, the scheduling flexibility of PUCCH repetition will be affected.

      - In our understanding, the RSRP threshold to determine whether to perform repetition is not necessary. Firstly, it should be noted that RSRP variation within an NTN cell is small, which is different from TN. Even in higher layer procedures in NTN, where RRM measurements are available and much more accurate than RSRP measurement in physical layer, time and location based handover solutions are designed in Rel-17 to replace legacy RSRP based handover solution. Hence, RSRP is not an efficient parameter to distinguish UEs within an NTN cell, which could have similar RSRP. Secondly, the RSRP threshold configured by network is for DL RSRP measurement, while the PUCCH repetition is a UL transmission, which are mismatched. UE’s RSRP measurement on DL signal cannot accurately reflect the PUCCH decoding performance at gNB. In legacy TN, since RSRP variation can be very large within a cell, DL RSRP measurement can reflect UL performance to some extent even if there is some inaccuracy caused by mismatch. While in NTN, since the RSRP variation is small, the inaccuracy could have significant impact, i.e., the PUCCH repetition cannot be well controlled by DL RSRP measurement. Thirdly, it has already been agreed that PUCCH repetition will be performed only when network broadcast the repetition factors in SIB. That is, there is already a mechanism to control whether to perform repetition. If the satellite is in a low orbit and high elevation angle, where link budget is high and no need of PUCCH repetition, network can just not configure the repetition factors in SIB. If the elevation angle is low or satellite orbit is high, network can enable the PUCCH repetition by configuring the repetition factor in the SIB. Since UEs within a cell has similar RSRP, such cell specific control mechanism is already enough. Defining an RSRP threshold is completely redundant. Based on above analysis, RSRP threshold should not be introduced, i.e., the working assumption above should not be confirmed.

      - Proposal 6: The working assumption on introducing RSRP threshold should not be confirmed, since defining RSRP threshold to configure PUCCH repetition is redundant and without benefits on performance.

      - Proposal 7: For PUCCH repetition for Msg4 HARQ-ACK, UE capable of PUCCH repetition reports the capability of PUCCH repetition for Msg4 HARQ-ACK regardless of whether the threshold is configured or not.

      - Reporting indication

      - In RAN1 #112[1], the options to indicate the information report whether the UE is capable of supporting repetition was discussed and following working assumption was agreed.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports

      - In Rel-17 Msg3 PUSCH repetition, separate PRACH preamble/occasion is used for the information transmission. Meanwhile, considering large preamble fragmentation already exists for the support of other features (RedCap,slicing,SDT,etc.) and the large cell sizes in NTN, furthermore, PRACH repetition which would be introduced in R18 coverage enhancement WI will consume more resources. Indication of PUCCH repetition capability via PRACH preamble/RO will result in further segmentation of the PRACH preambles which would lead to increased collision probability. However, if no further segmentation for indication of capability is performed, it may affect the accurate configuration of PUCCH repetition, such as being configured with repetition when no PUCCH repetition is needed.

      - Therefore, we prefer option B. There may be multiple types of UEs in an NTN cell. Considering forward compatibility and a UE does not always perform PUCCH repetition, additional restrictions need to be introduced to determine whether to use higher layer signaling in the Msg3 PUSCH to report capability. UE capable of repetition can determine whether to request repetition through higher layer signaling in Msg3 PUSCH if multiple values are configured by SIB. The higher layer of the UE incapable of repetition retains its original purpose regardless of whether multiple values are configured or not.

Proposal 8: When multiple repetition values is configured by SIB, capable of repetition UE to interpret the indication in the Msg3 PUSCH as capability reporting indication should be specified.

      - In addition, timing of when dedicated PUCCH configuration is provided is up to NW implementation though Msg4 PDSCH can include the configuration. That is, whether to configure the dedicated PUCCH resource in the Msg4 PDSCH is determined by NW implementation. So even after the HARQ for Msg-4, there is still case that common PUCCH will be reused, e.g., the HARQ feedback for subsequent PDSCHs after successful random access will be carried by the common PUCCH. To solve this issue, for the capability indication, there is a need to introduce additional restriction to define the validity duration of capability.

      - Proposal 9: The validity duration of capability indication needs to be defined, such as the validity duration of request/capability indication is valid till the configuration of dedicated PUCCH.

      - Dynamic indication

      - In RAN1 #112[1], the options to indicate the repetition number of PUCCH for Msg4 HARQ-ACK was discussed and following agreement was agreed.

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

For Alt 3, it may increase the complexity of the NW side and affect the detection complexity of the receiver. At the same time, due to the modification of the RNTI of the scrambling CRC, it may also increase the probability of missing detection of DCI. For Alt 4, although the implicit mapping has no impact on the overhead of DCI, generally the performance of PUCCH channels is better than that of PUSCH channels. Therefore, there may be situations where only PUSCH repetition is required without PUCCH repetition. In this case, it may cause unnecessary resource waste of PUCCH and access delay, and using other signaling, such as high-layer signaling, to define implicit mapping relationships can make dynamic scheduling inflexible. For Alt 2, not only does it change the design of DCI, but it also faces problems similar to Alt 4, e.g., dynamic scheduling is not only inflexible but also complex.

Therefore, we prefer Alt 1. Since Alt 1 may use existing bit field in the DCI, such as MCS, PRI, or DAI, to indicate the repetition number, the UE needs to know whether it is necessary to interpret a specific bit field as a repetition number indication to avoid misunderstandings about the purpose of the specific bit field on the UE side. For example, when the repetition capability of UE does not require coverage enhancement, it will not send a capability indication, and the UE will not interpret a specific bit field in the DCI scheduling Msg4 PDSCH. When the repetition capability of UE requires coverage enhancement, it will send a capability indication, and the UE needs to interpret a specific bit field in the DCI scheduling Msg4 PDSCH as a repetition number indication to obtain the repetition number of dynamic scheduling on the gNB. In this way, it is possible to avoid errors in the behavior when the repetition capability of UE is not required to be repeated but the number of repetitions is configured by the gNB, as UEs that require repetition and UEs that do not require repetition may complete for random access. Meanwhile, it can also avoid affecting the original purpose of the field.

Proposal 10: When the capability indication is sent by a UE, the UE to interpret the indication in the DCI scheduling Msg4 PDSCH as repetition number indication should be specified.

      - Frequency hopping

      - In RAN1 #112[1], how to perform frequency hopping for UE that needs to repeat PUCCH was discussed but no agreement was agreed.

Proposal 1-8_v0 For PUCCH repetition for Msg4 HARQ-ACK, intra-slot frequency hopping is used.

      - In NR, the frequency hopping protocols related to dedicate PUCCH are as follows. From these descriptions, it is clear that when dedicated PUCCH supports repetition, the UE can perform inter-slot frequency hopping only if interslotsFrequencyHopping is configured. Otherwise, intra-slot frequency hopping is performed by default. In other words, frequency hopping within a slot is not enabled by signaling, but is enabled by default. Similarly, for protocol descriptions of common PUCCH that do not support repetition, when the NR-U parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is not provided, frequency hopping is also supported. In our understanding, this frequency hopping method defaults to frequency hopping within a slot. In this way, dedicate PUCCH and common PUCCH have a common understanding. Therefore, if inter slot frequency hopping is not agreed, when the repetition factor for PUCCH for Msg4 HARQ-ACK is indicated in DCI scheduling Msg4 PDSCH, intra-slot frequency hopping is performed by default.

      - Proposal 11: For PUCCH repetition for Msg4 HARQ-ACK, UE capable of PUCCH repetition perform intra-slot frequency hopping if the repetition factor is indicated in DCI scheduling Msg4 PDSCH.

Dedicated PUCCH

– PUCCH-Config from 38.331 PUCCH-Config information element ... ... interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 0, 1, 3 or 4 is repeated over multiple slots. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6. ... 9.2.6 PUCCH repetition procedure from 38.213 ... - if the UE is not configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot ...

Common PUCCH

9.2.1 PUCCH Resource Sets from 38.212 … The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping. …

      - If inter slot frequency hopping is agreed, there are two frequency hopping types to choose. Considering the current specification does not clear text to do 'intra-slot' or ‘inter-slot’ FH, then anyhow spec change for that seem to be necessary. A similar scheme to dedicated PUCCH repetition can be considered, that is, defining enable signals for inter slot FH to indicate whether to perform inter slot frequency hopping. If the UE is not configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot. This way is consistent with the frequency hopping configuration of dedicated PUCCH to reduce protocol impact.

      - Proposal 12: For PUCCH repetition for Msg4 HARQ-ACK, if inter-slot frequency hopping is agreed, an additional enabling signaling need to be defined to indicate whether inter-slot frequency hopping is enabled.

- Conclusions

      - According to the analysis given above, we have the following observations and proposals:

      - Observation 1: DMRS bundling size can be determined based on the segment length for pre-compensation.

      - Proposal 1: A capability can be defined to indicate whether UE can perform DMRS bundling across different pre-compensation segments.

Proposal 2: For the determination of actual TDW based on new gNB configuration, at least the following behaviors should be required.

- UE reports capability on whether to support DMRS bundling across pre-compensation segments

- The new event of segmented pre-compensation is introduced for UEs not support DMRS bundling across pre-compensation segments.

Proposal 3: The PUCCH format-1 with 14 symbols should be used as the baseline for enhancement.

Proposal 4: The enhancement on DMRS bundling should also be applied for PUCCH with similar mechanism as PUSCH for VoIP.

Proposal 5: A bit field in Msg3 can be defined to indicate whether to support DMRS bundling for PUCCH for Msg4 HARQ-ACK.

      - Proposal 6: The working assumption on introducing RSRP threshold should not be confirmed, since defining RSRP threshold to configure PUCCH repetition is redundant and without benefits on performance.

      - Proposal 7: For PUCCH repetition for Msg4 HARQ-ACK, UE capable of PUCCH repetition reports the capability of PUCCH repetition for Msg4 HARQ-ACK regardless of whether the threshold is configured or not.

      - Proposal 8: When multiple repetition values is configured by SIB, capable of repetition UE to interpret the indication in the Msg3 PUSCH as capability reporting indication should be specified.

      - Proposal 9: The validity duration of request/capability indication needs to be defined, such as the validity duration of request/capability indication is valid till the configuration/application of dedicated PUCCH.

      - Proposal 10: When the capability indication is sent by a UE, the UE to interpret the indication in the DCI scheduling Msg4 PDSCH as repetition number indication should be specified.

      - Proposal 11: For PUCCH repetition for Msg4 HARQ-ACK, UE capable of PUCCH repetition perform intra-slot frequency hopping if the repetition factor is indicated in DCI scheduling Msg4 PDSCH.

      - Proposal 12: For PUCCH repetition for Msg4 HARQ-ACK, if inter-slot frequency hopping is agreed, an additional enabling signalling need to be defined to indicate whether inter-slot frequency hopping is enabled.

- Reference

- R1-2302230, Summary EOM on 9.11.1 Coverage enhancement for NR NTN, RAN1#112

- R1-2300704, Discussion on coverage enhancement for NTN, ZTE

- 3GPP TS 38.213, Physical layer procedures for control

- Appendix

Table 3 Simulation assumption for VoIP on PUSCH

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Sub-carrier Spacing | 15kHz |
| Channel Bandwidth | 2PRBs |
| Channel Model | NTN-TDL-C Rural 300 elevation angle, -8.46 for lgDS, 8.05 for K |
| UE Antenna Configuration | 1T1R, 2T1R |
| UE speed | 3 km/h |
| Frequency offset | 0 ppm |
| JCE | w/ JCE |
| HARQ configuration | w/o HARQ |
| Modulation | QPSK |
| PUSCH duration | 14 OS |
| PUSCH for VoIP | MCS index:2 TBS: 184bits DMRS configuration: Type I, DMRS symbol Number of repetitions: 20 BLER: 2% iBLER |

Table 4 Link-level simulation assumptions for PUCCH transmission

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 1-bit for HARQ-ACK for Msg4. |
| Carrier frequency | 2 GHz |
| SCS | 15kHz |
| Channel Bandwidth | 20MHz |
| Channel Model | NTN-TDL-C Rural 300 elevation angle, -8.46 for lgDS, 8.05 for K |
| UE Antenna Configuration | 1T1R |
| UE speed | 3 km/h |
| Modulation | BPSK |
| Frequency offset | 0/0.1 ppm |
| Frequency hopping | w/ inter-slot |
| BLER | For PUCCH format 1: DTX to ACK probability: 1%. BLER: 1% |
| Repetitions | w/ repetition. The number of repetitions is 2,4. |
| PUCCH duration | 14 OS |
| JCE | w/ or w/o JCE |
| Number of PRBs | 1 PRB |