3GPP TSG RAN WG1 #112bis-e	 	                                       R1-2303297

e-Meeting, April 17th – April 26th, 2023

Source:	ZTE

Title:	Discussion on improved GNSS operation for IoT-NTN

Agenda Item:	9.9.4

Document for:  Discussion

Introduction

In RAN1#112 [1], the scheme to enable the GNSS measurement in RRC-connected state were discussed for IoT device with some progress. In this contribution, we further share the views on improved GNSS operation in IoT NTN, including on success/completion of GNSS position fix, the configurations of GNSS measurement gap during long connection and the closed loop time correction.

Discussion

  - Configuration of GNSS measurement gap

For the configuration of GNSS measurement gap in long connection time, following agreements are agreed on the gap duration and start time. However, there are still remaining issue that need further studies.

Agreement On the length of GNSS measurement gap, which is aperiodically triggered by eNB, the gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. FFS: whether the gap duration is configured by eNB, or the gap duration is equal to the latest reported GNSS position fix time duration. Agreement On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, RAN1 can down select one of the following alternatives: - Alt 1: the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value - Alt 2: the start time should be based on the current GNSS validity duration with delay or without delay

For the gap duration of GNSS measurement gap, it is preferred to directly let the gap duration equal to the GNSS position fix time duration reported by UE. In RAN1#110bis-e, RAN1 has agreed that UE reports GNSS position fix time duration for measurement at least during the initial access stage. Since GNSS positioning is implemented at UE, UE should ensure that GNSS position fix can be completed within the reported GNSS position fix time duration. Therefore, it is not preferred to let eNB to configure a gap duration larger than the time duration reported by UE, which may waste resource and signaling. With above consideration, the gap duration should be directly equal to the reported GNSS position fix time duration without additional eNB configuration. Moreover, in our understanding, GNSS position fix time duration depends on the chipset of GNSS and is stable. Hence, it is preferred to report GNSS validity duration only once during initial access for the saving of signaling overhead.

Proposal 1: The duration of GNSS measurement gap should be equal to GNSS position fix time duration reported by UE with consideration of signaling and resource overhead.

As for the start of measurement gap, in our view, the start time is set to the end time of GNSS validity duration without delay, which can effectively save signaling overhead and maximize the effective time of GNSS validity duration. For example, as shown in Figure 1, the GNSS measurement gap will start immediately if the GNSS validity duration is expired, then there is no additional signaling to indicate the start time for GNSS measurement gap. And the new GNSS validity duration is re-accounted once the re-acquisition is completed successfully without additional distortion of the timeline at both base station and UE sides. If the GNSS measurement is failed, UE can go to IDLE since UE will not have a valid GNSS position. The implementation is simple and each GNSS validity duration can be fully utilized. While if Alt-1 is applied, how to determine the value X need further investigation. Besides, other issues may also need to be considered subsequently, e.g., how to handle the timing of HARQ-ACK for the MAC CE and how to handle the case when GNSS measurement is failed but old GNSS validity duration is not ended. Hence, Alt-1 is not preferred since it will lead to much more spec effort than Alt-2 but no obvious benefit.

Proposal 2: The start of GNSS measurement gap should be set to the expiration time of GNSS validity duration without delay to minimize spec effort.

Figure 1 An illustration of GNSS update in RRC connection

Besides aperiodic measurement gap triggered by eNB, other triggering methods as follows were also mentioned in RAN1#112 but not discussed in detail [1]:

- whether UE can send a request for network to trigger GNSS measurement aperiodically is needed.

- whether additional periodically GNSS measurement is needed with agreed timing/timer based GNSS measurement.

In our view, the methods mentioned above are not needed. For first bullet, the motivation is to avoid the case where UE wants to keep in RRC_CONNECTED mode but network does not trigger GNSS measurement. However, as UE autonomous GNSS re-acquisition has been agreed to be supported, UE can always have a chance to perform GNSS measurement before going to IDLE. Therefore, this additional trigger method is not needed. While for second bullet, the benefit periodic GNSS measurement compared to aperiodic GNSS measurement is that only one trigger can be enough for multiple measurements, which saves the signaling overhead. However, since UE autonomous re-acquisition has already been agreed, where trigger signaling is not needed for each GNSS measurement, the benefit of additionally supporting periodic measurement is not clear. Hence, triggering of periodic measurement is not needed.

Proposal 3: No need to support UE sending a request for network to trigger GNSS measurement aperiodically.

Proposal 4: No need to support additional periodically GNSS measurement on top of agreed timing/timer based GNSS measurement.

  - UE autonomous GNSS re-acquisition

In RAN1#111, UE autonomous re-acquisition of GNSS when UE does not receive eNB trigger has also been agreed. However, the details of mechanism and configuration still need to be studied.

If UE does not receive a trigger of GNSS measurement but it still wants to re-acquire GNSS to continue the communication, the GNSS measurement should be performed within a time period after expiration of current GNSS. A timer can be defined to specify the time for GNSS re-acquisition. The start of the timer can be set as the expiration time of current GNSS validity duration so that GNSS can be re-acquired as soon as possible. While the timer length can be equal to GNSS position fix time duration reported by UE, which ensures UE has enough time to complete GNSS measurement. From above consideration on the GNSS measurement timer, it can be observed that the start and length of the timer are similar to the GNSS measurement gap illustrated in previous clause. Hence, the same configuration method for GNSS measurement gap can be reused here. That is, the GNSS measurement timer length should be equal to GNSS position fix time duration reported by UE at initial access stage. Moreover, GNSS measurement timer will start at the expiration time of current GNSS validity duration if UE does not receive eNB trigger. The validity duration of new GNSS information will start after expiration of the new configured GNSS measurement timer if UE successfully re-acquires GNSS information. Otherwise, UE will go to IDLE state after the expiration of the GNSS measurement timer.

              - Proposal 5: For configuration of GNSS measurement timer in connected, the complexity and spec impact should also be minimized.

- The GNSS measurement timer starts at the expiration time of GNSS validity duration.

- The GNSS measurement timer length is equal to GNSS position fix time duration reported by UE at initial access stage.

  - On success of GNSS position fix

W.r.t how UE indicate success/completion of GNSS position fix, the following options needs to be further discussed:

Agreement The following alternatives can be considered to inform eNB the success of GNSS measurement at UE side after GNSS measurement in RRC connected. - Alt-1: The UE will report the new GNSS validity duration - Alt-2: The reception of any UL transmission from the UE at eNB after the GNSS measurement

As discussed in previous clauses, the GNSS measurement should be performed upon expiration of previous GNSS validity duration, which can maximize the utilization of each GNSS position fix and simplify the design. Therefore, once the GNSS position fix is failed, the UL synchronization will be lost and UE should go IDLE mode. With this consideration, once a UL transmission is received by eNB after the GNSS measurement, it means UE successfully obtain a new GNSS position which maintains the UL synchronization. That is, the reception of any UL transmission from UE at eNB can implicitly indicates the success of GNSS measurement. From above analysis, both Alt-1 and Alt-2 can work. Alt-2 is more preferred since any UL transmission can work as the success indication, which is simpler and may save signaling overhead.

Proposal 6: The reception of any UL transmission after GNSSS measurement is preferred to indicate success/completion of GNSS position fix without additional signaling overhead and scheduling delay.

Moreover, no matter which of Alt-1 and Alt-2 is finally selected, a restriction for the transmission should be introduced. As discussed previously, UE may lose UL synchronization after a failed GNSS measurement. In this case, UE is not able to transmit a failure indication to network. As a result, network should think the UE failed a GNSS measurement when it does not receive the success indication after the GNSS measurement, e.g., within a period of time. Then, network is able to release the connection to avoid resource waste. With such mechanism, when to send success indication of GNSS measurement should be specified. Otherwise, if the success indication is completely up to UE implementation, it is possible that UE sends a success indication after network determining to release the connection, which causes misalignment between UE and eNB. In order to achieve consensus between UE and eNB on whether a GNSS measurement is failed, restrictions should be specified for when to send success indication of GNSS measurement. A straightforward way is to define a waiting time window to illustrate during which time the success indication should be transmitted. The start of this waiting time window can start at the end of GNSS measurement gap or timer/timing. The length of the waiting time window may be configured by eNB, depending on how long the network will wait after a GNSS measurement. UE should send the success indication, e.g., report of new GNSS validity duration for Alt-1 of any UL transmission for Alt-2, within this waiting time window if the GNSS measurement is successful. If the GNSS measurement is failed, UE will not report the success indication. Note that when Alt-2 is applied, any UL transmissions within the time window may be valid for success indication, including UL transmission without additional DL scheduling (e.g., SR and configured grant PUSCH) and UL transmission with DL scheduling (e.g., PUSCH with dynamic grant). Network will think UE failed the GNSS measurement if it does not receive the success indication by the end of the waiting time window.

Proposal 7: The success indication of GNSS position fix should be transmitted within a time window after GNSS measurement. The GNSS position fix is thought failed if no success indication within the time window.

- The time window starts at the end of GNSS measurement gap or timer/timing.

- The time window length is configured by network.

  - Closed loop time correction

At the RAN1#112, the discussion on closed loop time correction is conducted and the following agreement is achieved.

Agreement At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration. - FFS: with legacy closed loop time correction or enhanced closed loop time correction - This mechanism is enabled/configured by eNB FFS: whether such mechanism will be specified depends on the outcome of this study

              - RAN1 has agreed to study mechanism to allow UL transmission after original GNSS validity duration expires with the help of closed loop time correction. However, the detailed implementation and whether to specify are still FFS. In our view, closed time correction mechanism has already been supported in current system, e.g., MAC CE TA command. However, since the closed loop correction mechanism is controlled by network, current GNSS validity duration which completely determined by UE cannot take this into consideration. Therefore, whether the closed time correction mechanism can extend the time of maintaining UL synchronization should be determined by eNB. To simplify the implementation, it is preferred to reuse current closed loop time correction mechanism and enhance the GNSS validity duration only. Initially, UE still report the GNSS validity duration without consideration of closed loop time correction. If network is able to maintain the UL synchronization after end of original GNSS validity duration, it can indicate UE to extend the GNSS validity duration with certain time duration. UE and eNB can achieve consensus on how long the new GNSS validity duration is. And the triggering mechanism of GNSS measurement gap and GNSS measurement timer/timing introduce in previous clauses, where the start time is set to the end of GNSS validity duration, can be remained unchanged no matter whether closed time correction is enabled.

              - Proposal 8: When closed loop time correction is used to maintain UL synchronization after expiry of original GNSS validity duration, eNB will indicate an extension of GNSS validity duration to UE.

- The GNSS validity duration of current GNSS position fix should be the sum of original GNSS validity duration and the extension indicated by eNB.

- The behaviors upon expiration of GNSS validity duration will keep unchanged no matter whether it is extended by eNB or not.

  - GNSS re-acquisition in C-DRX

              - In inactive state of C-DRX, UE will not detect PDCCH and thus does not receive signaling. Hence, UE re-acquiring GNSS information during inactive state of C-DRX will not interrupt communications, which should be supported. Moreover, it is preferred to specify the time when UE can reacquire the GNSS information to ensure full utilization of GNSS validity. In GNSS re-acquisition based on GNSS measurement gap or GNSS measurement timer, the end of GNSS measurement gap or timer can be naturally set as the start of GNSS validity duration, since UE and eNB can have consensus. While if UE can autonomously re-acquire GNSS at any time during inactive state, the start of GNSS validity duration will be unclear if the start of GNSS validity duration is defined in a similar way. Besides, it is possible that UE re-acquire GNSS very early and the residual GNSS validity duration in active state may be very limited. The GNSS validity duration will be wasted as shown in Figure 2(a). And with very limited GNSS validity duration, the eNB may be difficult to determine whether to schedule new transmission or to trigger a new GNSS measurement. Hence, in order to achieve consensus on GNSS validity and maximize the usage of GNSS information, GNSS position fix acquisition should be performed just before the switch from inactive to active time so that a fresh GNSS information can be obtained by UE as shown in Figure 2(b).

              - (a)

              - (b)

Figure 2 Illustration of GNSS re-acquisition during inactive sate of C-DRX

              - Proposal 9: UE should be supported to re-acquire GNSS during inactive state of C-DRX. The time for GNSS re-acquisition during inactive state of C-DRX should be specified as a time interval right before switch to active time to maximize the usage of GNSS validity duration.

Conclusion

In this contribution, analysis on improved GNSS operation for IoT-NTN is conducted with following proposals and observations:

Proposal 1: The duration of GNSS measurement gap should be equal to GNSS position fix time duration reported by UE with consideration of signaling and resource overhead.

Proposal 2: The start of GNSS measurement gap should be set to the expiration time of GNSS validity duration without delay to minimize spec effort.

Proposal 3: No need to support UE sending a request for network to trigger GNSS measurement aperiodically.

Proposal 4: No need to support additional periodically GNSS measurement on top of agreed timing/timer based GNSS measurement.

              - Proposal 5: For configuration of GNSS measurement timer in connected, the complexity and spec impact should also be minimized.

- The GNSS measurement timer starts at the expiration time of GNSS validity duration.

- The GNSS measurement timer length is equal to GNSS position fix time duration reported by UE at initial access stage.

Proposal 6: The reception of any UL transmission after GNSSS measurement is preferred to indicate success/completion of GNSS position fix without additional signaling overhead and scheduling delay.

Proposal 7: The success indication of GNSS position fix should be transmitted within a time window after GNSS measurement. The GNSS position fix is thought failed if no success indication within the time window.

- The time window starts at the end of GNSS measurement gap or timer/timing.

- The time window length is configured by network.

              - Proposal 8: When closed loop time correction is used to maintain UL synchronization after expiry of original GNSS validity duration, eNB will indicate an extension of GNSS validity duration to UE.

- The GNSS validity duration of current GNSS position fix should be the sum of original GNSS validity duration and the extension indicated by eNB.

- The behaviors upon expiration of GNSS validity duration will keep unchanged no matter whether it is extended by eNB or not.

              - Proposal 9: UE should be supported to re-acquire GNSS during inactive state of C-DRX. The time for GNSS re-acquisition during inactive state of C-DRX should be specified as a time interval right before switch to active time to maximize the usage of GNSS validity duration.

- Reference

- R1-2302120, Feature lead summary #3 of AI 9.11.4 on improved GNSS operations, RAN1#112