- 3GPP TSG RAN WG1 #112bis-e                                                                                      R1-2303299

# E-meeting, April 17th – 26th, 2023

Source:	ZTE

Title:	Discussion on the evaluation methodology of Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite radio interface technology

Agenda Item:	9.15.1

Document for:    Discussion

- Introduction

In RAN#99 meeting, the study item for self-evaluation towards the 3GPP submission of a IMT-2020 satellite radio interface technology is approved. The objectives of this study item are proposed in [1] as follows:

This study item will provide the description of the self-evaluation results towards IMT-2020 submission to ITU-R WP 4B against the technical performance requirements defined by Report ITU-R M.2514, using the evaluation criteria defined in the report, and complete the related compliance template and description templates. The candidate IMT-2020 RIT/SRIT(s) submission by 3GPP based on Rel-17 NTN (including both NR NTN and IoT NTN), will be evaluated and described as part of the study. Detailed objectives of this study item include: - Complete all required submission templates as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc] - Provide self-evaluation results against technical performance requirements for eMBB-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Peak data rate - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - Latency, including user plane latency and control plane latency - Energy efficiency, including both network and device - Mobility - Mobility interruption time - Provide self-evaluation results against technical performance requirements for mMTC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Connection density - Provide self-evaluation results against technical performance requirements for HRC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Reliability - Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4] IoT NTN will at least target self-evaluation against bullets c) and e) technical requirements, and NR NTN will target self-evaluation against all technical requirements (in bullets b) to e)). This study shall start with evaluating features that are supported by Rel-17 NTN (NR NTN + IoT NTN), as relevant for the above aspects.

In this contribution, the evaluation methodology is discussed.

- Discussion of the evaluation methodology

  - Scenarios and Channel model

There are 3 usage scenarios described in [2] for the satellite component of IMT-2020, that’s eMBB-s, mMTC-s, HRC-s. As satellite component of IMT-2020 is expected to be the complementary of terrestrial network, which means the service area of a satellite network is mainly to cover under served and unserved areas of terrestrial network,  all these 3 usage scenarios are suggested to be evaluated in Rural test environment. The channel model for Rural test environment can refer to sector 6.6.6 and 6.6.7.2 in [3].

Proposal 1: The test environment is Rural for all usage scenarios, and channel model for Rural in sector 6.6.6 and 6.6.7.2 in [3] can be used.

  - Satellite and terminal configurations

As described in [2], demonstration of compliance assuming handheld terminals is necessary and sufficient, other evaluations, e.g. for directional and MTD devices, may be provided by the proponent, though are not required. We can focus the self-evaluation on handheld terminals to evaluate against technical performance requirements defined in [2], thus the carrier frequency is assumed to be 2 GHz (S-band), and the bandwidth is up to 30 MHz. For peak data rate evaluation, if necessary, directional terminals such as VSAT can also be optionally considered with carrier frequency 20 GHz for DL and 30 GHz for UL (Ka-band) and bandwidth up to 400 MHz.

Proposal 2: Evaluation for handheld terminals and S-band should be considered as the baseline.

2.2.1 Satellite configuration

As suggested in [2], both GSO and Non-GSO should be considered. Therefore, in this self-evaluation GEO and LEO-600 are preferred to be evaluated. Single satellite can be considered as the starting point, and the beam layout of the satellite can be considered as 19 inner beams with 2 (for FRF=1) or 4 (for FRF=3) tiers of outer beams, details can be found in Figure 6.1.1.1-1 and Figure 6.1.1.1-2 in [4]. The antenna pattern of satellite is considered as the Bessel function provided in 6.4.1 in [3], and the polarization of satellite is circular. The example parameters given in sector 8.2.3 in [2] is related to the Set-1 satellite configuration, so we can consider Set-1 as the baseline. Since the service of satellite should cover a large area, low elevation angle of UE is preferred, thus the central beam center elevation can be considered as 12.5 degree for GEO and 30 degree for LEO. Detailed parameters of satellite can be found in Table 1.

Table 1  Satellite configuration for system level evaluation

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [3] | Section 6.4.1 in [3] |
| polarization |  | Circle | Circle |
| Adjacent beam spacing (ABS) on UV plane |  | - Band : 0.0061 Ka-band: 0.0027 | S-Band : 0.0668 Ka-band: 0.0267 |
| Central beam center elevation |  | 12.5 degrees | 30 degrees |
| Payload characteristics for DL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [3]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

2.2.2 UE configuration

As discussed above, UE is considered as handheld terminal, the parameter is the same as Table 6.1.1.1-3 in [4] and listed in Table 2 below.

Table 2  UE parameters for system level evaluation

| Characteristics | Handheld | VSAT |
| --- | --- | --- |
| Frequency band | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element | Directional Section 6.4.1 of [3] with 60 cm equivalent aperture diameter |
| Polarisation | Linear: +/-45°X-pol | circular |
| Rx Antenna gain | 0 dBi per element | 39.7 dBi |
| Antenna temperature | 290 K | 150 K |
| Noise figure | 7 dB | 1.2 dB |
| Tx transmit power | 200 mW (23 dBm) | 2 W (33 dBm) |
| Tx antenna gain | 0 dBi per element | 43.2 dBi |

2.2.3 Evaluation configuration

For connection density evaluation, the UE should have a density at least 500 per km2, thus 500 per km2 can be the baseline, for other evaluation the UE density is “10 UEs per spot beam”. In addition, FTP3 traffic model can be used for connection density evaluation, and full buffer can be used for other evaluation. The parameters for system level evaluation is listed in Table 3.

For link level evaluation, considering most of the service condition is LOS, channel model can be considered as NTN-CDL-C or NTN-CDL-D provided in section 6.9.1 in [3]. The parameters for link level evaluation is listed in Table 4 which can be referred to section 6.1.2 in [4].

Table 3  Parameters for system level evaluation

| Parameters | Values/Types/Configurations |  |  |
| --- | --- | --- | --- |
|  | Rural-eMBB-s | Rural-mMTC-s | Rural-HRC-s |
| Terminal type | Handheld, VSAT |  |  |
| Satellite orbit configuration | LEO, 600 km altitude GEO, 35786 km altitude |  |  |
| Spot beam pattern and frequency reuse factor | Hexagonal pattern, 19 inner beams, Total beams: 61 beams for FRF=1, 127 beams for FRF=3. |  |  |
| Service link frequency | Handheld: 2 GHz VSAT: 20 GHz for DL, 30 GHz for UL |  |  |
| Channel bandwidth | Handheld: 30 MHz VSAT: 400MHz |  |  |
| Subcarrier spacing | Handheld: 15 kHz VSAT: 60 kHz |  |  |
| Number of symbols per slot | 14 |  |  |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table 1 |  |  |
| UE characteristics | See Table 2 |  |  |
| Beam layout definition | See Table 6.1.1.1-4 in [4] |  |  |
| UE orientation | Handheld: random VSAT: towards the satellite |  |  |
| UE antenna height | 1.5 m |  |  |
| UE deployment | 100% outdoor, randomly and uniformly distributed over the area |  |  |
| UE density | 10 UEs per spot beam | 500 per km2 | 10 UEs per spot beam |
| UE mobility model | For mobility evaluations: Fixed and identical speed of 250 km/h of all UEs, randomly and uniformly distributed direction. For all other evaluations: Stationary | Stationary | Fixed and identical speed of 30 km/h of all UEs, randomly and uniformly distributed direction |
| Traffic model | Full buffer | With layer 2 PDU (Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Packet arrival follows Poisson arrival process for non-full buffer system-level simulation | Full buffer |
| Handover margin | 0 dB |  |  |
| UE attachment | RSRP |  |  |
| Receiver type | MMSE-IRC |  |  |
| Channel estimation | realistic |  |  |
| CSI feedback | Release 15 (impact of RTT should be considered) |  |  |

Table 4  Parameters for link level evaluation

| Parameters | S-band | Ka-band |
| --- | --- | --- |
| Carrier frequency | 2 GHz | DL 20 GHz, UL 30 GHz |
| Channel coding scheme | NR channel coding |  |
| Subcarrier spacing | 15 kHz | 60 kHz |
| Channel estimation | Realistic estimation |  |
| Frequency offset | Residual Frequency error after DL synchronisation: [0.1] ppm assuming UL pre-compensation |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1.1-8 in [4]] |  |
| Frequency tracking | drift pre-compensation is assumed |  |
| UE speed | 3 km/h, 0 km/s (stationary) |  |
| Channel model | NTN-CDL-C or NTN-CDL-D |  |
| Satellite antenna configuration | 1Tx/Rx | 1Tx/Rx |
| UE antenna configuration | S-band: (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| Phase noise Model | S-band phase noise modelling (optional) Ka-band phase noise modelling: phase noise profile according to TR 38.803 |  |

Proposal 3: The assumption listed in Table 1-4 should be considered as the baseline of parameters for self-evaluation.

  - Discussion on characteristics

There are total 14 technical performance requirements proposed in [1] to be evaluated, the detailed description of them can be found in section 7.2 in [2]. Each of the requirements is mapped to one or more of three high level assessment methods (simulation, analytical, inspection) defined in [2], the mapping table is given in sector 8.2.4 in [2] and listed in Table 5 below for convenience.

Table 5  Mapping table of technical performance requirements and assessment methods

| Characteristic for evaluation | High-level assessment method | Related section of Report ITU-R M.2514 |
| --- | --- | --- |
| Peak data rate | Analytical | § 7.2.1 |
| Peak spectral efficiency | Analytical | § 7.2.2 |
| User experienced data rate | Simulation and Analytical | § 7.2.3 |
| 5th percentile spectral efficiency | Simulation | § 7.2.4 |
| Average spectral efficiency | Simulation | § 7.2.5 |
| Area traffic capacity | Simulation and Analytical | § 7.2.6 |
| User plane latency | Analytical and Inspection | § 7.2.7.1 |
| Control plane latency | Analytical and Inspection | § 7.2.7.2 |
| Connection density | Simulation | § 7.2.8 |
| Energy efficiency | Inspection | § 7.2.9 |
| Reliability | Simulation | § 7.2.10 |
| Mobility | Simulation | § 7.2.11 |
| Mobility interruption time | Analytical | § 7.2.12 |
| Bandwidth | Inspection | § 7.2.13 |

For peak data rate and peak spectral efficiency, they can be derived from each other with bandwidth, thus they can be evaluated together. To get higher value of peak data rate, VSAT can be considered as the UE terminal, and correspondingly Ka-band with up to 400 MHz bandwidth should be considered. The required value of peak data rate and peak spectral efficiency for bandwidth up to 400 MHz can be found in Annex 2 in [2]. The assessment method can be analytical method and system level simulation, the system level simulation parameters can be found in Table 1 ~ Table 3.

For User experienced data rate, 5th percentile spectral efficiency, average spectral efficiency and area traffic capacity, system level simulation should be considered, and the parameters should be the same, thus they can jointly evaluated. For these characteristics, UE is considered as handheld, and frequency band is considered as S-band with up to 30 MHz bandwidth.

For user plane latency and control plane latency, only analytical method is considered, and the method can refer to [2].

For connection density, system level simulation is considered, the detailed steps can be found in [5]. This characteristic is mainly evaluated for IoT-NTN, and the UE is considered as handheld and mobility is considered as stationary.

For energy efficiency, inspection method is considered which can refer to section 7.2 in [2].

For reliability and mobility, system level simulation and link level simulation are both considered, UE is considered as handheld, the requirements can refer to [2] and the detailed simulation steps can refer to [5]. The simulation parameter for mobility and reliability can be the same as 5th percentile spectral efficiency evaluation except for UE speed, the difference can refer to parameters of eMBB-s (for mobility simulation) and HRC-s (for reliability simulation) in Table 3.

For mobility interruption time and bandwidth, analytical and inspection method are considered respectively, the method can refer to section 7.2 in [2].

Proposal 4: For peak data rate and peak spectral efficiency evaluation, VSAT and Ka-band with up to 400 MHz should be considered. For other characteristics, handheld and S-band with up to 30 MHz should be considered.

- Conclusions

In this contribution, discussion on the methodology of self-evaluation is provided with following proposals:

Proposal 1: The test environment is Rural for all usage scenarios, and channel model for Rural in sector 6.6.6 and 6.6.7.2 in [3] can be used.

Proposal 2: Evaluation for handheld terminals and S-band should be considered as the baseline.

Proposal 3: The assumption listed in Table 1-4 should be considered as the baseline of parameters for self-evaluation.

Proposal 4: For peak data rate and peak spectral efficiency evaluation, VSAT and Ka-band with up to 400 MHz should be considered. For other characteristics, handheld and S-band with up to 30 MHz should be considered.

Reference

- RP-230736, “Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology”.

- Report ITU-R M.2514-0, “Vision, requirements and evaluation guidelines for satellite radio interface(s) of  IMT-2020”.

- 3GPP TR 38.811, Study on New Radio (NR) to support non-terrestrial networks.

- 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN).

- Report ITU-R M.2412-0, “Guidlines for evaluation of radio interface technologies for IMT-2020”.