3GPP TSG RAN WG1 #112bis-e	R1-2303301

e-Meeting, 17 – 26 April 2023

Agenda item:		9.2.3.1

Source:	CEWiT

Title:	Evaluation on AI/ML for Beam Management

Document for:		Discussion and Decision

# Introduction

In RAN#94 plenary meeting, a new SID on AI/ML for air-interface was approved [1]. One of the objectives of the SID was to evaluate the performance benefits of AI/ML based algorithms for agreed use cases, including CSI feedback enhancements, beam management and positioning accuracy enhancements. For AI/ML based beam management two cases are considered: spatial domain beam prediction and temporal domain beam prediction [2].

Following agreements were made in the RAN1#112 meeting[6].

Agreement

- Further study the impact of quantization error of inputted L1-RSRP (for training and inference) for AI/ML model for beam management.

  - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model.

Agreement

- Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy

.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

- Note: This is only for evaluation discussion

Agreement

- For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

Agreement

- Further study the impact of quantization error of inputted L1-RSRP (for training and inference) for AI/ML model for beam management.

  - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model.

Agreement

- Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy

.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

- Note: This is only for evaluation discussion

Agreement

- For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

Agreement

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study the following options as AI/ML model inputs

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID

    - E.g., measurements of Set B of beams together with default values (e.g. 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector.

    - Detailed assumption can be reported by companies.

  - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: Specification impact can be discussed separately.

Agreement

- Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable)

  - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

- Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

Agreement

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study the following options as AI/ML model inputs

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID

    - E.g., measurements of Set B of beams together with default values (e.g. 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector.

    - Detailed assumption can be reported by companies.

  - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: Specification impact can be discussed separately.

Agreement

- Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable)

  - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

- Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

In this contribution we present our evaluation results of beam prediction in spatial domain and our views on the evaluation methodologies and KPI’s.

# Evaluation Results

In this section we explain the evaluation assumptions and present our results for spatial domain beam prediction.

## Evaluation Assumptions

For generating the dataset, we considered a 21-cell layout with 3GPP Uma scenario with 200m inter-site distance. Using this setup, we performed multiple random UE drops and thereby generating approximately 30000 UE per sectors. Out of the samples generated, 90% of the samples were used for training, and 10% for testing. The UEs were associated to the gNB based on the coupling loss.

The antenna configuration for the gNB and UE are given in the table in Appendix. In the simulations, we assume that the gNB is equipped with a single panel, which can generate a total of 32 CSI-RS beams. The UE is equipped with 2 panels can generate a total of 8 beams. The beam related assumptions are summarized in the table in Appendix.

The dataset consists of L1-RSRP measurements from all the transmit beams. At the UE side, we assume that the UE performs measurement using all the 8 receive beams and then reports back either all the 8 L1-RSRP measurements or reports back the L1-RSRP measurement using the best UE-side Rx beam, where the best UE-side Rx beam is the UE-side Rx beam with the maximum L1-RSRP for a fixed Tx beam.

For the selection of beams for Set B, we assume that Set B is fixed across both training and inference. As for the number of beams pairs in Set B, we consider multiple values to analyze the tradeoff between performance and overhead reduction.

## Model Description

We evaluate the performance of two different models in this contribution,

- model-a which is a low-complexity model and

- model-b which is a higher complexity model.

For both the AI/ML model structure use ReLU as the activation function. The cost function used was categorical cross entropy and an Adam optimizer was used to minimize the loss. The RSRP values were normalized to have zero mean and unit variance before giving it as input to the AI/ML model.

Model-a is a sequential model with two hidden layers and a dropout layer.

Model-b is a CNN with a residual connection.

The details of both the models are summarized in Table 1.

| Parameter | Parameter Value |  |  |
| --- | --- | --- | --- |
| Model description | Model-a 3 dense layers of size Set-B Softmax activation at output layer. |  | Model-b 2 Residual layer Softmax activation at output layer |
| Loss function | Categorical cross entropy |  |  |
| Optimization function | ADAM with learning rate of .001 |  |  |
| Dataset size | ~30000 samples, 90% training, 10% testing |  |  |
| Number of parameters | 82500 | 41090 |  |

Table 1 AI/ML model details

Table 1 AI/ML model details

## Evaluation Results

Based on the agreements and discussions, we report the following KPIs in this document:

- Beam prediction accuracy:

  - Top-1 (%): the percentage of the Top-1 genie-aided beam is Top-1 predicted beam

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams, for K=3, 5, and 10.

  - Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

- CDF of L1-RSRP difference between the predicted best beam and genie-aided best beam.

- Average L1-RSRP difference between the predicted best beam and genie-aided best beam.

- RS overhead reduction:

  - For the definition of RS overhead reduction, we select the given equation,

RS overhead reduction [%] = 1-NM

  - N is the number of beam/beam pairs in Set B

  - M is the total number of beams/beam pairs to be predicted.

The baseline scheme that we use is exhaustive beam sweeping, where the best beam is selected based on the measurement from all the beams on Set A.

For all our evaluations, we trained the model on one single sector, unless otherwise mentioned.

### Results for Tx-beam prediction

In Tx-beam prediction, the UE reports the L1-RSRP using the best UE side Rx side beam. Set-A consists of RSRP measurement for each Tx beam using the corresponding best UE side Rx beam and Set-B is a sub-set of Set-A.

With AI/ML model, the performance of two scenarios are evaluated, in which each scenario has fixed Set-B of beams for training and inference and the size of Set-B is varied across the scenarios. The details of the two scenarios are given below.

Case 1: Set-B has 16 beams. The fixed beam pattern used for this scenario is show in Figure 1.

Case 2: Set-B has 8 beams. The fixed beam pattern used for this scenario is show in Figure 1.

The KPI results for Tx-Beam prediction is summarized in Table 2 for a single sector. In the Tables we show the Top-K beam predication accuracy for K=1, and 3 and the average RSRP difference. The CDF plot for RSRP difference is shown in Figure 2.

Figure 1 Beam patterns for Set-B

Figure 1 Beam patterns for Set-B

Transmit Beams

Selected beam

Not selected beam

Case-1

Transmit Beams

Selected beam

Not selected beam

Case-2

Transmit Beams

Selected beam

Not selected beam

Case-1

Transmit Beams

Selected beam

Not selected beam

Case-2

| Scenario, Set-B pattern Model, | Set sizes |  | Overhead reduction | Average L1-RSRP difference (dB) | Top-1 accuracy | Top-3 accuracy |
| --- | --- | --- | --- | --- | --- | --- |
|  | Set A | Set B |  |  |  |  |
| Baseline | 32 | 32 | 0% | 0 | 100% | 100% |
| 100% outdoor, Case 1, model-a | 32 | 16 | 50% | 0.118 | 81.3% | 96.85% |
| 100% outdoor, Case-1, model-b | 32 | 16 | 50% | 0.097 | 83.68% | 97.55% |
| 100% outdoor, Case 2, model-b | 32 | 8 | 75% | 0.678 | 64.78% | 86.98% |
| 100% outdoor, Case-2, model-b | 32 | 8 | 75% | 0.577 | 67.34% | 88.68% |
| 80%/20% outdoor/indoor, Case 1, model-a | 32 | 16 | 50% | 0.132 | 79.41% | 96.64% |
| 80%/20% outdoor/indoor, Case 1, model-b | 32 | 16 | 50% | 0.132 | 78.81% | 96.98% |
| 80%/20% outdoor/indoor, Case 2, model-a | 32 | 8 | 75% | 0.845 | 59.96% | 84.77% |
| 80%/20% outdoor/indoor, Case 2, model-b | Table 2 Tx-Beam predication results Table 2 Tx-Beam predication results 32 | 8 | 75% | 0.723 | 61.3% | 86.69% |

Figure 2 RSRP difference CDF plots

Figure 2 RSRP difference CDF plots

UMa 100% outdoor UEs

UMa 100% outdoor UEs

UMa 100% outdoor UEs

UMa 100% outdoor UEs

The results show that AI/ML models can achieve performance close to baseline scheme with reduced overhead. For e.g. for 100% outdoor users, AI/ML model can achieve a Top-3 accuracy of 96.85% with an overhead reduction of 50%. Also, there is a marginal performance gain while using a complex model(model-b) as when compared to a simple model. When we compare the performance of the AI/ML model across different outdoor/indoor UEs ration, we notice that the performance of the AI/ML model degrades as the number of indoor users increases.

Observation 1: For a sufficiently large Set-B size, the AI/ML model can perform as good as the baseline scheme with reduced overhead.

Observation 2: The performance of AI/ML degrades as the number of indoor users increases.

### Results for generalization evaluation

In the RAN1#111 meeting, it was agreed that the generalization evaluations need to study three different cases. In this section we present our results for model generalization for the three different cases. For generalization evaluations, we generated dataset for three different configurations by varying the outdoor/indoor users ratios. We generated dataset for three different scenarios by varying the outdoor/indoor distributions. We choose outdoor/indoor ratios of 100%, 80% and 60% for generating three different datasets.

For the sake of clarity, we refer the dataset generated for three different configurations as follows.

- Dataset-1: UMa outdoor/indoor ratio of 100%./0%

- Dataset-2: UMa outdoor/indoor ratio of 80%./20%

- Dataset-3: UMa outdoor/indoor ratio of 60%./40%

- Dataset-4: UMi outdoor/indoor ratio of 100%./0%.

All the other parameters are same for the three configurations and are summarized in the Appendix.

As per the agreement made the previous RAN#1 meetings, we evaluate three different cases of generalization.

- Case 1 The AI/ML model is trained and tested using dataset-2.

- Case 2: The AI/ML model is trained on dataset-1 and tested on dataset-2.

- Case 3: The AI/ML model is trained on dataset-2 and tested on dataset-1.

- Case 4: The AI/ML model is trained on a mix of dataset-1 and dataset-2 and tested on dataset-3.

- Case 5: The AI/ML model is trained on dataset-1 and tested on dataset-4

For all the evaluations here, we use a fixed Set-B pattern which has 16 L1-RSRP values. The details of the Set-B pattern is same as that was described in the previous section.

The results for generalization evaluation are summarized in Table 3. We evaluated the different cases mentioned above using the two models, model-a and model-b, which are described in the previous section. For all the evaluations, we used the data from a single sector for both training and testing. The CDF plots for RSRP difference is shown in Figure 3.

| Generalization Type/Model Type | Top-1 Accuracy | Top-3 Accuracy | Average RSRP Difference (dB) |
| --- | --- | --- | --- |
| Case-1/model-a | 79.41% | 96.64% | 0.132 |
| Case-1/model-b | 78.81% | 96.98% | 0.132 |
| Case-2/model-a | 71.86% | 94.86% | 0.19 |
| Case-2/model-b | 73.62% | 95.33% | 0.19 |
| Case-3/model-a | 74.86% | 96.4% | 0.12 |
| Case-3/model-b | 78.71% | 97.4% | 0.10 |
| Case-4/model-a | 78.82% | 96.54% | 0.13 |
| Case-4/model-b | 82.62% | 97.7% | 0.10 |
| Case-5/model-a | 77.44% | 96.03% | 0.11 |
| Case-5/model-b | 77.65% | 96.75% | 0.10 |

Table 3 Generalization results

Table 3 Generalization results

Figure 3 RSRP CDF plots for generalization evaluation

We observe from the results that, the performance of the model that is trained only on data from one scenario is low when compared to the case when the model is trained on a mix of data from different scenarios. For e.g., for case-2 when the model is trained only on data with 100% outdoor users and then tested on data from 80%/20% outdoor/indoor users, the Top-1 accuracy is 71.86%. But when the training data is a mix of both 100% outdoor users and 60%/40% outdoor users, the Top-1 accuracy becomes 78.82%.

Also, we observed that using a more complex model, model-b, generally increase the prediction accuracy.

Observation 3: Training on a mix of data with different UE distribution can improve the performance of the AI/ML model.

Observation 4: Model trained on dataset generated using UMa channel can be used for dataset generated using UMi channel with minimal performance degradation.

# Conclusion

In this contribution we make the following observations.

Observation 1: For a sufficiently large Set-B size, the AI/ML model can perform as good as the baseline scheme with reduced overhead.

Observation 2: The performance of AI/ML degrades as the number of indoor users increases.

Observation 3: Training on a mix of data from different scenarios can improve the performance of the AI/ML model.

Observation 4: Model trained on dataset generated using UMa channel can be used for dataset generated using UMi channel with minimal performance degradation.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e

- Chairman’s Notes, RAN1#109-e meeting

- Chairman’s Notes, RAN1#110, Toulouse, France, 22-26 August 2022

- Chairman’s Notes, RAN1#110bis-e, e-meeting, 10-19 October 2022

- Chairman’s Notes, RAN1#111, Toulouse, France, 14-18 November 2022

- Chairman’s Notes, RAN1#112, Athens, Greece, 27 February – 4 March 2023

- R1-2207068, Evaluation on AI/ML for beam management, CEWiT

- R1-2210107, Evaluation on AI/ML for beam management, CEWiT

- R1-2212423, Evaluation on AI/ML for beam management, CEWiT

- R1-2301689, Evaluation on AI/ML for beam management, CEWiT

# Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE distribution | - For spatial domain beam prediction: - Option 1: 100% outdoor - Option 2: 80% outdoor - Option 3: 60% outdoor - For time domain prediction: 100% outdoor - 10 UE per sector |
| BS Antenna Configuration | antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Azimuth angles (degrees) = [-78.75, -56.25, -33.75, -11.25, 11.25, 33.75, 56.25, 78.75] Zenith angles (degrees) = [85.5, 94.5, 99, 103.5] |
| UE Antenna Configuration | antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Azimuth angles (degrees) = [ -67.5, -22.5, 22.5, 67.5] |
| BS Tx Power | 40dBm (baseline) |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |