3GPP TSG RAN WG1 #112bis-e	R1-2303303

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.3.2

Source:	CEWiT

Title:	Discussion on subband non-overlapping full duplex

Document for:	Discussion

# Introduction

A new study item on “Study on evolution of NR duplex operation” was approved in RAN#94-e [1]. The discussions started in meeting RAN1#109-e. This contribution discusses the potential areas of study related to non-overlapping subband full duplex (SBFD).

# Enhancements for configuring Sub-band

A node capable of non-overlapping SBFD, can transmit and receive simultaneously in different sub-bands (SBs) in frequency domain. Thus, a gNB capable of SBFD can perform DL and UL at the same time but different sub-bands within the same carrier. In this contribution, we focus on only SBFD operation within a TDD carrier according to the following agreement in RAN1#109-e [2].

Agreement

At least study SBFD operation within a TDD carrier.

Fig. 1 shows an UL SB in the middle part of the carrier, whereas Fig. 2 illustrates another scenario in which the UL SB is at one side of the carrier. Consider a scenario where BWP 1 in Fig. 1 is provided as a DL BWP to a UE. In that case, the complete UL SB and two guard bands are within the BWP given to the UE. Similarly, BWP 2 can be another way of providing a DL BWP to a UE, where a part of the UL SB and only one guard band is contained within the BWP. Again, BWP 3 in Fig. 2, contains a part of the UL SB and only one guard band. In SBFD, the gNB is not expected to perform DL operation within the UL SB. Hence, the portion of DL BWP overlapping with UL SB and guard band will be deactivated for the UE. The SBFD aware UE should be aware of the UL SB and the guard band to avoid unnecessary monitoring for DL reception in the portion of DL BWP overlapping with UL SB and guard band. The frequency domain location of the full UL SB can be provided to the UE. The frequency domain location can be provided as a combination of the starting subcarrier of the UL SB and the bandwidth in terms of subcarriers/RBs/RBGs. The information can be provided as cell specific signal.

Fig. 1:  UL SB in middle part of carrier

Fig. 2:  UL SB at one end of carrier

Observation 1: DL BWPs allocated to UEs overlaps partially or fully with the UL SB and creates unnecessary monitoring at the UE.

Proposal 1: Proposal 1: The frequency domain location of the full UL SB is provided to the SBFD aware UE in terms of starting RB and number of RBs.

Similarly, the SBFD operation can be enabled for a set of time resources, in which the UL SB is active. The SBFD aware UE should be aware of the SBFD active time resources to avoid unnecessary monitoring. The granularity of the SBFD active time resource can be at symbol level or slot level. The advantage of symbol level granularity is increased flexibility in scheduling, but at the cost of signaling overhead. Similarly, the slot level granularity reduces signaling but impacts flexibility. Further, the slot level restricts the activation of SBFD in case a few symbols of the slot are configured for high priority signals. E.g., if SSB is configured in a symbol of the slot then the SBFD cannot be activated in the entire slot. As an alternate method the granularity of SBFD active time resource can be resource type within a slot. E.g., the SBFD (or UL SB) is active in DL symbols of a slot.

Observation 2: Following observations are made about time granularity of SBFD operation.

- Symbol level granularity for SBFD active time resource leads to signaling overhead.

- Slot level granularity impacts flexibility and restricts activation of SBFD in case a few symbols of the slot are configured for high priority signals.

Proposal 2: Support time granularity of SBFD active time resource as resource type within a slot.

The SBFD configuration can be provided to the UE using explicit signalling. In case of explicit signaling the gNB explicitly indicate to the UE the UL SB and SBFD active time resource. The indication should configure the UE to either activate the UL SB for a time resource or skip the operation configured in UL SB, using conventional methods, in the time resource. In the former case the UE update the BW of active UL BWP based on UL SB indication, whereas in the latter the UE adapt the DL BWP excluding the UL SB. Further, the explicit indication helps is conflict resolution by setting the priority level for the indication. E.g., UE skip any DL scheduling overlapping with high priority UL SB.

Observation 3: The explicit indication of SBFD configuration avoids uncertainty, defines UE behaviour and helps in conflict resolution.

Proposal 3: Support explicit indication of SBFD configuration to the UE, where the configuration comprises UL SB, SBFD active time resource and priority level.

The gNB can activate SBFD operation periodically and aperiodically. Therefore, the SBFD active time resource can be periodic and aperiodic. The type can be configured by gNB along with indication of SBFD active time resource. In case of periodic, the indication of SBFD active time resource is semi-static using RRC and it comprises starting time, duration, and periodicity. In case of aperiodic, the configuration comprises type, slot offset, symbol offset, and duration.

Proposal 4: Support periodic and aperiodic activation of SBFD operation

- Periodic configuration consists of starting time, duration, and periodicity.

- Aperiodic configuration comprises slot offset, symbol offset, and duration.

One of the major drawbacks of the SBFD operation is self-interference (SI) created by simultaneous DL transmission and UL reception at gNB. The guard bands can be used between the frequency resources for DL and UL operation to minimize the impact of SI. There are 2 ways to provide information about guard band to the UE:

1. Explicit indication in terms of time and/or frequency

2. As a part of PDSCH rate matching

In the case of the first option, it is seen from the above figures that the guard band is always in conjuction with the UL SB in time domain. Thus, providing explicit time domain information for the guard band when explicit time domain information is already provided for the UL SB is redundant. Similarly, in frequency domain, the guard bands are supposed to start where the UL SBs end. Further, in practical scenarios, the bandwidth of the guard band will be less as compared to the bandwidth of the DL SBs. Thus, the signalling overhead required to signal the guard bands will be less as compared to signalling of DL SBs. In the case of the second option, the guard band is indicated as a part of PDSCH rate matching as reserved resources which cannot be used for DL/UL. Based on this, the following observation and proposal is made.

Proposal 5: Study the following for indication of guard bands:

- Explicit indication in terms of time and/or frequency

  - The time location is not explicitly provided for guard bands.

  - Only the bandwidth of the guard band(s) is provided in terms of subcarriers/RBs/RBGs.

- As a part of PDSCH rate matching

In the above described scenario, the UE can figure out the DL SBs implicitly once it knows the time and frequency locations of the UL SB (and guard). For a UE configured with a DL BWP, it can exclude the UL SB and the guard to implicitly figure out the DL SB(s). Thus, the following proposal is made.

Proposal 6: The frequency location of DL SBs are implicitly derived.

- BWP and TDD Configuration related enhancements

In SBFD, the gNB should be able to schedule DL reception to a UE and UL transmission from another UE simultaneously. However, the existing 5G-NR specification, the gNB has various limitations in scheduling simultaneous UL/DL. In 5G-NR, the configuration provided by TDD_UL_DL_Config_common is common for all UEs in a cell and DL/UL configuration provided for a time resource cannot be overwritten using TDD_UL_DL_Config_dedicated or SFI format. Therefore, the SBFD operation will be limited to the F symbols configured by TDD_UL_DL_Config_common. E.g., if a slot is indicated as DL by TDD_UL_DL_Config_common, then all the UEs in the cell will be DL and the gNB cannot schedule UL transmission from the UEs in that slot. Therefore, gNB cannot operate in SBFD in the slot. Similarly, in 5G-NR specification, the SFI cannot override the DL/UL configuration provided by TDD_UL_DL_Config_dedicated. Hence, the gNB cannot enable SBFD dynamically at a UE if a resource is semi-statically configured as DL/UL.

Observation 4: The SBFD operation is limited to F symbols configured by TDD_UL_DL_Config_common.

Proposal 6: Support TDD configurations provided by TDD_UL_DL_Config_dedicated and SFI formats overriding TDD_UL_DL_Config_common

Further, UE supports TDD configurations and slot formats starting with DL slots/symbols, which again restricts the SBFD operation to certain symbols or certain slot formats. E.g., as shown in Fig. 4,  UE 1 has UL BWP in slot 1 while UE 2 has DL BWP in the same slot. In such a case, it will be beneficial to provide UE 1 with a TDD configuration starting with UL. Instead of slots, the granularity in time can be symbols/groups of slots/frames as well. To allow such flexibility of scheduling and efficient utilization of available bandwidth for SBFD, TDD configurations and SFI formats starting with UL slots/symbols can be introduced.

Observation 5: TDD configurations and SFI formats starting with UL slots/symbols improves flexibility of scheduling SBFD and allows efficient utilization of available bandwidth for SBFD.

Proposal 7: TDD configurations starting with UL slots/symbols are supported.

Fig. 4  Illustration to show need for TDD configuration starting with UL

Currently in NR, the gNB assigns BWPs to UEs in a UE specific way. Multiple BWPs are assigned to a UE in RRC out of which one BWP remains active for a UE at a given instant of time. The gNB can signal the UE to dynamically switch from one BWP to another BWP from the multiple BWPs assigned to the UE in RRC. If the BWP is switched, the UE follows the same TDD configuration that has been provided to it in RRC. However, for efficient SBFD operation, the gNB might need to dynamically change the TDD configuration at the UE along with the BWP switching. E.g., UE1 has active BWP1 when the gNB is operating in non-SBFD mode. Now, the gNB wants to operate in SBFD mode and for that purpose, it needs to switch the active BWP of UE1 to BWP2 with a new TDD configuration to accommodate other UEs with different TDD configurations within the same band. This new TDD configuration allows the gNB to efficiently operate SBFD in the network. Currently in NR, to do this, the gNB has to provide a new TDD configuration using RRC which is semi-static in nature. Instead, the gNB can  provide multiple BWPs to the UE each associated with a different TDD configuration. Whenever, the UE switches the active BWP, the UE uses the TDD configuration associated with the new active BWP. An example is shown in Fig. 3. In Fig. 3a, the DL/UL BWP pair is associated with the configuration DUUUUU while in Fig. 3b,  the DL/UL BWP pair is associated with the configuration DDDDDU. The gNB has the flexibility to choose between the 2 configurations based on traffic requirements. Based on the above justification, the following is proposed.

Fig. 3  Different BWP pairs associated with different TDD configurations for the same UE

Proposal 8:  gNB configuring separate TDD configuration for each BWP pair of the UE is supported.

- Resource allocation

- SPS and CG related enhancements

The following agreement was made in RAN1# 111 [4].

Agreement

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

- Resource allocation in frequency domain including frequency hopping

- Resource allocation in time domain

- Power domain

- Spatial domain

FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Currently in NR, only one frequency domain resource assignment (FDRA) is provided for a particular grant free transmission ID. Once activated, the UE transmits PUSCH with the given resource assignment. However, these UL resources might change in frequency domain dynamically due to changes in SB sizes/location. This can be managed in 2 ways:

a. Different FDRAs for different slot types (SBFD and non-SBFD slots).

b. Provision to change the FDRA of the PUSCH transmission already configured.

Similar changes will be applicable for SPS PDSCH.

Proposal 9: Study FDRA related enhancement for SPS PDSCH and CG PUSCH.

- Repetition of signals/channels

One of the major use cases of SBFD is to enhance UL coverage by boosting UL SNR. The boosting in SNR is done by repetition of UL signals/channels like PUCCH, SRS etc.

Proposal 10: Study enhancements related to repetition of UL signals/channels e.g., PUCCH, SRS etc. for coverage enhancement.

- SBFD operation in SSB symbols

The following agreement was made in RAN1# 111 [4].

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Allowing SBFD operation in SSB symbols will have an impact on initial access especially for legacy UEs. An idle UE performing SSB measurements will experience UE-to-UE inter subband CLI. Further, for legacy UEs, any impact on SSB due to SBFD operation cannot be informed to the UEs. E.g., UL transmission in the UL SB happens in an SSB symbol with a certain SSB ID. A legacy UE will still try to perform SSB measurements in that symbol which is unnecessary. Thus, the following proposal is made.

Proposal 11: SBFD operation is not allowed in SSB symbols.

- Collision handling

The following agreement was made in RAN1# 112 [5].

Agreement

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.

For each option, additional conditions may apply to determine whether the option is applicable.

The following agreement was made in RAN1#110bis-e.

Agreement

Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE

If there are, whether/how to avoid/handle such collision cases (as second step)

If BWP of a UE is overlapping with the subband for SBFD operation, then SBFD operation can create conflicts with signals configured for the UE in the overlapping portion. E.g., UE is scheduled to receive  PDSCH in slot n, which is indicated as SBFD active time resource, and the frequency resources scheduled for PDSCH overlaps with UL SB. In that case, the gNB is expected to perform either transmission of PDSCH in the scheduled resources or UL operation in the UL SB, creating a conflict.

Observation 6: DL Signals/channels scheduled at the UE overlaps partially or fully with UL SB and SBFD active time resources creating conflicts.

The conflict resolution should consider various factors such as importance of the signals/channels creating the conflict, type of configuration/scheduling, etc. The gNB can indicate priority level for the SBFD configuration. E.g., if PRACH is configured in UL SB, then UL SB should be given priority over other DL signals. In that case the gNB indicates the SBFD active time resource overlapping with PRACH occasion as high priority to the UE, so that the UE skip any DL signal/channel overlapping with UL SB.  Further, the configuration of signals/channels to a UE can be done by higher layers or DCI. Similarly, the parameters of SBFD can be configured to the UE semi-statically or dynamically. The priority of signal/channel/parameter varies depending on the mechanism of scheduling/configuring. E.g., in 5G-NR specification, PDSCH scheduled by DCI overrides CSI-RS reception configured by higher layers. Therefore, the collision handling should consider scheduling mechanism of the colliding signal/channel and configuring mechanism of parameters of SBFD. Similarly, if dynamically indicated SBFD configuration conflicts with PDSCH scheduled semis-statically then the dynamic configuration should be prioritized.

Observation 7: Consider the following factors for conflict resolution.

- Priority of signals/channels creating conflict

- Type of configuration and scheduling

Proposal 12: gNB indicating priority for the SBFD configuration is supported.

Proposal 13: Dynamic configuration overriding semi-static configuration is supported.

# Conclusion

The following observations and proposals are made in this contribution,

Observation 1: DL BWPs allocated to UEs overlaps partially or fully with the UL SB and creates unnecessary monitoring at the UE.

Proposal 1: Proposal 1: The frequency domain location of the full UL SB is provided to the SBFD aware UE in terms of starting RB and number of RBs.

Observation 2: Following observations are made about time granularity of SBFD operation.

- Symbol level granularity for SBFD active time resource leads to signaling overhead.

- Slot level granularity impacts flexibility and restricts activation of SBFD in case a few symbols of the slot are configured for high priority signals.

Proposal 2: Support time granularity of SBFD active time resource as resource type within a slot.

Observation 3: The explicit indication of SBFD configuration avoids uncertainty, defines UE behaviour and helps in conflict resolution.

Proposal 3: Support explicit indication of SBFD configuration to the UE, where the configuration comprises UL SB, SBFD active time resource and priority level.

Proposal 4: Support periodic and aperiodic activation of SBFD operation

- Periodic configuration consists of starting time, duration, and periodicity.

- Aperiodic configuration comprises slot offset, symbol offset, and duration.

Proposal 5: Study the following for indication of guard bands:

- Explicit indication in terms of time and/or frequency

  - The time location is not explicitly provided for guard bands.

  - Only the bandwidth of the guard band(s) is provided in terms of subcarriers/RBs/RBGs.

- As a part of PDSCH rate matching

Proposal 6: The frequency location of DL SBs is implicitly derived.

Observation 4: The SBFD operation is limited to F symbols configured by TDD_UL_DL_Config_common.

Proposal 6: Support TDD configurations provided by TDD_UL_DL_Config_dedicated and SFI formats overriding TDD_UL_DL_Config_common

Observation 5: TDD configurations and SFI formats starting with UL slots/symbols improves flexibility of scheduling SBFD and allows efficient utilization of available bandwidth for SBFD.

Proposal 7: TDD configurations starting with UL slots/symbols are supported.

Proposal 8:  gNB configuring separate TDD configuration for each BWP pair of the UE is supported.

Proposal 9: Study FDRA related enhancement for SPS PDSCH and CG PUSCH.

Proposal 10: Study enhancements related to repetition of UL signals/channels e.g., PUCCH, SRS etc. for coverage enhancement.

Proposal 11: SBFD operation is not allowed in SSB symbols.

Observation 6: DL Signals/channels scheduled at the UE overlaps partially or fully with UL SB and SBFD active time resources creating conflicts.

Observation 7: Consider the following factors for conflict resolution.

- Priority of signals/channels creating conflict

- Type of configuration and scheduling

Proposal 12: gNB indicating priority for the SBFD configuration is supported.

Proposal 13: Dynamic configuration overriding semi-static configuration is supported.

# References

- RP-213591, New SI: Study on evolution of NR duplex operation, RAN#94-e, Dec. 2021

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e, May 2022

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110bis-e, October 2022

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111, November 2022

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112, Feb.-March 2023