3GPP TSG RAN WG1 #112-bis-e			R1-2303305

e-Meeting, April 17th –April 26th, 2023.

Source: 	CEWiT

Title:	Discussion on evaluation methodology and initial results for beam 	enhancement on sidelink FR2 operation

Agenda: 	9.4.3

Document for:	Decision

## Introduction

New radio (NR) sidelink (SL) communication is used to share information between device to device without or with only minimal involvement of the gNB. SL is evolved in NR Rel-15 and stabilized in Rel-17 for FR1 operation. But FR2 support for SL is limited. Rel-18 is planning to introduce FR2 enhancements/optimization mainly focusing on beam management aspects. In RAN#97, it is agreed to evaluate the enhancements for FR2 till RAN#98. The detailed objective of the study is as follows [1],

- Study and specify enhanced SL operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18)

  - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

  - Work is limited to the support of SL beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing SL-CSI framework and reusing Uu beam management concepts wherever possible.

    - Beam management in FR2 licensed spectrum considers SL unicast communication only.

Base on this objective in RAN # 110b-e initial simulation scenarios and parameters are agreed for analysing the FR2 enhancements. Following are the agreement from last meeting.

Agreement

- In evaluation methodology for commercial deployment scenario for sidelink operation on FR2

- Reuse indoor layout defined for SL-U with pairs topology and without WiFi nodes

  - FFS: total number of UEs deployed in the layout

  - Companies should report how UEs are paired

- FFS: whether to consider the cluster-based topology defined for SL-U

- Note: for the evaluation, there is no Uu link in this indoor layout

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, reuse layout option 3 in Section A.2.1.1 of TR 36.843 with

- Option 1: 7 macro sites with 3 cells per site

- Option 2: a single site

- Companies should report how UEs are paired

- FFS: total number of UEs deployed in the layout

- FFS: whether Uu and PC5 use same carrier

FFS: ISD for this layout option 3

Agreement

For the indoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the total number of UEs is 12 pairs/20 MHz with scaling factors of 1, ½ or 1/3.

Agreement

For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, the number of UEs per cell is 60 with scaling factors of 1, ½ or 1/3.

Agreement

For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, Uu link has different carrier as PC5 in the simulation is the baseline

- Optional: Uu link has same carrier as PC5 in the simulation.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the outdoor layout, the channel model reuses the procedures and parameters for UMi - Street Canyon specified in TR 38.901.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for the indoor layout, the channel model reuses the procedures and parameters for InH mixed office specified in TR 38.901.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for UE antenna parameters, reuse the antenna element pattern and antenna array configuration for pedestrian UE and cellular UE as in Table 6.1.4-6 and Table 6.1.4-7 of TR 37.885.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, consider at least the following parameters:

- Carrier frequency: 30 GHz

- Sub-carrier spacing: 120 kHz(baseline), 60 kHz(optional)

- Simulation bandwidth: 100 MHz (baseline), 200 MHz (optional)

- UE receiver noise figure: 13 dB (baseline), 10 dB (optional)

- UE Tx power: 23 dBm (EIRP should not exceed 43 dBm)

- UE speed: 3 km/h

Agreement

For the outdoor layout defined in the evaluation methodology for commercial deployment scenario for sidelink operation on FR2, ISD is 200 meters.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, support at least the following traffic model:

- Option 1: periodic traffic mode 3

  - Packet size scaling factor is up to companies’ porting

- Option 2: FTP model 3 with arrival rate satisfying one of the followings:

  - BO low load: 10%-25%

  - BO mid load: 35%-50%

  - BO high load: above 55%

  - Packet size is up to companies’ reporting

- Option 3: XR traffic models including cloud gaming, virtual reality, and augmented reality.

- It is up to each company to use either Option 1 or 2 or 3 or mixed of them.

Agreement

When reporting the simulation results for sidelink operation on FR2, companies should report the used resource allocation scheme.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, performance metric includes UPT, latency and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

FFS: UE satisfaction as section 7.2 in TR 38.838 for XR traffic evaluation.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor layout, do not support UE-to-UE 2D distance smaller than 10m.

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, for outdoor layout, in the pathloss model for UMi – Street Canyon in TR38.901, antenna height of base station (hBS) is replaced by antenna height of UE (hUT).

Agreement

In evaluation methodology for commercial deployment scenario for sidelink operation on FR2, optionally support performance metric of UE satisfaction as section 7.2 in TR38.838 for XR traffic evaluation.

Agreement

For sidelink beam management, RAN1 is to study.

- how transmit beam(s) training and/or receive beam(s) training is performed

- whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified

- the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.)

Agreement

RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance.

Agreement

RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance.

Agreement

RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

This contribution provides further discussion on beam management related aspects over sidelink.

## Beam management for Sidelink

3GPP NR standard has defined beam management procedures, including beam measurement, beam sweeping, beam reporting, and beam determination, for both Time Division Duplexing (TDD) and Frequency Division Duplexing (FDD) systems, in sub 6 and mmWave bands. 3GPP NR has introduced Channel State Information Reference Signals (CSI RSs) and Synchronization Signal Blocks (SS blocks) for beam management in the Downlink (DL) and Sounding Reference Signals (SRSs) for beam management in the Uplink (UL). The illustration of a transmission of the beams between two UEs as follows in the Figure 1.

Till Rel 17, sidelink beam management is optimised for FR1 operation. But in 5G advance use cases more stringent requirements on data rate and reliability are expected from sidelink technology. Therefore, mmWave optimization is important aspect to realise such future use cases.  One of the important aspects to be optimised is beam management capability over sidelink. We can expect more performance improvement with respect to throughput and reliability with the beam management in the sidelink communication system which needs to confirm under different beam management mechanisms in this study. Therefore, it is expected to explicitly study the beam management aspects of sidelink and determine the gaps in specification.

Figure 1: Beams representation between two UEs

## Beam Pairing:

In NR Uu link, the general downlink beam management procedure is that gNB sends downlink reference signals (e.g., SSB, CSI-RS), which are based on transmission configuration indicator (TCI) states and hence is beam based. The transmission of SSB is periodic, while the transmission of CSI-RS could be in periodic, semi-persistence or aperiodic manners.

In initial access, a UE first measures the RSRP of SSB and selects a SSB which applies a particular downlink transmit beam. Then, the UE makes a PRACH transmission on a RACH occasion which is associated with the selected SSB beam. With the PRACH is received by gNB, gNB knows the proper downlink transmit beam to serve this UE.

In 5G NR, as per 3GPP standards set of processes that are defined P-1, P-2 and P-3 for beam management.

P-1 Procedure: P-1 is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s). For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams. For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams.

P-2 Procedure: P-2 is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s). From a possibly smaller set of beams for beam refinement than in P-1. Note that P-2 can be a special case of P-1

P-3 procedure: P-3 is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming.

Following the principle of Uu link beam management, a transmitter UE sends sidelink reference signals and a receiver UE perform the sidelink beam measurement based on the sidelink reference signals. Similar to P-1 procedure in Uu link S-SSB based P-1 procedure can be adopted for initial beam selection in sidelink.

S-SSB will be transmitted by a UE acting as synchronization reference UE (SyncRefUE). This is done primarily for slot level synchronization between V2X devices when there is no base station (BS) (e.g., gNB, eNB or NB) and/or global navigation satellite. The time-frequency structure of S-SSB is shown in Figure 2. In time-domain S-SSB is transmitted in NsymbS-SSB orthogonal frequency division multiplexing (OFDM) symbols, numbered from 0 to NsymbS-SSB-1, where SL primary synchronisation signal (S-PSS) occupies 1st and 2nd symbol, SL secondary synchronisation signal S-SSS occupies 3rd and 4th symbol and physical SL broadcast channel (PSBCH) with associated demodulation reference signal (DM-RS) are mapped on to 0th and the 5th or 7th symbols after the S-SSS depending upon normal or extended cyclic prefix. Last symbol is used as guard period. Value of  NsymbS-SSBis 13 for a normal cyclic prefix and 11 for an extended cyclic prefix. In the frequency domain, S-SSB occupies 132 contiguous subcarriers numbered from 0 to 131. S-SSBs can be transmitted from different panels or beams and P1 procedure can be carried out. For reporting  suitable beam for Rx UE feedback channel can be used similar like HARQ feedback over PSFCH.

Figure 2: SL synchronization signal block structure for normal cyclic prefix

Proposal 1: In sidelink beam management, S-SSB based P-1 procedure for beam selection can be supported.

Proposal 2: Study further methods to enable beam selection and beam reporting mechanisms using S-SSB for P-1 procedure. PSFCH can be used for beam reporting for P1 procedure.

Currently, CSI-RS is being used in NR SL for link adaptation for unicast links based on the reported channel quality indicator (CQI) from receive (Rx) UE to transmit (Tx) UE. CQI is determined based on the channel measurement using CSI-RS transmitted in the physical SL shared channel (PSSCH) region from Tx UE. To better spectral efficiency, rank adaption for unicast is supported by measuring the rank using CSI-RS and is indicated using rank indicator (RI). CQI and RI combined represent the channel state information (CSI) that can be reported by Rx-UE to Tx-UE for link and rank adaption. In NR SL CSI is reported using SL CSI reporting medium access control- control element (MAC-CE) container as shown in Figure 3, RI indicates a value of the rank indicator for SL CSI reporting. The length of the field is 1 bit as up to two layers are supported for SL unicast link. CQI indicates channel quality for SL CSI reporting, the length of the field is 4 bits. R is for reserved bits, set to 0. The existing framework of CSI-RS or with some changes can be used for beam management.

| RI | CQI | R | R | R |
| --- | --- | --- | --- | --- |

Figure 3: SL CSI reporting MAC-CE for channel quality and rank indication

DMRS is used by receiver to produce channel estimates for demodulation of associated physical channel. DMRS design and mapping is specific to each sidelink channels viz PSBCH, PSCCH, PSSCH. The DMRS associated channel (PSBCH/PSCCH/PSSCH) is used to estimate channel coefficients and RSRP. The existing framework of DMRS or with some changes can be used for beam management for sidelink.

Either CSI-RS and/or DMRS can be used for beam refinement at Tx UE and Rx UE can be used as P2 and P3 procedure in NR. New MAC-CE reporting structure should be introduced for reporting P2 beam measurements. For P3 procedure reporting can be disabled.

Proposal 3: Study of beam refinement in sidelink using SL-CSI-RS or PSCCH/PSSCH DMRS for P-2 and P-3 procedures, including possible modifications to legacy SL-CSI-RS configuration and PSCCH/PSSCH DMRS.

Proposal 4: Study further methods to enable beam selection and beam reporting mechanisms using SL-CSI-RS or PSCCH/PSSCH DMRS for P-2 and P-3 procedures.

## Beam Maintenance:

In NR Uu link beam management, after initial beam pairing, a UE has to perform beam maintenance. The purpose of beam maintenance is to refined the initial acquired beam as per the channel selectivity in terms of time and frequency.  Another purpose of beam maintenance is beam switching. If the current serving beam pair becomes worse, then gNB and UE could switch the serving beam pair.

Similar procedures can be adopted for sidelink as well. For beam management purpose P-2 procedure can be evoked using SL-CSI-RS or PSCCH/PSSCH DMRS. P2 procedure refines a SSB beam to a narrower beam-via SL-CSI RS or PSCCH/PSSCH DMRS. The SL-CSI-RS can be configured to by transmitted by Tx UE periodically/aperiodically to acquire suitable beam for beam maintenance.  Further, P3 procedure should be defined to refine UEs receiver beam for given transmitting beam selected using P-2 procedure.

Proposal 5: Study the beam reference signal configurations and feedback reporting mechanisms for the beam switching for beam maintenance in sidelink.

The transmitting UE will transmit beam reference signal (CSI-RS or PSSCH/PSCCH DMRS) as per the configured configurations and expect a feedback of beam indication parameters from the receiving UE to select the best beam(s). The beam indication can be performed over a MAC-CE signalling or using a feedback channel (PSFCH).

The beam indication should include atleast one of the Beam-id, beam-strength and beam-validity. Beam-id denotes the identity of the best beams determined by the receiving UE. This indicates one or more than one best beams based on pre-configured threshold. If multiple beams are to be reported, the length of the field will be N bits for N number of beams. The bit position in the field corresponding to the best beams will be set to 1. In another method, if the single beam is configured to be reported, then the bitmap equal to  log2N bits and value indicated the best beam-id or index pointing to the best beam-id for UE-2. Beam-strength is the measured beam strength. Beam-validity field will indicate the time duration for which the beam will be valid. It can be reported using at least one of the ways as mentioned below, one way is to report its absolute value, which is measured as number of symbols/mini-slots/slots/sub-frames/frames/sec/m-sec or using probability over a time interval e.g., probability of beam validity for successive symbol/mini slots/slots/sub frames/ frame after a symbol/slot/sub frame or frame in which beam measurements is reported. In case of absolute value, it can be quantized using max possible absolute value and pre-configured resolution as given below:

QuantizeOpTabsoluteVresolution

Tabsolute is the maximum value of absolute time, and Vresolution is resolution to indicate validity, which will be pre-configured by the network. Resolution can be in terms of the number of symbols/mini slots/slots/sub frames/frames/sec/ms or micro sec. In another way, UE-1 or network will fix a probability threshold and provided it to UE-2. UE-2 will report a time indication showing beam valid probability above configured threshold. The time indication can be a one-bit indicator for showing probability above or below the threshold or it can be a time window in terms of number of symbols/mini-slots/ slots/sub frames or frames. If multiple beams are above threshold, then there will be multiple validities for multiple reported beams. This can be done by a bitmap for validity with each bit representing about probability value being above/below the probability threshold for a particular beam.

Proposal 6: In sidelink, the beam indication is performed using MAC-CE or PSFCH.

Proposal 7: In sidelink beam management, the beam indication should include at least one of the Beam-id, beam-strength and beam-validity.

## Beam Failure Recovery:

In the NR, UE detects Beam Failure based on L-RSRP measurements over BFR CSI-RS and try to switch to on of the configured set of beams. If a predefined number of Beam Failure is detected, Ue trigger Beam Failure Recovery (BFR) process using fixed PRACH preambles. The predefined number of beam failure to trigger this process is defined by beamFailureInstanceMaxCount in RRC. UE sends PRACH with the ID specified in BFR-SSB-Resource.ra-PreambleIndex.

In SL similar beam failure detection procedure can be adopted. A special semipersistent CSI-RS can be configured with fixed number of failure detection count to trigger BFR. As SL does not have PRACH new mechanism need to derive for BFR procedure. S-SSB or CSI-RS can be reused with additional configurations for BFR which need to be further investigated.

Proposal 8: For beam failure detection special CSI-RS resources should be configure semi persistently with L1-RSRP measurement for detecting failure.

Proposal 9: In SL beam management, for SL BFR different procedures should be investigated including S-SSB based, and CSI-RS based procedure.

## Conclusion:

The contribution draws the following observations and proposals,

Proposal 1: In sidelink beam management, S-SSB based p-1 procedure for beam selection can be supported.

Proposal 2: Study further methods to enable beam selection and beam reporting mechanisms using S-SSB for P-1 procedure. PSFCH can be used for beam reporting for P1 procedure.

Proposal 3: Study of beam refinement in sidelink using SL-CSI-RS or PSCCH/PSSCH DMRS for P-2 and P-3 procedures, including possible modifications to legacy SL-CSI-RS configuration and PSCCH/PSSCH DMRS.

Proposal 4: Study further methods to enable beam selection and beam reporting mechanisms using SL-CSI-RS or PSCCH/PSSCH DMRS for P-2 and P-3 procedures.

Proposal 5: Study the beam reference signal configurations and feedback reporting mechanisms for the beam switching for beam maintenance in sidelink.

Proposal 6: In sidelink, the beam indication is performed using MAC-CE or PSFCH.

Proposal 7: In sidelink beam management, the beam indication should include at least one of the Beam-id, beam-strength and beam-validity.

Proposal 8: For beam failure detection special CSI-RS resources should be configure semi persistently with L1-RSRP measurement for detecting failure.

Proposal 9: In SL beam management, for SL BFR different procedures should be investigated including S-SSB based, and CSI-RS based procedure.

## References

- RP-221938 - Revised WID on NR sidelink evolution

- R1-2301845   FL summary #3 for AI 9.4.3 Enhanced sidelink operation on FR2 licensed spectrum.

- "RAN1 Chair’s Notes" , 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023