3GPP TSG RAN WG1 #112bis-e                                                                               R1-2303313

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.1

Source:	Lenovo

Title:	Channel access mechanism for sidelink on FR1 unlicensed spectrum

Document for:	Discussion

# Introduction

In RAN1#112 meeting, for Rel-18 NR sidelink transmission on unlicensed spectrum (SL-U), the relevant channel access mechanisms have been extensively discussed including some fundamental issues, e.g., CAPC determination, UE-to-UE COT sharing, CPE duration, and CWS adjustment. The reached agreements are listed below.

Agreement The CAPC level that should be used for S-SSB transmissions: - Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission Agreement The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2 Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

In this contribution, we further elaborate our views on channel access mechanism and procedure for sidelink transmission on FR1 unlicensed spectrum.

# Discussion

## Frame based equipment channel access mechanism

Wireless signal transmission on unlicensed spectrum should meet the requirements of regulation subject to the management of the located country/region. According to the regulation on Frame based equipment (FBE) channel access mechanism, a device where the transmit/receive structure has a periodic timing with a declared periodicity equal to the Fixed Frame Period (FFP) between 1 and 10ms, and a single CCA slot, i.e., one-shot CCA with at least 9μs sensing interval. Transmissions can start only at the beginning of the FFP immediately following a successful one-shot CCA. If the FBE device finds the channel occupied, no transmission occurs on that channel during the following FFP. The channel occupancy time (COT) cannot be greater than 95% of the FFP and has to be followed by an Idle Period until the start of the next FFP such that the Idle Period is at least 5% of the COT, with a minimum of 100μs. An FBE structure with a fixed frame period is shown in Figure 1. Additionally, within each COT, the regulation allows multiple DL-to-UL transmission switching subject to a one-shot CCA. There is no requirement of gap between transmission bursts. UEs served by the gNB that wins the contention can transmit within the COT, where the winning gNB is the initiating device and the served UEs are responding devices in FBE. An FBE initiating device may change its FFP at most every 200ms.

Figure 1 Fixed Frame Period Structure for FBE

In Rel-16 NR-U, FBE based channel access is specified and named as semi-static channel acess. Even in Rel-17 URLLC, FBE is supported for IIoT use cases where the operator (the factory) may have the capability to clear the environment to make sure there is no other RAT deployed in the same carrier.

When unlicensed spectrum is used for sidelink transmission, FBE should be considered as one channel access mechanism for sidelink transmission so as to exploit the benefit of avoidance of inter-UE blocking.

Based on above analysis, we have below proposals:

Proposal 1: FBE-based channel access is supported for sidelink transmissions in FR1 unlicensed spectrum.

## Load based equipment channel access mechanism

Compared with FBE, load-based equipment (LBE) channel access procedure is more complicated, such as generation of random backoff counter, contention window size update, channel access priority class and maximum channel occupancy. In Rel-16 NR-U, several types of channel access procedures are specified: Type-1, Type-2A/2B/2C. Different channel access types can be applied for different cases. The detailed description is listed in TS37.213 and not repeated here.

When unlicensed spectrum is used for sidelink transmission, it is straightforward to reuse both Type-1 and Type 2A/2B/2C channel access types for sidelink transmissions. For a UE intending to initiate a COT for sidelink transmission over unlicensed spectrum, a CAPC should be determined before performing Type-1 channel access procedure. Since there are two tables specified in TS37.213 as shown above, Table 4.2.1-1 defined for UL transmission can be reused for sidelink transmission to determine a CAPC.

According to the objective of the Rel-18, gNB initiated channel occupancy cannot be shared with sidelink, however the WID does not specify whether uplink and sidelink transmission can share the same UE initiated channel occupancy duration. Similarly, it is rather easier to define separate uplink and sidelink type-1 channel access procedure within the Rel-18 time frame which means uplink and sidelink does not share the same UE initiated COT.

Proposal 2: Support separate channel access procedure for uplink and sidelink in Rel-18 i.e., uplink and sidelink does not share the same UE initiated COT.

Furthermore, since there are 4 channel access priority classes defined for channel access procedure while 8 L1 priorities included in SCI, a L1 priority to channel access priority class mapping needs to be defined. A simplest way is to map every two consecutive L1 priorities to one channel access priority class so as to establish a linkage between 8 L1 priorities to 4 UL CAPC values defined in Table 4.2.1-1 in TS37.213. One example is shown in below Table 1 by mapping 8 L1 priority levels to4 UL CAPC values.

Table 1 Mapping from traffic priority level to sidelink CAPC value by reusing 4 UL CAPC values

| Traffic priority level | CAPC () |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 3 | 2 |
| 4 | 2 |
| 5 | 3 |
| 6 | 3 |
| 7 | 4 |
| 8 | 4 |

Based on above analysis, we have below proposals:

Proposal 3: From RAN1’s perspective, mapping every two consecutive L1 priorities to one channel access priority class is feasible, mapping every two consecutive L1 priorities to one channel access priority class is feasible for sidelink transmissions in FR1 unlicensed spectrum.

## Burst-based sidelink transmissions

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. Different to SCI decoding-based sensing mechanism, LBT procedure is performed based on energy detection in each sensing slot. In detail, if the detected energy on one channel in one sensing slot is lower than the energy detection threshold, then the channel is deemed as empty or clear or available in that sensing slot; otherwise, the channel is deemed as occupied or non-available in that sensing slot.

For unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT). According to Table 4.2.1-1 in TS37.213, the MCOT is related to channel access priority class (CAPC) value. Consequently, when a UE performs a successful channel access Type 1, the UE can occupy the channel with maximum 2ms, 4ms, 6ms or 10ms, depending on the corresponding CAPC value which is determined based on the traffic priority. The higher the CAPC value, the longer the MCOT on the unlicensed spectrum. As shown in Table 4.2.1-1 in TS37.213, for CAPC value of 3 or 4, the MCOT can be 10ms, which is equivalent to 10 slots in case of 15kHz subcarrier spacing, 20 slots in case of 30kHz subcarrier spacing, 40 slots in case of 60kHz subcarrier spacing.

To avoid any risk of losing the channel, similar to DL burst and UL burst in TS37.213, a sidelink burst can be defined as a set of transmissions from a UE without any gaps greater than 16us. Multiple consecutive slots can be included in the sidelink burst. Each PSCCH/PSSCH starts from symbol 0 of a slot to avoid complicated designs at both Tx UE side and Rx UE side. One example is shown in Figure 2, where a UE initiates a COT via LBT Cat.4 using CAPC 2, then it can contiguously transmit 4 PSCCH/PSSCHs in the four consecutive slots without sensing. So the sidelink burst can include the four slots from symbol 0 of slot n to symbol 9 of slot n+3.

Figure 2 Burst based transmission for sidelink

Proposal 4: For burst-based sidelink transmission, from a Tx UE’s point of view, no gap symbol is included in between any two consecutive slots.

As specified in Rel-16 NR-U and Rel-17 above 52.6GHz, one DCI can be used to schedule multiple PDSCHs or PUSCHs so as not only to save DCI overhead but also avoid the risk of losing the channel over unlicensed spectrum. It is straightforward to extend this multiple PDSCHs or PUSCHs scheduling via a single DCI to sidelink transmission over unlicensed spectrum. With multi-slot scheduling, after initiating a COT via performing channel access Type 1, a UE can transmit a single SCI to schedule multiple consecutive slots for PSSCH transmissions within the COT so that the signaling overhead for PSCCH can be greatly reduced and the saved PSCCH resources can be used for PSSCH transmissions. Furthermore, since the PSSCH transmissions in the multiple consecutive slots are performed within the COT, there is no need to perform LBT between two consecutive slots.

In addition, there are several issues need to be solved if multi-slot scheduling is supported for sidelink evolution:

- The maximum number of slots schedulable by a single SCI needs to be determined based on the increasing SCI payload size and the maximum COT duration, e.g., whether to support max 8 slots scheduled by a single SCI.

- Since Rel-16/17 sidelink supports up to 3 resources reserved within 32 slots for one TB, whether the multiple PSSCHs scheduled by a single SCI are carrying a same TB or different TBs should be known to a UE.

- Existing Rel-16 PSFCH feedback is corresponding to one PSSCH. If multiple PSFCHs are transmitted for multiple PSSCHs carrying a same TB, the PSFCH resource is wasted. In that sense, in case of a same TB repetition on the multiple scheduled PSSCHs, a single bit of HARQ-ACK feedback for the TB carried on one PSFCH is sufficient.

Proposal 5: Multiple PSSCHs scheduled by a single SCI is supported for sidelink transmissions in FR1 unlicensed spectrum.

## Channel access types for sidelink transmissions

### Transmissions on a single channel

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. It does make sense that a Tx UE initiated COT can be shared to one or more Rx UEs when the requirements of COT sharing is satisfied. After initiating a COT by performing Type 1 channel access procedure, Tx UE, named UE1 for simplicity, performs contiguous sidelink transmissions without any gap larger than 16us in time domain, and then can transmit COT sharing information to share some slots within the COT to Rx UE (e.g., UE2) for transmitting PSCCH and associated PSSCH. Upon reception of the COT sharing information from UE1, UE2 can perform Type 2A or Type 2B channel access procedure and transmit PSCCH/PSSCH within the COT if the Type 2A/2B channel access procedure is successful.

Furthermore, if HARQ-ACK feedback is enabled, then PSFCH transmission can be contained within the COT so that an Rx UE can perform Type 2A or Type 2B channel access procedure and transmit PSFCH within the COT if the Type 2A/2B channel access procedure is successful. To leave the gap for Tx/Rx switching and performing Type 2A/2B channel access, one symbol can be reserved as gap before each PSFCH transmission.

In legacy SL operation, a UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period. If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A or a SCI format 2-B has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception.

For Rel-18 sidelink transmission on unlicensed spectrum, by performing channel access Type 1, a UE can initiate a COT with MCOT equal to 2ms, 4ms, 6ms, or 10ms dependent on concrete CAPC value. If periodic PSFCH is configured for unlicensed spectrum as legacy, the COT may be interrupted by the periodic PFSCH occasions. One example is shown in Figure 3 with 4 slots as one PSFCH periodicity. It is noted that the periodic PSFCH occasions may be used by a Rx UE to transmit PSFCH corresponding to PSSCHs transmitted by the COT initiator UE or other UEs other than the COT initiator UE (e.g., a UE sharing the COT for transmitting PSSCHs). Therefore, to support legacy PSFCH operation, it is required that a responding UE can transmit periodic PSFCH(s) in a COT to UE(s) other than the COT initiator UE.

Proposal 6: A responding UE can transmit PSFCH(s) in a COT to UE(s) other than the COT initiator UE.

Figure 3 Periodic PSFCH configuration

On the other hand, considering UE processing capability, it may be not possible for a Rx UE to decode a PSSCH and transmit the corresponding PSFCH within the COT. One example is shown in Figure 4, where CAPAC value of 2 is used for initiating the COT so the MCOT can have 4 slots. Two slots are assumed as the processing delay for PSFCH transmission. When HARQ-ACK feedback is enabled, then PSFCHs corresponding to PSSCHs in slot n and slot n+1 can be transmitted within the COT so that an Rx UE can perform Type 2A or Type 2B channel access procedure for transmitting the PSFCHs within the COT. However, for PSSCHs in both slot n+2 and slot n+3, due to the processing delay, the Rx UE can’t transmit PSFCHs for both PSSCHs within the COT and has to perform Type-1 channel access procedure for transmitting the PSFCHs outside of current COT. For quickly accessing the unlicensed channel, Type-1 channel access procedure with CAPC value of 1 can be adopted for transmitting a PSFCH outside of COT.

Figure 4 PSFCH transmissions inside/outside of COT

Regarding channel access Type 2A/2B/2C for SL, there is one FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16us. There is no such issue in Rel-16 NR-U since the LBT type for UL transmission is indicated in DCI and UE simply follows the indication. However, for Rel-18 SL over unlicensed spectrum, it is not clear which LBT type should be performed in case of 16us gap. We propose specifying UE behaviors by standards and using Type 2C if the corresponding transmission is not longer than 584us or Type 2B otherwise.

Proposal 7: In case of 16us gap, UE applies Type 2C channel access if the corresponding transmission is not longer than 584us or Type 2B otherwise.

### Transmissions on multiple channels

The agreement in RAN1#110bis-e for multi-channel dynamic channel access has established the NR-U UL channel access procedures as the baseline, allowing to further study enhancements for SL-U operation.

In our opinion, a technically reasonable enhancement is the case where a Tx UE intends to transmit different transport blocks on multiple LBT channels and all LBT channels corresponding to a transport block pass the LBT.

However, before discussing channel access for such a case, RAN1 needs to have a technical agreement whether transmissions of multiple TBs over different LBT channels in a slot is supported or not. In our understanding, there is no similar behaviour supported in NR-U or SL-licensed so far. We also note that transmitting multiple TBs in the same slot may have consequences on the required scheduling flexibility transmit power and the CM/PAPR of the resulting transmission if, and such a behaviour may need a feasibility confirmation by RAN4. This may result in constraints to the scheduling flexibility that render this feature overall unattractive.

Proposal 8: Before discussing relaxations of channel access behaviour for SL-U in case of multiple TB transmissions on a carrier, RAN1 needs to have the technical discussion and agreement whether such a new transmission behaviour will be supported. This may involve RAN4's feedback on the feasibility and corresponding constraints.

## UE-to-UE COT sharing operation

Up to and including NR Release 17, NR-U has provided mechanisms to operate NR-RAT in unlicensed spectrum to share a channel occupancy time (COT) with another device. The benefit of such COT sharing is that a device initiating a COT can grant another device the right to use the initiated COT without requiring the target device to initiate its own COT. Initiating a COT usually involves a certain sensing time before transmissions are allowed, while starting transmission in a shared COT requires only a fraction of the sensing time required for initiating a COT.

A COT is characterized by one or more of the following properties:

- COT Initiator: The node initiating a COT, e.g. following a channel access procedure as specififed in 3GPP TS 37.213

- Channel Access Priority Class: One or more classes that imply e.g. required sensing duration when initiating a COT, MCOT, e.g. as defined in 3GPP TS 37.213.

- MCOT: The maximum COT duration of a channel occupancy, counted or measured from the first transmission after initiation of the COT.

In the traditional cellular topology with a centralized scheduler, usually a gNB initiates a COT and may share this COT with one or more UEs; or a UE initiates a COT and could share this COT with a gNB. The sharing of a COT is indicated by a specific field in downlink control information or uplink control information. The COT sharing indicator may include means to indicate the maximum remaining duration of the COT being shared.

The following definitions are used for the purpose of sidelink channel access mechanisms:

Tx UE: A UE that transmits a COT sharing indicator via a sidelink connection

Rx UE: A UE that receives a COT sharing indicator via a sidelink connection

COT Initiator: A sidelink device that initiated a channel occupancy e.g., Tx UE

COT Donor: A sidelink device that transmits a COT sharing indicator, e.g. a Tx UE. The COT Donor may be identical to the COT Initiator

COT Recipient: A sidelink device that receives a COT sharing indicator, e.g. a Rx UE

The UE-to-UE COT sharing principles should take into consideration that the COT initiator which is the Tx UE may have connection with multiple Rx UEs or destinations as shown in Figure 5. The COT initiator or the Tx UE could share its initiated COT with multiple Rx UEs or destinations. The COT recipient acting as Rx UEs after receiving the COT sharing indicator could transmit data or feedback in the remaining channel duration.

Figure 5 UE to UE COT sharing

The UE-to-UE COT sharing operation requires new SCI format to incorporate COT sharing indicator similar to DCI format 2_0 and further discussions are necessary how to transmit the COT sharing indicator to multiple Rx UEs or destination ids. The COT sharing indictor should include the remaining duration in terms of number of physical slots, channel access priority value, destination id of the COT recipient. If there are more than one COT recipients, then RAN1 should discuss how to signal more than one destination ids along with the COT sharing indicator. Hence how to define one to one and one to many of the SCI format carrying the COT sharing indicator should be discussed in RAN1.

COT initiator can share the COT sharing indicator together with a destination ID and in such case COT recipient may transmit a unicast PSSCH towards the COT initiator in the shared COT. However, we need to restrict the COT recipient for one-to-many case, to limit the collision in the shared COT. COT sharing indicator can be restricted using a range value i.e., communication range UEs within the communication range may become the COT recipient which further utilizes the remaining channel occupancy duration indicated in the COT sharing indicator for transmission. UEs outside the communication range does not become the COT recipient which means UEs outside the communication range may not use the remaining channel occupancy duration for transmission.

Proposal 9: COT sharing indication includes remaining duration in terms of number of physical slots, channel access priority value, destination id and/or range of the COT recipient.

Proposal 10: COT initiator should have the flexibility to transmit COT sharing indicator to a one-one or one to many UEs/destinations

COT initiator which is the Tx UE should have the flexibility to transmit unicast, groupcast or broadcast data in the initiated COT and if there are no more data to be transmitted within the COT may share the initiated COT to multiple Rx UEs or destination ids. COT recipient should have the flexibility to use the shared COT to transmit unicast, groupcast, broadcast data.

Proposal 11: COT recipient should have the flexibility to use the shared COT to transmit unicast, groupcast, broadcast data

COT recipient need to transmit PSCCH/PSSCH, PSFCH in the shared COT to any UEs or destination ids unless the one of the transmissions is made to UE or source-destination id that provided the COT sharing indicator in that shared COT

Proposal 12: COT recipient could use the shared COT to make PSSCH, PSFCH transmissions to any UEs or destination ids with a restriction that at least one transmission is to be made to the UE or source-destination id that provided the COT sharing indicator

For the detailed COT recipient discussion, previous agreement is captured here

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

L2 Source ID and L2 destination ID are each 24 bit provided by higher layer and transmitted in the MAC header, however the physical layer transmits 16 bit L1 destination ID and 8 bit L1 source ID that are truncated portion of the L2 IDs. 2nd SCI includes 16 bit L1 destination ID and 8 bit L1 source ID. SCI transmitting COT sharing indicator includes one pair of Source-destination IDs. Providing other additional IDs supported by that UE-1 via MAC CE increases the resource overhead. Also transmitting through MAC CE increases the latency and, in such case, additional IDs may be decoded by the COT recipients later compared to the COT sharing indicator transmitted using SCI. In order to avoid explicit transmission of additional IDs using MAC CE, additional IDs are provided using a combination of explicit or implicit method to save resource overhead and latency.

If COT initiator shares the COT sharing indicator immediately after initiating the channel occupancy duration, then SCI carrying L1 IDs transmitted by the COT initiator to schedule PSCCH/PSSCH transmission within the channel occupancy duration can be a potential additional IDs implicitly.

Source and destination IDs in SCIs transmitted from the slot where the COT sharing indicator is transmitted until the slot where the COT duration is shared to the recipient are taken as forward indications for these additional source and destination IDs.

Additionally, in case the COT initiator would want to selectively choose the additional IDs then an additional bit may be included in SCI in each slot to inform when the source-destination pair included in that SCI can be considered as additional IDs to the COT recipients.

Figure 6: Forward indication to provide additional L1 IDs

Proposal 13: Support additional IDs transmission to the COT recipient in the same COT implicitly

Proposal 14: SCIs transmitted by the COT initiator containing source-destination IDs in every slot can be leveraged to provide additional IDs to the COT recipients implicitly

The agreement from RAN1#112 defines a responding UE as a target of a PSCCH/PSSCH transmission of a shared COT. A prior agreement states that a responding UE may use the shared COT for S-SSB transmissions. Therefore a synchronization reference UE would only be a responding UE (and therefore able to share the COT) if it has been a target of the initiating UE.

Observation 1: By existing agreements, a UE would only be allowed to transmit S-SSB by COT sharing if the UE has been a target of the initiating UE.

In our opinion this is too restrictive. In order to support efficient transmissions of S-SSB in a shared COT, we think one of the following mechanisms should be adopted:

- Option A: A responding UE over a shared COT for purposes of S-SSB transmissions can be any UE receiving the COT sharing indicator

- Option B: The ‘additional ID’ functionality with the COT sharing indicator is supported and indicates one or more SLSS IDs + Iic to identify which synchronisation reference UE is allowed to use the shared COT for transmissions of S-SSB

- Note: Neither of the proposed options are meant to allow such a UE to transmit signals/channels other than S-SSB, unless the UE is a target of a PSCCH/PSSCH transmission by a COT initiator.

Option A would allow any UE receiving the COT sharing indicator to transmit S-SSB by COT sharing. Option B would allow that only if the UE’s SLSS ID and Iic match one of the ‘additional ID(s)’.

Proposal 15: In order to support efficient transmissions of S-SSB in a shared COT, we think one of the following mechanisms should be adopted:

- Option A: A responding UE over a shared COT for purposes of S-SSB transmissions can be any UE receiving the COT sharing indicator

- Option B: The ‘additional ID’ functionality with the COT sharing indicator is supported and indicates one or more SLSS IDs + Iic to identify which synchronisation reference UE is allowed to use the shared COT for transmissions of S-SSB

- Note: Neither of the proposed options are meant to allow such a UE to transmit signals/channels other than S-SSB, unless the UE is a target of a PSCCH/PSSCH transmission by a COT initiator.

In a sidelink environment, a UE have sidelink connections to and from multiple UEs. Consequently, a UE may receive COT sharing indicators from many COT donor UEs. Since each of the indicated shared COTs results from a different donor device that may have e.g. remaining COT durations, priority class. COT recipient needs to select which of the indicated shared COTs to use and to abide by the rules pertaining to the selected COT.

Proposal 16: RAN1 needs to study mechanism for COT recipient to select one COT sharing indicator/COT donor

## Shared resource indication

In Rel-16 NR-U, after initiating a COT by performing LBT Cat.4, a UE can perform contiguous uplink transmissions without any gap in time domain, and then share a part of or the whole remaining COT to gNB for downlink transmission. The CG-UCI includes a field of COT sharing information indicating the shared resource. In detail, COT sharing information transmitted in slot n indicates the shared resource started from slot n+O with a duration of D slots, where the values of O and D are joint encoded.

Regarding the shared resource indication for sidelink transmission over FR1 unlicensed spectrum, the straightforward way is to reuse what Rel-16 NR-U has specified, i.e., the SCI transmitted in slot n includes a field of COT sharing information indicating the shared resource started from slot n+O with a duration of D slots, where the values of O and D are joint encoded.

As specified in TS38.212 for Rel-16 NR-U, in order to avoid any ambiguity for COT sharing, it clearly states that, if a UE indicates COT sharing other than "no sharing" in a CG PUSCH within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent CG PUSCHs, if any, occurring within the same UE's initiated COT such that the same DL starting point and duration are maintained. Hence, the similar mechanism should be reused for Rel-18 unlicensed sidelink transmission, i.e., for COT sharing, if a UE indicates COT sharing other than "no sharing" in a SCI format within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent transmission occasions, if any, occurring within the same UE's initiated COT such that the same starting point and duration of the shared resources are maintained.

Based on above analysis, we have below proposals:

Proposal 17: COT sharing information in SCI jointly indicates the starting slot and the number of consecutive slots of shared resources.

Proposal 18: If a UE indicates COT sharing other than "no sharing" in a SCI format within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent transmission occasions, if any, occurring within the same UE's initiated COT such that the same starting point and duration of the shared resources are maintained.

## Type 2A channel access for S-SSB, PSFCH

Even though there has been no official agreement in RAN1#110bis-e for S-SSB and PSFCH channel access based on Type 2A, we think the FL proposal Proposal 3 (VI) [from R1-2210709] is a reasonable step forward, however we think that the same Type 2A based channel access should be supported for PSFCH and S-SSB.

Proposal 19: Type 2A channel access procedure is applicable for S-SSB and PSFCH transmissions for a UE without a shared channel occupancy, with a restriction on the frequency and total duration of transmissions accessing the channel with such Type 2A usage in a time interval. The detailed restriction criteria are FFS.

- If the restriction criteria cannot be met, Type 1 channel access is applicable for the S-SSB/PSFCH transmission(s) in question.

- FFS: whether Type 2 channel access procedures can be applied to groupcast option 1 (if supported)

- FFS: how to define the value of  within the energy detection threshold calculation for S-SSB and PSFCH transmissions

## Contention window size update

The type-1 channel access describing contention window size (CWS) adaptation procedure described in 3GPP TS 37.213 for NR-U specifically designed for unicast PDSCH, PUSCH transmission. The NR-U contention window size adaptation procedure considers amount of HARQ feedback reception from TB, CBG based transmission within the reference duration. However, such CWS adjustment procedure should be studied for sidelink channel access mechanism considering unicast, groupcast and broadcast type of PSCCH/PSSCH transmission, transmission in a reference duration to multiple Rx UEs/destination ids and different HARQ feedback types especially for groupcast type e.g., Groupcast HARQ feedback option 1 (common NACK), Groupcast HARQ feedback option 2(dedicated ACK/NACK) defined in NR Rel16 Sidelink.

Existing uplink containing no HARQ-ACK procedure for the contention window size update may not be suitable for sidelink considering that in sidelink there are different reasons for the lack of HARQ feedback e.g., groupcast option-1.

Observation 2: The uplink contention window size update procedure cannot be directly applied to sidelink.

Proposal 20: RAN1 to study on the contention window size update procedure considering different cast type, transmission/reception from multiple Rx UEs or destinations and different HARQ feedback types e.g., groupcast option-1.

While some enhancements for CWS update are required, even if HARQ-ACK feedback is available for unicast transmissions, the corresponding PSFCH transmission may be dropped if the associated priority indicated in the SCI is a high value (=low priority). While this is not a problem in Rel-17 sidelink, this may have an impact to unlicensed channels since the dropped PSFCH would have been important for CWS adjustment purposes (i.e. to increase or reset CWS, depending on the HARQ-ACK feedback). One way to solve this would be to set the priority field in the SCI to a very low level, i.e. reducing the risk of dropping, however this will have serious implications since the priority field is defined in 38.321 as reflecting the “value of the highest priority of the logical channel(s), if any, and a MAC CE, if included, in the MAC PDU”.

Consequently we think that another mechanism is need to protect HARQ-ACK from dropping if it is associated with PSSCH in the reference window. We therefore propose to include a 1-bit indicator in the SCI to indicate if the corresponding PSSCH is within the UE’s reference window. An Rx UE would then use the lowest priority value for the corresponding PSFCH when determining the priority of PSFCH. The value indicated in the priority field is still used for higher layer purposes.

Proposal 21: To avoid priority-based dropping of HARQ-ACK associated with a CWS reference window, a Tx UE includes a 1-bit field to indicate if the corresponding PSFCH falls within the TX UE’s reference window. If so, an Rx UE uses the lowest priority value for the corresponding PSFCH priority determination (instead of the 3-bit field in the SCI).

For the detailed contention window mechanism, earlier RAN1 agreement is as follows:

Agreement - RAN1 is to study the definition of a “SL reference duration” following the NR-U principle and RAN1 is to agree on the definition before down-selection to an option for CW adjustment for SL HARQ-ACK feedback enabled/disabled and each cast type - In Type 1 SL channel access procedure, further study the following cases and options. Other options are not precluded. - CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled): - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value. - Option 5: If a collision indicator is received, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - FFS: the (pre-)configuration ratio values - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p; otherwise CWp is increased. - FFS whether groupcast option 1 (NACK-only) with SL-HARQ feedback enabled can be supported for SL-U. If supported, further study the following options (at least if all transmissions within the latest SL reference duration are groupcast option 1 transmissions) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: - If ‘NACK’ or a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - When neither ‘NACK’ nor a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, - Option A: CWp is reset to CWmin,p for every priority class p∈1,2,3,4. - Option B: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 3: An ACK-only procedure is used instead of a NACK-only procedure. In this case, if at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p, otherwise CWp is increased - Option 4: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 5 (option 3+legacy): ACK feedback is performed when a TB is successfully decoded in addition to the legacy NACK-only procedure. In this case, if ACK only is received related to any transmissions within the latest SL reference duration then CWp=CWmin,p, otherwise CWp is increased. - CW adjustment for unicast with SL-HARQ feedback enabled (at least In case only unicast PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 2: If at least one ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p ; otherwise CWp is increased. - FFS the case when UE is operating with different SL-HARQ feedback schemes (e.g., UE has concurrent broadcast transmission + unicast with SL-HARQ enabled, or GC option 1 + GC option 2, etc in the SL reference duration).

Especially for cases when no feedback is available, we think that we should resort to some mechanism to update the contention window. Otherwise we see the substantial risk that SL UEs might try to cheat the system e.g. by consistently disabling HARQ-ACK feedback to keep the CWS at its minimum interval. Therefore we support and prefer the following:

- CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled):

  - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U

  - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value.

- CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration):

  - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

    - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’

    - FFS: how to calculate the ratio

    - FFS: the (pre-)configuration ratio values

- Support groupcast option 1 (NACK-only) with SL-HARQ feedback enabled for SL-U.

  - Option 4: CW is adjusted according to CR/CBR measurement

Proposal 22: Support the following contention window update mechanisms:

- CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled):

  - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U

  - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value.

- CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration):

  - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

    - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’

    - FFS: how to calculate the ratio

    - FFS: the (pre-)configuration ratio values

- Support groupcast option 1 (NACK-only) with SL-HARQ feedback enabled for SL-U.

  - Option 4: CW is adjusted according to CR/CBR measurement

## HARQ feedback delay indicator

Non-numerical value as feedback timing was introduced which eventually postponed the transmission of the generated HARQ feedback while a subsequent HARQ transmission of numerical value as part of the DL DCI triggers the transmission of the previously generated HARQ feedback for those HARQ processes that was transmitted using non-numerical value indication. The Rel-16 sidelink HARQ feedback reporting may get delayed due to LBT failures and channel occupancy limitation. Such delay in the sidelink HARQ feedback reception at the Tx UE may also affect the reporting of SL HARQ feedback to gNB.

Currently the feedback timing for sidelink transmission is (pre)configured in a resource pool and not dynamically signalled and there should be some signalling enhancement needed to dynamically indicate the non-numerical feedback timing value to the Rx UEs or destinations to delay the transmission of SL HARQ feedback reporting. Further a trigger needs to be subsequently transmitted to request the transmission of SL HARQ feedback of those SL HARQ process previously indicated with non-numerical HARQ feedback timing value.  Such mechanism may also beneficial for gNB to delay the HARQ feedback transmission from a Tx UE using the non-numerical HARQ feedback timing value in DCI format 3_0.

Proposal 23: Support delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value dynamically indicated in SCI

## S-SSB transmissions

When UE intends to synchronize via S-SSB, it will try to find a synchronization reference with a priority level as high as possible. Therefore, we should provide an earlier channel access opportunity for a synchronization reference with a higher priority level. Such a design can be reflected by means of setting CPE starting positions based on the priority levels of synchronization reference.

Proposal 24: For S-SSB transmission in SL-U, CPE starting positions can be based on priority levels of synchronization reference, which can be obtained by UE based on (pre-)configuration.

## CP Extension (CPE)

Related to CP extension, below agreement were achieved from RAN1#111 and RAN1#112.

Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

To (pre-)configured multiple CPE starting positions in a resource pool for PSCCH/PSSCH transmission according to various priority level, it is beneficial to the high priority transmission with longer CPE length occupying channel (RB set) earlier than the low priority transmission with shorter CPE length in mode 2. This kind of enchantment can avoid the potential hidden node issue and be good for pre-empting low priority transmission in some resource limitation scenario.

Proposal 25: Support to (pre-)configure the multiple CPE starting positions according to priority level (e.g., CAPC or L1)

When multiple CPE starting positions are (pre-)configured according to priority level and multiple UEs with difference priority level selecting orthogonal frequency resources (i.e., FDMed resources) in the same slot, UE’s transmission with short CPE length will be blocked by the UE’s transmission with long CPE length. In this way, to selecting a default CPE starting position for multiple UEs with FDMed resources based on resource pool (pre-)configuration in mode 2 is needed.

Proposal 26: Support to define a criteria for selecting a default CPE starting position in mode 2.

The similar problem also appears in mode 1, network can indicate a common CPE starting position for multiple UEs scheduled in same slot with orthogonal frequency resources (i.e., FDMed resources). There is no doubt this increases the resource efficiency in system point of view.

Proposal 27: Support to indicate a certain CPE starting position for transmission UE in mode 1.

## LBT impact on resource selection

As shown in figure 7, based on legacy resource selection mechanism in SL communication mode 2, a UE performs resource selection at time slot n. The candidate resources within resource selection window [n+T1， n+T2] are determined based on the monitoring result within sensing window [n-T0, n-Tproc,0). The UE will exclude the reserved resources within resource selection window (in blue) and report available resources as candidate resources to higher layer.

Figure 7 Sensing based resource selection in sidelink communication

In this way, the resources 1,2,3,4 (in yellow) may be determined as candidate resources and are reported to higher layer. But for unlicensed band, to consider the LBT mechanism:

- When a resource (e.g., resource 1 or 2) is selected before reserved resource, the transmission on selected resource may have impact on the LBT of transmission on reserved resource.

- When a resource (e.g., resource 3 or 4) is selected after reserved resource, the transmission on reserved resource may have impact on the LBT of transmission on selected resource.

The legacy resource selection mechanism needs to be enhanced with considering LBT mechanism. For example,

- To exclude the resource before the reserved resource based on the LBT type 1 information of the reserved resource.

- To exclude the resource after the reserved resource based the LBT type 1 information of the candidate resource.

Here, the LBT type 1 information of the reserved resource is received from inter-UE’s SCI and detected by physical layer, so the resource exclusion procedure should be performed by L1.

Proposal 29: RAN1 study the LBT impact on resource selection with below enhancements:

- To exclude the resource before a reserved resource based on the LBT type 1 information of the reserved resource.

- To exclude the resource after a reserved resource based the LBT type 1 information of the candidate resource.

In addition, to further consider the COT sharing mechanism on unlicensed band, the following aspects can be studied:

- If a COT initialized for transmission on reserved resource and the COT can be shared to resource selection UE, whether the resource 3 or 4 can be regarded as the candidate resource reported to higher layer.

- If a resource selection UE performs transmission on resource 1 or 2 and initialized COT by the resource selection UE can be shared to the resource reservation UE, whether the resource 1 or 2 can be regarded as the candidate resource reported to higher layer.

Proposal 30: Study following aspects with considering the COT sharing mechanism:

- If a COT initialized for transmission on reserved resource and the COT can be shared to resource selection UE, whether the resource 3 or 4 can be regarded as the candidate resource reported to higher layer.

- If a resource selection UE performs transmission on resource 1 or 2 and initialized COT by the resource selection UE can be shared to the resource reservation UE, whether the resource 1 or 2 can be regarded as the candidate resource reported to higher layer.

Also, LBT needs to be performed before transmitting on the reserved resource and the LBT outcome on the reserved resource is not guaranteed. Hence, excluding candidate resources before and after the reserved resource may lead to overall resource in-efficiency. To avoid resource in-efficiency, following two methods can be further investigated.

- UE can select candidate resource before and after the reserved resources and while allowing COT initiated UE to share COT to the UE that reserved resource. Hence, when reserved resource is overlapped with the UE’s transmission resource, UE may perform COT sharing allowing transmission on the reserved resources

- UE can select candidate resource before and after the reserved resources while COT initiated UE may perform type 2 LBT on the reserved resource by selecting one of the multiple CPE starting position on the reserved resource to check whether the UE that reserved resource is transmitting based on the LBT outcome.

Observation 3: LBT outcome on the reserved resource is not guaranteed and excluding such resource resource in the candidate resource selection may lead to resource in-efficiency

Proposal 31: Support allowing COT initiated UE to share channel occupancy duration to the UE that reserved resource

Proposal 32: Support multiple CPE starting position on the reserved resource while allowing COT initiated UE to select one of the starting positions on the reserved resource for type 2 LBT to check the LBT outcome and transmission status of the UE on the reserved resource.

# Conclusion

In this contribution, we focus on the channel access mechanism and procedure for sidelink transmission on FR1 unlicensed spectrum and have below observations and proposals:

Proposal 1: FBE-based channel access is supported for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 2: Support separate channel access procedure for uplink and sidelink in Rel-18 i.e., uplink and sidelink does not share the same UE initiated COT.

Proposal 3: From RAN1’s perspective, mapping every two consecutive L1 priorities to one channel access priority class is feasible, mapping every two consecutive L1 priorities to one channel access priority class is feasible for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 4: For burst-based sidelink transmission, from a Tx UE’s point of view, no gap symbol is included in between any two consecutive slots.

Proposal 5: Multiple PSSCHs scheduled by a single SCI is supported for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 6: A responding UE can transmit PSFCH(s) in a COT to UE(s) other than the COT initiator UE.

Proposal 7: In case of 16us gap, UE applies Type 2C channel access if the corresponding transmission is not longer than 584us or Type 2B otherwise.

Proposal 8: Before discussing relaxations of channel access behaviour for SL-U in case of multiple TB transmissions on a carrier, RAN1 needs to have the technical discussion and agreement whether such a new transmission behaviour will be supported. This may involve RAN4's feedback on the feasibility and corresponding constraints.

Proposal 9: COT sharing indication includes remaining duration in terms of number of physical slots, channel access priority value, destination id and/or range of the COT recipient.

Proposal 10: COT initiator should have the flexibility to transmit COT sharing indicator to a one-one or one to many UEs/destinations

Proposal 11: COT recipient should have the flexibility to use the shared COT to transmit unicast, groupcast, broadcast data

Proposal 12: COT recipient could use the shared COT to make PSSCH, PSFCH transmissions to any UEs or destination ids with a restriction that at least one transmission is to be made to the UE or source-destination id that provided the COT sharing indicator

Proposal 13: Support additional IDs transmission to the COT recipient in the same COT implicitly

Proposal 14: SCIs transmitted by the COT initiator containing source-destination IDs in every slot can be leveraged to provide additional IDs to the COT recipients implicitly

Observation 1: By existing agreements, a UE would only be allowed to transmit S-SSB by COT sharing if the UE has been a target of the initiating UE.

Proposal 15: In order to support efficient transmissions of S-SSB in a shared COT, we think one of the following mechanisms should be adopted:

- Option A: A responding UE over a shared COT for purposes of S-SSB transmissions can be any UE receiving the COT sharing indicator

- Option B: The ‘additional ID’ functionality with the COT sharing indicator is supported and indicates one or more SLSS IDs + Iic to identify which synchronisation reference UE is allowed to use the shared COT for transmissions of S-SSB

- Note: Neither of the proposed options are meant to allow such a UE to transmit signals/channels other than S-SSB, unless the UE is a target of a PSCCH/PSSCH transmission by a COT initiator.

Proposal 16: RAN1 needs to study mechanism for COT recipient to select one COT sharing indicator/COT donor

Proposal 17: COT sharing information in SCI jointly indicates the starting slot and the number of consecutive slots of shared resources.

Proposal 18: If a UE indicates COT sharing other than "no sharing" in a SCI format within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent transmission occasions, if any, occurring within the same UE's initiated COT such that the same starting point and duration of the shared resources are maintained.

Proposal 19: Type 2A channel access procedure is applicable for S-SSB and PSFCH transmissions for a UE without a shared channel occupancy, with a restriction on the frequency and total duration of transmissions accessing the channel with such Type 2A usage in a time interval. The detailed restriction criteria are FFS.

- If the restriction criteria cannot be met, Type 1 channel access is applicable for the S-SSB/PSFCH transmission(s) in question.

- FFS: whether Type 2 channel access procedures can be applied to groupcast option 1 (if supported)

- FFS: how to define the value of  within the energy detection threshold calculation for S-SSB and PSFCH transmissions

Observation 2: The uplink contention window size update procedure cannot be directly applied to sidelink.

Proposal 20: RAN1 to study on the contention window size update procedure considering different cast type, transmission/reception from multiple Rx UEs or destinations and different HARQ feedback types e.g., groupcast option-1.

Proposal 21: To avoid priority-based dropping of HARQ-ACK associated with a CWS reference window, a Tx UE includes a 1-bit field to indicate if the corresponding PSFCH falls within the TX UE’s reference window. If so, an Rx UE uses the lowest priority value for the corresponding PSFCH priority determination (instead of the 3-bit field in the SCI).

Proposal 22: Support the following contention window update mechanisms:

- CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled):

  - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U

  - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value.

- CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration):

  - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

    - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’

    - FFS: how to calculate the ratio

    - FFS: the (pre-)configuration ratio values

- Support groupcast option 1 (NACK-only) with SL-HARQ feedback enabled for SL-U.

  - Option 4: CW is adjusted according to CR/CBR measurement

Proposal 23: Support delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value dynamically indicated in SCI

Proposal 24: For S-SSB transmission in SL-U, CPE starting positions can be based on priority levels of synchronization reference, which can be obtained by UE based on (pre-)configuration.

Proposal 25: Support to (pre-)configure the multiple CPE starting positions according to priority level (e.g., CAPC or L1)

Proposal 27: Support to define a criteria for selecting a default CPE starting position in mode 2.

Proposal 28: Support to indicate a certain CPE starting position for transmission UE in mode 1.

Proposal 29: RAN1 study the LBT impact on resource selection with below enhancements:

- To exclude the resource before a reserved resource based on the LBT type 1 information of the reserved resource.

- To exclude the resource after a reserved resource based the LBT type 1 information of the candidate resource.

Proposal 30: Study following aspects with considering the COT sharing mechanism:

- If a COT initialized for transmission on reserved resource and the COT can be shared to resource selection UE, whether the resource 3 or 4 can be regarded as the candidate resource reported to higher layer.

- If a resource selection UE performs transmission on resource 1 or 2 and initialized COT by the resource selection UE can be shared to the resource reservation UE, whether the resource 1 or 2 can be regarded as the candidate resource reported to higher layer.

Observation 3: LBT outcome on the reserved resource is not guaranteed and excluding such resource resource in the candidate resource selection may lead to resource in-efficiency

Proposal 31: Support allowing COT initiated UE to share channel occupancy duration to the UE that reserved resource

Proposal 32: Support multiple CPE starting position on the reserved resource while allowing COT initiated UE to select one of the starting positions on the reserved resource for type 2 LBT to check the LBT outcome and transmission status of the UE on the reserved resource.

# References

RP-213678, “New WID on NR Sidelink evolution”, RAN#94-e, Dec. 2021.

Chairman’s notes on RAN1#112