3GPP TSG RAN WG1 #112bis-e                                                                               R1-2303314

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.2

Source:	Lenovo

Title:	Physical layer design framework for sidelink on FR1 unlicensed spectrum

Document for:	Discussion

# Introduction

In RAN1#112 meeting, for Rel-18 NR sidelink transmission on unlicensed spectrum (SL-U), the relevant physical layer design framework has been extensively discussed including some fundamental issues, e.g., structure for PSCCH/PSSCH/PSFCH, PSSCH frequency domain resource allocation, guard band usage, PSFCH enhancement as well as SSB designs. The reached agreements are listed below.

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

In this contribution, we further elaborate our views on physical layer design framework for sidelink transmission on FR1 unlicensed spectrum.

# Discussion

## Interlace-based structure

Wireless signal transmission on unlicensed spectrum should meet the requirements of regulation subject to the management of the located country/region. Hence, the design of uplink waveform for NR-U PUSCH/PUCCH should meet the regulation requirements on unlicensed spectrum.

The requirements mainly include two aspects:

- Occupied channel bandwidth (OCB): the bandwidth containing 99% of the power of the signal, shall be between 80% and 100% of declared Nominal Channel Bandwidth;

- Maximum PSD (power spectrum density) with a resolution bandwidth of 1MHz, e.g., 10dBm/MHz.

These two requirements dictate that a signal which occupies a small portion of the channel bandwidth cannot be transmitted at the maximum available power at the UE due to the PSD and OCB constraints.

In Rel-16 NR over unlicensed spectrum (NR-U), interlace-based waveform is adopted as uplink waveform for unlicensed spectrum. Each interlace is equally distributed within the whole carrier bandwidth and spans more than 80% carrier bandwidth. The regulation requirements of OCB can be met if a signal or channel can occupy at least one interlace. Moreover, two adjacent PRBs of one interlace are separated by above 1 MHz distance and power boosting can be adopted for each PRB of one interlace. E.g., for 20MHz bandwidth and 15kHz SCS, all the available 106 PRBs are partitioned into 10 interlaces. Each interlace has 11 or 10 PRBs and is equally distributed within the 20MHz bandwidth. In this way, each interlace spans more than 80% system bandwidth so that the regulation requirements of OCB can be met. Moreover, 10 PRBs of one interlace are equally spaced in frequency so that two adjacent PRBs of one interlace are separated by 1.8MHz distance. Hence, one PRB within one interlace can be transmitted with 10dBm maximum power.

Figure 1 Interlace-based structure for 15kHz SCS

Different to Rel-16 NR-U where the minimum resource allocation unit is a interlace, a concept of sub-channel is defined in existing NR sidelink as the minimum resource allocation unit, where a subchannel comprises of multiple contiguous PRBs. Sidelink transmission/reception happens in a resource pool which can be configured with ‘M’ sub-channels with each subchannel including ‘N’ contiguous PRBs. The possible sub-channel size can be configured within the set of {n10, n12, n15, n20, n25, n50, n75, n100}. A resource pool for sidelink does not span across entire sidelink carrier or LBT sub-band. Furthermore, each sub-channel is a self-contained structure containing PSCCH, PSSCH and PSFCH resources.

- PSCCH is located in the lowest interlace of a sub-channel.  From Table 1, number of subchannels in a resource pool of 20MHz of bandwidth and number of remaining PRBs that may not meet the subchannel size are shown. Since the granularity of PSCCH/PSSCH scheduling is one subchannel size then we cannot ignore smaller subchannel size and RAN1 needs to discuss how to treat these remaining PRBs in an RB Set for PSCCH/PSSCH interlacing.

Table 1 Sidelink resource pool structure with 20MHz BW (106 RBs@15KHz SCS)

| Subchannel size (PRBs) | No of sub-channels in a resource pool with 20MHz BW (106PRBs @15KHz SCS) | Remaining PRBs (less than the subchannel size) |
| --- | --- | --- |
| 10 | 10 | 6 |
| 12 | 8 | 10 |
| 15 | 6 | 1 |
| 20 | 5 | 6 |
| 25 | 4 | 6 |
| 50 | 2 | 6 |
| 75 | 1 | 31 |
| 100 | 1 | 6 |

In addition, there are multiple numerologies defined for NR including 15, 30, 60 kHz for FR1. Rel-16 NR-U has agreed the number of interlaces is determined according to the subcarrier spacing, i.e., 10 interlaces with 10 or 11 PRBs per interlace for 15 kHz subcarrier spacing and 5 interlaces with 10 or 11 PRBs per interlace for 30kHz subcarrier spacing. For 60 kHz subcarrier spacing, it is not specified in Rel-16 NR-U for interlace-based structure.

Table 2 Max transmission bandwidth configuration NRB for FR1 (450 – 7125 MHz)

For the combination of 20MHz bandwidth and 60 kHz subcarrier spacing, candidate PRB-based interlace designs are listed in Table 3, wherein M is the number of interlaces and N is the number of PRBs per interlace. Generally, the combination of 4 interlaces and 6 PRBs per interlace can’t meet the regulation requirement of at least 80% occupancy ratio. The combination of 3 interlaces and 8 PRBs per interlace can nearly meet the occupancy ratio requirement. Instead, the combination of 2 interlaces and 12 PRBs per interlace can obviously meet the requirement.

Table 3 Candidate interlace structure for 20MHz bandwidth and 60 kHz SCS

| SCS | Candidate design | Occupied channel bandwidth per interlace | Percentage of occupied channel bandwidth per interlace over declared channel bandwidth |
| --- | --- | --- | --- |
| 60 kHz (total 24 PRBs) | M = 4, N = 6 | 15.12MHz (21 PRBs) | 75.6% |
|  | M = 3, N = 8 | 15.84MHz (22 PRBs) | 79.2% |
|  | M = 2, N = 12 | 16.56MHz (23 PRBs) | 82.8% |

Hence, for FR1 unlicensed spectrum, all the subcarrier spacings can be supported, i.e., 10 interlaces for 15kHz subcarrier spacing, 5 interlaces for 30kHz subcarrier spacing and 2 interlaces for 60kHz subcarrier spacing.

The interlacing RB based structure for PSFCH interlace is agreed in the previous meeting for 15kHz and 30kHz SCS, the interlacing structure for PSFCH need not be the same as that of the corresponding PSCCH/PSSCH interlacing. The PSFCH interlacing can contain a greater number of interlaces compared to the PSCCH/PSSCH interlace in a resource pool to provide tradeoffs between the common interlace and dedicated interlace and to optimally use the interlaces to meet the OCB requirement. Considering that, then L PSFCH interlace can be equal to one PSCCH/PSSCH subchannel and L PSFCH interlace can be equal to one PSCCH/PSSCH interlace. After finalizing the PSFCH interlace design, then RAN1 needs to discuss how to configure common interlace and dedicated interlace for PSFCH and the placement of common interlace within an RB set and the content to be transmitted on the common interlace considering all cast type’s HARQ feedback options.

Since the interlace design is already agreed for PSFCH design, then the interlace based RB transmission for PSFCH configuration contains both common and the dedicated interlace, which is provided by Alt-1.

Based on above analysis, we have below proposals:

Proposal 1: RAN1 should discuss the how to use the remaining PRBs in a RB set

Proposal 2: Support PSFCH interlace design containing L PSFCH interlace equals one PSCCH/PSSCH subchannel, placement of common interlace and the content to be transmitted in the common interlace

Proposal 3: 2 RB-based interlaces are specified for subcarrier spacing of 60kHz for sidelink transmission in FR1 unlicensed spectrum.

Resource pool configuration is usually up to RAN2 to decide and traditionally multiple resource pool considering different priority levels are supported and the selection of resource pool by MAC is according to the priority of the packet to be transmitted and the channel busy ratio. Although a resource pool configuration containing one or more RB sets were agreed at the previous meetings, a resource pool with subset of an RB set is for further study. In our understanding, multiple resource pools within an RB set is not precluded and hence there is no restriction on (pre-)configuring multiple resource pools within an RB set. RAN1 may further study on how to configure multiple resource pool in an RB set considering interlacing RB structure to meet the OCB requirement.

Proposal 4: Support configuring multiple resource pools within an RB set configured with interlaced RB-based structure

## Multiple candidate starting positions for sidelink

Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

When unlicensed spectrum is used for sidelink transmission, for the purpose of fair coexistence, the channel access procedure is still required before any sidelink transmission. Due to unpredictability of LBT, a Tx UE can’t know when the channel is available for sidelink transmission. If Tx UE prepares PSCCH and associated PSSCH only from symbol 0, then when Tx UE performs a successful LBT and obtains the channel from symbol 7 of one slot, the Tx UE has to wait or transmit some reservation signal and starts transmitting the prepared PSCCH/PSSCH from symbol 0 of the next slot. In this way, one half of the slot is wasted. In order to improve the spectrum utilization efficiency, multiple candidate starting positions are needed for sidelink, the more candidate starting positions, the higher spectrum utilization efficiency.

On the other hand, if multiple candidate starting positions are supported, then multiple PSCCHs/PSSCHs corresponding to multiple candidate starting positions in the slot are prepared by Tx UE before performing LBT, which inevitably leads to high complexity on Tx UE. From a Rx UE’s perspective, it has to blind detect PSCCH/PSSCH from the first candidate starting position in a slot. In that sense, the implementation complexity for the Rx UE is high.

It is noted that support of the multiple candidate starting positions is only necessary for initial slot of a COT initiated by Tx UE. After initial slot, there is only a single candidate starting position, i.e., symbol 0, for the subsequent sidelink transmissions. Hence, two candidate starting positions can be supported for sidelink transmission for trade-off.

In RAN1#112 meeting on the TBS determination for two starting symbols a working assumption was agreed that the reference number of symbols is determined by (pre-configuration), and the value range is still FFS. In our view the reference number of symbols for TBS determination could be predefined in the specification determined the configured number of symbols related to the second staring symbol, and it is unnecessary to introduce another configuration field to configured the reference number of symbols, the value of reference number of symbols could be the total number of symbols determined by the 1st starting symbol or 2nd staring symbol or the average number of symbols determined by the configured two starting symbols.

Proposal 5: For TBS determination it is unnecessary to introduce additional configuration, and the reference number of symbols could be derived based on the (pre-)configured 1st and/or 2nd starting symbol, e.g., total number of symbols determined by 1st or 2nd starting symbol or average number of symbols determined by 1st and 2nd starting symbol.

Furthermore, to ease Rx UE complexity due to extra blind detection on additional starting positions or unnecessary blind detection on non-SL signals, a signal can be considered for Rel-18 SL-U. For example, a Tx UE can transmit the signal in the beginning of a COT after successfully performing Type-1 channel access procedure. This signal can be a sequence with high detection probability and predefined for NR sidelink. From Rx UE’s point of view, the Rx UE does not need to blind detect each starting position for decoding PSCCH. It shall not start decoding PSCCH unless it detects the signal in one starting position. Since UE only needs to perform a sequence detection, the UE complexity is quite low. However, the power saving gain is significant since the UE can skip the PSCCH decoding if the channel is occupied by other non-SL systems, e.g., Wi-Fi, LAA, NR-U.

Proposal 6: Study defining a signal for easing UE blind detection complexity and saving UE battery power.

Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

For a slot with 2 candidate starting positions, when PSCCH/PSSCH is transmitted from the first candidate starting position, the first symbol in the slot (i.e., symbol 0) is used for AGC usage. And when PSCCH/PSSCH is transmitted from the second candidate starting position, the second candidate starting symbol (e.g., symbol 4 or symbol 7) in the slot can be used for AGC usage.

For example, as shown in below figure 2, if the access channel of a UE-A is successful in RB set 1 before symbol 0, the UE-A performs PSCCH/PSSCH transmission in RB set 1 from the first candidate starting symbol (i.e., symbol 0). Meanwhile, if the access channel of a UE-B is successful in RB set 2 before symbol 4, the UE-B performs PSCCH/PSSCH transmission in RB set 2 from the second candidate starting symbol (i.e., symbol 4). There will have power impact at Rx UE (e.g., UE-C) side in the slot, here UE-C performs reception on both RB set 1 and RB set 2. To avoid this impact, the second AGC symbol for the UE-A in RB set 1 is needed. Even the UE-B starts its transmission from symbol 0 in RB set 2, the second AGC is also needed for both UE-A and UE-B, because they do not know the transmission situation on the other RB set.

Figure 2 UE-A performs PSCCH/PSSCH transmission in RB set 1 and UE-B performs PSCCH/PSSCH transmission in RB set 2

But if a UE-A performs PSCCH/PSSCH transmission in RB set 1 and RB set 2 simultaneously (i.e., full bandwidth), the second AGC symbol (e.g., symbol 4 or symbol 7) can be used for PSSCH transmission with considering resource efficiency, i.e., the second AGC symbol is not used for AGC purpose, as shown in figure 3.

Figure 3 the UE-A performs PSCCH/PSSCH transmission in RB set 1 and RB set 2

From Tx UE point of view, to flexible determine the number of AGC symbol (i.e.,1 or 2 symbol(s) for AGC purpose) of a PSCCH/PSSCH transmission is reasonable and efficient. So we support Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details.

Proposal 7: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, regarding Tx UE behaviour, Option 3 is supported.

Regrading Rx UE behaviour, if above option 3 is supported, Tx UE does not always perform PSCCH/PSSCH transmission with two AGC symbols. In this way, Rx UE is not required to monitor two AGC symbols all the time. For example, if Rx UE detects that the PSCCH/PSSCH is transmitted spanning on full bandwidth and/or determines the second symbol is used to transmit PSSCH, Rx UE can only monitor the first AGC symbol. So additional (pre-)configuration or indication information is needed for Rx UE determination, e.g., based on per resource pool (pre-)configuration or received control indication. So we support Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol, FFS details.

Proposal 8: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, regarding Rx UE behaviour, Option B is supported.

## Frame structure

When unlicensed spectrum is used for sidelink transmission, needless to say, the channel access procedure, also named LBT, is required before any sidelink transmission. Different to SCI decoding-based sensing mechanism, LBT procedure is performed based on energy detection in each sensing slot. In detail, if the detected energy on one channel in one sensing slot is lower than the energy detection threshold, then the channel is deemed as empty or clear or available in that sensing slot; otherwise, the channel is deemed as occupied or non-available in that sensing slot.

For unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT). According to Table 4.2.1-1 in TS37.213, the MCOT is related to channel access priority class (CAPC) value. Consequently, when a UE performs a successful channel access Type 1, the UE can occupy the channel with maximum 2ms, 4ms, 6ms or 10ms, depending on the corresponding CAPC value which is determined based on the traffic priority. The higher the CAPC value, the longer the MCOT on the unlicensed spectrum. As shown in Table 4.2.1-1 in TS37.213, for CAPC value of 3 or 4, the MCOT can be 10ms, which is equivalent to 10 slots in case of 15kHz subcarrier spacing, 20 slots in case of 30kHz subcarrier spacing, 40 slots in case of 60kHz subcarrier spacing.

To avoid any risk of losing the channel, similar to DL burst and UL burst in TS37.213, a sidelink burst can be defined as a set of transmissions from a UE without any gaps greater than 16us. Multiple consecutive slots can be included in the sidelink burst. Each PSCCH/PSSCH starts from symbol 0 of a slot to avoid complicated designs at both Tx UE side and Rx UE side. One example is shown in Figure 4, where a UE initiates a COT via LBT Cat.4 using CAPC 2, then it can contiguously transmit 4 PSCCH/PSSCHs in the four consecutive slots without sensing. So the sidelink burst can include the four slots from symbol 0 of slot n to symbol 9 of slot n+3.

Figure 4 Burst based transmission for sidelink

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. It does make sense that a Tx UE initiated COT can be shared to one or more Rx UEs when the requirements of COT sharing is satisfied. One example is shown in Figure 5. After initiating a COT by performing Type 1 channel access procedure, Tx UE, named UE1 for simplicity, performs contiguous sidelink transmissions without any gap larger than 16us in time domain, and then can share some slots within the COT to Rx UE (e.g., UE2) for transmitting PSCCH and associated PSSCH. There are one or more symbols which may be used as gap between two consecutive PSSCHs. If HARQ-ACK feedback is enabled, then PSFCH transmission occasions should be contained within the COT with at least one symbol as gap before or after each PSFCH transmission. Therefore, the structure of the COT is needed for a Rx UE which intends to share the COT for transmitting PSCCH/PSSCH and/or PSFCH. The COT structure should contain the remaining channel duration in terms of number of slots, CAPC or logical channel priority value as indicated by higher layer, over-riding configuration to over-ride the gap symbol, AGC symbol and PSFCH symbol configured as part of the resource pool to perform PSSCH burst transmission, a dedicated configuration provides the placement location for gap symbol and CP-Ext symbol within the COT needed for COT sharing purpose. The COT structure can also provide the dedicated PSFCH symbol placement in a within a COT. The COT sharing indicator can also be transmitted with the COT structure signaling. The Rx UE determines the reference starting slot depending on the slot where the COT structure signaling is successfully received, which is needed to execute the placement of dedicated symbols, over-riding symbols and to calculate the remaining COT duration for the COT sharing purpose.

Figure 5 UE-to-UE COT sharing for sidelink transmission

Based on above analysis, we have below proposals:

Proposal 9: For burst-based sidelink transmission, from a Tx UE’s point of view, no gap symbol is included in between any two consecutive slots

Proposal 10: Each PSCCH/PSSCH is transmitted from symbol 0 of a slot other than initial slot of a COT.

Proposal 11: COT structure information is needed for sidelink transmissions in FR1 unlicensed spectrum.

## HARQ

In Rel-16/17 sidelink transmission, a UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period.

If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A or a SCI format 2-B has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception.

For Rel-18 sidelink transmission on unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT), e.g., occupy maximum several or tens of slots, depending on the corresponding CAPC value and subcarrier spacing.

If legacy periodic PSFCH configuration is used for unlicensed spectrum, when a UE initiates a COT by performing Type-1 channel access procedure, the COT may be frequently interrupted by the periodic PFSCH occasion. This is especially true since the PSFCH period is relatively small, i.e., 1, 2 or 4 slots. One example is shown in Figure 6 with max 4 slots as one PSFCH periodicity. When the COT has a duration of more than 4 slots, like COT 3, it will be interrupted twice. This interruption brings below drawbacks:

- The Tx UE’s sidelink transmissions can’t be contiguously transmitted in time domain because the Tx UE needs to monitor the channel continuously and perform channel access type 2 before it resumes its sidelink transmission.

- Tx UE is quite likely to lose the channel. When the gap is larger than 16us, then there is risk of losing the channel. When the COT is interrupted by one PSFCH occasion and there is no any PSFCH transmitted during the occasion due to NACK-only based feedback or disabled HARQ-ACK feedback, considering one PSFCH occasion occupies 4 consecutive symbols about 280us, the probability of losing the channel is quite high.

Figure 6 Periodic PSFCH configuration

To address aforementioned drawbacks, PSFCH transmission occasions can be dynamically indicated by SCI. In detail, after initiating a COT by performing channel access type 1, a Tx UE can transmit SCI scheduling a PSSCH and dynamically indicating a slot level offset between the slot where the corresponding PSSCH is transmitted and the slot where the corresponding PSFCH occasion is included from a configured HARQ-ACK feedback timing set. For each Rx UE intending to transmit PSFCH, based on the indicated PSFCH transmission occasion, the Rx UE can perform Type 2A/2B/2C LBT in case of the indicated PSFCH transmission occasion inside of the COT or Type 1 LBT in case of the indicated PSFCH transmission occasion outside of the COT.

To support dynamically indicated one or multiple PSFCH transmission occasions, the HARQ-ACK feedback timing set comprises one or multiple subsets with each subset including one or multiple HARQ-ACK feedback timing values. When a subset including a single HARQ-ACK feedback timing value is indicated by the SCI, it implies the HARQ-ACK feedback is to be transmitted in the slot indicated by the single HARQ-ACK feedback timing value and the HARQ-ACK feedback is transmitted only once; When a subset including multiple HARQ-ACK feedback timing values is indicated by the SCI, it implies the HARQ-ACK feedback is to be transmitted in multiple slots with each indicated by the respective HARQ-ACK feedback timing value and the transmission times of the HARQ-ACK feedback is equal to the number of HARQ-ACK feedback timing values in the subset. An indicator in the SCI is used to indicate a specific subset from the HARQ-ACK feedback timing set. In this way, the PSFCH transmission occasion is fully determined based on the indicator. E.g., the HARQ-ACK feedback timing set is configured as {{+2}, {+3}, {+2, +4}, {+3, +5}, {+2, +4, +6}, {+3, +5, +7}, …} and all the values in each subset is with reference to the slot where the PSSCH is transmitted. Hence, when a PSSCH is transmitted in slot n, if the subset of {+2, +4, +6} is indicated, then the HARQ-ACK feedback for the PSSCH can have 3 transmission opportunities, i.e., slot n+2, n+4, n+6, with each determined by the timing offset value of 2, 4, or 6; if the subset of {+2} is indicated, then the HARQ-ACK feedback for the PSSCH has a single transmission opportunity, i.e., slot n+2, determined by the timing offset value of 2.Due to UE processing delay, some PSFCHs may not be responded in same COT for the corresponding PSSCHs. The Tx UE can suspend the transmission of these PSFCHs and trigger the retransmission in a new COT initiated by the Tx UE. The HARQ-ACK feedback timing set can include one inapplicable value, e.g., -1, for indicating the HARQ-ACK feedback is suspended.

In this way, the PSFCH transmission occasion can be inside of the COT initiated by Tx UE which ensures a high LBT successful probability due to channel access Type 2A/2B/2C required. The ongoing sidelink transmission from Tx UE will be not interrupted by the PSFCH transmission occasion especially when the PSFCH transmission occasion is located at the end of the COT.

Based on above analysis, we have below proposals:

Proposal 12: Support dynamically indicated PSFCH resources for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 13: Support suspending PSFCH when the PSFCH can’t be transmitted in same COT with corresponding PSSCH and triggering it in a new COT.

Proposal 14: Support HARQ-ACK feedback timing set comprising one or multiple subsets with each subset including one or multiple HARQ-ACK feedback timing values for dynamically indicating one or multiple PSFCH transmission occasions.

For sidelink transmission on unlicensed spectrum, if channel access procedure is successful, a Tx UE transmits a PSSCH to a Rx UE and waits for the reception of PSFCH from the Rx UE. Without such HARQ-ACK feedback, the Tx UE has to retransmit the PSSCH even though the Rx UE has correctly decoded the PSSCH. There are two risks that the Tx UE may not receive the PSFCH from the Rx UE:

- LBT risk: Due to unpredicted LBT outcome at Rx UE side, the sidelink HARQ-ACK feedback may not be transmitted if LBT at Rx UE side is failed.

- Hidden node interference: If LBT at Rx UE side is successful, the Rx UE transmits PSFCH to Tx UE. Due to hidden node interference on the shared unlicensed spectrum, the Tx UE may not correctly decode the PSFCH.

Therefore, when one sidelink HARQ-ACK feedback is to be transmitted on an unlicensed carrier, the sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities (TOs) and transmitted in the first transmission opportunity with LBT success. One example is shown in Figure 7, where one PSSCH transmitted in slot 1 can have 4 transmission opportunities in slot 4, 8, 12 and 16 with PSFCH periodicity of 4 slots.

In detail, for each PSSCH, the maximum number of transmission opportunities of the corresponding HARQ-ACK feedback can be configured. Each transmission opportunity is determined subject to UE processing time and PSFCH periodicity configuration. The initial transmission of the PSFCH is determined as legacy and the subsequent retransmissions are determined according to the PSFCH periodicity. Assuming the PSSCH is received in slot n and the first PSFCH is to be transmitted in slot n+x, wherein x is the slot level offset subject to UE processing time and PSFCH periodicity configuration, then the transmission opportunities for the sidelink HARQ-ACK feedback corresponding to the PSSCH comprise slot n+x, n+x+P, n+x+2P, …, n+x+(M-1)*P, where M is the maximum transmission opportunities for same HARQ-ACK feedback and P is the periodicity of PSFCH in a resource pool.

To avoid potential PSFCH resource collision among the initial transmission and subsequent retransmissions for same HARQ-ACK feedback, for a given PSSCH, when it is scheduled by associated SCI, the index of the transmission time of the PSFCH is used for calculating the PSFCH resource index. E.g., upon reception of a PSSCH, a UE determines an index of a PSFCH resource for a PSFCH initial transmission and retransmission as PID+MID+imodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI scheduling the PSSCH reception,  MID is the identity of the UE receiving the PSSCH as indicated by higher layers if the UE detects a SCI format indicating groupcast with ACK or NACK based HARQ-ACK feedback (i.e., Cast type indicator field value of "01"); otherwise, MID is zero. i is the transmission time, e.g., i=0 for initial transmission, i=1 for the second transmission, …, i=M-1 for the Mth transmission. RPRB, CSPSFCH is a total number of PSFCH resources available for multiplexing HARQ-ACK information in a PSFCH transmission.

Figure 7 Multiple transmission opportunities for one sidelink HARQ-ACK feedback

Proposal 15: Support multiple periodic PSFCH occasions per PSCCH/PSSCH transmission.

Proposal 16: Support PSFCH transmission time index for determining multiple PSFCH transmission and retransmission resources per PSCCH/PSSCH transmission.

## PSFCH structure

Regarding PSFCH transmission, if one PSFCH is transmitted on a full interlace, then the problem is the PSFCH capacity is quite limited. Therefore, PSFCH capacity increase is needed and there are multiple alternatives for further study as listed in FL summary [3].

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case - FFS: whether/how to reduce PAPR of PSFCH transmission

Firstly, for PSFCH transmission using 15kHz SCS and 30kHz SCS, the baseline solution should be one PSFCH transmission occupies one interlace without cyclic shift operation, i.e., different interlaces correspond to different PSFCH transmission. However, this baseline operation may lead to insufficient PSFCH capacity especially for groupcast option 2. Hence, the PSFCH capacity increase is needed with the prerequisite of meeting OCB requirements and PSD requirements on unlicensed spectrum.

For Alt 1-1a, since each PSFCH can carry one HARQ-ACK bit using PSFCH format 0, each PSFCH transmission occupies one common interlace and one dedicated PRB. The common interlace is used for meeting OCB requirements and the dedicated PRB is used to carry one HARQ-ACK bit as legacy PSFCH format 0. Alt 1-1a can provide plenty of PSFCH resources for PSFCH transmission of 15kHz SCS and 30kHz SCS. The drawback is the dedicated PRB may be adjacent to the one RB of the common interlace so that the PSD requirements may not be met. Similar issue may happen for Alt 3-2a if the dedicated PRB is adjacent to either of the 2 common PRBs.

Compared with Alt 1-1a, Alt 3-2a can provide more significant PSFCH resources. The value of K2 is equal to 2 as shown in both Figure 8 and Figure 9 which can meet OCB requirements with the least common PRBs. Similar to Alt 1-1a, since each PSFCH can carry a single HARQ-ACK bit using PSFCH format 0, K4 is set to 1 so that each PSFCH transmission in Alt 3-2a occupies one dedicated PRB and 2 common PRBs. To avoid the PSD issue when the dedicated PRB is adjacent to either of the 2 common PRBs and mitigate the IBE from the 2 common PRBs to the dedicated PRB, a guard band between the common PRB and the nearest possible dedicated PRB is needed. As shown in Figure 9, a first guard band between the lowest common PRB (i.e., PRB 0) and the first possible dedicated PRB (e.g., PRB 10) is placed, and a second guard band between the last possible dedicated PRB (e.g., PRB 95) and the highest common PRB (i.e., PRB 105) is placed.

Figure 8 PSFCH structure based on partial interlace for 15kHz SCS

Figure 9 PSFCH structure based on partial interlace for 15kHz SCS

For 60kHz SCS, the above Alt 3-1a can be extended to support PSFCH transmission, i.e., each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of an RB set. The above dedicated PRB and common PRBs are within 1 interlace.

It may need to be further studied whether the definition of the existing PSFCH transmission capability psfch-TxNumber is in any way affected by the transmission of common PRBs. It should also be avoided that a UE is required to transmit on more than 2 common PRBs in an RB set, e.g. for the case that multiple RPs are in the same RB set.

Based on above analysis, we have below proposals:

Proposal 17: For PSFCH transmission using 15kHz and 30kHz SCS and 60kHz SCS, support each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of an RB set.

Proposal 18: For PSFCH transmission using 15kHz and 30kHz SCS and 60kHz SCS, for each PSFCH transmission, support guard band between the nearest dedicated PRB and either of the 2 common PRBs, where 2 common PRBs locate at the two edges of an RB set.

Observation 1: Further study may be needed whether the definition of the existing PSFCH transmission capability psfch-TxNumber is affected by the transmission of common PRBs. It should also be avoided that a UE is required to transmit on more than 2 common PRBs in an RB set, e.g. for the case that multiple RPs are in the same RB set.

Common interlace/common PRBs dropping

In Alt 1-1a and Alt 3-2a there are some common resources, e.g., common interlace/PRBs, the common resources are used to meet the OCB requirement, if only one PSFCH is transmitted by the UE the transmission of common resources is necessary to meet the OCB requirement, however in sidelink one UE may need to transmit multiple PSFCHs in one PSFCH transmission occasion, if the transmissions of multiple PSFCHs could meet the OCB requirement themselves, the transmission on common resources is not necessary. The drawback of the transmission of common resources will limit the PSFCH coverage and cause IBE. So the UE could drop the transmission on the common resources if the transmissions of multiple PSFCHs could meet the OCB requirement themselves. Two approaches could be considered to drop the common resources, the first one is that UE performs multiple PSFCHs selection as legacy mechanism., based on the priority order, if the transmissions of selected PSFCHs could meet the OCB requirement the UE drops the transmission of common interlace/common PRB(s). The second approach is that the UE could firstly to select the PSFCHs to meet the OCB requirement.

Proposal 19: The UE could drop the transmission of common interlace/common PRBs if the transmission of multiple PSFCHs could meet the OCB requirement.

## One shot SL HARQ feedback

In Rel-16, as part of the NR-U feature one shot HARQ feedback report transmission with UE transmitting a snapshot of HARQ-ACK report of all HARQ process using PUCCH or PUSCH when a UE receives DCI format 1_1 contain one bit to request the transmission of one shot HARQ feedback.

The sidelink feedback transmission/reception by a UE could be affected due to LBT failures and there exists NR-U features such as one-shot HARQ feedback, non-numerical HARQ feedback timing indicator to counter this problem. Such feature could be studied and requires adaptation for sidelink unlicensed operation.

However, in sidelink the one shot HARQ feedback reporting might need to be associated to a destination, cast types and moreover there is only a limited number of HARQ process between a source-destination pair of UEs as shown in the below Figure 10. Hence the procedure of transmission of SL one shot HARQ feedback should consider different source-destination ids, cast types such as unicast, groupcast, broadcast, HARQ feedback options and blind retransmissions etc., Also reporting one shot HARQ feedback report from Tx UE to gNB for all SL HARQ process should be studied which could be useful when due to LBT failures there could be delay in sidelink HARQ feedback reporting to gNB.

Figure 10 Showing active HARQ processes between UE1, UE2 and UE3

Proposal 20: Support introducing the one-shot HARQ feedback trigger and reporting mechanism for sidelink unlicensed operation.

## Non-numerical feedback timing indicator

Non-numerical value as feedback timing was introduced which eventually postponed the transmission of the generated HARQ feedback while a subsequent HARQ transmission of numerical value as part of the DL DCI triggers the transmission of the previously generated HARQ feedback for those HARQ processes that was transmitted using non-numerical value indication. The Rel16 sidelink HARQ feedback reporting may get delayed due to LBT failures and channel occupancy limitation. Such delay in the sidelink HARQ feedback reception at the Tx UE may also affect the reporting of SL HARQ feedback to gNB.

Currently the feedback timing for sidelink transmission is (pre)configured in a resource pool and not dynamically signalled and there should be some signalling enhancement needed to dynamically indicate the non-numerical feedback timing value to the Rx UEs or destinations to delay the transmission of SL HARQ feedback reporting. Further a trigger needs to be subsequently transmitted to request the transmission of SL HARQ feedback of those SL HARQ process previously indicated with non-numerical HARQ feedback timing value.  Such mechanism may also beneficial for gNB to delay the HARQ feedback transmission from a Tx UE using the non-numerical HARQ feedback timing value in DCI format 3_0.

Proposal 21: Support delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for sidelink unlicensed operation.

## Resource pool structure

In Rel-16/17 NR sidelink communication, higher layer signaling configuration, synchronization procedure and physical channel structure are designed and specified based on resource pool. The data transmission/reception and resource allocation for both mode 1 and mode 2 are assigned within a resource pool. So, to consider less standardization workload, resource pool relevant design can be reused for sidelink on FR1 unlicensed spectrum.

Proposal 22: To reuse resource pool relevant design for sidelink on FR1 unlicensed spectrum.

## Physical channel design

In Rel-16/17 NR sidelink communication, potential PSCCH is assigned in each subchannel. When scheduling multiple subchannels, PSCCH is allocated in the lowest subchannel in frequency domain. The size of PSCCH in frequency domain can be {10, 12, 15, 20, 25} RBs. In time domain, the PSCCH occupies 2 to 3 symbols.  For Interlace-based structure, potential PSCCH can be still designed to assign in each interlace and to occupy 2 to 3 symbols.

Proposal 23: To study the location of PSCCH in frequency/time domain for interlace-based structure.

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission. In NR-U, when more than one RB set is used for transmission, the used interlace index(s) and RB set index(s) are indicated and the used interlace index(s) in different  RB sets are the same. This legacy indication mechanism can be reused to SL-U, i.e.,  the used interlace index(s) in different RB sets are always the same and explicitly indicating the used sub-channel index(s) and RB set index(s).

Proposal 24: Support that the used interlace index(s) in different RB sets are always same, and explicitly indicating the used sub-channel index(s) and RB set index(s).

For the location of PSCCH, we support the PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH. For the SL transmission spanning on multiple RB sets, to consider the reliability of PSCCH in legacy NR-SL, the reception UE can decode the transmission successfully based on only one PSCCH. There is no need to further pursue the diversity gain and combination gain in frequency domain for PSCCH. If LBT failure, transmitting UE will drop transmission on each RB set, rather than performing transmission on available RB set(s) on which LBT is success. So there is no obvious gain for PSCCH locating in every RB set of corresponding PSSCH.

Proposal 25: Support PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH.

In Rel-16/17 NR sidelink communication, to consider the difference usage scenario, the number of columns of DMRS can be various, i.e., 2 ,3 or 4 columns in one time slot. The time domain location of DMRS can be reused. The frequency domain location of DMRS should be designed based on interlace-based structure.

Proposal 26: To study 2 ,3 or 4 columns of DMRS in one time slot and frequency domain location of DMRS should be designed based on interlace-based structure.

In Rel-16/17 NR sidelink communication, unicast, groupcast opiton1/2 and broadcast were supported. For the sidelink communication on unlicensed band, each cast type should be supported.

Proposal 27: To support unicast, groupcast option 1/2 and broadcast for unlicensed spectrum。

For unicast and groupcast opiton1/2, the feedback information is transmitted on PSFCH channel. The PSFCH region is assigned the last second and third symbols at one time slot (the last symbol and the fourth symbol from last symbol are GP). This kind of design can be reused for unlicensed spectrum. In addition, each HARQ feedback should be transmitted on one interlace but not on RB on unlicensed band, the PSFCH resource may be insufficient. For example, if a UE performs a groupcast option 2 transmission on 20MHz bandwidth, the 20MHz bandwidth may include 10 interlaces in case of 15KHz SCS, that is only 10 PSFCH resources can be provided to maximum10 number UEs in this group. So besides legacy PSFCH resources, to study increasing the PSFCH resource for unlicensed spectrum.

Proposal 28: Besides legacy PSFCH resources, to study increasing the PSFCH resources for unlicensed spectrum.

## Resource allocation mode 1 and mode 2

### Mode 1

In mode 1, as mentioned in WID objective, Uu operation for mode 1 is limited to licensed spectrum only. In this way, legacy scheduling and feedback mechanism on Uu link for mode 1 can be reused. The difference between legacy resource allocation and resource allocation on unlicensed band is that the scheduled UE needs to perform a LBT operation before its scheduled sidelink transmission. The assistance information for LBT operation at UE side may be signaled from network and/or based on UE determination. The content of this assistance information should be studied. In addition, the LBT operation may be failure and the UE behavior and potential solution should be studied.

Proposal 29: Besides legacy scheduling and feedback mechanism, the assistance information and its content for UE performing LBT operation and the potential solution for LBT failure should be studied.

The sidelink grants are allocated by gNB using DCI format 3_0 and then the Tx UE performs LBT before making any transmission on the indicated sidelink resources. DCI field could indicate the channel access priority value together with the sidelink grant and the behavior of Tx UE as a COT initiator could be different in the following ways:

Tx UE after receiving the sidelink grant could autonomously choose the channel access priority class value to perform Cat 4 LBT and accordingly initiate channel occupancy duration. In this method, the selection of channel access priority class value by the Tx UE could be based on the priority/latency of the data in the buffer which is in accordance with the existing LCP rule. In the second method, if the DCI indicates the channel access priority value, then the Tx UE could perform the Cat 4 LBT and accordingly initiate channel occupancy duration.

Proposal 30: RAN1 should discuss the need for signalling the channel access priority value or maximum channel occupancy duration along with the sidelink grant.

The sidelink grants are allocated by gNB using DCI and then the Tx UE performs LBT before making any transmission on the indicated sidelink resources. When LBT fails on all the indicated resources more sidelink resources are required from gNB. How the Tx UE request more resource from gNB using the physical layer signalling should be discussed.

Proposal 31:  LBT failures on all the indicated resources in the sidelink grant may trigger UE to request more resources from gNB, RAN1 should discuss ways to request more resource from gNB.

### Mode 2

In mode 2, UE may perform autonomous resource selection for SL transmission. The LBT failure issue is also existing. When the transmission is transmitted on reserved resource within same COT, e.g., for retransmission of the same TB, LBT type 2 is performed for the retransmission, the LBT failure probability may be reduced. But to consider the long period traffic, i.e., the reserved resource is out of an initial COT, the UE should re-initial a COT, the LBT failure probability may be increased. So, the assistance information of a UE performing LBT operation is also needed to be signaled on sidelink. The assistance information can be used by the UE who performs LBT operation, or it can be used by the UE who performs autonomous resource selection.

Proposal 32: The assistance information of a UE performing LBT operation is needed to be signaled on sidelink.

Proposal 33: Study on how to use the signaled assistance information (including its content) for LBT operation and resource selection.

## Multi-slot scheduling

A sidelink device is required to pass a clear channel assessment (CCA) procedure, such as Cat 4 LBT to initiate a channel occupancy in an unlicensed carrier (shared spectrum). The sidelink transmission are allowed to be performed within the maximum channel occupancy duration defined by the respective channel access priority class which is counted or measured from the first transmission after initiation of the COT. Consecutive transmission within he occupied channel duration without gaps or gap less than 25µs is beneficial so that initiator device does not need to perform Cat 2 LBT to reaffirm the channel. In the current sidelink physical layer design upto three non-consecutive sidelink resources could be allocated or reserved within a boundary of 32 slots using the sidelink grant provided by DCI format 3_0 to the Tx UE and PSCCH transmission to the Rx UE while the same upto three resources could be reserved using mode 2. The current resource scheduling mechanism based on configured grant, dynamic scheduling using mode 1 and candidate resource selection, reservation mechanism using mode 2 requires changes to perform consecutive transmission which could be based on multi-slot scheduling.

Proposal 34: RAN1 could study the possibility of assigning more than 3 consecutive resources with a gap less than 25µs using multi-slot scheduling

## S-SSB design

Additional candidate S-SSB are added to compensate for the case that some legacy S-SSB occasions are unavailable due to LBT failure. As per the motivation of introducing additional candidate S-SSB occasion(s), once S-SSB is transmitted on the legacy S-SSB occasion(s), there is no need to perform S-SSB transmission on additional candidate S-SSB occasion(s) within the same S-SSB period. In other words, how to use the additional candidate S-SSB occasion(s) should be based on the S-SSB transmission status on legacy S-SSB occasion(s) within the same S-SSB period.

Proposal 35: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) (Alt 1).

Agreement Down-select one or support both of the followings: - Option 1: Additional candidate S-SSB occasions are excluded from resource pool - Option 2: Additional candidate S-SSB occasions belong to resource pool - Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

More candidate occasions typically result in more time-domain resources required. If additional candidate S-SSB occasions are excluded from resource pool, the system performance will be impacted at least in terms of the metrics as follows. (1) the system overhead will be increased linearly along with the number of additional candidate S-SSB occasions. In R16, the number of S-SSB occasions for 15kHz/30kHz SCS in FR1 are configured up to 1/2, respectively. To compensate the impact of LBT failure in unlicensed spectra, the number of S-SSB occasions can be configured up to 4/8 for 15kHz/30kHz SCS, respectively. In that case, the system overhead will be increased by as high as three times.  (2) the number of COT-based transmissions interrupted by discretely distributed additional candidate S-SSB occasions will also increase along with the number of additional candidate S-SSB occasions. Therefore, option 1 cannot be supported.

To reduce the performance loss, it is beneficial to place additional candidate S-SSB occasion(s) within resource pool. If UE-A identifies that S-SSB has been transmitted on legacy S-SSB occasions, then UE-A can attempt to utilize the additional candidate S-SSB occasions for PSSCH/PSCCH transmission purpose. In addition, for a COT-based transmission within the resource pool, the COT initiator can determine how to utilize the additional candidate S-SSB occasions located in the COT, thus avoiding the ambiguity on utilizing additional candidate S-SSB occasions.

Proposal 36: Additional candidate S-SSB occasion(s) belong to resource pool (Option 2).

Considering the uncertainty of channel access in unlicensed spectra, to achieve a flexible distribution of S-SSB occasions, it is preferred to define the number and location of additional candidate S-SSB occasions independently from that of legacy S-SSB occasions.

Proposal 37: The number and location(s) of additional candidate S-SSB occasions are separately configured (option 3).

Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Option 1-1: As a method to meet OCB requirement, interlaced transmission is straightforward to be applied for S-SSBs in FR1 unlicensed spectrum. However, different interlaces may contain different number of RBs even for given SCS and bandwidth. For example, as illustrated in Figure 11, each of the first six interlaces contains 11 RBs, while each of the last four interlaces contains only 10 RBs. Consequently, interlaced mapping of S-SSB (11RBs) may need different numbers of interlaces according to the exact interlace(s) used. For the sake of simplicity, interlaced mapping of S-SSB should be fixed in a range, such as the interlace(s) with the least indices.

Figure 11 An exemplary interlace pattern

Option 3-1: S-SSB using contiguous RBs is beneficial for reliability. Figure 12 shows an exemplary structure with repetition of S-SSB in frequency, wherein legacy S-SSB structure as specified in R16/R17 NR can be reused without modification.

Figure 12 An exemplary structure with repetition of S-SSB in frequency

Proposal 38: For S-SSB transmission within 1 RB set, either structure of Option 1-1 or Option 3-1 can be supported for 15 kHz and 30 kHz SCS.

As agreed in the last meeting, RAN1 further study how to transmit S-SSB when a SL BWP contains multiple RB sets. Regarding S-SSB, transmission across multiple RB sets can increase the channel access opportunities in unlicensed spectra. On the other hand, multiple RB sets are beneficial for providing sufficient RBs for S-SSB, especially for interlace RB-based S-SSB transmission. The design can be considered from following aspects.

- Whether one S-SSB occasion is confined within one RB set in the frequency domain

It’s straightforward to reuse the S-SSB designed for individual RB set scenarios, where each S-SSB occasion is confined within one RB set in the frequency domain. In that case, it is required to configure a proper subset of RB sets within the SL BWP for S-SSB transmission and thus achieve a balance between channel access opportunities for S-SSB and spectrum efficiency.

Alternatively, regarding the wider bandwidth available, one S-SSB occasion can span across multiple RB sets in the frequency domain.  Therefore, the symbols occupied by each S-SSB occasion can be reduced while providing sufficient for the payload size defined for R16 S-SSB PSBCH. In this case, the new structure is needed to be designed for S-SSB. Exemplary S-SSB structures are illustrated in Figure 13.

Proposal 39: Regarding S-SSB structure over multiple RB sets, the following can be considered:

- Each S-SSB occasion in frequency can be confined within one RB set.

- Each S-SSB occasion in frequency can span over multiple RB sets.

Figure 13 exemplary S-SSB structures over multiple RB sets

- Whether to use guard bands for S-SSB transmissions

If the S-SSB structure for individual RB set is reused in multiple RB sets scenarios, the guard band is suggested not to support S-SSB transmission. The motivation behind this is to simplify S-SSB structure while decreasing interference to a neighboring RB set not available for S-SSB transmission. If the one S-SSB occasion spans across multiple RB sets, the guard band in between the multiple RB sets can be used either for S-SSB transmission or not, which are illustrated by right side and left side in Figure 13 respectively.

Proposal 40: Regarding guard band resources, the following can be considered:

- In the case that each S-SSB occasion in frequency is confined within one RB set, the guard band is not used for S-SSB transmission.

- In the case that each S-SSB occasion in frequency spans over multiple RB sets, the guard band in between can be used either for S-SSB transmission or not.

- Whether S-SSB period boundaries for different RB sets are aligned or not

An exemplary distribution of S-SSB occasions over multiple RB sets is illustrated in Figure 14. In each RB set, the S-SSB occasions can be distributed in the same way following that defined for 3GPP SL R16, R17 or R18. The RB sets differ from each other in the starting boundaries, which are not aligned across multiple RB sets. This design is beneficial as it increases the density of S-SSB occasions in the time domain, without increasing the number of S-SSB occasions in each RB set.

Proposal 41: Support that the starting boundaries for S-SSB periods are not aligned across multiple RB sets.

Figure 14 an exemplary distribution of S-SSB occasions over multiple RB sets

## Congestion control mechanism

Congestion control mechanism was introduced in Rel-14 LTE-V2X and Rel-16/17 NR sidelink. The motivation of congestion control is that UE can adjust its transmission power and resource based on its own CBR measurement. In detailed, in mode 1, the network can adjust the resource configuration appropriately based on reported CBR from measurement UE. In mode 2, UE can autonomously adjust its own transmission parameters based on congestion control mechanism which is a function of CBR measurement. For SL-U, the resource pool may be not only used by 3GPP device, but the resource also can be obtained by other RAT, e.g., Wi-Fi system, based on channel access result. In this way, congestion control mechanism should be further supported to optimize in high traffic load for 3GPP system, and to study the potential measurement impact caused by the other RAT’s transmission.

Proposal 42: To support congestion control mechanism for SL-U.

Proposal 43: To study the potential measurement impact caused by the other RAT’s transmission.

# Conclusion

In this contribution, we focus on the physical layer framework for sidelink transmission on FR1 unlicensed spectrum and have below observations and proposals:

Proposal 1: RAN1 should discuss the how to use the remaining PRBs in a RB set

Proposal 2: Support PSFCH interlace design containing L PSFCH interlace equals one PSCCH/PSSCH subchannel, placement of common interlace and the content to be transmitted in the common interlace

Proposal 3: 2 RB-based interlaces are specified for subcarrier spacing of 60kHz for sidelink transmission in FR1 unlicensed spectrum.

Proposal 4: Support configuring multiple resource pools within an RB set configured with interlaced RB-based structure

Proposal 5: For TBS determination it is unnecessary to introduce additional configuration, and the reference number of symbols could be derived based on the (pre-)configured 1st and/or 2nd starting symbol, e.g., total number of symbols determined by 1st or 2nd starting symbol or average number of symbols determined by 1st and 2nd starting symbol.

Proposal 6: Study defining a signal for easing UE blind detection complexity and saving UE battery power.

Proposal 7: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, regarding Tx UE behaviour, Option 3 is supported.

Proposal 8: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, regarding Rx UE behaviour, Option B is supported.

Proposal 9: For burst-based sidelink transmission, from a Tx UE’s point of view, no gap symbol is included in between any two consecutive slots

Proposal 10: Each PSCCH/PSSCH is transmitted from symbol 0 of a slot other than initial slot of a COT.

Proposal 11: COT structure information is needed for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 12: Support dynamically indicated PSFCH resources for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 13: Support suspending PSFCH when the PSFCH can’t be transmitted in same COT with corresponding PSSCH and triggering it in a new COT.

Proposal 14: Support HARQ-ACK feedback timing set comprising one or multiple subsets with each subset including one or multiple HARQ-ACK feedback timing values for dynamically indicating one or multiple PSFCH transmission occasions.

Proposal 15: Support multiple periodic PSFCH occasions per PSCCH/PSSCH transmission.

Proposal 16: Support PSFCH transmission time index for determining multiple PSFCH transmission and retransmission resources per PSCCH/PSSCH transmission.

Proposal 17: For PSFCH transmission using 15kHz and 30kHz SCS and 60kHz SCS, support each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of an RB set.

Proposal 18: For PSFCH transmission using 15kHz and 30kHz SCS and 60kHz SCS, for each PSFCH transmission, support guard band between the nearest dedicated PRB and either of the 2 common PRBs, where 2 common PRBs locate at the two edges of an RB set.

Observation 1: Further study may be needed whether the definition of the existing PSFCH transmission capability psfch-TxNumber is affected by the transmission of common PRBs. It should also be avoided that a UE is required to transmit on more than 2 common PRBs in an RB set, e.g. for the case that multiple RPs are in the same RB set.

Proposal 19: The UE could drop the transmission of common interlace/common PRBs if the transmission of multiple PSFCHs could meet the OCB requirement.

Proposal 20: Support introducing the one-shot HARQ feedback trigger and reporting mechanism for sidelink unlicensed operation.

Proposal 21: Support delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for sidelink unlicensed operation.

Proposal 22: To reuse resource pool relevant design for sidelink on FR1 unlicensed spectrum.

Proposal 23: To study the location of PSCCH in frequency/time domain for interlace-based structure.

Proposal 24: Support that the used interlace index(s) in different RB sets are always same, and explicitly indicating the used sub-channel index(s) and RB set index(s).

Proposal 25: Support PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH.

Proposal 26: To study 2 ,3 or 4 columns of DMRS in one time slot and frequency domain location of DMRS should be designed based on interlace-based structure.

Proposal 27: To support unicast, groupcast option 1/2 and broadcast for unlicensed spectrum。

Proposal 28: Besides legacy PSFCH resources, to study increasing the PSFCH resources for unlicensed spectrum.

Proposal 29: Besides legacy scheduling and feedback mechanism, the assistance information and its content for UE performing LBT operation and the potential solution for LBT failure should be studied.

Proposal 30: RAN1 should discuss the need for signalling the channel access priority value or maximum channel occupancy duration along with the sidelink grant.

Proposal 31:  LBT failures on all the indicated resources in the sidelink grant may trigger UE to request more resources from gNB, RAN1 should discuss ways to request more resource from gNB.

Proposal 32: The assistance information of a UE performing LBT operation is needed to be signaled on sidelink.

Proposal 33: Study on how to use the signaled assistance information (including its content) for LBT operation and resource selection.

Proposal 34: RAN1 could study the possibility of assigning more than 3 consecutive resources with a gap less than 25µs using multi-slot scheduling

Proposal 35: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) (Alt 1).

Proposal 36: Additional candidate S-SSB occasion(s) belong to resource pool (Option 2).

Proposal 37: The number and location(s) of additional candidate S-SSB occasions are separately configured (option 3).

Proposal 38: For S-SSB transmission within 1 RB set, either structure of Option 1-1 or Option 3-1 can be supported for 15 kHz and 30 kHz SCS.

Proposal 39: Regarding S-SSB structure over multiple RB sets, the following can be considered:

- Each S-SSB occasion in frequency can be confined within one RB set.

- Each S-SSB occasion in frequency can span over multiple RB sets.

Proposal 40: Regarding guard band resources, the following can be considered:

- In the case that each S-SSB occasion in frequency is confined within one RB set, the guard band is not used for S-SSB transmission.

- In the case that each S-SSB occasion in frequency spans over multiple RB sets, the guard band in between can be used either for S-SSB transmission or not.

Proposal 41: Support that the starting boundaries for S-SSB periods are not aligned across multiple RB sets.

Proposal 42: To support congestion control mechanism for SL-U.

Proposal 43: To study the potential measurement impact caused by the other RAT’s transmission.

# References

- RP-213678, “New WID on NR Sidelink evolution”, RAN#94-e, Dec. 2021.

- Chairman’s notes on RAN1#112

- R1-2301931,	FL summary#4 for AI 9.4.1.2 SL-U physical channel design framework	Moderator (Huawei)