3GPP TSG RAN WG1 #112bis-e R1-2303318 e-Meeting, April 17th – April 26th, 2023 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 17.5.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on multicast HARQ-ACK information in a Type-3 HARQ-ACK codebook. Source to WG:; Lenovo. Source to TSG:; R1. Work item code:; NR_MBS-Core; Date:; 2023-04-04. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; According to Section 9.1.4, Type-3 HARQ-ACK codebook includes HARQ-ACK information for all configured HARQ processes on all configured serving cells. When a UE is configured to receive multicast on a serving cell and triggered to report a Type-3 HARQ-ACK codebook, this Type-3 HARQ-ACK codebook shall include multicast HARQ-ACK information bits for the serving cell. The multicast HARQ-ACK information bits can be enabled or disabled by RRC signaling or one bit in DCI format 4_2. For multicast PDSCH receptions scheduled by DCI formats associated with G-RNTI/G-CS-RNTI or multicast SPS PDSCH receptions with disabled HARQ-ACK information, how to transmit the corresponding HARQ-ACK information bits in Type-3 HARQ-ACK codebook needs to be resolved. One simple way is to adopt similar rule as what RAN1 has done for Type-1 HARQ-ACK codebook for disabled HARQ-ACK information bits, i.e., use “If a UE would report HARQ-ACK information associated with a G-RNTI for multicast or a G-CS-RNTI with disabled HARQ-ACK information, as described in clause 18, a value of the HARQ-ACK information is a UE implementation choice.” for Type-3 HARQ-ACK codebook generation.. -. Summary of change:; Copy “If a UE would report HARQ-ACK information associated with a G-RNTI for multicast or a G-CS-RNTI with disabled HARQ-ACK information, as described in clause 18, a value of the HARQ-ACK information is a UE implementation choice.” in section 9.1.4.. Consequences if not approved:; Type-3 HARQ-ACK codebook generation is not clear considering multicast PDSCH receptions with disabled HARQ-ACK information bits.. Clauses affected:; 9.1.4. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR.. ### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated numbers of HARQ processes NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is zero. If a UE would report HARQ-ACK information associated with a G-RNTI for multicast or a G-CS-RNTI with disabled HARQ-ACK information, as described in clause 18, a value of the HARQ-ACK information is a UE implementation choice. Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1 Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0 Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1 <Unchanged part omitted>