3GPP TSG RAN WG1 #112bis-e			R1-2303325

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.2

Source:	Ericsson

Title:	Co-channel coexistence between LTE sidelink and NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

In 3GPP RAN1#112 meeting, the following agreements were achieved.

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

The RAN1 status of the co-channel coexistence between LTE sidelink and NR sidelink was discussed in the RAN plenary meeting RAN#99. Specifically, the scope with respect to the subcarrier spacing, the transmission of NR PSFCH is revised in the WID [1], which is summarized below:

Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In this paper, we present our views on the issues and the corresponding solutions to operate LTE-NR co-channel coexistence.

# 2	Solutions for co-channel coexistence between LTE and NR sidelink

In this section, we discuss solutions for co-channel coexistence between LTE and NR sidelinks. First, we discuss a few baseline assumptions to ensure that the solution is useful in practice and to limit the scope of the discussions. Then, we discuss static coexistence solutions, including the Rel16 framework. Finally, we present our view on the dynamic co-channel coexistence of LTE and NR SLs.

## 2.1	Dynamic co-channel coexistence between LTE and NR sidelink

### Transmitting HARQ feedback over PSFCH

To address the power variations and AGC issue of transmitting PSFCH on LTE SL subframes, WID objective has set the following direction:

For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. In the above, - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH

Though this addresses the AGC issue, RAN1 shall aim to protect the key merit of NR SL, i.e., HARQ feedback for PSSCH transmission. It is beneficial to introduce some explicit conditions with respect to the selection of the NR SL candidate resources while reducing the possibility of excessive avoidance of resources. One straightforward approach to ensure the overlapping of PSFCH transmissions with LTE SL resources reserved for LTE SL transmissions is by means of two steps, whereby first the TX UE avoids selecting resources for PSCCH/PSSCH transmissions that overlap with PSFCH resource. In the next step, it is also necessary in some situations to impose another constraint at the RX UE to not transmit on these overlapping resources.

For dynamic resource pool sharing, to ensure the avoidance of PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain, the NR SL TX UE always avoids first selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or NR SL RX UE does not transmit on the overlapping resources.

However, depending on the resource pool configuration and the SL traffic situations in NR and LTE, there may be possibility of excessive avoidance of resources, which incurs increased delay for NR SL traffic.

In some cases, by following the principles of in-device coexistence mechanism with the NR SL module being aware of the LTE SL resource pool configuration, one can also choose a pre-configuration option, where the NR SL RX UE does not avoid PSFCH transmissions. This does not cause any detrimental effects for the coexistence operation provided if there exists an alignment between logical and physical slots with respect to PSFCH slot. We present below different examples to illustrate both cases of misaligned and aligned logical and physical slots with respect to PSFCH slot and the corresponding impact.

Let us denote by  t0SL, t1SL, …, tTmaxSL the set of subframes that may belong to a PSSCH resource pool for LTE SL transmission, which do not include SLSS and reserved subframes and hence not contiguous in time domain. RSSI detection is performed according to ty- Pstep*jSL and Pstep = 100 in ITS band. But the gap is not always equal to 100 ms (can be larger or smaller) due to the SLSS and reserved subframes that are not accounted. Similar situation occurs in NR SL as the slots for SLSS and reserved slots are not accounted. Altogether, the periodic relationship between PSFCH resources in NR and resources used for RSSI detection in LTE is not always guaranteed when they operate on different scales of logical slots than in physical slots.  Figure 1 shows an example where the mismatch between logical and physical slots makes it difficult for LTE SL UE to detect PSFCH transmissions based on RSSI measurement, whereby third and fourth PSFCH slots are perceived wrongly by the LTE SL UE. Specifically, in this example, PSFCH slots are aligned between logical and physical slots before SLSS slot while misaligned after the insertion of SLSS.

Figure 1: Illustration of the mismatch between logical and physical slots, which induces a problem in detecting periodicity of PSFCH transmission. PSFCH periodicity is set to 4 and two SLSS slots occur per 160 ms interval.

Figure 2: Illustration of the mismatch between logical and physical slots does not make a problem in detecting periodic PSFCH transmission. PSFCH periodicity is set to 2 and two SLSS slots occur per 160 ms interval.

Next, we introduce another example in Figure 2, whereby LTE SL UE still be able to detect periodic PSFCH transmission. In this example, two SLSS slots are placed together and PSFCH slots before the SLSS slots and after the SLSS slots are aligned. One of the SLSS slots is perceived as a PSFCH slot by the LTE SL UE, which has no or minimum impact on the LTE sensing and detection of NL SL PSFCH transmission. In both examples, LTE SL resource pool configuration is not considered. If one chooses LTE SLSS slots and NR SLSS slots judiciously in a certain combination, the issue of misalignment is further minimized.  This is illustrated in Figure 3.

Figure 3: Illustration of NR resource pool configuration with its SLSS and LTE resource pool configuration with its SLSS. Both configurations together help in alleviating the misalignment of logical and physical time slots with respect to PSFCH slots. NR PSFCH periodicity is set to 4 and two SLSS slots exist 160 ms interval.

Based on the above, we have the following observation and proposals:

In some cases, the mismatch between logical and physical slots can make a problem in detecting periodic PSFCH transmission and in some cases, there is none or minimum impact.

The mismatch between logical and physical slots is avoided by using an appropriate (pre-)configuration for NR SL (resource pool, SLSS, etc.).

In each PSFCH occasion, PSCCH/PSSCH RX UE transmits PSFCH (equivalently, does not avoid the PSFCH transmissions) only if there exists an alignment between logical and physical slots with respect to PSFCH slot.

### Timeline of sensing information shared by LTE SL module

Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax = 4 ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: any discussion on the earliest information, if needed.

In the last meeting, there was a discussion about the aging of information provided LTE SL module. If the information provided by the LTE SL module is too old, it may not be effective in resource selection facilitating dynamic resource pool sharing. Therefore, one way to solve this is to impose a constraint about the timeline within which the candidate information shared by the LTE SL module can be regarded as a valid information for resource selection.

For dynamic resource pool sharing, for details regarding the timeline on how the NR SL module uses the information shared by the LTE SL module, the information corresponding to the LTE SL subframes in the interval [n-T0, n-T1] is provided at the time n, where T0 = 1000 ms and T1 = 2 ms.

It is worth to mention that after receiving this information from LTE SL module, NR SL module utilizes within 4 ms to perform the resource selection according to the above mentioned agreement.

# 3 	Synchronization for coexistence

In the previous RAN WG1#110 meeting, it was concluded that Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe in the context of coexistence between LTE-V UEs and NR-V UEs through TDM-based semi-static resource pool partitioning.

At least for the case of the same subcarrier spacing of 15 kHz (which is the basis of our Observation 8) and same synchronization source of GNSS in LTE and NR (µ=0), there is no difference in the expressions of DFN (direct frame number) in both radio access technologies, and hence, it is straightforward to have the alignment between the boundaries of LTE SL sub-frame and NR SL slot.

Alternatively, if the synchronization sources are different in both radio technologies (e.g., different synchronization priorities), then there may be misalignment of time boundaries depending on the synchronization source in each case. We discuss below the feasible solutions to address this issue w.r.t. Type A devices:

- In cases where a) LTE UE is synchronized to GNSS while NR UE is synchronized SLSS originating from a UE connected to GNSS, b) LTE UE is synchronized to SLSS originating from a UE connected to GNSS while NR UE is synchronized to GNSS, and c) both LTE and NR UEs are synchronized to SLSSs originating from UEs connected to GNSS, the UEs are synchronized to GNSS either directly or indirectly and thus there is no issue.

- In cases where d) LTE UE is synchronized to GNSS, and NR UE is synchronized to SLSS originating from a UE that is synchronized to other than GNSS, e) LTE UE is synchronized to SLSS originating from a UE connected to other than GNSS while NR UE is synchronized to GNSS and f) LTE and NR UEs are synchronized to SLSSs originating from UEs connected to other than GNSS, new solution is required to make sure the successful operation of coexistence. That is, to enable that the NR SL relies on the synchronization source of LTE SL. This can be possible among type-A devices consisting of both NR and LTE SL modules.

- In case where g) LTE UE is synchronized to SLSS originating from a UE that is connected to other than GNSS while NR UE is either synchronized to GNSS or synchronized to SLSS originating from a UE connected to GNSS, a new solution is required to make sure the successful operation of coexistence. Potential options include:

  - 1) NR UE follows LTE SLSS or 2) NR UE transmits LTE SLSS, which can be easily possible among type-A devices.

To summarize, the defined synchronization issue can be resolved when all NR UEs are type-A devices and if all UEs (including LTE legacy and NR release-18) rely on LTE SLSS.

RAN1 specifies for type-A devices to follow LTE SLSS, if available, in the context of coexistence between LTE and NR sidelinks.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	In some cases, the mismatch between logical and physical slots can make a problem in detecting periodic PSFCH transmission and in some cases, there is none or minimum impact.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For dynamic resource pool sharing, to ensure the avoidance of PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain, the NR SL TX UE always avoids first selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or NR SL RX UE does not transmit on the overlapping resources.

Proposal 2	The mismatch between logical and physical slots is avoided by using an appropriate (pre-)configuration for NR SL (resource pool, SLSS, etc.).

Proposal 3	In each PSFCH occasion, PSCCH/PSSCH RX UE transmits PSFCH (equivalently, does not avoid the PSFCH transmissions) only if there exists an alignment between logical and physical slots with respect to PSFCH slot.

Proposal 4	For dynamic resource pool sharing, for details regarding the timeline on how the NR SL module uses the information shared by the LTE SL module, the information corresponding to the LTE SL subframes in the interval [n-T0, n-T1] is provided at the time n, where T0 = 1000 ms and T1 = 2 ms.

Proposal 5	RAN1 specifies for type-A devices to follow LTE SLSS, if available, in the context of coexistence between LTE and NR sidelinks.

# References

RP-230077, WID revision: NR sidelink evolution, OPPO, RAN#99, Rotterdam, Netherlands, Mar. 20th -23rd, 2023.