3GPP TSG RAN WG1 #112bis-e           	                                   R1-2303327

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 9.18

Source: MediaTek Inc.

Title: On maintenance for MC-Enhancement

Document for: Discussion and decision

- Introduction

During RAN1 #112, companies have extensive discussions and agreements on multi-carrier (MC) enhancement. Some agreements related to “multi-cell scheduling with a single DCI” are copied below [1]:

Agreement

Inclusion of PDCCH monitoring adaptation indication in DCI format 0_X/1_X is configurable.

Agreement

Inclusion of minimum applicable scheduling offset indicator in DCI format 0_X/1_X is configurable.

Agreement

For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following:

- If table defining combinations of co-scheduled cells for the set of cells is configured,

  - an indicator in the DCI is included and points to one row of the table.

  - …

- Otherwise,

  - The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells.

  - …

Besides, for multi-cell scheduling with a single DCI, there is one issue about PDCCH monitoring of non-scheduling cells not addressed yet:

- In cross-carrier scheduling, UE does not expect to monitor PDCCH candidates on non-scheduling cells.

To maintain the benefit of UE power saving, we think the same rule for non-scheduling cells should be applied to R18 multi-cell scheduling.

This contribution provides our views on maintenance for MC-Enhancement.

- On maintenance for multi-cell scheduling with a single DCI

2.1 PDCCH monitoring of non-scheduling cells

In 38.213 [2], for cross-carrier scheduling, it is designated that “UE does not expect to monitor PDCCH candidates on non-scheduling cells”, and this constraint applies to both scheduling DCIs and non-scheduling DCIs.

Observation 1: In 38.213 [2] 10.1, for cross-carrier scheduling, it is designated that “UE does not expect to monitor PDCCH candidates on non-scheduling cells” as shown below, and this constraint applies to both scheduling DCIs and non-scheduling DCIs.

10.1	 UE procedure for determining physical downlink control channel assignment

…

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell.

Observation 1 is important to UE power saving, since the PDCCH monitoring power on non-scheduling cell is saved completely. This is a critical appealing point for UE vendors to implement this kind of cross/multi carrier scheduling features.

Observation 2: Observation 1 is important to UE power saving, since the PDCCH monitoring power on non-scheduling cell is saved completely. This is a critical appealing point for UE vendors to implement this kind of cross/multi carrier scheduling features.

Hence, we have the following proposal:

Proposal 1: To maintain the benefit of UE power saving, the same rule for PDCCH monitoring of non-scheduling cells in legacy is applied to R18 multi-cell scheduling. The following exemplary 38.213 spec text revision can be considered:

- “A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to of PDSCH/PUSCH scheduling for that secondary cell in another serving cell.”

2.2 DCI field types of “PDCCH monitoring adaptation indication” and “minimum applicable scheduling offset indicator”

In RAN1 #112 [1], it is agreed that the following fields are included in DCI 0_X/1_X but the types of these fields are not defined yet:

Agreement

Inclusion of PDCCH monitoring adaptation indication in DCI format 0_X/1_X is configurable.

Agreement

Inclusion of minimum applicable scheduling offset indicator in DCI format 0_X/1_X is configurable.

Observation 3: In RAN1 #112, it is agreed the DCI fields of “PDCCH monitoring adaptation indication” and “minimum applicable scheduling offset indicator” are included in DCI 0_X/1_X (if configured), but the types of these fields are not defined yet.

We hence have the following proposal:

Proposal 2: The DCI fields of “PDCCH monitoring adaptation indication” and “minimum applicable scheduling offset indicator” should both be Type-2 fields (separate fields for each co-scheduled cell).

2.3 On DCI field positions for different co-scheduled cells combination.

In [3] Section 2.2, the topic of “dynamic DCI parsing based on the co-scheduled cell indicator in the DCI” was discussed, it is argued in [3] that the benefit of “dynamic DCI parsing based on the co-scheduled cell indicator” is not clear. In RAN1 #112, two methods based on co-scheduled cell indicator or FDRA are agreed as below, and it seems “dynamic DCI parsing based on the co-scheduled cell indicator in the DCI” should not be supported.

Agreement

For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following:

- If table defining combinations of co-scheduled cells for the set of cells is configured,

  - an indicator in the DCI is included and points to one row of the table.

  - …

- Otherwise,

  - The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells.

  - …

Observation 4: In [3] Section 2.2, it is argued that the benefit of “dynamic DCI parsing based on the co-scheduled cell indicator” is not clear. In RAN1 #112, two scheduled cells indication methods based on co-scheduled cell indicator or FDRA are agreed. Hence, it seems “dynamic DCI parsing based on the co-scheduled cell indicator in the DCI” should not be supported.

We hence have the following proposal:

Proposal 3: RAN1 to clarify that the DCI field position for each cell remains unchanged for different co-scheduled cells combination. One exemplary figure from [3] is shown in Figure 1.

Figure 1. An exemplary figure for DCI field position for each cell (from [3])

- Conclusion

In this contribution, we focus on the discussions for R18 maintenance for MC-Enhancement and have the following observations and proposals:

Observation 1: In 38.213 [2] 10.1, for cross-carrier scheduling, it is designated that “UE does not expect to monitor PDCCH candidates on non-scheduling cells” as shown below, and this constraint applies to both scheduling DCIs and non-scheduling DCIs.

10.1	 UE procedure for determining physical downlink control channel assignment

…

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell.

Observation 2: Observation 1 is important to UE power saving, since the PDCCH monitoring power on non-scheduling cell is saved completely. This is a critical appealing point for UE vendors to implement this kind of cross/multi carrier scheduling features.

Proposal 1: To maintain the benefit of UE power saving, the same rule for PDCCH monitoring of non-scheduling cells in legacy is applied to R18 multi-cell scheduling. The following exemplary 38.213 spec text revision can be considered:

- “A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to of PDSCH/PUSCH scheduling for that secondary cell in another serving cell.”

Observation 3: In RAN1 #112, it is agreed the DCI fields of “PDCCH monitoring adaptation indication” and “minimum applicable scheduling offset indicator” are included in DCI 0_X/1_X (if configured), but the types of these fields are not defined yet.

Proposal 2: The DCI fields of “PDCCH monitoring adaptation indication” and “minimum applicable scheduling offset indicator” should both be Type-2 fields (separate fields for each co-scheduled cell).

Observation 4: In [3] Section 2.2, it is argued that the benefit of “dynamic DCI parsing based on the co-scheduled cell indicator” is not clear. In RAN1 #112, two scheduled cells indication methods based on co-scheduled cell indicator or FDRA are agreed. Hence, it seems “dynamic DCI parsing based on the co-scheduled cell indicator in the DCI” should not be supported.

Proposal 3: RAN1 to clarify that the DCI field position for each cell remains unchanged for different co-scheduled cells combination. One exemplary figure from [3] is shown in Figure 1.

Figure 1. An exemplary figure for DCI field position for each cell (from [3])

- Reference

Chairman’s Notes, RAN1 #112 (9.9 Multi-carrier enhancements for NR)

3GPP TS 38.213 V17.5.0 5G; NR; Physical layer procedures for control

R1-2301429, “Multi-cell PUSCH/PUSCH scheduling with a single DCI”, Qualcomm Incorporated, RAN1 #112