3GPP TSG RAN WG1 #112-bis-e		R1-2303328

Online, April 17th – 26th , 2023

Agenda Item: 9.1.2

Source: MediaTek Inc.

Title: CSI enhancement

Document for: Discussion and Decision

# Introduction

In the RAN#94e meeting, the working item “NR MIMO evolution for downlink and uplink” for Rel-18 is approved. The objectives for DL CSI enhancement include [1]

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

-	Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

-	UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

- Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

- Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this technical document, we share our study results and views based on the agreements from the RAN1#112 meeting [2].

# CSI enhancement for high/medium UE velocities

## Multiple CQI feedback format

In RAN1#112 meeting, we have the following agreements on the slot association for X=1 CQI and the issue of X>1 CQIs [2]

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2

TS38.214 defines the concept of wideband reference CQI and subband differential CQI in the frequency domain (FD), with the wideband CQI being reported using 4 bits and the differential CQIs using 2 bits each. The subband CQIs are then derived by gNB as

csbn=cwb+cdiffn		n=0,…N3-1,

where csbn is the CQI in subband n, cwb is the 4-bit wideband reference CQI, and cdiffn is the 2-bit differential CQI in subband n. The CQI feedback overhead in the absence of such differential CQI is 4N3 bits considering 4-bit subband CQIs. This is reduced to 4+2N3 bits with the differential formulation. The rationale for this CQI feedback overhead reduction is that the subband CQIs do not change significantly from the wideband CQI.

With the agreement of X=2 time domain (TD) CQIs in time domain, the CQI overhead is 24+2N3 bits. However, since the 2nd TD CQI is not expected to vary significantly from a reference TD unit (i.e., 1st TD unit), it is possible to use a similar rationale as in the frequency domain to reduce the CQI feedback overhead. Further, in order to aid omission of UCI in capacity constrained PUSCH, the CQIs of TD unit 0 can be included in CSI part 1, and the CQIs of TD unit 1 are included in CSI part 2.

For X=2, three CQI format alternatives for the 2nd TD CQI have been proposed in RAN1#112:

- Alt1.1. Independent of the 1st TD CQI: A 4-bit wideband CQI and 2-bit sub-band CQIs

- Alt1.2. Differential reference CQI relative to the 1st TD CQI: A BR-bit wideband CQI and 2-bit sub-band CQIs

- Alt1.3. Differential reference and sub-band CQIs relative to the 1st TD CQI: A BR-bit wideband CQI and 1-bit sub-band CQIs

where 0≤BR<4 (0 implies that the reference CQI for the 2nd TD CQI is the reference CQI for the 1st TD CQI)

We describe our understanding of these enhancements in the following:

Alternative 1.2

Denote the CQI in subband n=0,…N3-1 of TD unit x=0,…X-1 as c(x)[n].

A 4-bit wideband reference CQI cwb(0) is reported for CQI time unit 0. Then, subband CQIs of time unit 0 are reporting using 2-bit differential CQIs cdiff(0)n relative to reference CQI, such that the CQIs for all subbands of time unit 0 can be reconstructed by the gNB as

c(0)n=cwb(0)+cdiff(0)n, n=0,…N3-1

Further, wideband CQI of TD unit 1 is reported using BR-bit differential CQIs cdw(x) relative to reference wideband CQI, such that the gNB reconstructs the wideband CQI of the TD unit 1 as

cwb(1)=cwb(0)+cdw(1)

Finally, subband CQIs of the TD unit 1 are reported using 2-bit differential CQIs cdiff(x)n relative to wideband CQI of the TD unit 1, such that the gNB reconstructed subband CQIs are

c(1)n=cwb(1)+cdiff(1)n, n=0,…N3-1

This is illustrated below for easy understanding:

The CQI feedback overhead of this method is 4+2N3+BR+2N3 bits, of which

- Part 1 CSI could contain CQIs of TD unit 0, i.e., c, cdiff(0)n,n=0,…N3-1 (4+2N3 bits)

- Part 2 CSI could contain differential CQIs of TD unit 1, i.e., cdw(1), cdiff(1)n,n=0,…N3-1(BR+2N3 bits)

Since the time domain CQIs do not vary much, the reference CQI of both TD units could be the same, resulting in BR=0, such that FD CQIs of both TD units could be derived from a single reference CQI as shown below:

Alternative 1.3

The CQI reporting mechanism for TD unit 0 remains same as above.

Wideband reference CQI of TD unit 0 is reported using BR-bit differential CQI cdw(1) relative to reference CQI, such that the gNB reconstructs the wideband CQI of TD unit 0 as

cwb(x)=cwb(0)+cdw(x), x=1,…X-1

Further, subband CQIs of TD unit 1 are reporting using 1-bit differential CQIs relative to subband CQIs of time unit 0, such that the gNB reconstructs the wideband CQI of TD unit 1 as

c(1)n=c(0)n+cdiff(1)n, n=0,…N3-1

The CQI feedback overhead of this method is 4+2N3+BR+N3 bits, of which

- Part 1 CSI could contain CQIs of time unit 0, i.e., c, cdiff(0)n,n=0,…N3-1 (4+2N3 bits)

- Part 2 CSI could contain differential CQIs of TD unit 1, i.e., cdw(x),c(1)n, n=0… N3-1 (BR+N3 bits)

The CQI feedback overhead of these alternatives with BR=0 is shown in Table 1. It is seen that Alt 1.3 has the most potential to minimize CQI feedback overhead. From the above analysis, we have the following observations and proposals.

Observation 1: With X=2 CQIs in the time domain, CQI feedback overhead of Rel-18 Type II codebook is twice that of Rel-16 Type II codebook.

Proposal 1: For the Rel-18 Type-II codebook refinement for high/medium velocities, methods to reduce CQI feedback overhead for X>1 time-domain CQIs are needed

Table 1 CQI feedback overhead

| N3 | Rel-16 eType II CQI feedback overhead | Rel-18 Type II CQI feedback overhead |  |  |
| --- | --- | --- | --- | --- |
|  |  | Alt 1.1 | Alt 1.2 | Alt 1.3 |
| 19 | 42 | 84 | 80 | 61 |
| 14 | 32 | 64 | 60 | 46 |
| 7 | 18 | 36 | 32 | 25 |

Proposal 2: For the Rel-18 Type-II codebook refinement for high/medium velocities, reduce multiple CQI feedback overhead using the concepts of reference CQI and differential CQIs

Proposal 3: For the Rel-18 Type-II codebook refinement for high/medium velocities, support differential reference and sub-band CQIs relative to the 1st TD CQI, with a 0-bit wideband CQI and 1-bit sub-band CQIs (i.e., Alt 1.3)

Proposal 4: For the Rel-18 Type-II codebook refinement for high/medium velocities, aid UCI omission of multiple time domain CQIs by including reference CQI(s) and certain differential CQI(s) in CSI Part 1 and including remaining CQIs in CSI Part 2.

## NZC bitmap design

In RAN1#112 meeting, we have the following agreements regarding the NZC bitmap [2]:

Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives (no later than RAN1#112bis-e): - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size to report the selected pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(, ,)} with , where is the threshold that can be configured by gNB, df1,s1,t1=minf1-f0, Mv-f1-f0+mins1-s0, L-s1-s0+mint1-t0, Q-t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively Nokia/NSB, Samsung, vivo, and ZTE raised concerns that, in their understanding, Alt3A violates previous agreements for “Q different two-dimensional bitmaps” and/or common DD basis selection across SD/FD basis pairs and hence, to some extent, objective 1 of the WID. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the down-selection of bitmap(s) for indicating the locations of the NZCs (in RAN1#112bis-e), the following is used as a guidance for evaluation: - Following the agreed EVM, use “UPT vs. overall overhead (including CQI and PMI)” to compare across alternatives, assuming at least FTP1 traffic model and Rel-16 Parameter Combinations (L, beta, pv) - Use only the supported codebook parameter values (e.g. Q, K, m, d, delta, N4) - Companies are to state their assumptions on UE-side prediction (e.g. ideal or realistic, CSI-RS type, CSI-RS overhead calculation in relation to UPT, assumptions on WCSI and l) and the use of rank adaptation

In the offline email discussion, a simpler version of Alt 4 was provided by vivo as follows:

For NZC bitmap in Type II CSI refinements for high/medium UEs, Q different bitmaps are supported for each layer, each of the Q bitmaps corresponds to DD basis q = 0 or 1.

- For each polarization, each of the Q bitmaps contains bits included in a set of SD basis and FD basis pairs {(s, f)}, satisfying min(f,Mv-f)+ min(|s-sref |, L-|s-sref |)≤D, where

  - s∈{0,…,L-1}, f∈{0,…,M-1}

  - sref∈{0,…,L-1} is the SD basis indicated by SCI

  - Two polarizations have same set of {(s, f)} in the bitmap

At the outset, we would like to state that in our understanding, Alt 3A neither violates WID nor does it violate any previous agreements because:

- L spatial domain bases are selected among N1N2 DFT bases in conventional way

- M frequency domain bases are selected among N3 DFT bases in conventional way

- Q doppler domain bases are selected among N4 DFT bases independent of SD and FD bases

- Although RAN1 # 111 agreed to have Q bitmaps, it still allowed for further study of bitmap reduction. Further, Alt 3A does not yield Q identical bitmaps in any case, and UE is always allowed free selection of NZCs, as illustrated in the figure below for the case of M=1, Q=2 and both Doppler components being selected in a bitmap of size MQ, as shown in Fig. 1 below:

Figure 1 NZC bitmap for Alt 3A with M = 1, Q = 2

Observation 2: For the Type-II codebook refinement for high/medium velocities, Alt 3A bitmap overhead reduction scheme neither violates WID nor any previous agreements.

The simplification in Alt 4 comes from the substitution of f0,t0=(0,0) and the fact that Q=2. This means that the strongest coefficient is always mapped to FD basis 0 (as in Rel-16 eType II) and DD basis 0. Since mint1-t0,Q-t1-t0=min1,2-1=1, the calculation of the distance metric reduces to

df1,s1,t1=minf1,Mv-f1+mins1-s0,L-s1-s0+1≤d, or

minf1,Mv-f1+mins1-s0,L-s1-s0≤d-1

Therefore, in our understanding, Alt 4 is not a fundamental simplification, but merely a result of the agreement of Q=2 and the understanding that f0,t0=(0,0). Further, the metric D in the simplification is related to the earlier metric d as D=d-1 as seen from the above equation. Given this understanding, Alt 4, by gNB configuration, forces zeros in certain SD-FD positions away from the SCI and these zero positions are identical in both polarizations and both Doppler bases. This is illustrated in Fig. 2 below:

Figure 2 NZC bitmap for Alt 4

In this figure, we assume that the SCI is located in the position sref=3 and the gNB configures D=d-1=2. The positions denoted by 1s can contain ZCs and NZCs, however, the positions denoted by 0s are frozen to be ZCs by the distance metric configuration. We see that a basic single bitmap for polarization 1 and DD basis 0 are repeated for (polarization 2, DD basis 0), (polarization 1, DD basis 1), (polarization 2, DD basis 1).

Observation 3: For the Type-II codebook refinement for high/medium velocities, Alt 4 bitmap overhead reduction scheme forces zeros in SD, FD positions away from the SCI by gNB configuration, and is not based on actual DL channel property observed by UE.

We compare the performance of the three Alts by simulation assumptions agreed in the last meeting, and the results are shown in Fig. 3. The details of NZC selection for each of these alternatives is described in the following.

Figure 3 UPT vs. overhead performance of NZC bitmap schemes

In Alt 1, Q bitmaps of size 2LM, and a total of K0=β×2LMQ NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers.

In Alt 3A, S=μMQ delay-Doppler pairs are indicated using a bitmap of size MQ. Subsequently, a total of K0=β×2LS NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers. An illustration of Alt 3A is shown in Fig. 4.

Figure 4 NZC indication by Alt 3A

In Alt 4, based on the configuration of D and parameters (L,M,Q), the bitmap size is calculated (as shown in Fig above). In this bitmap, a total of K0=β×2LMQ NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers.

Fig. 3 shows the results for μ=0.75 and D=2 and D=3. Parameter combinations 1~6 of Rel-16 eType II are used for performance evaluation, and the baseline for UPT gain is Rel-16 eType II with parameter combination 1. It is seen that Alt 3A can achieve better UPT-overhead tradeoff compared to Alt 1. Specifically, it can provide 50~60 bits overhead saving compared with Alt 1 with <1 % performance loss.

Further, the results show that Alt 4 (the latest simplified version) with D=3 cannot provide significant overhead gain over Alt 1, although it can maintain similar performance. However, with D=2, the performance starts to degrade at higher parameter combinations (> 2). The main reason is that Alt 4 forces zeros in certain SD, FD positions which could otherwise have been NZCs. Also, it yields same zero coefficient positions for SD beams in the two polarizations and DD bases 0 and 1, which could not be the case in practice (e.g.: SD beams depends on channel eigen directions, so all SD beams should be treated independently, and at low/mid UE speeds, DD basis 1 is expected to have more ZCs than DD basis 0).

Alt 3A becomes a natural choice when considering the codebook structure W1W2Wf⨂WdH, where the delay Doppler bases are jointly represented. Therefore, our first preference is to support Alt 3A. However, considering slightly higher spec effort involved with Alt 3A, we are fine with Alt 1.

Observation 4: NZC indication by Alt 3A can provide 50~60 bits overhead saving compared with Alt 1 with <1 % performance loss.

Observation 5: NZC indication by Alt 4 and D = 3 can achieve similar performance as Alt 1 without significant overhead saving.

Observation 6: NZC indication by Alt 4 and D = 2 degrades in performance especially at higher parameter combinations, due to forcing zero coefficients in certain SD, FD positions.

Proposal 5: For the Type-II codebook refinement for high/medium velocities, support NZC indication by Alt 1 as first preference and Alt 3A as second preference.

## On Parameter Combinations

Agreement On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook (if supported), for the purpose of choosing the supported Parameter Combinations - Regarding the codebook parameter pv, in addition to the supported values from the legacy specification, introduce as additional candidate values - pv =1/8 for v=1,2 (hence 1/16 for v=3,4) - pv =1/2 for v=1,2,3,4 - Regarding the codebook parameter b, in addition to the supported values from the legacy specification, introduce as an additional candidate value b = 1/8 - Regarding the codebook parameter L, the supported values from the legacy specification apply

We don’t see a strong reason to support additional parameter combinations than the ones supported in Rel-16 eType II, therefore we propose to retain the legacy parameter combinations.

Proposal 6: On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook, support the legacy parameter combinations in Rel-16 regular eType-II.

## UCI omission

In RAN #112, we had the following agreement regarding UCI omission for Type II codebook refinement for high/medium velocities [2]:

Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector): - Alt1. Prio(l,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.l+q - Note: This implies that DD basis is designated the highest priority - Alt2. Prio(l,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+l - Note: This implies that DD basis is designated the lower priority (after FD basis) - FFS: S(q) maps the index q according to a rule - Alt3. Prio(l,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l + l - Note: This implies that DD basis is designated the least priority - Alt4. Prio(l,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+l - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis) - FFS: S(q) maps the index q according to a rule FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m q=0,…,Q-1

The alternatives in the agreement can be represented in accordance with the highest to lowest priority of UCI transmission as:

- Alt 1: DD index → Layer index → SD index → FD index

- Alt 2: Layer index → SD index → FD index → DD index permuted

- Alt 3: Layer index → SD index → FD index → DD index

- Alt 4: Layer index → SD index → DD index → FD index

With the agreement of only Q=2 DD bases, it is expected that for low to medium UE speeds (which we think is the most practical scenario of deployment of this codebook), most of the significant codebook coefficients would be associated with the first DD basis and a few coefficients would be associated with the second DD basis. Therefore, the gNB should at least reconstruct the legacy SD, FD precoder corresponding to DD basis 0 accurately in case of UCI omission. This is not possible with Alt 1 and Alt 4 since DD indices are given higher or intermediate priority. We think that DD index should be given the lowest priority, with SD, FD and layer indices following the legacy rule. As for the remapping or permutation of DD indices, we think that it is not needed given that firstly, since q=0,1, there is no room for remapping apart from flipping the indices; and secondly, the UE may by its own implementation designate the strongest Doppler index as DD basis 0.

Observation 7: In case of UCI omission, gNB should be able to at least reconstruct the legacy SD, FD precoder corresponding to DD basis 0 accurately, which is not possible with Alt 1 and Alt 4.

Proposal 7: For the Type-II codebook refinement for high/medium velocities, regarding UCI omission, support Alt 3 (DD basis is designated the least priority without any permutation).

## Time-domain channel properties (TDCP)

In RAN1 #112, the following agreement was made regarding TDCP reporting [2].

Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 FFS: Value of Dbasic

The previous agreement states that TDCP reporting is based on wideband quantized time-domain correlation, where the number of reported coefficients is Y. It also states that the basic feature has Y=1 amplitude-only reporting. Additionally, an optional feature allows Y≥1 coefficients be reported with both quantized amplitude and phase.

Remaining issues include decisions on:

- The value Dbasic which acts as upper bound on reported lag value for the basic feature.

- The quantization scheme for amplitude and phase

- The TRS configurations used for TDCP calculation/reporting.

In this section, we will discuss the aforementioned remaining issues. But first, let us introduce the mathematical analysis for appropriate autocorrelation sampling time determination since this shall help with the Dbasic specification. We start with some definitions we will need.

fc≝ Carrier frequency

c≝ speed of light

τ≝ Time lag used for CSI auto-correlation calculation

A(τ)≝ Normalized auto-correlation function

Ts≝ Auto-correlation sampling time

fs≝ Auto-correlation sampling frequency

NTRS≝ Number of TRS bursts

fd≝ Dopper frequency

Δfd≝ Doppler frequency resolution

fd,max≝ Maximum unambiguous Doppler frequency

v≝ UE speed

Δv≝ UE speed resolution

Since the auto-correlation function is quantized in time, there exist a maximum doppler frequency that can be resolved. This is referred to as the maximum unambiguous (i.e., resolvable) doppler frequency, and is denoted as fd,max. Any doppler frequency higher than this value, will result in aliasing --- a situation we want to avoid. The value of fd,max can be derived using the Nyquist criterion, where

fd,max=fs2=12Ts,

We can treat Ts or fs as design parameters, which we want to choose such that a certain fd,max is achieved. Hence, we need fs to satisfy

fs≥2fd,max=2vmaxfcc

⟺ Ts≤c2vmaxfc

For example, if fc=3.5 GHz and vmax=120 Km/h, then Ts≤ 1.2857 ms.

Assuming subcarrier spacing of 30KHz, this means that we need to sample the channel at least once every 2 time slots (i.e., Ts=1 ms). If we violate this criterion, then we will not be able to take correct decision regarding doppler shift or UE speeds. To show an example of the seriousness of this issue, consider a UE moving at 120 Km/h and carrier frequency fc=3.5 GHz. We use CDL-A channel model and 20 TRS bursts with periodicity of 5 slots, i.e., (Ts=2.5 ms). Then, we plot the doppler spectrum (top) and the auto-correlation function A(τ) (bottom) as follows:

Figure 5. 20 TRS bursts with 5 slot separation.

The figure above depicts a maximum peak at a doppler shift of approximately 40 Hz.

Note also that the maximum doppler shift we expect to get is =±fcvc=±388.88 Hz.  On the other hand, the above figure shows a maximum doppler shift above -200 and below 200 Hz. This is yet another indication that the above figure with Ts=2.5 ms is not correctly depicting doppler spectrum.

Next, we repeat the same experiment and only change the TRS periodicity to 2 slots (i.e., Ts=1 ms). This results in the following figure, which is drastically different from the previous one.

Figure 6. 20 TRS bursts with 2 slot separation.

Now, we see a maximum peak of the doppler spectrum occurring at a doppler shift of approximately -325 Hz, and another lower peak at approximately 250 Hz. For completeness, we also show the result if we kept the TRS periodicity at Ts=1 ms but only used 8 TRS bursts (instead of 20). This results in the following figure.

Figure 7. 8 TRS bursts with 2 slot separation

The difference between using 8 TRS bursts and 20 is the lower resolution of the obtained doppler spectrum for the former. Nonetheless, the figure still predicts a maximum peak at a doppler shift of approximately  -333 Hz and a second peak at 266 Hz.

Even though we discuss the aliasing as it occurs in the doppler spectrum, the same effect happens in the time-domain in the autocorrelation sequence. Thus, to avoid doppler spectrum aliasing, the auto-correlation sequence must be sampled at time lags with a period ≤c2vmaxfc . This should be used as a guide in determining the value of Dbasic. Another way of seeing this aliasing behavior is that a too large sampling time will result in missing the zero-crossing of the autocorrelation sequence. Thus, we may not be able to distinguish between a slow-moving UE where the first zero-crossing has not yet occurred or a fast-moving UE with many zero-crossings have happened prior to autocorrelation sampling.

### 2.5.1 The lag upper bound (Dbasic)

The value of Dbasic can be defined as:

Dbasic ≝ “The maximum lag that gNB can configure for TDCP reporting for the basic feature”.

Further details may also be needed to specify to the UE which exact OFDM symbols are to be used for TDCP calculation and can help inform the choice for possible Dbasic values. The following two figures, Figure 8 and Figure 9 show examples for two-slot and single-slot TRS resources, respectively.

Figure 8. Dbasic examples for two-slot TRS configuration

Figure 9. Dbasic examples for one-slot TRS configuration

The above figures show that lag calculations can either be measured from the same respective OFDM symbol location within the TRS burst, or it can be measured from the first OFDM symbol of the first burst and the last OFDM symbol of second burst.

There are two key issues in determining the value Dbasic. The first is the ability of the receiver to maintain phase coherence across the times at which TRS is received, in addition to the buffering capability of earlier TRS-based CSI estimates. From this perspective, smaller values of Dbasic are preferred. The second issue is aliasing avoidance (see our earlier contribution in R1-2301573). A large value for Dbasic leaves room for situations where doppler aliasing can occur. This is yet another reason why smaller Dbasic is preferred.

Observation 8: Large Dbasic has negative implications for both buffering early CSI estimates for the entirety of the Dbasic duration, as well as losing phase coherence. It may result in aliasing.

Proposal 8: For TDCP reporting,

- Define Dbasic as number of symbols between the first symbol of the first TRS burst to first symbol of the second TRS burst.

- The value of Dbasic = 28 OFDM symbols.

### 2.5.2 TDCP Quantization

In this section, we will mainly focus on the basic TDCP feature where just one amplitude-only coefficient is quantized and reported. We will also shed some light on the optional feature, as well.

While potentially numerous quantization mechanisms can be applied, we prefer to adopt a scheme that is based on theR16 E-Type II codebook quantization. In Type II codebook, amplitude values are quantized using uniform step sizes in dB. The formula for the quantization steps can be written as:

pi=122bits-1-ibase∀i∈0,1,…, 2bits-1,

The step size can also be written as 20log1012-1base= -20baselog1012≈1base×6.0206 dB.

In TS 38.214 two different quantization tables are used in R16 E-Type II Using. In Table 5.2.2.2.5-3, the number of bits used =3 while the value of base=2 (which gives a step size ≈3 dB). On the other hand, Table 5.2.2.2.5-2 utilizes 4 bits and base=4 (which gives a step size ≈1.5 dB).

The quantization steps of Table 5.2.2.2.5-2 can be visualized as follows:

Figure 10. Quantization steps using (bits, base) = (4, 4)

We can see from Fig. 10 that the E-Type quantization mechanism has higher resolution (smaller step sizes) near the small values (i.e., around 0) while it has coarse resolution near 1. This, in fact, is the opposite of what is needed for TDCP quantization, at least based on the codebook switching use case where the optimal codebook switching point is around 10 Km/Hr (refer to our earlier contribution in R1-2301573). Simple modifications of the E-Type II quantization scheme can allow finer resolution around 1 and coarse resolution near 0. For instance, we can use quantization levels at 1-pi or 1-pi2 instead of pi. This results in the following quantization levels.

Figure 11. Quantization steps using (bits, base) = (4, 4).
(a) 1-pi (left), (b) 1-pi2(right)

From the above Figure we see that this quantization mechanism almost fixes the problem, where both schemes have the intended consequence of small step sizes near 1. However,

- The scheme 1-pi has its largest value well below 1 which is problematic.

- The other scheme 1-pi2 is very close to 1. However, the step sizes near 1 are perhaps too small to be useful, and the step sizes near 0 are too large.

For completeness, we also show the quantization levels obtained for the (bits, base) = (3, 2) in Fig. 12 below.

Figure 12. Quantization steps using (bits, base) = (3, 2).
(a) 1-pi (left), (b) 1-pi2(right)

We conduct simulations to evaluate the performance of TDCP quantization using the basic feature, i.e., Y=1 with quantized magnitude quantity of the wideband autocorrelation at lag = 28 OFDM symbols. We evaluate the performance under the codebook switching use case. We use multiple quantization configurations for the number of bits and the quantization step size (controlled by base).

In our simulations, the CSI reporting periodicity is 20 slots, while the TDCP is reported every 80 slots. The simulation is evaluated across as a total of 6000 time slots. For TDCP calculation, we use the formula agreed in RAN1 #111 with the arithmetic averaging, given by

At,τ≈n=0N-1Xnt+τ∙Xn*t12∙n=0N-1Xnt∙Xn*t+Xnt+τ∙Xn*t+τ

where Xn is the measured CSI, and n is the subcarrier index. For the lag value we choose (i.e., 28 OFDM symbols), the optimal threshold value we have is 0.99 which is the average autocorrelation magnitude for UEs at speed of 10 Km/hr. Using the reported TDCP, the CB switching decision is as follows:

CB=E-Type II,                            , if TDCP≤threshold Type I Single-Panel       , if TDCP>threshold

First, we start with the (bits, base) = (3,2) configuration. This means only 8 quantization steps exists and the step size is 3 dB. Fig. 13 shows the resulting performance.

Figure 13 Throughput (left), Quantization MSE (right), for quantized TDCP using both
1-p and 1-p2 and (bits, base) = (3, 2)

The above figure shows that the (3,2) configuration fails to adapt to the chaning UE speeds. Next, we show the performance under (4,4), (5,4) and (5,5) quantization paramters.

Figure 14. Throughput (left), Quantization MSE (right), for quantized TDCP using both
1-p and 1-p2 and (bits, base) = (4, 4), (5, 4) and (5, 5)

All the quantization parameter combinations used in Fig. 14 adapt well to changing UE speeds, except for (4, 4) using with the 1-pi scheme. This can also be seen from the quantization MSE (right).

A key observation from Figure 14 is that even though all 1-pi2 parameters with 4 and 5 bits considered in that figure do achieve great CB switching performance, their quantization error is worse than those using 1-pi at higher UE speeds (≥20 Km/Hr). This is a result of coarser resolution for small TDCP values (well below 1). While this is not an issue for CB switching, it can cause worse performance for the additional feature where Y>1 values are reported, where not all reported TDCP values are expected to be near 1.

Observation 9: If pi are the quantization levels from E-Type amplitude quantization, then using 1-pi for TDCP quantization offers better quantization performance compared to 1-pi2 for TDCP values well below 1.

Proposal 9: Use 1-pi TDCP amplitude quantization where pi are the quantization levels obtained from E-Type II amplitude quantization, where pi is given by

pi= 122bits-1-ibase ∀i∈{0,1,…,2bits-1}

Proposal 10: For TDCP amplitude quantization:

- Use 5-bit quantization

- Use 5≤ base ≤6 (i.e., 1 dB≤ quantization step sizes ≤1.2 dB).

### 2.5.3 TRS configurations used for TDCP calculation/reporting

Legacy TRS configurations have a minimum periodicity of 10 ms. Specifically, the periodicity can be 2μXp slots, where μ is the numerology and Xp∈{10,20,40,80}. Thus, only one periodic TRS will not be enough to obtain autocorrelation at lags below 10 slots (except for intra TRS burst where possible lags are 4, 14 and 18 OFDM symbols). Instead, multiple TRS resources can configured with some non-zero offset between any two TRS resource, where the UE can use all such configured TRS to compute TDCP at the required lags.

The options for configured TRSes can be:

- One periodic TRS resource with periodicity P1 + one or more periodic TRS resource(s) with periodicity P2. Each additional TRS resource can be used to calculate one TDCP coefficient. See Fig. 10 for an example.

- One periodic TRS resource with periodicity P1 + one or more aperiodic TRS resource(s). Each additional TRS resource can be used to calculate one TDCP coefficient. Subject to UE capability of supporting aperiodic TRS.

Figure 15. Two TRS resources with different periodicities.

Proposal 11: Multiple legacy TRS resources can be used for TDCP calculation. No TRS resource configuration enhancement is required.

# CSI enhancement for coherent JT

In this section, we continue the discussion on CSI enhancements for mTRP CJT.

## FD bases selection and reporting for Mode 1 codebook

In RAN1#112 meeting, the following agreement was reached regarding frequency domain (FD) bases selection in Mode 1 codebook for Coherent-JT [2]:

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: where is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource with , and is commonly selected across N CSI-RS resources - Alt2. independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

Alt 1 would be suitable when dominant FD bases are same across TRPs, but are separated by offsets (e.g. related to non-uniform inter-antenna distance and/or frequency offset across TRPs – as in co-site multi-panel deployments). It is also suitable for non-co-located mTRP deployments wherein the FD bases across TRPs only differ by propagation delays resolvable at the configured precoder subband granularity. Alt 2 would be a natural choice considering diverse mTRP deployments. However, considering UE implementation and feedback overhead, Alt 1 has the potential to maintain commonality with Mode 2 codebook, which was the original intention of Mode 1 codebook.

We compare the performance of Alt 1 and Alt 2 by simulations in Fig. 16. The simulation settings are presented in Appendix II. We do not assume dynamic TRP selection, such that it is always 4-TRP transmission. The best combination of SD bases (L1,L2,L3,L4) is chosen by the UE among the agreed combinations in the last meeting. The set of dominant FD bases are found per layer, post the SVD of the mTRP channel, similar to FD bases selection in Rel-16 eType II precoder. For Alt 1, the best FD offsets ϕn=0,1,…N3-1 are found per TRP such that the overall mTRP precoder post FD decompression is close to subband mTRP precoders.

Figure 16 Performance comparison of Mode 1 and Mode 2 feedback schemes

It is seen from the results that at low to medium ISDs (500m), the three codebook configurations perform within 2~3 % of each other. This is because the propagation delays at such ISDs are not resolvable with the configured subband size of 8 RBs, and all three configs yield similar FD bases. As the ISD becomes larger, certain propagation delays could be resolved by Alt 1, and hence it yields 2~3 % gain over Mode 2. Further, at such ISDs, it becomes more probable for a UE to be served by a few dominant TRPs, so Alt 2, which finds independent FD bases across TRPs, gives 1~2 % better performance than Alt 1 at higher parameter combinations.

Based on the above arguments and results, we make the observations and proposals.

Observation 10: Considering UE implementation, feedback overhead, and commonality with Mode 2, Alt 1 is a preferred choice for Mode 1 codebook FD bases selection

Observation 11: FD bases selection Alt 1 does not provide any performance benefit over Mode 2 at low to medium ISDs but provides 2~3 % average UPT gain at high ISD scenarios.

Observation 12: FD bases selection Alt 2 does not provide any performance benefit over Alt 1 at low to medium ISDs but provides 1~2 % average UPT gain at high ISD scenarios.

Proposal 12: On the Type-II codebook refinement for CJT mTRP, for mode-1, support the current formulation of Alt 1 with subband FD granularity and integral FD offset ϕn=0,1,…N3-1.

In the offline discussion, some companies prefer to have TRP specific oversampled FD offset for Alt 1, i.e., there would be an oversampling factor O3 for N3, and UE would need to calculate and feedback the best oversampling factor q3=1,…O3 per TRP. With such oversampling per TRP, UE would need to search among O3 orthogonal groups of FD bases and report the best group (q3) and the corresponding FD offset from the group, which not only increases UE complexity significantly but also increases the feedback overhead. Alternately, if we consider that the UE only needs to compute and feedback the slope of a phase ramp across frequency domain, it is unclear for the UE about the FD granularity to compute this slope and the FD granularity with which it would be applied at the gNB. In such case, UE might need to consider a very fine FD resolution to compute this slope. This increases the UE complexity tremendously.

Observation 13: Computing oversampled FD offset / slope of a phase ramp in frequency domain per TRP increases the UE complexity significantly.

Proposal 13: On the Type-II codebook refinement for CJT mTRP, for mode-1 Alt 1 codebook structure, do not support FD oversampling per TRP.

## On W2 quantization alternatives

In RAN1 #110bis-e meeting, the following agreement was made regarding W2 coefficient quantization for Rel-18 codebook [3]:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Two meetings ago, it was agreed to support quantization Alt 1 – with Cgroup,phase=1and Cgroup,amp=2 for the Rel-18 Type II codebook for CJT. However, there were extensive discussions on Alt 3 – which differs from Alt 1 by having Cgroup,amp=2N, i.e., each polarization of each TRP having its own amplitude reference. It was argued that such amplitude reference would improve performance in geographically distant TRP deployments.

However, amplitude variations across distant TRPs are implicitly taken care in the precoder computation from the joint mTRP channel and consequently in quantization Alt 1. Having separate amplitude references would artificially boost the number of NZC, thereby increasing overhead but not contributing meaningfully to performance improvement. Moreover, following the rationale of multi-TRP transmissions to ensure uniform coverage to all UEs, as long as the UE is close to one or two TRPs in the coordinating set, a single reference amplitude is enough to reap the benefit of CJT.

To support our arguments, we provide simulation results comparing the performance of Alt 1 and Alt 3 in Fig. 17. The simulation scenario and settings are as explained earlier. From the results, we observe that Alt 3 cannot provide consistent performance benefit over Alt 1. Further, the cost of this little performance benefit must be borne by the increased overhead of feeding back multiple reference amplitudes. Therefore, supporting quantization Alt 3 is not necessary.

Proposal 14: For Rel-18 Type II codebook for CJT, revert the working assumption on quantization Alt 3.

Figure 17 Performance comparison of quantization Alt 1 and Alt 3

## Parameter Combination Linkage

In RAN1#112 meeting, the following agreements were reached regarding parameter combinations for Type-II codebook refinement for CJT mTRP [2]:

Conclusion On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv, b} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv, b} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability Agreement On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {an} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,b} from where the value of {pv,b} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,b} for Rel-16-based refinement pv for layers 1-4; b; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16 Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations: NTRP; {Ln} combination. 2; {4,2}. 3; {2,4,2}, {4,2,2}. No other permutations are supported. FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6 Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, support linkage between the list of supported {Ln} combinations and list of supported {pv,b} combinations via pairing each combination for {pv,b} with at least one combination for {Ln}, for each NTRP value. - FFS (by RAN1#112bis-e): The exact list of supported pairs/linkage, or restriction of {Ln} when paired to each of {pv,b} - FFS (by RAN1#112bis-e): Whether/How to support configuration signalling for indicating the linkage - Note: While no additional codebook parameter will be introduced, the total number of SD basis vectors across CSI-RS resources can still be used as a criterion for choosing the supported pairs/linkage

Currently we have 16 options for SD combinations across TRPs and 6 options for (pv,β) combinations, giving a total of 96 SD-FD combinations (across all NTRP values). It is desired that for each NTRP value, the number of parameter combinations be nearly the same as legacy Rel-16 (i.e., 8). However, without any linkage between the two tables, the following issues arise:

- Four tables (one each for NTRP = 1,2,3,4) would be needed to list the ~8 allowed combinations of Ln and (pv,β).

- Since NL combinations of {Ln} are configured by gNB, when a particular (pv,β) combination is specified for each combination, it becomes hard to find a common single (pv,β) configuration allowed for all NL combinations.

To avoid these issues, we propose to link the options of Ln and (pv,β) by the parameter Ltot=n=1NLn. Since the total feedback overhead and performance depends on (Ltot,pv,β), the linkage criterion could depend on the feedback overhead vs. performance tradeoff. From the current options of Ln combinations, we have 6 unique values of Ltot for N>1, i.e., Ltot∈4,6,8,10,12,16. Therefore, as a starting point, the linkage can be defined in the following way:

| Combination | pv for layers 1-4 | β | Conditions on Ltot |
| --- | --- | --- | --- |
| 1 | 18,18,116,116 | 14 | Not applicable for Ltot∈4,6 |
| 2 | 18,18,116,116 | 12 | Not applicable for Ltot∈4 |
| 3 | 14,14,18,18 | 14 | - |
| 4 | 14,14,18,18 | 12 | - |
| 5 | 14,14,14,14 | 34 | Not applicable for Ltot∈12,16 |
| 6 | 12,12,12,12 | 12 | Not applicable for Ltot∈10,12,16 |

With this linkage, when the gNB configures NL combinations of {Ln}, it could configure at most one or two combinations of (pv,β) that are allowed for all values of Ltot among the NL combinations of {Ln}.

Observation 14: With the current combinations of {Ln} and pv,β, four tables (one each for NTRP = 1,2,3,4) would be needed to list the ~8 allowed parameter combinations

Observation 15: Since NL combinations of {Ln} are configured by gNB, with a particular (pv,β) combination specified for each combination, NL separate configurations of (pv,β) would be needed.

Proposal 15: For NTRP>1, link the options of Ln and (pv,β) by the parameter Ltot=n=1NTRPLn in the form of a table listing the allowed combinations of (pv,β) for a particular Ltot.

## UCI omission and CBSR

In RAN #112, we had the following agreement regarding UCI omission and CBSR for Type II codebook refinement for CJT mTRP [2]:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(l,l,m,n)=() .N.RI.P(m)+N.RI.l(n)+N.l+n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(l,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+l - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with , i.e., SD basis index over all resources: Prio(l,l,m,n) = 2Ltot.RI.P(m)+ RI.+RI.l(n)+ l FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource The same rank restriction is applied across NTRP CSI-RS resources

The alternatives for UCI omission can be represented in accordance with the highest to lowest priority of UCI transmission as:

- Alt 1: CSI-RS resource (TRP) → Layer → SD index → FD index

- Alt 2: Layer → SD index → FD index → CSI-RS resource (TRP)

- Alt 3: Layer → SD indexed across TRPs → FD index

When UE computes the CJT precoder according to certain mTRP hypothesis and UCI omission happens, the remaining precoder coefficients cannot be used to reconstruct a lower mTRP precoder. This is because the remaining precoder coefficients do not form the optimal eigen vectors of the lower mTRP composite channel. Due to this, we do not support Alt 2, where the CSI-RS resource index is given the least priority.

In fact, to achieve the goals of performance enhancement due to mTRP spatial diversity and coverage, we prefer Alt 1, such that all mTRP CJT precoder coefficients are utilized jointly to yield at least a beamforming (coverage) gain. However, given the simple extension from legacy single TRP rule, we are also open to consider Alt 3, where the SD beams are indexed across TRPs/ CSI-RS resources.

In all the above alternatives, how to index the CSI-RS/TRP indices is an issue. Since the gNB has access to the individual TRPs RSRPs during the initial mTRP selection phase, it can order the CSI-RS resource elements in the decreasing order of RSRPs, so that UE just uses the natural CSI-RS indices to calculate the priority rules. By such operation, gNB can always ensure that it obtains the precoder coefficients corresponding to the strongest TRPs without much omission.

Observation 16: When UE computes the CJT precoder according to certain mTRP hypothesis and UCI omission happens, the remaining precoder coefficients cannot be used to reconstruct a lower mTRP precoder.

Observation 17: CSI-RS resource index should be given highest priority to ensure that the UE at least benefits from a joint beamforming gain from multiple TRPs.

Proposal 16: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support Alt 1 (CSI-RS index highest priority) as first preference.

Proposal 17: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support Alt 3 (SD indexed across CSI-RS resources) as second preference.

Proposal 18: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, do not support any CSI-RS index permutation. The gNB indexes the CSI-RS resource elements according to certain rule and UE only uses the natural CSI-RS indices for calculating priority rules.

As for CBSR, we think that extending from legacy method of beam specific amplitude restriction, CSI-RS specific amplitude restriction may be supported. It might also be useful to consider SD beam specific amplitude restriction with SD beams indexed naturally across CSI-RS resources. Also, as per legacy, we prefer to retain soft amplitude restriction as being UE optional.

Finally, we think that CBSR could be configured to be off for certain strongest TRPs serving a UE.

Proposal 19: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, support SD beam specific amplitude restriction, with SD beams indexed naturally across CSI-RS resources.

Proposal 20: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, soft amplitude restriction should be a UE optional feature as per legacy.

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Observation 1: With X=2 CQIs in the time domain, CQI feedback overhead of Rel-18 Type II codebook is twice that of Rel-16 Type II codebook.

Observation 2: For the Type-II codebook refinement for high/medium velocities, Alt 3A bitmap overhead reduction scheme neither violates WID nor any previous agreements.

Observation 3: For the Type-II codebook refinement for high/medium velocities, Alt 4 bitmap overhead reduction scheme forces zeros in SD, FD positions away from the SCI by gNB configuration, and is not based on actual DL channel property observed by UE.

Observation 4: NZC indication by Alt 3A can provide 50~60 bits overhead saving compared with Alt 1 with <1 % performance loss.

Observation 5: NZC indication by Alt 4 and D = 3 can achieve similar performance as Alt 1 without significant overhead saving.

Observation 6: NZC indication by Alt 4 and D = 2 degrades in performance especially at higher parameter combinations, due to forcing zero coefficients in certain SD, FD positions.

Observation 7: In case of UCI omission, gNB should be able to at least reconstruct the legacy SD, FD precoder corresponding to DD basis 0 accurately, which is not possible with Alt 1 and Alt 4.

Observation 8: Large Dbasic has negative implications for both buffering early CSI estimates for the entirety of the Dbasic duration, as well as losing phase coherence. It may result in aliasing.

Observation 9: If pi are the quantization levels from E-Type amplitude quantization, then using 1-pi for TDCP quantization offers better quantization performance compared to 1-pi2 for TDCP values well below 1.

Observation 10: Considering UE implementation, feedback overhead, and commonality with Mode 2, Alt 1 is a preferred choice for Mode 1 codebook FD bases selection

Observation 11: FD bases selection Alt 1 does not provide any performance benefit over Mode 2 at low to medium ISDs but provides 2~3 % average UPT gain at high ISD scenarios.

Observation 12: FD bases selection Alt 2 does not provide any performance benefit over Alt 1 at low to medium ISDs but provides 1~2 % average UPT gain at high ISD scenarios.

Observation 13: Computing oversampled FD offset / slope of a phase ramp in frequency domain per TRP increases the UE complexity significantly.

Observation 14: With the current combinations of {Ln} and pv,β, four tables (one each for NTRP = 1,2,3,4) would be needed to list the ~8 allowed parameter combinations

Observation 15: Since NL combinations of {Ln} are configured by gNB, with a particular (pv,β) combination specified for each combination, NL separate configurations of (pv,β) would be needed.

Observation 16: When UE computes the CJT precoder according to certain mTRP hypothesis and UCI omission happens, the remaining precoder coefficients cannot be used to reconstruct a lower mTRP precoder.

Observation 17: CSI-RS resource index should be given highest priority to ensure that the UE at least benefits from a joint beamforming gain from multiple TRPs.

Proposal 1: For the Rel-18 Type-II codebook refinement for high/medium velocities, methods to reduce CQI feedback overhead for X>1 time-domain CQIs are needed

Proposal 2: For the Rel-18 Type-II codebook refinement for high/medium velocities, reduce multiple CQI feedback overhead using the concepts of reference CQI and differential CQIs

Proposal 3: For the Rel-18 Type-II codebook refinement for high/medium velocities, support differential reference and sub-band CQIs relative to the 1st TD CQI, with a 0-bit wideband CQI and 1-bit sub-band CQIs (i.e., Alt 1.3)

Proposal 4: For the Rel-18 Type-II codebook refinement for high/medium velocities, aid UCI omission of multiple time domain CQIs by including reference CQI(s) and certain differential CQI(s) in CSI Part 1 and including remaining CQIs in CSI Part 2.

Proposal 5: For the Type-II codebook refinement for high/medium velocities, support NZC indication by Alt 1 as first preference and Alt 3A as second preference.

Proposal 6: On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook, support the legacy parameter combinations in Rel-16 regular eType-II.

Proposal 7: For the Type-II codebook refinement for high/medium velocities, regarding UCI omission, support Alt 3 (DD basis is designated the least priority without any permutation).

Proposal 8: For TDCP reporting,

- Define Dbasic as number of symbols between the first symbol of the first TRS burst to first symbol of the second TRS burst.

- The value of Dbasic = 28 OFDM symbols.

Proposal 9: Use 1-pi TDCP amplitude quantization where pi are the quantization levels obtained from E-Type II amplitude quantization, where pi is given by

pi= 122bits-1-ibase ∀i∈{0,1,…,2bits-1}

Proposal 10: For TDCP amplitude quantization:

- Use 5-bit quantization

- Use 5≤ base ≤6 (i.e., 1 dB≤ quantization step sizes ≤1.2 dB).

Proposal 11: Multiple legacy TRS resources can be used for TDCP calculation. No TRS resource configuration enhancement is required.

Proposal 12: On the Type-II codebook refinement for CJT mTRP, for mode-1, support the current formulation of Alt 1 with subband FD granularity and integral FD offset ϕn=0,1,…N3-1.

Proposal 13: On the Type-II codebook refinement for CJT mTRP, for mode-1 Alt 1 codebook structure, do not support FD oversampling per TRP.

Proposal 14: For Rel-18 Type II codebook for CJT, revert the working assumption on quantization Alt 3.

Proposal 15: For NTRP>1, link the options of Ln and (pv,β) by the parameter Ltot=n=1NTRPLn in the form of a table listing the allowed combinations of (pv,β) for a particular Ltot.

Proposal 16: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support Alt 1 (CSI-RS index highest priority) as first preference.

Proposal 17: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support Alt 3 (SD indexed across CSI-RS resources) as second preference.

Proposal 18: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, do not support any CSI-RS index permutation. The gNB indexes the CSI-RS resource elements according to certain rule and UE only uses the natural CSI-RS indices for calculating priority rules.

Proposal 19: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, support SD beam specific amplitude restriction, with SD beams indexed naturally across CSI-RS resources.

Proposal 20: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, soft amplitude restriction should be a UE optional feature as per legacy.

# References

- RP-213598, “MIMO evolution for downlink and uplink,” Samsung.

- Chair’s notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- Chair’s notes, RAN1#110bis-e, e-Meeting, October 10th – 19th, 2022.

# Appendix I

SLS assumptions for CSI enhancement of high/medium UE velocities

| Parameter |  | Value |
| --- | --- | --- |
| Scenario |  | Urban macro (UMa) |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Frequency Range |  | FR1 only, 2 GHz. |
| Inter-BS distance |  | 200 m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256 QAM |
| Numerology | Slot/non-slot | 14 OFDM symbols per slot |
|  | SCS | 15 kHz |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU MIMO with rank adaptation |
| MIMO layers |  | Maximum MU layers 8 |
| CSI-RS periodicity |  | As shown in the tables |
| CSI feedback |  | CSI feedback periodicity: 5 slots (baseline) Scheduling delay: 4 ms (baseline) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 100% outdoor |
| UE receiver |  | MMSE-IRC |
| Spatial consistency mobility procedure |  | Disabled |

# Appendix II

SLS assumptions for CSI enhancement of coherent JT

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Outdoor1: Dense urban macro with 4 intra-cell TRPs |
| Frequency Range |  | 2 GHz |
| Inter-site distance |  | 200 m |
| Channel model |  | Based on TR 38.901. Difference in propagation delays between UE and NTRP TRPs is taken into account in the composite Channel Impulse Response (CIR) |
| Antenna setup and port layouts at each TRP |  | 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1) |
| BS Tx power |  | 44 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | According to TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256QAM |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO adaptation with up to rank 2 |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% for MU-MIMO 20 % for SU-MIMO |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Baseline for performance evaluation |  | Single TRP Rel-16 eTypeII |