R1- 2303329

3GPP TSG RAN WG1 Meeting #112 bis

Apr 17th – Apr 26th, 2023

| Agenda Item: | 9.1.3.1 |  |
| --- | --- | --- |
| AI Description: | Increased number of orthogonal DMRS ports Including increasing orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO. |  |
| Source: | MediaTek Inc. |  |
| Title: | Expanding DMRS System Capacity: Seamless Extensions of Existing Patterns |  |
| Document for: | Discussion and Decision |

- Introduction

In the RAN (Plenary) Meeting #94e, a new Work Item (WI) [1] was approved targeting MIMO evolution for Downlink (DL) and Uplink (UL). The 3rd objective of the DMRS enhancments include:

“Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

- Striving for a common design between DL and UL DMRS

- Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS”

The Demodulation Reference Signal (DMRS) is crucial for receiver operations where it enables the receiver to aquire accurate channel state inforation so that it can eliminate the channel’s distortion effect on transmitted symbols (i.e., channel equalization) prior to symbol demodulation.

The current maximum numbers of orthognoal DMRS ports are 4, 6, 8 and 12, which can be obtained using Type 1 and Type 2 DMRS with single and double OFDM symbols, repsectively. These values also dictate the maximum number of simultaneous transmission of data streams since data is always accompanied by DMRS for coherent reception. From this short description, it becomes clear that the current DMRS design limits the number of MU-MIMO data streams to ≤12.

In this technical document, we share our study results and views based on the agreements from the RAN1#112 meeting [2].

- MU-MIMO Operation

2.1 MU-MIMO between R15 and R18 ports

Support of Rel 15 (FD-OCC length 2) and Rel 18 (FD-OCC length 4) DMRS ports sharing the CDM group, can lead some practical issues depending on the legacy R15 UE implementation, specifically for DMRS processing.  This is mainly due to fact that the legacy UEs need to only consider FD-OCC length 2, which is more robust to long delay spread channel, hence, their performnace may be impacted when they faced with FD-OCC length 4. Moreover, if we have to cover multiple UEs under single CDM group with R15 and R18 DMRS ports, it would be hard to maintain orthogonalilty. The orthogonality could be easily broken across the ports of different UEs as UEs could have different calibration coefficients and even other channel conditions like propogation delays could be different for each UE. After all ignoring the channel effects, we can only ensure a maximum of single layer transmission for Legacy R-15 UE when co-scheduled with R-18 UE within single CDM. To multiplex R-15 and R-18 UEs within a single CDM, both the code-covers of OCC length 2 and OCC length 4 has to be orthogonal each other. Thus, ensuring the orthogonality between R-15 UE with OCC 2 and first half and second half of R-18 DMRS OCC 4, only single layer can be allocated to the Legacy UE (as shown in below figure). Whereas on the other hand, by efficient scheduling of UEs across the CDM groups, can provide more benefits by supporting all four ports within CDM group as against 3 ports in co-scheduling case. By considering all these scenarios, we prefer not to support MU-MIMO operation within single CDM group between R15 and R18 DMRS ports.

Proposal 1: To deprioritize the support of Rel 15 (FD-OCC length 2) and Rel 18 (FD-OCC length 4) DMRS ports sharing the CDM group.

Fig.1. Only single layer transmission support for R-15 UE when co-scheduled with R18 UE in 1 CDM group

2.2 MU-MIMO Restrictions Within a CDM Group

As discussed during RAN1-112, to avoid co-scheduled SU+MU DMRS ports exceeding the total number of DMRS ports that a UE can support, certain restrictions are needed on co-scheduled MU ports. Such restriction is also considered in the legacy Rel-15 DMRS design, as in TS 38.214 Section 5.1.6, please see below for reference.

For DM-RS configuration type 1,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

For DM-RS configuration type 2,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

Hence, following the legacy DMRS MU-MIMO restriction we would new restriction needs to be added to cover Rel-18 DMRS configurations. Here we propose that UE that is configured with new DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is supported given the following restrictions:

Proposal 2: For single symbol DMRS, if the DMRS ports of a UE are in two or more CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

Proposal 3: For double symbol DMRS, a UE does not expect DMRS ports from a co-scheduled UE in the same CDM group as the UE, unless the UE and the co-scheduled UE are each assigned with a different TD-OCC for their DMRS ports with respect to each other.

- Antenna Port field for PDSCH

During RAN1-112, the discussions on the antenna port signalling for PDSCH  was done and port index combinations were agreed for single symbol DMRS port mapping. Regarding two codeword transmission with more than 4-layers using single symbol DMRS, working assumption on the port indexing has been made in the last meeting. We would like to further extend the working assumption on the port combinations for 2 CW by adding new entries. The combinations are extended from Legacy port combinations and New ports are included with higher layer transmission. The new entries allow direct CDM group and CW association, i.e., no cross CDM group CW mapping, leading to more efficicnt UE processing as the UE side.

Proposal 4: Support confirming the working assumption with the following update.

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Support row 0-7 for 2 CWs in Table 4-0.

- Subject to UE capability, UE can indicate one of the following port combinations;

  - {0,1,2,3} only,

  - {4,5,6,7} only,

  - {0,1,2,3,4,5,6,7}.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRSCDM group(s)without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 0,1,2,3,10 |
| 5 | 2 | 0,1,8,2,3,10 |
| 6 | 2 | 0,1,8,2,3,10,11 |
| 7 | 2 | 0,1,8,9,2,3,10,11 |

- PTRS enhancements

PTRS enhancements are needed in UL as we tend to support 8Tx in Rel-18 UL. We are discussing the CB design for Ng = 2 and 4, where Ng is the number of coherent groups.  As there are four non coherent groups, each group can be associated with a panel in a use case. Therefore, 4 PTRS ports are needed where each PTRS port is associated with each panel (or coherent group).

Proposal 5: Support up to 4 port PTRS for UL 8Tx Operation, where 4 ports PTRS is only applicable to Ng = 4 configuration, where Ng denotes number of UL panels.

- Conclusion

Based on above discussion, we provide the following proposals related to DMRS enhancement:

Proposal 1: To deprioritize the support of Rel 15 (FD-OCC length 2) and Rel 18 (FD-OCC length 4) DMRS ports sharing the CDM group.

Proposal 2: For single symbol DMRS, if the DMRS ports of a UE are in two or more CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

Proposal 3: For double symbol DMRS, a UE does not expect DMRS ports from a co-scheduled UE in the same CDM group as the UE, unless the UE and the co-scheduled UE are each assigned with a different TD-OCC for their DMRS ports with respect to each other.

Proposal 4: Support confirming the working assumption with the following update.

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Support row 0-7 for 2 CWs in Table 4-0.

- Subject to UE capability, UE can indicate one of the following port combinations;

  - {0,1,2,3} only,

  - {4,5,6,7} only,

  - {0,1,2,3,4,5,6,7}.

Proposal 5: Support up to 4 port PTRS for UL 8Tx Operation, where 4 ports PTRS is only applicable to Ng = 4 configuration, where Ng denotes number of UL panels.

- References

- 3GPP RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021

- Chair’s notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.