3GPP TSG RAN WG1 #112-bis-e		R1-2303330

Apr 17th  – Apr 26th , 2023

Agenda Item: 9.1.4.2

Source: MediaTek Inc.

Title: SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for: Discussion

# Introduction

In the RAN (Plenary) Meeting #94e, a new Work Item (WI) was approved targeting MIMO evolution for Downlink (DL) and Uplink (UL). The 5th objective of the aformentioned WI is:

“Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices
Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study”

The lagacy specifications include support of up to 4 UL TX ports, allowing up to 4 trasnmission layers with Codebook and Non-Codebook based schemes.

- The codebook transmission scheme entails providing the UE with a specific Transmit Precoder Matrix Indicator (TPMI) chosen from a predefined codebook, based on non-precoded SRS transmission.

- The non-codebook scheme, on the other hand, relies on channel reciprocity, to deduce general precoding weights. Those weights are in turn used to precode SRS. Up to 4 SRS resources in one SRS Resource Set are configured, with only one port per SRS resource. The Network (NW) indicates to UE which precoding weights to use, in addition to the trasnsmission rank, through SRS Resource Indicators (SRI).

In this document, we discuss enhancememnts to TPMI and SRI targeting both codebook and non-codebook transmission to enable up to 8 TX UL operation .

# Full Coherent CB Design

In RAN1-112 it was agreed to reuse DL Type I single panel CB with oversampling factor of 1 is to be supported for UL 8Tx full coherent codebook design. Furthermore, RAN1 is still open to discuss on oversampling factor greater than 1. In this meeting, we try to present some of the evaluations on the impact of oversampling factors in the UL.  Specifically, for codebook design, two layouts are agreed for considerations, namely, (N1, N2) = (4,1) and (2,2), where N1 and N2 denotes number of antenna elements in the horizontal and vertical dimensions, respectively. To realize Type I CBs for coherent UL 8TX transmission with performance and DCI overhead trade-off, we tend to support Type I CBs without oversampling factor design for full coherent UEs.

DL Type I has adopted oversampling factors for the DFT vectors used in constructing the precoders to further improve spatial granularity. Oversampling factors used in DL Type I codebook both horizontal, O1, and vertical dimensions, O2, have a fixed value of 4. However, having such high oversampling for UL codebook can result in significant DCI payload for TPMI indication. To reduce the feedback overhead and verify the performance, we conduct simulations with lower oversampling factors. The oversampling factors are reduced from (4,4) to (2,2) and (1,1). The evaluation below were conducted for (N1, N2) = (2, 2) configuration. Based on the results below, we observe that increasing the oversampling factor does not benefit systems performance. In our study both UMa and UMi environments with 32Rx and 64Rx at the gNB, as per agreed EVM, were evaluated. As the ratio of Rx ports at the gNB to Tx ports at the UE increases, dominant receiver beamforming can be achieved in average throughputs. On the hand, we experience a marginal improvement in terms of cell edge performance in some cases of restricted rank (rank≤4) transmission. Thus, based on overall performance and overhead trade-off, we do not prefer to support oversampling factors greater than one.

Fig1. Avg. Throughput comparison with different oversampling factors

Observation 1: Unlike DL, increasing the DFT oversampling factor for purposes of full coherent 8 TX UL transmission precoder, does not result in significant performance improvement.

Proposal 1: Consider only (O1, O2) = (1,1) with DL Type I CBs for full coherent UE as there is not significant benefits observed with oversampling factor greater than 1.

Table 1. SE performance with different oversampling factors in UMa and UMi respectively.

|  |  | Avg. SE |  | cell edge SE |  |
| --- | --- | --- | --- | --- | --- |
| 32Rx, max 4-layer transmission | O1, O2=1 | 11.63 | 100% | 0.217 | 100% |
|  | O1, O2=2 | 11.78 | 101% | 0.229 | 106% |
| 64Rx, max 4-layer transmission | O1, O2=1 | 17.11 | 100% | 0.725 | 100% |
|  | O1, O2=2 | 17.13 | 100% | 0.745 | 103% |

|  |  | Avg. SE |  | cell edge SE |  |
| --- | --- | --- | --- | --- | --- |
| 32Rx, max 4-layer transmission | O1, O2=1 | 10.0 | 100% | 0.234 | 100% |
|  | O1, O2=2 | 10.21 | 102% | 0.255 | 109% |
| 64Rx, max 4-layer transmission | O1, O2=1 | 15.88 | 100% | 0.488 | 100% |
|  | O1, O2=2 | 15.96 | 101% | 0.483 | 99% |

# Partial and Non-Coherent CB Design

Partial and non-coherent CBs to be considered for multi-group (or panel, Ng>1) UE antenna layout where coherency is not ensured across the groups. When the UE cannot maintain phase coherence across all 8 TX ports, UL transmission on TX antennas will have to reflect this limitation in CBs. We need to design the CBs for those UEs which are partially or even non coherent. The antennas which are coherent should be considered as one group and the codebook weights corresponding cross-panels are considered as zeroes. In case of partial coherence, there could be number of possibilities on how many coherence groups can be formed from the 8Tx antennas. For the ease of codebook design, we prefer to start the CB design with Ng = 2 and then Ng = 4 can be followed.

In this paper, we present design of CB design for Ng=2. The approach we present aligns with the agreements made during the previous meeting and result us to CBs generated from the co-phased beams. This approach infact is well aligned with the procedure of Type I for UL and we no more use tabulated codebook matrices. The unified approach, we present here can be used for the CB generation of Legacy full coherent 2Tx/4Tx CBs as well. While designing the codebooks, we follow set of following principles,

- CBs are generated from common basis and co-phasing factors which can be used for both Ng = 2 and Ng = 4.

- We try to split the layers across the coherency groups and the aim is to support atleast one layer on each coherency group. This helps in utilizing the full power transmission incase PAs without maximum power rating are being used at each coherency group.

- More number of precoders are supported for low rank scenarios and higher rank precoders are less in number. This is a valid consideration as higher rank is chosen typically at higher SNRs and the UL precoder granularity will not provide significant gains in such scenarios. Moreover rank>4 is still considered to be optional in UL 8TX.

- The design approach is developed by limiting the DCI overhead to maximum of TPMI alone (excluding RI) to maximum of 6 bits.

Table II: CB generation procedure for Ng = 2

Layer 1: W1=A∅A00 where A∈11;1-1;1j;1-j; ∅∈{1,j,-1,-j} Example: a1a2∅a1∅a20000 or 0000a1a2∅a1∅a2 where a1a2 is the selected basis A

Layer 2: Layer split approach: W2=A0∅A000B∅B where A, B∈11;1-1;1j;1-j; ∅∈{1,j} Example: W2=a10a20∅a10∅a200b10b20∅b10∅b2 where a1a2 is the selected basis A

Layer 3: W3=AA0∅A-∅A00000B∅B where A, B∈11;1-1;1j;1-j; ∅∈{1,j} Details: Indication of the coherency group with maximum layers is included within TPMI.

Layer 4: W4=A0∅A000B∅B where A,B∈111-1;11j-j; ∅∈{1, j}

Layer 5: W5=AA000∅A-∅A0000000B∅BB-∅BBj∅B where A,B∈11;1-1;1j;1-j; ∅∈{1,j}; Details: Indication of the coherency group with maximum layers is included.

Layer 6: W=AAA000∅A-∅Aj∅A000000000B∅BB-∅BBj∅B where A,B∈11;1-1;1j;1-j; ∅∈{1,j}; Details: Indication of the coherency group with maximum layers is included.

Layer 7: W7=AA000∅A-∅A0000000B∅BB-∅BBj∅B where A∈111-1;11j-j;B∈11;1-1;1j;1-j; ∅∈{1,j}

Layer 8: W8=AA00∅A-∅A000000B∅BB-∅B where A∈111-1;11j-j;B ∈111-1;11j-j; ∅∈{1,j}

Proposal 2: Prioritize CB design for Ng = 2, specifically for partial coherent transmission.

Proposal 3: Support to split the layers across the coherency groups as it allows for efficient power utilization in case of different PAs associated with different coherency groups.

Observation 2: Considering all possible layer splitting precoders for 8Tx partial coherent CB construction can lead to large signaling overhead.

Table III: Possible layer distribution across coherency groups.

| Rank | All layers in one coherency Group | Layers split across 2 coherency Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) | (1,1) |
| 3 | (3,0), (0,3) | (2,1), (1,2) |
| 4 | (4,0), (0,4) | (2,2), (3,1), (1,3) |
| 5 |  | (4,1), (1,4), (2,3), (3,2) |
| 6 |  | (4,2), (2,4), (3,3) |
| 7 |  | (4,3), (3,4) |

Green colored entries: Our first preference. Yellow colored entries: second preference. Non highlighted: To be dropped.

Proposal 4: Restrict layer splitting combinations as explained in the Table III to limit the TPMI field size.

Proposal 5: Precoder structure as presented in Table II can be adopted for partial coherent CBs.

Proposal 6: Consider 2Tx basis and co-phasing factors to generate co-phased beams which will serve as the precoders of partially coherent UE (Ng = 2 or 4). The following predefined basis set is considered to generate all the possible precoders.

- 11;1-1;1j;1-j; 111-1;11j-j; ∅∈{1,j,-1,-j}

Proposal 7: Rank dependent co-phasing factors restriction to be considered for DCI overhead saving.

# Full Power Transmission

Full power transmission on non-coherent and partial-coherent 8 Tx UE is to be supported and the solutions can be reused from Rel-16. Three modes of full power operation based on UE capability has been addressed in Rel-16. Extending all the modes for UL 8TX needs some discussion, however, considering Mode 0 for UL 8TX would be the safest option to start with. In Mode 0 operation, we assume that all PAs are full rated and can transmit partial coherent precoded signal with full power potential. We also believe given the limited time remaining to complete the remaining issue with regards to 8 TX UL operation we may need to agree on atleast one full power mode (Mode 0) supported in R18 8TX to contain the effort needed for this feature.

Proposal 8: Support to reuse Rel-16 Mode 0 full power operation for UL 8Tx.

# Conclusion

Observation 1: Unlike DL, increasing the DFT oversampling factor for purposes of full coherent 8 TX UL transmission precoder, does not result in significant performance improvement.

Observation 2: Considering all possible layer splitting precoders for 8Tx partial coherent CB construction can lead to large signaling overhead.

Proposal 1: Consider only (O1, O2) = (1,1) with DL Type I CBs for full coherent UE as there is not significant benefits observed with oversampling factor greater than 1.

Proposal 2: Prioritize CB design for Ng = 2, specifically for partial coherent transmission.

Proposal 3: Support to split the layers across the coherency groups as it allows for efficient power utilization in case of different PAs associated with different coherency groups.

Proposal 4: Restrict layer splitting combinations as explained in the Table III to limit the TPMI field size.

Proposal 5: Precoder structure as presented in Table II can be adopted for partial coherent CBs.

Proposal 6: Consider 2Tx basis and co-phasing factors to generate co-phased beams which will serve as the precoders of partially coherent UE (Ng = 2 or 4). The following predefined basis set is considered to generate all the possible precoders.

- 11;1-1;1j;1-j; 111-1;11j-j; ∅∈{1,j,-1,-j}

Proposal 7: Rank dependent co-phasing factors restriction to be considered for DCI overhead saving.

Proposal 8: Support to reuse Rel-16 Mode 0 full power operation for UL 8Tx.

# References

[1] R1-2206994 “SRI/TPMI enhancement for enabling 8 TX UL transmission”, MediaTek

# Appendix

|  | Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h | Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h |
| --- | --- | --- |
| Parameter | Value | Value |
| Channel model | 38.901 | 38.901 |
| System bandwidth | 20 MHz | 20 MHz |
| gNB RX antenna setup and port layouts | Outdoor FWA: (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8) 𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8) 𝜆 | Indoor FWA: (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8) 𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8) 𝜆 |
| Antenna radiation pattern parameters | 38.901 | 38.901 |
| gNB receiver noise figure | 5dB | 5dB |
| gNB receiver | MMSE-IRC | MMSE-IRC |
| gNB scheduler | Single user with proportional fair | Single user with proportional fair |
| Modulation | - Up to 256QAM | - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation | SU-MIMO with rank adaptation |
| UE speed | 3 Km/hr | 3 Km/hr |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 | To be defined according to outcome of Proposal 2.1 |
| CBs | Legacy Concatenated, DL Type I | Legacy Concatenated, DL Type I |
| Precoder granularity | Wideband | Wideband |
| Power control | Open loop, - alpha = 0.8- P0= -50 dBm | Open loop, - alpha = 0.8- P0= -80 dBm |
| UE power rating | 32 dBm | 23 dBm |
| Metric | UL mean-user throughput | UL mean-user throughput |