3GPP TSG RAN WG1 #112-bis	R1-2303332

e-Meeting, April 17th – April 26th, 2023

Source: 	MediaTek Inc.

Title: 	Evaluation on low power WUS

Agenda item:	9.11.1

Document for:	Discussion

# Introduction

In RAN1#111, use cases, MR ultra-deep sleep state, coverage evaluation assumptions, and link-level simulation assumptions for main radio (MR) and low-power wake-up receiver (LPWUR) were discussed and endorsed. The endorsed agreements are listed in the appendix

In this contribution, we evaluated OFDMA-based and OOK-based LPWUR, comparing their PSG, use cases, and LLS performance under similar receiver and resource assumptions. We suggest combining these two technologies for improved power savings, lower latency, and reduced signaling overhead.

# Evaluation Assumptions

## Link-level Assumptions

### MDR/FAR Assumption

Agreement in the post-email discussion after RAN1#112 For link-level simulation of LP-WUS, the following table is used as starting point, - FFS for other assumptions if any - Note: The assumptions are not intended to limit the scope of the study or the design. Table XX. Simulation assumptions for LP-WUS (MDR/FAR assumption only) Attributes; Assumptions. MDR/FAR assumption; The miss-detection rate (MDR) of LP-WUS 1%, The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behavior, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials.

What it is: Assumptions to perform link-level (LLS) simulations, including waveforms, required time and frequency resources, and performance targets such as miss detection rate (MDR) and false alarm rate (FAR) are listed.

What the problem is: Alt 2 determines FAR target across a reference time duration of K WUS attempts. It will lead to a challenging FAR target per WUS such as 0.1/K% or 1/K%.

Why it matters: It is unclear whether FAR below 1% is achievable given a reasonable resource and a noise level. For example, R1-2301579 shows 1% FAR is only achievable by losing 3-dB MDR performance and given a 32-bit preamble modulated by OOK. Presumably, lower FAR will require more MDR loss or a longer preamble than 32 bits.

Support of Alt 2 (FAR target is determined across a reference time duration of one or multiple WUS attempts/trials) may result in the target FAR values below 1% or below 0.1%, which may not be feasible if there is no CRC for further confirmation.

Clarify the baseline of [0.1%, 1%] for the false-alarm rate (FAR) of LP-WUS agreed in RAN1#112 is defined by FAR per single WUS attempt/trial, otherwise, non-feasible FAR targets will be included.

### Frequency Error/Drifting

Working Assumption in the post-email discussion after RAN1#112 For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20.

What it is: Presumably, the oscillator and the RTC above refer to multiple clock models considering the required power consumption and the achievable performance used to evaluate time error. Typically, RTC refers to a 32KHz crystal oscillator (XO) with a low power consumption of 1uW.

What the problem is: The oscillator/clock error cannot be used for a mixer directly. The initial/residual frequency error to convert the RF signal is from the voltage-controlled oscillator (VCO) circuits, depending on how to implement a phase-locked loop (PLL) or a frequency-locked loop (FLL).

Why it matters: The current working assumption implies the RTC cannot be used by a mixer to down-convert the RF signal. But it is not true. Another problem is that the max frequency errors given by the clock are used as carrier frequency offset (CFO). But it is not correct as well.

Note: Based on the following two works of literature [1] and [2], the local oscillator (LO) used for a mixer is driven by a 32KHz RTC. The LO frequency error is not only caused by the RTC but also by the performance of FLL circuits. In [1], the amount of LO frequency fluctuation is limited to ±0.2MHz. Combined with LO frequency-tuning step, the overall LO frequency error is then bounded within ±0.5MHz.

Figure 1: How a 32KHz RTC drives a mixer to convert RF signal into baseband in [2].

Figure 2: FLL time-domain simulation where the error is bounded within ±0.5 MHz in [2].

Clock frequency errors, i.e., oscillator and RTC frequency errors in the working assumption, cannot be considered as the CFO assumption used to convert RF signal into baseband.

The working assumption of clock modeling cannot be used for carrier frequency error (CFO), e.g., 200 ppm. Remove option 1 (200, 0.1), option 2 (50, 0.1), and the context for the frequency error assumption to prevent misleading.

## Coverage Evaluation Assumptions

Agreement in RAN1#112 For coverage evaluation, the following is used, Delay spread; Urban: 300ns, optional: 1000ns, and companies to provide descriptions for such scenarios Rural: 300ns.

What it is: In TR 38.901, channel models are based on real channel measurements. Delay spreads are provided by normalized values among taps and a scaling factor such as 300ns or 1000ns.

What the problem is: From the TDL-C-300ns model, we can see the maximum delay span is 2.6us, larger than the CP length of 2.34us for 30kHz SCS. If we consider 1000ns, the maximum delay span is 8.6us, 3 times larger than CP.

Why it matters: The optional case of 1000ns will result in an overdesign of LPWUR, leading to additional power consumption or shortening the capable coverage. We cannot see any value to consider the case of 1000ns in the LPWUR design, which may outperform MR’s performance.

In the assumption of delay spread corresponding to the TDL models, the value of 1000ns has the max delay span of 8.6us, three times larger than the CP length of 2.34us for 30kHz SCS.

Change the assumption of delay spread from optional 1000ns to optional: 100ns, considering the agreed values of 300ns and 1000ns result in larger delay spans than the CP length for 30kHz SCS.

## Power Model Assumptions

### MR Ultra-deep Sleep

Agreement in RAN1#112 For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

What it is: The ultra-deep sleep has been introduced to FR1 MR, considering more power-consuming hardware components can be turned off. Since MR is expected to support IoT, wearables, and eMBB devices, two options for transition energy and time are listed to match a use-case device.

What the problem is: Alt 2 is still agreed upon as a starting point. Without confirmation, we cannot use it as an assumption for performance evaluation. Also, no recommendation on how a company should use these alternatives.

Why it matters: Using Alt 1 to evaluate LPWUR accompanied with all use cases may lead to optimistic results. If companies consider using LPWUR with a more complicated device, e.g., wearables or eMBB devices, Alt 2 should be considered as an evaluation assumption.

Confirm Alt 2: (40000, 800ms) being the transition energy and transition time for FR1 MR ultra-deep sleep state.

### LR Ramp-up Time

Agreement in RAN1#111 The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed.

What it is: The power model for LPWUR in FR1 defines two power states: Off and On, with a relative power of 0.001 for the Off state and 0.01/0.05/0.1/0.5/1/2/4 for the On state. The transition energy and the ramp-up time are FFS.

What the problem is: The relative power values cannot match the power consumption for an OFDMA-based LPWUR. Also, the ramp-up time needs to associate with the relative power values based on its RX architecture.

Why it matters: If the power model is inaccurate, it could lead to incorrect predictions of the power consumption, which could affect the reliability of the LP-WUR.

Note: Phase lock loop (PLL) lock time can be less than 1 millisecond, dependent on the type of algorithms used and the LPWUR sampling rate. In addition, a frequency lock loop (FLL) typically locks faster than a PLL. It seems unreasonable to define ramp-up time based on whether PLL/FLL is needed.

The current relative power values for LPWUR are up to 4 and they cannot reflect the need to support OFDMA-based LPWUR.

Add relative power values 20 and 40 for the power state ON to support OFDMA-based LPWUR and set ramp-up time to zero for OOK/FSK LPWUR with relative power values smaller than 20.

# Evaluation Results

## Power Saving Gains

Table 1: PSG evaluations for OOK-based and OFDMA-based LPWURs

| PSG w.r.t. R17 |  | OOK-based LPWUR Relative power = 0.1 |  | OFDMA-based LPWUR Relative power = 40 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No duty cycle | Duty cycle (0.3%) | No duty cycle | Duty cycle (0.3%) |
| IDRX | No RRM relax | -3% [Table 4] | 2% [Table 7] | -1021% [Table 10] | -4% [Table 13] |
|  | RRM relax (4 times) | 28%* [Table 5] | 35% [Table 8] | -1324% [Table 11] | 26%** [Table 14] |
| eDRX | No RRM relax | 37% [Table 6] | 47% [Table 9] | -4223% [Table 12] | 34% [Table 15] |
| * most likely needs periodic LP-SS ** may reuse NR PSS/SSS |  |  |  |  |  |

Table 2: PSG evaluations to combine OOK-based and OFDMA-based LPWURs

| PSG w.r.t. R17 |  | Combine OOK-based LPWUR and OFDMA-based LPWUR Relative power = 0.1+40×0.3125%=0.225 |
| --- | --- | --- |
|  |  | Duty cycle (0.3125%) is only used for OFDMA-based LPWUR |
| IDRX | No RRM relax | -10% |
|  | RRM relax (4 times) | 20% |
| eDRX | No RRM relax | 23% |

What it is: Power saving gain (PSG) for OOK-based, OFDMA-based LPWUR, and a combined LPWUR using OOK-based and OFDMA-based LPWUR. We observe 28% to 47% PSG for OOK-based LPWUR, 26% to 34% PSG for OFDMA-based LPWUR, and 20% to 23% PSG for the combined receiver.  For non-combined cases, a long duty cycle or periodic LP-SS will be needed to achieve the gain.

What the problem is: OOK-based LPWUR has less than 2% PSG without RRM relaxation, but RRM relaxation requires periodic LP-SS, which increases signaling overhead. OFDMA-based LPWUR can support RRM relaxation, but without a duty cycle, no PSG can be achieved due to high power consumption. Using a long duty cycle increases latency and cannot be used in RRC CONNECTED.

Why it matters: The combination of OOK-based and OFDMA-based LPWURs with a long duty cycle on the OFDMA-based LPWUR can result in a more efficient solution for saving power while also maintaining low latency and low signaling overhead. OOK-based LPWUR can be used for monitoring aperiodic LPWUS, while OFDMA-based LPWUR can be used for RRM relaxation. We observe a PSG from 20% to 23% to R17, providing a better power-saving solution with low latency and low signaling overhead without the need for LP-SS and a long duty cycle on LPWUS monitoring.

OOK-based LPWUR has low PSG without RRM relaxation, but RRM relaxation requires periodic LP-SS with increased overhead, while OFDMA-based LPWUR can support RRM relaxation, but a duty cycle increases latency and cannot be used in RRC CONNECTED.

RAN1 should use a combination of two different receiver approaches - one based on OOK and the other based on OFDMA. The OOK-based approach would be used to monitor a wake-up signal, while the OFDMA-based approach would use a longer duty cycle to perform synchronization and RRM measurement.

## Coverage Performance

What it is: In this section, we will clarify the following points for performance comparison in AWGN channel:

- To calibrate the theoretical performance of binary FSK with non-coherent detection (NC-BFSK), we consider the comparison with respect to Eb/No, which is the ratio of energy per bit to noise power spectral density. Eb/No is a measure of signal-to-noise ratio (SNR) in digital communication systems. The theoretical bit error rate (BER) of NC-BFSK in AWGN channel is given by: BER = 0.5 *exp (-0.5 * Eb/No).

- If we have two OOK sequences that are orthogonal to each other, meaning that their inner product is zero, then the error rate performance of detecting one of them can be matched to the theoretical performance of NC-BFSK. This is because the orthogonality implies that there is no interference between the two sequences, and the detection can be done by correlating the received signal with one of the sequences and comparing it with a threshold. The SNR of this correlation is equal to Eb/No, and the BER is the same as NC-BFSK.

- For the non-coherent detection over SSS and based on OFDM receiver, we use a detection design that utilizes local frequency domain correlation (for the usage in frequency selectivity channels). This means that we correlate the received signal with a local version of the SSS in each subcarrier, and then combine the results across subcarriers. This detection design is sub-optimal for AWGN channel because it does not exploit the full diversity of the SSS signal, which spans multiple subcarriers. Therefore, the performance of this detection design is inferior to the theoretical performance of NC-BFSK in AWGN channel.

Why it matters: Overall, we observe a good matching in the error rate performances among the non-coherent signaling schemes, given that the candidate signals for UE to detect exhibit orthogonality in AWGN channels. This means that the signals are well separated in terms of energy and do not interfere with each other. The orthogonality can be achieved by using different frequencies (as in FSK), different time slots (as in OOK), or different codes (as in SSS).

What it is: In this section, we compare the performance of different signaling schemes in TDL-C fading channel. We clarify the following points for the performance comparison:

- We use the received SNR as the metric for the performance comparison, which is defined as the ratio of the average received signal energy to the average receiver noise power within the signal bandwidth. This is because we need to obtain a certain SNR level for the MIL analysis, and the received SNR reflects the actual signal quality at the receiver.

- We assume that the detection is subject to a false alarm rate (FAR) of 1%, which means that we set a threshold on the correlation outcome of the low power receiver to reject the noise samples. For OFDM-based low power receiver, we find that the threshold design has a significant impact on the missing detection rate (MDR) performance, which is the probability of missing a valid signal sample. Therefore, we also show the MDR performance without FAR filtering for OFDM-based low power receiver to illustrate this effect.

- For OOK signaling, we use two orthogonal OOK sequences, i.e., 1010 and 0101, that modulate four chips in an OFDM symbol duration. Since we use a relatively longer chip duration than the channel mean delay spread, the performance of OOK can be superior to SSS with non-coherent detection. This is because OOK can exploit all multipath diversity by energy detection. As a result, we observe a sharper MDR slope for OOK than for SSS with non-coherent detection.

Why it matters: Overall, OOK-based signaling and SSS-based signaling can achieve comparable performance in fading channels. However, with a sufficient OOK chip duration, OOK-based signaling also shows better performance than SSS-based signaling, in terms of diversity order (MDR slope) and robustness with respect to FAR filtering design. This means that OOK-based signaling can achieve a lower MDR with a lower received SNR, and it is less sensitive to the threshold design for FAR filtering.

Table 3: Link Budget (MIL) Analysis

| Scenarios | Urban 2.6GHz |  |
| --- | --- | --- |
| Description of LP WUS | OOK Sequence | NR SSS |
| System configuration |  |  |
| Carrier frequency (GHz) | 2.60 |  |
| Pathloss model | NLOS TDL-C |  |
| Target error rate (BLER/MDR etc.) | 1% initial BLER (no retransmission) |  |
| Company reporting Assumptions for LP-WUS/WUR |  |  |
| False alarm rate (FAR) | 1% |  |
| Channel Structure | 4-chip sequences | 127-chip sequences |
| Number of information bits delivered | 1 | 1 |
| Waveform | OOK | OFDMA |
| Coding Scheme | None |  |
| Frequency Domain Allocation (MHz) | 4.32 | 3.81MHz |
| Guard band (MHz) | 0.72 | N/A |
| Time Domain Allocation (Y ms) | 0.5 |  |
| Efficiency(bit/s/Hz) | 0.000396825 |  |
| Receiver structure | zero-IF | OFDMA |
| Frequency error/drifts | 0 ppm |  |
| ADC bit-width | 4 | Ideal |
| Sampling rate (MHz) | 3.84 | 20 |
| Parameters for BB BPF/LPF | 5th-Order Butterworth with 4.32MHz | Ideal |
| Other assumptions if not listed above |  |  |
| - Non-coherent: Both receivers apply non-coherent detection - Timing: Perfect Timing - Channel: TDL-C with a mobile speed of 3km per hour and RMS delay spread of 300ns (long) |  |  |
| Transmitter |  |  |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 100000000 |  |
| (3c) bandwidth used for the evaluated channel (Hz) | 4320000 | 3810000 |
| Receiver |  |  |
| (10) Number of receiving antenna elements | 1 |  |
| (13) Receiver noise figure (dB) | 15 |  |
| (19) Required SNR (dB) | -3 | -1.5 |
| (22) Receiver sensitivity | -93.26 | -92.30 |
| (22bis) MCL | 150.67 | 149.17 |
| (23) Hardware link budget, MIL | 152.02 | 150.52 |
| Calculation of available path loss |  |  |
| (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) | 292.12 | 267.42 |

What it is: In this section, we compare the coverage performance of different signaling schemes in the TDL-C fading channel. We clarify the following points for the performance comparison:

- For the performance comparison, we assume the OOK-based low-power receiver and the OFDMA-based low-power use non-corner detection and have a receiver noise figure of 15 dB.

- We use the same information bit, and comparable time and frequency resources for the comparison. For OFDM, 3.81MHz is used to map 127 chips on 127 RE. For OOK, 4.32MHz is used to map 4 chips on 144 RE. Both schemes use 1 OFDM symbol in the time domain to transmit one information bit.

Why it matters: Overall, OOK-based signaling and SSS-based signaling can achieve comparable coverage in fading channels for a non-coherent detector given the same among of time and frequency resources, and the data rate. It means a hybrid signaling approach that combines OFDM and OOK signaling can achieve comparable coverage in fading channels while avoiding the need for a long duty cycle or broadcasting a new reference signal based on OOK.

By using a hybrid signaling approach, we can take advantage of the benefits of both OFDM and OOK-based signaling. Specifically, we can use OFDM to support RRM relaxation, which can reduce signaling overhead, and use OOK-based signaling to achieve low PSG without a long duty cycle. With a sufficient OOK chip duration, the hybrid approach can also achieve better diversity order and robustness compared to SSS-based signaling.

Overall, the hybrid signaling approach can provide a more efficient and effective solution for LPWUR in fading channels. It can achieve comparable performance to existing signaling methods while avoiding their limitations. Therefore, we recommend further investigation and testing of this approach to determine its feasibility and potential benefits.

When using non-coherent detection with the same amount of time and frequency resources and information bits, both OOK-based signalling and SSS-based signalling show comparable performance in terms of MIL and MDR. In other words, there is no significant difference in the performance of these two signalling methods when non-coherent detection is used under these conditions.

RAN1 should consider a hybrid OFDM and OOK signaling approach with non-coherent detection for comparable coverage in fading channels without long duty cycles or broadcasting new signals.

# Conclusion

In this contribution, we have the following observations and proposals.

Observation 1	Support of Alt 2 (FAR target is determined across a reference time duration of one or multiple WUS attempts/trials) may result in the target FAR values below 1% or below 0.1%, which may not be feasible if there is no CRC for further confirmation.

Proposal 1	Clarify the baseline of [0.1%, 1%] for the false-alarm rate (FAR) of LP-WUS agreed in RAN1#112 is defined by FAR per single WUS attempt/trial, otherwise, non-feasible FAR targets will be included.

Observation 2	Clock frequency errors, i.e., oscillator and RTC frequency errors in the working assumption, cannot be considered as the CFO assumption used to convert RF signal into baseband.

Proposal 2	The working assumption of clock modeling cannot be used for carrier frequency error (CFO), e.g., 200 ppm. Remove option 1 (200, 0.1), option 2 (50, 0.1), and the context for the frequency error assumption to prevent misleading.

Observation 3	In the assumption of delay spread corresponding to the TDL models, the value of 1000ns has the max delay span of 8.6us, three times larger than the CP length of 2.34us for 30kHz SCS.

Proposal 3	Change the assumption of delay spread from optional 1000ns to optional: 100ns, considering the agreed values of 300ns and 1000ns result in larger delay spans than the CP length for 30kHz SCS.

Proposal 4	Confirm Alt 2: (40000, 800ms) being the transition energy and transition time for FR1 MR ultra-deep sleep state.

Observation 4	The current relative power values for LPWUR are up to 4 and they cannot reflect the need to support OFDMA-based LPWUR.

Proposal 5	Add relative power values 20 and 40 for the power state ON to support OFDMA-based LPWUR and set ramp-up time to zero for OOK/FSK LPWUR with relative power values smaller than 20.

Observation 5	OOK-based LPWUR has low PSG without RRM relaxation, but RRM relaxation requires periodic LP-SS with increased overhead, while OFDMA-based LPWUR can support RRM relaxation, but a duty cycle increases latency and cannot be used in RRC CONNECTED.

Proposal 6	RAN1 should use a combination of two different receiver approaches - one based on OOK and the other based on OFDMA. The OOK-based approach would be used to monitor a wake-up signal, while the OFDMA-based approach would use a longer duty cycle to perform synchronization and RRM measurement.

Observation 6	When using non-coherent detection with the same amount of time and frequency resources and information bits, both OOK-based signalling and SSS-based signalling show comparable performance in terms of MIL and MDR. In other words, there is no significant difference in the performance of these two signalling methods when non-coherent detection is used under these conditions.

Proposal 7	RAN1 should consider a hybrid OFDM and OOK signaling approach with non-coherent detection for comparable coverage in fading channels without long duty cycles or broadcasting new signals.

# Reference

Alpman E, Khairi A, Dorrance R, Park M, Somayazulu VS, Foerster JR, Ravi A, Paramesh J, Pellerano S. 802.11 g/n compliant fully integrated wake-up receiver with− 72-dBm sensitivity in 14-nm FinFET CMOS. IEEE Journal of Solid-State Circuits. 2018 Apr 23;53(5):1411-22 [link]

Liu R, KT AB, Dorrance R, Dasalukunte D, Kristem V, Lopez MA, Min AW, Azizi S, Park M, Carlton BR. An 802.11 ba-based wake-up radio receiver with Wi-Fi transceiver integration. IEEE Journal of Solid-State Circuits. 2019 Dec 24;55(5):1151-64. [link]

# Appendix

## Agreements in RAN1#112

Conclusion:

The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group.

Agreement

The following characteristics for target use cases are considered in the study item:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - FFS: latency

  - primary for small form devices

  - power-sensitive

  - static, nomadic or limited mobility

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - FFS: latency

  - primary for small form devices,

  - power-sensitive

  - low/medium speed, FFS: high speed

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - FFS: latency

  - devices form is various and not restricted

  - power-sensitive

  - low/medium speed, FFS: high speed

Note: other use cases/characteristics are not precluded if any.

Agreement

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

- Alt 1: (15000, 400ms)

- Alt 2: ([40000], [800ms])

Company to report which alternative they use for which use cases.

Agreement

For coverage evaluation, the following is used,

| Number of RX chains at the UE’s MR antenna elements for UE | Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS. |
| --- | --- |
| Number of RX chains antenna elements for LP-WUR | 1 Rx Note: agreed in RAN1#110bis |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Reference data rates for MR eMBB | Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Reference PDCCH configuration | SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,. |
| Pathloss model (select from LoS or NLoS) | Urban: NloS Rural: NloS and LoS |
| Bandwidth | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| Delay spread | Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns |
| UE velocity | Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |

Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis.

Agreement

For link-level simulation of LP-WUS, the following table is used as starting point,

- FFS for other assumptions if any

- Note: The assumptions are not intended to limit the scope of the study or the design.

Table XX. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used |
| Channel structure | - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence). |
| SCS of OFDM generator for NR signal | 30kHz/15KHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials |
| Code scheme | Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….) |
| gNB Channel BW | 20MHz, FFS other values |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| Filter | X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional |
| Sampling Rate | - Companies to report. |
| ADC bit width | 1-bit, 4-bit, 8-bit, ideal and other options are not precluded |
| Channel Model | See link coverage assumption table (will copy and paste here) |
| Impairment modelling | - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report |

Working Assumption

- For evaluation of LP-WUR frequency and time errors, the following is used,

| Parameter | Value |
| --- | --- |
| Oscillator max frequency error [ppm], Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |
| RTC max frequency error [ppm] | 20 |

- Company to report how to use the clocks for LR on/off states

  - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state.

  - For Option 3/4,

    - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,

      - e.g., option 3/4 is not applicable

        - when MR is in ‘ultra-deep sleep state’ with [0.015] power units and LR is in off state or,

        - when LR monitoring power less than [TBD] power unit,

    - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

  - If MR is in other state than ‘ultra-deep sleep state’, the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

    - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1

    - When frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

    - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

    - FFS: Frequency displacement (Fd), defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).

    - Model 2: random frequency drifting, FFS details

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

    - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (linear region)

    - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

    - The transition between transient and linear region (from synchronization or calibration point/time) occurs at time [Ts= (Fe-Fr)/( F’)]

    - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

    - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous synchronization/calibration (Tr); Te= ΔT+ Tr

    - Model 2: random time drifting, FFS details

- FFS: Phase noise model

## PSG Settings

Table 4: OOK-based LPWUR, IDRX, No RRM relax, No duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 0.1 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 2580.35 |
|  | Power saving gain = -3% |
|  | baseline schemes = 2502.35 |

Table 5: OOK-based LPWUR, IDRX, RRM relax, No duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 0.1 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep Sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 2ms |
|  | Relax 4 times. Perform RRM every 4 I-DRX cycles. |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 1356.8 |
|  | Power saving gain = 28% |
|  | baseline schemes =1883.65 |

Table 6: OOK-based LPWUR, eDRX, No RRM relax, No duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| eDRX configuration | e-DRX cycle = 61.44s |
|  | i-DRX cycle in PTW [s] = 5.12 s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 0.1 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 110 ms Energy consumption = 4225.25 |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 36379.99 |
|  | Power saving gain = 37% |
|  | baseline schemes = 57544.84 |

Table 7: OOK-based LPWUR, IDRX, No RRM relax, Duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Duty Cycle = 0.3125% (monitor 0.5ms per 160ms) |
|  | ON state power[unit] = 0.1 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 2453.95 |
|  | Power saving gain = 2% |
|  | baseline schemes = 2502.35 |

Table 8: OOK-based LPWUR, IDRX, RRM relax, Duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Duty Cycle = 0.3125% (monitor 0.5ms per 160ms) |
|  | ON state power[unit] = 0.1 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep Sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 2ms |
|  | Relax 4 times. Perform RRM every 4 I-DRX cycles. |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 4952.12 |
|  | Power saving gain = 35% |
|  | baseline schemes =7534.6 |

Table 9: OOK-based LPWUR, eDRX, No RRM relax, Duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| eDRX configuration | e-DRX cycle = 61.44s |
|  | i-DRX cycle in PTW [s] = 5.12 s |
| LP-WUR (LP) | Duty Cycle = 0.3125% (monitor 0.5ms per 160ms) |
|  | ON state power[unit] = 0.1 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 110 ms Energy consumption = 4225.25 |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 30389.59 |
|  | Power saving gain = 47% |
|  | baseline schemes = 57544.84 |

Table 10: OFDMA-based LPWUR, IDRX, No RRM relax, No duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 40 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 28052.35 |
|  | Power saving gain = -1021% |
|  | baseline schemes = 2502.35 |

Table 11: OFDMA-based LPWUR, IDRX, RRM relax, No duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 40 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep Sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 2ms |
|  | Relax 4 times. Perform RRM every 4 I-DRX cycles. |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 107,315.2‬ |
|  | Power saving gain = -1324% |
|  | baseline schemes =7526.4 |

Table 12: OFDMA-based LPWUR, eDRX, No RRM relax, No duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| eDRX configuration | e-DRX cycle = 61.44s |
|  | i-DRX cycle in PTW [s] = 5.12 s |
| LP-WUR (LP) | Continuously monitoring |
|  | ON state power[unit] = 40 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 110 ms Energy consumption = 4225.25 |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 1259035.99 |
|  | Power saving gain = -2088% |
|  | baseline schemes = 57544.84 |

Table 13: OFDMA-based LPWUR, IDRX, No RRM relax, Duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Duty Cycle = 0.3125% (monitor 0.5ms per 160ms) |
|  | ON state power[unit] = 40 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 2502.35 |
|  | Power saving gain = -4% |
|  | baseline schemes = 2502.35 |

Table 14: OFDMA-based LPWUR, IDRX, RRM relax, Duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| DRX configuration | I-DRX cycle = 1.28s |
| LP-WUR (LP) | Duty Cycle = 0.3125% (monitor 0.5ms per 160ms) |
|  | ON state power[unit] = 40 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 0 ms, Energy consumption = 0 (Deep sleep) |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 2ms |
|  | Relax 4 times. Perform RRM every 4 I-DRX cycles. |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 5,529.6‬ |
|  | Power saving gain = 26% |
|  | baseline schemes =7526.4 |

Table 15: OFDMA-based LPWUR, eDRX, No RRM relax, Duty cycle [back to Table 1]

| Items | Descriptions |
| --- | --- |
| Case identifier | Case A7: provide power and latency for different RRM assumptions |
| eDRX configuration | e-DRX cycle = 61.44s |
|  | i-DRX cycle in PTW [s] = 5.12 s |
| LP-WUR (LP) | Duty Cycle = 0.3125% (monitor 0.5ms per 160ms) |
|  | ON state power[unit] = 40 |
|  | OFF state power[unit] = 0.001 |
| Main Radio (MR) | Ramp-up time for ultra-deep sleep[ms] = 400ms |
|  | Ultra-deep sleep relative power [unit] = 0.015 |
|  | Ramp-up/down energy [unit*ms] = 15000 |
|  | Sync/re-sync = 110 ms Energy consumption = 4225.25 |
|  | Power model = TR 38.875 (IoT and wearables) |
| Other configurations | PEI used = Yes |
|  | Number of SSB before PO / PEI = 6 |
| Traffic | Per group/UE = group |
|  | Mean arrival time = 128s |
|  | R_E = 1% |
|  | I-DRX cycle = 1280ms |
|  | Number of UE = 10 |
| Measurement | RRM included = Yes |
|  | Measured by MR |
|  | Measurement Period = 8ms |
| Miscellaneous | LPWUS FAR = 1% |
|  | total time used for evaluation = 1.28s |
| Power consumption | Relative Power Consumption = 37915.99 |
|  | Power saving gain = 34% |
|  | baseline schemes = 57544.84 |