3GPP TSG RAN WG1 #112-bis	R1-2303333

e-Meeting, April 17th – April 26th, 2023

Source: 	MediaTek Inc.

Title: 	Low power WUS receiver architectures

Agenda item:	9.11.2

Document for:	Discussion

# Introduction

In RAN1#112, agreements for OFDMA-based LPWUR architectures were endorsed. Companies were encouraged to provide details on power consumption reduction compared to the Main Radio (MR), including components such as LNA/amplifier, Oscillator/PLL, ADC, and BB processing complexity. The agreements are listed in Appendix.

This contribution provides views on OOK-based LPWUR and OFDMA-based LPWUR. It also explains how and why to combine these two receivers, including power consumption models and noise figure values.

# OOK-based LPWUR

Agreement in RAN1#110-bis-e Study the homodyne/zero-IF architecture with baseband envelope detection based on at least the following diagram for LP-WUR. - Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram. - 1-bit or multi-bit ADC is applied.

What it is: A LPWUR architecture supports OOK waveform demodulation and synchronization. The receiver does not require I/Q branches for digital BB processing and multi-bit ADC is not necessary for OOK demodulation.

What the problem is: Phase noises resulted from a multi-path channel and frequency errors degrade performance of using In-phase signal only to demode OOK waveform. The receiver losses part of transmitted signal when the phase noise turns the in-phase signal to the quadrature component. Additionally, the performance of baseband envelope detection degrades when the zero/DC level cannot be estimated properly. The transmitted signal below the estimated zero/DC level is lost and cannot be enveloped for OOK demodulation.

Why it matters: For better coverage and interference suppression, the homodyne/zero-IF architecture should consider I/Q branches for digital BB processing. Additionally, baseband envelope detection should be done in digital domain after auto gain control (AGC) to find the correct zero/DC level.

Using I/Q branches rather than a single in-phase branch in the homodyne/zero-IF architecture with baseband envelope detection can improve the system's resilience to multi-path channels and frequency errors.

RAN1 should consider adding I/Q branches in the homodyne/zero-IF architecture with baseband envelope detection to improve the system's robustness and handle multi-path channels and frequency errors caused by low accuracy LO and not using PLL.

The power consumption of adding I/Q branches to a zero-IF architecture can be comparable to that of FSK receivers, at around 0.05, 0.1 and 0.5, respectively.

# OFDMA-based LPWUR

Agreement in RAN1#112 For OFDMA-based signals/channels, study the receiver architectures based on the following diagrams: - Proponent companies should at least provide details on power consumption reduction compared to the MR regarding the RF and digital BB processing. - The potential power reduction compared to the main radio may come from e.g.: - Oscillator/PLL with relaxed performance requirements

What it is: For OFDMA-based LPWUR, proponents should provide details on power consumption reduction by lowering performance requirements of components such as the LNA/amplifier, oscillator/PLL, ADC, and reducing the complexity of the BB processing.

What the problem is: The LPWUR must be able to detect the existing NR signal while relaxing the performance requirements.

Why it matters: If the performance requirements cannot be relaxed, there will be no power reduction compared to the MR, making the power consumption too large to be a LPWUR.

Note: Relaxing the frequency error of an oscillator/PLL from 10ppm to 200ppm increases the number of frequencies that need to be searched, resulting in increased complexity and power consumption. However, the resulting residual frequency error can be less than 1ppm, which requires less energy to maintain tracking. Our evaluation shows it is feasible to search a PSS signal using a 3.84MHz sampling rate with 200ppm frequency error.

It is feasible to search a PSS signal with a 200ppm frequency error and 3.84MHz sampling rate, although it requires increased complexity.

For OFDMA-based LPWUR, RAN1 should consider lowering the performance requirements for oscillator/PLL from 10ppm to 50 or 200ppm to reduce the RF power consumption.

Figure 1: Searching for a PSS signal using a 200ppm CFO and 3.84MHz sampling rate

# Combination of OOK-based and OFDMA-based LPWUR

Figure 2: Combining OOK-based and OFDMA-based LPWUR with the independent local oscillators.

What it is: The combination of OOK-based LPWUR and OFDMA-based LPWUR using a duty cycle, with OOK-based LPWUR based on the ZERO-IF architecture and OFDMA-based LPWUR operated with a duty cycle to monitor PSS/SSS and provide functions such as timing and frequency tracking and serving cell RRM measurement.

Where the problem is: Independent local oscillators provide improved detection performance on the OFDM signal but come with increased hardware complexity. Additionally, various combinations are possible, and it requires effort to find the optimal combination.

Why it matters: Combining the two technologies can result in better spectrum efficiency and energy saving, achieving 20% PSG with RRM relaxation of 4 times and 23% of PSG with eDRX of 61.44s.

Note: It is possible to further simplify the combined receiver by relaxing the frequency error of an oscillator/PLL from 10ppm to 200ppm, supporting I/Q branches for OOK-based LPWRU and sharing I/Q branches and the 200ppm LO.

RAN1 should consider the LPWUR architecture which supports periodic PSS and SSS detection with a duty cycle operation, as well as aperiodic OOK payload demodulation.

Figure 3: Combining OOK-based and OFDMA-based LPWUR with the shared local oscillators.

# Conclusion

In this contribution, we have the following observations and proposals.

Observation 1	Using I/Q branches rather than a single in-phase branch in the homodyne/zero-IF architecture with baseband envelope detection can improve the system's resilience to multi-path channels and frequency errors.

Proposal 1	RAN1 should consider adding I/Q branches in the homodyne/zero-IF architecture with baseband envelope detection to improve the system's robustness and handle multi-path channels and frequency errors caused by low accuracy LO and not using PLL.

Proposal 2	The power consumption of adding I/Q branches to a zero-IF architecture can be comparable to that of FSK receivers, at around 0.05, 0.1 and 0.5, respectively.

Observation 2	It is feasible to search a PSS signal with a 200ppm frequency error and 3.84MHz sampling rate, although it requires increased complexity.

Proposal 3	For OFDMA-based LPWUR, RAN1 should consider lowering the performance requirements for oscillator/PLL from 10ppm to 50 or 200ppm to reduce the RF power consumption.

Proposal 4	RAN1 should consider the LPWUR architecture which supports periodic PSS and SSS detection with a duty cycle operation, as well as aperiodic OOK payload demodulation.

# Reference

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

# Appendix

## RAN1#112 agreements

Agreement

Study the parallel receiver architectures (as examples that can be captured in the TR) for FSK based on the following diagrams:

- Parallel homodyne architecture receiver

  - The observations made for homodyne/zero-IF architecture with baseband envelope detection in RAN1#110b/111 are also applicable here.

- Parallel heterodyne architecture receiver

  - The observations made for heterodyne architecture with IF envelope detection in RAN1#110b/111 are also applicable here.

- Note: Other architectures are not precluded.

- The OOK receiver architectures agreed for study in RAN1#110bis-e are also examples that can be captured in the TR

Agreement

Study the receiver architectures (as examples that can be captured in the TR) for FSK with frequency to amplitude conversion based on the following diagrams:

- Homodyne architecture receiver with frequency to amplitude conversion

  - I/Q branches are required for frequency to amplitude conversion in digital BB.

- Heterodyne architecture receiver with frequency to amplitude conversion

  - Companies provide the exact type FFS what type(s) of frequency to amplitude conversion being is studied.

- Note: Other architectures are not precluded.

Agreement in RAN1#112

For OFDMA-based signals/channels, study the receiver architectures based on the following diagrams:

- I/Q branches are required for digital BB processing.

- Digital BB processing may or may not include FFT (companies to provide details on how).

- For sequence-based OFDM signals/channels, digital BB processing includes sequence correlation in either time domain (without FFT) or frequency domain (after FFT).

- Proponent companies should at least provide details on power consumption reduction compared to the MR regarding the RF and digital BB processing.

  - Companies are encouraged to provide the break-down for the components.

  - The potential power reduction compared to the main radio may come from e.g.:

    - Lower performance LNA/amplifier

    - Oscillator/PLL with relaxed performance requirements

    - ADC with lower sampling rate and smaller bit-width

    - Reduced BB processing complexity compared to the MR

  - Companies are encouraged to provide the performance analysis corresponding to the considered power consumption considering the impact of e.g. phase noise, I/Q mismatch.

  - Companies to report whether the LP WUR is assumed to share components with MR. In case of component sharing, the potential impact on the MR ultra-deep sleep state should be considered.

  - Companies to report the possible number of information bits

- In addition, companies should consider the power consumption in the OFF state and the transition energy.

Agreement

For the study on LP WUR architecture, power consumption relative to the deep sleep state of the MR is provided.

- Deep sleep state of non-RedCap UE should be assumed