3GPP TSG RAN WG1 #112-bis	R1-230xxxx

e-Meeting, April 17th – April 26th, 2023

Source: 	MediaTek Inc.

Title: 	L1 signal design and procedure for low power WUS

Agenda item:	9.13.3

Document for:	Discussion

# Introduction

In RAN1#112, different LPWUS generation and LPWUR monitoring behaviors for MC-ASK, MC-FSK, and CP-OFDMA waveforms have been defined. The details of the agreements are listed in Appendix 6.1.

This contribution provides our view on how CP-OFDMA and MC-ASK waveforms can be used simultaneously to support duty cycle, timing synchronization, and potentially serving cell RRM relaxation.

# L1 Design

## Combining OFDMA and OOK signal

Figure 1: The proposed LPWUS format including NR PSS, OOK Preamble (optional), payload, and CRC.

What it is: A gNB has to broadcast PSS and SSS for the MR, and broadcast an OOK preamble or LPSS for LPWUR. It will be a waste if the LPWUR can monitor PSS directly for timing synchronization.

Where the problem is: It is not clear whether the OOK preamble is needed if LPWUR can monitor NR PSS directly, due to the limitations of LPWUR such as a 200 ppm frequency error in its local oscillator (LO), low sampling rate of 3.84Mhz, and low bit resolution of ADC and no I/Q branches in a mixer.

Why it matters: It is important to understand this technique, as it can provide a better balance between signaling overhead and energy saving.

By monitoring PSS and SSS, LPWUR can prevent broadcasting periodic LPSS and NR SSB, thus achieving timing synchronization, maintaining a duty cycle, and monitoring serving cell RRM.

RAN1 should study how LPWUR monitors periodic NR PSS and SSS for a duty cycle and serving cell relaxation, while also monitoring OOK-based LPWUS to reduce hardware complexity and power consumption. This would ensure the best balance between signaling overhead and energy saving.

## Aperiodic OOK wake-up indication

Figure 2: OOK-based LPWUR with the timing synchronization provided by another receiver

What it is: An OOK-based receiver monitors OOK LPWUS with timing synchronization provided by an independent receiver. This receiver can monitor aperiodic LPWUS given pre-defined monitoring occasions, with the configurations provided by the main radio and timing provided by the independent receiver. An envelop detector is used to decode the LPWUS, and only the low-bit resolution of ADC is needed due to the two levels of On-Off Keying. Auto Gain Control (AGC) is added to prevent multi-path fading channel and phase noise, with the DC level estimated by the independent receiver.

Where the problem is: It is unclear how the timing estimate can be provided independently from another LPWUR and how the power consumption will be required if both receivers are supported simultaneously. Additionally, when the OOK LPWUS passes through a fading channel, LPWUR may not find the proper DC level and the proper receive signal power level, which could cause phase noise and truncation of the transmitted signal. Auto Gain Control (AGC) is used to solve this issue by estimating the DC level with the average of the received signal power from the independent receiver that monitors PSS and SSS.

Why it matters: Without the timing synchronization provided by the independent receiver, the OOK LPWUR would not be able to decode the OOK-based LPWUS. The Auto Gain Control is also important to ensure that the signal passes through the fading channel and phase noise can be recovered by the envelope detector. Without the DC level estimation, most of the transmitted signal will be truncated with a wrong ADC range and the envelope detector.

The OOK LPWUR can maintain its low complexity by using an independent receiver to provide timing synchronization and the DC level estimation, to decode the OOK-based LPWUS.

An OOK-based LPWUR should have an independent receiver to provide timing synchronization and the DC level estimation, as well as an AGC module to estimate the ADC range for improved signal reception.

## Periodic OFDMA signal for synchronization and RRM offloading

Figure 3: OFDMA-based LPWUR with the PSS candidates provided by MR

Figure 4: How MR generates PSS candidates for OFDMA-based LPWUR

What it is: OFDMA-based LPWUR is a receiver that provides timing information and the DC level estimation (or RSRP) via the existing OFDM-based signal such as PSS and SSS broadcasted by the serving cell. It considers no I/Q branches, a 200 ppm LO, and the ADC with 3.84MHz and 4-bit resolution.

Where the problem is: Lowering the LO accuracy from 10ppm to 200ppm increases the search frequency range, and leads to a high power consumption and a risk of false alarm.

Why it matters: Timing synchronization and RSRP measurement are important for LPWURs, and a false alarm can lead to incorrect measurements, incorrect operations, and transition energy waste of the main radio (MR)

Note: The main radio (MR) can generate the PSS candidates with fewer hypotheses to reduce the computational complexity. Additionally, the MR can stay awake before LPWUR has built up its synchronization to prevent any false alarm during the initial time and frequency sync. Some initial evaluations are below and settings are in Appendix 6.2.

Lowering the LO accuracy from 10ppm to 200ppm results in higher power consumption and a risk of false alarms.

To reduce the power consumption and the risk of false alarms, the main radio (MR) should generate the PSS candidates with fewer hypotheses and stay awake before LPWUR has built up its synchronization.

Figure 5: Auto-correlation results of NR PSS from the OFDMA-based LPWUR with no I/Q and 200ppm CFO.

Figure 6: Initial evaluation results for the combination of OOK-based and OFDM-based LPWUR

# Procedure

## LPWUR Initiation

Figure 7: Continuous monitoring, corresponds to LP-WUR is ON all the time

Figure 8: How to prevent a false alarm risk due to continuous monitoring for aperiodic LPWUS

What it is: For continuous monitoring, LPWUR has no pre-defined search space or monitoring occasions. In this case, it must detect LPWUS on every single received sample.

What the problem is: With 1% FAR, LPWUR will wake up the MR with a chance of 98.2% after 400 attempts of continuous monitoring, which may lead to wrongly waking the MR up and wasting transition energy.

Why it matters: Continuous monitoring increases LPWUR’s power consumption and wrongly waking MR up will waste transition power for turning hardware components on and off.

- After LPWUR continues monitoring over 104.2 μs at a sampling rate of 3.84 MHz, LPWUR will most likely wake MR up due to a false alarm with a 98.2% FAR.

To prevent the risk of a high FAR, UE should not turn MR off before LPWUR receives the first LPWUS or synchronizes with the network successfully, and LPWUR should not expect to receive another LPWUS after a certain time.

# Conclusion

In this contribution, we have the following observations and proposals.

Observation 1	By monitoring PSS and SSS, LPWUR can prevent broadcasting periodic LPSS and NR SSB, thus achieving timing synchronization, maintaining a duty cycle, and monitoring serving cell RRM.

Proposal 1	RAN1 should take advantage of LPWUR to monitor periodic NR PSS and SSS for a duty cycle and serving cell relaxation, while also monitoring OOK-based LPWUS to reduce hardware complexity and power consumption. This would ensure the best balance between signaling overhead and energy saving.

Observation 2	The OOK LPWUR can maintain its low complexity by using an independent receiver to provide timing synchronization and the DC level estimation, to decode the OOK-based LPWUS.

Proposal 2	An OOK-based LPWUR should have an independent receiver to provide timing synchronization and the DC level estimation, as well as an AGC module to estimate the ADC range for improved signal reception.

Observation 3	Lowering the LO accuracy from 10ppm to 200ppm results in higher power consumption and a risk of false alarms.

Proposal 3	To reduce the power consumption and the risk of false alarms, the main radio (MR) should generate the PSS candidates with fewer hypotheses and stay awake before LPWUR has built up its synchronization.

Observation 1	After LPWUR continues monitoring over 104.2 μs at a sampling rate of 3.84 MHz, LPWUR will most likely wake MR up due to a false alarm with a 98.2% FAR.

Proposal 4	To prevent the risk of a high FAR, UE should not turn MR off before LPWUR receives the first LPWUS or synchronizes with the network successfully, and LPWUR should not expect to receive another LPWUS after a certain time.

# Reference

RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

# Appendix

## Agreements

Agreement (modified as shown below in red during Thursday session)

For MC-ASK waveform generation, where K is size of IFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further

- Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are

  - OOK=1 means all SCs are modulated

  - OOK=0 means all SCs are zero power (from base-band point of view)

- Option OOK-2: Parallel M-bit OOK in frequency domain,

  - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around

  - OOK=1 means all SCs in segment are modulated

  - OOK=0 means all SCs in segment are zero power (from base-band point of view)

  - FFS architecture.

- Option OOK-3: Multi-tone single-bit OOK

  - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around

  - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view)

  - OOK=0 means all SCs in all segments are zero power (from base-band point of view)

  - FFS architecture

- Option OOK-4: Transform M-bit OOK in time domain

  - N SCs of OOK-1 are generated by a transformation (DFT/Least square)

    - N’ samples are generated from M-bits

    - signal modification may or may NOT be used

    - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’

  - N’ can be the same as K

- FFS modulated SCs are e.g. QAM symbols, sequences or other signals

  - Companies to report their assumptions

- potential guard-band SCs are zero power (from base-band point of view)

- [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted]

- Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

Agreement

Study synchronization signal used by LP-WUR, if needed, based on

- Option 1: aperiodic signal transmitted as part of LP-WUS

  - FFS: Whether the signal can additionally be transmitted separately from LP-WUS

- Option 2: periodic signal transmitted separately from LP-WUS

- Option 3: Option1 + Option2

Agreement

For M-bit MC-FSK generation study further the following options

- Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around.

  - segment comprises one sub-carrier or multiple contiguous SCs

  - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view)

- Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around.

  - segment comprises one sub-carrier or multiple contiguous SCs

  - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view)

- M >0

- N >1

- Study how to generate segment in time domain, e.g. OOK-1 or OOK-4

- Other options are not precluded.

Agreement

For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study

- FDM/TDM multiplexing with other NR transmissions

- link performance

- impact to legacy UEs

- impact on gNB

Agreement

Study further pros and cons of the following monitoring behaviors of LP-WUR

- Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states

- Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

Agreement

Study potential measurement metric used for RRM measurements performed by LP-WUR.

- examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal

- companies to report assumption of signal used for measurements

Agreement

- When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least

  - raw information bit-size

  - [time/frequency resources (including any guard bands), if applicable]

  - [total energy of LP-WUS across the time/frequency resources]

  - FFS: false alarm probability/rate

  - FFS: misdetection probability/rate

are kept [comparable or fixed]. Study at least

  - impact of timing error

  - impact of frequency error

  - impact of phase noise and I/Q imbalance, if applicable

  - impact of ADC resolution and sampling rate

  - impact of interference

  - impact of delay spread

  - impact of doppler spread

- Companies to report

  - how they modelled SINR

  - time/frequency resources (including any guard bands) for the scheme

  - false alarm probability/rate and misdetection probability/rate

  - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms

- When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

## LLS settings

| Items | Values |
| --- | --- |
| Carrier frequency (GHz) | 2.6GHz |
| BW (MHz) | 4.32MHz |
| Guard band (MHz) | 0.72 MHz |
| Frequency error | 200 ppm |
| Adjacent subcarrier interference | No PDSCH mapped on resources other than that for WUS and guard band (the same EPRE) |
| ADC bits | 4 bits |
| RX sampling rate | 3.84MHz |
| Channel structure | data: 16bits (32 chips), CRC: 8 bits (16 chips) |
| Channel coding |  |
| Carrier Frequency | 2.6GHz |
| Waveform | OOK for data OFDM for timing synchronization |
| Channel structure | Option 3: Payload+CRC |
| SCS of OFDM generator for NR signal | 30kHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) |
| WUS duration | Number of OFDM symbols: 8, 16, 32,64 symbols |
| MDR/FAR assumption | The miss-detection rate (MDR) of LP-WUS 1%, |
| Code scheme | 1/2 Manchester code |
| gNB Channel BW | 20MHz |
| LP-WUS BW | 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS |
| Filter | 5-th Order filter (e.g. Butterworth, Chebyshev, …) with 4.32 MHz bandwidth, |
| Adjacent subcarrier interference | No PDSCH mapped on resources other than that for WUS and guard band; |
| Sampling Rate | 3.84Mhz |
| ADC bit width | 4-bit |
| Channel Model | AWGN |