3GPP TSG RAN WG1 #112bis-e		                                                        R1-2303335

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 9.2.1

Source: MediaTek Inc.

Title: Discussion on general aspects of AI/ML LCM

Document for: Discussion & Decision

# Introduction

In RAN1#110e meeting, RAN1 agreed to study the following aspects for life cycle management (LCM).

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - UE capability and [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability

In RAN1#110bis-e meeting, RAN1 clarified the boundaries between different collaboration levels, i.e., level x/y and level y/z. Furthermore, RAN1 made much progress on the different components of LCM, especially for model selection, activation, deactivation, switching, and fallback, model monitoring. RAN1 agreed to study various approaches for achieving good performance across different scenarios/configurations/sites and to consider additional aspects for the initial list of common KPIs for evaluating performance benefits of AI/ML.

In RAN1#111 meeting, RAN1 decided to study both functionality-based and model-ID-based LCM. Furthermore, RAN1 made the working assumption for the definitions of ‘proprietary model’ and ‘open-format model’, both of which are considered as two separate model format categories for RAN1 discussion. RAN1 also made the working assumption for the definitions of ‘model identification’ and ‘functionality identification’ as well as ‘model update’ and ‘model parameter update’.

In RAN1#112 meeting, RAN1 categorized model delivery/transfer cases considering training location, model storage location, and model format and figured out 6 cases. Some progresses were made on model identification and functionality identification.

In this contribution, we share our views on the functional framework and different aspects of LCM.

# Functional Framework

The framework defined for RAN intelligence includes the functions of data collection, model training, model inference, and Actor. It can be considered as a starting point for Rel-18 AI/ML over air interface. The function of data collection is the most fundamental one. In RAN1#110b-e Meeting, RAN1 drew the conclusion that data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. In our understanding, data collection should provide input data to model training, model inference and model monitoring functions.

The function of model training performs AI/ML model training, validation and testing. If model training and model inference is performed at the same entity, model transfer/delivery is required. RAN1 is discussing different model transfer/delivery cases considering the possibilities that model storage and model training may or may not be co-located. But we don’t think model storage needs to be considered as a function and it can be considered as one step of model training. It is expected that model delivery from the model training entity to the model storage entity will not have specification impact.

Besides the functions of data collection, model training and model inference, one additional function needs to be considered is model monitoring, which is used to monitoring the performance of the AI/ML model and triggers the corresponding control of the AI/ML model including model activation/deactivation, fallback to non-AI operation, or even triggers model retraining and update. In the functional framework for RAN intelligence, model monitoring is also mentioned, but is not identified as one essential functional block. The main reason is that model monitoring in RAN intelligence is purely network implementation specific.

However, in Rel-18 AI/ML over air interface, the model monitoring may require signalings and procedures to collect data and provide the performance feedback to the peer entity. The outcome of the model monitoring is used for model control.  The performance feedbacks can be intermediate KPIs that are used to evaluate the AI/ML model performance, and they can also be the system-level KPIs that are used to evaluate the overall system performance. Considering the potential specification impacts of modal monitoring, it should be identified as one functional block in the framework.

Model Inference

Model Training

Model Monitoring

Model delivery/ transfer

Model Performance Feedback

Output

Training data

Inference data

Data collection

Monitoring data

Model control

Figure 1 Example of Functional Framework for AI/ML over air interface

Just as illustrated in Figure 1, the AI/ML functional framework for air interface includes the functions of data collection, model training, model inference and model monitoring. Besides the functions in the framework, the interactions and processes between those functions should be considered. The major steps regarding these data flows are mentioned below:

- Training data needs to be sent from data collection to model training.

- Similarly, inference data (different from training data) needs to be sent from data collection to model inference.

- Similarly, monitoring data (different from training data, inference data) needs to be sent from data collection to model monitoring.

- Model transfer/delivery need to be propagated from model training to model inference.

- Model performance feedback is sent from model monitoring to model training for model update.

- Outputs of the model needs to be sent from model inference to model monitoring to monitor the model performances.

- Model monitoring sends the model control decision to model inference.

Proposal 1: The AI/ML functional framework for air interface includes the functions of data collection, model training, model inference and model monitoring. Take Figure 1 as starting point for functional framework discussion.

# LCM

## Common and Use Case Specific Aspects

In previous meetings, we observed that some components of LCM were discussed in both agenda items of the general aspects and use case specific aspects. Therefore, some discussions may be duplicated and achieved overlapped agreements. It will help the discussion if we can split the LCM components to general aspects and use case specific aspects in a clearer way.

Although one-sided and two-sided model are defined as two types of inference operations, the ways for data collection, model training, model deployment, model monitoring, model update and model transfer are distinctively different between one-sided and two-sided model.

For model inference, the AI/ML operation for one-sided and two-sided model is totally different. For two-sided mode, inference is performed jointly across the UE and the network, i.e., the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

For CSI compression with two-sided model, three types of training collaborations were identified and the procedures for different training approaches were discussed and agreed for CSI compression enhancement.

For one-sided model, modal training is only performed at one side, e.g.  UE side or network side. For example, for the sub use case BM-Case1 and BM-Case2, two alternatives are agreed, i.e., Alt.1: AI/ML model training at NW side; Alt.2: AI/ML model training at UE side. It is observed that the training process for one-sided and two-sided model are different and model training for two-sided model is more complicated than one-sided model.

For data collection, the general aspects need to be considered commonly for different use cases. The general aspects include the purposes of data collection, the requirements of data collection as well as the possible ways to perform data collection. However, the requirements and detailed procedure of data collection may be different for different use cases. Taking data collection for training for example, if model is trained at one side, e.g., Type1 training for the two-sided model or training for one-sided model, it is very likely that the data is collected and stored at the side where training is performed. If model is trained at both sides, e.g., Type 2 and Type 3 training for the two-sided model, the dataset may be shared or separately owned by both sides. How to share the dataset or separately collect the data for two-sided model should be further studied.

The mechanism of model monitoring both one-sided and two-sided model may also be different. The purpose of model monitoring is to monitor the inference performance of the AI/ML model, which may trigger model update, i.e., re-train or fine tune an AI/ML model to improve the model inference performance. The input of the model monitoring function is the output of the inference function. The model monitoring function should try to derive the intermediate KPIs or the system performance to evaluate the inference performance. The output of the model monitoring function is the decision on whether to retain or fine-tune the AI/ML model. The decision should be passed to the training function. For one-sided model, if model training and model inference are located at the same entity, model monitoring at least for intermediate KPI is performed by the same entity. For two-sided model, where to locate the monitoring function needs to be studied further with following possibilities, i.e., model monitoring at UE side, network side or both sides.

Considering the differences between one-sided and two-sided model in terms of data collection, model training, model monitoring and model inference, it is straightforward to study LCM for one-sided model and two-sided model separately for different use cases.  Then we can discuss later what can be abstracted from the use case specific conclusions as general aspects for LCM.

Proposal 2: The mechanisms for data collection, model training, model monitoring and model inference are use case specific and studied for each use case.

Even if LCM for one-sided and two-sided model are different in the aspects of data collection, model training, model inference and model monitoring, some other aspects can be common for one-sided and two-sided model.

For Type 1 training in two-sided model, model transfer is required. However, model transfer for one-sided model may not be necessarily if model training and model inference are located at the same entity. For example, for the sub use case BM-Case1 and BM-Case2, it’s possible that AI/ML model training and inference are performed at the same entity, i.e., at the UE side or at the network side. In this case, model transfer is not required. Even though the need of model transfer is use case specific, model transfer can be performed in a common way with common format. It is also desired that one common mechanism for model management is desired for different use cases with either one-sided or two-sided model.

Model/functionality Identification is used to align the common understanding of a model/functionality between UE and network. The procedure of model/functionality identification should be common for different use cases, even though the detailed information to be aligned may be use case specific. Model selection, model switching, model activation/deactivation, fallback through network control can be common for different use cases, even though the detailed information/triggering rely on which to make the control decision should be considered for each use case.

Proposal 3: The mechanisms for model transfer, model configuration, model selection, model switching, model activation/deactivation, fallback and UE capability reporting can be common for different use cases and be studied in the general aspect.

In RAN1#111 and RAN1#112 meeting, RAN1 made the following agreements for functionality-based and model-ID-based LCM.

Agreement (RAN1#111) For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate functionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs. Agreement (RAN1#112) For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

Although RAN1 agreed to study both functionality-based and model-ID-based LCM procedures, we don’t think we are intended to design two distinct mechanisms for each component in LCM procedures. Now the boundary between Functionality and model is quite vague. For functionality-based LCM we may also need functionality ID. The only difference between functionality-based LCM and model-ID-based LCM is whether each model and the corresponding model ID is visible to network. Even if UE may have multiple AI/ML models for the functionality, the model operation at the UE side for the functionality is transparent to network side for functionality-based LCM. Therefore, the difference between functionality-based and model-ID-based LCM is whether the procedures are anchored on the functionalities or models, and whether model ID information is required and aligned between UE and network.  It is desired that model-ID-based and functionality-based LCM share as much commonality as possible in design principle, procedure and mechanism.

Proposal 4: RAN1 doesn’t intend to design two distinct mechanisms for functionality-based and model-ID-based LCM. Functionality-based and model-ID-based LCM procedures should share as much commonality as possible.

## Data Collection

In RAN1#110b-e Meeting, RAN1 drew the conclusion that data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FL also suggested the aspects to be studied for data collection. Enhanced RS design or configuration, enhanced UE measurement and report, and assistance data to UEs and networks should be studied in use case specific agenda items. The general aspect agenda can focus on entities responsible for data collection, signaling for indicating/requesting data collection and dataset sharing.

Since data collection is used for different purposes, the function of data collection may not be a single functional entity but may be multiple functional entities serving different purposes of functions in LCM. For example, the datasets for model training may be collected and stored somewhere if offline training is performed. For supervised learning, ground-true data also needs to be collected for model training. For model inference, the inputs are live data collected over the air interface. For model monitoring, the ground-true data may also need to be collected depending on whether intermediate KPIs are evaluated and data distribution for input and/or output is observed. It is desired that the function entities of data collection for different purposes is located with the corresponding functions to avoid dataset transferring over the air interface. In other words, if model training and update is performed at the UE side, UE should be responsible for data collection used for model training. If model monitoring is performed at the UE side, UE should be responsible for data collection used for model monitoring. This principle is also applicable to network side.

Proposal 5: Data collection comprises multiple functional entities serving for different purposes of functions. The functional entity of data collection is co-located with the function for which the dataset is used.

For data collection, whether existing methods can be used should be studied, including identifying these existing methods and their potential extensions. If the existing and extension of the existing methods are not sufficient, new mechanism for data collection can be considered. However, such evaluation should be based on the requirements of data collection. First of all, we need to discuss which aspects can be considered as the requirements of data collection for different purposes. In our understanding, the data collection requirements can consider the following aspects: data size, data collection latency, validity, security and privacy. The definition of data collection requirements can help RAN1 and RAN2 to figure out appropriate methods and procedures to collect data for different purposes.

Data collection requirements for different purposes may be different. For example, data collection for offline training takes time and does not have stringent latency requirements.  The data collection function may broadly collect data from a large number of UEs over a relatively long period of time. UE may log data for a certain period of time. The accumulated data may be collected all at once. The size of the data would be large. As a result, data collection for offline training is characterized by less stringent latency requirements, large data sizes, and longer validity time. Since the data collected for training may be located in an OTT server, it may or may not be a trusted server. Security and privacy issues need to be considered.

Compared to data collection for training, data collection for monitoring may have opposite requirements. ​Whenever data is collected to monitor intermediate KPIs or system performance KPIs in the model monitoring process, it needs to reflect the instantaneous performance. Data collection for monitoring may be performed on-demand or periodically while the AI/ML algorithm is in use. Data collection for offline training is characterized by time-critical latency requirements, limited data size, and short validity time. It is expected that data collection requirements for model inference are similar as for model monitoring.

Proposal 6: Data collections should consider the requirements of data size, latency, validity, security and privacy. Consider distinct data requirements for different purposes, e.g., offline training and model monitoring/inference.

Considering the case that data may be collected for offline training by an OTT server, similar as model transfer, data can be transferred to the OTT server through UP tunnel. For UE-sided model, there are generally two ways mentioned for model training, i.e., the AI/ML model is trained at the network side and transferred to UE then (collaboration level z) or the AI/ML model is trained at the UE side (collaboration level y). If the UE-sided model is trained at the UE side, it is not practical to collect data and train model purely on UE devices. The practical way is that the data is collected by UEs and transferred to an OTT server, where big data set is built, and the AI/ML model is trained and developed there, just as illustrated in Figure 2.

How to collect the data to the OTT server needs to be considered.

- Option 1: Data transfer to UE to OTT server with RAN awareness

- Option 2: Data transfer from UE to OTT server without RAN awareness

In option 1, RAN node is aware of the UE data transfer. When UE data arrives at RAN node, RAN node processes those data, add RAN related information and transfer the UE data to the OTT server through 5GS. Based on current system architecture, UE data is transferred through CP. The UE data can be included in a container. In this case, RAN node doesn’t know what information is being transferred from the UE to the OTT server. Current data collection methods including immediate/log MDT, L1 measurement, L3 measurement, early measurement and UAI can also serve the same purpose. In this case, RAN node knows what information is delivered as UE data.

In option 2, RAN node is not aware of the UE data transfer, which is delivered through UP. Even if the data transfer through UP channel is transparent to air interface, it may not be out of 3GPP scope and requires SA involvement, especially SA2. In last RAN2 meeting, existing EVEX framework was proposed to be studied as one of the data collection methods. We assume it is one variation of option 2.

Figure 2 Data transfer from UE to an OTT server

Proposal 7: RAN1 consider UE data transfer from UE to the OTT server with the following ways. FFS on others.

- Option 1: Data transfer to UE to OTT server with RAN awareness

- Option 2: Data transfer from UE to OTT server without RAN awareness

If data is collected by the network side, following two options can be considered for data collection:

- Alt 1: Utilize existing or extension of existing measurement/report procedures

- Alt 2: Utilize (new)procedure particular for data collection request and control

In Alt 1, data collection relies on the measurement and report mechanisms, no matter whether such mechanisms already exist in L1 and/or L3 or is enhanced for different AI use cases. In Alt 2, data collection can be requested and controlled by a new procedure. For example, ground-true data or assistant data for model training and model monitoring may be requested by the peer entity. For ground-true data collection, the mechanism to start and stop the data collection need to be considered. For the assistant data, the mechanism to request such assistant data from peer entity should also be considered.

Proposal 8: Study the following mechanisms for data collection: utilize existing or extension of existing L1/L3 measurement and report procedure or utilize procedure particular for data collection request and control.

For CSI compression training Type 3 (Separate training at NW side and UE side), how to perform dataset exchange should be considered. Similar as the clarification of model transfer, dataset exchange can be performed in the following two ways:

- Proprietary way: Dataset exchange is transparent to 3GPP signalling over the air interface.

- 3GPP specified way: Dataset exchange is supported by 3GPP signaling and mechanism.

If dataset exchange is performed in a proprietary way, it is assumed that there is no specification impact and not need to be discussed in 3GPP. Even if this solution is out of 3GPP scope, we need try to understand how it works. One possible way is that that the OTT server of UE side and network side communicates with each other directly based on certain business agreements between those vendors during the offline training phase. Based on this assumption, it is very likely that model update will have the similar procedure. It is expected that both initial AI/ML model deployment and model update requires relatively a long period to make the AI models be developed, tested, certified, and deployed at both the UE and gNB. It is unclear how to perform fine tuning for the model which has been already deployed at the UE and gNB with collection of the fresh data over the air interface.

If the dataset exchange is performed in 3GPP specified way, one concern mentioned is the overhead due to dataset exchange over the air interface considering that the dataset is large. Another relevant question is whether data format for dataset exchange also needs to be specified. In our understanding, 3GPP specified way means the procedure and signaling to trigger and control dataset exchange is specified. It doesn’t necessarily mean that the data format needs to be specified. Modal fine tuning is in nature to be supported with collection of the fresh data over the air interface.

RAN1 can study both proprietary way and 3GPP specified way and even combination of them for dataset exchange for two-sided model Type 3 training.

Proposal 9: Study the proprietary way, 3GPP specified way and combination of them for dataset exchange for two-sided model Type 3 training.

## Model Control and Model Monitoring

In RAN1#110b-e meeting, RAN1 made following agreements related to model selection, activation, deactivation, switching, and fallback. It is not convenient to repeat the long word list when we talk about those operations. We can call the above operations as model control for short to facilitate the discussion.

Proposal 10: Use ‘model control’ to stand for model selection, activation, deactivation, switching, and fallback to facilitate the discussion.

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

Although the terminology discussion is deprioritized, it is worthwhile to clarify the difference among those terms related to model control, including model selection, modal activation, deactivation, and model switching. In our understanding, model selection is to select the first AI/ML model at the very beginning when AI/ML operation is enabled. After that, model activation/deactivation, switching and fallback is performed based on the model monitoring and change of scenarios/configurations/sites. Modal switching can be realized by deactivating the current model and activating a new model. Even though the purposes of model switching, and model activation/deactivation may be different, the actual operation may be the same. It is worthwhile to clarify the difference between them if both model switching and model activation/deactivation are kept on the table and studied.

Proposal 11: Clarify model selection as the operation to select the first AI/ML model at the very beginning when AI/ML is enabled.

Proposal 12: Clarify the difference between model switching and model activation/deactivation and discuss the need to keep both.

RAN1 also agreed to study model monitoring at least for the purposes of model control and model update and elaborated on how model monitoring interacts with model control for the use case of CSI compression and BM. There hasn’t been relevant agreement for positioning yet.

Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - Alt1. NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - Alt2. UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Table 1 is trying to match the use case specific agreements with general aspect agreements and makes those agreements in different places keep consistent. Furthermore, how modal monitoring interacts with model control should be discussed.

It can be observed that for at least for UE sided models and two-sided models, network-decided approach is considered for all use cases, including CSI compression, BM-case 1 and BM-case 2. For the case of network-decided and network-initiated mechanism, modal monitoring is performed at the network side. For the case of network-decided and UE-initiated mechanism, UE performs model monitoring, sends model control request to network and network decides how to perform model control based on the inputs from the UE. It’s possible that the network may also performs model monitoring.

Proposal 13: For network-decided and network-initiated mechanism, model monitoring is performed at the network side.

Proposal 14: For network-sided and UE-initiated mechanism, consider the cases that model monitoring is performed at the UE side or at both UE and network side.

UE-decided approach is only considered for UE-sided model, like BM-case 1/2 where AI/ML training, inference as well as model monitoring is performed at the UE side. However, there are some uncertainties for UE-decided approach, e.g., what events/conditions are required to be configured by the network and whether UE needs to report its decision to the network. Furthermore, UE-decided approach assumes that model monitoring is performed at the UE side.

If UE-decide approach is UE-autonomous without reporting UE’s decision to the network, it is suitable for the collaboration level x. UE behaviours for model monitoring and modal control is UE implementation specific and will not be specified.

Proposal 15: For UE-decided mechanisms, model monitoring is performed at the UE side.

Proposal 16: If model control is UE-autonomous without reporting the decision to the network, model monitoring and model control at the UE side is implementation specific and will not be specified.

Table 1 Network-decided vs. UE-decided for different use cases

| Network-decided, Network-initiated, |  | Network-decided, UE-initiated |  |
| --- | --- | --- | --- |
| - CSI compression (Alt 1) - BM-case 1 and BM-case 2 (Alt 2) |  | - CSI compression (Alt 2), - BM-case 1 and BM-case 2 (Alt 3) |  |
| UE-decided, decision reported to network, network configuration | UE autonomous, decision reported to network |  | UE autonomous, decision not reported to network |
| BM-case 1 and BM-case 2 (Alt 1) |  |  |  |

## Model ID

In RAN2#119b-e meeting, RAN2 assumed that a model is identified by a model ID. Its usage is FFS. In RAN1#110b meeting, RAN1 made following agreements for model ID.

Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations. FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality-based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations

According to the discussion for each use case/functionality, a set of specific models e.g., scenario-/configuration-specific and site-specific models may be developed. Therefore, multiple AI models for the same functionality will be supported. For model-ID-based LCM, the multiple models are visible to the peer entity, e.g., the UE-sided models are visible to the network side. For functionality-based LCM, even if there may be multiple AI models at the UE side, the model switching operation at the UE side is transparent to the network.

During model training and model development, each model should be identified by an identifier and even a version number considering the model can be updated. Considering the AI/ML model is kept proprietary and left to implementation, it’s very likely that different vendors may have different sets of AI/ML models even for the same functionality. Therefore, model IDs in each vendor’s repository is used for model development and management, which is implementation and vendor specific. We call it original model ID in Figure 1 to avoid potential confusion.

In RAN2#121 meeting, RAN2 assumed that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified. If the network is responsible for model control, i.e., model activation/deactivation/switching, it needs to re-structure the UE vendor specific model ID lists to another form of model IDs, which are manageable by the network. We call it global model ID in Figure 1. Each global model ID of this type should be able to uniquely identify each AI model known to the network.  Certain network functions or OAM may be responsible for model ID management, where the linkage between the original vendor-specific model IDs and the global model IDs are maintained. Therefore, the model ID globally identifying each AI/ML model is a permanent ID, which is assigned and controlled by the network.

Since each AI/ML model is identified by the global model ID at the network side, it can be used in UE capability reporting, indicating which AI/ML model is supported by the UE. For collaboration level y without model transfer, indicating the model ID in UE capability also implies the availability of the model at the UE. For collaboration level z with model transfer, indicating the model ID in UE capability only implies that UE is capable of executing the AI/ML model, but the corresponding model may not be available at the UE and needs to be downloaded from the network. Therefore, the global model ID is used to identify the AI/ML model which is being model transferred from the network to the UE.

For two-sided model, the global model ID is also used to pair the AI/ML models between the UE and the network. When a global model ID is assigned to a UE part model of two-sided model, the network pairs the UE part model with a network part model.

The global model ID may have multiple fields and very long length, it’s not efficient to use the global model ID directly for model control. During model control in model-ID-based LCM, an appropriate AI/ML model among a set of AI/ML models is chosen for usage, which can fit the scenario/configuration/site. Considering model activation/deactivation/switch/selection only occurs when UE is in CONNECTED state, a temporary model index can be assigned to each model through model configuration. Model activation/deactivation/switch/selection and fallback can rely on the model index, similar as SCell index for model activation, deactivation, switching and fallback. Based on the model index, both UE and network knows which AI/ML is being in use and monitored.

Proposal 17: The global model ID is a permanent ID, which is assigned and managed by the network.

Proposal 18: The global model ID for each AI/ML model is used for the following purposes:

- Model test certification

- UE capability reporting to indicate which AI/ML model is supported by the UE

- Model transfer to indicate which AI/ML model is being transferred to the UE

- Model paring of the AI/ML models between the UE and network for two-sided model

Proposal 19: A model index is assigned to each model for model activation/deactivation/switch/selection/fallback by the network through model configuration.

Figure 3 Different types of model IDs for different purposes and managed by different network entities

## Model Identification

In last RAN1 meeting, RAN1 made following working assumptions for model/functionality identification.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Model identification and functionality identification were defined a general way, which consider both of the cases that UE shares UE-side model information to network or network shares network-side model information to the UE. For the case that network shares network-sided information to the UE, since the network-sided model is trained and generated by the network side itself, network can align the information for the model/functionality directly through DL signalings or messages. For the case that UE shares UE-sided model information to the network, how the model/functionality identification procedure is initiated and performed by the UE should be considered.

The UE-sided model, no matter it’s one-sided model or two-sided model, there are two possible ways to deploy the AI/ML model to the UE side: AI/ML model is generated and trained at the network side and delivered to the UE side; AI/ML model is generated and trained at the UE side.

The network and UE should refer to the same AI/ML model unambiguously during AI/ML collaborations and LCM. For the former case, since the AI/ML model is generated at network side, it is in nature known to the network side. The AI/ML model is deployed to the UE side through model transfer procedure, which may also provide model-related information to the UE. For the latter case, the AI/ML model is generated at the UE side, which is unknown to the network side. UE needs to align with the network to share certain information for the AI/ML model. UE may provide model-related information to the network through model identification procedure.  After model identification, the related information for the multiple AI/ML models for different functionalities will be kept at the network side for each UE.

There are generally different ways to support model identification, i.e., either with or without UE report over the air interface.  If model identification is performed by UE report over air interface, it can be performed through RRC layer or NAS layer. The mechanism for model identification should be discussed in RAN2 and SA2. RAN1 needs to discuss what information should be shared between UE and network for model/functionality identification.

Proposal 20: Study what associated information needs to be provided through model identification for the case that UE sided model is generated and training at the UE side and leave the model identification procedure to RAN2 discussion.

RAN2 also assumed that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. This is general assumption, regardless of whether the AI/ML mode is at UE side, network side or at both sides. Each AI/ML model is designed for a specific use case and appliable for certain scenario/configuration considering the generalization performance. Information on model input needs to be known for each AI/ML model and data collection can collect the required data for training, inference, and monitoring.

Proposal 21: For each AI/ML model, at least following associated information needs to be known. FFS on other information.

- Model functionality (e.g., beam management, CSI compression, positioning)

- Applicable scenario/configuration and site

- Information on model input

- Information on model output

- Information on pairing between UE-sided part and network-sided part of two-sided model

## Model Transfer/Delivery

In last RAN1 meeting, RAN1 made the working assumptions that gNB-UE collaboration Level y-z boundary is defined based on whether model delivery is transparent to 3GPP signaling over the air interface or not.

Where the AI/ML models are stored determines through which tunnel (CP or UP) the AI/ML model should be transferred. RAN2 is now discussing following options for model transfer:

- Option 1: Model transfer/delivery between UE and gNB. For this option, CP and UP solutions can be studied

- Option 2: Model transfer/delivery between UE and CN. For this option, CP and UP solutions can be studied

- Option 3: Model transfer/delivery between UE and LMF. For this option, CP solution can be studied

- Option 4: Model transfer/delivery between UE and server. The option may be transparent to 3GPP, and it can be left to implementation

When the AI/ML is transferred between the network and UE, one important issue is in what kind of format the model is transferred. Two approaches were mentioned in RAN1 discussion. One approach is that the AI/ML model is transferred in the format of runtime image, i.e., one entity transfers the AI/ML model runtime image directly to the other entity. The other approach is that the AI/ML model is transferred through a specific format specified by 3GPP. However, the model transfer format should also be discussed by RAN2. RAN1 should discuss what kind of information needs to be transferred, i.e., model structure+ parameters or parameters only. Furthermore, RAN2 is evaluating the pros and cons for each model transfer/delivery option. The information about the requirements of model transfer/delivery in terms of model size, latency as well as the model update frequency is required to consider the applicability of those solutions.

Proposal 22: For model transfer, RAN1 focuses on what kind of information needs to be delivered for model transfer. RAN1 leaves model transfer channel (CP or UP) and model format to RAN2 discussion. RAN1 should provide inputs for the requirements of model transfer/delivery in terms of model size, latency and model update frequency to RAN2 to evaluate the applicability of different model transfer/delivery solutions.

Table 2 Different options to support model transfer/delivery

# Conclusion

Functional Framework

Proposal 1: The AI/ML functional framework for air interface includes the functions of data collection, model training, model inference and model monitoring. Take Figure 1 as starting point for functional framework discussion.

General aspect and use case specific aspect

Proposal 2: The mechanisms for data collection, model training, model monitoring and model inference are use case specific and studied for each use case.

Proposal 3: The mechanisms for model transfer, model configuration, model selection, model switching, model activation/deactivation, fallback and UE capability reporting can be common for different use cases and be studied in the general aspect.

Proposal 4: RAN1 doesn’t intend to design two distinct mechanisms for functionality-based and model-ID-based LCM. Functionality-based and model-ID-based LCM procedures should share as much commonality as possible.

Data collection

Proposal 5: Data collection comprises multiple functional entities serving for different purposes of functions. The functional entity of data collection is co-located with the function for which the dataset is used.

Proposal 6: Data collections should consider the requirements of data size, latency, validity, security and privacy. Consider distinct data requirements for different purposes, e.g., offline training and model monitoring/inference.

Proposal 7: RAN1 consider UE data transfer from UE to the OTT server with the following ways. FFS on others.

- Option 1: Data transfer to UE to OTT server with RAN awareness

- Option 2: Data transfer from UE to OTT server without RAN awareness

Proposal 8: Study the following mechanisms for data collection: utilize existing or extension of existing L1/L3 measurement and report procedure or utilize procedure particular for data collection request and control.

Proposal 9: Study the proprietary way, 3GPP specified way and combination of them for dataset exchange for two-sided model Type 3 training.

Model control and model monitoring

Proposal 10: Use ‘model control’ to stand for model selection, activation, deactivation, switching, and fallback to facilitate the discussion.

Proposal 11: Clarify model selection as the operation to select the first AI/ML model at the very beginning when AI/ML is enabled.

Proposal 12: Clarify the difference between model switching and model activation/deactivation and discuss the need to keep both.

Proposal 13: For network-decided and network-initiated mechanism, model monitoring is performed at the network side.

Proposal 14: For network-sided and UE-initiated mechanism, consider the cases that model monitoring is performed at the UE side or at both UE and network side.

Proposal 15: For UE-decided mechanisms, model monitoring is performed at the UE side.

Proposal 16: If model control is UE-autonomous without reporting the decision to the network, model monitoring and model control at the UE side is implementation specific and will not be specified.

Model ID

Proposal 17: The global model ID is a permanent ID, which is assigned and managed by the network.

Proposal 18: The global model ID for each AI/ML model is used for the following purposes:

- Model test certification

- UE capability reporting to indicate which AI/ML model is supported by the UE

- Model transfer to indicate which AI/ML model is being transferred to the UE

- Model paring of the AI/ML models between the UE and network for two-sided model

Proposal 19: A model index is assigned to each model for model activation/deactivation/switch/selection/fallback by the network through model configuration.

Model Identification

Proposal 20: Study what associated information needs to be provided through model identification for the case that UE sided model is generated and training at the UE side and leave the model identification procedure to RAN2 discussion.

Proposal 21: For each AI/ML model, at least following associated information needs to be known. FFS on other information.

- Model functionality (e.g., beam management, CSI compression, positioning)

- Applicable scenario/configuration and site

- Information on model input

- Information on model output

- Information on pairing between UE-sided part and network-sided part of two-sided model

Model transfer/delivery

Proposal 22: For model transfer, RAN1 focuses on what kind of information needs to be delivered for model transfer. RAN1 leaves model transfer channel (CP or UP) and model format to RAN2 discussion. RAN1 should provide inputs for the requirements of model transfer/delivery in terms of model size, latency and model update frequency to RAN2 to evaluate the applicability of different model transfer/delivery solutions.

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, Electronic Meeting, Dec. 6 - 17, 2021.

- TR 37.817 V17.0.0 Study on enhancement for Data Collection for NR and EN-DC