3GPP TSG RAN WG1 #112bis-e           	                                   R1-2303343

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 9.17.10

Source: MediaTek Inc.

Title: On UE feature discussion for Rel-18 MC enhancements

Document for: Discussion and decision

- Introduction

This contribution provides our views on UE features for Rel-18 MC enhancements.

- On UE feature for multi-cell scheduling with a single DCI

In RAN1 #110be [1], it is agreed that:

Agreement:

- Confirm below working assumption reached in RAN1#110 meeting.

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- …

In RAN1 #111 [2], it is agreed that:

Agreement:

- For a set of cells which is configured for multi-cell scheduling, up to 4 cells within the set of cells are supported.

  - A DCI format 0_X/1_X can schedule PUSCH(s)/PDSCH(s) on a combination of co-scheduled cells among the same set of cells.

In RAN1 #112 [3], it is agreed that:

Agreement

Following is supported in Rel-18 multi-cell scheduling

- A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X.

- Up to 4 sets of cells can be configured per PUCCH group.

- …

- Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell.

  - The value of N is reported as UE capability.

  - …

We hence have the following proposals:

Proposal 1: For R18 multi-cell scheduling with a single DCI, introduce the following UE capabilities:

- Supportable maximum number of co-scheduled cells in one set (for DL and UL separately)

  - Candidate values: {2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one PUCCH group (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one cell group (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one PUCCH group with the same scheduling cell (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one cell group with the same scheduling cell (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- On UE feature for multi-carrier UL Tx switching

For UE feature of multi-carrier UL Tx switching, we have the following proposal:

Proposal 2: For R18 UL Tx switching, the following UE feature need to be introduced.

| Feature group | Components | Prerequisite FG | Type | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| ULTxSwitchingBandPair-r18 | - {bandIndexUL1-r18, bandIndexUL2-r18} - Switching among bands {3bands, 4bands} - uplinkTxSwitchingPeriod-r18 {n35us, n140us, n210us} - uplinkTxSwitching-OptionSupport-r18{switchedUL, dualUL, both} | ULTxSwitchingBandPair-r16 | Per BC | Optional |

- Conclusion

In this contribution, we focus on the discussions on UE features for Rel-18 MC enhancements and have the following proposals:

Proposal 1: For R18 multi-cell scheduling with a single DCI, introduce the following UE capabilities:

- Supportable maximum number of co-scheduled cells in one set (for DL and UL separately)

  - Candidate values: {2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one PUCCH group (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one cell group (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one PUCCH group with the same scheduling cell (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

- Supportable maximum number of sets in one cell group with the same scheduling cell (for DL and UL separately)

  - Candidate values: {1, 2, 3, 4}

  - Type: Per BC

Proposal 2: For R18 UL Tx switching, the following UE feature need to be introduced.

| Feature group | Components | Prerequisite FG | Type | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| ULTxSwitchingBandPair-r18 | - {bandIndexUL1-r18, bandIndexUL2-r18} - Switching among bands {3bands, 4bands} - uplinkTxSwitchingPeriod-r18 {n35us, n140us, n210us} - uplinkTxSwitching-OptionSupport-r18{switchedUL, dualUL, both} | ULTxSwitchingBandPair-r16 | Per BC | Optional |

- Reference

Chairman’s Notes, RAN1 #110be

Chairman’s Notes, RAN1 #111

Chairman’s Notes, RAN1 #112