3GPP TSG RAN WG1 #112-bis-e			           R1-2303345

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 9.7.2

Source: MediaTek Inc.

Title: On NW Energy Saving Enhancements for Cell DTX/DRX Mechanism

Document for: Discussion and Decision

# Introduction

In RAN#98-e meeting, there approved the work item scope for Rel-18 network energy saving [1]. One item of the work scope is related to cell DRX/DRX mechanism:

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In RAN1#112 meeting, the following agreements are achieved [2][3]:

Agreement - RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX - Further discussions on other aspects are not precluded

Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

Based on the above, this contribution will discuss design of cell DTX/DRX and alignment with UE DRX for potential decisions.

# Design of cell DTX/DRX and Alignment with UE DRX

In order to enhance the network energy efficiency, cell DTX/DRX can be applied to reduce the transmission and reception power of base stations and UE. However, this may cause impact to the legacy UEs that are in idle or inactive state and rely on certain signals or channels for, e.g., synchronization and/or positioning. Therefore, it is important to ensure that these signals or channels, such as TRS and PRS, are not affected by the non-active periods of cell DTX/DRX.

Observation 1: To comply with the constraint, “The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided”, signals or channels that can be utilized by idle/inactive (legacy) UEs should not be impacted by non-active periods of cell DTX/DRX.

Proposal 1: TRS and PRS should not be impacted by non-active periods of cell DTX/DRX, considering the usage by idle/inactive (legacy) UEs.

Another challenge of applying cell DTX/DRX is to maintain the quality of service for latency sensitive applications, such as voice over IP (VoIP) and augmented reality (AR). These applications require low end-to-end delay and high reliability for data transmission. However, if data retransmissions are prohibited in the non-active periods of cell DTX/DRX, this may increase the latency and packet loss for these applications. Therefore, it is desirable to allow data retransmissions in the non-active periods of cell DTX/DRX.

One possible solution to balance network energy saving and latency impact to restrict new data transmission to the active periods of cell DTX/DRX, while allowing retransmission(s) of previously scheduled data in both active and non-active periods of cell DTX/DRX. This can reduce the network energy consumption caused by new data transmission in the non-active periods of cell DTX/DRX, while ensuring the reliability and timeliness of data retransmission(s) for latency constrained applications.

Observation 2: To avoid excess latency for latency constrained use cases, e.g., VoIP and/or AR, schedule of data retransmissions should be allowed in non-active periods of cell DTX/DRX.

Proposal 2: New data of UE specific scheduling and PDSCH/PUSCH is not expected in non-active periods of cell DTX/DRX, while retransmission(s) of scheduled data in active periods of cell DTX/DRX is still allowed.

Alignment and aggregation of the active periods of cells and UEs can save significant energy for scenarios where there are many active UEs with low data rates, such as VoIP service [4][5]. In such cases, the signaling overhead for coordinating the cell and UE activities may outweigh the energy savings from sleep states. Therefore, a more efficient way to align and aggregate the cell and UE active periods is needed to reduce the energy overhead and improve the NES performance.

One possible solution is to specify a cell-wise indication mechanism similar to the Rel-17 TRS for idle/inactive UEs, where network can broadcast multiple configurations of cell DTX/DRX using SIB and indicate which configuration is applicable for all UEs using paging indications (including paging DCI and PEI). This can resolve the signaling overhead issue and enhance the NES gain. Note that the application delay and how to align UE DRX and Cell DTX/DRX after receiving the indication still require specific discussion and decision for this feature.

Observation 3: In [4] and [5], alignment and aggregation of cell and UE active periods can achieve significant network energy saving (NES) gain, particularly for light load scenario. However, there can still be a large amount of active UEs in light load scenario, e.g., as VoIP service.

Proposal 3: Efficient cell-wise signaling design is developed for alignment and aggregation of cell and UE activities so as to avoid excess UE-specific signaling overhead due to a large amount of active UEs (e.g., in VoIP service).

Proposal 4: Analogous mechanism to Rel-17 TRS for idle/inactive, i.e., SIB for configuration broadcast and paging indications for activation/adaptation of candidate configuration(s), is specified.

- FFS: Application delay and alignment of UE DRX and Cell DTX/DRX after receiving the indication.

# Summary

In this contribution the following observations and proposals are provided:

Observation 1: To comply with the constraint, “The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided”, signals or channels that can be utilized by idle/inactive (legacy) UEs should not be impacted by non-active periods of cell DTX/DRX.

Proposal 1: TRS and PRS should not be impacted by non-active periods of cell DTX/DRX, considering the usage by idle/inactive (legacy) UEs.

Observation 2: To avoid excess latency for latency constrained use cases, e.g., VoIP and/or AR, schedule of data retransmissions should be allowed in non-active periods of cell DTX/DRX.

Proposal 2: New data of UE specific scheduling and PDSCH/PUSCH is not expected in non-active periods of cell DTX/DRX, while retransmission(s) of scheduled data in active periods of cell DTX/DRX is still allowed.

Observation 3: In [4] and [5], alignment and aggregation of cell and UE active periods can achieve significant network energy saving (NES) gain, particularly for light load scenario. However, there can still be a large amount of active UEs in light load scenario, e.g., as VoIP service.

Proposal 3: Efficient cell-wise signaling design is developed for alignment and aggregation of cell and UE activities so as to avoid excess UE-specific signaling overhead due to a large amount of active UEs (e.g., in VoIP service).

Proposal 4: Analogous mechanism to Rel-17 TRS for idle/inactive, i.e., SIB for configuration broadcast and paging indications for activation/adaptation of candidate configuration(s), is specified.

- FFS: Application delay and alignment of UE DRX and Cell DTX/DRX after receiving the indication.

# Reference

RP-223540, “New WID: Network energy savings for NR”, RAN#98-e

“Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0”, online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112/Report

RP-230565, “Status report for WI: Network energy savings for NR”, Rapporteur (Huawei), RAN#99

3GPP TR 38.864, “Study on network energy savings for NR (Release 18)”, Dec., 2022; online available: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3987

R1-2212259, “NW Energy Savings Performance Evaluation”, MediaTek, RAN1#111