3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303346

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.15.1

Source:	MediaTek Inc.

Title:	Evaluation methodology for IMT-2020 satellite

Document for:	Discussion

# 1	Introduction

RAN#99 agreed on a Study Item for a Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology [1]. In this contribution we provide basic simulation assumptions to be used for the self-evaluation for IoT NTN for fulfilment of the “Connection Density” requirement

# 2	Discussion

## 2.1	Background

ITU-R requirement

For the submission to ITU-R, our view is that the IoT NTN technology (NB-IoT and eMTC) should target fulfilment of at least the “Connection Density” requirement from ITU-R Report M.2514 [2].

The Connection Density requirement is defined as the following:

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2). A connection density of at least 500 devices per km2 should be supported. The requirement was derived assuming up to and including a 30 MHz bandwidth.

The proponent should report the connection density achievable by the candidate RITs/SRITs, and identify the assumed frequency band(s) of operation and the channel bandwidths in that(those) band(s).

The evaluation methodology for the systems simulations for determining the connection density should follow the principles outlined in § 7.1.3 of Report ITU-R M.2412 with needed adaptations.

The evaluation is conducted in the Rural-mMTC-s test environment (see § 8.2), applicable to handheld devices or, optionally, MTD. Suitable evaluation configuration parameters according to § 8.2 for this test environment should be used and declared by the proponent of the evaluation.

## 2.2	Simulation parameters for IoT NTN Connection Density

Principles for parameter selection

We provide some system simulation assumptions for analysing Connection Density for IoT NTN in Table A and some link level parameters in Table B. The principles used to define these assumptions were:

- Reuse relevant parameters for radio configuration from IMT-2020 submission (Rel15/16) – see end of document to compare

- Reuse where possible from TR38.821 parameter sets.

- Applies the “example” evaluation parameters from ITU M.2514 [2].

- We assume the total bandwidth needed to meet the requirement should be reported

It is also expected that many of these parameters could be applicable for Connection Density modelling for NR, if such evaluation is desired.

Satellite constellation

We have no strong view on the satellite constellation to use. However, in [2] an example evaluation configuration is described for LEO 600km and aligned with Set-1 parameters from [3]. Also other performance requirements will likely use LEO 600km. Therefore, to minimise overall effort for companies, we have based the simulation parameters on LEO-600km constellation.

It is also important the note the following text: “The parameters are solely for the purpose of consistent evaluation of the candidate satellite radio interface(s) and relate only to specific test environments used in these evaluations. They should not be considered as the values that must be used in any deployment of any IMT-2020 system nor should they be taken as the default values for any other or subsequent study in ITU or elsewhere. They do not necessarily themselves constitute any requirements on the implementation of the system.”

We understand this to mean that whichever configuration is evaluated, then it does not constrain the IMT-2020 satellite applicability only to that deployment scenario of the 3GPP NTN technology modelled.

Traffic model

For IMT-2020 it was also allowed to use “full buffer” or “non-full buffer” traffic to analyse Connection Density.

An example traffic model in [2] seems to be the same as the “non-full buffer” model evaluated for Connection Density as part of IMT-2020 for terrestrial systems. Within the IMT-2020 submission, for the “non-full buffer traffic” model, a 10 second latency bound was required for 32-byte packets to be successfully transmitted by each UE. This was defined within the “Simulation Process”. If non-full buffer is used, application of the 10 second latency bound for IMT-2020 satellite submission should be confirmed as part of the 3GPP simulation methodology definition.

Channel model

More discussion is required on the channel model to use, and whether to consider LOS only, flat fading only, and maybe other parameters discussed in [4].

Observation 1: While many parameters seem to be reusable from past evaluations, further confirmation is needed. In particular, the “channel modelling” details need further discussion.

Table A: SLS parameters for Connection Density performance evaluation

| Radio Access | NB-IoT | eMTC |
| --- | --- | --- |
| Data transmission procedure | RRC resume, data after msg5, RRC suspend | RRC resume, data after msg5, RRC suspend |
| Waveform | DL: OFDMA UL: SC-FDMA | DL: OFDMA UL: SC-FDMA |
| Duplexing | HD-FDD | HD-FDD |
| Channel Bandwidth | 180kHz | 1.08MHz |
| Numerology | 15kHz) (3.75kHz could be discussed as alternative) | 15kHz |
| Total Aggregated Bandwidth | B = 180kHz x N (Bmax = 30MHz TBD: if scaling by 200kHz needed, i.e. standalone or non-anchor channel raster? | B = 1.08MHz x M (Bmax = 30MHz TBD: if scaling by 1.4MHz needed, due to channel raster of 100kHz |
| PRACH configuration | TBD | TBD |
| PUSCH Scheduling Unit | Single tone (as numerology) – see above | 180kHz |
| Power control parameter | TBD | TBD |
| UL DMRS | 2 symbols per 14 OFDM symbols |  |
| SINR calculation | TBD |  |
| Criteria for selection for serving beam | Maximizing RSRP |  |
| Configuration scenario | See Table A1 |  |
| Frequency band | See Table A1 |  |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table A1 |  |
| Satellite antenna pattern | See Table A1 |  |
| Satellite polarization configuration | See Table A1 |  |
| Beam layout | See Table A1 |  |
| Number of beams | See Table A1 |  |
| Frequency re-use factor | See Table A1 |  |
| Polarization re-use | See Table A1 |  |
| Deployment scenario | See Table A1 |  |
| Fast fading model? | TBD if needed – relates to channel model discussion |  |
| Propagation conditions | See Table A1 |  |
| UEs outdoor/indoor distribution | See Table A1 |  |
| UEs coverage distribution | X= 500 UEs per km2 with random and uniform distribution in all the Voronoi cell area associated to each beam. The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |
| UE configuration | Handheld & MTD. See Table A3 for more details. |  |
| UE mobility model | Stationary |  |
| Handover Margin | To be reported by the companies? (1dB / 2dB in TN case, 0dB in NR NTN) |  |
| UE attachment | RSRP |  |
| UE/eNB Receiver type | MMSE-IRC |  |
| Scheduler | To be reported by the companies |  |
| Traffic model (if non-full buffer used) Note: We are open to consider full buffer as an alternative/option | With layer 2 PDU (Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Packet arrival follows Poisson arrival process for non-full buffer system-level simulation (Note: 10 seconds max latency budget required for IMT-2020 terrestrial and we assume the same could apply here, but views welcome.) |  |
| Channel estimation | Realistic |  |
| Performance metric | Connection density of UEs (X) per km2 (see definition and example simulation process) |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. |  |  |

Table A1: Deployment scenario assumptions

| Configuration scenario | LEO-600 (See rationale at start of section 2.2) |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table A2 Note: Same satellite characteristics should be considered for both single and multi-satellite simulations |
| Satellite antenna pattern | See section 6.4.1 in [4]: Bessel function |
| Satellite polarization configuration | Circular |
| Beam layout definition | Singles satellite simulation: See Table A4 |
| Frequency re-use factor | TBD (1 or 3) If FRF =3, then for NB-IoT there would be 1 NB-IoT channel per sub-band (so minimum of 3 channels in total in the system) |
| Polarization re-use | Disabled |
| Channel model | TBD – further discussion needed. |
| Deployment scenario | Rural |
| Propagation conditions | Base-line: Clear Sky, Line of sight (Can we just consider LOS?) |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |

Table A2: Set-1 satellite parameters from [3] (may be common for all scenarios)

| Satellite orbit |  | LEO-600 |
| --- | --- | --- |
| Satellite altitude |  | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 2 m |
| Satellite EIRP density |  | 34 dBW/MHz |
| Satellite Tx max Gain |  | 30 dBi |
| 3dB beamwidth |  | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 50 km |
| Payload characteristics for UL transmissions |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 2 m |
| G/T |  | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 30 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [4]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [4] dB. |  |  |

Table A3: UE characteristics

| Characteristics | Handheld & MTD |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 1) with omni-directional antenna element |
| Rx Antenna gain | 0 dBi |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi |
| UE height | 1.5m |

Table A4: Beam layout definition for single satellite simulation, from [3]

| Scenario | Scenario C2 – LEO600 |
| --- | --- |
| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| Number of beams | Baseline: 19-beam layout considering wrap-around mechanism (i.e. 18 beams surrounding the central beam and allocated on 2 distinct "tiers") |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: Central beam boresight direction computed based on elevation angle target |

## 2.3	Additional parameters for any Link-level simulations for IoT NTN Connection Density (only NB-IoT covered)

Table B captures suggested parameters for LLS, proposed as a starting point. These were adapted from the IMT-2020 submission. Currently it only includes NB-IoT, but corresponding parameters for eMTC can be added too.

Table B: Additional parameters for IoT NTN Connection Density LLS

| Physical channel | NPUSCH |
| --- | --- |
| Simulation bandwidth | Single Tone |
| Number of users in simulation | 1 |
| Link-level Channel model | TBD |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS | 256 |
| Modulation order | [BPSK-π/2, QPSK-π/4] |
| Number of Resource units | 2,3,4,5,6,8,10 |
| Number of repetition | 1,2,4,8,16 |
| Channel estimation | LMMSE |
| Channel coding scheme | Turbo code |

Proposal 1: Consider the IoT NTN Connection Density parameters for “System-level” simulation (section 2.2) and “Link-level” simulations (section 2.3) in this document as a starting point for further discussion and refinement during RAN1#112bis-e.

## 2.4	Simulation process for IoT NTN

The following below is a possible simulation process for “IoT NTN” adapted for IMT-2020 SAT from IMT-2020 evaluation methodology. Note: Full changes compared to IMT-2020 are not shown.

Possible evaluation methods to evaluate the connection density requirement:

− non-full buffer system-level simulation;

− full-buffer system-level simulation followed by link-level simulation.

The following steps are used to evaluate the connection density based on non-full buffer system-level simulation.

Step 1: Set system user number per beam as N.

Step 2: Generate the user packet according to the traffic model.

Step 3: Run non-full buffer system-level simulation to obtain the packet outage rate. The outage rate is defined as the ratio of the number of packets that failed to be delivered to the destination receiver within a transmission delay of less than or equal to 10s to the total number of packets generated in Step 2.

Step 4: Change the value of N and repeat Step 2-3 to obtain the system user number per beam N’ satisfying the packet outage rate of 1%.

Step 5: Calculate connection density by equation C = N’ / A, where the beam area A is the geographical area per beam (central beam only?)

The requirement is fulfilled if the connection density C is greater than or equal to the connection density requirement.

The simulation bandwidth used to fulfill the requirement should be reported. Additionally, it is encouraged to report the connection efficiency (measured as N’ divided by simulation bandwidth) for the achieved connection density.

The following steps are used to evaluate the connection density based on full-buffer system-level simulation followed by link-level simulation.

Step 1: Perform full-buffer system-level simulation using the evaluation parameters for Rural-mMTC test environment, determine the uplink SINRi for each percentile i=1…99 of the distribution over users, and record the average allocated user bandwidth Wuser.

Step 2: Perform link-level simulation and determine the achievable user data rate Ri for the recoded SINRi and Wuser values.

Step 3: Calculate the packet transmission delay of a user as Di = S/Ri, where S is the packet size.

Step 4: Calculate the traffic generated per user as T = S/Tinter-arrival, where Tinter-arrival is the inter-packet arrival time.

Step 5: Calculate the long-term frequency resource requested under SINRi as Bi = T/(Ri/Wuser). Step 6: Calculate the number of supported connections per beam, N = W / mean(Bi). W is the simulation bandwidth. The mean of Bi may be taken over the best 99% of the SINRi conditions.

Step 7: Calculate the connection density as C = N / A, where the beam area A is the geographical area of the beam.

The requirement is fulfilled if the 99th percentile of the delay per user Di is less than or equal to 10s, and the connection density is greater than or equal to the connection density requirement.

The simulation bandwidth used to fulfil the requirement should be reported. Additionally, it is encouraged to report the connection efficiency (measured as N divided by simulation bandwidth) for the achieved connection density.

Proposal 2: Consider the Connection Density simulation process in this document as a starting point for IoT NTN Connection Density.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1: While many parameters seem to be reusable from past evaluations, further confirmation is needed. In particular, the “channel modelling” details need further discussion.

Proposal 1: Consider the IoT NTN Connection Density parameters for “System-level” simulation (section 2.2) and “Link-level” simulations (section 2.3) in this document as a starting point for further discussion and refinement during RAN1#112bis-e.

Proposal 2: Consider the Connection Density simulation process in this document as a starting point for IoT NTN Connection Density.

# 4	References

RP-230736, New SID, “Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology”, RAN#99, March 2023.

Report ITU-R M.2514: “Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020”

TR 38.821 v16.1.0, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”, May 2021.

TR 38.811 v15.4.0, “Study on New Radio (NR) to support non-terrestrial networks (Release 15)”, September 2020.