3GPP TSG RAN WG1 #112bis-e			R1-2303355

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.12.3

Title:	Dynamic switching between waveforms

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In RAN1#112, the following agreements were made for dynamic waveform switching:

Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant. Conclusion The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In the contribution we discuss our views on dyanmic waveform switching for further NR uplink coverage enhancements.

# Discussion

## Application scenarios/formats for switching

In terms of application for waveform switching, current RRC parameters can be used to switch UE between DFT-S-OFDM and CP-OFDM.

- pusch-config

- transformPrecoder in configuredGrantConfig IE

- msg3-transformPrecoder

- msgA-transformPrecoder

The first parameter in the list configures dynamic uplink grants whereas the second parameter configures uplink configured grants. The third and the fourth RRC parameters above configure PUSCH for RACH procedure. The benefit for dynamic waveform switching for msg3 or for msgA is not clear. The new functionality can be limited to normal dynamic PUSCH grants and for configured uplink grants.

As for the DCI formats that support dynamic waveform switching, it was agreed in RAN1-110b-e that at least DCI format 0_1 and format 0_2 are supported. In our view, this is sufficient as the benefit of supporting fallback DCI is not clear.

We propose the following for supported DCI formats:

Proposal 1: Do not support DCI format 0_0 for triggering dynamic waveform switching.

## DCI size alignment

In previous RAN1-112 meeting, DCI size alignment was discussed but no agreement was reached. The latest FL proposal from RAN1-112 [2] is as follows:

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

In our view, the most important aspect in the discussion is to agree on a method that will provide DCI size alignment without any ambiguity at UE side for any of the DCI fields. Next, minimizing the specification impact can be a preferred approach. The current specifications follow per-format alignment (by zero padding) for DCI alignment with C-RNTI (e.g., between UL scheduling DCI and DL scheduling DCI). Similar per-format approach can be considered for aligning DCI size between CP-OFDM and DFT-S-OFDM.

The current specification also defines per-field alignment for DCI with CS-RNTI. This is to ensure that the NDI field is located by the UE correctly. If we follow similar approach, Option-1 or Option-4 seem to require smaller specification impact.

We propose the following:

Proposal 2: Per-format alignment is preferred between two DCI with C-RNTI.

Proposal 3: Per-field alignment is preferred between DCI with C-RNTI and DCI with CS-RNTI.

## PHR enhancements to support dynamic waveform switching

During the discussions in last RAN1-112 meeting, the following proposal was discussed for PHR enhancements to support dynamic waveform switching:

FL proposal 3-1: Support reporting of power headroom information applicable to a reference PUSCH using a target waveform different from waveform of actual PUSCH: - Support at least the case where reference PUSCH has same RB allocation and modulation order as actual PUSCH (FFS: RB allocation not compatible with target waveform) - FFS: Type of power headroom information (power headroom and/or Pcmax,c) - FFS: Whether it can be included in same PHR as legacy PH types - FFS: Triggering of information

Providing additional assistance information to gNB can help make better waveform switching decisions. Such PHR reporting enhancements as listed above can potentially offer some gain. However, such enhancements will also introduce additional signalling overhead. How much gain is expected by providing such additional PHR information needs to be quantified to better understand the benefits.

We suggest that the study on such assistance information should continue to determine potential gains. We propose the following:

Proposal 4: Potential performance gains by PHR enhancements should be justified against additional signaling overhead.

# Conclusions

We have the following proposals:

Proposal 1: Do not support DCI format 0_0 for triggering dynamic waveform switching.

Proposal 2: Per-format alignment is preferred between two DCI with C-RNTI.

Proposal 3: Per-field alignment is preferred between DCI with C-RNTI and DCI with CS-RNTI.

Proposal 4: Potential performance gains by PHR enhancements should be justified against additional signaling overhead.

# References

- RP-221858, Revised WID on Further NR coverage enhancements, China Telecom, June 2022.

- R1-2302222, “Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM”, March 2023.