3GPP TSG RAN WG1 Meeting #112bis-e	R1-2303357

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.9.3

Source:	MediaTek Inc.

Title:	Disabling of HARQ for IoT NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#98e [1], the revised WID on IoT NTN enhancements was endorsed for Release 18. Rel-17 IoT-NTN is considered as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective for disabling of HARQ feedback.

According to the WID, disabling of HARQ feedback for IoT NTN has been agreed in RAN meetings.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

This contribution aims to discuss aspects related to disabling of HARQ feedback for IoT NTN.

- 2 Discussion on disabling of HARQ feedback

2.1 Analysis of DL peak data rates in NB-IoT NTN

In LTE eMTC and NB-IoT specifications, the HARQ process with adaptive retransmissions is asynchronous on DL and UL. In both directions, the receiver tries to decode the transport block (TB) with soft combining of HARQ re-transmissions. In case the transmission is a new TB, the soft buffer should be flushed. In case a retransmission occurred, soft combining of current HARQ re-transmission and previous HARQ transmissions should be performed. The UE knows a scheduled TB is re-transmitted from the new data indicator (NDI) as indicated on DCI.

In satellite systems, larger values of RTD can be typically experienced due to the large channel propagation delay between the UE and the satellite, which depends on the satellite orbit and the elevation angle. HARQ stalling occurs due to the longer satellite RTT between the device and the network, where the UE needs to wait for the reception of a packet or the UL grant to transmit a new packet with HARQ re-transmissions.

In SI phase of IoT NTN, link budget results have been captured and summarized in TR 36.763 [2]. The link budget results would suggest that at very low SNR, repetition levels of 16 or 32 with TBS mapped to 10 RUs will be sufficient in most scenarios.

In cellular NB-IoT, the max TBS is 2536 bits with QPSK modulation as shown in Appendix. Figure 1 illustrates the scheduling of DL packets with 2 HARQ processes. The DCI#1 scheduled NPDSCH #1 can be transmitted during the scheduling delay of the DCI#0 and scheduled NPDSCH #0, and network schedules the NPDSCH #1 right next to NPDSCH #0.

Figure 1: 2 HARQ processes for legacy NB-IoT

Table 1 shows DL maximum data rates of NB-IoT for Cellular, LEO, and GEO with HARQ feedback enabling with Max TBS=2536 bits with QPSK. With repetitions scheduled by the eNB, the impact of the satellite RTD on the NB-IoT data rates in GEO due to HARQ stalling is significant with a 61% reduction of peak data rate observed with 16 repetitions and 10 RUs per TBS compared to cellular (this means TB is transmitted over 16*10 RUs = 160 ms assuming SCS=15 kHz); for LEO, it is not significant with a 7% reduction of peak data rate observed with 16 repetitions. Without repetitions, the impact of the satellite RTD on the NB-IoT data rates in GEO and LEO is significant. For LEO it is relatively smaller than that in GEO.

Observation 1: Peak data rate reduction becomes lower with higher repetitions used in NB-IoT NTN scenarios with HARQ feedback always enabled.

Table 1: peak data rate of enabling HARQ

|  | 2-HARQ Processes TBS=2536 bits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DL without repetitions for NPDCCH and NPDSCH | DL with 4 repetitions for NPDCCH and NPDSCH | DL with 8 repetitions for NPDCCH and NPDSCH | DL with 16 repetitions for NPDCCH and NPDSCH | DL with 32 repetitions for NPDCCH and NPDSCH |
| Cellular | 126.8 kbps | 49.2 kbps | 27.1 kbps | 14.3 kbps | 7.3 kbps |
| LEO | 74.6 kbps (~41% reduction) | 38.7 kbps (~21% reduction) | 23.6 kbps (~13% reduction) | 13.2 kbps (~7% reduction) | 7.1 kbps (~3% reduction) |
| GEO | 8.7 kbps (~93% reduction) | 7.8 kbps (~84% reduction) | 6.9 kbps (~74% reduction) | 5.6 kbps (~61% reduction) | 4.1 kbps (~44% reduction) |

2.2 Configure enabling/disabling on HARQ feedback in IoT NTN

Disabling of HARQ feedback was discussed in last few meetings and agreements are made in RAN1 and RAN2 for options of disabling HARQ feedback in Rel-18 IoT NTN.

Agreement (RAN1 110) For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above Agreement (RAN1 112) Confirm the following working assumption with the following update: Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes. - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”. For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A. Agreements (RAN2 119): - Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability - For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN - From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT). Agreements (RAN2 119b): - For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1. - Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback

In RAN1 112, RAN1 made agreement on how to configure/indicate enabling/disabling of HARQ feedback for downlink transmission for NB-IoT NTN, two new indication alternatives were involved, RRC per-HARQ process bitmap and DCI solution.

As illustrated in Figure 2, there are four cases on how to configure/indicate enabling/disabling of HARQ feedback for downlink transmission for NB-IoT NTN with the two new indication alternatives:

- when both RRC per-HARQ process bitmap and DCI solution are configured, support DCI indication to override RRC bitmap configuration

- when RRC per-HARQ process bitmap solution is configured and DCI solution is not configured, support RRC bitmap configuration

- when RRC per-HARQ process bitmap solution is not configured and DCI solution is configured, support DCI solution

- when both RRC per-HARQ process bitmap solution and DCI solution are not configured, the legacy procedure should be supported where all HARQ processes with feedback enabled

Figure 2: enabling/disabling of HARQ feedback for downlink transmission for NB-IoT NTN

Observation 2: Both RRC per-HARQ process bitmap solution and DCI solution are new indication alternatives.

Observation 3: When both RRC per-HARQ process bitmap solution and DCI solution are not configured, the legacy procedure where all HARQ processes with feedback enabled should be supported.

For disabling of HARQ feedback in IoT NTN, enabling/disabling HARQ feedback per HARQ process via UE specific RRC signaling is aligned with RAN2 agreements with low specification impact. As IoT device is more delay tolerant, enabling/disabling HARQ feedback per HARQ process via UE specific RRC signaling is enough to ensure network flexibility in HARQ feedback enabling/disabling scheduling.

There may be the case when the HARQ process is utilized for MAC CE but HARQ feedback is disabled for the HARQ process by RRC per-HARQ process bitmap. For the HARQ process, the network may override RRC disabled configuration by DCI with re-interpreted fields. The DCI solution needs to be enabled by a higher layer parameter npdsch-HarqFeedbackDisableOverride-Config via UE-specific RRC signalling based on UE capability.

Proposal 1: For NB-IoT, DCI-based overridden indication is supported if additional higher layer parameter is enabled by UE-specific RRC signalling based on UE capability.

Proposal 2: DCI-based overridden indication only applied to semi-statically HARQ feedback disabled processes.

The network can enable HARQ feedback for the HARQ process with repetitions 16 or higher. Based on analysis provided in the previous section, which showed that disabling HARQ feedback has marginal impact on peak data rates for larger repetitions. This is the simplest way for DCI to indicate HARQ feedback enabling/disabling with re-interpreted “Repetition number” field.

Another reasonable way for DCI to indicate HARQ feedback enabling/disabling is re-interpreting “HARQ-ACK resource” field. If “HARQ-ACK resource” field ≠ ‘0000’, the network can indicate or override RRC disabled configuration with enabled HARQ feedback for the HARQ process. In other words, if “HARQ-ACK resource” field = ‘0000’, then RRC configuration with RRC per-HARQ process bitmap is used or disabled HARQ feedback for the HARQ process is used if no RRC per-HARQ process bitmap configured. This seems a reasonable way considering that if the HARQ feedback is disabled, then the “HARQ-ACK resource” field becomes redundant since HARQ feedback is not transmitted.

Proposal 3: Support DCI-based overridden indication via re-interpreted fields (e.g. “Repetition number” field, “HARQ-ACK resource” field).

The restriction of the scheduling with “HARQ-ACK resource” field = ‘0000’ for transmission of HARQ feedback, could be further mitigated in case combining with “Repetition number” field, where HARQ feedback is enabled if repetitions are 16 or higher as follows:

- If “Repetition number” field indicates 16 repetitions or higher, HARQ feedback for the HARQ process is enabled (for any value of “HARQ-ACK resource” field)

- If “Repetition number” field indicates fewer than 16 repetitions, and “HARQ-ACK resource” field ≠ ‘0000’, HARQ feedback for the HARQ process is enabled

- If “Repetition number” field indicates fewer than 16 repetitions, and “HARQ-ACK resource” field = ‘0000’, RRC configuration for the HARQ process with RRC per-HARQ process bitmap is used or HARQ feedback for the HARQ process is disabled if no RRC per-HARQ process bitmap configured

Hence, we make the following proposal:

Proposal 4: DCI with re-interpreted fields to override RRC configuration to enable HARQ feedback for DL transmission per HARQ process

- If “Repetition number” field is ‘0100’ or above – i.e. 16 repetitions or higher

- If “HARQ-ACK resource” field ≠ ‘0000’.

Note: If “Repetition number” field indicates fewer than 16 repetitions, and “HARQ-ACK resource” field = ‘0000’, RRC configuration for the HARQ process with RRC per-HARQ process bitmap is used or HARQ feedback for the HARQ process is disabled if no RRC per-HARQ process bitmap configured.

2.3 HARQ feedback for scheduling multiple TB

Agreement (RAN1 109e) For IoT NTN, further study the potential issues due to enabling/disabling on HARQ feedback for downlink transmission - Issue A: SPS PDSCH - Issue B: (N)PDSCH/(N)PDCCH scheduling restriction - Issue C: HARQ feedback for scheduling multiple TB - Issue D: HARQ bundling for eMTC HD-FDD - Issue F: NPRACH capacity - Issue G: Serving cell/satellite change during data transfer (FFS: for eMTC and/or NB-IoT) - Other issues are not excluded Note: The “Issues” in common for eMTC and NB-IoT can be separately discussed.

Multiple TB is one optional feature considering unicast and multicast for Rel-16 NB-IoT. There is no multi-TB HARQ feedback in SC-MTCH. There is multi-TB HARQ feedback in unicast, with consideration of HARQ ACK bundling.

If higher layer parameter npdsch-MultiTB-Config is enabled and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI, the Number of scheduled TB for Unicast is 1 bit, where value 0 indicates a single TB is scheduled and value 1 indicates multiple TB are scheduled.

For NB-IoT, if Number of scheduled TB for Unicast is present and multiple TB are scheduled, the multi-TB (two TBs) are scheduled together without “HARQ process number” field in DCI. However, as specicfied in TS 36.213 of Section 16.4.1.5, If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB, otherwise, HARQ process ID of 0 shall be assumed.

TS 36.213 Section 16.4.1.5 If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB, otherwise, HARQ process ID of 0 shall be assumed.

There are two ways to decide enabling or disabling HARQ feedback of multi-TB:

- decide enabling or disabling HARQ feedback of multi-TB respectively on the HARQ process ID of 0 for the first TB and HARQ process ID of 1 for the second TB.

- decide enabling or disabling HARQ feedback of multi-TB only on HARQ process ID of 0 for all TBs.

Observation 4: There are two ways to decide enabling or disabling HARQ feedback of multi-TB:

- decide enabling or disabling HARQ feedback of multi-TB respectively on the HARQ process ID of 0 for the first TB and HARQ process ID of 1 for the second TB.

- decide enabling or disabling HARQ feedback of multi-TB only on HARQ process ID of 0 for all TBs.

If way (1) is utilized, there will involve more discussion on the cases, when one TB is enabled and the other TB is disabled, and how DCI solution can be done for different TBs, and so on. While way (2) can be a better way for progress.

Proposal 5: HARQ feedback enabling/disabling for multiple TB only based on HARQ process ID of 0 for all TBs.

3 Conclusion

In this contribution, the following proposals were made

Observation 1: Peak data rate reduction becomes lower with higher repetitions used in NB-IoT NTN scenarios with HARQ feedback always enabled.

Observation 2: Both RRC per-HARQ process bitmap solution and DCI solution are new indication alternatives.

Observation 3: When both RRC per-HARQ process bitmap solution and DCI solution are not configured, the legacy procedure where all HARQ processes with feedback enabled should be supported.

Proposal 1: For NB-IoT, DCI-based overridden indication is supported if additional higher layer parameter is enabled by UE-specific RRC signalling based on UE capability.

Proposal 2: DCI-based overridden indication only applied to semi-statically HARQ feedback disabled processes.

Proposal 3: Support DCI-based overridden indication via re-interpreted fields (e.g. “Repetition number” field, “HARQ-ACK resource” field).

Proposal 4: DCI with re-interpreted fields to override RRC configuration to enable HARQ feedback for DL transmission per HARQ process

- If “Repetition number” field is ‘0100’ or above – i.e. 16 repetitions or higher

- If “HARQ-ACK resource” field ≠ ‘0000’.

Note: If “Repetition number” field indicates fewer than 16 repetitions, and “HARQ-ACK resource” field = ‘0000’, RRC configuration for the HARQ process with RRC per-HARQ process bitmap is used or HARQ feedback for the HARQ process is disabled if no RRC per-HARQ process bitmap configured.

Observation 4: There are two ways to decide enabling or disabling HARQ feedback of multi-TB:

- decide enabling or disabling HARQ feedback of multi-TB respectively on the HARQ process ID of 0 for the first TB and HARQ process ID of 1 for the second TB.

- decide enabling or disabling HARQ feedback of multi-TB only on HARQ process ID of 0 for all TBs.

Proposal 5: HARQ feedback enabling/disabling for multiple TB only based on HARQ process ID of 0 for all TBs.

4 References

- RP-223519, Moderator (MediaTek), Revised WID on IoT NTN enhancements, 12-16 December, 2022

- TR 36.763, Release 17

- R1-2301804, Lenovo, FLS#2 on disabling of HARQ feedback for IoT NTN

- R1-1912125, MediaTek, Delay-tolerant re-transmission mechanisms in NR-NTN, RAN1#99, November 2019

5 Appendix

TS 36.213 Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

TS 36.211 Table 16.4.1.3-2: Number of repetitions () for NPDSCH.