3GPP TSG RAN WG1 #112bis-e								   R1-2303359

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.1.1

Source: 	MediaTek Inc.

Title:	Unified TCI framework extension for multi-TRP

Document for: 	Discussion and Decision

Introduction

In this contribution, we will share our views on following issues for unified TCI extension:

- TCI state indication and activation

- TCI selection from the indicated joint/DL/UL TCI states

- UL power control for UL MTRP

Discussion

TCI state indication and activation

The following issues are discussed in this session:

- TCI state indication/activation and corresponding UE behaviours for S-DCI based MTRP

- Common TCI state ID activation/update for a set of CCs configured in the same CC list

TCI state indication/activation and corresponding UE behaviours for S-DCI based MTRP

In RAN1#111 [1], the following was agreed for S-DCI based MTRP:

Agreement from RAN1#111 On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

According to the agreement, for S-DCI based MTRP, the existing TCI field should be able to indicate joint/DL/UL TCI state(s) for one of the two TRPs or both TRPs. To our interpretation, the followings should be supported for TCI state indication/activation:

- For joint DL/UL TCI mode, a full-set or any sub-set of {1st joint TCI state, 2nd joint TCI state} can be mapped to a TCI codepoint of the existing TCI field. One example is shown in Table 1.

Table 1. Example of TCI state activation in joint DL/UL TCI mode

| TCI codepoint | 1st Joint TCI State | 2nd Joint TCI State |
| --- | --- | --- |
| 000 | Joint TCI state ID = a | Joint TCI state ID = b |
| 001 | Joint TCI state ID = c |  |
| 010 |  | Joint TCI state ID = d |
| 011 | Joint TCI state ID = e | Joint TCI state ID = f |

- For separate DL/UL TCI mode, a full-set or any sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} can be mapped to a TCI codepoint of the existing TCI field). One example is shown in Table 2.

Table 2. Example of TCI state activation in separate DL/UL TCI mode

| TCI codepoint | 1st DL TCI State | 1st UL TCI State | 2nd DL TCI State | 2nd UL TCI State |
| --- | --- | --- | --- | --- |
| 000 | DL TCI state ID = a | UL TCI state ID = m | DL TCI state ID = b | UL TCI state ID = n |
| 001 | DL TCI state ID = c | UL TCI state ID = o |  |  |
| 010 |  |  | DL TCI state ID = d | UL TCI state ID = p |
| 011 | DL TCI state ID = e |  | DL TCI state ID = f |  |
| 100 |  | UL TCI state ID = q | DL TCI state ID = g | UL TCI state ID = r |
| 101 | DL TCI state ID = h |  |  | UL TCI state ID = s |

When a TCI activation command activates a sub-set of {1st joint TCI state, 2nd joint TCI state} or {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} to a TCI codepoint, UE should be able to differentiate each activated joint/DL/UL TCI state is specific to the 1st or 2nd joint/DL/UL TCI state of the TCI codepoint.

Since beam indication DCI may not indicate TCI states for both TRPs, there are two possible UE behaviours if UE receives a beam indication DCI indicating TCI state(s) only for one TRP:

- Alt1: UE switches from MTRP to STRP operation for all channels/signals that apply the indicated TCI states

- Alt2: UE applies the newly indicated TCI state(s) for the one TRP and keeps the current applied TCI state(s) for another TRP

In previous agreements made in this AI, NW can use either RRC or DCI signaling to select the indicated TCI states for target channels and switching between MTRP to STRP for each channel can be individually done by the RRC or DCI-based TCI selection. It would be functionally redundant if switching between MTRP to STRP operation can be also done through TCI state indication. In this sense, we see Alt2 is a better choice, and UE shall maintain the indicated TCI state(s) even it is not included in the TCI states mapped to the indicated TCI codepoint.

Proposal 1: For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {1st  joint TCI state, 2nd joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

- If the UE receives a TCI codepoint mapped with a full-set of {1st joint TCI state, 2nd joint TCI state}, the UE applies the corresponding joint TCI states in the full-set

- If the UE receives a TCI codepoint mapped with a sub-set of {a first joint TCI state, a second joint TCI state}, the UE applies the corresponding joint TCI state in the sub-set, and keeps the currently applied joint TCI state not in the sub-set

- Note: When a TCI activation command activates a sub-set of {1st joint TCI state, 2nd joint TCI state} to a TCI codepoint, UE should be able to differentiate each activated joint TCI state is specific to the 1st or 2nd joint TCI state of the TCI codepoint. How to achieve above is up to RAN2 design.

Proposal 2: For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

- If the UE receives a TCI codepoint mapped with a full-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE applies the corresponding DL/UL TCI states in the full-set.

- If the UE receives a TCI codepoint mapped with a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE applies the corresponding DL/UL TCI state(s) in the sub-set, and keeps the currently applied DL/UL TCI state(s) not in the sub-set

- Note: When a TCI activation command activates a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} to a TCI codepoint, UE should be able to differentiate each activated DL/UL TCI state is specific to the 1st or 2nd DL/UL TCI state of the TCI codepoint. How to achieve above is up to RAN2 design.

Common TCI state ID activation/update for a set of CCs configured in the same CC list

In RAN1#111, there was a discussion on whether to support the follow two cases in unified TCI extension:

- Whether a CC list can be comprised of a mix of STRP CC(s) and MTRP CC(s)

- Whether a CC list can be comprised of a mix of S-DCI based MTRP CC(s) and M-DCI based MTRP CC(s)

In Rel-17 unified TCI framework, multiple CCs in the same band can be configured in a CC list (i.e., simultaneousTCI-UpdateListX), and one TCI state activation/indication command can apply to all the CCs in the same CC list to reduce signaling overhead. Since Rel-17 unified TCI framework is tailored for STRP operation, the same signaling can be used for all CCs configured with STRP operation. However, in unified TCI extension for MTRP, the TCI state activation/indication for different MTRP modes are not common, and it is also quite different from the TCI state activation/indication for STRP. It would be a problem how to achieve common TCI state activation/indication by a single command.

Table 3. Comparison of TCI state activation/indication between different TRP modes

| TRP Mode | TCI state activation | TCI state indication |
| --- | --- | --- |
| STRP operation | A MAC-CE maps a joint TCI state or a full/sub-set of DL and UL TCI states to TCI codepoint(s) | A DCI format 1_1/1_2 indicates a TCI codepoint |
| S-DCI based MTRP operation | A MAC-CE maps a full/sub-set of 2 joint TCI states or a full/sub-set of two pairs of DL and UL TCI states to TCI codepoint(s) | A DCI format 1_1/1_2 indicates a TCI codepoint |
| M-DCI based MTRP operation | A MAC-CE maps a joint TCI state or a full/sub-set of DL and UL TCI states to TCI codepoint(s) specific to a coresetPoolIndex value included in the same MAC-CE | A DCI format 1_1/1_2 indicates a TCI codepoint specific to a coresetPoolIndex value |

In our view, there are use cases for a CC list comprising of a mix of STRP CC(s) and MTRP CC(s), however, all the MTRP CC(s) should operate in either S-DCI or M-DCI mode since it may not be practical to deploy ideal and non-ideal backhauls at the same time. Regarding how to support common TCI state activation and indication for STRP CC(s) and MTRP CC(s) in a same CC list can be further discussed.

Proposal 3: On unified TCI framework extension, support the followings:

- A CC list (simultaneousTCI-UpdateListX) for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP

- A CC list (simultaneousTCI-UpdateListX) for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP

- FFS: How to support common TCI state ID activation/update for above two cases

TCI selection from the indicated joint/DL/UL TCI states

In Rel-17 unified TCI framework, whether a target channel/RS apply the unified TCI state is (semi-statically) fixed, which is either defined in specification (i.e., always apply the indicated TCI state) or configured according to RRC configuration. In Rel-18 unified TCI extension, two joint/DL/UL TCI states can be indicated simultaneously to a BWP, thus it’s necessary to further specify how to determine which indicated joint/DL/UL TCI state(s) shall be applied to a target channel/signal. In our view, TCI selection in Rel-18 unified TCI extension should follow the same principle as in Rel-17 to avoid unnecessary signaling overhead and complexity. However, we also see that dynamic switching between STRP/MTRP or different TRPs is still necessary to enable flexible scheduling for some channels/RSs (e.g., dynamically scheduled PDSCH/PUSCH).

Proposal 4: TCI selection in Rel-18 unified TCI extension should follow the same principle as in Rel-17 to avoid unnecessary signaling overhead and complexity, however, dynamic switching between STRP/MTRP or different TRPs is still necessary to enable flexible scheduling for some channels/RSs (e.g., dynamically scheduled PDSCH/PUSCH).

Based on above principle, TCI selection for the following channels/signals are discussed in this session:

- (S-DCI) PDSCH scheduled/activated by DCI 1_1/1_2 if the [TCI selection field] is present

- (S-DCI) PDSCH scheduled/activated by DCI 1_1/1_2 if the [TCI selection field] is not present

- (S-DCI) PUSCH scheduled/activated by DCI 0_1/0_2 (including DG and Type2 CG)

- (S-DCI) Type1 CG-PSUCH

- (S-DCI) AP-CSI-RS for CSI/BM

- (S-DCI) SRS for CB/NCB/AS and AP SRS for BM

- (M-DCI) PUCCH

- (M-DCI) SRS for CB/NCB/AS and AP-SRS for BM

(S-DCI) PDSCH scheduled/activated by DCI 1_1/1_2 if the [TCI selection field] is present

In RAN1#111, for S-DCI based MTRP operation, it was agreed to introduce a new DCI field (i.e., [TCI selection field]) in DCI 1_1/1_2 for TCI selection of PDSCH reception scheduled/activated by DCI 1_1/1_2, where the presence of the new DCI field is RRC-configurable. However, it is still unclear that the presence of the new DCI field is configured per CORESET, BWP, or serving cell. We have concern on per CORESET configuration since it will lead to complicated UE behaviors especially for the default behaviors before UE decodes the scheduling/activation DCI. For per BWP configuration or per serving cell configuration, we are open to the two alternatives.

Proposal 5: On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field]) in DCI format 1_1/1_2 is configured either per BWP or per serving cell

In RAN1#112, RAN1 has agreed to use the codepoints “00”, “01”, and “10” of the [TCI selection field] in DCI 1_1/1_2, and whether/how to use the codepoints “11” of the [TCI selection field] is still under FFS. In our view, current agreed behaviors already can support all the S-DCI based MTRP schemes, thus we fail to see the need to use the codepoint “11” for new functionalities not supported in Rel-17, e.g., switching the applying order of the first and second joint/DL TCI states or dynamic switching between different S-DCI based MTRP schemes.

Table 5. UE behaviors corresponding to different codepoints of the TCI selection field

| Codepoint of the TCI selection field | UE behavior |
| --- | --- |
| 00 | The UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 |
| 01 | The UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 |
| 10 | The UE shall apply both the 1st and the 2nd indicated joint/DL TCI states to the scheduled/activated PDSCH reception |
| 11 | FFS: Whether and how to use the codepoint "11" of the [TCI selection field] |

Proposal 6: On unified TCI framework extension for S-DCI based MTRP, the codepoint “11” of the [TCI selection field] is reserved

(S-DCI) PDSCH scheduled/activated by DCI 1_1/1_2 if the [TCI selection field] is not present

In RAN1#112, for S-DCI based MTRP operation, several alternatives were agreed for TCI selection of PDSCH reception scheduled/activated by DCI 1_1/1_2 if the [TCI selection field] is not present in DCI 1_1/1_2. For Alt2, it is unclear how to enable MTRP operation if the [TCI selection field] is not present in DCI 1_1/1_2. For Alt3A, MTRP operation is not possible if PDCCH is not configured to apply both indicated joint/DL TCI states. Regarding Alt4, we have concern on whether at most 8 codepoints of the existing TCI field can support both TCI state update for all channels/signals and TCI state selection for PDSCH reception, and we fail to see the need to use another DCI field to support the same functionality as the [TCI selection field]. For Alt1 and Alt3, we are open to these two alternatives. On the other hand, since which indicated joint/DL TCI state(s) is applied can be determined semi-statically/statically based on Alt1/Alt3, UE should be able to buffer the received signal according to Alt1 or Alt3 before the DCI is decoded.

Proposal 7: On unified TCI framework extension for S-DCI based MTRP, when the [TCI selection field] is absent in DCI format 1_1/1_2, support either Alt1 or Alt3 for PDSCH reception scheduled/activated by DCI format 1_1/1_2 regardless the scheduling offset is larger or smaller than a threshold

(S-DCI) PUSCH scheduled/activated by DCI 0_1/0_2 (including DG and Type2 CG)

In RAN1#112, RAN1 agreed to use the existing SRS resource indicator in DCI 0_1/0_2 for TCI selection of PUSCH transmissions in S-DCI based MTRP, where the UE behaviors corresponding to the codepoints “00”, “01”, “10”, and “11” of the existing SRS resource indicator in DCI 1_1/1_2 are defined as follows:

Table 6. UE behaviors corresponding to different codepoints of the SRS resource set indicator

| Codepoint of SRS resource set indicator | UE behavior |
| --- | --- |
| 00 | The UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) |
| 01 | The UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) |
| 10 | For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) FFS: For SDM and SFN based PUSCH Tx scheme |
| 11 |  |

Regarding TDM based PUSCH Tx scheme, based on current agreement, there is a fixed mapping between the 1st/2nd indicate joint/UL TCI state and the 1st/2nd SRS resource set regardless of the codepoints “10” and “11”, and different indicated joint/UL TCI states still can be applied to a PUSCH Tx occasion according to the associated   SRS resource set that can be switched by the codepoints “10” and “11”. In our view, the same behavior can be used for SDM/SFN based PUSCH Tx schemes regardless of codepoints “10” and/or “11”, i.e., the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource in the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource in the second SRS resource set. Note that the adoption of the codepoints “10” and/or “11” is discussed in STxMP AI.

Proposal 8: On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

- For SDM and SFN based PUSCH Tx schemes, if the DCI format 0_1/0_2 indicates codepoint "10" and/or “11” for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource corresponding to the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) corresponding to the SRS port(s) corresponding to the indicated SRS resource in the second SRS resource set.

- Note: Whether the codepoints “10” and/or “11” of the existing SRS resource set indicator is/are supported is discussed separately in STxMP AI

(S-DCI) Type1 CG-PUSCH

For Type1 CG, we see similar RRC configuration used for PUCCH can be provided per Type1 CG configuration for TCI selection, i.e., to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to the corresponding CG-PUSCH transmission.

Proposal 9: On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided per Type1 CG configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding CG-PUSCH transmission

(S-DCI) AP-CSI-RS for CSI/BM

In Rel-17 unified TCI framework, whether an AP-CSI-RS resource set (for CSI/BM) follows the indicated joint/DL TCI state is configured per CSI-AssociatedReportConfigInfo in CSI-AperiodicTriggerState, i.e., when qcl-info is absent for aperiodic CSI RS, the UE shall use QCL information included in the indicated joint/DL TCI state.

In Rel-18 unified TCI extension for MTRP, as we mentioned above, TCI selection should follow the same principle as in Rel-17 to avoid unnecessary signaling overhead and complexity. In this sense, we prefer to use RRC configuration in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState to inform that the UE shall apply the first or the second indicated joint/DL TCI state to an AP-CSI-RS resource set, which simply extends the Rel-17 design (i.e., based on RRC configuration) to enable TCI selection among two indicated joint/DL TCI states. Note that even the TCI selection is done by RRC, the same AP-CSI-RS resource still can be associated with different TRPs through different AP trigger-states, and NW can trigger one of the AP trigger-states dynamically through DCI format 0_1/0_2.

In RAN1#112, there were proposals that use a fixed rule to determine which indicated joint/DL TCI state should be applied to some special cases, e.g., AP-CSI-RS for enhanced group-based beam reporting or NCJT CSI measurement. In our view, for the case if no explicit RRC configuration (including qcl-info and the RRC configuration for TCI selection) is provided, the indicated joint/DL TCI states can be mapped to the resource sets or resource groups with fixed rules as follows:

- For enhanced group-based beam reporting, if qcl-info/qcl-info2 for an AP-CSI-RS resource set configured is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState and no RRC configuration is provided to the AP-CSI-RS resource set for TCI selection, the UE shall apply the first indicated joint/DL TCI state to the first CMR set and the second indicated joint/DL TCI state to the second CMR set.

- For an AP-CSI-RS resource set configured with two Resource Groups (for NCJT CSI), if qcl-info for an AP-CSI-RS resource set configured is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State and no RRC configuration is provided to the AP-CSI-RS resource set for TCI selection, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2.

Proposal 10: On unified TCI framework extension for S-DCI based MTRP, if qcl-info/qcl-info2 for an aperiodic CSI-RS resource set configured for CSI/BM is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set

- For enhanced group-based beam reporting, if above RRC configuration is not provided to the first and second aperiodic CSI-RS resource sets, the UE shall apply the first indicated joint/DL TCI state to the first aperiodic CSI-RS resource set and the second indicated joint/DL TCI state to the second aperiodic CSI-RS resource set.

- For NCJT CSI measurement, if above RRC configuration is not provided to the aperiodic CSI-RS resource set configured with two Resource Groups, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2.

(S-DCI) SRS for CB/NCB/AS and AP-SRS for BM

Similar to CSI-RS, for an SRS resource set configured with followUnifiedTCIstateSRS = “enable”, we also prefer to use RRC configuration in the SRS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to an AP-CSI-RS resource set. However, for the case if two SRS resource sets for CB/NCB are configured and the RRC configuration for TCI selection is not provided for them, a fixed rule can be used.

Proposal 11: On unified TCI framework extension for S-DCI based MTRP, if followUnifiedTCIstateSRS = “enable” is provided to a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, an RRC configuration can be provided to the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the aperiodic CSI-RS resource set

- If two SRS resource sets for CB/NCB are configured and above RRC configuration is not provided to the two SRS resource sets for CB/NCB, the UE shall apply the first indicated joint/UL TCI state to the 1st SRS resource set for CB/NCB and the second indicated joint/UL TCI state to the 2nd SRS resource set for CB/NCB.

(M-DCI) PUCCH

In RAN1#112, the following options are identified for PUCCH in M-DCI based MTRP:

- Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission

- Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted.

- Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted.

For Option 1, we see it is straightforward to directly configure a coresetPoolIndex value in a corresponding PUCCH resource (or PUCCH resource group), and the UE shall apply the indicated joint/UL TCI state specific to the configured coresetPoolIndex value. However, this will cause a different RRC design compared with S-DCI based MTRP. To unify the potential RRC design for S-DCI and M-DCI based MTRP, we prefer Option 2 that introduces a similar RRC configuration as S-DCI based MTRP to inform that the UE shall apply the first or the second indicated joint/UL TCI state to a PUCCH resource/resource group, where the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

For PUCCH transmission with HARQ-ACK feedback, whether to introduce a special rule can be further discussed. However, it is not necessary in our view since the DCI with DL assignment can dynamically select a PUCCH resource associated with the same coresetPoolIndex value through the PUCCH resource indicator. We also don’t think we need to introduce any enhancement to PUCCH transmission with LLR triggered for TRP-specific BFR.

Proposal 12: On unified TCI framework extension for M-DCI based MTRP, support only Option 2 for PUCCH transmission

(M-DCI) SRS for CB/NCB/AS and AP-SRS for BM

Similar to SRS in S-DCI based MTRP, for an SRS resource set configured with followUnifiedTCIstateSRS = “enable”, we also prefer to use RRC configuration in the SRS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to an AP-CSI-RS resource set, where the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

However, for the case if two SRS resource sets for CB/NCB are configured for STxMP PUSCH+PUSCH and the RRC configuration for TCI selection is not provided for them, a fixed rule can be used. In RAN1#111 and RAN1#112, the following was agreed in AI 9.1.4.1:

Agreement from AI 9.1.4.1 in RAN1#112 - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set Agreement from AI 9.1.4.1 in RAN1#112 - Among the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH, the SRS resource set with lower set ID is the first SRS resource set.

In our view, these associations introduced in AI 9.1.4.1 can be reused for TCI selection for SRS for CB/NCB and Type1 CG w/o additional/redundant signaling overhead. Based on the association between SRS resource set and coresetPoolIndex value defined in AI 9.1.4.1, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the SRS resources set associated with the same coresetPoolIndex value if the SRS resources set is configured to follow the indicated joint/UL TCI state by followUnifiedTCIStateSRS.

Proposal 13: On unified TCI framework extension for S-DCI based MTRP, if followUnifiedTCIstateSRS = “enable” is provided to a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, an RRC configuration can be provided to the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the aperiodic CSI-RS resource set, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- If two SRS resource sets for CB/NCB are configured for STxMP PUSCH+PUSCH and above RRC configuration is not provided to the two SRS resource sets for CB/NCB, the UE shall apply the indicated joint/UL TCI state specific to coresetPoolIndex value 0 to the 1st SRS resource set for CB/NCB and the indicated joint/UL TCI state specific to coresetPoolIndex value 1 to the 2nd SRS resource set for CB/NCB.

(M-DCI) Other channels/signals

For the following channel and signal in M-DCI based MTRP, we prefer to discuss after RAN1 concludes on the TCI selection scheme for PUCCH/SRS.

- Type1 CG-PUSCH

- AP CSI-RS for CSI/BM

UL Power control for UL MTRP schemes

In RAN1#112, the following was agreed for UL PC for UL MTRP schemes:

Agreement from RAN1#112 On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

Since RAN4 has replied to RAN1 LS on UE power limitation for STxMP in FR2 in this meeting (R1-2302265), RAN1 can start to discuss the max Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) for STxMP.

Based on the RAN4 reply, both per-UE and per-panel power limitation should be applied to a same UE for STxMP. In current UL PC procedure in TS 38.213, UL Tx power is determined per PUSCH/PUCCH transmission occasion based on single UE-configured maximum output power. Take PUSCH transmission as an example:

[dBm]

However, for S-DCI based SDM/SFN STxMP, UE needs to determine two UL Tx power according to the indicated joint/UL TCI states for a same Tx occasion, where each UL Tx power is applied to a corresponding UE panel. Meanwhile, to limit UL Tx power per panel for STxMP, two UE-configured maximum output power maybe needed at least for S-DCI based STxMP PUSCH/PUCCH. One potential solution is introducing per-indicated-TCI-state UL Tx power determination (which has been agreed in RAN1#112) and UE-configured maximum output power for the case if two indicated joint/UL TCI states are applied to a same PUSCH/PUCCH Tx occasion(s), i.e., the UE shall determine UL Tx power individually for each indicated joint/UL TCI state based on a corresponding UE-configured maximum output power, the UL PC parameter setting for PUSCH/PUCCH/SRS included in the indicated joint/UL TCI state, if any, and the PL-RS included in the indicated joint/UL TCI state. Each determined UL Tx power is applied to the PUSCH/PUCCH antenna port(s) associated with the corresponding indicated joint/UL TCI state.

Proposal 14: On unified TCI framework extension, study whether and how to introduce per-indicated-TCI-state UL Tx power determination at least for S-DCI based STxMP PUSCH/PUCCH, i.e., the UE shall determine UL Tx power individually for each indicated joint/UL TCI state based on a corresponding UE-configured maximum output power, the UL PC parameter setting for PUSCH/PUCCH/SRS included in the indicated joint/UL TCI state, if any, and the PL-RS included in the indicated joint/UL TCI state.

- Each determined UL Tx power is applied to the PUSCH/PUCCH antenna port(s) that applies the corresponding indicated joint/UL TCI state

For M-DCI based PUSCH+PUSCH STxMP, UE can determine UL Tx power for each PUSCH Tx occasion based on current UL PC procedure in TS 38.213, thus per-panel power limitation can be guaranteed by the legacy UE-configured maximum output power. However, to meet the per-UE power limitation, M-DCI based PUSCH+PUSCH STxMP should be also considered as a part of prioritizations for transmission power reductions in current spec of TS 38.213. In our view, for the case if the starting symbols of two overlapped PUSCH Tx occasions are not aligned, the UE shall allocate the power for the earlier PUSCH Tx occasion first, regardless of the priorities of the overlapped PUSCH Tx occasions. Such behavior is already supported by current specification since the UE does not include power for transmissions starting after the symbol of transmission occasion i when the UE determines a total Tx power for serving cells in a frequency range in a symbol of transmission occasion i. If the starting symbols of two overlapped PUSCH Tx occasions are aligned, the UE allocates the power for the PUSCH with higher priority. On how to define the priority between two overlapped PUSCH Tx occasions, this can be further discussed.

Proposal 15: M-DCI based PUSCH+PUSCH STxMP should be taken into account of the clause of TS 38.213 for prioritizations for transmission power reductions

- FFS: Enhancement to priority rule for two PUSCH Tx occasions in the same serving cell

Conclusion

In this contribution, extension of unified TCI framework for MTRP operation was discussed. Based on the discussion in the previous sections, we made the following proposals and observations:

TCI state update and activation

Proposal 1: For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {1st  joint TCI state, 2nd joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

- If the UE receives a TCI codepoint mapped with a full-set of {1st joint TCI state, 2nd joint TCI state}, the UE applies the corresponding joint TCI states in the full-set

- If the UE receives a TCI codepoint mapped with a sub-set of {a first joint TCI state, a second joint TCI state}, the UE applies the corresponding joint TCI state in the sub-set, and keeps the currently applied joint TCI state not in the sub-set

- Note: When a TCI activation command activates a sub-set of {1st joint TCI state, 2nd joint TCI state} to a TCI codepoint, UE should be able to differentiate each activated joint TCI state is specific to the 1st or 2nd joint TCI state of the TCI codepoint. How to achieve above is up to RAN2 design.

Proposal 2: For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

- If the UE receives a TCI codepoint mapped with a full-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE applies the corresponding DL/UL TCI states in the full-set.

- If the UE receives a TCI codepoint mapped with a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE applies the corresponding DL/UL TCI state(s) in the sub-set, and keeps the currently applied DL/UL TCI state(s) not in the sub-set

- Note: When a TCI activation command activates a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} to a TCI codepoint, UE should be able to differentiate each activated DL/UL TCI state is specific to the 1st or 2nd DL/UL TCI state of the TCI codepoint. How to achieve above is up to RAN2 design.

Proposal 3: On unified TCI framework extension, all CCs in a same CC list (simultaneousTCI-UpdateListX) for common TCI state ID activation/update can be configured with either STRP operation, S-DCI based MTRP operation, or M-DCI based MTRP operation

- FFS: Whether to increase more than 4 CC lists in unified TCI extension for MTRP

TCI selection from the indicated joint/DL/UL TCI states

Proposal 4: TCI selection in Rel-18 unified TCI extension should follow the same principle as in Rel-17 to avoid unnecessary signaling overhead and complexity, however, dynamic switching between STRP/MTRP or different TRPs is still necessary to enable flexible scheduling for some channels/RSs (e.g., dynamically scheduled PDSCH/PUSCH).

Proposal 5: On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field]) in DCI format 1_1/1_2 is configured either per BWP or per serving cell

Proposal 6: On unified TCI framework extension for S-DCI based MTRP, the codepoint “11” of the [TCI selection field] is reserved

Proposal 7: On unified TCI framework extension for S-DCI based MTRP, when the [TCI selection field] is absent in DCI format 1_1/1_2, support either Alt1 or Alt3 for PDSCH reception scheduled/activated by DCI format 1_1/1_2 regardless the scheduling offset is larger or smaller than a threshold

Proposal 8: On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

- For SDM and SFN based PUSCH Tx schemes, if the DCI format 0_1/0_2 indicates codepoint "10" and/or “11” for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource corresponding to the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) corresponding to the SRS port(s) corresponding to the indicated SRS resource in the second SRS resource set.

- Note: Whether the codepoints “10” and/or “11” of the existing SRS resource set indicator is/are supported is discussed separately in STxMP AI

Proposal 9: On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided per Type1 CG configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding CG-PUSCH transmission

Proposal 10: On unified TCI framework extension for S-DCI based MTRP, if qcl-info/qcl-info2 for an aperiodic CSI-RS resource set configured for CSI/BM is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set

- For enhanced group-based beam reporting, if above RRC configuration is not provided to the first and second aperiodic CSI-RS resource sets, the UE shall apply the first indicated joint/DL TCI state to the first aperiodic CSI-RS resource set and the second indicated joint/DL TCI state to the second aperiodic CSI-RS resource set.

- For NCJT CSI measurement, if above RRC configuration is not provided to the aperiodic CSI-RS resource set configured with two Resource Groups, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2.

Proposal 11: On unified TCI framework extension for S-DCI based MTRP, if followUnifiedTCIstateSRS = “enable” is provided to a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, an RRC configuration can be provided to the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the aperiodic CSI-RS resource set

- If two SRS resource sets for CB/NCB are configured and above RRC configuration is not provided to the two SRS resource sets for CB/NCB, the UE shall apply the first indicated joint/UL TCI state to the 1st SRS resource set for CB/NCB and the second indicated joint/UL TCI state to the 2nd SRS resource set for CB/NCB.

Proposal 12: On unified TCI framework extension for M-DCI based MTRP, support only Option 2 for PUCCH transmission

Proposal 13: On unified TCI framework extension for S-DCI based MTRP, if followUnifiedTCIstateSRS = “enable” is provided to a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, an RRC configuration can be provided to the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the aperiodic CSI-RS resource set, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- If two SRS resource sets for CB/NCB are configured for STxMP PUSCH+PUSCH and above RRC configuration is not provided to the two SRS resource sets for CB/NCB, the UE shall apply the indicated joint/UL TCI state specific to coresetPoolIndex value 0 to the 1st SRS resource set for CB/NCB and the indicated joint/UL TCI state specific to coresetPoolIndex value 1 to the 2nd SRS resource set for CB/NCB.

UL Power control for UL MTRP schemes

Proposal 14: On unified TCI framework extension, study whether and how to introduce per-indicated-TCI-state UL Tx power determination at least for S-DCI based STxMP PUSCH/PUCCH, i.e., the UE shall determine UL Tx power individually for each indicated joint/UL TCI state based on a corresponding UE-configured maximum output power, the UL PC parameter setting for PUSCH/PUCCH/SRS included in the indicated joint/UL TCI state, if any, and the PL-RS included in the indicated joint/UL TCI state.

- Each determined UL Tx power is applied to the PUSCH/PUCCH antenna port(s) that applies the corresponding indicated joint/UL TCI state

Proposal 15: M-DCI based PUSCH+PUSCH STxMP should be considered in the clause of TS 38.213 for prioritizations for transmission power reductions when the UE allocates Tx power to PUSCH/PUCCH/ PRACH/SRS transmissions so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i.

- FFS: Priority for two PUSCH Tx occasions in the same serving cell and the two PUSCH Tx occasions have the same starting symbol

Reference

- 3GPP RAN1 #112, “Chairman’s Notes RAN1 #112”, February 2023

- 3GPP RAN1 #111, “Chairman’s Notes RAN1 #111”, November 2022

- 3GPP RAN1 #110bis-e, “Chairman’s Notes RAN1 #110bis-e”, October 2022

- 3GPP RAN1 #110, “Chairman’s Notes RAN1 #110”, August 2022

- 3GPP RAN1 #109e, “Chairman’s Notes RAN1 #109-e”, May 2022