3GPP TSG RAN WG1 #112bis-e		     R1-2303360

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.1.2

Source: 	MediaTek Inc.

Title:	UL Tx Timing Management for MTRP

Document for: 	Discussion and Decision

Introduction

In this contribution, we will share our views on the following issues:

- UE implementation issue

- Association between TAG and UL transmission

- RACH enhancement for both intra-cell and inter-cell MTRP

- RACH enhancement for intra-cell MTRP

- RACH enhancement for inter-cell MTRP

Discussion

UE implementation issue

In RAN1#111 [1], the followings were agreed:

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e: - Alt 1: Introducing a time gap X between two UL transmissions associated with two different TA values - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused - FFS: How X is determined - Alt 2: Reduce the overlapping duration of one of the two UL transmissions - Alt 3: Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap - Other alternatives are not precluded TBD: how to capture the downselected alternative(s) in the specifications in case specification impact is deemed needed.

Figure 1. Three alternatives to handle UL transmissions applying two Tas if UE doesn’t support STxMP

For the case if a UE doesn’t support UL STxMP, the UE is not able to handle the UL transmissions if they are overlapped after applied two different TA values, respectively. Meanwhile, the UE also need a processing time to perform two the UL transmissions associated with two different TA values; thus a switching gap X is necessary. The value of X can be a UE capability and 140 𝜇𝑠 should be candidate value. On definition of the switching gap X, we prefer to use an absolute time since the time offset between two UL transmissions after applying two different TA values may not be an integer of symbols. In summary, we prefer Alt1 + Alt3 as a solution to address the UE implementation issue.

Proposal 1: For M-DCI based MTRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, introducing a time gap X between two UL transmissions after applying two different TA values, where the time gap X is a UE capability defined in 𝜇𝑠, e.g., 140 𝜇𝑠.

Proposal 2: For M-DCI based MTRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, the UE does not expect the two UL transmissions after applying two different TA values overlap to each other in time domain

Association between TAG and UL transmission

In RAN1#112 [1], the following were agreed:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

In Rel-18 unified TCI extension, the following was agreed for M-DCI based MTRP [3]:

Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value

Based on the above agreement, there is an association between the activated joint/UL TCI state(s) and each coresetPoolIndex value for M-DCI based MTRP. If one joint/UL TCI state is not activated, there is no association between the joint/UL TCI state and a coresetPoolIndex value in Rel-18 unified TCI extension. On the other hand, all the activated joint/UL TCI states can be used for UL channels/signals, thus it is not necessary to mention the activated UL/joint TCI states “of UL signals/channels”. In summary, the sub-bullet in the agreement should be modified as in the following proposal:

Proposal 3: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

RACH enhancement for both intra-cell and inter-cell MTRP

In RAN1#110bis [3], the followings were agreed for both intra-cell and inter-cell MTRP:

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI

Issue: How to determine the TAG/TAG-ID corresponding to a RACH procedure initiated by UE

When two TAGs are configured within SpCell, it may be still ambiguous that which TAG should apply the initial TA value indicated by RAR in response to PRACH transmission initiated by UE on the SpCell. In our view, this issue can be resolved by pre-defined rule, e.g., the initial TA value shall be applied to the TAG with lowest TAG-ID. For 2-step RACH, since it is also initiated by UE, there is no anchor to let UE know an absolute TA command in MsgB should be applied to which TAG. In our view, this issue can be simply resolved in a similar way.

Proposal 4: If two TAGs are configured within SpCell for M-DCI based MTRP, which TAG should apply the initial TA value indicated by RAR in response to PRACH transmission triggered by UE can be determined based a pre-defined rule, e.g., the initial TA value shall be applied to the TAG with lowest TAG-ID.

Proposal 5: If two TAGs are configured within SpCell for M-DCI based MTRP, support a pre-defined rule to determine which TAG should apply the absolute TA command in MsgB, e.g., the absolute TA command shall be applied to the TAG with lowest TAG-ID.

RACH enhancement for intra-cell MTRP

RAN1#110bis [3], the followings were agreed for intra-cell MTRP:

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

Since it is already agreed that a TAG ID can be associated with a joint/UL TCI state, we see Alt7 is also a feasible solution to determine the TAG/TAG-ID corresponding to a RACH procedure for intra-cell MTRP, where each joint/DL TCI state can be associated with a TAG as well. Then, the initial TA value indicated by an RAR in response to a PRACH transmission triggered by DCI format 1_0 on a CORESET is applied to the TAG corresponding to the joint/DL TCI state used as the QCL assumption for the CORESET, as shown in Figure 2.

Figure 2. Potential solution to determine which TAG should apply the initial TA value indicated by RAR

Proposal 6: If two TAGs are configured within a serving cell for intra-cell M-DCI based MTRP, to indicate/determine which TAG should apply the initial TA value indicated by RAR in response to PRACH transmission triggered by PDCCH order on the serving cell, support the following:

- Alt7:  Each joint/DL TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the joint/DL TCI state used to receive the PDCCH order

RACH enhancement for inter-cell MTRP

In RAN1#110bis [3], the followings were agreed for inter-cell MTRP:

Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order - FFS: Explicit indication or implicit indication through PDCCH order

In RAN1#112 [1], the following was agreed for LTM:

Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

We see similar scheme can used for inter-cell MTRP as well, i.e., the reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of additional PCI.

Proposal 7: On how to determine which PRACH configuration to be used in the RACH procedure triggered by PDCCH order for inter-cell MTRP, the reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of additional PCI.

Conclusion

In this contribution, UL Tx timing management for MTRP was discussed. Based on the discussion in the previous sections, we made the following proposals:

Proposal 1: For M-DCI based MTRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, introducing a time gap X between two UL transmissions after applying two different TA values, where the time gap X is a UE capability defined in 𝜇𝑠 and 140 𝜇𝑠 should be included as a candidate value.

Proposal 2: For M-DCI based MTRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, the UE does not expect the two UL transmissions after applying two different TA values overlap to each other in time domain

Proposal 3: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

Proposal 4: If two TAGs are configured within SpCell for M-DCI based MTRP, which TAG should apply the initial TA value indicated by RAR in response to PRACH transmission triggered by UE can be determined based a pre-defined rule, e.g., the initial TA value shall be applied to the TAG with lowest TAG-ID.

Proposal 5: If two TAGs are configured within SpCell for M-DCI based MTRP, support a pre-defined rule to determine which TAG should apply the absolute TA command in MsgB, e.g., the absolute TA command shall be applied to the TAG with lowest TAG-ID.

Proposal 6: If two TAGs are configured within a serving cell for intra-cell M-DCI based MTRP, to indicate/determine which TAG should apply the initial TA value indicated by RAR in response to PRACH transmission triggered by PDCCH order on the serving cell, support the following:

- Alt7:  Each joint/DL TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the joint/DL TCI state used to receive the PDCCH order

Proposal 7: On how to determine which PRACH configuration to be used in the RACH procedure triggered by PDCCH order for inter-cell MTRP, the reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of additional PCI.

Reference

- 3GPP RAN1 #112, “Chairman’s Notes RAN1 #112”, February 2023

- 3GPP RAN1 #111, “Chairman’s Notes RAN1 #111”, November 2022

- 3GPP RAN1 #110bis, “Chairman’s Notes RAN1 #110bis-e”, October 2022