3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303364

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | xxxx | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [R17] Draft 38.212 CR on SUL indicator and pusch-Config/pucch-Config for DCI 0_0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In 38.212 7.3.1.1.1, for UL/SUL indicator in DCI 0_0, there is one scenario not addressed: - Only one carrier of NUL/SUL configured with pusch-Config but no carriers configured with pucch-Config (E.g. only cell-specific pucch-ConfigCommon is configured) Looking at the RAN1 #90bis agreement (UE specific RRC signalling (re-)configures the location of the PUCCH …), it only regulates the UE-specific RRC configuration. Hence, for the scenario mentioned above, it seems more reasonable to transmit PUSCH on the carrier configured with pusch-Config. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define the PUSCH transmission behavior (transmit PUSCH on the carrier configured with pusch-Config) on NUL/SUL of DCI 0_0 under the following scenario: - Only one carrier of NUL/SUL configured with pusch-Config but no carriers configured with pucch-Config (E.g. only cell-specific pucch-ConfigCommon is configured). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Undefined UE behavior for the sencario mentioned aboove. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis: The draft CR is intended for RAN1 consensus to address this issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.3.1.1.1	Format 0_0

<Unchanged Text Omitted>

-	UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured;

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is configured on only one carrier of UL or SUL, and the higher layer parameter pucch-Config is not configured on UL and SUL, the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pusch-Config is configured;

-	If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured.

-	If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted.

<Unchanged Text Omitted>