3GPP TSG RAN WG1 #112bis-e                                                       R1-2303365 e-Meeting, April 17th – April 26th, 2023

Agenda item:	7.2

Source: 	MediaTek Inc.

Title: 	On rate match pattern periodicity

Document for:	Discussion and decision

# Introduction

In Rel-17, NCD-SSB can be configured to RedCap UEs via dedicated BWP configuration. However, a normal non-RedCap UE is not aware of the presence of NCD-SSB and cannot rate match around NCD-SSB unless instructed by its serving gNB. Unfortunately, the maximum periodicity of RateMatchPattern is only 40ms which is less than the configurable periodicities of 80ms and 160ms for NCD-SSB. Considering trade-off between signaling overhead and specification completeness, we hence propose to add periodicities of n80 and n160 to RateMatchPattern to avoid unnecessary resource waste when NCD-SSB is configured with a periodicity of 80ms or 160ms at least for 15kHz and 30kHz SCS.

The related WI is NR_newRAT-Core for Rel-17. The companion draft CR is provided in R1-2303366 in [1].

# Discussion

As seen in TS 38.331, NCD-SSB can be configured with a periodicity of {5, 10, 20, 40, 80, or 160} msec.

NonCellDefiningSSB information element NonCellDefiningSSB-r17 ::= SEQUENCE { absoluteFrequencySSB-r17 ARFCN-ValueNR, ssb-Periodicity-r17 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S ssb-TimeOffset-r17 ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 } OPTIONAL, -- Need S ... }

From 38.331, the maximum periodicity value from periodicityAndPattern is n40.

38.331 RateMatchPattern ::= SEQUENCE { rateMatchPatternId RateMatchPatternId, patternType CHOICE { bitmaps SEQUENCE { resourceBlocks BIT STRING (SIZE (275)), symbolsInResourceBlock CHOICE { oneSlot BIT STRING (SIZE (14)), twoSlots BIT STRING (SIZE (28)) }, periodicityAndPattern CHOICE { n2 BIT STRING (SIZE (2)), n4 BIT STRING (SIZE (4)), n5 BIT STRING (SIZE (5)), n8 BIT STRING (SIZE (8)), n10 BIT STRING (SIZE (10)), n20 BIT STRING (SIZE (20)), n40 BIT STRING (SIZE (40)) } OPTIONAL, -- Need S ... }, controlResourceSet ControlResourceSetId }, subcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond CellLevel dummy ENUMERATED { dynamic, semiStatic }, ..., [[ controlResourceSet-r16 ControlResourceSetId-r16 OPTIONAL -- Need R ]] }

Besides, from 38.214, there is statement as follows.

Clause 5.1.4.1 of TS 38.214 The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec.

But for NCD-SSB, gNB may configure a longer periodicity than CD-SSB such as 40ms/80ms/160ms. With a RateMatchPattern periodicity shorter than that of NCD-SSB, some REs that are not used by NCD-SSB will be wasted.

Therefore, we have following proposal:

Proposal 1: Add periodicities of n80 and n160 to periodicityAndPattern in RateMatchPattern.

- Send LS to RAN2

# Conclusion

In this contribution, we have the following proposal.

Proposal 1: Add periodicities of n80 and n160 to periodicityAndPattern in RateMatchPattern.

- Send LS to RAN2

# Reference

- R1-2303366, “Draft CR for 38.214 on rate match pattern periodicity,” MediaTek Inc., RAN1 #112bis-e, April 2023