3GPP TSG RAN WG1 #112bis-e                                          R1- 2303367

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.4.1.1

Source: 	MediaTek Inc.

Title: 	Discussion on channel access mechanism

Document for:	Discussion/Decision

Introduction

After discussion in the meetings from RAN1 #109-e [1] to RAN1 #112 [5], the following high-level topics were generated for SL-U channel access mechanism:

- Channel access mechanisms for SL-U

- CP extension

- Sharing of channel occupancy time (COT)

- Multi-consecutive slots transmission (MCSt)

- Multiple channel access procedures

- Resource allocation enhancements (mode 1 and mode 2) in SL-U

In this contribution, we further provide our views on the above topics for SL-U channel access mechanism and show our evaluation results and corresponding observations.

Discussions

Channel access mechanism for SL-U

Type 2A channel access

In RAN1 #111 meeting, we achieved the following agreement on the discussion of Type 2A channel access.

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

The current restrictions of Type 2A channel access for S-SSB transmission without a shared COT is actually from the specification of 3GPP. While in 3GPP specification, there is no definition on the observation period. To that point, we think it is also unnecessary for SL-U to define such a period.

Observation 1: Observation period is not defined in legacy NR-U for the case of performing Type 2A channel access for discovery burst transmission without a shared channel occupancy.

For the last FFS on Type 2A applicability for PSFCH without a shared COT. We do not support Type 2A channel access used for PSFCH transmission without shared COT considering the following reasons:

- In legacy NR-U, only the transmission satisfying the specific conditions can use Type 2A to access the channel without a shared COT, i.e., the transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is at most 1ms, and the discovery burst duty cycle is at most 1⁄20. The HARQ feedback involved in PUCCH should use Type 1 channel access to initiate a COT for transmission. Correspondingly, Type 1 channel access should also be used for PSFCH transmission in SL-U.

- Other issues will be encountered if Type 2A channel access can be used for both S-SSB and PSFCH transmission without shared COT (e.g., how to determine the total/combined transmission duration/duty cycle with considering of both S-SSB and PSFCH transmission).

- The transmission of S-SSB or PSCCH/PSSCH with higher priority may be blocked if Type 2A channel access is used for PSFCH transmission without a shared COT.

Based on the above considerations, we have the following proposal:

Proposal 1: UE performs Type 1 channel access procedures for PSFCH transmissions without a shared channel occupancy and Type 2 channel access procedures within a shared channel occupancy.

Additionally, in legacy NR-U, a UE may switch from Type 1 channel access to Type 2A channel access for its transmission when satisfying the following conditions:

TS 37.213 Section 4.2.1.0.0 If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as described in clause 11.1.1 of [6], the following is applicable: - The UE may switch from Type 1 channel access procedures as described in clause 4.2.1.1 to Type 2A channel access procedures as described in clause 4.2.1.2.1 for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy. In this case, if the UL transmissions are PUSCH transmissions on configured resources, the UE may assume any priority class for the channel occupancy shared with the gNB.

Considering COT sharing behavior will also be supported for SL-U, the above issue will be encountered in SL-U as well and we think this mechanism in NR-U can be reused to optimize the channel access procedure and improve resource efficiency. With this mechanism, the UE actually can switch from a COT initiating UE (i.e., Type 1 channel access to initiate a COT) to a COT sharing UE (i.e., Type 2A channel access to share a COT). Thus the general requirement of COT sharing should be respected (e.g., the destination ID of the UE’s PSCCH/PSSCH transmission)

Proposal 2: If a UE determines the duration in time domain and the location in frequency domain of a remaining COT initiated by COT initiator, the UE may switch from Type 1 channel access procedures to Type 2A channel access procedures for its corresponding SL transmissions within the determined resources of the remaining COT.

- FFS details (e.g., CAPC level, transmission/channel type, ID information).

CP extension

In RAN1 #112, we achieved the following agreement on SL-U CP extension operation:

Agreement #112 A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

CPE for PSFCH and S-SSB

Regarding the configuration (e.g., (pre-)configured in each RP, pre-defined or indicated) on CPE starting position of PSFCH, it may be impacted by the resource configuration of PSFCH transmission. For example, if the resource for PSFCH is (pre-)configured, the CPE starting position can also be (pre-)configured to achieve FDMed transmission of multiple PSFCH transmissions. While if the resource for PSFCH is dynamically indicated by COT initiating UE, the CPE starting position can also be dynamically indicated by the COT initiating UE in this case to generate a specific transmission gap, for example, a gap less than 16 us.

Observation 2: The resource configuration of PSFCH (e.g., (pre-)configured or dynamically indicated) may have impact on the configuration of CPE starting position (e.g., (pre-)configured in each RP, pre-defined or indicated).

For the transmission of S-SSB, considering there may exist different priorities among synchronization references, different CPE starting positions can be used in this case to guarantee the SyncRef UE with high priority can access the channel earlier.

Proposal 3: Multiple CPE starting positions should be (pre-)configured for S-SSB transmission.

Regarding the issue of single or multiple CPE starting positions for PSCCH/PSSCH transmission, we discuss our perspectives from three aspects based on the latest agreement in RAN1 #112:

- Applicable scenario for multiple CPE starting positions

- Criteria for defining a default CPE starting position when multiple CPE starting positions are (pre-)configured

- Criteria for selecting one CPE starting position when multiple CPE starting positions are (pre-)configured

Multiple CPE starting positions

Applicable scenarios

We discuss the applicable scenarios for multiple CPE starting positions from the perspective of outside and inside a COT.

For outside COT operation, considering the distributed feature of SL, synchronized transmission starting position for multiple UEs will result in a large intra-cell collision if the transmission from multiple UEs are initial transmission on a same SL resource. As shown in Figure 1, both UE 1 and UE 2 finish Type 1 channel access before slot n. if UE 1 and UE 2 access the channel at the 1st symbol of slot n in the way of synchronization (e.g., only one CP extension starting position), the transmissions from UE 1 and UE 2 will collide with each other.

Figure 1. Illustration of intra-cell collision for SL-U with synchronized transmission.

Observation 3: Synchronized transmission for SL-U results in intra-cell collision between initial transmissions outside a COT.

To solve this issue, a sensing slot level channel access together with multiple CPE durations/starting positions can be used. As an example, shown in Figure 2, the symbol immediately before the starting symbol (supposing the 2nd symbol within a slot is the starting symbol in this example) can be further divided into multiple sensing slots. The other symbols in this slot can be used for data transmission(s) if the sensing slot level channel access is successful. Multiple CPE durations/starting positions can be used to align the boundary between the end of sensing slot level channel access and the start of transmission. Besides, the CPE duration/starting position can be determined according to the CAPC value of the traffic from different UEs to achieve prioritized channel access. As the example shown in Figure 2, UE 1 within a shorter sensing slot level channel access can occupy the channel with an earlier CPE starting position (or longer CPE duration), which blocks the sensing slot level channel access of UE 2 and the collision can be avoided.

Figure 2. Illustration of sensing slot level channel access with multiple CPE durations/starting positions within a symbol.

Observation 4: Sensing slot level channel access together with multiple CPE starting positions can better avoid intra-cell initial transmission’s collision outside a COT.

The above observations motivate the following proposals:

Proposal 4: Multiple CPE starting positions are supported outside a COT.

For inside COT operation, in legacy NR-U, different transmission gap lengths can be generated by different CPE durations/starting positions. The supported durations of CPE in NR-U are:

- 0 (i.e., no CP extension): the scheduled UL transmission is outside of a gNB-initiated channel occupancy time. UE applies Type 1 channel access to initiate its own COT.

- C1*symbol length – 25 us: To generate a gap of 25 us between two UL transmissions within a gNB-initiated COT as illustrated in Figure 3(a).

- C2*symbol length – 16 us – TA: To generate a gap of 16 us between a DL transmission and an UL transmission in a gNB-initiated COT as illustrated in Figure 3(b).

- C3*symbol length – 25 us – TA: To generate a gap of 25 us between a DL transmission and an UL transmission in a gNB-initiated COT as illustrated in Figure 3(c).

Figure 3(a). CPE (C1*symbol length – 25 us) for a scheduled uplink transmission following another scheduled uplink transmission in a gNB-initiated COT where the TA values for the two uplink transmissions are assumed to be approximately the same.

Figure 3(b): CPE (C2*symbol length – 16 us – TA) to generate a gap of 16 us between a DL transmission and an uplink transmission in a gNB-initiated COT.

Figure 3(c). CPE (C3*symbol length – 25 us – TA) to generate a gap of 25 us between a DL transmission and an uplink transmission in a gNB-initiated COT.

Figure 3. Utilization of CPE to generate different gaps between two transmissions in legacy NR-U.

Observation 5: Different CPE starting positions can be used to generate different gaps between two transmissions inside a COT.

In SL-U, we already agreed that Type 2A / 2B / 2C channel access can be applied for different transmission gaps of >= 25us / at least 16us / <=16us inside a COT, respectively. It is reasonable to support multiple CP extension durations/starting positions to create different transmission gaps inside a COT. Otherwise, maybe only one single Type 2 channel access can be applied, which is contrary to our intention of the agreement on the utilization of Type 2A / 2B / 2C channel access.

Observation 6: It is agreed that Type 2A / 2B / 2C channel access can be applied to different gaps between two transmissions inside a COT.

Therefore, we think different CPE durations/starting positions are necessary to guarantee the application of Type 2A / 2B / 2C inside a COT.

Proposal 5: Multiple CPE starting positions are supported inside a COT.

Criteria for defining a default CPE starting position

When multiple CPE starting positions are (pre-)configured, a default/common CPE starting position can be defined to achieve FDMed transmission. In our opinion, as shown in Figure 4, the reservation information or the status of the resource being occupied (e.g., reserved or non-reserved resource) can be used as a criterion for defining a default CPE starting position, which can be detailed as the following items:

- If CPE is performed before a non-reserved resource, i.e., the resource is not reserved by the channel access executer and other UE(s),multiple CPE starting positions within one or two symbols can be used. In this case, after the channel access procedures of different UEs are completed, the transmissions from different UEs can be started from a subset of multiple CPE starting positions based on the traffic priority. UE then randomly chooses a single CPE starting position within the subset of multiple CPE starting positions. With random CPE starting position selection, initial transmission collision can be avoided among different non-reserved UE’s transmission.

- If CPE is performed before a reserved resource including the resource reserved by the channel access executer and/or the resource reserved by the other UE(s), a default CPE starting position can be used. In this case, after the channel access procedures of different UEs are completed, the transmissions of different UEs can be started from the single CPE starting position, and FDMed transmission can be achieved.

Figure 4. The illustration of default and multiple CPE starting positions.

Observation 7: For the case of multiple CPE starting positions, a default CPE starting position can be used before the reserved resource to achieve FDM transmission. A single CPE starting position can be selected before the non-reserved resource to avoid initial transmission collision

Based on the above observation, we have the following proposal:

Proposal 6: Resource reservation information can be used as a criterion for defining a default CPE starting position from the set of multi-CPE starting positions. The resource reservation information includes both the resource reservation information from the CPE transmitting UE and other UE(s)

- If resource reservation information of the transmission resource slot is available, UE’s transmission is started from the default CPE starting position.

- If resource reservation information of the transmission resource slot is not available, UE’s transmission is started from a subset of multi-CPE starting positions based on the traffic priority. UE randomly chooses a single CPE starting position within the subset of multi-CPE starting positions.

Criteria for selecting one of the multiple CPE starting positions

As mentioned before, multiple CPE starting positions can be used before the non-reserved resource. In this case, one of the multiple CPE starting position should be selected for starting the transmission. Considering the prioritized transmission mechanism in legacy SL, we think the same principle can be used here. To that point, CAPC level is preferred as the criterion for selecting one of the multiple CPE starting positions to grant the UE with higher priority traffic a higher channel access opportunity.

Proposal 7: CAPC value can be used as a criterion for selecting one of the multiple CPE starting positions.

Single CPE starting position

In fact, the motivation of single CPE starting position is to achieve FDMed transmission for multiple UEs. From our point of view, one default CPE starting position defined within multiple CPE starting positions can also realize this purpose. Correspondingly, it is unnecessary to define a single CPE starting position for PSCCH/PSSCH transmission.

Observation 8: FDM transmission of PSCCH/PSSCH from multiple UEs can be achieved by defining a default CPE starting position from the set of multiple CPE starting positions. It is unnecessary to define a single CPE starting position for PSCCH/PSSCH transmission.

Proposal 8: Only support multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH transmission.

On the other hand, for inside COT operation, the COT initiating UE may share the COT for transmissions by multiple UEs. In this case, if the COT initiator can schedule the transmissions of the multiple UEs like gNB/AP scheduling UEs/STAs in NR-U/WiFi, a single CPE starting position can be dynamically indicated by the COT initiating UE for multiple UEs to access the shared COT in the way of FDM.

Proposal 9: Single CPE starting position can be achieved by the indication of COT initiating UE inside a COT.

Sharing of channel occupancy time (COT)

From our perspective, there are three main issues for the discussion of COT sharing:

- The utilization of the shared COT

- Content of COT indicator

- Container of COT indicator

In RAN1 #112, we achieved an agreement on who can use the shared COT. While in RAN1 #111, we also had a discussion on the proposal of the content and container of COT indicator, which is also attached below.

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

COT sharing for PSFCH transmission

For the case of COT sharing used for PSFCH transmission, a remaining FFS is whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator. One key issue is the blind detection of PSFCH for the UE(s) other than the initiator. It can be further divided into the following two cases.

- Case 1: The resource within the shared COT for PSFCH transmission is (pre-)configured.

- Case 2: The resource within the shared COT for PSFCH transmission is dynamically indicated by the COT initiating UE.

In Case 1, the resource location for PSFCH transmission is a prior information for the UE(s) other than the initiator. Correspondingly, the complexity of PSFCH detection may not be increased for the other UE(s) in this case. In Case 2, for the UE(s) cannot receive COT indication from the COT initiator, the potential resource for PSFCH transmission is unknown for these UEs, which may further result in the blind detection issue. Therefore, from our point of view, the resource configuration method for PSFCH transmission should be considered when treating this FFS.

Observation 9: The resource configuration (e.g., (pre-)configured or dynamically indicated) for PSFCH transmission may have impact on whether a responding UE can transmit PSFCH(s) to the UE(s) other than the initiator when using the shared COT.

Proposal 10: For (pre-)configured PSFCH resource within a shared COT, a responding UE can transmit PSFCH(s) to UE(s) other than the initiator. For dynamically indicated PSFCH resource within a shared COT, a responding UE cannot transmit PSFCH(s) to UE(s) other than the initiator.

Contents of COT sharing information

For the contents of COT sharing information, we had an extensive discussion in RAN1 #110b-e and the proposal is attached above. In general, we are fine the with the current proposal but with the following observations. As stated in the agreement on Type 2 channel access in RAN1 #110 meeting, when a responding UE use a shared COT from the COT initiator, the responding UE can utilize Type 2A/2B/2C channel access before the scheduled resource according to different gap durations. In NR-U, as we also mentioned in Section 2.3.1, different CPE starting positions can be used to generate different gap durations between two transmissions inside a COT. Therefore, the channel access type together with the information of CPE starting position should be delivered by the COT sharing information to provide the basic information for the responding UE accessing the channel. Then combined with the current proposal, we have the following proposal for the contents of COT sharing information:

Proposal 11: COT sharing information should include information of CPE starting position, channel access type, L1 ID, CAPC level, remaining COT duration, RB set(s) in the COT, starting offset of the shared COT, and responding UE’s transmission.

Multi-consecutive slots transmission (MCSt)

Due to limit time, MCSt mechanism for SL-U is not treated in the GTW of RAN1 #112, but in RAN1 #110b-e, the following related agreements were achieved:

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (, remaining PDB, and ) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (, remaining PDB, and ) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

In legacy SL Mode 2, the resource is selected based on the following steps:

- Higher layer provides a set of parameters (L1 priority, PDB, number of sub-channels, RRP) to L1 per TB.

- L1 report a subset of candidate single-slot resources to higher layer.

- Higher layer randomly selects the resource(s) for initial/re-transmission from the reported subset of candidate single-slot resources.

Only one set of parameters is provided from higher layer when L1 is triggered for reporting a subset of candidate resources in legacy SL. This principle we think can be reused for SL-U even in MCSt or the case of multiple TBs. Besides, we think Option 2 can be realized by triggering Option 1 multiple times. Thus Option 1 is a more general solution and has less spec impact compared with Option 2.

Observation 10: Option 2 can be realized by triggering Option 1 multiple times.

Additionally, to assist the resource exclusion and selection procedures in L1, the information of (lowest) CAPC value of traffic, consecutive slots number can be delivered to L1 when L1 is triggered for reporting a subset of candidate resources for MCSt. A relaxed resource exclusion and selection procedure can be studied to achieve MCSt, e.g., a relaxed RSRP threshold or remaining resource portion after resource exclusion based on the information delivered from higher layer.

Observation 11: The information of CAPC and consecutive slots number of MCSt can assist L1 to better select resources for MCSt.

When L1 reports a subset of candidate resources for MCSt, Option B seems follows the behaviour in legacy SL with only one subset of candidate single-slot resource is reported. However, if no further enhancement for L1 resource exclusion and selection procedure is considered in Option B, which has the possibility that the reported resource set SA does not contain any multi-consecutive slots. The same issue also involved in Option C.

Observation 12: Enhanced mechanisms (e.g., RSRP adjustment) are needed for L1 to achieve resource selection of multi-consecutive slots.

Based on the above observations, we have the following proposals:

Proposal 12: The information of CAPC value and consecutive slots number should be provided when L1 is triggered for reporting a subset of candidate resources for MCSt.

Proposal 13: Option 1 and Option A below are preferred for MCSt operation in SL-U, and enhanced mechanisms should be studied when L1 reports a subset of candidate resources for MCSt.

- When L1 is triggered for reporting a subset of candidate resources for MCSt,

  - Option 1: Only one set of parameters (prio, remaining PDB, L and P) is provided for the resource selection procedure in L1

- When L1 reports a subset of candidate resources for MCSt,

  - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time

Additional LBT time in SL resource allocation

Per agreement of RAN1 #110b-e, we have an FFS on whether/how to consider the additional LBT time in SL resource allocation, which is also related to the topic of resource allocation enhancement for SL-U. From our point of view, SCI sensing in SL is used to address the intra-RAT resource collision to avoid the interference by resource reservation in the proactive approach. LBT sensing, to meet the regulator requirement, is to address inter-RAT/intra-RAT interference by sensing the resource immediately before the transmission in the reactive approach. Thus, both of them have their advantages depending on the scenario and use cases. For SL-U, it would be reasonable to consider a combined solution by using both SCI sensing mechanism and LBT sensing mechanism jointly. However, the uncertainty of LBT may introduce additional time consumption for legacy SL resource allocation and thus may invalidate the selected resources.

Additionally, we actually already had another agreement at RAN1 #109-e to emphasize the enhanced mechanisms for Mode 1 and Mode 2 RA needed to be studied when introducing additional LBT operation as copied below:

Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation procedure needs to be updated / enhanced due to shared spectrum channel access - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2

From our point of view, it is necessary RAN 1 should have an independent topic to discuss the essential enhanced mechanisms for resource allocation due to additional channel access procedure. From this point of view, the related issues are detailed in an individual Section 2.6.

Proposal 14: For SL-U, RAN1 should study essential enhancements for resource allocation due to additional channel access procedure.

Multi-channel access procedures

Due to limit time, multi-channel access procedures for SL-U is not treated in the GTW of RAN1 #112, but in RAN1 #111, the following related agreements were achieved:

Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

In legacy NR-U, a difference between Type A and Type B DL multi-channel access is the channel access type performed on each channel. For Type A DL multi-channel access, Type 1 channel access with modifications on counter maintenance is performed on each channel. For Type B DL multi-channel access, only one channel is selected to perform Type 1 channel access, while for the other channels, it is required to sense the channel for at least a sensing interval of 25 ms immediately before the transmission. Two different DL multi-channel access procedures specify different channel access requirement on each channel and may result in different channel access duration.

For SL-U, if the CAPC value for PSFCH transmission is agreed to be always set as 1, the difference between Type A and Type B multi-channel access is marginal. While if it is agreed that any CAPC value can be used for PSFCH transmission, there may exist a large difference between these two mechanisms. Therefore, we think the discussion of whether type A or type B or both NR-U DL multi-channel access procedures will be supported for PSFCH can be postponed after there is an agreement on the CAPC value setting for PSFCH transmission.

Observation 13: The CAPC value of PSFCH may have impact on the utilization of Type A/Type B NR-U DL multi-channel access for PSFCH transmission.

Regarding the multi-channel access procedures for S-SSB, as analyzed in our companion paper on SL-U physical channel design, S-SSB transmission on multi-RB sets will introduce many complex issues like OCB restriction, multi-channel access procedure, and may also degrade the spectrum efficiency. Therefore, multiple RB sets for S-SSB transmission should not be supported from our side and the corresponding FFS should be removed.

Proposal 15: The transmission of S-SSB on frequency domain should be restrained within one RB set, thus multi-channel access is not supported for S-SSB transmission.

Resource allocation enhancement

Mode 1

For Mode 1, the resource is selected/reserved by gNB and the channel access is performed by UE. It implies that the resource selection/reservation (at gNB side) is decouple with the channel access procedure (at UE side). Essentially, there is the timeline impact on Mode 1 resource allocation procedure due to the introduction of the LBT sensing depending on the reserved or non-reserved transmission.

For the transmission on the non-reserved resources, e.g., the first transmission of the periodic traffic or the initial transmission of the aperiodic traffic, the SL-U resource allocation in the grant by gNB should consider the additional time for channel access procedure at the UE side in addition to the existing UE processing time. Thus, there may be the impact on the timeline for Mode 1 operation. Moreover, channel access related/updated information (e.g., CAPC value/contention window size/ random-backoff counter information) may need to be known by gNB to help allocate the resources located after channel access procedure.

For the transmission on the reserved resources, gNB may need to consider the sufficient time gap between the reserved resources during the resource selection for the potential channel access procedure. Or gNB may have to overbook some contiguous resources to address potential channel access failure. Thus, it may also require some channel access related/updated information to be known at gNB for the proper resource selection to avoid invalid resource allocation.

In general, the uncertainty of channel access procedure may introduce additional time consumption for legacy Mode 1 resource allocation procedure and thus may invalidate the allocated resources. To solve this issue, the channel access related/updated information can be reported from UE to gNB for aiding Mode 1 resource allocation at the gNB side. The channel access related information may include, for example, the CAPC value of the traffic/contention window size/random back-off counter, etc. Assisted by the channel access related/updated information, the gNB can schedule resources for initial transmission and/or re-transmission with a proper estimated LBT protection margin. Besides, as described previously, the channel access information like contention window size may be adjusted during the transmission. The adjustment related information of the contention window size should also report to gNB to assist the following resource allocation.

Observation 14: The uncertainty of LBT may introduce additional time consumption for legacy SL resource allocation and thus may invalidate the scheduled or configured resources in Mode 1.

Observation 15: For SL-U Mode 1 resource allocation, the resource assignment at gNB side may be impacted by channel access information (e.g., contention window size), which is available at the UE side.

Based on the above observations, we have the following proposal:

Proposal 16: Mode 1 UE can report channel access information (e.g., contention window size) to gNB for aiding Mode 1 resource allocation.

Mode 2

Impact of LBT failure on SCI-based resource selection

Essentially, SCI sensing in SL is used to address the intra-RAT resource collision to avoid the interference by resource reservation in the proactive approach. LBT sensing, to meet the regulator requirement, is to address inter-RAT/intra-RAT interference by sensing the resource immediately before the transmission in the reactive approach. Thus, both of them have their advantages depending on the scenario and use cases. For SL-U, it would be reasonable to consider a unified solution by using both SCI sensing mechanism and LBT sensing mechanism jointly. Essentially, it is possible considering the different stages to perform SCI sensing and LBT sensing in the procedure.

For mode 2 operation, SCI sensing for resource reservation and LBT sensing are happened at the same UE. Thus, LBT information (e.g., LBT access type, contention window size) can be available at the device. Similar to the Mode 1 operation, there can be some timeline impact for Mode 2 sensing/selection procedure depending on the traffic (e.g., CAPC of the traffic/CW size).

For the transmission on the non-reserved resources, e.g., the first transmission of the periodic traffic or the initial transmission of the aperiodic traffic, the UE may perform LBT at first and then determine the selection window for resource selection based on the LBT success time. In this case, it is possible that there is a gap between LBT success occasion and the resource selected for transmission. This can be handled by additional short defer sensing or CP extension to handle the gap. Determination of the starting point of the selection window according to the LBT success occasion can avoid the invalid resource selection as much as possible. Alternatively, the UE may perform resource selection at first and then perform LBT immediately before the selected resource. In this case, the starting point of the resource selection window should take into account the LBT operation time in addition to the processing time T1. Besides, considering the resources are selected before LBT procedure in this case, it may invalidate the selected resources due to potential LBT failure. Therefore, some solutions (e.g., overbooking) should be considered meanwhile for this case.

For the transmission on the reserved resources, the UE may have to determine the trigger time for LBT operation according to the occasion of the reserved resource for transmission. That is, LBT operation is performed up to the reserved resource (i.e., performed some time earlier than the reserved resource for transmission) taking into account LBT counter and the issue of potential LBT failure.

To further study the impact of additional LBT time on legacy SL Mode 2 SCI-based resource allocation procedure as described above, we provide the corresponding evaluation results in Figure 5. In this evaluation, 10 pairs of SL-U UE are dropped in an indoor scenario. In the simulation, when the BO increased from low to medium and high, more selection windows will be spanned to select enough resources for the corresponding traffic, i.e., the latency will be increased from low to medium and high BO. The max contention window size in LBT channel access equals to 127. The other related parameters are summarized in Appendix 1.

Figure 5. Average UPT of SL-U for legacy SCI-based RA w/ and w/o impact from LBT (no RA enhancement used here)

It can be observed from Figure 5 that compared with legacy SL SCI-based resource selection, the introduction of LBT operation can degrade the UPT performance of the system under all cases of low/medium/high BO. Besides, it can be observed that for the case of high BO, the introduction of LBT operation significantly degrades the UPT performance. The reason is that under high BO, the LBT operation will suffer a larger random time duration, which further results in more invalidated resources selected by SCI-based resource selection. Generally, we have the following observations from the analysis and evaluation results:

Observation 16: The uncertainty of LBT operation may introduce additional time consumption for legacy SL resource allocation and thus may invalidate the selected resources in Mode 2 and degrade the UPT performance of the system.

Observation 17: Essential enhancements should be studied to solve the timeline issue for SL resource allocation procedures due to additional time consumption introduced by LBT operation.

Enhancements to combat the potential LBT failure

Figure 6. Enhanced SCI-based resource selection with LBT operation

As shown in Figure 6, after the packet (aperiodic/periodic) arrival, the random back-off (BO) counter N can be generated/dropped according to the CAPC value. Besides, the LBT channel access procedure can be triggered as soon as the packet is arrived. Additionally, with the random back-off counter N, the approximate duration of LBT channel access can be predicted. Then to combat the potential LBT channel access failure, a protection gap following the (pre-)predicted LBT finish point can be configured/indicated, which can grant more flexible time to try LBT channel access. The selection window position and the candidate resource set can be determined after the packet arrival or after the determination of the LBT end followed by a protection gap margin. During the selection window, resource overbooking mechanism can be (pre-)configured/indicated to combat the potential LBT failure. Additionally, alternatively to the random resource selection within the selection window, select resource as soon as possible after the LBT channel access procedure is finished may better retain the channel and improve the efficiency of COT utilization. For the case that LBT channel access is finished while the packet transmission is not ready, the UE should step into the self-defer duration, and a short LBT channel access shall be performed immediately before the intended transmission.

Proposal 17: Study enhancements (e.g., overbooking/protection margin) for SL-U resource allocation to combat the impact introduced by additional channel access procedure.

Enhanced mechanism to fulfil the gap between LBT end and transmission start

For the case that LBT procedure is finished but the SL transmission point has not yet arrived, enhancement between the end of the LBT procedure and the start of the SL transmission should be considered from our point of view. Specifically, Rel-16/Rel-17 SL UE only have the capability to start PSCCH/PSSCH transmission at the granularity of slot. As working assumption in RAN1 #110b-e [3], the number of starting symbol(s) within a slot for SL-U can be increased but may still have a limited number (e.g., 2 starting symbols within a slot). As a result, the gap between the LBT finish point and the transmission starting symbol may be larger than one symbol. Considering the maximum length of CPE operation is limited (e.g., one symbol and/or 1/2/4 symbols for 15/30/60 kHz SCS, respectively), the other enhancements should be considered to retain the channel as soon as the LBT is finished. For example, the repetition of the following SL transmission can be transmitted here to fulfil the gap mentioned above and block the LBT/transmission of the other UEs, including the UEs from intra-cell and inter-cell, during this period.

Proposal 18: For the gap between LBT end and SL transmission start, the repetition of SL transmission can be used to retain the channel.

VLP operation

Very-Low-Power (VLP) operation has been widely used or considered in some regions (e.g., EU/CN/US) for the important 5/6 GHz spectrum as summarized below:

- EU：ECC Decision (20)01 supports Lower Power Indoor (LPI) and VLP devices in 5925~6425MHz. VLP is specified for indoor and outdoor use with max EIRP 14dBm and max PSD -8 dBm/MHz (i.e., 5dBm for 20MHz).

- CN：VLP operation is supported at 5725-5850MHz with max EIRP 14dBm.

- US：So far, it only supports standard and/or LPI devices in 5945~7125MHz. However, it will issue a ruling about VLP device for hotspots and short-range application

Similar to short control signaling mechanism and OCB exemption for the simplification of channel access in time and frequency domain, respectively, VLP can be considered as a power domain approach to meet regulator requirement.

Considering that commercial use cases such as smart home, personal access network will be focused for SL-U use cases, the low power operation may not cause any coverage problem issue. Instead, VLP essentially may reduce the interference significantly due to the short communication range and more friendly for co-existence subject to the regulator requirements based on extensive studies. Additionally, the VLP operation provides the possibility for on-chip PA implementation to support the low cost/power devices which are dominated/required for the unlicensed spectrum usage. For VLP operation, it may also simplify the implementation since LBT operation may not be necessary and hence make the design and implementation easier, more friendly for the IoT device and fast deployment.

Therefore, we can see that VLP can be applied to the important short-range communication use cases such as wearable devices, in-car communications, SL-based industry IoT. So, it can make SL-U more competitive in terms of the cost and power consumption. Thus, it is worth studying the potential spec impact and benefits of VLP operation for SL-U.

To evaluate the applicability and performance of VLP operation, we conduct corresponding simulations in section 2.8    under different configurations (e.g., scenario, traffic model, etc.). In general, the evaluation results show that with VLP operation, the fairness of coexistence between SL-U and NR-U can be achieved. And more importantly, compared with higher power mode (e.g., max Tx power of 18 dBm), the performance of both SL-U and NR-U can be improved with VLP operation (e.g., max Tx power of 5dBm) together with no LBT operation. The specific evaluation results and observations can be found in section 2.8    .

Proposal 19: Study whether/how to support VLP operation for SL-U.

RLF detection

For Rel-16/Rel-17 SL, the radio link failure (RLF) shall be detected with one of the following triggers [7]: 1) the maximum number of retransmissions for a specific destination has been reached; or 2) T400 expiry for a specific destination; or 3) the maximum number of consecutive HARQ DTX for a specific destination has been reached.

For SL-U, the HARQ feedback may be blocked due the LBT channel access failure at the Rx UE side, which will result in the increment of DTX at the Tx UE side and further has the potential to incorrectly trigger HARQ-based RLF detection. This may lead to the frequent HARQ-based FLR detection for SL-U especially in the case of heavy network loading.

Additionally, the results of LBT channel access can also reflect the quality of the radio link, which thus is regarded as one of the RLF triggers in legacy NR-U, i.e., consistent LBT failure based RLF detection. Therefore, for the RLF detection mechanism in SL-U, consistent LBT failure can also be considered as one of the triggers.

Observation 18: The consistent LBT failure based RLF detection in legacy NR-U should be supported as one RLF trigger in SL-U.

Observation 19: The LBT failure at Rx UE side before the PSFCH occasion(s) can result in the absence of PSFCH reception(s) at the Tx UE side and may further incorrectly trigger HARQ-based RLF detection.

These observations motivate the following proposal:

Proposal 20: Study solutions to combat the impact of LBT failure on the RLF detection in SL-U.

Evaluation results

For the part of evaluation methodology, the following agreements are achieved in RAN1 #110 meeting:

Agreement The following evaluation scenario can be used for evaluating performance of SL-U designs, resource allocation schemes, and coexistence study with another RAT in a shared channel. - Scenario 1 (commercial use cases) – recommended: - Evaluation methodology baseline is NR-U from TR 38.889 with the following updates. - Indoor layout - Option 1: a pairs topology for SL-U from R1-2205033 – recommended - a = 20m, b = 60m, c = 20m, d = 80 m - There are two operators to model two RATs at a time. The red one is SL-U UE, the blue one is Wi-Fi or NR-U. - For NR-U / Wi-Fi, the same number of UEs / Wi-Fi STA as the total number of SL-U devices are dropped in the area. The NR-U UE / Wi-Fi nodes are dropped uniformly per gNB/AP per 20 MHz. - Companies should report if they used a different number of UEs / Wi-Fi STA as the total number of SL-U devices, as an additional evaluation scenario. - For evaluation of unicast traffic, the topology of SL-U is pair topology and the SL-U UEs are dropped uniformly at random in the area. - Companies should report how SL-U UEs are paired - 6 SL-U pairs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz - For evaluation of groupcast traffic, SL-U UEs are dropped uniformly at random in the area, SL-UEs form groupcast UE group based on TX-RX UE distancing, the distance is provided by each company. - Companies should report how SL-U UEs form a group - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz - For evaluation of broadcast traffic, SL-U UEs are dropped uniformly at random in the area. - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz - Option 2: SL UE clusters (R1-2203146) - Indoor layout and UE dropping model with N = 3 or 6 clusters and each with M=5 UEs - Each cluster is a circle, with a central point and radius Rmax = 15 or 10m and Rmin = 5 or 1m - No overlapping among the N clusters - For coexistence, there are two operators to model two RATs at a time, where the red one is Wi-Fi AP or NR-U gNB. NR-U UE / Wi-Fi STA are dropped uniformly per gNB/AP. - Simulation bandwidth can be larger than 20MHz (e.g., 80MHz) - Channel model follows NR InH Mixed Office model used in NR-U (TR38.889) - Traffic model - Option 1: R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10) - FFS whether/how the PDB requirement can be captured - Option 2: FTP model 3 with arrival rate satisfying one of the followings: - BO Low load: 10%~25% - BO Mid load: 35%~50% - BO High load: above 55% - Option 3: XR cloud gaming model in TR38.838 - FFS whether/how the PDB requirement can be captured - It is up to each company to use either Option 1 or 2 or Option 3 or mixed of them - Interference model: - Layout option 1: Explicit modelling of NR-U / WiFi transmissions (as per TR38.889) - Note, for the interference traffic model: - The same or equivalent traffic model setting as SL-U should be used as much as possible to achieve equal load (e.g., SL-U RAT offered load equal the interfering RAT’s offered load). - The same number of traffic flows should be used between SL-U and the interfering RAT (e.g., 10 UEs with 10 flows, and 5 STAs with 2 flows each, one for DL and one for UL) - Companies should report if they used a different assumption, as an additional evaluation scenario. - Performance metric: UPT, latency, and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure. - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation - FFS for groupcast and broadcast - Fair coexistence criterion between SL-U and the interfering RAT (e.g., according to NR-U TR38.889)

Fairness coexistence

In the following evaluation results, the indoor layout option 1 agreed in RAN1 #110 meeting: a pairs topology for SL-U is adopted. The unicast traffic and traffic model of FTP mode 3 with low/medium/high BO are used in the coexistence evaluation between SL-U and NR-U. For SL-U, 5 pairs of UEs are generated per 20MHz where only Tx UE will transmit packet to Rx UE in a SL-U pair. For NR-U, the total traffic flow (i.e., 5 flow in total) is kept same with that of SL-U to achieve an equal traffic load. The paring RSRP threshold of SL-U is set as -82 dBm in the following evaluation results. The other detailed evaluation configuration is summarized in Appendix 1.

Different coexistence scenarios including NR-U + NR-U, NR-U + SL-U, and SL-U + SL-U are evaluated. For each coexistence scenario, the results under different BO of 16%, 35% and 55% are simulated to show the coexistence performance under low/medium/high traffic load, respectively. Energy detection threshold for LBT is configured as -72dBm for both NR-U and SL-U. To improve the channel access efficiency of SL-U as analysed previously, both NR-U and SL-U can start transmission at the end of LBT procedure. For NR-U, the COT is initiated by gNB and then shared to UE for UL transmission. Additionally, in the coexistence evaluation, the maximum Tx power of SL-U is set as 18dBm (non-VLP mode) and 5dBm (VLP mode), respectively, to show the impact of VLP operation on the performance of both NR-U and SL-U under the coexistence scenario.

The UPT performance of NR-U in the coexistence scenario is provided in Figure 7. It is regarded as fair coexistence if the UPT of NR-U or Wi-Fi operator is not degraded when the coexisting operator is replaced with SL-U. In Figure 7, the UPT performance of NR-U is provided in 5 cases, where the performance of NR-U in case 1 can be used as a baseline.

- Case 1(baseline): Coexistence scenario of NR-U + NR-U.

- Case 2: Coexistence scenario of NR-U + SL-U, where the max Tx power is 18dBm and LBT is enabled for SL-U.

- Case 3: Coexistence scenario of NR-U + SL-U, where the max Tx power is 18dBm and LBT is disabled for SL-U.

- Case 4: Coexistence scenario of NR-U + SL-U, where the max Tx power is 5dBm and LBT is enabled for SL-U.

- Case 5: Coexistence scenario of NR-U + SL-U, where the max Tx power is 5dBm and LBT is disabled for SL-U.

Figure 7. The average UPT of NR-U UL under different cases

The corresponding raw data of Figure 7 is provided in Table 1.

Table 1. Evaluation results of average UPT of NR-U UL under different cases

| Traffic loading | Low load BO range for NR-U in NR-U + NR-U: 10%~25% |  |  |  |  | Medium load BO range for NR-U in NR-U + NR-U: 35%~50% |  |  |  |  | High load BO range for NR-U in NR-U + NR-U: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | NR-U in NR-U+ NR-U | NR-U in NR-U+ SL-U |  |  |  | NR-U in NR-U+ NR-U | NR-U in NR-U+ SL-U |  |  |  | NR-U in NR-U+ NR-U | NR-U in NR-U+ SL-U |  |  |  |
| SL-U Max Tx power [dBm] | - | 18 | 18 | 5 | 5 | - | 18 | 18 | 5 | 5 | - | 18 | 18 | 5 | 5 |
| SL-U LBT | - | O | × | O | × | - | O | × | O | × | - | O | × | O | × |
| UPT [Mbps] | 92 | 92 | 89 | 95 | 95 | 63 | 63 | 55 | 76 | 76 | 34 | 32 | 28 | 42 | 41 |
| Delay [ms] | 7.21 | 7.46 | 8.01 | 6.73 | 6.73 | 22.84 | 23.16 | 25.39 | 9.47 | 9.53 | 159 | 182 | 203 | 43.41 | 45.77 |
| BO | 16% | 19% | 19% | 17% | 17% | 35% | 36% | 43% | 35% | 35% | 55% | 57% | 58% | 53% | 54% |

As shown in Figure 7 and Table 1, for case 2, the fairness can be achieved from a similar UPT performance of NR-U UL when the coexisting operator is changed from NR-U to SL-U. The major reason makes the fairness can be achieved is that for NR-U, the gNB can enjoy a more relaxed noise figure value against SL-U UE (5dB vs. 9dB in the evaluation), which increases the opportunity of channel access for NR-U when coexistence with SL-U nodes.

However, for case 3, the fairness cannot be achieved with a reduced UPT performance of NR-U UL when the coexisting operator is changed from NR-U to SL-U due to the increased interference from SL-U without LBT channel access.

Observation 20: In indoor scenario with symmetric traffic at low/medium/high loads in 20MHz bandwidth at 5GHz, the evaluation results of UPT show that the fairness coexistence between NR-U and SL-U can be achieved for the case of SL-U with 18dBm max Tx power and enabled LBT operation.

Observation 21: LBT is necessary to stabilize system interference especially for non-coordinated SL-U deployment for the case of SL-U with 18dBm max Tx power.

VLP operation with FTP model 3 traffic

Additionally, it can be also observed from Figure 7 and Table 1 that when the max Tx power of SL-U is changed from 18dBm to 5dBm, in both cases of SL-U with (case 4) and without (case 5) LBT operation, the fairness can be achieved with an improved UPT performance of NR-U UL when the coexisting operator is changed from NR-U to SL-U. The major reason is that under VLP mode (i.e., 5dBm), the limited coverage range of SL-U results in a very low interference to NR-U system, which is more beneficial to NR-U system compared to SL-U with max Tx power of 18dBm.

Observation 22: Compared with higher SL-U max Tx power (e.g., 18dBm), the UPT performance of NR-U can be improved for the case of lower SL-U max Tx power (e.g., 5dBm) in coexistence scenario.

Additional to the performance of NR-U, the UPT performance of SL-U in the coexistence scenario is also evaluated. In Figure 8, the UPT performance of SL-U is provided considering 4 cases, where the performance of SL-U in case 1 can be used as a baseline.

- Case 1(baseline): Coexistence scenario of SL-U + SL-U.

- Case 2: Coexistence scenario of NR-U + SL-U, where the max Tx power is 18dBm and LBT is enabled for SL-U.

- Case 3: Coexistence scenario of NR-U + SL-U, where the max Tx power is 5dBm and LBT is enabled for SL-U.

- Case 4: Coexistence scenario of NR-U + SL-U, where the max Tx power is 5dBm and LBT is disabled for SL-U.

Note that the for the coexistence scenario of NR-U + SL-U, the UPT performance of SL-U @18dBm without LBT is not provided because the fairness cannot be achieved in this case as described previously.

Figure 8. The average UPT of SL-U under different cases

The corresponding raw data of Figure 8 is provided in Table 2.

Table 2. Evaluation results of average UPT of SL-U under different cases

| Traffic loading | Low load BO range for NR-U in NR-U + NR-U: 10%~25% |  |  |  | Medium load BO range for NR-U in NR-U + NR-U: 35%~50% |  |  |  | High load BO range for NR-U in NR-U + NR-U: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | SL-U in SL-U+ SL-U | SL-U in NR-U+ SL-U |  |  | SL-U in SL-U+ SL-U | SL-U in NR-U+ SL-U |  |  | SL-U in SL-U+ SL-U | SL-U in NR-U+ SL-U |  |  |
| SL-U Max Tx power [dBm] | 18 | 18 | 5 | 5 | 18 | 18 | 5 | 5 | 18 | 18 | 5 | 5 |
| SL-U LBT | O | O | O | × | O | O | O | × | O | O | O | × |
| UPT [Mbps] | 70 | 72 | 75 | 101 | 16 | 33 | 42 | 71 | 3.8 | 9 | 15 | 49 |
| Delay [ms] | 10.7 | 10.3 | 9.0 | 5.9 | 130 | 56.7 | 49.5 | 9.4 | 392 | 320 | 212 | 16.7 |
| BO | 17% | 16% | 15% | 8% | 84% | 63% | 54% | 29% | 99% | 89% | 83% | 56% |

It can be observed from Figure 8 and Table 2 that the UPT performance of SL-U in the coexistence scenario can achieve highest value under the case of lower max Tx power (5dBm, VLP) and no LBT operation. The major reason is that under VLP mode (i.e., 5dBm), the limited coverage range of SL-U results in a very low interference level, and the no LBT channel access procedure reduces the latency to start a transmission on the channel, which further improve the system throughput of SL-U.

Observation 23: Compared with higher SL-U max Tx power (e.g., 18dBm), the UPT performance of SL-U can be improved for the case of lower SL-U max Tx power (e.g., 5dBm) together with no LBT operation in the coexistence scenario.

VLP operation with XR clouding gaming traffic

The XR clouding gaming traffic is also simulated to further evaluate the ability of SL-U VLP operation to support XR traffic type. The data rate of XR used in the simulation is 30Mbps. The PER and PDB are set as 99% and 15ms, respectively. According to TR38.838, UE satisfaction and system capacity are used as the performance metric for XR traffic evaluation, which is also detailed below [8]:

- UE satisfaction: A UE is declared as a satisfied UE if all the considered streams meet their own PER and PDB requirements, i.e., more than a certain percentage of packets are successfully transmitted within a given air interface PDB.

- System capacity: System capacity is identified as KPI for capacity study, which is defined as the maximum number of users per cell with at least Y% of UEs being satisfied, where Y=90 (baseline) or 95 (optional).

The evaluation results of SL-U in the modes of non-VLP (i.e., 18dBm max Tx power) and VLP (i.e., 5dBm max Tx power) are presented in Figure 9 and Figure 10, respectively. Besides, in Figure 10, we also provide the evaluation results of “decode 1 SCI” and “decode 2 SCI” to compare the effect of the decoded SCI number on the performance SL-U.

Figure 9. Satisfied UE rate of XR traffic for SL-U @18dBm with LBT operation

Figure 10. Satisfied UE rate of XR traffic for SL-U @5dBm without LBT operation under different SCI decoding number

It can be observed from Figure 9 that for the case SL-U max Tx power is 18dBm and LBT operation is executed, the satisfied UE rate can meet the requirement of 95% only when the SL-U pair number is relatively small (i.e., 1 to 3 pairs in Figure 9). While with the increased pair number of SL-U, the satisfied UE rate will degrade significantly (i.e., 4 to 6 pairs in Figure 9) due to the increased latency and interference. In Figure 10, it can be observed that for the case that SL-U max Tx power is 5dBm and LBT operation does not executed, the satisfied UE rate can meet the requirement of 95% even with the SL-U pairs number is relatively large (i.e., 4 to 8 pairs in Figure 10). Besides, if the SCI decoding number can be increased from 1 to 2, the satisfied UE rate can be further increased and meet the requirement of 95% even with the SL-U pairs number is large (i.e., 9 and 10 pairs in Figure 10).

Observation 24: Compared with higher SL-U max Tx power (e.g., 18dBm), the SL-U with lower max Tx power (e.g., 5dBm) can better support XR traffic with an increased UE satisfaction rate and system capacity.

Observation 25: Compared with SL-U with 1 SCI decoding number, 2 SCI decoding number can improve the UE satisfaction rate and system capacity especially for larger SL-U pair number.

Summary

Observation 1: Observation period is not defined in legacy NR-U for the case of performing Type 2A channel access for discovery burst transmission without a shared channel occupancy.

Observation 2: The resource configuration of PSFCH (e.g., (pre-)configured or dynamically indicated) may have impact on the configuration of CPE starting position (e.g., (pre-)configured in each RP, pre-defined or indicated).

Observation 3: Synchronized transmission for SL-U results in intra-cell collision between initial transmissions outside a COT.

Observation 4: Sensing slot level channel access together with multiple CPE starting positions can better avoid intra-cell initial transmission’s collision outside a COT.

Observation 5: Different CPE starting positions can be used to generate different gaps between two transmissions inside a COT.

Observation 6: It is agreed that Type 2A / 2B / 2C channel access can be applied to different gaps between two transmissions inside a COT.

Observation 7: For the case of multiple CPE starting positions, a default CPE starting position can be used before the reserved resource to achieve FDM transmission. A single CPE starting position can be selected before the non-reserved resource to avoid initial transmission collision

Observation 8: FDM transmission of PSCCH/PSSCH from multiple UEs can be achieved by defining a default CPE starting position from the set of multiple CPE starting positions. It is unnecessary to define a single CPE starting position for PSCCH/PSSCH transmission.

Observation 9: The resource configuration (e.g., (pre-)configured or dynamically indicated) for PSFCH transmission may have impact on whether a responding UE can transmit PSFCH(s) to the UE(s) other than the initiator when using the shared COT.

Observation 10: Option 2 can be realized by triggering Option 1 multiple times.

Observation 11: The information of CAPC and consecutive slots number of MCSt can assist L1 to better select resources for MCSt.

Observation 12: Enhanced mechanisms (e.g., RSRP adjustment) are needed for L1 to achieve resource selection of multi-consecutive slots.

Observation 13: The CAPC value of PSFCH may have impact on the utilization of Type A/Type B NR-U DL multi-channel access for PSFCH transmission.

Observation 14: The uncertainty of LBT may introduce additional time consumption for legacy SL resource allocation and thus may invalidate the scheduled or configured resources in Mode 1.

Observation 15: For SL-U Mode 1 resource allocation, the resource assignment at gNB side may be impacted by channel access information (e.g., contention window size), which is available at the UE side.

Observation 16: The uncertainty of LBT operation may introduce additional time consumption for legacy SL resource allocation and thus may invalidate the selected resources in Mode 2 and degrade the UPT performance of the system.

Observation 17: Essential enhancements should be studied to solve the timeline issue for SL resource allocation procedures due to additional time consumption introduced by LBT operation.

Observation 18: The consistent LBT failure based RLF detection in legacy NR-U should be supported as one RLF trigger in SL-U.

Observation 19: The LBT failure at Rx UE side before the PSFCH occasion(s) can result in the absence of PSFCH reception(s) at the Tx UE side and may further incorrectly trigger HARQ-based RLF detection.

Observation 20: In indoor scenario with symmetric traffic at low/medium/high loads in 20MHz bandwidth at 5GHz, the evaluation results of UPT show that the fairness coexistence between NR-U and SL-U can be achieved for the case of SL-U with 18dBm max Tx power and enabled LBT operation.

Observation 21: LBT is necessary to stabilize system interference especially for non-coordinated SL-U deployment for the case of SL-U with 18dBm max Tx power.

Observation 22: Compared with higher SL-U max Tx power (e.g., 18dBm), the UPT performance of NR-U can be improved for the case of lower SL-U max Tx power (e.g., 5dBm) in coexistence scenario.

Observation 23: Compared with higher SL-U max Tx power (e.g., 18dBm), the UPT performance of SL-U can be improved for the case of lower SL-U max Tx power (e.g., 5dBm) together with no LBT operation in the coexistence scenario.

Observation 24: Compared with higher SL-U max Tx power (e.g., 18dBm), the SL-U with lower max Tx power (e.g., 5dBm) can better support XR traffic with an increased UE satisfaction rate and system capacity.

Observation 25: Compared with SL-U with 1 SCI decoding number, 2 SCI decoding number can improve the UE satisfaction rate and system capacity especially for larger SL-U pair number.

Proposal 1: UE performs Type 1 channel access procedures for PSFCH transmissions without a shared channel occupancy and Type 2 channel access procedures within a shared channel occupancy.

Proposal 2: If a UE determines the duration in time domain and the location in frequency domain of a remaining COT initiated by COT initiator, the UE may switch from Type 1 channel access procedures to Type 2A channel access procedures for its corresponding SL transmissions within the determined resources of the remaining COT.

- FFS details (e.g., CAPC level, transmission/channel type, ID information).

Proposal 3: Multiple CPE starting positions should be (pre-)configured for S-SSB transmission.

Proposal 4: Multiple CPE starting positions are supported outside a COT.

Proposal 5: Multiple CPE starting positions are supported inside a COT.

Proposal 6: Resource reservation information can be used as a criterion for defining a default CPE starting position from the set of multi-CPE starting positions. The resource reservation information includes both the resource reservation information from the CPE transmitting UE and other UE(s)

- If resource reservation information of the transmission resource slot is available, UE’s transmission is started from the default CPE starting position.

- If resource reservation information of the transmission resource slot is not available, UE’s transmission is started from a subset of multi-CPE starting positions based on the traffic priority. UE randomly chooses a single CPE starting position from the subset of multi-CPE starting positions.

Proposal 7: CAPC value can be used as a criterion for selecting one of the multiple CPE starting positions.

Proposal 8: Only support multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH transmission.

Proposal 9: Single CPE starting position can be achieved by the indication of COT initiating UE inside a COT.

Proposal 10: For (pre-)configured PSFCH resource within a shared COT, a responding UE can transmit PSFCH(s) to UE(s) other than the initiator. For dynamically indicated PSFCH resource within a shared COT, a responding UE cannot transmit PSFCH(s) to UE(s) other than the initiator.

Proposal 11: COT sharing information should include information of CPE starting position, channel access type, L1 ID, CAPC level, remaining COT duration, RB set(s) in the COT, starting offset of the shared COT, and responding UE’s transmission.

Proposal 12: The information of CAPC value and consecutive slots number should be provided when L1 is triggered for reporting a subset of candidate resources for MCSt.

Proposal 13: Option 1 and Option A below are preferred for MCSt operation in SL-U, and enhanced mechanisms should be studied when L1 reports a subset of candidate resources for MCSt.

- When L1 is triggered for reporting a subset of candidate resources for MCSt,

  - Option 1: Only one set of parameters (, remaining PDB,  and ) is provided for the resource selection procedure in L1

- When L1 reports a subset of candidate resources for MCSt,

  - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time

Proposal 14: For SL-U, RAN1 should study essential enhancements for resource allocation due to additional channel access procedure.

Proposal 15: The transmission of S-SSB on frequency domain should be restrained within one RB set, thus multi-channel access is not supported for S-SSB transmission.

Proposal 16: Mode 1 UE can report channel access information (e.g., contention window size) to gNB for aiding Mode 1 resource allocation.

Proposal 17: Study enhancements (e.g., overbooking/protection margin) for SL-U resource allocation to combat the impact introduced by additional channel access procedure.Proposal 21: For the gap between LBT end and SL transmission start, the repetition of SL transmission can be used to retain the channel.

Proposal 18: For the gap between LBT end and SL transmission start, the repetition of SL transmission can be used to retain the channel.

Proposal 19: Study whether/how to support VLP operation for SL-U.

Proposal 20: Study solutions to combat the impact of LBT failure on the RLF detection in SL-U.

Reference

[1] Chair's notes RAN1#109-e

[2] Chair's notes RAN1#110

[3] Chair's notes RAN1#110b-e

[4] Chair's notes RAN1#111

[5] Chair's notes RAN1#112

[6] 3GPP TS 37.213, Physical layer procedures for shared spectrum channel access

[7] 3GPP TS 38.331, NR Radio Resource Control (RRC) Protocol specification

[8] 3GPP TR 38.838, Study on XR (Extended Reality) evaluations for NR

[9]	RP-213678, “New WID on NR sidelink evolution”, RAN #94, OPPO, LG Electronics

- Appendix 1

Table 3. Summary of evaluation configurations for indoor scenario at 5GHz

| Carrier frequency | 5GHz |
| --- | --- |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| SCS | 30KHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm |
| NR-U UE/STA Tx Power | 18dBm |
| SL-U UE Tx Power | 18dBm 5dBm (for the evaluation of VLP) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for NR-U UE dropping | -82dBm |
| SL-U pairing RSRP threshold | -82dBm |
| CCA-ED | -72dBm |
| Max COT length | 6ms |
| UE receiver | MMSE-IRC |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |