3GPP TSG RAN WG1 #112-bis-e	R1-2303370

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	5

Source:	MediaTek Inc.

Title:	Discussion on RAN2 LS on SL resource (re)selection

Document for:	Discussion and Decision

- Introduction

A RAN2 LS was sent to RAN1 [1] indicating that the impact of LBT failure to the SL resource (re)selection procedure is discussed by RAN2. It is RAN2 understanding, that UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission. The issue is whether such new resource (re)selection trigger is also applicable for the multiple consecutive slots transmission (MCSt) case. Therefore, RAN1 is asked to take the above information into consideration and to provide feedback, if there are any concerns with such behavior. The original text of the RAN2 LS is copied below

(below is copied from RAN2 LS R2-2302036) 1. Overall Description: RAN2 discussed the impact of LBT failure to the SL resource (re)selection procedure. It is RAN2 understanding, that UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission. It is FFS whether such new resource (re)selection trigger is also applicable for the multiple consecutive slots transmission (MCSt) case. 2. Actions To RAN1 group. ACTION: RAN2 kindly asks RAN1 to take the above information into consideration and to provide feedback, if there are any concerns with such behavior.

RAN1 is requested to provide the feedback on the above Actions regarding the impact of LBT failure indication to SL resource (re)selection for MCSt case. In this contribution, the issue of LBT failure impact on resource (re)selection is analysed.

- Discussions

RAN1 has made following agreements on MCSt case of SL-U in RAN1 #110b-e:

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB,LsubCh andPrsvp_Tx) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCh and Prsvp_Tx) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCh sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCh sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

Currently, RAN1 has no consensus on the utilization of MCSt, e.g., whether it is used for one TB transmission, and/or multiple TB transmissions, or both. From our perspective, that may have impact on the resource (re)configuration.

- If MCS is used for multiple TBs, e.g., each slot of the MCS is used for the transmission of one TB. It further implies each LBT failure indication corresponds to a failed transmission of one TB. In this case, a resource (re)selection shall be triggered to (re)select a new resource for the TB with LBT failure indication.

- If MCS or partial slots of MCS is used for one TB, i.e., multiple slots for the transmission of one TB. In this way, the reserved slots number may larger than the actually slot number required for one TB transmission, i.e., overbooked slots for one TB to combat the potential LBT failure. In this case, the resource (re)selection shall not be triggered when LBT failure indication from lower layer is detected unless it is before the last overbooked slot for one TB transmission.

Furthermore, the subset of candidate resources L1 reports to higher layers for MCSt may also impact the resource (re)selection procedure.

- If Option A is used, only candidate multi-slot resources (a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time) is available in the reported subset of candidate resource from L1. Therefore, the resource (re)selection may based on a granularity of multi-slot resources (e.g., a new MCS will be (re)selected even only one or partial slot(s) of original MCS encountered with LBT failure).

- If Option B is used, candidate single-slot resources is available in the reported subset of candidate resource from L1. Therefore, the resource (re)selection may based on a granularity of single-slot resource (e.g., a new single or partial slot(s) will be (re)selected for the case of single or partial slot(s) of original MCS encountered with LBT failure, respectively)

In summary, from RAN1 perspective, the LBT failure indication can be used as a trigger of resource (re)selection for both single and multiple consecutive slots transmission. Ans the usage of MCSt (e.g., for one or multiple TB) may have further impact on the resource (re)selection, which is actually up to RAN2.

Proposal 1:  For the impact of LBT failure on SL resource (re)selection, reply to RAN2 as follows:

- Regarding LBT failure indication for the case of MCSt, whether it can be used as a new trigger for resource (re)selection depends on the MCS is used for one TB transmission and/or multiple TB transmissions.

Additionally, it is not clear from our side what is the specific behavior of resource (re)selection if the LS. In our understanding, it may be involved with the following options:

- Option 1: (Re)select different resource (e.g., slot) within the same RB set for which LBT failure is indicated.

- Option 2: (Re)select different RB set for which LBT failure has not been indicated.

The issue for Option 2 may be

- Assuming a total of 80MHz bandwidth, only 4 RB sets are available. If each LBT failure indication triggers a switch of RB set, it may be easy to find all RB sets are indicated with a LBT failure.

- For legacy NR-U, only when consistent LBT failure is triggered, the UE will switch the active UL BWP to a UL BWP for which consistent LBT failure has not been triggered.

Proposal 2: It should be clarified by RAN2 the resource (re)selection is for different resource within the same RB set for which LBT failure is indicated; and/or for different RB set for which LBT failure has not been indicated.

Besides, per RAN2 agreement, the granularity of LBT failure indication is per RB set. If it is RAN2 consensus that the resource (re)selection is for different RB set, how to accumulate the LBT failure counter in this case should be clarified.

Proposal 3: It should be clarified by RAN2 the accumulation method of LBT failure counter if the resource (re)selection is aiming for different RB set.

- Conclusions

Based on the discussion and analysis above, following proposals are given.

Proposal 1:  For the impact of LBT failure on SL resource (re)selection, reply to RAN2 as follows:

- Regarding LBT failure indication for the case of MCSt, whether it can be used as a new trigger for resource (re)selection depends on the MCS is used for one TB transmission and/or multiple TB transmissions.

Proposal 2: It should be clarified by RAN2 the resource (re)selection is for different resource within the same RB set for which LBT failure is indicated; and/or for different RB set for which LBT failure has not been indicated.

Proposal 3: It should be clarified by RAN2 the accumulation method of LBT failure counter if the resource (re)selection is aiming for different RB set.

# References

- R2-2302036, “LS to RAN1 on SL resource (re)selection”, RAN2, 3GPP TSG-RAN WG2 Meeting #121, February, 2023.