3GPP TSG RAN WG1 #112-bis-e	R1-2303371

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	5

Source:	MediaTek Inc.

Title:	Discussion on RAN2 LS on comparison of SL-RSRP and SD-RSRP measurements

Document for:	Discussion and Decision

- Introduction

A RAN2 LS was sent to RAN1 [1] indicating that during the work on session continuity enhancements within NR_SL_relay_enh work item, a new measurement report trigger has been proposed yet with no consensus in RAN2:

Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE.

The key issue here is if the new event requires the comparison of SL-RSRP (RSRP measurement on unicast PC5 link) and SD-RSRP measurements, whether the comparison can be used for the purposes of triggering a measurement report.

The original text of the RAN2 LS is copied below

(below is copied from RAN2 LS R2-2302234) 1. Overall Description: During the work on session continuity enhancements within NR_SL_relay_enh work item a new measurement report trigger has been proposed yet with no consensus in RAN2: Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE This new event requires either the comparison of SD-RSRP (RSRP measurement on broadcast PC5 link used for discovery messages) measurements from different UEs, or the comparison of SL-RSRP (RSRP measurement on unicast PC5 link) and SD-RSRP measurements. During the discussion some companies expressed concerns on direct comparison of SD-RSRP and SL-RSRP due to the power control mechanism applied on PC5 unicast links, while other companies think the network can handle such comparison by using an appropriate offset in the measurement event. Before agreeing to the introduction of this measurement report trigger, RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements: Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report? 2. Actions To RAN WG1 and RAN WG 4 ACTION: RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements, more specifically to the question above.

RAN1 is requested to provide the feedback on the feasibility of the comparisons of RSRP measurements. In this contribution, this issue is analysed.

- Discussions

From RAN1 perspective, it is confused whether/how to compare the SL-RSRP and SD-RSRP measurement considering the two measurement is based on different mechanisms. Specifically, the SL-RSRP may be measured based on PC5 link with TPC, while the SD-RSRP measurement may not be involved with TPC. Besides, whether the accuracy of the SD-RSRP measurement (e.g., due to the variance of bandwidth) can meet the requirement is also not clear. Considering these observations, maybe the criterion on the comparison result should be clarified at first by RAN 2. For example, it is used for the comparison of SINR at Rx UE side and/or for the pathloss, etc. Otherwise, it is hard for RAN1 to say whether the  comparison of SL-RSRP and SD-RSRP measurement is feasible or not.

Proposal 1: For the comparison of SL-RSRP and SD-RSRP measurement, reply RAN2 as follows:

- It should be clarified by RAN2 at first the criterion on the comparison between SL-RSRP and SD-RSRP.

- Conclusions

Based on the discussion and analysis above, following proposal is given.

Proposal 1: For the comparison of SL-RSRP and SD-RSRP measurement, reply RAN2 as follows:

It should be clarified by RAN2 at first the criterion on the comparison between SL-RSRP and SD-RSRP.

# References

- R2-2302234, “LS on Comparison of SL-RSRP and SD-RSRP measurements”, RAN2, 3GPP TSG-RAN WG2 Meeting #121, February 2023.