3GPP TSG RAN WG1#112bis-e                          R1- 2303374

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.1

Source:	Transsion Holdings

Title:	Discussion of channel access mechanism for sidelink in unlicensed spectrum

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, a new WID had been approved. And in RAN#95e meeting, this WID had been revised as the following:[1]

- Objective of SI or Core part WI or Testing part WI To check in RAN#97 for objectives 1 and 3, taking into account the progress on objectives 2 and 4, aiming to have specification work for both objective 1 and 3. ...... - Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - ......

In this contribution, we focus on the channel access mechanism for sidelink in unlicensed spectrum, including EDT determination, channel access procedures, CAPC and CW adjustment, CP extension, COT sharing, multi-channel access, multi-consecutive slots transmission, long term sensing and FBE operation.

Discussion

EDT determination

Energy detection is the only criterion to determine whether the channel is busy or not, so the setting of the energy detection threshold (EDT) will greatly affect the probability of successful channel access. In NR-U/LAA, the setting of the threshold value depends on several parameters, such as channel bandwidth, transmit power, etc.. Another factor is whether carrier frequencies are shared with other wireless access technologies (e.g., WiFi) for long-term, or whether the network deployment can guarantee that the carrier frequency are only used by NR. When NR coexists with other wireless access technologies on the same carrier frequency, the maximum threshold for a carrier with a 20 MHz bandwidth is -72 dBm. When NR is the only technology using carrier frequency, for a carrier with 20 MHz bandwidth, the threshold is set to -62 dBm unless a lower threshold is required by the regulatory requirements. For uplink transmissions of NR-U/LAA, RRC signaling can also be used to configure thresholds to meet regulatory requirements.

For sidelink unlicensed access system, the transmitting power and operating bandwidth of both transmitting and receiving UE are similar to the UE of NR system. Therefore, the EDT determination method for NR-U/LAA uplink can be used as a starting point for the study of EDT determination method for sidelink unlicensed access system.

Proposal 1: The EDT determination method for NR-U/LAA uplink can be used as a starting point for the study of EDT determination method for sidelink unlicensed access system.

Channel access procedures

Multiple nodes (eNB/gNB/UE/WiFi AP) using different radio access technologies need to coexist on unlicensed spectrum. And the coordination of resource scheduling among these nodes is not possible. To share the unlicensed spectrum fairly with LAA/WiFi, new channel access mechanisms are introduced in NR-U to assist in resource scheduling. Two channel access procedures are defined in NR-U specification: dynamic channel access procedure and semi-static channel access procedure.

The key technology for the dynamic channel access procedure is listen-before-talk (LBT), in which the transmitter needs to evaluate the channel availability prior to transmission and uses random back-off time in case of conflict. Two types of dynamic channel access procedure are defined in the NR specification: Type 1 DL/UL channel access procedure and Type 2 DL/UL channel access procedure. The former is used to start DL or UL transmission at the beginning of the COT, while the later is used for COT sharing and DRS transmission. According to the duration of the transmission gap in the COT, Type 2 channel access procedures has three sub-types:

- Type 2A, used when the transmission gap in the COT is 25us or greater (FBE), or for DRS transmission;

- Type 2B, used when the transmission gap in the COT is 16us;

- Type 2C, used when the transmission gap in the COT is up to 16us;

As analyzed above, in order to fairly co-exist with NR-U/LAA/WiFi system, channel access procedure should be introduced for the sidelink unlicensed access system. It is beneficial to use the channel access procedure defined in NR-U as a starting point for the study of channel access procedures in sidelink unlicensed access system. This is because it saves the workload of standardization.

It had been agreed at RAN1#109-e meeting that Type 1 and Type 2 (2A/2B/2C) channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are taken as baseline for NR sidelink operation in a shared channel. A typical application scenario for Type 1 channel access procedure is to initiate a sidelink transmission at the beginning of the COT. Similarly, for Type 2 (2A/2B/2C) channel access procedure, the typical application scenario is applied to sidelink transmission within the COT.

According to the European regulation (ETSI EN 301 893) [2], short control signaling can be used with the following limitations:

“Within an observation period of 50ms, the number of Short Control Signalling Transmissions by the equipment shall be equal to or less than 50;

and the total duration of the equipment's Short Control Signalling Transmissions shall be less than 2 500 µs within said observation period.”

According to the limitation of short control signaling, only some relatively sparse signals/channels can use the short control signaling transmission mechanism. Considering the short transmission time of PSFCH, e.g., less than one slot, their use of short control signaling mechanism is expected to have little impact on the performance of other RATs.

In RAN1#111 meeting, RAN1 made the following agreement [3] to support Type 2A channel access procedure for S-SSB transmissions from a UE without a shared channel occupancy with a time duration and a duty cycle constraint. But the applicability of using the same channel access mechanism (Type 2A without a shared COT) for PSFCH transmission is left for further study.

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

In NR-U, the short control signaling mechanism is used for discovery burst transmission (SSB) from the gNB. However, in NR-U, the gNB still needs to perform a Type 2A channel access procedure to ensure that the channel is idle before the discovery bust transmission. It is understood to be particularly friendly to transmissions from other RATs coexisting in the same shared channel. In addition, in order to fairly co-exist with NR-U in the same shared channel, the short control signaling (i.e. PSFCH) should use the same transmission mechanism used in NR-U, i.e., perform Type 2A channel access procedure prior to PSFCH transmission.

Proposal 2: Type 2A channel access procedure is used for PSFCH transmission.

CAPC

In RAN1#110bis-e meeting, the following agreements were made [4]:

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

In NR-U, the channel access priority of the downlink transmission is different from that of the uplink transmission, which means that the downlink transmission has a shorter defer duration than the uplink transmission. Therefore, in high-load scenarios, downlink transmission has a slightly higher probability of occurring than uplink transmission. In other words, a gNB serving multiple UEs has a slightly higher probability of winning channel competition compared to a UE serving only one gNB for uplink transmission. Similarly, in broadcast or groupcast, COT is initiated by one UE, while in unicast it is initiated by multiple UEs. From a fairness point of view, different mp values can be configured here for the same CAPC value to enable priority or non-priority channel access based on different COT intentions. Therefore, mp=1 can be used with p=1, and it is up to UE implementation to apply mp=1 or mp=2 for S-SSB/PSFCH/PSCCH/PSSCH transmission.

Proposal 3: mp=1 can be used with p=1, and it is up to UE implementation to apply mp=1 or mp=2 for S-SSB/PSFCH/PSCCH/PSSCH transmission.

Contention window adjustment

In RAN1#112 meeting, the following agreement was made [5]:

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

In NR-U, the end timing of DL reference duration can be either the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. The concept of MCSt is very similar to the “transmission burst”. Following the same principle, the end of the MCSt transmission that contains at least one PSSCH with ACK/NACK HARQ-ACK enabled can be defined as the end timing of SL reference duration.

Proposal 4: For MCSt, the end timing of SL reference duration can be the end of the MCSt transmission that contains at least one PSSCH with ACK/NACK HARQ-ACK enabled.

In RAN1#110bis-e meeting, the following agreement was made [4]:

Agreement ...... - FFS whether groupcast option 1 (NACK-only) with SL-HARQ feedback enabled can be supported for SL-U. If supported, further study the following options (at least if all transmissions within the latest SL reference duration are groupcast option 1 transmissions) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: - If ‘NACK’ or a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - When neither ‘NACK’ nor a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, - Option A: CWp is reset to CWmin,p for every priority class p∈1,2,3,4. - Option B: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 3: An ACK-only procedure is used instead of a NACK-only procedure. In this case, if at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p, otherwise CWp is increased - Option 4: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 5 (option 3+legacy): ACK feedback is performed when a TB is successfully decoded in addition to the legacy NACK-only procedure. In this case, if ACK only is received related to any transmissions within the latest SL reference duration then CWp=CWmin,p, otherwise CWp is increased. - CW adjustment for unicast with SL-HARQ feedback enabled (at least In case only unicast PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 2: If at least one ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p ; otherwise CWp is increased. - FFS the case when UE is operating with different SL-HARQ feedback schemes (e.g., UE has concurrent broadcast transmission + unicast with SL-HARQ enabled, or GC option 1 + GC option 2, etc in the SL reference duration).

For the case of groupcast option 1 (NACK-only based SL feedback), the SL Rx UE can only send ‘NACK’ in the same PSFCH resource when the corresponding PSSCH is not correctly decoded. Then the contention window size adjustment procedure based on HARQ-ACK feedback cannot be used at SL Tx UE side, because only NACK will be received at Tx UE side. In this case, if any NACK is received, the legacy principle of increasing CW for every priority class can be reused. For the case where the SL Tx UE does not receive HARQ-NACK feedback, it is most likely because the sidelink channel conditions between the SL Tx UE and all SL Rx UEs are good. And that all SL Rx UEs are interfered is unlikely happened. Therefore, in case of no ‘NACK’ is received, reset CWp to CWmin,p.

Proposal 5: If ‘NACK’ is received, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

- When no ‘NACK’ is received related to any transmissions within the latest SL reference duration,

  - CWp is reset to CWmin,p for every priority class p∈1,2,3,4.

CP extension

It can be found that in a sidelink slot, there is a guard symbol at the end of the PSSCH channel or PSFCH channel. And for 15kHz/30kHz SCS, the guard symbol can be 70.3us/35.2us respectively, which is greater than 25us. In unlicensed spectrum, when the gap is greater than 25us, the next transmission will need to use a Type 1 channel access procedure, which increases the probability of losing the channel. In order to use the Type 2 channel access procedure, it is necessary to ensure that the gap between two consecutive transmissions is small enough, even less than the duration of a symbol. This can be solved by CP extension that makes the transmission start earlier than the symbol boundary.

In NR-U, three types of CPE are introduced for different scenarios, of which, two types of CPE include TA values. For COT sharing, to ensure that the interval between transmission bursts at the COT initiating UE is 16us/25us, the value of timing advance should be considered when calculating CPE. Considering that the COT initiating UE and the responding UE may have different TA values, the TA values of both the COT initiating UE and the responding UE should be considered.

Proposal 6: For COT sharing, the TA values of both the COT initiating UE and the responding UE should be considered when calculating CPE.

On the other hand, in case of consecutive SL transmissions, since they come from the same SL Tx UE, the TA value does not need to be considered when calculating the CPE.

Proposal 7: In case of consecutive SL transmissions, the TA value does not need to be considered when calculating the CPE.

In RAN1#112 meeting, the following agreement was made [5]:

Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

For the PSFCH transmission within a COT, the CPE is used to create a gap equal to 25us or 16us, or even less than 16us. Considering there is only one guard symbol before the PSFCH symbol in a sidelink slot, Option 1 should be applied for PSFCH transmission within a COT. Similarly, for COT sharing, the CPE of PSCCH transmission/PSSCH transmission should be limited to one symbol before the AGC symbol.

On the other hand, for out of COT transmissions, if the same resources are configured for different PSCCH transmissions/PSSCH transmissions, different lengths of CPE can be used to block the LBTs of different PSSCH transmissions, thus avoiding interference between different PSSCH transmissions.

Proposal 8:

- For COT sharing, the CPE of PSCCH/PSSCH/PSFCH transmission should be limited within one symbol just before the next AGC symbol.

- For out of COT transmissions, the CPE of PSCCH/PSSCH/PSFCH transmission can be limited within at most 2 symbols just before the next AGC symbol.

COT sharing

In RAN1#111 meeting, the following agreement was made [4]:

Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE.

Regarding PSFCH transmission, when a UE needs to transmit multiple SL HARQ feedback in a PSFCH occasion, the SL HARQ feedback may be for different PSSCH transmissions from different UEs. In this case, some SL HARQ feedback may be within the same COT as the PSSCH transmitting UE, while some may be outside the COT. Therefore PSFCH transmitting UEs may need to perform different types of LBTs that have different sense lengths and may result in using different starting positions of CPEs to access the channel. In this sense, it is not fair for the PSFCH transmitting UE to self-block its own transmission. Therefore, a better approach is that when a responding UE is transmitting at least one PSFCH intended for the COT initiating UE, the responding UE is able to use the COT to send SL HARQ feedback to other UEs in the same PSFCH occasion.

Proposal 9: When a responding UE is transmitting at least one PSFCH intended for the COT initiating UE, the responding UE is able to use the COT to send SL HARQ feedback to other UEs in the same PSFCH occasion.

Multi-channel access

In RAN1#111 meeting, the following agreement was made [3]:

Agreement - For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

In NR-U, for the DL multi-channel access procedure, the gNB is required to perform LBT independently on each shared channel or RB set that the gNB intends to transmit. gNB transmits on the channel on which the channel access procedure succeeds. On the other hand, for the UL multi-channel access procedure, the UE needs to perform LBT on all shared channels or RB sets that it intends to transmit. The UE can transmit only if the channel access procedure is successful for all channels. Otherwise, the UE does not transmit on any channel.

DL multi-channel access procedure can also be applied if S-SSB can be configured within each RB set and multiple S-SSBs are supported within multiple RB sets.

Proposal 10: The NR-U DL multi-channel access procedure should be used as baseline for the transmission of S-SSB for multiple SSB transmissions on multiple channels.

Multi-consecutive slots transmission

The motivation for supporting multi-contiguous slots transmission is mainly to reduce the frequency of UEs performing LBTs to access the channel after acquiring the COT, and to preserve the COT as much as possible to transmit the data of the sidelink UEs. Also the mechanism of multi-contiguous slots transmission ensures that the data of the sidelink UE is transmitted in the next slot as soon as possible. It would be beneficial to support this feature in the SL-U operation of Rel-18, but given the potential standardization efforts, the main intent needs to be achieved while keeping the mechanism as simple as possible. That is, to reuse existing sidelink designs as much as possible. For example, the TRIV, FRIV and MCS should be the same for different transmissions scheduled by a single SCI.

Proposal 11: For multi-consecutive slots transmission, reuse the existing sidelink designs as much as possible.

In RAN1#110bis-e meeting, the following agreement was made [4]:

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

According to the existing mode 2 resource selection procedure, L1 layer reports a subset of candidate single-slot resources that can be selected for transmission based on a set of parameters (L1 priority, PDB, number of subchannels, and reservation period) provided by the upper layer. These resources are then randomly selected at the upper layer for the initial and re-transmission of a single TB. Therefore, there is no guarantee that the selected resources can and will be in consecutive slots. In addition, MCSt should support multi-TB transmission (in addition to single-TB transmission). In this case, multiple sets of upper layer parameters should be provided to L1 for candidate resource reporting. At the last meeting, issues regarding MCSt’s Mode 2 resource selection procedure were discussed and several of these options were agreed to be selected in the future meeting.

In NR sidelink, the UE determines the re-transmission number by the UE implementation, where the maximum re-transmission number is (pre-)configured in the resource pool. Following the same design principle, only one set of parameters should be provided for the resource selection procedure in L1 (Option 1). Regarding Option 2, if multiple sets of parameters are provided for the resource selection procedure in L1, this would lead to a more complex resource selection procedure, as the L1 layer would need to decide which set of parameters to use in the resource exclusion procedure; or do a separate resource exclusion procedure based on each set of parameters. Both approaches would have many impacts on the specification.

Proposal 12: When L1 is triggered for reporting a subset of candidate resources for MCSt, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

Regarding option A, the UE can decide the number of consecutive slots for MCSt, so the sensing granularity changes from a single-slot candidate resource (NR sidelink baseline) to a multi-slot candidate resource. Regarding option B, the L1 layer does not know how many such multi-slot candidate resources are available, and therefore may provide the MAC with a resource set in which no or only a few single-slot candidates are contiguous. Based on the above analysis, option A can guarantee that enough multi-slot candidate resources will be reported, while option B cannot. Therefore, to maintain randomness and avoid potential resource collisions as much as possible, option A should be supported.

Proposal 13: When L1 reports a subset of candidate resources for MCSt, L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time.

Long term sensing

In NR-U, L3 RSSI measurement and channel occupancy reporting from UE have been introduced to reduce or mitigate the impact of interference e.g. from hidden nodes. L3 RSSI measurement is a indicator of the channel occupancy and it can reflect the interference status from neighbouring nodes. gNB can effectively allocate resources in unlicensed spectrum for the DL/UL transmission based on the channel occupancy reporting.

In sidelink operation, the resource pool is configured by gNB. Further, for resource allocation Mode 1, gNB schedules resources for sidelink transmission. However, in sidelink unlicensed access system, gNB does not operate on the unlicensed spectrum, so the gNB does not know the interference situation on the unlicensed spectrum. If the gNB lacks the information about the interference in the unlicensed spectrum, then the gNB can not effectively allocate resources in the resource pool for the sidelink transmission. Hence, L3 RSSI measurement and channel occupancy reporting from sidelink UE should be supported in sidelink unlicensed access system.

Proposal 14: L3 RSSI measurement and channel occupancy reporting from sidelink UE should be supported in sidelink unlicensed access system.

FBE operation

FBE operation can be applied with the guarantee that the LBE nodes are absent on a long term basis (e.g., by level of regulation). Compared to LBE operation, FBE operation achieves lower channel access complexity because there is no need to perform random fallback. And if FBE nodes are synchronized, the network can achieve the following: Ability to use frequency reuse factor 1. Considering the benefits of lower channel access complexity and higher spectrum efficiency, FBE operation should be studied in sidelink unlicensed access system.

Proposal 15: FBE operation should be studied in sidelink unlicensed access system.

Conclusion

In this contribution, we focus on the channel access mechanism for sidelink in unlicensed spectrum, including EDT determination, channel access procedures, CAPC and CW adjustment, CP extension, COT sharing, multi-channel access, multi-consecutive slots transmission, long term sensing and FBE operation. Based on the discussion in section 2, we provide the following proposals:

Proposal 1: The EDT determination method for NR-U/LAA uplink can be used as a starting point for the study of EDT determination method for sidelink unlicensed access system.

Proposal 2: Type 2A channel access procedure is used for PSFCH transmission.

Proposal 3: mp=1 can be used with p=1, and it is up to UE implementation to apply mp=1 or mp=2 for S-SSB/PSFCH/PSCCH/PSSCH transmission.

Proposal 4: For MCSt, the end timing of SL reference duration can be the end of the MCSt transmission that contains at least one PSSCH with ACK/NACK HARQ-ACK enabled.

Proposal 5: If ‘NACK’ is received, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

- When no ‘NACK’ is received related to any transmissions within the latest SL reference duration,

  - CWp is reset to CWmin,p for every priority class p∈1,2,3,4.

Proposal 6: For COT sharing, the TA values of both the COT initiating UE and the responding UE should be considered when calculating CPE.

Proposal 7: In case of consecutive SL transmissions, the TA value does not need to be considered when calculating the CPE.

Proposal 8:

- For COT sharing, the CPE of PSCCH/PSSCH/PSFCH transmission should be limited within one symbol just before the next AGC symbol.

- For out of COT transmissions, the CPE of PSCCH/PSSCH/PSFCH transmission can be limited within at most 2 symbols just before the next AGC symbol.

Proposal 9: When a responding UE is transmitting at least one PSFCH intended for the COT initiating UE, the responding UE is able to use the COT to send SL HARQ feedback to other UEs in the same PSFCH occasion.

Proposal 10: The NR-U DL multi-channel access procedure should be used as baseline for the transmission of S-SSB for multiple SSB transmissions on multiple channels.

Proposal 11: For multi-consecutive slots transmission, reuse the existing sidelink designs as much as possible.

Proposal 12: When L1 is triggered for reporting a subset of candidate resources for MCSt, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

Proposal 13: When L1 reports a subset of candidate resources for MCSt, L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time.

Proposal 14: L3 RSSI measurement and channel occupancy reporting from sidelink UE should be supported in sidelink unlicensed access system.

Proposal 15: FBE operation should be studied in sidelink unlicensed access system.

Reference

- RP-220300		WID revision: NR sidelink evolution, 	OPPO, 	2022-03

- ETSI EN 301 893

- Draft_Minutes_report_RAN1#111_v020, 2022-11

- Draft_Minutes_report_RAN1#110bis-e_v010, 2022-10

- Draft_Minutes_report_RAN1#112_v020, 2023-03