3GPP TSG RAN WG1#112bis-e                              	                R1- 2303376

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.2

Source:	Transsion Holdings

Title:	Discussion of co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, a new WID had been approved. And in RAN#99 meeting, this WID had been revised as the following: [1]

- Objective of SI or Core part WI or Testing part WI ...... - Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only - ......

In RAN1#110bis-e meeting, the following agreements were made: [2]

Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

In this contribution, we focus on the co-channel coexistence for LTE sidelink and NR sidelink, including the solution of dynamic resource partitioning.

Discussion

Dynamic resource pool partitioning - Use of LTE sensing

A list of parameters that should be shared by the LTE sidelink module has been identified during the last meeting, including the time and frequency locations of LTE transmission, resource reservation periods, SL RSRP and/or SL RSSI measurement results, priority, time and frequency location of resources used for own LTE SL transmissions, candidate resource set SA or SB, LTE logical subframe related information and subframes that are not monitored by the LTE sidelink UE due to half-duplex constraints. Among them, the parameters “the time and frequency locations of LTE transmission, resource reservation periods” are used by the LTE sidelink module to determine the resources reserved for other LTE sidelink UEs. Other parameters such as "SL RSRP measurement results, priority" are used by the LTE sidelink module to exclude resources reserved by other LTE sidelink UEs from the available resources. The last parameter "Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE" can be used to indicate the resources that are not sensed by the LTE sidelink module. In summary, the above parameters are useful for dynamic resource pool sharing. Regarding how and when to share these sensing and resource reservation information, that can be left for implementation.

- Proposal 1: For co-channel coexistence in Rel-18, for the study of dynamic resource pool sharing, the information shared by the LTE SL module to the NR SL module contains at least the following parameters:

- LTE sensing results include

- Time and frequency locations of LTE transmissions

- Resource reservation periods

- SL RSRP measurement results

- Priority

- Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

In RAN1#112 meeting, the following agreement was made: [3]

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied

Some remaining issues about SL RSRP threshold were not addressed in the last meeting. Four alternatives were agreed for down-selecting at the next meeting. It is difficult to achieve a balance between NR SL performance and LTE SL performance with either NR SL RSRP based or LTE SL RSRP based resource exclusion. At this point, it may be difficult to envision whether the LTE threshold or the NR threshold is more appropriate. Considering that Alt 3 is a super set of all three alternatives, Alt 3 may be more suitable.

It has been agreed in RAN#99 that NR SL UE needs to avoid selecting resources for PSCCH/PSSCH transmissions, whose corresponding PSFCH transmission occasions overlap in the time domain with LTE SL reservations. In other words, for a NR SL slot with NR PSFCH, it is not overlapped with LTE SL slots. Therefore, Alt 4 is not necessary..

Proposal 2: For NR SL resource (re)selection procedure, SL RSRP threshold list should be separately (pre)configured for dynamic resource pool sharing.

In RAN1#112 meeting, the following agreement was made: [3]

Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

The issue of identifying NR SL resources that overlap with LTE SL resources used for LTE SL transmission was discussed at the last meeting and two options were agreed for future down-selecting. Regarding Option 1, it is assumed that the LTE SL transmission always has priority over NR SL transmission when NR SL transmission overlaps with LTE SL transmission. This is not practical and will degrade NR SL transmission performance. Regarding Option 2, it is consistent with the in-device coexistence method (i.e. prioritizing one NR SL transmission over LTE SL transmission or prioritizing one LTE SL transmission over NR SL transmission), which is realistic and reduces the standardization efforts.

- Proposal 3: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

- UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

In RAN1#112 meeting, the following proposal was discussed [4], but no consensus was achieved.

Proposal 2-3(I): - In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, all the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved only once for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module

Assuming that LTE SL resources in non-monitored subframes are only repeatedly reserved once, this may result in candidate resource sets containing resources reserved by LTE SL UEs for their own transmissions. This will lead to an increase in resource collisions. The ideal case is to exclude non-monitored resources a predefined number of times, which is the Q value already defined in the NR SL and LTE SL.
Proposal 4:

- In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module

- For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, all the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times according to the formula of Q in TS 36.213 for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool

- The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

Conclusion

In this contribution, we have discussed the co-channel coexistence for LTE sidelink and NR sidelink, including the solution of resource partitioning. Based on the discussion in section 2, we provide the following proposals:

- Proposal 1: For co-channel coexistence in Rel-18, for the study of dynamic resource pool sharing, the information shared by the LTE SL module to the NR SL module contains at least the following parameters:

- LTE sensing results may include

- Time and frequency locations of LTE transmissions

- Resource reservation periods

- SL RSRP measurement results

- Priority

- Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

Proposal 2: For NR SL resource (re)selection procedure, SL RSRP threshold list should be separately (pre)configured for dynamic resource pool sharing.

- Proposal 3: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

- UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

Proposal 4:

- In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module

- For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, all the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times according to the formula of Q in TS 36.213 for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool

- The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

Reference

- RP-230077		WID revision: NR sidelink evolution, 	OPPO, 	2023-03

- Draft_Minutes_report_RAN#110bis-e_v010, 2022-10

- Draft_Minutes_report_RAN1#112_v020, 2023-03

- R1-2301855	Feature lead summary #3 for AI 9.4.2: Co-channel coexistence for LTE sidelink and NR sidelink, LGE, 2023-03